MOD	J/58A5/1

22.5I		6)	An administration operating a non-geostationary-satellite system in the fixed-satellite service which is in compliance with the limits in Nos. 22.5C, 22.5D and 22.5F shall be considered as having fulfilled its obligations under No. 22.2 with respect to any geostationary-satellite network, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-geostationary-satellite system and the geostationary-satellite network, provided that the epfd radiated by the non-geostationary-satellite system in the fixed-satellite service into any operating geostationary fixed-satellite service earth station does not exceed the operational and additional operational limits given in Tables 22-4A, 22-4A1, 22-4B and 22-4C, when the diameter of the earth station antenna is equal to the values given in Table 22-4A, 22-4A1 or 22-4C, or the gain of the earth station is equal to or greater than the values given in Table 22-4B for the corresponding orbital inclination of the geostationary fixed-satellite service satellite. Except as otherwise agreed between concerned administrations, an administration operating a non-geostationary-satellite system in the fixed-satellite service that is subject to the limits in Nos. 22.5C, 22.5D and 22.5F and which radiates epfd into any operating geostationary fixed-satellite service earth station at levels in excess of the operational or additional operational limits given in Tables 22-4A, 22-4A1, 22-4B and 22-4C, when the diameter of the earth station antenna is equal to the values given in Table 22-4A, 22-4A1 or 22-4C, or the gain of the earth station is equal to or greater than the values given in Table 22-4B for the corresponding orbital inclination of the geostationary fixed-satellite service satellite, shall be considered to be in violation of its obligations under No. 22.2. The provisions of Resolution [XXX] (WRC-03) shall apply in the event of non-compliance with the single-entry operational and additional operational limits in Section II of Article 22 by a non-geostationary-satellite system in the fixed-satellite service that is subject to the limits in Nos. 22.5C, 22.5D and 22.5F.      (WRC-03)