MOD	IND/62/13

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 Mobile-satellite (Earth-to-space) Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Space research (space-to-Earth) |  |  |

14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) Radio astronomy 5.149

Reasons:	ITU-R studies carried out under Resolution 216 (Rev.WRC-2000) have demonstrated that appropriately designed and operated AMSS systems can operate on a secondary basis in the 14-14.5 GHz band without causing harmful interference to the primary services operating in the band. Studies have also shown the feasibility of AMSS systems sharing with systems using secondary allocations in the band and amend the Table of Frequency Allocations in Article 5. The phrase “except aeronautical mobile-satellite” may be removed from the secondary MSS allocation for 14-14.5 GHz in the Table of Frequency Allocations. No further modifications to the RR to protect other services from harmful interference would need to be incorporated as the regulatory status of a secondary service and related procedures are well defined in the Radio Regulations. Since the service is to be secondary, it is obliged to protect all primary services and pre-existing systems of secondary services can claim protection from harmful interference, and also the AMSS must accept harmful interference from primary services and pre-existing secondary services.