MOD	CHN/79/1

# 1	Limits for the interference into frequency assignments in conformity with the Regions 1 and 3 Plan or with the Regions 1 and 3 List or into new or modified assignments in the Regions 1 and 3 List

Under assumed free-space propagation conditions, the power flux-density of a proposed new or modified assignment in the List shall not exceed the value of –103.6 dB(W/(m2 · 27 MHz)).

With respect to § 4.1.1 a) or b) of Article 4, an administration in Region 1 or 3 shall be considered by the Bureau as being affected if the minimum orbital spacing between the wanted and interfering space stations, under worst-case station-keeping conditions, is less than 9°.

However, an administration shall not be considered as affected if either of the following two conditions are met:

a)	under assumed free-space propagation conditions, the power flux-density at any test point within the service area associated with any of its frequency assignments in the Plan or in the List or for which the procedure of Article 4 has been initiated, does not exceed the following values:15

–147     dB(W/(m2 · 27 MHz))	for  0°	 θ < 0.23°

–135.7 + 17.74 log θ     dB(W/(m2 · 27 MHz))	for  0.23°	 θ < 2.0°

–136.7 + 1.66 θ2      dB(W/(m2 · 27 MHz))	for  2.0°	 θ < 3.59°

–129.2 + 25 log θ     dB(W/(m2 · 27 MHz))	for  3.59°	 θ < 9°

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.

b)	the effect of the proposed new or modified assignments in the List is that the equivalent downlink protection margin16 corresponding to a test point of its assignment in the Regions 1 and 3 Plan or List, or for which the procedure of Article 4 has been initiated, including cumulative effect of any previous modification to the List or any previous agreement, does not fall more than 0.45 dB below 0 dB or, if already negative, more than 0.45 dB below the value resulting from:

–	the Regions 1 and 3 Plan and List as established by WRC-2000; or

–	a proposed new or modified assignment to the List in accordance with this Appendix; or

–	a new entry in the Regions 1 and 3 List as a result of successful application of Article 4 procedures.

NOTE – In performing the calculation, the effect at the receiver input of all the co-channel and adjacent-channel signals is expressed in terms of one equivalent co-channel interfering signal. This value is usually expressed in decibels.

1	5 	For the protection of analogue assignments brought in service before 17 October 1997, the following values shall be used until 1 January 2015:

–147     dB(W/(m2  27 MHz))			for 0°             0.44°

–138 + 25 log      dB(W/(m2  27 MHz))	for 0.44°        9°.

1	6 	For the definition of the equivalent protection margin, see § 3.4 of Annex 5.