ADD	CHN/87/3

5.ZZ	In the band 14-14.5 GHz, the power flux-density produced on the territory of China, by any aircraft earth station in the aeronautical mobile-satellite service shall not exceed, unless otherwise agreed by the affected administration:

132 + 0.5*		dB(W/(m2 in any 1 MHz))	for	   40

112			dB(W/(m2 in any 1 MHz))	for	40    90

Reasons:	To indicate the specific protection requirements of the fixed service on the territory of China.