MOD	KOR/109A6/2

5.543A 	In Bhutan, Korea (Rep. of), Indonesia, Iran (Islamic Republic of), Japan, Maldives, Mongolia, Myanmar, Pakistan, the Dem. People’s Rep. of Korea, Sri Lanka, Thailand and Viet Nam, the allocation to the fixed service in the band 31-31.3 GHz may also be used by systems using high altitude platform stations (HAPS) in the ground-to-HAPS direction. The use of the band 31-31.3 GHz by systems using HAPS shall not cause harmful interference to, nor claim protection from, other types of fixed-service systems or other co-primary services, taking into account No. 5.545. Systems using HAPS in the band 31-31.3 GHz shall not cause harmful interference to the radio astronomy service having a primary allocation in the band 31.3-31.8 GHz, taking into account the protection criterion of 141 dB(W/(m2 · 500 MHz)). In order to ensure the protection of satellite passive services, the level of unwanted emissions from a ground station of a system using HAPS into the band 31.3-31.8 GHz shall be limited to –106 dB(W/MHz) under a clear-sky condition, and may be increased up to –100 dB(W/MHz) under rainy conditions in accordance with rain attenuation, provided that effective impact on the passive services is not in excess of –106 dB(W/MHz).      (WRC-03)

Reasons:	For the operation of systems using HAPS in the FS in the band 31.0-31.3 GHz, the proposed modification of No. 5.543A is suitable for the protection of passive services in the 31.3-31.8GHz band, taking into account the result of ITU-R studies. And the Republic of Korea intends to put its country name in the No. 5.543A in order to facilitate the appropriate harmonization with neighbouring countries who are listed in the No. 5.543A and intend to implement HAPS.