MOD	VTN/112/2

# 6	Limits to the change in the power flux-density of assignments in the Regions 1 and 3 Plan or List to protect the fixed-satellite service (space-to-Earth) in the band 11.7-12.2 GHz in Region 2 or in the band 12.2-12.5 GHz in Region 3, and of assignments in the Region 2 Plan to protect the fixed-satellite service (space-to-Earth) in the band 12.5-12.7 GHz in Region 1 and in the band 12.2-12.7 GHz in Region 3

...

In the case an administration in Region 3 intends to protect the assignments of its FSS networks, of which the notification information has been submitted to BR prior to 3 June 2000, having the isotropic gain (dBi) of the receiving earth station antenna in the direction of maximum radiations of 33.3 dBi, the administration is considered as not being affected if the proposed new or modified assignment in the Region 1 List, or if a proposed modification to the Region 2 Plan, gives a power flux-density anywhere over any portion of its own territory of its overlapping frequency assignments in its FSS networks less than:

–186.5	dB(W/(m2 · 40 kHz)) 	for	0°	 θ < 0.054°

–164.0 + 17.74 log θ	dB(W/(m2 · 40 kHz))	for	0.054°	 θ < 1.8°

–162.3 + 0.89 θ2	dB(W/(m2 · 40 kHz))	for	1.8°	 θ < 5.0°

–157.5 + 25 log θ	dB(W/(m2 · 40 kHz))	for	5.0°	 θ < 10.57°

–131.9	dB(W/(m2 · 40 kHz))	for	10.57°	 θ

where θ is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.