MOD	VNT/112/3

ANNEX  4     (WRC-03)

Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not
subject to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the 
List or proposed new or modified assignments in the List for Regions 1 and 3, in Region 1 (12.5-12.7 GHz) and in
Region 3 (12.2-12.7 GHz) with respect to the Plan or 
proposed modifications to the Plan for Region 2, in Region 3 
(12.2-12.5 GHz) with respect to the Plan, the List or proposed 
new or modified assignments in the List for Region 1

(See Article 7)

...

With respect to § 7.1 and 7.2 of Article 7, coordination of a space station in the fixed-satellite service (space-to-Earth) of Region 2 is required when, under assumed free-space propagation conditions, the power flux-density on the territory over any portion of the service area of the overlapping frequency assignments in the broadcasting-satellite service of an administration in Region 3, of which the BSS Plan has national coverage only and has been brought into service before 3 June 2000, exceeds the value derived from the expressions given below:

–147 	dB(W/(m2 · 27 MHz))	for	0°	 θ < 0.23°

–135.7 + 17.74 log θ  	dB(W/(m2 · 27 MHz))	for	0.23°	 θ < 1.8°

–134.0 + 0.89 θ2 	dB(W/(m2 · 27 MHz))	for	1.8°	 θ < 5.0°

–129.2 + 25 log θ 	dB(W/(m2 · 27 MHz))	for	5.0°	 θ < 10.57°

–103.6 	dB(W/(m2 · 27 MHz))	for	10.57°	 θ

where  is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.