MOD	VTN/112/4

ANNEX  6MOD 39

Criteria for sharing between services

...

# 2	Antenna sizes40 and total noise temperatures

The range of antenna sizes and total noise temperatures considered for the protection of the fixed-satellite service and the broadcasting-satellite service are given in the following Table:

| Receive earth station antenna diameter (m) | 0.45 | 0.60 | 0.80 | 1.20 | 2.4 | 5.0 | 8.0 | 11.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Receive earth station noise temperature (K) | 110 | 110 | 125 | 150 | 150 | 200 | 250 | 250 |
| Total link noise temperature (K) | 174 | 174 | 198 | 238 | 238 | 317 | 396 | 396 |

The total link noise temperature was calculated from the receive earth station noise temperature (which includes the antenna temperature, the receive amplifier temperature and the noise increase resulting from feeder losses), and adding 2 dB for all other sources of noise (uplink noise, GSO interference, cross polarization isolation and frequency reuse interference).

...

# 4	Power flux-density to protect FSS and BSS with specific antenna diameters41

The Table below contains required power flux-density levels for the protection of FSS and BSS with the characteristics in Section 2 above using the criteria specified in Section 3 above.

| Required power flux-density (pfd) in dB(W/(m2 · 27 MHz)) corresponding to different antenna diameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbital separation between wanted and interfering space stations | 45 cm | 60 cm | 80 cm | 120 cm | 240 cm | 500 cm | 800 cm | 1 100 cm |
| 0° | 134.2 | 136.7 | 138.7 | 142.2 | 147.4 | 152.5 | 155.6 | 158.2 |
|  > 0 | For any value of the orbital separation  between the wanted and interfering space stations, the applicable pfd should be relaxed from the value corresponding to 0° orbital separation by adding the off-axis antenna discrimination, as calculated under the assumptions in Section 1 above. |  |  |  |  |  |  |  |

3	9 	Sections 1 and 2 of Part A of this Annex are applicable when the services of Regions 1 or 3 are involved. Section 3 of Part A is applicable to all Regions.

4	0	The 45 cm diameter in the range of antennas to be protected is applied to the Region 3 BSS Plan which has been brought into service before 3 June 2000, and Region 3 FSS which has been submitted the notification information to the Bureau before 3 June 2000.

4	1	The 45 cm diameter in the range of antennas to be protected is applied to the Region 3 BSS Plan which has been brought into service before 3 June 2000, and Region 3 FSS which has been brought into service before 3 June 2000.