MOD	IAP/5/130

5.552A	The allocation to the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz is designated for use by high altitude platform stations. The use of the bands 47.2-47.5 GHz and 47.9-48.2 GHz by high altitude platforms in the fixed service is subject to the provisions of No. 9.22. See Resolution [HAPS 28-31] (WRC-03).     (WRC-03)

Reasons:	Consequential to the suppression of Resolution 122. While studies have been completed, HAPS systems still need to be subject to the provisions of Article 9 to ensure coordination with the FSS at 47 GHz. The reference to Resolution [HAPS 28-31] reflects the intent of WRC-2000 that the identification of 2  300 MHz of FS spectrum at 27.5-28.35 GHz and 31-31.3 GHz in certain countries is intended as an alternative for the HAPS designation at 47 GHz which is problematic in those countries due to excessive rain attenuation. Nos. 9.17 and 9.18, which apply for the coordination of terrestrial stations, including HAPS, with earth stations, are currently applicable without having to be called out in a footnote to Article 5 of the Radio Regulations. No. 9.22 is a new provision (see IAP/5/131 below) that is intended to address the previously unaddressed coordination case of transmitting HAPS ground-based stations with receiving space station of the FSS, when the HAPS ground station appears in the coverage area of a satellite network.