| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 7 to Document 13-E |
|  |  |  | 13 March 2003 |
|  |  |  | Original: English/ French/ Spanish |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 7 |  |

Agenda item 1.7 – Amateur services

Sub-Part 7A – Agenda item 1.7.1 – Amateur services

Proposals

ARTICLE  25

Amateur services

Section I  –  Amateur service

SUP	EUR/13A7/1

25.1	§ 1

Reasons:	It is the sovereign right of each Member State to regulate its telecommunications. If an administration chooses to prohibit international communications, it should be the concern of the administration to enforce this rule and not a general obligation.

MOD	EUR/13A7/2

25.2	§ 2	1)	Transmissions between amateur stations of different countries shall be limited to communications conforming to amateur service activities as defined in No. 1.56 and to remarks of a personal character.

Reasons:	Simplification of the Radio Regulations and shortening the text of No. 25.2, which defines the content of amateur communications.

ADD	EUR/13A7/3

25.2.A		2)	Transmissions between amateur stations shall not be encoded for the purpose of obscuring their meaning, except with the authority of the relevant administration granted to meet a particular operational need.

Reasons:	Simplification of the Radio Regulations and clarification of the wording that messages should be in principle in conformity with amateur activities as defined in No. 1.56, exceptional cases excluded.

MOD 	EUR/13A7/4

25.3		2)	Amateur stations may be used for transmitting international communications on behalf of third parties only in case of emergencies or disaster situations.

Reasons:	Clarification and specific indication when third party communications are allowed.

MOD	EUR/13A7/5

25.4		3)	The preceding provisions may be modified by administrations of the countries concerned.

Reasons:	Allowing administrations to divert from provision No. 25.3 to satisfy national requirements.

MOD	EUR/13A7/6

25.5	§ 3	1)	Administrations shall determine whether or not a person seeking a licence to operate an amateur station shall prove that he is able to send and receive correctly texts in Morse code signals.

Reasons:	Tasking administrations to determine whether or not to maintain the Morse code skill in the amateur services on a national level.

MOD	EUR/13A7/7

25.6		2)	Administrations shall verify the operational and technical qualifications of any person wishing to operate an amateur station.

Reasons:	Simplification of the Radio Regulations.

SUP	EUR/13A7/8

25.7	§ 4

Reasons:	Simplification of the Radio Regulations and a redundant provision. See No. 15.2, which provides that “Transmitting stations shall radiate only as much power as is necessary to ensure a satisfactory service”.

SUP	EUR/13A7/9

25.8	§ 5	1)

Reasons:	Simplification of the Radio Regulations and a redundant provision. The text reiterates concepts included generically in the Radio Regulations and applicable to all radio services.

NOC	EUR/13A7/10

25.9		2)	During the course of their transmissions, amateur stations shall transmit their call sign at short intervals.

ADD	EUR/13A7/11

25.9A 		Administrations are advised to take the necessary steps to allow amateur stations to prepare for and meet communication needs in the event of a disaster.

Reasons:	Recognition of the value of amateur communications during disaster situations.

Section II  –  Amateur-satellite service

NOC	EUR/13A7/12

25.10	§ 6	The provisions of Section I of this Article shall apply equally, as appropriate, to the amateur-satellite service.

MOD	EUR/13A7/13

25.11	§ 7	Administrations authorizing space stations in the amateur-satellite service shall ensure that sufficient earth command stations are established before launch to guarantee that any harmful interference caused by emissions from a station in the amateur-satellite service can be terminated immediately (see No. 22.1).

Reasons:	Simplification of the Radio Regulations.

Sub-Part 7B – Agenda item 1.7.2 – Identification of stations

Proposals

ARTICLE  19

Identification of stations

Section I  –  General provisions

NOC	EUR/13A7/14

19.1 to 19.28

Section II  –  Allocation of international series and assignment of call signs

NOC	EUR/13A7/15

19.28A to 19.44

Section III  –  Formation of call signs

NOC 	EUR/13A7/16

19.45 to 19.48

SUP	EUR/13A7/17

19.49 	c)

Reasons:	At the present time, some countries cannot have amateur call signs because of the restriction imposed by No. 19.49 when the letters O or I are used in the national identifier. Modes of radiocommunication in current use in the amateur services are such that there is no difficulty distinguishing between the numbers 0 and 1, and the letters O and I respectively.

Suppression of No. 19.49 c) would remove the restriction for some identifiers, thus adding more flexibility for administrations, especially those unable to have amateur call signs at present.

NOC	EUR/13A7/18

19.50 to 19.66

NOC	EUR/13A7/19

19.67		Amateur and experimental stations

MOD	EUR/13A7/20

19.68	§ 30	1)

–	one character (see No. 19.50.1) and a single digit (other than 0 or 1), followed by a group of not more than four characters, the last of which shall be a letter, or

–	two characters and a single digit (other than 0 or 1), followed by a group of not more than four characters, the last of which shall be a letter.

Reasons:	No. 19.68 in its present form limits amateur and experimental stations call-sign suffixes to “a group of not more than three letters”. This restriction places a limit on the number of possible call-sign combinations and their formulation and prohibits the use of certain combinations for special events.

It is therefore proposed to revise the present Article in such a way that more flexibility is allowed for administrations to issue call signs.

Such a change would considerably expand the number of possible call-sign combinations and provide administrations with increased flexibility without creating conflict with the call-sign formats specified for stations in other services.

NOC	EUR/13A7/21

19.69		2)	However, the prohibition of the use of the digits 0 and 1 does not apply to amateur stations.

ADD	EUR/13A7/22

19.69A 		3)	In special occasions for temporary use, administrations may waive the requirement of the call sign containing not more than four trailing characters.

Reasons:	Providing additional flexibility for administrations.

NOC	EUR/13A7/23

19.70 to 19.71

Section IV  –  Identification of stations using radiotelephony

MOD	EUR/13A7/24

19.72	§ 32	Stations using radiotelephony shall be identified as indicated in Nos. 19.73 to 19.82A.

Reasons: Update of the Radio Regulations.

NOC	EUR/13A7/25

# 19.73 to 19.82

ADD	EUR/13A7/26

19.82A 	§ 35A	Amateur stations and experimental stations

–	a call sign (see No. 19.68).

Reasons:	Update of the Radio Regulations.

Sub-Part 7C – Agenda item 1.7.3 – Review of the terms and definitions
 of Article 1 to the extent required as a consequence of 
changes made in Article 25

Proposals

ARTICLE  1

Terms and definitions

Section III  –  Radio services

NOC	EUR/13A7/27

1.56		amateur service:  A radiocommunication service for the purpose of self-training, intercommunication and technical investigations carried out by amateurs, that is, by duly authorized persons interested in radio technique solely with a personal aim and without pecuniary interest.

NOC 	EUR/13A7/28

1.57		amateur-satellite service:  A radiocommunication service using space stations on earth satellites for the same purposes as those of the amateur service.

Reasons:	Studies conducted to date have not indicated the need for any consequential changes to Article 1.

______________