| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 8 to Document 13-E |
|  |  |  | 13 March 2003 |
|  |  |  | Original: English/ French/ Spanish |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 8 |  |

Agenda item 1.8

SUB-PART 8A

Agenda item 1.8.1 – Issues related to unwanted emissions, 
boundary between spurious and out-of-band emissions

Introduction

WRC-03 agenda item 1.8.1 invites the Conference to consider the boundary between the spurious and out-of-band emissions with a view to include the boundary in Appendix 3.

The proposal herein amends Articles 1 and 3 and Appendix 3 to take into account the most recent studies identifying the unwanted emissions to which the limits of Appendix 3 Section II apply. These studies define the out-of-band and spurious domains of an emission and determine the boundary between them:

–	Recommendation ITU-R SM.329-10, “Unwanted emissions in the spurious domain”;

–	Recommendation ITU-R SM.1541-1, “Unwanted emission in the out-of-band domain";

–	Recommendation ITU-R SM.1539-1, “Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329”.

Based on the results of the studies undertaken in Task Group 1/5, Europe proposes the modification of Articles 1 and 3 and the modification of Appendix 3 by providing specific exceptions from the general 2.5BN boundary based on Recommendations ITU-R SM.1539 and ITU-R 1541 (Method A in the CPM Report).

The sequence of the text contained in Section II, existing paragraphs 8-10 in Appendix 3 of the Radio Regulations (2001), has been altered for clarity and in order to address potential ambiguities in the interpretation of the reference bandwidths relating to the various services. As a related issue on the agenda item, a reference bandwidth for a further type of radar has also been included subsequent to work undertaken in WP 8B.

Proposals

ARTICLE  1

Terms and definitions

Section VI  –  Characteristics of emissions and radio equipment

ADD	EUR/13A8/1

1.146bis		out-of-band domain (of an emission): The frequency range, immediately outside the necessary bandwidth but excluding the spurious domain, in which out-of-band emissions generally predominate.

Reasons:	Out-of-band emissions, defined based on their source, occur in the out-of-band domain and, to a lesser extent, in the spurious domain. Spurious emissions likewise may occur in the out-of-band domain as well as in the spurious domain.

ADD	EUR/13A8/2

1.146ter		spurious domain (of an emission): The frequency range beyond the out-of-band domain in which spurious emissions generally predominate.

Reasons:	Adoption of these two definitions will provide a means to distinguish between frequency ranges within which the emission limits of Appendix 3, Section II either apply or do not apply.

ARTICLE  3

Technical characteristics of stations

MOD	EUR/13A8/3

3.6		Transmitting stations shall conform to the maximum permitted power levels specified in Appendix 3.

MOD	EUR/13A8/4

3.7		Transmitting stations shall conform to the maximum permitted power levels for out-of-band emissions, or unwanted emissions in the out-of-band domain, specified for certain services and classes of emission in the present Regulations. In the absence of such specified maximum permitted power levels transmitting stations should, to the maximum extent possible, satisfy the requirements relating to the limitation of the out-of-band emissions, or unwanted emissions in the out-of-band domain, specified in the most recent ITU-R Recommendations (see Resolution 27 (Rev. WRC-97)*).

Reasons:	Slight modification of Nos. 3.6 and 3.7 to accommodate emissions specified using both the existing definitions of out-of-band emission and spurious emission, and the definitions proposed above.

MOD	EUR/13A8/5

APPENDIX  3  (Rev.WRC-03)

Table of maximum permitted
 power levels for spurious or spurious domain emissions

(See Article 3)

Reasons:	Section I of this Appendix applies to spurious emissions, while Section II applies to unwanted emissions in the spurious domain. The proposed title encompasses both types of emission limits.

1	The following sections indicate the maximum permitted levels of certain unwanted emissions, in terms of power as indicated in the tables, of components supplied by a transmitter to the antenna transmission line. Section I, which provides spurious emissions limits, is applicable until 1 January 2012 to transmitters installed on or before 1 January 2003; Section II, which limits emissions in the spurious domain, is applicable to transmitters installed after 1 January 2003 and to all transmitters after 1 January 2012. The provisions of No. 4.5 apply to unwanted emissions not covered in Sections I or II.

2	Spurious and spurious domain emissions (covered by Sections I and II) from any part of the installation, other than the antenna and its transmission line, shall not have an effect greater than would occur if this antenna system were supplied with the maximum permitted power at the frequency of that emission.

3	These levels shall not, however, apply to emergency position-indicating radiobeacon (EPIRB) stations, emergency locator transmitters, ships’ emergency transmitters, lifeboat transmitters, survival craft stations or maritime transmitters when used in emergency situations.

4	For technical or operational reasons, more stringent levels than those specified may be applied to protect specific services in certain frequency bands. The levels applied to protect these services, such as safety and passive services, shall be those agreed upon by the appropriate world radiocommunication conference. More stringent levels may also be fixed by specific agreement between the administrations concerned. Additionally, special consideration of transmitter spurious or spurious domain emissions may be required for the protection of safety services, radio astronomy and space services using passive sensors. Information on the levels of interference detrimental to radio astronomy, Earth exploration satellites and meteorological passive sensing is given in the most recent version of Recommendation ITU-R SM.329.

5	Spurious or spurious domain emission limits (covered by Sections I and II) for combined radiocommunication and information technology equipment are those for the radiocommunication transmitters.

Reasons:	The revised paragraphs reflect the distinction between the types of emissions to which the limits of Sections I and II apply.

NOC

Section I  –  Spurious emission limits for transmitters installed on 
or before 1 January 2003 (valid until 1 January 2012)

MOD	EUR/13A8/6

Section II  –  Spurious domain emission limits for transmitters installed after 1 January 2003 
and for all transmitters after 1 January 2012

Application of these limits

7	The frequency range of the measurement of spurious domain emissions is from 9 kHz to 110 GHz or the second harmonic if higher.

Reasons:	The provisions of this section apply to unwanted emissions in the spurious domain, here called “spurious domain emissions,” as opposed to the spurious emissions addressed in Section I.

8	The spurious domain emission levels are specified in the following reference bandwidths:

–	1 kHz between 9 kHz and 150 kHz

–	10 kHz between 150 kHz and 30 MHz

–	100 kHz between 30 MHz and 1 GHz

–	1 MHz above 1 GHz.

Reasons:	For clarity, the reference bandwidths relating to most of the services are best placed as part of the initial text in this section and are transferred from the existing paragraph 10. In addition, the provisions of this section apply to unwanted emissions in the spurious domain, here called “spurious domain emissions,” as opposed to the spurious emissions addressed in Section I.

9	As a special case, the reference bandwidth of all space service spurious domain emissions should be 4 kHz.

Reasons:	For clarity, the above text warrants a separate paragraph number and is transferred from the existing paragraph 10. In addition, the provisions of this section apply to unwanted emissions in the spurious domain, here called “spurious domain emissions,” as opposed to the spurious emissions addressed in Section I.

10	As a special case for radar systems, the reference bandwidths for defining spurious domain emission levels should be calculated for each particular system. Thus, for the four general types of radar pulse modulation utilized for radionavigation, radiolocation, acquisition, tracking and other radiodetermination functions, the reference bandwidth values are determined using the following:

–	for fixed-frequency, non-pulse-coded radar, one divided by the radar pulse length, in seconds (e.g. if the radar pulse length is 1 s, then the reference bandwidth is 1/1 s  1 MHz);

–	for fixed-frequency, phase coded pulsed radar, one divided by the phase chip length, in seconds (e.g. if the phase coded chip is 2 s long, then the reference bandwidth is 1/2 s  500 kHz);

–	for frequency modulated (FM) or chirped radar, the square root of the quantity obtained by dividing the radar bandwidth in MHz by the pulse length, in microseconds (e.g. if the FM is from 1 250 MHz to 1 280 MHz or 30 MHz during the pulse of 10 s, then the reference bandwidth is (30 MHz/10 s)1/2  1.73 MHz);

–	for radars operating with multiple waveforms the reference bandwidth is determined empirically from observations of the radar emission and are obtained following the guidance given in Recommendation ITU-R M.1177;

–	in the case of radars, where the bandwidths determined as above, are greater than 1 MHz, then a reference bandwidth of 1 MHz should be used.

Reasons:	This assists in clarification of the reference bandwidths for defining limits of spurious domain emissions and maintains consistency with the existing version of RR Appendix 3. Moreover, in cases where previously stipulated reference bandwidths exceeding 1 MHz could not be directly used as the measurement bandwidth, since they are not commonly available on spectrum analysers, it is proposed to use a maximum of 1 MHz reference bandwidth. Additionally, measurement bandwidths are specified within Recommendation ITU-R M.1177. Based on the output of the last WP 8B meeting, it is proposed to cover the case of multiple waveform radars in Appendix 3 (fourth bullet above). The text below the bullets has been deleted since the revised version of Recommendation ITU-R M.1177 provides acceptable measurement methods for all types of radars. Text has been transferred from existing paragraph 9. In addition, the provisions of this Section apply to unwanted emissions in the spurious domain, here called “spurious domain emissions”, as opposed to the spurious emissions addressed in Section I.

10bis	Guidance regarding the methods of measuring spurious domain emissions is given in the most recent version of Recommendation ITU-R SM.329. The e.i.r.p. method specified in that Recommendation should be used when it is not possible to accurately measure the power supplied to the antenna transmission line, or for specific applications where the antenna is designed to provide significant attenuation in the spurious domains. Additionally, the e.i.r.p. method may need some modification for special cases.     (WRC-03)

Specific guidance regarding the methods of measuring spurious domain emissions from radar systems is given in the most recent version of Recommendation ITU-R M.1177.

Reasons:	Text for guidance from both existing paragraphs 8 and 9 are best combined. In addition, the provisions of this section apply to unwanted emissions in the spurious domain, here called “spurious domain emissions”, as opposed to the spurious emissions addressed in Section I.

11	The emission limits of this section apply to all emissions, including harmonic emissions, intermodulation products, frequency conversion products and parasitic emissions, at frequencies in the spurious domain (see Fig. 1). The upper and lower parts of the spurious domain extend outward from a boundary determined using Annex 1.

Reasons:	Since the boundary between the out-of-band and spurious domains is determined using Annex 1, the information is no longer needed here.

________________________________________________________________________________

Reasons:	The Figure illustrates the text description of the locations of the out-of-band and spurious domains from the previous paragraph.

-

Reasons:	The information used to determine the boundary between the out-of-band and spurious domains is now found in Annex 1.

11bis	For the case of a single satellite operating with more than one transponder in the same service area, and when considering the limits for spurious domain emissions as indicated in § 11 of this Appendix, spurious domain emissions from one transponder may fall on a frequency at which a second, companion transponder is transmitting. In these situations, the level of spurious domain

emissions from the first transponder is well exceeded by the fundamental or out-of-band domain emissions of the second transponder. Therefore, the limits of this Appendix should not apply to those emissions of a satellite that fall within either the necessary bandwidth or the out-of-band domain of another transponder on the same satellite, in the same service area (see Fig. 2).

Transponders A, B, C and D are operating on the same satellite in the same service area. Transponder A is not required to meet spurious domain emission limits in frequency ranges  and , but is required to meet them in frequency ranges  and .     (WRC-03)

# 12	Examples of applying 43  10 log (P) to calculate attenuation requirements

Where specified in relation to mean power, spurious domain emissions are to be at least x dB below the total mean power P, i.e. –  x dBc. The power P (W) is to be measured in a bandwidth wide enough to include the total mean power. The spurious domain emissions are to be measured in the reference bandwidths given in the Recommendation. The measurement of the spurious domain emission power is independent of the value of necessary bandwidth. Because the absolute emission power limit, derived from 43  10 log (P), can become too stringent for high-power transmitters, alternative relative powers are also provided in Table II.

Example 1

A land mobile transmitter, with any value of necessary bandwidth, must meet a spurious domain emission attenuation of 43  10 log (P), or 70 dBc, whichever is less stringent. -The reference bandwidths used to define the limits for spurious domain emissions are defined in § 8 to § 10 of this Appendix. Applying this is in the frequency range between 30 MHz and 1 GHz gives a reference bandwidth of 100 kHz.

With a measured total mean power of 10 W:

–	Attenuation relative to total mean power  43  10 log (10)  53 dBc.

–	The 53 dBc value is less stringent than the 70 dBc, so the 53 dBc value is used.

–	Therefore: Spurious domain emissions must not exceed 53 dBc in a 100 kHz bandwidth, or converting to an absolute level, they must not exceed 10 dBW – 53 dBc  _ 43 dBW in a 100 kHz reference bandwidth.

With a measured total mean power of 1 000 W:

–	Attenuation relative to total mean power = 43 + 10 log (1 000)  73 dBc.

–	The 73 dBc value is more stringent than the 70 dBc limit, so the 70 dBc value is used.

–	Therefore: Spurious domain emissions must not exceed 70 dBc in a 100 kHz bandwidth, or converting to an absolute level, they must not exceed 30 dBW – 70 dBc  _ 40 dBW in a 100 kHz reference bandwidth.

Example 2

A space service transmitter with any value of necessary bandwidth must meet a spurious domain emission attenuation of 43  10 log (P), or 60 dBc, whichever is less stringent. To measure spurious domain emissions at any frequency, Note 10 to Table II indicates using a reference bandwidth of 4 kHz.

With a measured total mean power of 20 W:

–	Attenuation relative to total mean power  43  10 log (20)  56 dBc.

–	The 56 dBc value is less stringent than the 60 dBc limit, so the 56 dBc value is used.

–	Therefore: Spurious domain emissions must not exceed 56 dBc in a 4 kHz reference bandwidth, or converting to an absolute level, they must not exceed 13 dBW – 56 dBc  _ 43 dBW in a 4 kHz reference bandwidth.

TABLE  II     (WRC-03)

Attenuation values used to calculate maximum permitted spurious domain emission 
power levels for use with radio equipment

| Service category in accordance with Article 1, or equipment type15 | Attenuation (dB) below the power supplied to the antenna transmission line |  |
| --- | --- | --- |
| All services except those services quoted below: | 43  10 log (P), or 70 dBc, whichever is less stringent |  |
| Space services (earth stations)10, 16 | 43  10 log (P), or 60 dBc, whichever is less stringent |  |
| Space services (space stations)10, 17 | 43  10 log (P), or 60 dBc, whichever is less stringent |  |
| Radiodetermination14 | 43  10 log (PEP), or 60 dB, whichever is less stringent |  |
| Broadcast television11 | 46  10 log (P), or 60 dBc, whichever is less stringent, without exceeding the absolute mean power level of 1 mW for VHF stations or 12 mW for UHF stations. However, greater attenuation may be necessary on a case by case basis |  |
| Broadcast FM | 46  10 log (P), or 70 dBc, whichever is less stringent; the absolute mean power level of 1 mW should not be exceeded |  |
| Broadcasting at MF/HF | 50 dBc; the absolute mean power level of 50 mW should not be exceeded |  |
| SSB from mobile stations12 | 43 dB below PEP |  |
| Amateur services operating below 30 MHz (including those using SSB)16 | 43  10 log (PEP), or 50 dB, whichever is less stringent |  |
| Services operating below 30 MHz, except space, radiodetermination, broadcast, those using SSB from mobile stations, and amateur12 | 43  10 log (X), or 60 dBc, whichever is less stringent, where X  PEP for SSB modulation, and X  P for other modulation |  |
| Low-power device radio equipment13 | 56  10 log (P), or 40 dBc, whichever is less stringent |  |
| Emergency transmitters18 | No limit |  |
| P: mean power in watts supplied to the antenna transmission line, in accordance with No. 1.158. When burst transmission is used, the mean power P and the mean power of any spurious domain emissions are measured using power averaging over the burst duration. |  |  |
| PEP: peak envelope power in watts supplied to the antenna transmission line, in accordance with No. 1.157. |  |  |

TABLE II (end) dBc: decibels relative to the unmodulated carrier power of the emission. In the cases which do not have a carrier, for example in some digital modulation schemes where the carrier is not accessible for measurement, the reference level equivalent to dBc is decibels relative to the mean power P.

10 Spurious domain emission limits for all space services are stated in a 4 kHz reference bandwidth. 11 For analogue television transmissions, the mean power level is defined with a specified video signal modulation. This video signal has to be chosen in such a way that the maximum mean power level (e.g. at the video signal blanking level for negatively modulated television systems) is supplied to the antenna transmission line. 12 All classes of emission using SSB are included in the category “SSB”. 13 Low-power radio devices having a maximum output power of less than 100 mW and intended for short-range communication or control purposes; such equipment is in general exempt from individual licensing. 14 For radiodetermination systems (radar as defined by No. 1.100), spurious domain emission attenuation (dB) shall be determined for radiated emission levels, and not at the antenna transmission line. The measurement methods for determining the radiated spurious domain emission levels from radar systems should be guided by Recommendation ITU-R M.1177. (WRC-03) 15 In some cases of digital modulation (including digital broadcasting), broadband systems, pulsed modulation and narrow-band high-power transmitters for all categories of services, there may be difficulties in meeting limits close to  250% of the necessary bandwidth. 16 Earth stations in the amateur-satellite service operating below 30 MHz are in the service category “Amateur services operating below 30 MHz (including those using SSB)”. (WRC-2000) 17 Space stations in the space research service intended for operation in deep space as defined by No. 1.177 are exempt from spurious domain emission limits. (WRC-03) 18 Emergency position-indicating radio beacon, emergency locator transmitters, personal location beacons, search and rescue transponders, ship emergency, lifeboat and survival craft transmitters and emergency land, aeronautical or maritime transmitters. (WRC-2000)

ADD	EUR/13A8/7

Annex  1

Determination of the boundary between the out-of-band
and spurious domains

1	In general the boundary between the out-of-band and spurious domains occurs at frequencies that are separated from the centre frequency of the emission by 250% of the necessary bandwidth.

2	Except as provided below, the boundary between the out-of-band and spurious domains occurs at frequencies that are separated from the centre frequency of the emission by the values shown in Table 1. For most systems, the centre frequency of the emission is the centre of the necessary bandwidth. For multichannel or multicarrier transmitters/transponders, where several carriers may be transmitted simultaneously from a final output amplifier or an active antenna, the centre frequency of the emission is taken to be the centre of the 3 dB bandwidth of the transmitter or transponder and the transmitter or transponder bandwidth is used in place of the necessary bandwidth for determining the boundary. Some systems specify unwanted emissions relative to channel bandwidth, or channel spacing. These may be used as a substitute for the necessary bandwidth in Table 1, provided they are found in ITU-R Recommendations.

TABLE  1

Values for frequency separation between the centre frequency 
and the boundary of the spurious domain

| Frequency range | Narrow-band case |  | Normal separation | Wideband case |  |
| --- | --- | --- | --- | --- | --- |
|  | for BN < | Separation |  | for BN > | Separation |
| 9 kHz < fc < 150 kHz | 250 Hz | 625 Hz | 2.5 BN | 10 kHz | 1.5 BN + 10 kHz |
| 150 kHz < fc < 30 MHz | 4 kHz | 10 kHz | 2.5 BN | 100 kHz | 1.5 BN + 100 kHz |
| 30 MHz < fc < 1 GHz | 25 kHz | 62.5 kHz | 2.5 BN | 10 MHz | 1.5 BN + 10 MHz |
| 1 GHz < fc < 3 GHz | 100 kHz | 250 kHz | 2.5 BN | 50 MHz | 1.5 BN + 50 MHz |
| 3 GHz < fc < 10 GHz | 100 kHz | 250 kHz | 2.5 BN | 100 MHz | 1.5 BN + 100 MHz |
| 10 GHz < fc < 15 GHz | 300 kHz | 750 kHz | 2.5 BN | 250 MHz | 1.5 BN + 250 MHz |
| 15 GHz < fc < 26 GHz | 500 kHz | 1.25 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |
| fc > 26 GHz | 1 MHz | 2.5 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |
| NOTE – In Table 1, fc is the centre frequency of the emission and BN is the necessary bandwidth. If the assigned frequency band of the emissions extends across two frequency ranges, then the values corresponding to the higher frequency range shall be used for determining the boundary. |  |  |  |  |  |

Example 1: The necessary bandwidth of an emission at 26 MHz is 1.8 kHz. Since 2.5 BN is only 4.5 kHz, the minimum separation applies. The spurious domain begins 10 kHz each side of the centre of the necessary bandwidth.

Example 2: The necessary bandwidth of an emission at 8 GHz is 200 MHz. Since the wideband case applies for BN > 100 MHz at that frequency, the spurious domain begins 400 MHz each side of the centre of the necessary bandwidth. Using the general separation formula, the out-of-band domain would have extended to 2.5  200 MHz  500 MHz either side of the centre frequency.

3	Tables 2 and 3 show exceptions to Table 1 for narrow-band and wideband cases, respectively, applicable to particular systems or services and frequency bands.

TABLE 2

Narrow-band variations for systems or services and frequency bands

| System or service | Frequency range | Narrow-band case |  |
| --- | --- | --- | --- |
|  |  | for BN < (kHz) | Separation (kHz) |
| Fixed service | 14 kHz-1.5 MHz | 20(1) | 50 |
|  | 1.5-30 MHz | 80(2) | 200 |
| (1) This is based on an assumption that the maximum value of the necessary bandwidth is about 3 kHz for the frequency range 14 kHz-1.5 MHz. The value of 50 kHz separation is extremely large as compared with the necessary bandwidth. It is because unwanted emissions of high power transmitters under modulated conditions have to be below the spurious limit (70 dBc) at the boundary between the out-of-band and spurious domains. (2) This is based on an assumption that the maximum value of the necessary bandwidth is about 12 kHz for the frequency range 1.5-30 MHz. The value of 200 kHz separation is extremely large as compared with the necessary bandwidth. It is because unwanted emissions of high power transmitters under modulated conditions have to be below the spurious limit (70 dBc) at the boundary between the out-of-band and spurious domains. Also, if future systems in the fixed service operating in this frequency range require a necessary bandwidth larger than 12 kHz, it may become necessary to review the 200 kHz separation. It should be noted that for medium or low power transmitters (e.g. below 1 kW), a smaller value may be appropriate as the minimum separation. This matter requires further study. |  |  |  |

TABLE 3

Wideband variations for systems or services and frequency bands

| System or service | Frequency range | Wideband case |  |
| --- | --- | --- | --- |
|  |  | for BN > | Separation |
| Fixed service | 14-150 kHz | 20 kHz | 1.5 BN + 20 kHz |
| Fixed-satellite service (FSS) | 3.4-4.2 GHz | 250 MHz | 1.5 BN + 250 MHz |
| FSS | 5.725-6.725 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 7.25-7.75 GHz and 7.9-8.4 GHz | 250 MHz | 1.5 BN + 250 MHz |
| FSS | 10.7-12.75 GHz | 500 MHz | 1.5 BN + 500 MHz |
| Broadcasting-satellite service (BSS) | 11.7-12.75 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 12.75-13.25 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 13.75-14.8 GHz | 500 MHz | 1.5 BN + 500 MHz |

4	For primary radar systems, the variation of the boundary between the out-of-band and spurious domains is described in Recommendation ITU-R SM.1541.

Reasons:	Annex 1 is added for the following reasons:

–	Section II of this Appendix states that the emission limits apply to unwanted emissions in the spurious domain. This Annex is needed to determine the boundary between the out-of-band and spurious domains, and thus the frequencies to which the emission limits of Section II apply.

–	Table 1, taken from Recommendation ITU-R SM.1539, shows the normal boundary of 2.5 BN, along with the narrow-band and wideband exception. The information in the Recommendation, along with the text of existing paragraphs 11 and 11bis, have been included, though they have been shortened to bring them in line with the form of other Appendices.

–	Tables 2 and 3 are also taken from Recommendation ITU-R SM.1539.

–	In case of radar systems the boundary depends on the type of radar which is being considered; this variation is described in Recommendations ITU-R SM.1541 and ITU-R SM.1539.

SUB-PART 8B

Agenda item 1.8.2 – Issues related to the protection of passive services

# 1	Introduction

According to WRC-03 agenda item 1.8.2, it should be studied if, in bands allocated to the passive services, more stringent spurious emission limits than those given in Appendix 3 of the Radio Regulations are required, while also assessing the impact on all concerned services of implementing or not implementing such limits.

## 1.1	Protection of bands allocated to the Earth exploration-satellite service (passive) with a primary status

For the EESS (passive), the studies on  the compatibility analysis with space active services have indicated that the probability of interference from unwanted emissions is often above the percentage of time during which the interference criteria may be exceeded.

In the case of terrestrial services, the ITU-R “band-by-band” studies have shown that the EESS (passive) protection criteria are met in some bands without restrictions on the deployment of terrestrial systems or in other bands using appropriate mitigation techniques. These mitigation techniques should not place any unacceptable constraints on the terrestrial services concerned. It is proposed that WRC-03 consider whether the results of these “band-by-band” studies are appropriate to be included within the Radio Regulations.

It is additionally proposed to identify bands where further work is needed and where regulatory measures may be needed at WRC-07. The additional work with respect to the specific frequency band pairs currently identified, should be completed by WRC-07.

## 1.2	Protection of bands allocated to the radio astronomy service with a primary status

For radio astronomy, the compatibility analysis with active services has shown that any potential interference as a result of unwanted emissions from terrestrial services can be resolved on a national basis. However, in some cases there are discrepancies between levels of unwanted emissions specified in the design phase of space stations and the requirements of the radio astronomy service in order to meet their protection requirements. ITU-R conducted “band-by-band” studies in order to assess the compatibility between the passive service operating in identified bands and the active services operating in adjacent bands. Based on these studies it is proposed to specify limits for some bands where studies in the draft new Recommendation [BbB] have concluded that the RAS protection criteria may be met and that the limits would not unduly constrain the development of active services. In some other bands it is proposed to set up threshold levels. It is additionally proposed to identify bands where further work is needed and where regulatory measures may be needed at WRC-07.

# 2	Proposals

## 2.1	On the protection of the Earth exploration-satellite (passive) service (EESS)

This section provides regulatory solutions to implement limits within Article 5 to protect the Earth exploration-satellite (passive) service bands from unwanted emission of active services. It is additionally proposed to insert a Resolution to identify bands where further studies should be done.

ARTICLE  5

Frequency allocations

MOD	EUR/13A8/8

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 31-31.3 FIXED 5.543A ADD 5.XX1 MOBILE Standard frequency and time signal-satellite (space-to-Earth) Space research 5.544 5.545 5.149 |  |  |

ADD	EUR/13A8/9

5.XX1	In order to avoid unacceptable interference to the Earth exploration-satellite (passive) service in the band 31.3-31.5 GHz, the unwanted emissions power delivered to the antenna of any station in the fixed service operating in the band 31.0-31.3 GHz shall be limited to –38 dBW in a 100 MHz reference bandwidth in the band 31.3-31.5 GHz (see also No. 5.543A).

Reasons:	This footnote is related to No. 5.543A as proposed to be modified in EUR/13A13/3.

MOD	EUR/13A8/10

51.4-55.78 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 51.4-52.6 FIXED ADD 5.XX2 MOBILE 5.547 5.556 |  |  |

ADD	EUR/13A8/11

5.XX2	In order to avoid unacceptable interference to the Earth exploration-satellite (passive) service in the band 52.6-54.25 GHz, the unwanted emissions power delivered to the antenna of any station in the fixed service operating in the band 51.4-52.6 GHz shall be limited to –33 dBW in a 100 MHz reference bandwidth in the band 52.6-54.25 GHz.

ADD	EUR/13A8/12

DRAFT  RESOLUTION  [EUR/13A8-1]  [EESS_PROTECTION_1.4 GHz]
(WRC-03)

Compatibility analysis between the Earth exploration-satellite (passive) service and other services around 1 400 MHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that unwanted emissions from active services may cause unacceptable interference to the Earth exploration-satellite service (passive);

b)	that the frequency band 1 400-1 427 MHz is allocated to the Earth exploration-satellite (passive) service, radio astronomy and space research (passive) services on a primary basis;

c)	that the frequency band 1 350-1 400 MHz is allocated to the fixed, mobile and radiolocation services on a primary basis;

d)	that the frequency band 1 427-1 429 MHz is allocated to the fixed, mobile except aeronautical and space operation (Earth-to-space) services on a primary basis;

e)	that the frequency band 1 429-1 452 MHz is allocated to the fixed and mobile on a primary basis;

f)	that, for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect the Earth exploration-satellite (passive) service in specific bands;

g)	that the protection of the band 1 400-1 427 MHz is essential since it provides the capability to measure soil moisture and sea surface salinity on a global basis and continuously in time;

h)	that two Earth exploration-satellite (passive) service systems are being designed to operate in the band 1 400-1 427 MHz,

recognizing

a)	that Recommendation ITU-R SM.[BbB] (band-by-band) does not address the protection of the Earth exploration-satellite (passive) service operating in the band 1 400-1 427 MHz from unwanted emissions of fixed and mobile services operating in the bands 1 350 to 1 400 MHz and 1 427 to 1 452 MHz;

b)	that additional provisions may be required to protect the Earth exploration-satellite (passive) service in the band 1 400-1 427 MHz from unwanted emissions of active services operating in the bands 1 350 to 1 400 MHz and 1 427 to 1 452 MHz;

c)	that it is necessary to ensure an equitable burden sharing for achieving compatibility between active and passive services,

resolves

1	to invite ITU-R to conduct compatibility analyses between Earth exploration-satellite (passive) service operating in the band 1 400-1 427 MHz and actives services operating in the bands 1 350 to 1 400 MHz and 1 427 to 1 452 MHz;

2	to recommend that WRC-07 review the results of these studies and consider regulatory measures to ensure the protection of the Earth exploration-satellite (passive) service operating in the band 1 400-1 427 MHz from unwanted emissions of active services operating in adjacent or nearby bands taking into account the impact on all concerned services of implementing or not implementing such measures,

urges administrations

1	to provide the relevant characteristics of systems of active services operating in the bands 1 350 to 1 400 MHz and 1 427 to 1 452 MHz;

2	to contribute to the compatibility studies between the fixed, mobile and radiolocation services and Earth exploration-satellite (passive) service around 1 400 MHz.

ADD	EUR/13A8/13

DRAFT  RESOLUTION  [EUR/13A8-2/EESS_PROTECTION_23, 31, 50 GHz]
(WRC-03)

Compatibility analyses between the Earth exploration-satellite (passive) service
and inter-satellite and fixed-satellite services around
23.6 GHz, 31.3 GHz and 50.2 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that unwanted emissions from active services may cause unacceptable interference to the Earth exploration-satellite service (passive);

b)	that the frequency bands 23.6-24, 31.3-31.5 GHz are allocated to the Earth exploration-satellite (passive) service, radio astronomy and space research (passive) services on a primary basis;

c)	that the frequency band 50.2-50.4 GHz is allocated to the Earth exploration-satellite (passive) service and space research (passive) services on a primary basis;

d)	that the frequency band 22.55-23.55 GHz is allocated to the inter-satellite service on a primary basis;

e)	that the frequency band 30-31 GHz is allocated to the fixed-satellite (Earth-to-space) service on a primary basis;

f)	that the frequency band 47.2-50.2 GHz is allocated to the fixed-satellite (Earth-to-space) service on a primary basis;

g)	that the frequency band 50.4-51.4 GHz is allocated to the fixed-satellite (Earth-to-space)  service on a primary basis;

h)	that for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect the Earth exploration-satellite (passive) service in specific bands;

i)	that the protection of the passive bands 23.6-24 GHz, 31.3-31.5 GHz and 50.2-50.4 GHz is essential since those bands provide measurements on the water vapour content, temperature (associated with a window channel) on a global basis and continuously in time;

j)	that the corresponding Earth exploration-satellite (passive) service systems are already operating in the above passive bands;

k)	that WRC-2000 adopted Recommendation 66 on the studies of the maximum permitted levels of unwanted emissions,

recognizing

a)	that Recommendation ITU-R SM.[BbB] (band-by-band) already addressed the protection of the Earth exploration-satellite (passive) service operating in the above passive bands from unwanted emissions of space services operating in the bands identified in considering d), e), f) and g);

b)	that corresponding annexes in Recommendation ITU-R SM.[BbB] (band-by-band) need further refinement;

c)	that additional provisions may be required to protect the Earth exploration-satellite (passive) service in the above passive bands from unwanted emissions of actives services operating in the bands identified in considering d), e), f) and g);

d)	that it is necessary to ensure an equitable burden sharing for achieving compatibility between active and passive services,

resolves

1	to invite ITU-R to consider the existing compatibility analyses between Earth exploration-satellite (passive) service operating in the above bands and active services operating in the bands identified in considering d), e), f) and g);

2	to recommend that WRC-07 review the results of these studies in order to consider regulatory measures to ensure the protection of the Earth exploration-satellite (passive) service operating in the above passive bands from unwanted emissions of active services operating in the bands identified in considering d), e), f), and g), taking into account the impact on all concerned services of implementing or not implementing such measures,

urges administrations

1	to provide the relevant characteristics of systems of active services operating in the bands identified in considering d), e), f) and g) and of passive systems operating in the above-mentioned passive bands;

2	to contribute to the review of the existing compatibility studies between the inter-satellite or fixed-satellite services and Earth exploration-satellite (passive) service around the frequencies 23.6, 31.3 and 50.2 GHz.

* * *

## 2.2	On the protection of the radio astronomy service (RAS)

The following text provides a regulatory solution for implementing pfd limits and threshold levels for GSO systems and epfd limits and threshold levels for non-GSO systems based on the epfd concept as defined in Article 22 in the relevant bands.

The results obtained from the band-by-band studies indicating compatibility between RAS and active services shall be the basis for bands where limits will apply. It should be noted that there are already limits included in the Radio Regulations (see Nos. 5.443B, 5.511A and 5.551G) or RAS bands protected from unwanted emissions (see No. 5.208A). It is thus additionally proposed to incorporate these limits for the protection of these RAS bands. Depending on the outputs of the

discussions on agenda item 1.15 dealing with footnote 5.443B and agenda item 1.32 dealing with footnote 5.551G, there would be a need to review these footnotes and to ensure consistency between all the proposals addressing protection of RAS bands with regards to unwanted emissions from space services.

For the case of non-GSO space stations, the limits and threshold levels are specified in terms of  equivalent power flux-density (epfd) at a radio astronomy station resulting from unwanted emissions from all space stations of a non-geostationary-satellite system in a frequency band allocated on a primary basis and for all conditions and for all methods of modulation. These unwanted emissions should not exceed the specified limits for more than 2% of time at any radio astronomy station according to ITU-R RA.1513. The epfd calculation is performed in accordance with Recommendations ITU-R S.1586 and ITU-R M.1583. The epfd limits and threshold levels are derived from Recommendation ITU-R RA.769 and the epfd is evaluated using the reference antenna pattern and the maximum antenna gain given in draft new Recommendation RA.[PATTERN NGSO].

The pfd/epfd limits or threshold levels:

–	are applicable to any radio astronomy station for which complete notification information has been received by the Bureau prior to the reception of the complete advance publication information of the satellite network;

–	should be defined depending on the type of observation supported at the considered radio astronomy station (single-dish or VLBI observation) and shall be specified in the reference bandwidth defined in Recommendation ITU-R RA.769;

–	may be exceeded on the territory of any country whose administration has so agreed;

–	shall be excluded from  examination performed by the Bureau under Articles 9 and 11 of the Radio Regulations.

This method would require a modification of Appendix 4 to insert additional characteristics of radio astronomy stations, in particular the type of observation and the minimum operating elevation angle of the antenna.

Proposals

ARTICLE  5

ADD	EUR/13A8/14

5.XX3	Space services operating in the following bands: 137-138 MHz, 387-390 MHz, 400.15-401 MHz, 1 452-1 492 MHz, 1 525-1 559 MHz, 1 559-1 610 MHz, 1 613.8-1 626.5 MHz, 2 670-2 690 MHz, 5 010-5 030 MHz, 21.4-22.0 GHz, and 42-42.5 GHz shall comply with the limits given in Annex 1 to Resolution [EUR/13A8-3] [RAS_PROTECTION] (WRC-03).

MOD	EUR/13A8/15

75.2-137.175 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 137-137.025 SPACE OPERATION (space-to-Earth) METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208A 5.209 ADD 5.XX3 SPACE RESEARCH (space-to-Earth) Fixed Mobile except aeronautical mobile (R) 5.204 5.205 5.206 5.207 5.208 |  |  |
| 137.025-137.175 SPACE OPERATION (space-to-Earth) METEOROLOGICAL-SATELLITE (space-to-Earth) SPACE RESEARCH (space-to-Earth) Fixed Mobile-satellite (space-to-Earth) 5.208A 5.209 ADD 5.XX3 Mobile except aeronautical mobile (R) 5.204 5.205 5.206 5.207 5.208 |  |  |

MOD	EUR/13A8/16

137.175-148 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 137.175-137.825 SPACE OPERATION (space-to-Earth) METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208A 5.209 ADD 5.XX3 SPACE RESEARCH (space-to-Earth) Fixed Mobile except aeronautical mobile (R) 5.204 5.205 5.206 5.207 5.208 |  |  |
| 137.825-138 SPACE OPERATION (space-to-Earth) METEOROLOGICAL-SATELLITE (space-to-Earth) SPACE RESEARCH (space-to-Earth) Fixed Mobile-satellite (space-to-Earth) 5.208A 5.209 ADD 5.XX3 Mobile except aeronautical mobile (R) 5.204 5.205 5.206 5.207 5.208 |  |  |

MOD	EUR/13A8/17

335.4-410 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 387-390 FIXED MOBILE Mobile-satellite (space-to-Earth) 5.208A 5.254 5.255 ADD 5.XX3 |  |  |
| ... |  |  |
| 400.15-401 METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208A 5.209 ADD 5.XX3 SPACE RESEARCH (space-to-Earth) 5.263 Space operation (space-to-Earth) 5.262 5.264 |  |  |

MOD	EUR/13A8/18

1 350-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 452-1 492 FIXED MOBILE except aeronautical mobile BROADCASTING 5.345 5.347 BROADCASTING- SATELLITE 5.345 5.347 ADD 5.XX3 | 1 452-1 492 FIXED MOBILE 5.343 BROADCASTING 5.345 5.347 BROADCASTING-SATELLITE 5.345 5.347 ADD 5.XX3 |  |
| 5.341 5.342 | 5.341 5.344 |  |

MOD	EUR/13A8/19

1 525-1 610 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 525-1 530 SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.351A Earth exploration-satellite Mobile except aeronautical mobile 5.349 | 1 525-1 530 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.351A Earth exploration-satellite Fixed Mobile 5.343 | 1 525-1 530 SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.351A Earth exploration-satellite Mobile 5.349 |
| 5.341 5.342 5.350 5.351 5.352A 5.354 ADD 5.XX3 | 5.341 5.351 5.354 ADD 5.XX3 | 5.341 5.351 5.352A 5.354 ADD 5.XX3 |
| 1 530-1 535 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.351A 5.353A Earth exploration-satellite Fixed Mobile except aeronautical mobile | 1 530-1 535 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.351A 5.353A Earth exploration-satellite Fixed Mobile 5.343 |  |
| 5.341 5.342 5.351 5.354 ADD 5.XX3 | 5.341 5.351 5.354 ADD 5.XX3 |  |
| 1 535-1 559 MOBILE-SATELLITE (space-to-Earth) 5.351A 5.341 5.351 5.353A 5.354 5.355 5.356 5.357 5.357A 5.359 5.362A ADD 5.XX3 |  |  |
| 1 559-1 610 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) 5.329A 5.341 5.362B 5.362C 5.363 ADD 5.XX3 |  |  |

MOD	EUR/13A8/20

1 610-1 660 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) Radiodetermination-satellite (Earth-to-space) |
| 5.341 5.355 5.359 5.363 5.364 5.365 5.366 5.367 5.368 5.369 5.371 5.372 ADD 5.XX3 | 5.341 5.364 5.365 5.366 5.367 5.368 5.370 5.372 ADD 5.XX3 | 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.372 ADD 5.XX3 |

MOD	EUR/13A8/21

2 520-2 700 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 670-2 690 FIXED 5.409 5.410 5.411 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (Earth-to-space) 5.351A Earth exploration-satellite (passive) Radio astronomy Space research (passive) | 2 670-2 690 FIXED 5.409 5.411 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (Earth-to-space) 5.351A Earth exploration-satellite (passive) Radio astronomy Space research (passive) | 2 670-2 690 FIXED 5.409 5.411 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (Earth-to-space) 5.351A Earth exploration-satellite (passive) Radio astronomy Space research (passive) |
| 5.149 5.412 5.419 5.420 ADD 5.XX3 | 5.149 5.419 5.420 ADD 5.XX3 | 5.149 5.419 5.420 5.420A ADD 5.XX3 |

MOD	EUR/13A8/22

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 000-5 150 AERONAUTICAL RADIONAVIGATION 5.367 5.443A 5.443B 5.444 5.444A ADD 5.XX3 |  |  |

MOD	EUR/13A8/23

18.6-22.21 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 21.4-22 FIXED MOBILE BROADCASTING- SATELLITE 5.530 ADD 5.XX3 | 21.4-22 FIXED MOBILE | 21.4-22 FIXED MOBILE BROADCASTING- SATELLITE 5.530 ADD 5.XX3 5.531 |

MOD	EUR/13A8/24

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) 5.551AA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551G ADD 5.XX3 |  |  |

Appendix  4  (WRC-2000)

Consolidated list and tables of characteristics for use in the 
application of the procedures of Chapter III

ANNEX  2A

Characteristics of satellite networks’ earth stations 
or radio astronomy stations2     (WRC-2000)

MOD	EUR/13A8/25

## A.17	Compliance with aggregate power flux-density limits

a)	For satellite systems operating in the radionavigation-satellite service in the band 1 164-1 215 MHz, the calculated aggregate power flux-density produced at the Earth’s surface by all the space stations within all radionavigation-satellite systems, as defined in No. 5.328A.

-

ADD	EUR/13A8/26

## A.18	Commitment regarding compliance with Resolution [EUR/13A8-3] [RAS_PROTECTION] (WRC-03)

For space systems for which Resolution [EUR/13A8-3] [RAS_PROTECTION] (WRC-03) applies, a commitment that the filed system will comply with Resolution [EUR/13A8-3] [RAS_PROTECTION] (WRC-03).

# C	Characteristics to be provided for each group of frequency assignments for a satellite antenna beam or an earth station or radio astronomy station antenna

ADD	EUR/13A8/27

## C.13bis	Type of observations

The type of the observations to be done in the frequency bands shown in § C.3 b). The different types of observations are: single-dish or VLBI.

ADD	EUR/13A8/28

## C.13ter	Radio astronomy station minimum observation elevation angle

The minimum elevation angle at which the radio astronomy station conducts single-dish or VLBI observations in the frequency band.

ARTICLE  29

Radio astronomy service

ADD	EUR/13A8/29

29.14 	§ 11	In order to avoid unacceptable interference to the radio astronomy service from unwanted emissions of space stations, Resolution [EUR/13A8-3] [RAS_PROTECTION] (WRC-03) shall be applied.

ADD	EUR/13A8/30

DRAFT  RESOLUTION  [EUR/13A8-3]  [RAS_PROTECTION] (WRC-03)

Protection of the radio astronomy stations from unwanted
emissions of space stations

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that unwanted emissions from active services may cause unacceptable interference to the radio astronomy service;

b)	that, for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect the RAS service in specific bands;

c)	that Recommendation ITU-R SM.[BbB] provides compatibility analysis between  passive services and active services in adjacent and nearby bands;

d)	that out-of-band emissions can be controlled to a certain extent in the design process of space stations through careful design methods but that some narrow-band spurious emissions can be generated through some uncontrollable physical mechanisms;

e)	that consultation between administrations may lead to innovative solutions offering the potential for rapid implementation;

f)	that WRC-2000 adopted Recommendation 66 on the studies of the maximum permitted levels of unwanted emissions,

noting

a)	that Recommendation ITU-R M.1583 provides a methodology based on the epfd concept for calculation of interference resulting from unwanted emissions from non-GSO satellite systems of the mobile-satellite or radionavigation-satellite services into radio astronomy stations;

b)	that Recommendation ITU-R S.1586 provides a methodology based on the epfd concept for calculation of interference resulting from unwanted emissions from non-GSO systems of the fixed-satellite service into radio astronomy stations;

c)	that draft new Recommendation ITU-R RA.[PATTERN NGSO] provides antenna patterns to be used for compatibility analyses between non-GSO systems and RAS stations based on the epfd concept;

d)	that Recommendation ITU-R RA.1513 recommends acceptable levels of data loss to radio astronomy observations, stating in particular that the percentage of data loss caused by any system should be lower than 2%;

e)	that the Bureau is not involved in the consultation procedure defined in this Resolution,

recognizing

a)	that there is an uncertainty in the pre-launch assessment of the levels of unwanted emissions;

b)	that it is necessary to ensure an equitable burden sharing for achieving compatibility between active and passive services;

c)	that Recommendation ITU-R SM.[BbB] does not address all bands where, for technical or operational reasons, more stringent spurious emission limits than the general limits in Appendix 3 may be required to protect the RAS service;

d)	that for some bands where RAS protection criteria are difficult to be met, a consultation procedure may be needed,

resolves

1	that space stations for which complete advance publication information is received by the Bureau as of 5 July 2003, which are planned to operate in one or several space service bands listed in Annex 1, shall comply at any radio astronomy station with the unwanted emission limits of Annex 1 in the specific radio astronomy bands associated to this or these above-mentioned space service band(s);

2	that the radio astronomy stations to be taken into account in applying resolves 1 are those which are operating in the frequency band(s) for which Annex 1 applies and which are notified before the date of reception of the complete advance publication information of the space station referred to in resolves 1;

3	that the limits given in Annex 1 may be exceeded on the territory of any country whose administration has so agreed;

4	that, if an administration considers that the design of any of its space stations can not meet the unwanted emissions threshold levels given in Annex 2 at a radio astronomy station, this administration shall consult with the administration operating this radio astronomy station until a mutually acceptable solution has been found;

5	that the space stations to be considered in the application of resolves 4 are those for which complete advance publication information is received by the Bureau as of 5 July 2003;

6	that the radio astronomy stations to be taken into account in applying resolves 4 are those which are operating in the frequency band(s) where the space station can not meet the unwanted emission threshold levels referred to in resolves 4 and which are notified before the date of reception of the complete advance publication information of the space station referred to in resolves 5;

7	that, in the event that the unwanted emission levels from a space station to which resolves 4 applies and which has been brought into service exceed the threshold levels contained in Annex 2 at a radio astronomy station as defined in resolves 6, the notifying administration of such space station shall consult with the administration operating this radio astronomy station in order to achieve a mutually acceptable solution, taking into account relevant ITU-R Recommendations agreed by administrations concerned to be used in this consultation process;

8	to invite ITU-R to complete as a matter of urgency and in time for consideration by WRC-07, the studies in all frequency bands listed in Annex 3;

9	to recommend that WRC-07 review the results of the studies identified in resolves 8 and consider the appropriate measures, in particular the inclusion of new paired bands in Annex 1 or Annex 2, as appropriate, taking into account the impact on all concerned services of implementing or not implementing limits or threshold levels for the bands given in Annex 3,

urges administrations

1	to take all appropriate steps, as from the design phase, to ensure that space stations, which are planned to operate in one or several space service allocations listed in Annex 2, meet the unwanted emission threshold levels given in Annex 2 at any radio astronomy station;

2	to provide the relevant characteristics of systems operating in the bands listed in Annex 3;

3	to contribute to the compatibility studies relating to the active space services and the radio astronomy service operating in the bands identified in Annex 3.

ANNEX  1  TO  DRAFT  RESOLUTION  [EUR/13A8-3]  [RAS_PROTECTION] (WRC-03)

Unwanted emission limits

The unwanted emission limits related to geostationary space stations are given in Table 1-1 in terms of power flux-density (pfd) produced at a radio astronomy station in a reference bandwidth.

In Table 1-1 the pfd value given in the third, fifth and seventh column (associated with the reference bandwidth contained in the adjacent column) has to be met by space stations operating in the bands indicated in the first column at the radio astronomy station operating in the band mentioned in the second column.

The unwanted emission limits related to space stations of non-geostationary systems are given in Table 1-2 in terms of equivalent power flux-density (epfd) produced at a radio astronomy station in a reference bandwidth not to be exceeded during a given percentage of time.

In Table 1-2 the epfd value given in the third, fifth and seventh column (associated with the reference bandwidths contained in the adjacent column) has to be met by space stations operating in the bands indicated in the first column at the radio astronomy station operating in the band mentioned in the second column. The epfd at a given radio astronomy station shall be evaluated by using the antenna pattern and the RAS maximum antenna gain given in draft new Recommendation RA.[PATTERN NGSO]. Guidance on the calculation of epfd can be found in Recommendations ITU-R S.1586 and ITU-R M.1583. The elevation angles of the radio astronomy stations to be taken into account in the epfd calculation are those higher than the minimum elevation angle θmin of the radio telescope. In the absence of such information a value of 5o shall be taken. The percentage of time during which the epfd level shall not be exceeded is mentioned in Note (1) of Table 1-2.

Table 1-1

pfd limits for unwanted emissions from geostationary space stations
at a radio astronomy station

| Space service bands | Radio astronomy band | Single dish continuum |  | Single dish spectral lines |  | VLBI(1) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | pfd(2) | Reference bandwidth | pfd(2) | Reference bandwidth | pfd(2) |
| MHz | MHz | dB(W/m2) | MHz | dB(W/m2) | kHz | dB(W/m2) |
| 1 452-1 492 1 525-1 559 | 1 400-1 427 | –180 | 27 | –196 | 20 | –166 |
| 1 525-1 559 1 559-1 610 1 613.8-1 626.5 | 1 610.6-1 613.8 | NA | NA | –194 | 20 | –166 |
| 2 670-2 690 | 2 690-2 700 (in Regions 1 and 3) | –177 | 10 | NA | 20 | –161 |
| 5 010-5 030 | 4 990-5 000 | –171 | 10 | NA | 50 | –153 |
| GHz | GHz | – | – | – | – | – |
| 21.4-22.0 | 22.21-22.5 | NA | NA | NA | 250 | –128 |
| 42.0-42.5 | 42.5-43.5 | –137 | 1 000 | –153 | 500 | –116 |
| (1) The reference bandwidth used for spectral line observations has also been used as reference bandwidth for VLBI observations. In VLBI bands, where no spectral line observations are conducted, the reference bandwidth for VLBI observations has been determined using the assumption of Recommendation ITU-R RA.769 for a typical spectrometer channel (3 km/s). (2) Integrated over the reference bandwidth with an integration time of 2 000 s. |  |  |  |  |  |  |

Notes for the Conference

NOTE A – The frequency band 42.0-42.5 GHz needs confirmation (agenda item 1.32 – Resolution 128).

NOTE B – The pfd values specified for VLBI observations should be updated using the latest version of Recommendation ITU-R RA.769.

Table 1-2

epfd(1) limits for unwanted emissions from non-GSO satellite systems
at a radio astronomy station

| Space service bands | Radio astronomy band | Single dish continuum |  | Single dish spectral lines |  | VLBI(2) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | epfd(3) | Reference bandwidth | epfd(3) | Reference bandwidth | epfd(3) |
| MHz | MHz | dB(W/m2) | MHz | dB(W/m2) | kHz | dB(W/m2) |
| 137-138 387-390 400.15-401 (No. 5.208A) | 150.05-153.0 | –238 | 2.95 | NA | 10 | –228 |
|  | 322-328.6 | –240 | 6.6 | –255 | 10 | –228 |
|  | 406.1-410 | –242 | 3.9 | NA | 10 | –228 |
|  | 608-614 | –241 | 6 | NA | 10 | –228 |
| 1 452-1 492 1 525-1 559 | 1 400-1 427 | –243 | 27 | –259 | 20 | –229 |
| 1 525-1 559 | 1 610.6-1 613.8 | NA | NA | –258 | 20 | –230 |
| 1 559-1 610 | 1 610.6-1 613.8 | NA | NA | –258 | 20 | –230 |
| 1 613.8-1 626.5 | 1 610.6-1 613.8 | NA | NA | –258 | 20 | –230 |
| 2 670-2 690 | 2 690-2 700 (in Regions 1 and 3) | –246 | 10 | NA | 20 | –230 |
| 5 010-5 030 | 4 990-5 000 | –245 | 10 | NA | 50 | –227 |
| GHz | GHz | – | – | – | – | – |
| 21.4-22 | 22.21-22.5 | NA | NA | NA | 250 | –215 |
| 42.0-42.5 | 42.5-43.5 | –230 | 1 000 | –246 | 500 | –209 |
| (1) These epfd limits shall not be exceeded for more than 2% of time. (2) The reference bandwidth used for spectral line observations has also been used as reference bandwidth for VLBI observations. In VLBI bands, where no spectral line observations are conducted, the reference bandwidth for VLBI observations has been determined using the assumption of Recommendation ITU-R RA.769 for a typical spectrometer channel (3 km/s). (3) Integrated over the reference bandwidth with an integration time of 2 000 s. |  |  |  |  |  |  |

Notes for the Conference

NOTE A – The frequency band 42.0-42.5 GHz needs confirmation (agenda item 1.32 – Resolution 128).

NOTE B – The epfd values specified for VLBI observations should be updated using the latest version of Recommendation ITU-R RA.769.

ANNEX  2 TO  DRAFT  RESOLUTION  [EUR/13A8-3]  [RAS_PROTECTION] (WRC-03)

Unwanted emission threshold levels

The unwanted emission threshold levels related to geostationary space stations are given in Table 2-1 in terms of power flux-density (pfd) produced at a radio astronomy station in a reference bandwidth.

In Table 2-1 the pfd value given in the third, fifth and seventh column (associated with the reference bandwidth contained in the adjacent columns) has to be met by space stations operating in the bands indicated in the first column at the radio astronomy station operating in the bands mentioned in the second column.

The unwanted emission threshold levels related to space stations of non-geostationary systems are given in Table 2-2 in terms of equivalent power flux-density (epfd) produced at a radio astronomy station in a reference bandwidth not to be exceeded during a given percentage of time.

In Table 2-2 the epfd value given in the third, fifth and seventh column (associated with the reference bandwidth contained in the adjacent columns) has to be met by space stations operating in the bands indicated in the first column at the radio astronomy station operating in the band mentioned in the second column. The epfd at a given radio astronomy station shall be evaluated by using the antenna pattern and the RAS maximum antenna gain given in draft new Recommendation RA.[PATTERN NGSO]. Guidance on the calculation of epfd can be found in Recommendations ITU-R S.1586 and ITU-R M.1583. The elevation angles of the radio astronomy stations to be taken into account in the epfd calculation are those higher than the minimum elevation angle θmin of the radio telescope. In the absence of such information a value of 5o shall be taken. The percentage of time during which the epfd level should not be exceeded is mentioned in Note (1) of Table 2-2.

Table 2-1

Unwanted emission pfd thresholds from geostationary space stations
at a radio astronomy station

| Space service bands | Radio astronomy band | Single dish continuum |  | Single dish spectral lines |  | VLBI(1) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | pfd(2) | Reference bandwidth | pfd(2) | Reference bandwidth | pfd(2) |
| MHz | MHz | dB(W/m2) | MHz | dB(W/m2) | kHz | dB(W/m2) |
| 1 670-1 710 | 1 660-1 670 | –181 | 10 | –194 | 20 | –166 |
| 2 655-2 670 | 2 690-2 700 (in Regions 1 and 3) | –177 | 10 | NA | 25 | –161 |
| GHz | GHz | – | – | – | – | – |
| 22.55-23.55 | 22.21-22.5 | –146 | 290 | –162 | 250 | –128 |
| 22.55-23.55 24-24.05 | 23.6-24 | –147 | 400 | –161 | 250 | –129 |
| 30-31.3 31.8-32.3 | 31.3-31.8 | –141 | 500 | NA | NA | NA |
| (1) The reference bandwidth used for spectral line observations has also been used as reference bandwidth for VLBI observations. In VLBI bands, where no spectral line observations are conducted, the reference bandwidth for VLBI observations has been determined using the assumption of Recommendation ITU-R RA.769 for a typical spectrometer channel (3 km/s). (2) Integrated over the reference bandwidth with an integration time of 2 000 s. |  |  |  |  |  |  |

Table 2-2

Unwanted emission epfd threshold levels(1) from non-GSO satellite systems
at a radio astronomy station

| Space service bands | Radio astronomy band | Single dish continuum |  | Single dish spectral lines |  | VLBI(2) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | epfd(3) | Reference bandwidth | epfd(3) | Reference bandwidth | epfd(3) |
| MHz | MHz | dB(W/m2) | MHz | dB(W/m2) | kHz | dB(W/m2) |
| 1 670-1 710 | 1 660-1 670 | –246 | 10 | –259 | 20 | –231 |
| 2 655-2 670 | 2 690-2 700 (in Regions 1 and 3) | –246 | 10 | NA | 25 | –230 |
| GHz | GHz | – | – | – | – | – |
| 21.4-22 22.55-23.55 | 22.21-22.5 | –233 | 290 | –249 | 250 | –215 |
| 22.55-23.55 24-24.05 | 23.6-24 | –235 | 400 | –249 | 250 | –217 |
| 30-31.3 31.8-32.3 | 31.5-31.7 | –231 | 500 | NA | NA | NA |
| (1) These epfd threshold levels shall not be exceeded for more than 2% of time. (2) The reference bandwidth used for spectral line observations has also been used as reference bandwidth for VLBI observations. In VLBI bands, where no spectral line observations are conducted, the reference bandwidth for VLBI observations has been determined using the assumption of Recommendation ITU-R RA.769 for a typical spectrometer channel (3 km/s). (3) Integrated over the reference bandwidth with an integration time of 2 000 s. |  |  |  |  |  |  |

aNNEX  3  TO  DRAFT  RESOLUTION  [eur/13a8-3]  [ras_protection] (wrc-03)

Further compatibility analysis between
space services and the RAS service

The number of potential interference situations, due to unwanted emissions from space services into radio astronomy service bands, may be high, particularly if non-adjacent band interference is also taken into account. A partial list of such bands and situations, where compatibility studies should be conducted or continued is given in Table 3-1. These studies should determine if spurious emission limits more stringent than the general limits contained in Appendix 3 of the Radio Regulations may be required to protect the radio astronomy service from unwanted emissions of the space services, taking into account the impact on all concerned services of implementing or not implementing such limits.

TABLE 3-1

Band pairs to be considered for further studies

| Space service bands | Radio astronomy band |
| --- | --- |
| MHz | MHz |
| 137-138 | 150.05-153.0 (No. 5.208A) |
| 387-390 | 322-328.6 (No. 5.208A) |
| 400.15-401 | 406.1-410 (No. 5.208A) |
| 620-790 (No. 5.311) | 608-614 |
| 1 670-1 710 | 1 660-1 670 |
| 2 655-2 670 | 2 690-2 700 |
| 2 670-2 690 | 2 690-2 700 (in Region 2) |
| GHz | GHz |
| 10.7-10.95 | 10.6-10.7 |
| 21.4-22.0 | 22.21-22.5 |
| 22.55-23.55 | 22.21-22.5 |
| 22.55-23.55 | 23.6-24 |
| 24-24.05 | 23.6-24 |
| 30-31.3 | 31.3-31.8 |
| 31.8-32.3 | 31.5-31.7 |

______________