MOD	EUR/13A9/11

RESOLUTION  331  (Rev.WRC-03)

Transition to the Global Maritime Distress and Safety System (GMDSS) and phasing out the distress and safety system described in Appendix 13

The World Radiocommunication Conference (Geneva, 2003),

noting

that all ships subject to the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended, were required to be fitted for the Global Maritime Distress and Safety System (GMDSS) by 1 February 1999,

noting further

a)	that many administrations have taken steps to implement the GMDSS also for classes of vessels not subject to SOLAS, 1974, as amended;

b)	that an increasing number of vessels not subject to SOLAS, 1974, as amended, are making use of the techniques and frequencies of the GMDSS prescribed in Chapter VII;

c)	that some administrations and vessels, not subject to SOLAS, 1974, as amended, may wish to continue to use provisions of Appendix 13 for distress and safety communications for yet a few years;

d)	that it would be costly for administrations to maintain in parallel for an excessive period of time shore-based facilities necessary to support both the old and new distress and safety systems;

e)	that there may be a need to maintain existing shore-based distress and safety services described in Appendix 13 for some years after this Conference so that vessels not subject to SOLAS, 1974, as amended and not yet using the techniques and frequencies of the GMDSS will be able to obtain assistance from these services until such time as they are able to participate in the GMDSS;

f)	that the International Maritime Organization (IMO) has decided that:

–	listening watches on 2 182 kHz are no longer mandatory on board SOLAS ships after 1 February 1999;

–	listening watches on VHF channel 16 shall continue; a final date for the cessation of mandatory watchkeeping on channel 16 is yet to be determined;

–	at the earliest opportunity, digital selective calling on VHF channel 70 will be used universally for initial distress, urgency and safety alerting, using VHF channel 16 as the radiotelephony channel following the initial alert;

g)	that WRC-97 decided that listening watch on 2 182 kHz will no longer be obligatory after 1 February 1999;

h)	that the Radio Regulations require GMDSS vessels to keep watch on the appropriate digital selective calling (DSC) distress frequencies;

i)	that IMO has identified a need for a VHF channel for immediate voice intership calling so that all SOLAS ships can establish and conduct communications with each other for distress, safety and general communications purposes until DSC on channel 70 becomes a capability commonly used by both SOLAS ships and non-SOLAS vessels;

j)	that separate provisions in the existing Radio Regulations designate VHF channel 16 and the frequency 2 182 kHz as the international channels for general calling by radiotelephony;

k)	that the Radio Regulations establish that ship stations should, when practicable, keep watch on VHF channel 13;

l)	that several administrations have established Vessel Traffic Service (VTS) systems and require their vessels to keep watch on local VTS channels;

m)	that ships that are required by SOLAS to carry a radio station have been equipped with DSC, and many vessels subject to national carriage requirements are also being equipped with DSC, but the majority of vessels that carry a radio station on a voluntary basis might not yet have DSC equipment;

n)	that similarly, many administrations have established distress and safety services based on DSC watchkeeping, but the majority of port stations, pilot stations and other operational coast stations might not yet have been equipped with DSC facilities;

o)	that for the reasons in noting further m) and n) listed above, it will be necessary for some stations in the maritime mobile service to continue for some years to call each other by radiotelephony in certain situations;

p)	that IMO has urged administrations to require all seagoing vessels under national legislation, and encourage all vessels voluntarily carrying VHF radio equipment to be fitted with facilities for transmitting and receiving distress alerts by DSC on VHF channel 70 no later than 1 February 2005,

considering

a)	that the operation of the GMDSS described in Chapter VII and the distress and safety system described in Appendix 13 differ in many crucial aspects, such as means and methods of alerting, communication facilities available, announcement and transmission of maritime safety information, etc.;

b)	that operation of the two systems in parallel for a long period would cause ever-increasing difficulties and incompatibilities between vessels operating in the two different systems and may thus seriously degrade safety at sea in general;

c)	that the GMDSS overcomes the deficiencies of the aural watch-keeping on maritime distress and calling frequencies on which the distress and safety system described in Appendix 13 relies, by replacing these watches by automatic watch, i.e. digital selective calling and satellite communication systems;

d)	that the listening watch on 2 182 kHz on board SOLAS ships and at some coast stations has ceased in accordance with the decisions of IMO and ITU mentioned in noting further f) and g) above,

resolves

1	to retain, as an interim measure, the provisions permitting use of VHF channel 16 and the frequency 2 182 kHz for general voice-calling;

2	to urge all administrations to assist in enhancing safety at sea by:

–	encouraging all vessels to make use of the GMDSS as soon as possible;

–	requiring all seagoing vessels required to carry and encouraging all ships voluntarily carrying VHF radio equipment to be fitted with facilities for transmitting and receiving distress alerts by DSC on VHF channel 70 no later than 1 February 2005;

–	encouraging, where appropriate, establishment of suitable shore-based facilities for GMDSS, either on an individual basis or in cooperation with other relevant parties in the area;

–	encouraging vessels to limit their use of VHF channel 16 and the frequency 2 182 kHz for calling to the minimum necessary, noting the provisions of No. 52.239 of the Radio Regulations;

3	that administrations may, taking account of all aspects involved, such as:

–	decisions by IMO and ITU on aural watch on 2 182 kHz and VHF channel 16;

–	the GMDSS radio systems available in the area concerned;

–	the compatibility problems mentioned in considering a) and b) above;

–	the density and classes of vessels normally in the area;

–	the geographical nature of the area and general navigational conditions within the area;

–	other adequate measures taken to ensure safety communications for vessels sailing in the area,

when the development on transition to the GMDSS and the prevailing conditions in the area makes it reasonable to do so, release their ship stations and coast stations within the area concerned from the obligations described in Appendix 13 on listening watch on VHF channel 16 or 2 182 kHz or both;

when doing so, administrations should:

–	inform IMO of their decisions and submit to IMO details on the area concerned;

–	inform the Secretary-General on the necessary details for inclusion in the List of Coast Stations,

resolves further

that the Secretary-General should ensure that such arrangements and details regarding the area concerned be indicated in relevant maritime publications,

-

instructs the Secretary-General

to communicate this Resolution to IMO and the International Civil Aviation Organization (ICAO).

-

***