MOD	ASP/25/134

52.221A		2)	Limited safety-related calling on the carrier frequencies 12 290 kHz and 16 420 kHz shall be permitted only to and from rescue coordination centres (see No. 30.6.1), subject to the safeguards of Resolution (XXX) (WRC-03). The alternative carrier frequencies 12 359 kHz and 16 537 kHz may be used by ship stations and coast stations for calling on a simplex basis, provided that the peak envelope power does not exceed 1 kW.     (WRC-03)

Reasons:	The addition of this footnote permits calling to and from stations that have search and rescue responsibilities, i.e. rescue coordination centres. Because of the very limited number of rescue coordination centres that are, or plan to be, operating in these bands, the additional traffic and potential for interfering with distress and safety traffic is very low.