| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 31 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| proposals for the work of the conference |  |
| agenda item 1.31 |  |

1.31	to consider the additional allocations to the mobile-satellite service in the 1-3 GHz band, in accordance with Resolutions 226 (WRC-2000) and 227 (WRC-2000)

Introduction

Taking into consideration the view of APT Member States’ input documents the APT proposes:

1)	to make a global allocation to MSS (space-to-Earth) in the band 1 520-1 525 MHz associated with the establishment of appropriate thresholds to trigger coordination with the other allocated services. In addition, MSS use in this band shall not claim protection from the MS service in Japan. The proposals are consistent with Method D of the CPM Report relating to Resolution 226 (§ 2.8.1.3.4);

2)	to make a global allocation to MSS (Earth-to-space) in the band 1 670-1 675 MHz associated with regulatory measures to ensure an appropriate sharing environment between MSS and the already allocated services. The proposals are consistent with Method A of the CPM Report relating to Resolution 227 (§ 2.8.2.3.1);

3)	as a consequence of making the new MSS allocations, to remove the existing allocations to MSS in Region 2 in the bands 1 492-1 520 MHz and 1 675-1 710 MHz.

These proposals contribute to alleviating the spectrum shortage for MSS and also harmonizing allocations between ITU Regions. The first set of proposals is associated with Resolution 226 and the second set with Resolution 227.

Proposals

Resolution  226

article  5

MOD	ASP/25/218

1 350-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 492-1 520 FIXED MOBILE except aeronautical mobile | 1 492-1 520 FIXED MOBILE 5.343 | 1 492-1 520 FIXED MOBILE |
| 5.341 5.342 | 5.341 5.344 | 5.341 |

MOD	ASP/25/218A

| 1 520-1 525 FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (space-to-Earth) ADD 5.348A ADD 5.NZL | 1 520-1 525 FIXED MOBILE 5.343 MOBILE-SATELLITE (space-to-Earth) 5.348A ADD 5.NZL | 1 520-1 525 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) MOD 5.348A ADD 5.NZL |
| --- | --- | --- |
| 5.341 5.342 MOD 5.348 | 5.341 5.344 MOD 5.348 | 5.341 MOD 5.348 |

Reasons:	These proposals contribute to alleviate the spectrum shortage for MSS and also harmonize allocations between ITU Regions.

MOD	ASP/25/219

5.348	The use of the band 1 520-1 525 MHz by the mobile-satellite service is subject to coordination under No. 9.11A. However, no coordination threshold in Article 21 for space stations of the mobile-satellite service with respect to terrestrial services shall apply to the situation referred to in No. 5.343. With respect to the situation referred to in No. 5.343, the requirement for coordination in the band 1 520-1 525 MHz will be determined by band overlap. In the band 1 520-1 525 MHz stations in the mobile-satellite service shall not claim protection from the stations of the fixed service. No. 5.43A does not apply.

Reasons:	Consequential modification to reflect the proposed MSS band reduction in Region 2 and to provide that MSS cannot claim protection from fixed service systems. Protection of terrestrial services is ensured through footnotes in Appendix 5.

MOD	ASP/25/220

5.348A	In the band 1 520-1 525 MHz, the coordination threshold in terms of the power flux-density levels at the surface of the Earth in application of No. 9.11A for space stations in the mobile-satellite (space-to-Earth) service, with respect to the land mobile service use for specialized mobile radios or used in conjunction with public switched telecommunication networks (PSTN) operating within the territory of Japan, shall be –150 dB(W/m2) in any 4 kHz band for all angles of

arrival, instead of those given in Table 5-2 of Appendix 5. The above threshold level of the power flux-density shall apply until it is changed by a competent world radiocommunication conference. In the band 1 520-1 525 MHz stations in the mobile-satellite service shall not claim protection from the MS station in Japan.

Reasons:	Consequential modification to reflect the proposed MSS band reduction in Region 2.

ADD	ASP/25/221

5.NZL	In the band 1 520-1 525 MHz, the coordination threshold in terms of the power flux-density levels at the surface of the Earth in application of No. 9.11A for space stations in the mobile-satellite (space-to-Earth) service, with respect to the fixed services operating within the territory of New Zealand, shall be –138 dB(W/m2) in any 1 MHz band for all angles of arrival, instead of those given in Table 5-2 of Appendix 5. The above threshold level of the power flux-density shall apply until it is changed by a competent world radiocommunication conference.

Reasons:	To protect specific fixed service systems in New Zealand.

ARTICLE  21

ADD	ASP/25/222

TABLE  21-4A

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
| 1 520-1 525 MHz (Applicable to the territory of the United States in Region 2 between the longitudes 71° W and 125° W) | Mobile-satellite (space-to-Earth) | 0°  δ  4° | 4° < δ  20° | 20° δ 60° | 60°  δ  90° |  |
|  |  | –181.0 | –193.0  20 log δ | –213.3  35.6 log(δ) | –150.0 | 4 kHz |
| 1 520-1 525 MHz (Applicable to all other territory of the United States in Region 2) | Mobile-satellite (space-to-Earth | 0°  δ  43.4° |  | 43.4° < δ ≤ 60° | 60° < δ  90° |  |
|  |  | –155.0 |  | 213.3 + 35.6 log δ | –150.0 | 4 kHz |

Reasons:	The limit is to protect the AMT stations in the mobile service, based on CPM text (see § 2.8.1.4).

appendix  5  (wrc-2000)

ANNEX  1

MOD	ASP/25/223

TABLE  5-2

| Reasons: A consequential modification to reflect that the band has been reduced to 1 520-1 525 MHz. Frequency band (MHz) | Terrestrial service to be protected | Coordination threshold values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GSO space stations |  | Non-GSO space stations |  |  |
|  |  | pfd (per space station) calculation factors (NOTE 2) |  | pfd (per space station) calculation factors (NOTE 2) |  | % FDP (in 1 MHz) (NOTE 1) |
|  |  | P | r dB/ degrees | P | r dB/ degrees |  |
| 1 520-1 525 | Analogue FS telephony (NOTE 5) | – 146 dB(W/m2) in 4 kHz and – 128 dB(W/m2) in 1 MHz | 0.5 | – 146 dB(W/m2) in 4 kHz and – 128 dB(W/m2) in 1 MHz | 0.5 |  |
|  | All other cases (MOD NOTE 4) | – 128 dB(W/m2) in 1 MHz | 0.5 | – 128 dB(W/m2) in 1 MHz | 0.5 | 25 |

MOD	ASP/25/224

NOTE 4 – Exceptions for the band 1 520-1 525 MHz are as follows:

Reasons:	A consequential modification to reflect that the band has been reduced to 1 520-1 525 MHz.

APPENDIX  7  (WRC-2000)

MOD	ASP/25/225

TABLE  8a

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | ... | Mobile-satellite, land-mobile satellite, maritime mobile- satellite |
| --- | --- | --- | --- | --- |
| Frequency bands (MHz) |  |  |  | 1 520-1 530 1 555-1 559 2 160-2 200 1 |
| Transmitting terrestrial service designations |  |  |  | Fixed, mobile |
| Method to be used |  |  |  | § 1.4.6 |
| Modulation at earth station 2 |  |  |  | N |
| Earth station interference parameters and criteria | p0 (%) |  |  | 10 |
|  | n |  |  | 1 |
|  | p (%) |  |  | 10 |
|  | NL (dB) |  |  | 0 |
|  | Ms (dB) |  |  | 1 |
|  | W (dB) |  |  | 0 |
| Terrestrial station parameters | E (dBW) in B 3 | A |  | 37 4 |
|  |  | N |  | 37 |
|  | Pt (dBW) in B | A |  | 0 |
|  |  | N |  | 0 |
|  | Gx (dBi) |  |  | 37 |
| Reference bandwidth | B (Hz) |  |  | 4  103 |
| Permissible interference power | Pr( p) (dBW) in B |  |  | –176 |
| 1 In the band 2 160-2 200 MHz, the terrestrial station parameters of line-of-sight radio-relay systems have been used. If an administration believes that, in this band transhorizon systems need to be considered, the parameters associated with the frequency band 2 500-2 690 MHz may be used to determine the coordination area. 2 A: analogue modulation; N: digital modulation. 3 E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth. 4 This value is reduced from the nominal value of 50 dBW for the purposes of determination of coordination area, recognizing the low probability of high power emissions falling fully within the relatively narrow bandwidth of the earth station. 5 The fixed-service parameters provided in the column for 163-167 MHz and 272-273 MHz are only applicable to the band 163-167 MHz. |  |  |  |  |

Reasons:	To align Table 8a with proposal ASP/25/218A.

SUP	ASP/25/226

RESOLUTION  226  (WRC-2000)

Sharing studies for, and possible additional allocations to, the
mobile-satellite service (space-to-Earth) in the 1-3 GHz range,
including consideration of the band 1 518-1 525 MHz

Reasons:	A consequence of finding a global allocation for MSS in the band 1 520-1 525 MHz.

Resolution 227

ARTICLE  5

MOD	ASP/25/227

1 660-1 710 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 670-1 675 METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE 5.380 MOBILE-SATELLITE (Earth-to-space) 5.341 ADD 5.UUU ADD 5.VVV ADD 5.QQQ |  |  |

MOD	ASP/25/227A

| 1 675-1 690 METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 675-1 690 METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 675-1 690 METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |
| --- | --- | --- |
| 5.341 | 5.341 | 5.341 |
| 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) Fixed Mobile except aeronautical mobile | 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) | 1 690-1 700 METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) |
| 5.289 5.341 5.382 | 5.289 5.341 5.381 | 5.289 5.341 5.381 |

| 1 700-1 710 FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 700-1 710 FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 1 700-1 710 FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |
| --- | --- | --- |
| 5.289 5.341 | 5.289 5.341 | 5.289 5.341 5.384 |

Reasons:	To show modifications to the Table of Frequency Allocations for Method A, and indicating that it is premature to allocate MSS in the band 1 668-1 670 because ITU-R studies have not yet been completed.

ADD	ASP/25/228

5.UUU	In the band 1 670-1 675 MHz, stations in the mobile-satellite service shall not cause harmful interference to stations in the meteorological aids service in those countries that have existing MetAids service in this band and that plan to continue such use. The use of this band by the mobile-satellite service is subject to coordination under No. 9.11A.

Reasons:	Footnote to trigger MSS coordination and to minimize the effect on MetAids of the new MSS allocation.

ADD	ASP/25/229

5.VVV	In the band 1 670-1 675 MHz, stations in the mobile-satellite service shall be subject to coordination under No. 9.11A, and shall not cause harmful interference to stations in the radio astronomy service operating in the band 1 660-1 670 MHz.

Reasons:	Footnote to trigger MSS coordination and to not cause interference to RAS by the MSS allocation.

ADD	ASP/25/230

5.QQQ	The mobile-satellite service in the 1 670-1 675 MHz band shall be subject to coordination under No. 9.11A and shall not cause harmful interference to the meteorological-satellite service.

Reasons:	Footnote to trigger MSS coordination and to not cause interference to MetSat by the MSS allocation.

APPENDIX  7  (WRC-2000)

MOD	ASP/25/231

TABLE  7a

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | ... | Mobile- satellite |  | ... |
| --- | --- | --- | --- | --- | --- |
| Frequency bands (MHz) |  |  | 1 670-1 675 |  |  |
| Receiving terrestrial service designations |  |  | Fixed, mobile |  |  |
| Method to be used |  |  | § 1.4.6 |  |  |
| Modulation at terrestrial station 1 |  |  | A | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) |  | 0.01 | 0.01 |  |
|  | n |  | 2 | 2 |  |
|  | p (%) |  | 0.005 | 0.005 |  |
|  | NL (dB) |  | 0 | 0 |  |
|  | Ms (dB) |  | 33 | 33 |  |
|  | W (dB) |  | 0 | 0 |  |
| Terrestrial station parameters | Gx (dBi) 3 |  | 35 | 35 |  |
|  | Te (K) |  | 750 | 750 |  |
| Reference bandwidth | B (Hz) |  | 4  103 | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B |  | –131 | –107 |  |
| 1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 1 675-1 710 MHz may also be used to determine a supplementary contour. 3 Feeder losses are not included. |  |  |  |  |  |

Reasons:	The coordination distance with respect to MetAids is a predetermined coordination distance (see Appendix 7, Table 10), hence no entry is required in Table 7. Further consequential modifications are made to the frequency ranges.

MOD	ASP/25/232

TABLE  9a

Parameters required for the determination of coordination distance for a transmitting earth station
in bands shared bidirectionally with receiving earth stations

| Space service designation in which the transmitting earth station operates |  | ... | Mobile-satellite |  | ... |
| --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  |  | 1.670-1.675 |  |  |
| Space service designation in which the receiving earth station operates |  |  | Meteorological-satellite |  |  |
| Orbit 6 |  |  | Non-GSO | GSO |  |
| Modulation at receiving earth station 1 |  |  | N | N |  |
| Receiving earth station interference parameters and criteria | p0 (%) |  | 0.006 | 0.011 |  |
|  | n |  | 3 | 2 |  |
|  | p (%) |  | 0.002 | 0.0055 |  |
|  | NL (dB) |  | 0 | 0 |  |
|  | Ms (dB) |  | 2.8 | 0.9 |  |
|  | W (dB) |  | 0 | 0 |  |
| Receiving earth station parameters | Gm (dBi) 2 |  | 30 | 45 |  |
|  | Gr (dBi) 4 |  | 19 9 | 8 |  |
|  | min 5 |  | 5 | 3 |  |
|  | Te (K) 7 |  | 370 | 118 |  |
| Reference bandwidth | B (Hz) |  | 106 | 4  103 |  |
| Permissible interference power | Pr( p) (dBW) in B |  | –145 | –178 |  |

Reasons:	Consequential modifications to the frequency ranges in the Tables of Appendix 7.

MOD	ASP/25/233

TABLE  10

Predetermined coordination distances

| Reasons: Consequential modifications to the frequency ranges in the Tables of Appendix 7. Frequency sharing situation |  | Coordination distance (in sharing situations involving services allocated with equal rights) (km) |
| --- | --- | --- |
| Type of earth station | Type of terrestrial station |  |
| ... |  |  |
| Ground-based in the bands: 400.15-401 MHz 1 670-1 675 MHz | Station in the meteorological aids service (radiosonde) | 580 |
| Aircraft (mobile) in the bands: 400.15-401 MHz 1 670-1 675 MHz | Station in the meteorological aids service (radiosonde) | 1 080 |
| ... |  |  |

ARTICLE  9

Procedure for effecting coordination with or 
obtaining agreement of other administrations1, 2, 3, 4, 5, 6

Section II  –  Procedure for effecting coordination10, 11

Sub-Section IIA  –  Requirement and request for coordination

ADD	ASP/25/234

9.16bis 	ka)	for a typical earth station of a satellite network for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to No. 9.11A, in respect of specific earth stations operating in the opposite direction of transmission, in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission and where the coordination area of the earth station includes the territory of another country or the earth station is located within the coordination area of another earth station;

Reasons:	Article 9 provision No. 9.16bis to provide for coordination with typical MSS earth stations that are not covered in other provisions referenced under No. 9.11A. It restricts the effect to satellite systems called upon to coordinate via a footnote, whereas, changing No. 9.17A would have applied to all satellite systems.

MOD	ASP/25/235

9.11A	e)	for a station for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to this provision, the provisions of Nos. 9.12 to 9.16bis are applicable;     (WRC-03)

Reasons:	A consequence of adding No. 9.16bis as above.

(Proposal number ASP/25/236 is not used.)

ARTICLE  5

SUP	ASP/25/237

5.377

Reasons:	A consequence of removing allocation of MSS in the band 1 675-1 710 MHz for Region 2.

SUP	ASP/25/238

RESOLUTION  227  (WRC-2000)

Sharing studies for, and possible additional allocations to, the mobile-satellite 
service (Earth-to-space) in the 1-3 GHz range, including consideration
 of the band 1 683-1 690 MHz

Reasons:	A consequence of finding a global allocation for MSS in the band 1 670-1 675 MHz.

____________