MOD	ARB/27A28/7

NOTE 4 – Exceptions for the band 1 492-1 525 MHz are as follows:

4.1	For the land mobile service on the territory of Japan (MOD No. 5.348A): –150 dB(W/m2) in 4 kHz at all angles of arrival is applicable to all satellite space-to-Earth emissions.

4.2	For the aeronautical mobile service for telemetry in the territory of the United States, the requirement for coordination is determined by )the following pfd thresholds, in 4 kHz:

–156	dB(W/m2)	for	0º	≤ δ ≤ 4º

–156.4 + 0.111 δ	dB(W/m2)	for	4º	< δ ≤ 60º

–150	dB(W/m2)	for	60º	< δ ≤ 90º

where δ is the angle of arrival (degrees).

4.3	For the aeronautical mobile service for telemetry on the territory of the administrations listed in No. 5.342: 
–140 dB(W/m2) in 4 kHz at all angles of arrival.

Reasons:	To define the pfd coordination thresholds for the protection of terrestrial services.