| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 29 to Document 27-E |
|  |  |  | 16 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Arab States Common Proposals |  |
| Proposals for the work of the conference |  |
| Agenda item 1.32 |  |

1.32	to consider technical and regulatory provisions concerning the band 37.5-43.5 GHz, in accordance with Resolutions 128 (Rev.WRC-2000) and 84 (WRC-2000)

Resolution 128

# 1	Proposal

The Arab Administrations support Method C of the CPM Report to WRC-03 which proposes replacing No. 5.551G by two new footnotes to apply to the non-GSO in the band 42.0-42.5 GHz as well as to GSO. The levels derived from the protection criteria in Recommendation ITU-R RA.769 would apply at radiotelescopes notified prior to the submission of advance publication information by the subject satellite network or system. Consequently, Resolution 128 can be suppressed.

ARTICLE  5

MOD	ARB/27A29/1

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) 5.551AA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F ADD 5.551GX ADD 5.551GY |  |  |

SUP	ARB/27A29/2

5.551G

Reasons:	To be replaced with two new footnotes.

ADD	ARB/27A29/3

5.551GX	The equivalent power flux-density produced by all the space stations in any non-geostationary-satellite system in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service (space-to-Earth) system operating in the 42-42.5 GHz band shall not exceed at the radio astronomy station (depending on the type of observations conducted) for more than 2% of the time:

–	230 dB(W/m2) in 1 GHz (for single dish continuum observations) in the band 42.5-43.5 GHz;

–	–246 dB(W/m2) in any 500 kHz (for single dish spectral-line observations) in the band 42.5*-43.5 GHz; or

–	–209 dB(W/m2) in any 500 kHz (for VLBI observations) in the band 42.5-43.5 GHz.

These epfd shall be evaluated using the antenna pattern given in Annex 2 to Recommendation ITU-R S.1586 and a maximum RAS antenna gain of 93 dBi shall apply for elevation angles higher than the minimum operating angle θmin of the radiotelescope (for which a default value of 5° should be adopted in the absence of notified information).

These values shall apply at any radio astronomy station that has been notified to ITU either before 4 July 2003 or before the date of receipt of the advance publication information (API) of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	Consequential to the ITU-R studies.

ADD	ARB/27A29/4

5.551GY	The power flux-density produced by any geostationary space station in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service (space-to-Earth) operating in the band 42-42.5 GHz shall not exceed at a radio astronomy station (depending on the type of observations conducted):

–	137 dB(W/m2) in 1 GHz (for single dish continuum observations) in the band 42.5-43.5 GHz;

–	–153 dB(W/m2) in any 500 kHz (for single dish spectral-line observations) in the band 42.5*–43.5 GHz; or

–	–116 dB(W/m2) in any 500 kHz (for VLBI observations) in the band 42.5-43.5 GHz.

These values shall apply at any radio astronomy station that has been notified to ITU either before 4 July 2003 or before the date of receipt of the advance publication information (API) of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	Consequential to the Arab proposal and to the ITU-R studies.

APPENDIX  4  (WRC-2000)

Consolidated list and tables of characteristics for use in the 
application of the procedures of Chapter III

ANNEX  2A

Characteristics of satellite networks’ earth stations 
or radio astronomy stations2     (WRC-2000)

MOD	ARB/27A29/5

## A.17	Compliance with equivalent power flux-density limits

...

b)	For non-geostationary-satellite systems operating in the fixed-satellite service and broadcasting-satellite service in the band 42-42.5 GHz the calculated equivalent power flux-density at the site of a radio astronomy station in the band 42.5-43.5 GHz, as defined in No. 5.551GX.

Reasons:	Consequential to the Arab proposal and to the ITU-R studies.

ADD	ARB/27A29/6

## A.18	Compliance with power flux-density limits

For geostationary satellites operating in the fixed-satellite service and broadcasting-satellite service in the band 42-42.5 GHz, the calculated power flux-density produced at the site of a radio astronomy station in the band 42.5-43.5 GHz, as defined in No. 5.551GY.

Reasons:	Consequential to the Arab proposal and to the ITU-R studies.

SUP	ARB/27A29/7

RESOLUTION  128  (Rev.WRC-2000)

Protection of the radio astronomy service in the 42.5-43.5 GHz band

Reasons:	This Resolution is no longer required, since the studies have been completed for the provisional limits identified in 5.551G for the protection of the radio astronomy from satellite service networks.

***

Resolution 84

# 1	Introduction

Studies have been performed within ITU-R to assess the effect on the fixed service in the 37.5-42.5 GHz band from the non-GSO and GSO FSS pfd limits included in Table 21-4. For non-GSO FSS satellites, the results of these studies are contained in Recommendation ITU-R F.1484, which recommends that the current pfd levels in Table 21-4 of Article 21 are sufficient to protect the fixed service. For the GSO FSS case, the results of these studies are contained in Recommendation ITU-R SF.1573. ITU-R has agreed that the current pfd levels in Table 21-4 of Article 21 provide a sufficient level of protection to the fixed service.

# 2	Proposal

The Arab Administrations support the ITU studies conclusion that there is no need to change the current pfd limits in Article 21 where it shows that these limits are sufficient to protect the fixed service, and consequently Resolution 84 (WRC-2000) can be suppressed. Also Arab Administrations propose to suppress Nos. 21.16.11, 21.16.12 and 21.16.13, which are notes to Table 21-4 of Article 21.

Since ITU-R has not completed the studies with regard to the use of other portions of the 37.5-42.5 GHz band for high density requested in invites 6 of Resolution 84 (WRC-2000) on the appropriate criteria and techniques, the Arab Group is addressing a development of a new Resolution based on invites 6 of Resolution 84 (WRC-2000).

ARTICLE  5

SUP	ARB/27A29/8

5.551AA

Reasons:	Providing alternative footnotes since the ITU-R studies showed that these pfd limits provided in Table 21-4 of Article 21 are sufficient to protect the fixed service.

ADD	ARB/27A29/9

5.551AAA	While addressing the sharing conditions with the fixed service in the bands 37.5-40 GHz and 40.5-42.5 GHz, the power flux-density at the Earth’s surface from any fixed-satellite service satellite should be at the level(s) required to meet the fixed-satellite service link availability and performance objectives of the subject applications. In any case, the levels shall not exceed the applicable power flux-density limits in Table 21-4.

Reasons:	Consequential to the ITU-R studies that any FSS satellite should meet the pfd limits provided in Table 21-4 of Article 21 in order to protect the fixed services.

MOD	ARB/27A29/10

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA MOBILE SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |

Reasons:	Consequential to the Arab proposal.

MOD	ARB/27A29/11

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) ADD 5.551AAA BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551G |  |  |

Reasons:	Consequential to the Arab proposal.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

MOD	ARB/27A29/12

TABLE  21-4 (continued)

| Reasons: Consequential to the Arab proposal. Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| ... |  |  |  |  |  |  |
| 37.5-40 GHz | Fixed-satellite (non-geostationary-satellite orbit) Mobile-satellite (non-geostationary-satellite orbit) | –120 10 | –120 + 0.75( – 5) 10 |  | –105 10 | 1 MHz |
| 37.5-40 GHz | Fixed-satellite (geostationary-satellite orbit) Mobile-satellite (geostationary-satellite orbit) | –127 | 5°-20° | 20°-25° | –105 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) | –107 + 0.4 ( – 20) |  |  |
| ... |  |  |  |  |  |  |
| 40.5-42 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –115 10 | –115 + 0.5( – 5) 10 |  | –105 10 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –120 | 5°-15° | 15°-25° | –105 | 1 MHz |
|  |  |  | –120 + ( – 5) | –110 + 0.5 ( – 15) |  |  |

MOD	ARB/27A29/13

TABLE  21-4 (end)

| Reasons: Consequential to the Arab proposal. Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –120 10 | –120 + 0.75( – 5) 10, |  | –105 10 | 1 MHz |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –127 | 5°-20° | 20°-25° | –105 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) | –107 + 0.4 ( – 20) |  |  |

SUP	ARB/27A29/14

_______________

16	21.16.11

SUP	ARB/27A29/15

_______________

17	21.16.12

SUP	ARB/27A29/16

_______________

18	21.16.13

Reasons:	Consequential to the Arab proposal.

SUP	ARB/27A29/17

RESOLUTION  84  (WRC-2000)

Power flux-density limits in the bands 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service

Reasons:	This Resolution is no more required since the ITU-R studies showed that the pfd limits provided in Table 21-4 of Article 21 are sufficient to protect the fixed service, and hence the studies are completed.

_____________

* 	Arab Administrations are still considering the lowest frequency in the 42.5-43.5 GHz range at which the spectral line protection level should begin and that it is premature to propose a value for this WRC.