MOD	B/35/16

TABLE  21-4 (continued)

| Reasons: Since there is no special definition for HEO satellites to reflect their particular characteristics, the FS in the 10.7-12.75 GHz band should be protected by the adoption of adequate limits on pfd produced by all types of non-GSO satellites including those in highly-elliptical orbits. These pfd levels should be limited mainly for the lowest angles of arrival in order to avoid situations in which highly-elliptical orbits may repeatedly appear within the main beams of FS antennas for significant duration of time. The proposed limits are identified as Mask E in the ITU-R CPM-02 Report. Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 10.7-12.75 GHz | Fixed-satellite (space-to-Earth) (non-geostationary-satellite orbit) | –147 | –147 + 1.75(– 5) | –112 | 1 MHz |
| ... |  |  |  |  |  |