ADD	KOR/109A4/2

5.WAS1 	The use of the 5 150-5 250 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems including radio local area networks, which shall be operated in accordance with the following conditions (see also Resolution [RLAN1](WRC-03)):

–	deployment of wireless access systems including radio local area networks shall be restricted to indoor use only;

–	the mean equivalent isotropically radiated power density limit of any wireless access system station, including radio local area network stations, shall not exceed 10 mW (10 dBm) in any 1 MHz band (or equivalently 0.04 mW 
(–14 dBm) in any 4 kHz band).

In the band 5 150-5 250 MHz, stations in the mobile service shall not claim protection from earth stations in the fixed-satellite service. The provisions of No. 5.43A do not apply to the mobile service allocation in the band 5 150-5 250 MHz.

NOTE – KOR/109A3/9 and KOR/109A4/2 are identical.

Reasons:	The use of RLAN in the 5 150-5 250 MHz band must be considered carefully. Considering the importance of sharing the radio resource with the existing services in this band, the Republic of Korea recognizes the need for the protection of the existing MSS feeder links. In addition, in order to simplify the footnotes that may be required to specify the technical restrictions on relevant systems that facilitate sharing of the spectrum, these restrictions are incorporated in the form of Resolutions. This footnote can support the protection of the existing services.