| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION DEVELOPMENT BUREAU ITU-D STUDY GROUPS | Document 1/114(Rev.1)-E 25 September 2000 Original: English |
| THIRD MEETING OF STUDY GROUP 1: GENEVA, 11 - 15 SEPTEMBER 2000 THIRD MEETING OF STUDY GROUP 2: GENEVA, 18 - 22 SEPTEMBER 2000 |  |

FOR ACTION

Question 7/1: 	Universal access/service

STUDY GROUP 1

SOURCE:	ZAMBIA

TITLE:	CASE STUDY: UNIVERSAL SERVICE/ACCESS

________

Action required:

It is proposed that the document be inserted among the case studies considered in the Final Report on Question 7/1.

Abstract:

The document gives an overview of the situation in Zambia on universal service/access.

1.	INTRODUCTION

Zambia is a landlocked country with an area of 752,620 square kilometres and a population of about 10 million people. The country is situated in the southern part of Africa and shares borders with eight other African countries, these being Angola, Botswana, Democratic Republic of Congo, Malawi, Mozambique, Namibia, Tanzania and Zimbabwe.

Zambia is a member of the Organisation of African Unity (OAU) and also of the regional groupings of Southern African Development Community (SADC) and the Common Market of Eastern and Southern Africa (COMESA), which has its Secretariat in the Zambian Capital, Lusaka. Zambia is the current Chairman of the Pan-African Telecommunications Union (PATU). Zambia is also a signatory to the Lome Convention of ACP countries.

The Republic of Zambia gained political independence from Great Britain on the 24th of October 1964. Up until 1991, when multi-party elections were held in the first truly democratic setting, the Zambian economy was based on socialistic principles major business concerns such as the Copper Mines, National Airline, National Railways, Road Transport, National Broadcasting and Posts and Telecommunications were either wholly or in majority shareholding owned and controlled by the Government.

Although Zambia is a tropical country, the high altitude rising to some 2 164 metres above sea level ensures a generally cool environment. There are four seasons - cool dry season (April to August); hot dry season (August to November) and the warm wet season (November to April). The warmest moth is October (30-35 degrees Celsius) and the coolest is June (5-10 degrees Celsius). The country’s vegetation is mainly savannah with rainfall averaging 750 mm/year in the South and 1 270 mm/year in the North.

2	THE TELECOMMUNICATIONS SECTOR.

Zambia’s telecommunications sector is fully liberalised. Prior to liberalisation, postal and telecommunication services were provided by the Posts and Telecommunications Corporation (PTC) under the direct supervision of the Ministry responsible for Communications. During this period, the services offered 	by both institutions were concentrated mainly in urban areas.

2.1	Liberalisation.

In July, 1994, Parliament passed the Telecommunications Act. The Act ushered in a new era of private participation in the provision of telecommunication networks and services.

The Telecommunication Act provided for the separation of Policymaking, Regulatory and Operational functions in the sector.

Government, through the Ministry of Communications and Transport, is responsible for policy making.

The Telecommunications Act established the Communications Authority of Zambia (CAZ), which is responsible for the management and interpretation of policy through regulation and licensing. CAZ may grant licences for the provision and operation of telecommunication services to eligible individuals or organisations subject to prescribed conditions.

The passing of the Act also saw the break up of PTC into two autonomous operators. The Postal Division of PTC became the Zambia postal Services Corporation (ZAMPOST) and the Telecommunications Division became the Zambia Telecommunications Company (ZAMTEL).

2.2	Communications Authority of Zambia (CAZ).

The Communications Authority of Zambia (CAZ) was established by Government in order to regulate the provision of telecommunication services in the country. The main functions of CAZ are:

- To develop rules and procedures that would promote and safeguard social requirements and competition in the telecommunications sector;

- To protect the interest of users and consumers of telecommunication services;

- To create a fair playing field for operators and investors;

- To administer effective and efficient utilisation of the Radio Frequency spectrum.

2.3	Zambia Telecommunications Company Limited (ZAMTEL).

ZAMTEL, a national telecommunications operator, was formed as result of the split of the PTC. The company is still the pre-dominant provider of basic services such as Public Switched Telephone Service, International Access, Facsimile and Telex ZAMTEL is currently wholly Government owned.

ZAMTEL has a Universal Service Obligation (USO) to provide service to rural and sparsely populated areas even where service provision is not profitable. Following liberalisation, the company has been earmarked for privatisation and, at this year’s budget presentation, a 20% minority shareholding in Zamtel was announced

The telecommunications sector has opened up to competition and, so far, it has seen the entry of new operators in Mobile Cellular Service and other Value Added Services such as Public Call Offices and Internet provision.

Services such as Callback and Refiling, although yet to be prohibited by law, are 	discouraged.

3.	SERVICE PROVISION.

3.1	Sector Growth.

Although the sector in Zambia is still in its infancy, current liberalisation has stimulated some growth with new participants entering the market. By the end of 1998, there were four companies offering telecommunication services. Telecel and Zamcell joined Zamtel as providers of cellular phone services, Zynex operates Public Call Offices (PCO’s) and Zamnet and Coppernet are Internet Service Providers (ISPs). (The current operators in the sector and the types of services provided are shown in Table 1).

There are also a number of Customer Premise Equipment (CPE) providers, dealing in such equipment as Private Branch Exchanges (PBX’s), telephone hand sets, facsimile 	machines, telephone answering machines, mobile telephone handsets, etc.

The country still has a long way to go before the demand for telecommunication services can be considered as satisfied. In most areas, telephone penetration, measured as the number of direct exchange lines per 100 population, is less than 1 There is also a considerable disparity in telephone service distribution between urban and rural areas with the density averaging at 1.2 and 0.4 respectively.

3.2	The Mobile Telephone Market.

Mobile telephone services deserve special mention as the main growth area in the 	sector since the advent of liberalisation. There are currently three cellular phone operators.

Zamcell is the latest cellular phone operator to enter the market. The company has introduced, for the first time in Zambia, the pan-European digital Global Mobile System for communication (GSM) and is making arrangements to secure regional and international roaming agreements.

Telecel’s target is to have 12,000 subscribers in Lusaka and another 12,000 on the Copperbelt. When the network, was commissioned in 1997 it used Code Division Multiple Access (CDMA) digital technology supplied by Motorola. Telecel has since replaced the CDMA network with a GSM one. Telecel’s network has been expanded to cover most major towns on the Copperbelt.

Zamtel’s cellular network covers Lusaka and the Copperbelt and is based on analogue Advanced Mobile Phone Service (AMPS) technology supplied by NEC of Japan. There are 4 488 subscribers currently connected to the Zamtel cellular network. Plans are at an advanced stage for the Zamtel cellular network to migrate from analogue to digital using GSM technology. The planned expansion will be in two phases. The first phase, expected to be completed before year end 2001, will cover the major towns along the industrialised line of rail South-North run of the country The projected subscriber base for the first phase is about 25 000. The second phase will see service extended to the rest of the country.

4.	THE ZAMTEL NETWORK.

4.1	Overview.

Telecommunication Services available in the country include basic voice telephony, facsimile, Public Call Offices, Telex, Telegraphy, leased data circuits, Internet and radio and TV broadcasting. The most important of these services still remains basic telephony.

Zamtel owns most of the telecommunications infrastructure in the country and provides all wired public telephone services and the main national and international long distance circuits. The company operates a countrywide telecommunication services network that covers all administrative districts. The districts are served by a variety of telephone exchanges ranging from the old crossbar switches to modern digital ones. The network is connected mainly via analogue terrestrial microwave routes and satellite links using VSAT technology. The microwave links also provide a nation wide distribution network for radio and television.

4.2	Telephone Network.

The national network consists of 94 telephone exchanges (some of which combine local and transit functions), 8 satellite exchanges, 2 transit exchanges and one international gateway. Of these, 31 are digital systems, 44 are analogue electronic, 18 are analogue crossbar and one is a manual system. All the satellite exchanges are digital, linked to parent exchanges by cable PCM (six systems) and radio PCM (two systems). The digital international gateway has just been upgraded to handle CCS 7 signalling. The development trend is towards full digitalisation.

The total installed capacity is 126 760 of which some 70% are served by digital exchanges.

Telephone service is available in all the administrative districts and the national telephone density is 0.88 Direct Telephone Lines per 100 population. The growth rate for telephone service is about 3.5 %.

Sixty-five telephone exchanges, which consist of 41 rural and 24 urbancrossbar systems are very old and are no longer in production. These require replacement in order to 	achieve a higher grade of telephone service.

4.3	Public Call Offices (PCO’s).

Zamtel operates some 1 030 PCO’s installed nationwide. Eight hundred and eighty of these are token operated with denominations of 1 unit for local calls and 5 units for long distance national calls. A hundred and fifty of the PCO’s are card operated.

4.4	Telex Network.

The Telex network covers the whole country. Larger cities are connected through 	Time Division Multiplex (TDM) equipment, while Voice Frequency Telegraphy (VFT) circuits serve remote areas.

There are two exchanges in operation with a combined capacity of 4 504 lines. A 	Stored Program Controlled (SPC) exchange was commissioned in 1976 in Lusaka 	and a digital Telex exchange was commissioned in 1989 in Kitwe. The Lusaka exchange is old and plans to phase it out are in place. Due to declined usage of the Telex service, there are no plans to expand existing capacity.

4.5	National Transmission Network.

The national switching centres are connected by 28 microwave and 28 VHF/UHF 	radio links. Six of the microwave links, including the high usage route linking the Capital, Lusaka, to the Copperbelt region, are digital.

4.6	International Transmission Links.

International telecommunication service is provided via terrestrial radio over the Panaftel network and also via satellite using two Standard A Earth Stations working with the Intelsat global satellite network. The first Earth Station, Mwembeshi 1-A commissioned in 1985, operates in the Indian Ocean Region (IOR) and the second 	one, Mwembeshi 2-A, commissioned in 1988, operates in the Atlantic Ocean Region (AOR). Installation works are going on to replace the aged IOR earth station with a new digital system.

Low Cost Time Division Multiple Access (LC-TDMA) digital satellite technology has been implemented for Mwembeshi 2-A Earth Station.

4.7	Domestic Satellite (DOMSAT) Network.

In 1997, the central hub station for the DOMSAT network using Demand Assigned Multiple Access (DAMA) technology was commissioned in Lusaka. The aim of the network is to provide telephone and data services to customers in remote areas not yet connected to the Public Switched Telephone Network (PSTN).

To date, six remote stations are connected to the network and there are plans to connect more. The DOMSAT network is also capable of providing VSAT services for private companies and other organisations interested in setting up their own local or wide area data transmission networks.

4.8	Wireless Local Loop (WILL).

In order to promote development in rural areas with scattered communities and also to address its Universal Access/ Service Obligations, Zamtel is installing WILL telephone systems all over the country. WILL systems are also being installed in the subscriber distribution networks of other areas as a solution to the high incidence of theft and vandalism of copper cable.

4.9	Cellular Telephone Network.

The company first launched its cellular telephone service in the Capital, Lusaka, in 1995. The technology used is analogue Advanced Mobile Phone System (AMPS) with an initial capacity of 5 000 lines. The service has since expanded to cover the Copperbelt region and the number of connected subscribers stands at 4 488. Plans are in place for further expansion to cover the rest of the country using GSM technology. The two phase National Cellular Network project is expected to be completed by year end 2002.

Two other cellular phone operators, Telecel and Zamcell have set up networks the country. (Details in Paragraph 3.2)

4.10	Internet Service.

Zamtel became the second Internet Service Provider (ISP) in Zambia in 1997, with capacity to handle about 1000 subscribers from a Point of Presence (POP) in Lusaka. A higher capacity link has just been installed and a second POP is planned for the Copperbelt to cater for anticipated growth of Internet traffic. A third ISP, Coppernet, became operational in 1999.

5.	FUTURE INVESTMENTS.

5.1	Demand for Telecommunication services.

The current combined capacities of Zamtel and other service providers fail far short of satisfying customer demand. There is need to expand and modernise existing 	networks and services and to introduce new ones in order to meet both real and latent demand.

Developing of telecommunications is very capital intensive, especially since, like is the case with Zambia as a developing country, virtually all plant and machinery have to be imported. The changed circumstances in a liberalised economy also mean that, Zamtel, for example, can no longer depend upon the traditional support from external aid donors for soft loans and grants. The company has to devise new ways and means of securing funding for development of network infrastructure.

5.2	Fibre Optic Cables.

There are plans to install high capacity fibre optic cables to serve high traffic domestic routes. The cables will cater for existing and anticipated demand.

5.3	Spur Links.

There are plans to replace all existing open wire carrier systems with digital radio links in ring formation to serve as alternative routing to the star formation microwave links already in operation.

5.4	Wireless Local Loop (WILL) Systems.

The WILL system has proved cost effective in the provision of service to rural and farming communities. It has also proved its worth in the fight with vandalism and theft of copper cables. For these reasons, investment in WILL technology countrywide will continue. It is planned to install only digital WILL systems.

5.5	Satellite Communications.

The Mwembeshi 2-A Earth Station has been upgraded by provision of Intermediate Data Rate (IDR) and Digital Circuit Multiplex Equipment (DCME) to enable the antenna to meet the Intelsat operating requirements. Plans are underway to replace the fifteen year old Mwembeshi 1-A Earth Station with a fully digital system.

A number of private VSAT stations have been licensed to provide private wide area data networks.

5.6 	Regional Interconnectivity.

Plans are at an advanced stage to set up a regional network to link countries in the COMESA region. The proposed network, COMTEL, will link member countries using fibre optic cable and ATM technology. It has been proposed that the network is run as a joint venture between COMESA telecommunications operators and an offshore registered private company.

There are similar plans to connect member countries of the SADC region.

5.7	Data Communication Network.

There is need for an interconnected data communications network for the SADC region. Plans are underway for the implementation of the project.

5.8	Outside Plant (OSP).

Most of the OSP network consists of PVC insulated cable which is mostly procured locally from Metal Fabricators of Zambia (ZAMEFA). OSP comprises one of the major operational expenditures for Zamtel. Large quantities of cable are used to replace old and vandalised cables.

5.9	Global Mobile Personal Communication by Satellite (GMPCS).

Zambia is committed to adopt strategies that ensure that the country is not lagging too far behind in adopting new technologies such as GMPCS. The policy, regulatory, legal, technological, operational, social and economic factors relating to GMPCS have been studied. Government is considering the signing of the ITU GMPCS Memorandum of Understanding (MoU).

6.	CONSTRAINTS.

Development of telecommunications is a continuous process. The network requires to keep pace with increased demand and changing technologies. In the past developments in the telecommunications infrastructure in Zambia were achieved mainly through loans from international financial institutions such as the African Development Bank (ADB), World Bank, Commercial banks and assistance through bilateral aid from co-operating countries. In the present circumstances, there is no flow of such assistance.

New operators will require substantial capital injection in order to take off while newly privatised entities will require to secure alternative sources of project financing after being detached from Government.

The high taxation on imported equipment and generally the lack capital required for project implementation are the major constraints that are facing the sector.

Furthermore, the interest rates charged by local banks are prohibitive. Moreover, local banks lack the capacity to lend the huge sums of money associated with telecommunication development projects.

7.	UNIVERSAL ACCESS/SERVICE.

7.1	Background.

Before the creation of Zmatel and the Communications Authority of Zambia (CAZ) in 1994 as respective operating and regulatory entities of the national telecommunications network, Government, through the Posts and Telecommunications Corporation (PTC), was directly involved in the evolution of the network. Medium and long term development were part of National Development Plans. PTC was also responsible for radio frequency allocation and monitoring and also for Type Approval of subscriber terminal equipment. The rural network benefited from the favourable economic environment of the late sixties and early seventies and achieved considerable growth.

After liberalisation, one of the major immediate problems faced by CAZ was that of Universal Access/Service. While there exists strong Government goodwill for provision of at least basic telephone service to all parts of the nation, new methods of achieving this in the new competitive environment had to be worked out. CAZ is, at this stage, unequal to the task both in expertise and resources.

Universal Access/Service is currently a subject of national debate among the stakeholders and, while everyone understands and supports the goals of, at least, offering access to basic telephone service to every citizen, where there is less unanimity are the ways and means of achieving this.

Although not formally defined, the concept of Universal Access/service Obligation in Zambia, as is the case in many developing countries, is understood more as the provision of Access to, at least, basic telephone service of acceptable quality, at affordable cost and within reasonable distance. The concept of provision of Universal Service is considered as out of the capability of developing economies.

7.2	Universal Access Fund (UAF).

The setting up of a UAF for the provision of telecommunications services to rural and remote areas was alluded to in the Telecommunications Act of 1994. What was not made clear was how this fund was going to be set up and administered.

7.2.1	Fund Capitalisation.

One proposal for raising capital for UAF was through direct tax on revenues of operators of the so-called profitable networks. The idea emanates from the apparent success of the Road Tax in the transport sector where a percentage of revenues from fuel sales is used for maintenance of roads. It has been argued here that this may prove to be impractical since, unlike fuel, most telecommunications services are not sold on

Cash-on-Delivery basis. If UAF is to be collected on billing, like is currently the case with Value Added Tax (VAT), the operator would have to be paying huge amounts on billing, bad debts inclusive. There are also fears that, as is suspected with the Road Tax, the money collected under UAF could be diverted to fund other Government priority areas.

A variation of the above is that of capitalising UAF from a national blanket tax. It is argued that apart from taxing only those in formal employment, some of whom do not have telephone service, the tax net would not capture those in informal employment, some of whom do enjoy telephone service.

7.2.2	Network Operation and Maintenance.

Another important question is that of ownership and operation of the rural networks under UAF. It does not make economic sense for the Government to set up another operating entity while at the same time putting up Zamtel for privatisation. CAZ, who are supposed to oversee administration of the fund can not go into operations and the idea of funding a private operator through UAF seems too complicated to be considered seriously.

7.3	Free Rural Licence.

One of the first proposals to be considered was the offer of a free operating licence for rural networks as opposed to the very expensive licences for networks such as urban, national, international and cellular. The main concern with this solution is the geographic and economic definition of Rural Area. In addition, since telecommunications is recognised as a development engine, there is also the question of how long before an area under the free licence scheme could be graduated from Rural to Urban status.

7.4	Concessionary Obligations.

Another option under consideration is the making of the provision of service to rural areas a condition of granting licences for the other networks. Apart from the economic considerations, the technical viability of, for example, a cellular network operator setting up a separate network to cater for the rural areas would need to be considered. Such a network would need to be dedicated to the particular area and should not be just an extension of the operator’s cellular capability. As is the case with Free Rural Licence (Paragraph 6.3), the question of definition of Rural Area is important here.

  - Play or Pay.

The scheme, under which an operator chooses to either provide universal access or pay in lieu, has also been discussed. Complications here include determination of levels of payments from non-players. The impending privatisation of Zamtel also has a bearing on the adoption of such a scheme – the new owners might choose not to play.

7.6	New Technologies.

The settlement of the question of Universal Access/Service in Zambia has been considerably slowed down by the delayed realisation of the promise of new ways and means of offering service to remote areas by utilising Low Earth Orbit satellites and other satellite based technologies. The pan-African project RASCOM (Regional African Satellite Communication Organisation), for example, includes plans to extend service to within five kilometres of every African village.

8.	CONCLUSIONS.

Zambia, like many other developing countries of Africa, is striving to develop its telecommunications sector. In this age where the world is becoming a global village, there is need for countries to recognise the importance of putting in place strategies that will allow for development to take in all spheres of life, including the field of telecommunications.

The Zambian Government is currently engaged in a broad-based reform programme aimed at restructuring the economy and developing the private sector.

The Government has, therefore, recognised the need for private participation in the telecommunications sector so as to raise capital for the development of telecommunications and enhance efficiency in the provision of services.

The question of Universal Access/Service is a very important one. All stakeholders should put in every effort to see to it that it is resolved. The benefit of access to at least basic telephone service should be extended to every citizen. It might have been easy for Zamtel as a monopoly to extend service to remote subscribers. The current situation, however, should be looked at differently. It is worth noting that revenue from the national network, which might have been used to subsidise provision of service to remote areas, has declined considerably because of shared traffic with the new cellular operators.

Given in the Appendices are some national indicators to give an idea of how the country fares as a typical example of a Least Developed Country with a telephone density of less than One DEL per hundred of population (The current value of the national currency is about K 3 000 to 1 US $).

TABLE 1- SERVICE AVAILABILITY AND PROVISION.

| SERVICE |  |  | COMPANY |  |  |
| --- | --- | --- | --- | --- | --- |
| TYPE | TELECEL | ZAMCELL | ZAMNET | ZAMTEL | ZYNEX |
| FACSIMILE |  |  |  | * |  |
| LDC |  |  |  | * |  |
| INTERNET |  |  | * | * |  |
| MOBILE-N | * | * |  | * |  |
| MOBILE-R |  | * |  |  |  |
| PAGING |  |  |  |  | * |
| PCO-CD |  |  |  | * | * |
| PCO-CN |  |  |  | * |  |
| PSDN |  |  |  |  |  |
| PSTN |  |  |  | * |  |
| SES |  |  |  | * |  |
| TELEX |  |  |  | * |  |

LEGEND.

LDS		LEASED DATA CIRCUIT.

PSTN		PUBLIC SWITCHED TELEPHONE NETWORK.

PSDN		PUBLIC SWITCHED DATA NETWORK.

MOBILE-N	NATIONAL MOBILE TELEPHONE SERVICE.

MOBILE-R	MOBILE TELEPHONE SERVICE WITH INTERNATIONAL ROAMING.

PCO-CN	COIN OPERATED PUBLIC CALL OFFICE.

PCO-CD	CARD OPERATED PUBLIC CALL OFFICE.

SES			INTERNATIONAL SATELLITE EARTH STATION.

ZAMBIA

NATIONAL INDICATORS

APPENDIX A

POPULATION BY PROVINCE (x 1 000)

|  | 1980 | 1990 | 1994 | 1995 | 1996 | 1997 | 1998 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total National | 5 662 | 7 759 | 8 799 | 9 095 | 9 397 | 9 712 | 10 036 |  |
| Rural | 3 403 | 4 810 | 5 440 | 5 618 | 5 814 | 6 018 | 6 226 |  |
| Urban | 2 259 | 2 949 | 3 359 | 3 477 | 3 583 | 3 694 | 3 810 |  |
| Central | 512 | 771 | 888 | 915 | 939 | 964 | 989 |  |
| Rural | 361 | 549 | 635 | 654 | 673 | 693 | 713 |  |
| Urban | 151 | 222 | 253 | 261 | 266 | 271 | 276 |  |
| Copperbelt | 1 251 | 1 458 | 1 587 | 1 623 | 1 645 | 1 668 | 1 691 |  |
| Rural | 221 | 219 | 235 | 239 | 241 | 245 | 248 |  |
| Urban | 1 030 | 1 239 | 1 352 | 1 384 | 1 404 | 1 423 | 1 443 |  |
| Eastern | 651 | 1 004 | 1 149 | 1 193 | 1 245 | 1 299 | 1 355 |  |
| Rural | 588 | 915 | 1 046 | 1 086 | 1 133 | 1 182 | 1 233 |  |
| Urban | 63 | 89 | 103 | 107 | 112 | 117 | 122 |  |
| Luapula | 421 | 565 | 616 | 631 | 646 | 662 | 678 |  |
| Rural | 366 | 479 | 522 | 534 | 547 | 560 | 574 |  |
| Urban | 55 | 86 | 94 | 97 | 99 | 102 | 104 |  |
| Lusaka | 691 | 991 | 1 242 | 1 315 | 1 388 | 1 462 | 1 543 |  |
| Rural | 140 | 167 | 210 | 223 | 239 | 257 | 276 |  |
| Urban | 551 | 824 | 1 032 | 1 092 | 1 148 | 1 206 | 1 267 |  |
| Northern | 512 | 926 | 1 050 | 1 088 | 1 124 | 1 161 | 1 200 |  |
| Rural | 361 | 799 | 902 | 934 | 964 | 995 | 1 027 |  |
| Urban | 151 | 127 | 148 | 154 | 160 | 166 | 173 |  |
| N/Western | 303 | 438 | 495 | 510 | 528 | 547 | 562 |  |
| Rural | 262 | 378 | 426 | 439 | 454 | 470 | 482 |  |
| Urban | 41 | 60 | 69 | 71 | 74 | 77 | 80 |  |

| Southern | 672 | 965 | 1 081 | 1 114 | 1 161 | 1 210 | 1 261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rural | 505 | 754 | 845 | 874 | 910 | 947 | 986 |
| Urban | 167 | 221 | 236 | 240 | 251 | 263 | 275 |
| Western | 486 | 638 | 691 | 704 | 721 | 738 | 756 |
| Rural | 404 | 558 | 619 | 634 | 652 | 670 | 689 |
| Urban | 82 | 80 | 72 | 70 | 69 | 68 | 67 |

NOTES:	1980 and 1990 are census figures while 1994 to 1998 are projections
 	Average population growth rate: 3.1 per cent
	Average number of births per woman: 6.5 (1992) and 6.1 (1996)
	Infant mortality rate (per 1 000): 107 (1992) and 109 (1996)
	Under five mortality rate (per 1 000): 191 (1992) and 197 (1996)

APPENDIX B

EMPLOYMENT (INFORMAL SECTOR)

1993	1996

Total Rural Male	844 128	926 494

Total Rural Female	950 285	1 109 022

Total Rural Both	1 794 413	2 035 516

Total Urban Male	107 850	217 019

Total Urban Female	121 041	229 852

Total Urban Both	228 891	446 871

Total Male	951 978	1 143 513

Total Female	1 071 326	1 338 874

Total Both	2 023 034	2 482 387

APPENDIX C

EMPLOYMENT (FORMAL SECTOR x 1 000)

1993	1994	1995	1996	1997 	1998

Total	520.0	496.0	485.0	479.4	475.2	465.0

Transport and	29.0	29.0	36.5	38.3	45.9	46.1
Communications

Agriculture, Forestry	82.8	78.3	69.1	68.3	58.9	57.5
And Fisheries

Mining and Quarrying	58.2	51.2	52.2	47.7	44.5	39.4

Manufacturing	67.6	57.1	55.7	47.4	47.1	43.3

Electricity and Water	5.7	5.1	5.0	4.4	5.0	4.8

Construction	22.1	17.5	10.5	13.1	17.1	18.3

Catering, Hotels	49.3	49.9	41.4	46.8	48.9	49.9
and Restaurants

Finance, Insurance, Real	37.0	34.1	41.9	37.6	37.8	37.6
Estate and Business Services

Community, Social and	168.3	173.8	172.6	125.8	169.8	169.9

Redundancies	4 873.0	10 025.0	7 564.0	4 146.0	4 474.0	9 699.0

APPENDIX D

INDUSTRIAL PRODUCTION

1993	1994	1995	1996	1997 	1998

Electricity Generated	7 868.6	8 140.9	8 129.4	7 130.1	7 856.2	7 280.0
(Million kW).

Electricity Consumption	6 960.4	6 739.0	7 147.0	3 559.0	6 354.4	6 474.4
(Million kW).

Copper Production	403.5	360.2	307.1	314.6	308.9	242.2
(x 1 000 Tonnes)

Zinc (x 1 000 Tonnes)	5.6	0.1	-	-	-	-

Lead (x 1 000 Tonnes)	1.6	-	-	-	-	-

Coal (x 1 000 Tonnes)	329.0	135.0	152.0	128.1	165.0	-

Cobalt (x 1 000 Tonnes)	4.2	2.6	2.8	4.8	4.2	-

APPENDIX E

INDEX OF INDUSTRIAL PRODUCTION (1980 = 100)

1993	1994	1995	1996	1997 	1998

Total Index	       88.3	       77.3	       73.1	  71.3	 77.1	 74.2

Mining	       73.5	 60.9	 54.5	  62.3	       62.9	 60.4

Manufacturing	 111.8	101.1	 98.5	  84.4	 96.9	 94.3

Electricity	 85.3	 88.3	 89.7	  77.5	 89.8	 82.8

APPENDIX F

GROSS DOMESTIC PRODUCT (GDP COMPONENTS x BILLION KWACHA)

1994	1995	1996	1997	1998

Government Final Consumption		293.6	464.0	 721.6	 898.0	951.5

Private Final Expenditure Consumption		1 780.8	2 168.7	3 015.9	3 747.2	4 428.6

Gross Fixed Capital Formation		253.6	373.4	442.8	675.1	908.1

Increase in Stocks		69.0	104.8	 63.1	  77.3	 95.0

Exports		806.5	1 082.3	1 237.4	1 519.4	1 621.7

Total GDP		2 240.7	2 998.3	3 944.8	5 169.0	5 921.1

Per Capita GDP (x 1 000 Kwacha)	    	 255.8	      329.1	417.4	 528.5	586.3

APPENDIX G

INFLATION RATES AND PURCHASING POWER OF THE LOCAL CURRENCY

1995	1996	1997 	1998

Metropolitan Low Income Group	       	46.1	 34.6	 17.2	 31.1

Metropolitan High Income Group		 40.8	 36.3	 19.1	 30.0

Non-Metropolitan Income Group		 49.5	 34.8	 19.0	 30.7

Composite		 46.0	 35.2	 18.6	 30.6

Purchasing Power of One Kwacha		 74.1	 51.8	 41.6	 33.5

APPENDIX H

CONSUMER PRICE INDEX (1994 = 100)

1995	1996	1997 	1998

Metropolitan Low Income Group			 135.8	 192.8	 237.8	 280.9

Metropolitan High Income Group			 135.3	 188.3	 235.1	 276.8

Non-Metropolitan Income Group			 134.2	 196.3	 245.0	 288.9

Wholesale Price Index (1996 = 100)			396 390.2	466 145.5	568 451.5	268 749.6

Price Index of Building Materials (1974 = 100)		348 133.9	28 233.2	435 974.9	541 531.3

APPENDIX I

MONEY AND BANKING (MILLION KWACHA)

1994	1995	1996	1997	1998

Money Supply		138 829.9	216 050.6	272 916.7	379 797.7	360 104.9

Bank Loans and Advances		144 168.2	213 121.3	397 322.1	328 421.9	575 171.9

Savings and Time Deposits		173 682.8	221 522.7	288 615.7	305 542.7	258 898.8

APPENDIX J

PUBLIC FINANCE (BILLION KWACHA)

1994 	1995	1996	1997 	1998

Total Revenue and Grants		503.4	 763.1	 618.4	1 246.7	1 496.7

Total Revenue		 502.8	 757.5	 593.0	959.3	1 163.7

Current Revenue		 502.6	 757.5	 592.8	  958.9	1 162.9

Tax Revenue		  21.9	 671.7	 544.7	  933.7	1 089.6

Non-Tax Revenue	 	 80.7	  85.8	  48.1	   25.2	  88.3

Capital Revenue		   0.2	   -	   0.2	0.4	0.8

Grants		   0.6	  5.6	  25.4	 287.4	 333.0

Total Expenditure and Net Lending		530.8	743.4	 866.8	1 713.8	1 710.5

Total Expenditure		 492.2	727.7	 842.6	1 575.6	1 572.2

Current Expenditure	      	 260.5	440.4	 720.0	  906.1	1 160.9

Capital Expenditure		 231.7	287.3	 122.0	  669.5	 411.3

Overall Surplus (Deficit)	    	  (27.4)	 19.7	(248.4)	(467.1)	(213.8)

Financing		 27.4	(19.7)	 248.4	  467.1	213.8

Abroad		 73.9	488.3	 187.4	  772.4	 280.2

Domestic		(46.5)	(508.0)	  61.0	 (305.3)	 (66.4)

APPENDIX K

BALANCE OF PAYMENTS (MILLION KWACHA)

1995		1996		1997

Exports		1 133 874.57		1 274 224.25		1 500 362.12

Imports		(725 310.66)		(1 076 202.56)		(1 326 979.07)

Trade Balance		408 563.91		198 021.68		173 383.05

Non-Factor Services (Net)		(567 281.49)		(698 142.51)		801 649.03

Investment Income (Net)		 (258 633.17)		(273 469.51)		286 720.13

Current Account Balance		 (146 861.57)		(405 330.04)		(619 871.61)

Overall Balance		 (249 932.38)		(193 044.85)		(242 387.38)

APPENDIX L

EDUCATION (1994 FIGURES)

____________________________________________________________

Primary School	1 507 660

Secondary School	 199 154

Teacher Training Colleges	 4 598

Vocational Training Institutes	4 888

Universities	3 734

APPENDIX M

HEALTH

1993	  	 1994		1995

Number of Hospitals	  	82		    82	 	  84

Number of Beds in Hospitals		17 077		16 999	   	 16 960

Number of Health Centres	 	 1 106		 1 110	   	  1 082

Number of Beds in Health Centres	 	 9 322		 9 539	   	  9 502

Zambia Telecommunications Company Limited

Telephone Exchange Statistics

| EXCHANGE | EQUIPPED CAPACITY |  | WORKING LINES |  | CLASS |  | TECHNOLOGY |  | MAKE |  | CONNECTED CAPACITY % |  | DEMAND | WAITING LIST | MANUFAC-TURER | YEAR INSTALLED |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIRPORT | 512 |  | 265 |  | URBAN |  | ELECTRONIC |  | MCR |  | 51.76 |  | 272 | 7 | ITT | 1980 |  |
| CHELSTON | 3 000 |  | 1 786 |  | URBAN |  | ELECTRONIC |  | NXIE |  | 59.53 |  | 2 368 | 582 | ITT | 1984 |  |
| CHILANGA | 1 200 |  | 510 |  | URBAN |  | DIGITAL |  | E 10 |  | 42.50 |  | 689 | 179 | ALCATEL | 1996 |  |
| CHINIKA | 2 500 |  | 1 261 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 50.44 |  | 1 507 | 246 | NEC | 1990 |  |
| CHIRUNDU | 96 |  | 83 |  | RURAL |  | ELECTRONIC |  | MCR |  | 86.46 |  | 210 | 127 | STK | 1987 |  |
| CHISAMBA | 256 |  | 88 |  | RURAL |  | ELECTRONIC |  | MCR |  | 34.38 |  | 93 | 5 | STK | 1987 |  |
| CHONGWE | 96 |  | 82 |  | RURAL |  | ELECTRONIC |  | MCR |  | 85.42 |  | 83 | 1 | STK | 1988 |  |
| EMMASDALE | 3 872 |  | 1 862 |  | URBAN |  | DIGITAL |  | NEAX 62E |  | 48.09 |  | 2 449 | 587 | NEC | 1989 |  |
| KAFUE | 2 000 |  | 1 053 |  | URBAN |  | ELECTRONIC |  | NX 1E |  | 52.65 |  | 1 166 | 113 | ITT | 1982 |  |
| LUANGWA | 96 |  | 74 |  | RURAL |  | ELECTRONIC |  | MCR |  | 77.08 |  | 74 | 0 | ITT | 1984 |  |
| LUSAKA | 16 500 |  | 11 027 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 66.83 |  | 11 908 | 881 | NEC | 1989 |  |
| MAKENI | 1 000 |  | 901 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 90.10 |  | 1 212 | 311 | NEC | 1990 |  |
| MUMBWA | 400 |  | 253 |  | URBAN |  | CROSS BAR |  | ARF |  | 63.25 |  | 333 | 80 | ERICSSON | 1980 |  |
| NAMALUNDU | 96 |  | 96 |  | RURAL |  | ELECTRONIC |  | MCR |  | 100.00 |  | 99 | 3 | STK | 1987 |  |
| NAMPUNDWE | 96 |  | 96 |  | RURAL |  | ELECTRONIC |  | MCR |  | 100.00 |  | 101 | 5 | STK | 1988 |  |
| RIDGEWAY | 5 500 |  | 4 582 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 83.31 |  | 4 827 | 245 | NEC | 1994 |  |
| ROMA | 3 500 |  | 2 803 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 80.09 |  | 3 433 | 630 | NEC | 1989 |  |
| SIAVONGA | 576 |  | 285 |  | URBAN |  | ELECTRONIC |  | MCR |  | 49.48 |  | 308 | 23 | STK | 1987 |  |
| WOODLANDS | 7 000 |  | 4 331 |  | URBAN |  | DIGITAL |  | E 10B |  | 61.87 |  | 4 737 | 406 | ALCATEL | 1987 |  |
| CHAMBESHI | 500 |  | 192 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 38.40 |  | 315 | 123 | NEC | 1991 |  |
| CHILILABOM | 2 000 |  | 939 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 46.95 |  | 1 125 | 186 | NEC | 1993 |  |
| CHILNGOLA | 3 000 |  | 2 604 |  | URBAN |  | CROSSBAR |  | ARF |  | 86.80 |  | 2 882 | 276 | ERICSSON | 1981 |  |
| CHINSALI | 192 |  | 116 |  | RURAL |  | ELECTRONIC |  | MCR |  | 60.42 |  | 117 | 1 | ITT | 1983 |  |
| ISOKA |  | 256 |  | 133 |  | RURAL |  | ELECTRONIC |  | MCR |  | 51.95 | 134 | 1 | ITT |  | 1983 |
| ITIMPI |  | 500 |  | 125 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 25.00 | 139 | 14 | NEC |  | 1991 |
| KABOMPO |  | 128 |  | 93 |  | RURAL |  | ELECTRONIC |  | MCR |  | 72.66 | 142 | 49 | ITT |  | 1985 |
| KALULUSHI |  | 3 000 |  | 748 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 24.93 | 777 | 29 | NEC |  | 1993 |
| KAPUTA |  | 240 |  | 34 |  | RURAL |  | DIGITAL |  | 4300 R |  | 14.17 | 38 | 4 | ALCATEL |  | 1996 |
| KASAMA |  | 2 000 |  | 1 077 |  | URBAN |  | DIGITAL |  | E 10 |  | 53.85 | 1 130 | 53 | ALCATEL |  | 1995 |
| KASEMPA |  | 128 |  | 114 |  | RURAL |  | ELECTRONIC |  | MCR |  | 89.06 | 174 | 60 | ITT |  | 1985 |
| KAWAMBWA |  | 256 |  | 80 |  | RURAL |  | ELECTRONIC |  | MCR |  | 31.25 | 114 | 34 | ITT |  | 1983 |
| KITWE |  | 12 000 |  | 6 622 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 55.18 | 7 117 | 495 | NEC |  | 1991 |
| LUANSHYA |  | 3 000 |  | 2 000 |  | URBAN |  | CROSSBAR |  | ARF |  | 66.67 | 2 734 | 734 | ERICSSON |  | 1978 |
| LUWINGU |  | 256 |  | 74 |  | RURAL |  | ELECTRONIC |  | MCR |  | 28.91 | 74 | 0 | ITT |  | 1984 |
| MANSA |  | 1 000 |  | 656 |  | RURAL |  | DIGITAL |  | NEAX 61E |  | 65.60 | 872 | 216 | NEC |  | 1990 |
| MASAITI |  | 96 |  | 82 |  | RURAL |  | ELECTRONIC |  | MCR |  | 85.42 | 180 | 98 | STK |  | 1989 |
| MBALA |  | 640 |  | 374 |  | RURAL |  | ELECTRONIC |  | MCR |  | 58.44 | 383 | 9 | ITT |  | 1983 |
| MINDOLO |  | 3 000 |  | 1 606 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 53.53 | 1 705 | 99 | NEC |  | 1993 |
| MPIKA |  | 768 |  | 314 |  | RURAL |  | ELECTRONIC |  | MCR |  | 40.89 | 390 | 76 | ITT |  | 1983 |
| MPONGWE |  | 96 |  | 29 |  | RURAL |  | DIGITAL |  | 4 300R |  | 30.21 | 83 | 54 | ALCATEL |  | 1998 |
| MPOROKOSO |  | 256 |  | 99 |  | RURAL |  | ELECTRONIC |  | MCR |  | 38.67 | 99 | 0 | ITT |  | 1983 |
| MPULUNGU |  | 192 |  | 170 |  | RURAL |  | ELECTRONIC |  | MCR |  | 88.54 | 171 | 1 | ITT |  | 1983 |
| MUFULIRA |  | 3 000 |  | 1 658 |  | URBAN |  | CROSSBAR |  | ARF |  | 55.27 | 1 940 | 282 | ERICSSON |  | 1980 |
| MUFUMBWE |  | 96 |  | 61 |  | RURAL |  | ELECTRONIC |  | MCR |  | 63.54 | 88 | 27 | ITT |  | 1985 |
| MUNGWI |  | 128 |  | 95 |  | RURAL |  | ELECTRONIC |  | MCR |  | 74.22 | 95 | 0 | ITT |  | 1983 |
| MWENSE |  | 128 |  | 51 |  | RURAL |  | ELECTRONIC |  | MCR |  | 39.84 | 69 | 18 | ITT |  | 1983 |
| MWINILUNGA |  | 128 |  | 75 |  | RURAL |  | ELECTRONIC |  | MCR |  | 58.59 | 122 | 47 | ITT |  | 1985 |
| NAKONDE |  | 192 |  | 128 |  | RURAL |  | ELECTRONIC |  | MCR |  | 66.67 | 128 | 0 | ITT |  | 1983 |
| NCHELENGE |  | 128 |  | 77 |  | RURAL |  | ELECTRONIC |  | MCR |  | 60.16 | 107 | 30 | ITT |  | 1983 |
| NDOLA MAIN |  | 9 000 |  | 4 947 |  | URBAN |  | DIGITAL |  | E 10 |  | 54.97 | 5 327 | 380 | ALCATEL |  | 1987 |
| IND. NORTH |  | 1 300 |  | 951 |  | URBAN |  | DIGITAL |  | E 10 |  | 73.15 | 1 066 | 115 | ALCATEL |  | 1987 |
| IND. SOUTH |  | 1 000 |  | 271 |  | URBAN |  | DIGITAL |  | E 10 |  | 27.10 | 288 | 17 | ALCATEL |  | 1987 |
| KABUSHI |  | 2 200 |  | 1 009 |  | URBAN |  | DIGITAL |  | E 10 |  | 45.86 | 1 297 | 288 | ALCATEL |  | 1987 |
| KANSENSHI |  | 2 000 |  | 1 214 |  | URBAN |  | DIGITAL |  | E 10 |  | 60.70 | 1 445 | 231 | ALCATEL |  | 1987 |
| NORTHRISE |  | 1 300 |  | 657 |  | URBAN |  | DIGITAL |  | E 10 |  | 50.54 | 1 012 | 355 | ALCATEL |  | 1987 |
| PAMODZI |  | 1 300 |  | 349 |  | URBAN |  | DIGITAL |  | E 10 |  | 26.85 | 446 | 97 | ALCATEL |  | 1987 |
| SAMFYA |  | 256 |  | 143 |  | RURAL |  | DIGITAL |  | MCR |  | 55.86 | 235 | 92 | ITT |  | 1983 |
| SOLWEZI |  | 1 000 |  | 682 |  | RURAL |  | DIGITAL |  | NEAX 61E |  | 68.20 | 801 | 119 | NEC |  | 1990 |
| ZAMBEZI |  | 128 |  | 120 |  | RURAL |  | ELECTRONIC |  | MCR |  | 93.75 | 198 | 78 | ITT |  | 1985 |
| CHADIZA |  | 200 |  | 63 |  | RURAL |  | CROSSBAR |  | ARK |  | 31.50 | 135 | 72 | ERICSSON |  | 1981 |
| CHAMA |  | 128 |  | 74 |  | RURAL |  | ELECTRONIC |  | MCR |  | 57.81 | 95 | 21 | STK |  | 1987 |
| CHIBOMBO |  | 128 |  | 67 |  | RURAL |  | ELECTRONIC |  | MCR |  | 52.34 | 67 | 0 | STK |  | 1988 |
| CHIPATA A |  | 800 |  | 679 |  | URBAN |  | CROSSBAR |  | ARF |  | 84.88 | 976 | 297 | ERICSSON |  | 1981 |
| CHIPATA H |  | 1 000 |  | 592 |  | URBAN |  | CROSSBAR |  | C 23 HC |  | 59.20 | 596 | 4 | HITACHI |  | 1989 |
| CHOMA A |  | 800 |  | 530 |  | URBAN |  | CROSSBAR |  | ARF |  | 66.25 | 566 | 36 | ERICSSON |  | 1979 |
| CHOMA H |  | 1 000 |  | 309 |  | URBAN |  | CROSSBAR |  | C 23 H |  | 30.90 | 315 | 6 | HITACHI |  | 1991 |
| GWEMBE |  | 200 |  | 49 |  | RURAL |  | CROSSBAR |  | ARK |  | 24.50 | 49 | 0 | ERICSSON |  | 1980 |
| ITEZHI-TEZHI |  | 96 |  | 72 |  | RURAL |  | ELECTRONIC |  | MCR |  | 75.00 | 74 | 2 | STK |  | 1987 |
| KABWE |  | 4 000 |  | 2 612 |  | RURAL |  | ELECTRONIC |  | NX 1E |  | 65.30 | 3 128 | 516 | ITT |  | 1982 |
| KALABO |  | 128 |  | 93 |  | RURAL |  | ELECTRONIC |  | MCR |  | 72.66 | 131 | 38 | ITT |  | 1983 |
| KALOMO |  | 400 |  | 165 |  | RURAL |  | CROSSBAR |  | ARK |  | 41.25 | 171 | 6 | ERICSSON |  | 1979 |
| KAOMA |  | 256 |  | 149 |  | RURAL |  | ELECTRONIC |  | MCR |  | 58.20 | 255 | 106 | ITT |  | 1983 |
| KAPIRI |  | 384 |  | 250 |  | URBAN |  | ELECTRONIC |  | MCR |  | 65.10 | 255 | 5 | ITT |  | 1982 |
| KATETE |  | 300 |  | 178 |  | RURAL |  | CROSSBAR |  | ARK |  | 59.33 | 353 | 175 | ERICSSON |  | 1981 |
| L/STONE |  | 5 000 |  | 1 812 |  | URBAN |  | DIGITAL |  | E 18B |  | 36.24 | 2 037 | 225 | ALCATEL |  | 1987 |
| LUKULU |  | 96 |  | 71 |  | RURAL |  | ELECTRONIC |  | MCR |  | 73.96 | 77 | 6 | ITT |  | 1983 |
| LUNDAZI |  | 384 |  | 223 |  | RURAL |  | ELECTRONIC |  | MCR |  | 58.07 | 263 | 40 | STK |  | 1987 |
| MAAMBA |  | 256 |  | 138 |  | RURAL |  | ELECTRONIC |  | MCR |  | 53.91 | 139 | 1 | STK |  | 1987 |
| MAZABUKA |  | 1 000 |  | 776 |  | URBAN |  | CROSSBAR |  | ARF |  | 77.60 | 978 | 202 | ERICSSON |  | 1980 |
| MFUWE |  | 96 |  | 83 |  | RURAL |  | ELECTRONIC |  | MCR |  | 86.46 | 129 | 46 | STK |  | 1987 |
| MKUSHI |  | 384 |  | 319 |  | RURAL |  | ELECTRONIC |  | MCR |  | 83.07 | 348 | 29 | ITT |  | 1982 |
| MONGU |  | 1 000 |  | 738 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 73.80 | 858 | 120 | NEC |  | 1990 |
| MONZE |  | 1 000 |  | 414 |  | URBAN |  | CROSSBAR |  | ARF |  | 41.40 | 416 | 2 | ERICSSON |  | 1980 |
| NAMWALA |  | 128 |  | 107 |  | RURAL |  | ELECTRONIC |  | MCR |  | 83.59 | 108 | 1 | STK |  | 1987 |
| NYIMBA |  | 200 |  | 63 |  | RURAL |  | CROSSBAR |  | ARK |  | 31.50 | 99 | 36 | ERICSSON |  | 1981 |
| PEMBA |  | 300 |  | 51 |  | RURAL |  | CROSSBAR |  | ARK |  | 17.00 | 51 | 0 | ERICSSON |  | 1980 |
| PETAUKE |  | 400 |  | 197 |  | RURAL |  | CROSSBAR |  | ARK |  | 49.25 | 406 | 209 | ERICSSON |  | 1981 |
| SENANGA |  | 256 |  | 132 |  | RURAL |  | ELECTRONIC |  | MCR |  | 51.56 | 167 | 35 | ITT |  | 1983 |
| SERENJE |  | 384 |  | 213 |  | RURAL |  | ELECTRONIC |  | MCR |  | 55.47 | 215 | 2 | ITT |  | 1982 |
| SESHEKE |  | 240 |  | 79 |  | RURAL |  | DIGITAL |  | 4300 R |  | 32.92 | 79 | 0 | ALCATEL |  | 1996 |
| SINAZONGWE |  | 160 |  | 26 |  | RURAL |  | DIGITAL |  | 4300 R |  | 16.25 | 114 | 88 | ALCATEL |  | 1998 |
| SINDA |  | 200 |  | 45 |  | RURAL |  | CROSSBAR |  | ARK |  | 22.50 | 50 | 5 | ERICSSON |  | 1981 |
| ZIMBA |  | 100 |  | 49 |  | RURAL |  | MANUAL |  | PMBX |  | 49.00 | 53 | 4 | PLESSEY |  | 1972 |
| LUSAKA MOB |  | 4 500 |  | 3 752 |  | URBAN |  | DIGITAL |  | NEAX 61E |  | 83.38 | 3 752 | 0 | NEC |  | 1995 |
| TOTALS | 130 988 |  | 77 377 |  |  |  |  |  |  |  | 59.07 |  | 88 933 | 11 556 |  |  |  |

Zambia Telecommunications Company Limited

Some Important Milestones in the Development of Telecommunication Services in Zambia

| YEAR | EVENT |
| --- | --- |
| 1913 | First manual telephone exchange installed in Livingstone. |
| 1931 | First wireless stations installed at Mpika and Kabwe. |
| 1933 | First trunk services between Northern Rhodesia (Zambia), Southern Rhodesia (Zimbabwe) and South Africa. |
| 1957 | Subscriber Trunk Dialling introduced in main centres. |
| 1958 | Telex services introduced. |
| 1964 | Manual exchanges replaced by Strowger Step-by-Step automatic systems. |
| 1967 | Lusaka-Kabwe 960 Channel microwave link commissioned. |
| 1974 | First satellite Earth Station, Mwembeshi 1A, commissioned. |
| 1974 | Lusaka-Livingstone 960 Channel microwave link commissioned. |
| 1978 | Lusaka-Nakonde Panaftel microwave link commissioned. |
| 1980 | Lusaka-Chipata microwave link commissioned. |
| 1980 | Lusaka-Mongu microwave link commissioned. |
| 1985 | First digital exchange, AXE 10, introduced serving as International Gateway. |
| 1985 | International Direct Dialling (IDD) service introduced. |
| 1987 | First digital local exchange, Alcatel E10 B, installed in Ndola. |
| 1988 | Second satellite Earth Station, Mwembeshi 2A, commissioned. |
| 1995 | AMPS Mobile Telephone Service introduced in the capital, Lusaka. |
| 1997 | First Domestic Satellite (DOMSAT) network service introduced at Sesheke. |
| 1997 | The ZAMTEL.zm Internet Service introduced. |

Zambia Telecommunications Company Limited

Annual Route Summaries of International Telephone Traffic

(Outgoing Traffic in Minutes)

| ROUTE | 1994/5 | 1995/6 | 1996/7 | 1997/8 | 1998/9 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| ANGOLA | 0 | 119 | 0 | 0 | 0 | 119 |
| AUSTRALIA | 49 365 | 81 945 | 97 066 | 54 957 | 0 | 283 333 |
| BELGIUM | 100 511 | 102 056 | 105 606 | 123 830 | 107 982 | 539 985 |
| BOTSWANA | 287 725 | 271 177 | 260 922 | 260 972 | 314 867 | 1 395 663 |
| CANADA | 163 357 | 247 164 | 264 826 | 147 669 | 166 182 | 989 198 |
| DENMARK | 33 342 | 61 866 | 5 797 | 0 | 0 | 101 005 |
| ETHIOPIA | 0 | 2 344 | 0 | 0 | 0 | 2 344 |
| FINLAND | 21 259 | 23 059 | 2 356 | 0 | 0 | 46 674 |
| FRANCE | 375 065 | 353 758 | 242 384 | 108 205 | 208 881 | 1 288 293 |
| GERMANY | 426 677 | 446 048 | 501 640 | 277 287 | 272 249 | 1 923 901 |
| GREAT BRITAIN | 2 878 313 | 2 843 902 | 2 758 748 | 2 199 435 | 2 244 143 | 12 924 541 |
| HONG KONG | 0 | 0 | 0 | 0 | 9 636 | 9 636 |
| INDIA | 382 517 | 471 372 | 463 427 | 425 875 | 514 689 | 2 257 880 |
| ITALY | 174 648 | 172 976 | 158 026 | 446 830 | 321 597 | 1 274 077 |
| JAPAN | 93 422 | 118 236 | 115 414 | 127 545 | 122 403 | 577 020 |
| KENYA | 303 708 | 257 221 | 276 654 | 326 776 | 313 990 | 1 478 349 |
| LESOTHO | 2 906 | 137 | 0 | 15 | 0 | 3 058 |
| MALAWI | 126 249 | 55 490 | 200 324 | 239 943 | 202 309 | 824 315 |
| NETHERLANDS | 174 498 | 179 472 | 346 162 | 300 482 | 257 409 | 1 258 023 |
| NORWAY | 24 169 | 43 527 | 4 291 | 0 | 0 | 71 987 |
| SINGAPORE | 0 | 0 | 0 | 0 | 4 490 | 4 490 |
| SOUTH AFRICA | 3 120 173 | 3 547 665 | 3 964 818 | 4 682 620 | 4 554 053 | 19 869 309 |
| SWAZILAND | 7 152 | 2 124 | 1 141 | 26 | 0 | 10 443 |
| SWEDEN | 52 260 | 189 852 | 17 734 | 0 | 0 | 259 846 |
| TANZANIA | 203 130 | 225 347 | 254 370 | 269 192 | 277 669 | 1 229 708 |
| UGANDA | 41 069 | 27 550 | 28 037 | 12 395 | 3 022 | 112 073 |
| USA (AT&T) | 474 267 | 574 488 | 558 780 | 512 520 | 615 875 | 2 735 930 |
| USA (MCI) | 200 055 | 295 540 | 655 527 | 946 727 | 1 118 577 | 3 216 426 |
| ZIMBABWE | 1 212 120 | 1 422 358 | 1 350 354 | 1 455 081 | 1 469 803 | 6 909 716 |
| TOTAL | 10 927 937 | 12 016 793 | 12 634 404 | 12 918 382 | 13 099 826 | 61 597 342 |

Zambia Telecommunications Company Limited

Annual Route Summaries of International Telephone Traffic

(Incoming Traffic in Minutes)

| ROUTE | 1994/5 | 1995/6 | 1996/7 | 1997/8 | 1998/9 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| ANGOLA | 0 | 1 944 | 0 | 0 | 0 | 1 944 |
| AUSTRALIA | 103 707 | 129 465 | 159 077 | 58 313 | 647 | 451 209 |
| BELGIUM | 139 961 | 123 964 | 115 711 | 123 526 | 128 296 | 631 458 |
| BOTSWANA | 110 989 | 465 031 | 409 422 | 634 008 | 814 739 | 2 434 189 |
| CANADA | 469 263 | 572 278 | 609 857 | 875 415 | 774 400 | 3 356 213 |
| DENMARK | 84 775 | 91 768 | 68 782 | 89 324 | 65 430 | 400 079 |
| FINLAND | 79 393 | 54 198 | 23 444 | 13 964 | 31 225 | 202 224 |
| FRANCE | 202 512 | 252 154 | 151 245 | 173 455 | 602 300 | 1 381 666 |
| GERMANY | 282 793 | 277 659 | 261 959 | 317 267 | 235 453 | 1 375 131 |
| GREAT BRITAIN | 2 600 003 | 3 048 378 | 2 530 186 | 2 447 993 | 2 679 062 | 13 305 622 |
| GREECE | 15 492 | 43 229 | 38 917 | 24 048 | 48 763 | 170 449 |
| HONG KONG | 0 | 0 | 0 | 23 800 | 23 727 | 47 527 |
| INDIA | 261 569 | 374 745 | 268 153 | 446 768 | 397 740 | 1 748 975 |
| ITALY | 485 748 | 447 593 | 853 268 | 1 433 336 | 1 344 763 | 4 565 068 |
| JAPAN | 171 708 | 192 568 | 195 035 | 217 435 | 183 903 | 960 649 |
| KENYA | 138 465 | 233 611 | 251 764 | 271 047 | 233 413 | 1 128 300 |
| LESOTHO | 0 | 9 380 | 9 629 | 0 | 0 | 19 459 |
| MALAWI | 60 189 | 2 542 | 88 | 313 415 | 212 901 | 589 135 |
| NETHERLANDS | 161 956 | 247 426 | 301 712 | 364 592 | 305 019 | 1 380 705 |
| NORWAY | 113 968 | 16 874 | 128 027 | 126 627 | 129 728 | 515 224 |
| SINGAPORE | 0 | 0 | 0 | 16 116 | 0 | 16 116 |
| SOUTH AFRICA | 4 485 727 | 4 630 411 | 4 977 382 | 6 221 964 | 6 661 674 | 26 977 158 |
| SWAZILAND | 7 114 | 21 446 | 506 | 29 | 26 | 29 121 |
| SWEDEN | 109 490 | 96 153 | 84 079 | 103 661 | 128 553 | 521 936 |
| TANZANIA | 170 579 | 174 842 | 226 756 | 339 531 | 272 991 | 1 184 699 |
| UGANDA | 16 314 | 18 826 | 9 973 | 17 993 | 24 834 | 87 940 |
| USA (AT&T) | 2 449 201 | 1 697 741 | 1 873 847 | 1 984 864 | 1 977 239 | 9 982 892 |
| USA (MCI) | 1 477 330 | 1 557 038 | 2 477 254 | 2 399 831 | 1 255 383 | 9 166 836 |
| ZIMBABWE | 1 252 722 | 1 159 746 | 1 100 599 | 2 134 883 | 1 766 998 | 7 414 948 |
| TOTAL | 15 452 924 | 15 899 175 | 17 130 160 | 21 178 477 | 20 301 945 | 89 962 681 |

Zambia Telecommunications Company Limited

International Telephone And Telex Charges

(US $ Per Minute, Effective May 1, 1999)

COUNTRY	OLD RATE	NEW RATE	OFF PEAK	
			RATE

AFGHANISTAN	2.90	2.60	2.00

ALASKA	2.40	2.20	1.60

ALBANIA	2.80	2.50	1.90

ALGERIA	2.40	2.20	1.60

AMERICAN SAMOA	2.40	2.20	1.60

ANDORRA	2.20	2.00	1.50

ANGOLA	1.80	1.60	1.20

ANTIGUA	2.40	2.20	1.60

ARGENTINA	2.40	2.20	1.60

ASCENSION ISLAND	2.40	2.20	1.60

AUSTRALIA	2.20	2.00	1.50

AUSTRIA	2.40	2.20	1.60

AZERBAIJAN	2.80	2.50	1.90

AZORES	2.40	2.20	1.60

BAHAMAS	2.40	2.20	1.60

BAHRAIN	2.40	2.20	1.60

BANGLADESH	2.40	2.20	1.60

BARBADOS	2.40	2.20	1.60

BELIZE	2.90	2.60	2.00

BENIN	2.40	2.20	1.60

BERMUDA	2.40	2.20	1.60

BOLIVIA	2.40	2.20	1.60

BHUTAN	2.40	2.20	1.60

BOPHUTHATSWANA	1.80	1.60	1.20

BOSNIA AND HERZEGOVINA	2.40	2.20	1.60

BOTSWANA	1.80	1.60	1.20

BRAZIL	2.90	2.60	2.00

BRUNEI	2.40	2.20	1.60

BULGARIA	2.80	2.50	1.90

BURKINA FASO	2.80	2.50	1.90

BURMA	2.90	2.60	2.00

BURUNDI	1.80	1.60	1.20

CAMBODIA	2.90	2.60	2.00

CAMEROON	2.40	2.20	1.60

CANADA	2.20	2.00	1.50

CANARY ISLAND	2.40	2.20	1.60

CAPE VERDE	2.90	2.60	2.00

CAYMAN ISLAND	2.40	2.20	1.60

CENTRAL AFRICAN REPUBLIC   	2.40	2.20	1.60

CHAD	2.90	2.60	2.00

CHILE	2.90	2.60	2.00

CHINA	2.80	2.50	1.90

COLOMBIA	2.90	2.60	2.00

COMOROS	1.80	1.60	1.20

CONGO	2.80	2.50	1.90

CONGO D.R. (SHABA REGION)   	1.80	1.60	1.20

CONGO D.R. (THE REST)	2.40	2.20	1.60

COOK ISLAND	2.40	2.20	1.60

COSTA RICA	2.40	2.20	1.60

CYPRUS	2.40	2.20	1.60

COTE D'IVORE	2.80	2.50	1.90

CROATIA	2.40	2.20	1.60

KIRIBATI	2.40	2.20	1.60

CUBA	2.90	2.60	2.00

CZECH REPUBLIC	2.40	2.20	1.60

SLOVAKIA	2.40	2.20	1.60

DENMARK	2.20	2.00	1.50

DIEGO GARCIA	2.80	2.50	1.90

DJIBOUTI	1.80	1.60	1.20

DOMINICA	2.40	2.20	1.60

DOMINICAN REPUBLIC	2.40	2.20	1.60

ECUADOR	2.40	2.20	1.60

EAST TIMOR	2.40	2.20	1.60

EGYPT	1.80	1.60	1.20

EL SALVADOR	2.90	2.60	2.00

EQUATORIAL GUINEA	2.40	2.20	1.60

ERITREA	2.90	2.60	2.00

ESTONIA	2.80	2.50	1.90

ETHIOPIA	1.80	1.60	1.20

FAROE ISLANDS	2.20	2.00	1.50

FIJI	2.40	2.20	1.60

FINLAND	2.20	2.00	1.50

FRANCE	2.20	2.00	1.50

FRENCH GUINEA	2.40	2.20	1.60

FRENCH POLYNESIA	2.40	2.20	1.60

GABON	2.90	2.60	2.00

GAMBIA	2.40	2.20	1.60

GEORGIA	2.80	2.50	1.90

GERMANY	2.20	2.00	1.50

GHANA	2.40	2.20	1.60

GIBRALTAR	2.40	2.20	1.60

GREECE	2.80	2.50	1.90

GREENLAND	2.40	2.20	1.60

GRENADA	2.40	2.20	1.60

GUADELOUPE	2.90	2.60	2.00

GUAM	2.40	2.20	1.60

GUATEMALA 	2.40	2.20	1.60

GUINEA	2.80	2.50	1.90

GUINEA BISSAU	2.80	2.50	1.90

GUYANA	2.40	2.20	1.60

HAITI	2.90	2.60	2.00

HAWAII	2.40	2.20	1.60

HONDURAS	2.40	2.20	1.60

HONG KONG	2.40	2.20	1.60

HUNGARY	2.40	2.20	1.60

ICELAND	2.80	2.50	1.90

INDIA	2.20	2.00	1.50

INDONESIA	2.40	2.20	1.60

IRAN	2.40	2.20	1.60

IRAQ	2.40	2.20	1.60

IRELAND	2.40	2.20	1.60

ISRAEL	2.40	2.20	1.60

ITALY	2.20	2.00	1.50

JAMAICA	2.40	2.20	1.60

KOREA	2.40	2.20	1.60

JAPAN	2.20	2.00	1.50

JORDAN	2.80	2.50	1.90

KENYA	1.80	1.60	1.20

KUWAIT	2.40	2.20	1.60

LAOS	2.90	2.60	2.00

LATVIA	2.80	2.50	1.90

LEBANON	2.90	2.60	2.00

LESOTHO	1.80	1.60	1.20

LIBERIA	2.40	2.20	1.60

LIECHTENSTEIN	2.80	2.50	1.90

LITHUANIA	2.80	2.50	1.90

LYBIA	2.40	2.20	1.60

LUXEMBOURG	2.40	2.20	1.60

MACAO	2.80	2.50	1.90

MACEDONIA	2.40	2.20	1.60

MADAGASCAR	1.80	1.60	1.20

MADEIRA	2.40	2.20	1.60

MALAWI	1.80	1.60	1.20

MALAYSIA	2.40	2.20	1.60

MALDIVES 	2.40	2.20	1.60

MALI	2.80	2.50	1.90

MALTA	2.80	2.50	1.90

MARSHALL ISLANDS	2.90	2.60	2.00

MARTINIQUE	2.90	2.60	2.00

MAURITANIA	2.90	2.60	2.00

MAURITIUS	1.80	1.60	1.20

MELILA	2.40	2.20	1.60

MEXICO	2.90	2.60	2.00

MICRONESIA	2.40	2.20	1.60

MARINA	2.40	2.20	1.60

MOLDOVIA	2.80	2.50	1.90

MONACO	2.40	2.20	1.60

MONGOLIA	2.90	2.60	2.00

MONTSERRAT	2.40	2.20	1.60

MOROCCO	2.40	2.20	1.60

MOZAMBIQUE	1.80	1.60	1.20

MYANMAR	2.90	2.60	2.00

NAMIBIA	1.80	1.60	1.20

NAURU	2.40	2.20	1.60

NEPAL	2.40	2.20	1.60

NETHERLANDS	2.20	2.00	1.50

NETHERLANDS ANTILLES	2.40	2.20	1.60

NEW CALEDONIA	2.40	2.20	1.60

NEW ZEALAND	2.40	2.20	1.60

NICARAGUA	2.40	2.20	1.60

NIEU	2.40	2.20	1.60

NIGER	2.40	2.20	1.60

NIGERIA	2.40	2.20	1.60

NORWAY	2.20	2.00	1.50

OMAN	2.80	2.50	1.90

PAKISTAN	2.80	2.50	1.90

PALAU	2.90	2.60	2.00

PANAMA	2.40	2.20	1.60

PAPUA NEW GUINEA	2.40	2.20	1.60

PARAGUAY	2.80	2.50	1.90

PHILIPPINES	2.40	2.20	1.60

POLAND	2.40	2.20	1.60

PORTUGAL	2.40	2.20	1.60

PUERTO RICO	2.90	2.60	2.00

QATAR	2.40	2.20	1.60

REUNION	2.80	2.50	1.90

ROMANIA	2.80	2.50	1.90

RUSSIAN FEDERATION	2.80	2.50	1.90

RWANDA	1.80	1.60	1.20

SAN MARINO	2.40	2.20	1.60

SAUDI ARABIA	2.90	2.60	2.00

SENEGAL	2.80	2.50	1.90

SEYCHELLES	1.80	1.60	1.20

SIERRA LEONE	2.40	2.20	1.60

SINGAPORE	2.40	2.20	1.60

SLOVENIA	2.40	2.20	1.60

SOLOMON ISLANDS	2.40	2.20	1.60

SOMALIA	1.80	1.60	1.20

SOUTH AFRICA	1.80	1.60	1.20

SPAIN	2.80	2.50	1.90

SRI LANKA	2.40	2.20	1.60

ST. HELENA	2.40	2.20	1.60

ST. KITTS AND NEVIS	2.40	2.20	1.60

ST. LUCIA	2.40	2.20	1.60

ST. PIERRE AND MIQUELON	2.90	2.60	2.00

SAO TOME AND PRINCIPE	2.90	2.60	2.00

ST. VINCENT	2.40	2.20	1.60

SUDAN	2.40	2.20	1.60

SURINAME	2.90	2.60	2.00

SWAZILAND	1.80	1.60	1.20

SWEDEN	2.20	2.00	1.50

SWITZERLAND	2.80	2.50	1.90

SYRIA	2.40	2.20	1.60

TAIWAN	2.90	2.60	2.00

TANZANIA	1.80	1.60	1.20

THAILAND	2.90	2.60	2.00

TOGO	2.80	2.50	1.90

TOKELAU	2.90	2.60	2.00

TONGA	2.40	2.20	1.60

TORTOLA	2.40	2.20	1.60

TRINIDAD AND TOBAGO	2.40	2.20	1.60

TUNISIA	2.40	2.20	1.60

TURKEY	2.80	2.50	1.90

TURKS AND CAICOS ISLAND	2.40	2.20	1.60

TUVALU	2.40	2.20	1.60

UGANDA	1.80	1.60	1.20

UKRAINE	2.80	2.50	1.90

UNITED ARAB EMIRATES	2.90	2.60	2.00

UNITED KINGDOM	2.20	2.00	1.50

UNITED STATES OF AMERICA   	2.20	2.00	1.50

URUGUAY	2.90	2.60	2.00

VANUATU	2.40	2.20	1.60

VENDA	1.80	1.60	1.20

VENEZUELA	2.40	2.20	1.60

VIET NAM	2.90	2.60	2.00

VIRGIN ISLANDS	2.40	2.20	1.60

YUGOSLAVIA	2.40	2.20	1.60

WALLIS AND FUTUNA	2.90	2.60	2.00

WESTERN SAMOA	2.90	2.60	2.00

YEMEN ARAB REP.	2.90	2.60	2.00

YEMEN PEOPLE'S REP.	2.80	2.50	1.90

ZIMBABWE	1.80	1.60	1.20