| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION DEVELOPMENT BUREAU ITU-D STUDY GROUPS | Document 1/186(Rev.1) Document 2/205(Rev.3)-E 11 December 2001 Original: English |
| FOURTH MEETING OF STUDY GROUP 1: CARACAS (VENEZUELA), 3 - 7 SEPTEMBER 2001 FOURTH MEETING OF STUDY GROUP 2: CARACAS (VENEZUELA), 10 - 14 SEPTEMBER 2001 |  |

Questions:	All

STUDY GROUP 2

SOURCE:	CHAIRMEN OF THE JOINT GROUP ON WTDC-98 RESOLUTION 9

TITLE:	WTDC-98 RESOLUTION 9: REVIEW OF NATIONAL SPECTRUM MANAGEMENT AND USE OF THE SPECTRUM. STAGE 1: 29.7-960 MHZ

________

Resolution 9 WTDC-98

Wtdc-98 Resolution 9: 
Review of national spectrum management and use of the spectrum stage 1: 29.7-960 MHz

## Table of Contents

Table of contents		2

Disclaimer		4

Preface			4

1	Executive summary		5

2	Introduction		5

2.1	World Telecommunication Development Conference (WTDC-98) Resolution 9		5

2.2	ITU-D and ITU-R activities in preparing a response to WTDC-98 Resolution 9		5

2.2.1	Establishment of an ITU-D and ITU-R Joint Group		5

2.2.2	The work programme		5

2.2.3	The Questionnaire		6

2.2.4	The invitation to the Administrations of Member States and to Sector Members		6

3	General analysis of the replies to the Questionnaire		7

4	Analysis of Part I of the Questionnaire (specific questions on national radio
frequency spectrum management)		7

4.1	General		7

4.2	Work products produced and under development		8

5	Analysis of Part II of the Questionnaire (general questions on national spectrum management)		9

5.1	General		9

5.2	Legal		9

5.2.1	Questions grouped under this category		9

5.3	Financial		11

5.3.1	Questions grouped under this category		11

5.4	Spectrum planning		13

5.4.1	Questions grouped under this category		13

5.4.2	Question 3		13

5.4.3	Question 4		14

5.4.4	Question 9		15

5.4.5	Question 10		16

5.4.6	Question 11		17

5.5	Spectrum control (including monitoring and technical analyses of interference		18

5.5.1	Questions grouped under this category		18

5.5.2	Monitoring stations		20

5.5.3	Fixed stations		21

5.5.3.1	HF stations		21

5.5.3.2	V/UHF stations		22

5.5.3.3	SHF stations		22

5.5.4	Mobile stations		22

5.5.4.1	HF stations		23

5.5.4.2	V/UHF stations		23

5.5.4.3	SHF stations		24

5.5.5	General comment		24

5.6	Information technology		24

5.6.1	Questions grouped under this category		24

5.6.2	Provisional analysis		25

5.7	Organisational		25

5.7.1	Questions grouped under this category		25

5.7.2	Introduction 		25

5.7.3	Questions 15 and 16 (staff employed in spectrum management duties		26

5.7.4	Question 17 (structure of the national spectrum management organization		26

5.8	Use of ITU Handbooks		27

5.8.1	Questions grouped under this category		27

5.8.2	Response to question 18		28

6	Requests for assistance in spectrum management (identified in the Questionnaire)		28

6.1	Introduction		28

6.2	Results of the replies		29

7	Proposed further action		31

7.1	Within the Joint Group for WTDC-98 Resolution 9		31

7.2	Within the Telecommunication Development Sector		31

7.3	Within the Radiocommunication Sector		31

Annexes

Annex 1	An example extract of the trilingual version of the International Allocation
Table presented in the 1998 edition of the Radio Regulations (S5) that has
been developed to facilitate the examination of national allocation tables
presented in French, Spanish or English without the need to incur
translation costs 		32

Annex 2-1	Analysis of the Questionnaire Part II: Legal aspects		33

Annex 2-2	Analysis of the Questionnaire Part II: Financial aspects		35

Annex 2-3	Analysis of the Questionnaire Part II: Spectrum planning		38

- Examples of national frequency allocation tables

- Example of national spectrum management strategy

Annex 2-4	Analysis of the Questionnaire Part II: Spectrum control (including monitoring
and technical analyses of interference)		52

Annex 2-5	Analysis of the Questionnaire Part II: Information technology		60

Annex 2-6	Organisational aspects		62

Annex 3-1	Analysis of requests for assistance in spectrum management		67

Annex 3-1	Analysis of requests for assistance in spectrum management (continued)		74

Annex 4	Country data		76

Annex 5	Listing of individuals and organizations identified by the Administrations as Focal Points for providing spectrum management information		81

Annex 6	Cover Page of the ITU-D Sector Web Page that has been designed to promote improved information exchange between national spectrum managers on a range of spectrum management matters		100

Annex 7	Overview of radiocommunication services operating in the frequency range
	29.7 MHz to 960 MHz		101

Disclaimer

Opinions expressed in the report are those received from different sources and are not recommendations. The mention of specific companies or products does not imply any endorsement or recommendations by the ITU.

Preface from the Directors of BDT and BR

Communication is a tool crucial to the success of modern national economies. Radio is essential to the information society, as it is a uniquely versatile and cost-effective communications medium and national policies will endeavour to provide an environment in which radio can make the greatest possible contribution to the social, economic, public safety and security needs of the country. The prime objective of national spectrum management is, therefore, to enable a country to manage effectively its use of the finite resources of the radio frequency spectrum and satellite orbits (in particular the geostationary-satellite orbit), within the framework of  international treaty obligations.

It is with this objective in mind that the World Telecommunication Development Conference (WTDC-98) adopted Resolution 9, “Participation of countries, particularly Developing Countries, in frequency spectrum management”. This Resolution required the Directors of ITU-D and ITU-R to develop a report on current and foreseen national uses of the radio frequency spectrum. Further, it required the Directors of the Telecommunication Development and Radiocommunication Sectors to consider and implement effective means to encourage and facilitate the active participation of both developing and least developed countries in the preparation of this report.

In accordance with this Resolution, the BDT and BR have both provided a high level of encouragement and support to the ITU-D and ITU-R Study Groups in their co-operative effort to produce a report on the first stage of the study: “WTDC-98 Resolution 9: Review of national spectrum management and use of the spectrum. Stage 1: 29.7-960 MHz”. This report provides a detailed analysis of how spectrum management is undertaken in a wide variety of countries throughout the world with particular emphasis on the needs of the Developing and Least Developed Countries. It will be an extremely useful tool to help developing countries to identify the assistance required so that future work in the ITU can be focused on their specific needs. It will also provide a benchmark to measure how far these requirements and needs have been satisfied.

Both Directors are pleased to see that a revision of Resolution 9 will be submitted to WTDC-02 recommending that the work continues to review the use of spectrum between 960 MHz to 3 000 MHz. This will be a considerable to challenge to those participating in the work and both Directors look forward to providing continued assistance through the resources of the BDT and BR.

Abstract

This Report has been prepared as a joint ITU-D and ITU-R response to World Telecommunication Development Conference (WTDC-98) Resolution 9, that requires the development of a report dealing with the use of the radio spectrum, with a primary objective to assist Developing Countries in fulfilling their national spectrum management functions.  During their meetings in 1998 and 1999, the two sectors agreed to establish a Joint Group to progress the work and also agreed to distribute a Questionnaire on national spectrum management to Administrations of Member States and to Sector Members. The Joint Group has met three times and worked by electronic correspondence to process the responses to the Questionnaire that was in two parts, one part dealing with national frequency allocation tables and use of the frequency range 29.7 - 960 MHz, the other part dealing with general aspects of national spectrum management. One of the key objectives is to identify specific problems that Developing Countries might be experiencing in national spectrum management.

1.	Executive summary

This Report has been prepared as a joint ITU-D and ITU-R response to World Telecommunication Development Conference (WTDC-98) Resolution 9, that requires the development of a report dealing with the use of the radio spectrum, with a primary objective to assist Developing Countries in fulfilling their national spectrum management functions.  During their meetings in 1998 and 1999, the two sectors agreed to establish a Joint Group to progress the work and also agreed to distribute a Questionnaire on national spectrum management to Administrations of Member States and to Sector Members. The Joint Group has had three meetings and worked by electronic correspondence to process the responses to the Questionnaire that was in two parts, one part dealing with national frequency allocation tables and use of the frequency range 29.7 - 960 MHz, the other part dealing with general aspects of national spectrum management. One of the key objectives is to identify specific problems that Developing Countries might be experiencing in national spectrum management.

This is the first stage of the Report. It has been adopted by ITU-D Study Groups at their meetings in Caracas, Venezuela, September 2001 and by ITU-R Study Group 1 at its meeting in Geneva, Switzerland, November 2001.

### 2.	Introduction

2.1	World Telecommunication Development Conference (WTDC-98) Resolution 9

Resolution 9, adopted by the World Telecommunication Development Conference (WTDC-98), requires the Directors of ITU-D and ITU-R to develop a report on current and foreseen national uses of the radio frequency spectrum. This resolution also requires the Directors of the Telecommunication Development and Radiocommunication Sectors to consider and implement effective means to encourage and facilitate the active participation of both developing and least developed countries in the preparation of this report.

2.2	ITU-D and ITU-R activities in preparing a response to WTDC-98 Resolution 9

2.2.1	Establishment of an ITU-D and ITU-R Joint Group

In response to this resolution, ITU-R Study Group 1 agreed with the proposal from ITU-D to establish a joint ITU-R and ITU-D group, to be named the "Joint Group on Resolution 9", and defined a proposed programme of work that would lead towards the production of the required report. This proposed work programme was submitted for review and adopted by ITU-D Study Groups 1 and 2 during their meetings in Geneva in September 1999.

2.2.2	The work programme

As envisioned in the work program for the Joint Group on Resolution 9, the scope of the proposed work included:

- collecting selected information from all Member States and all Sector Members of the Radiocommunication and Telecommunication Development Sectors, through the use of a two-part questionnaire (BDT-CA/8 - BR-CA/71) distributed jointly by the Radiocommunication and Telecommunication Development Sectors;

- using the spectrum management expertise in the Joint Group on Resolution 9 to analyze the collected information; and

- produce a report for review by ITU-R Study Group 1 and ITU-D Study Groups 1 and 2. Member States and Sector Members were requested to complete the questionnaire by 31 January 2000.

Most of the detailed work was undertaken by electronic correspondence. The first review of the responses to this questionnaire was performed at a meeting of the Joint Group on Resolution 9 in Geneva on 6 and 7 March 2000. Progress reports were presented at the ITU-D Study Group meetings September 2000 and at the ITU-R Study Group 1 meeting October 2000. A second meeting was held 20-22 March 2001 and the third meeting 6-8 June 2001.

    - The Questionnaire

Part I of the Questionnaire dealt with “Specific Questions on National Spectrum Management Relating to Resolution 9”. Member States and Sector Members were invited to provide information on the use of the radio spectrum between 29.7 – 960.0 MHz. This frequency range was chosen by agreement at the preparatory meeting of the Joint Group and endorsed by ITU-D Study Groups 1 and 2 and ITU-R Study Group 1. As WTDC-98 Resolution 9 envisaged that the report would be completed in stages, further frequency ranges could be examined in the future.

Part II of the Questionnaire dealt with “General Questions on National Spectrum Management” and contains 18 questions on technical, policy, financial and legal aspects of spectrum management. This attachment is a revision of a general questionnaire that was distributed previously by the ITU-R (CA/401 dated 20 January 1993). The responses to the questions in Attachment 2 will be used both in the preparation of the report required in Resolution 9, and also in planning the future work program of ITU-R Study Group 1.

Administrative Circular CA/08 (BDT) - CA/71 (BR) was distributed in paper form to Administrations of Member States of the ITU, Members of the Development Sector and Members of the Radiocommunication Sector. However, this Circular was also available in electronic format from the ITU web site and consequently, all respondents were urged to submit the requested information in electronic form. In particular, an extract of Article S5 of the 1998 editon of the Radio Regulations (Allocation Table for the frequency bands from 29.7 to 960 MHz) was included in the Circular for use by those administrations without a national table of frequency allocations.

2.2.4	The invitation to the Administrations of Member States and to Sector Members

Administrations of Member States were invited:

- to respond to the questions on national spectrum allocations, in addition, to provide the technical parameters of systems in use in given frequency bands; and

- to describe, in detail, specific problems in national spectrum management currently faced by the responding Member State.

Sector Members were requested to provide the technical parameters of systems currently available for given frequency bands.

Both Member States and Sector Members were invited to forward their responses to this two-part questionnaire, before 31 January 2000 (later extended to 31 March 2001), to the Telecommunication Development Bureau.

### 3.	General analysis of the replies to the Questionnaire

A total of 75 responses to the Questionnaire was received. This total comprised 4 responses from Telecommunication Operators and 71 responses from Administrations. Only those responses from Administrations were used in the analysis of the general spectrum management questionnaire and the National Spectrum Allocation Tables. To enable a representative comparison, these 71 responses have been separated into the following categories :

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 5 | 9 | 14 | 20 |
| Americas | 2 | 10 | - | 12 | 17 |
| Asia and Pacific | 1 | 8 | 5 | 14 | 20 |
| Europe and CIS | 13 | 14 | - | 27 | 38 |
| Arab States | - | 4 | - | 4 | 5 |
| TOTAL | 16 | 41 | 14 | 71 | 100 |

Taking into account that there are 189 Members States of the ITU, the ratio of responses received for each  region can be established :

Africa	:	30%

Americas	:	34%

Asia and Pacific	:	37%

Europe and CIS	:	49%

Arab States	:	21%

4.	Analysis of Part I of the Questionnaire (Specific questions on national radio frequency spectrum management)

  - General

Radio frequency spectrum management has traditionally been practiced in a two-step process – first through a series of policy decisions, a spectrum allocation strategy is established that designate given frequency bands for use by specified radio services; and then through a series of licensing decisions which implement the established allocation strategy, designated operators are provided authorization to operate given transmitters on specified frequencies. Measures have also been developed over the years for determining the effectiveness of established spectrum allocation strategies; these measures include 1) the extent to which interference is avoided between licensed radio systems, 2) the efficiency achieved in the use of the spectrum, 3) the cost-effectiveness of the spectrum dependent services provided, and 4) the availability of radiocommunication equipment that operates in compliance with the established allocation strategy.

International spectrum allocation strategies have been established at a series of ITU Radiocommunication Conferences and are presented in the Radio Regulations, Article S.5 (1998), and established national spectrum allocation strategies of a number of administrations are also available in the form of various graphical charts, published tables, and Internet web pages. The Working Group established to implement Resolution 9 of Valletta recognized that spectrum managers in many developing countries have a critical need for improved access to this wealth of information from which their respective national strategies can be developed.

Project objectives

For the purpose of focusing the work to be performed, two project objectives were defined by the working group –

- to facilitate a continuing dialog among spectrum managers throughout the world that will permit the existing expertise to be applied to existing problems in a timely and effective manner; and

- to develop certain information technology tools holding promise for improving the mutual understanding of the respective national spectrum allocation strategies developed in disparate environments differing linguistically, culturally, technically, politically, and economically.

  - Work products produced and under development

Through the joint efforts of the ITU-R and D Bureaux, a CD-Rom has been assembled which contains all of the 75 responses to the questionnaire (CA/08 and CA/71); copies of this “Resolution 9” CD-Rom are available on request from the BDT Secretariat. This CD-Rom presents all of the responses in *.pdf format and all can be viewed using an Adobe Acrobat reader which can be downloaded at no cost over the Internet. In each of the responses presented, an individual is identified who has been designated as the respondent’s “point of contact” -- see Annex 5 for a compilation of all the points of contact identified.

A trilingual version of the International Allocation Table presented in the Radio Regulations, Article S5 (1998 edition), has been compiled for the frequency bands between 29.7 and 960 MHz -- see Annex 1.3 for some representative pages of this table. This table (in Word format) is intended to serve as a translation tool to permit the immediate recognition of the spectrum allocations presented in the responses regardless of the language used by the respondent (English, French, or Spanish). To more easily identify the differences in spectrum allocations between the three ITU Regions, the table has been compiled with a single set of non-overlapping frequency bands.

Some work has been initiated toward the development of a set of database tools that would permit improved searches of all the national allocation tables compiled during the preparation of this report. This database development is continuing.

During the preparation of this report, a website was established i) to permit timely review of the draft work-products produced and ii) to test the feasibility of using this medium for satisfying Project Objective (1) stated above – to promote effective information exchange between spectrum managers who are physically dispersed throughout the world. The address of this webpage is:

http://www.itu.int/ITU-D/study_groups/SGP_1998-2002/JGRES09/Res9_Index.html

## 5	Analysis of Part II of the Questionnaire (general questions on national spectrum management)

5.1	General

Part II of the Questionnaire was developed by ITU-D and ITU-R Study Group 1 and follows on from previous similar Questionnaires. The results of the previous Questionnaire were attached to the Circular Letter (BDT-CA/08 - BR-CA/71). The questions asked in the current Questionnaire concern the following key aspects of national spectrum management:

- Legal

- Financial

- Spectrum planning (technical issues)

- Spectrum control (including monitoring and technical analyses of interference)

- Information Technology for spectrum management

- Organisation of spectrum management

- Use of ITU products, especially handbooks

Each of the following sections are structured to give: a statement of the questions asked; a brief explanatory review to place the particular aspect in context; identification of any obvious misunderstanding of the questions and a tabular presentation of the analysis of the responses grouped into region and country category (developed, developing or least developed). For each of the categories, a tabular “country-by-country” presentation is given in the annexes.

5.2	Legal

    - Questions grouped under this category

Q1: Do you have a national law governing spectrum management?

- Last date this law was changed or modified?

- Are any actions planned to change this law?

Q2: Have you published regulations and procedures for national spectrum management (e.g. radio services, license requirements etc.)

Laws covering the spectrum resource are as important as those that govern land and water use. Some administrations have a single law covering general telecommunications including radiocommunication while others clearly distinguish between the two because they consider the operating environment and requirements for management are so different. Whichever option is chosen, the law (or parts of the law) covering radiocommunication should establish basic concepts, authorities, broad goals and objectives, and responsibilities. It should give recognition to the existence of the radio spectrum as a national resource and the need to govern it in the interest of all citizens. It should therefore establish the right of the national government to regulate radiocommunication use, including authorization of use and enforcement of spectrum management rules. Furthermore, the law covering radiocommunication should establish the right of citizens and the government to own and operate radiocommunication equipment. Final elements that should be covered are the requirements for public access to the spectrum management decision process and for government responsiveness to the public input. Therefore, the radiocommunication law should require that the spectrum management authority provide the public with information on its decisions, including a written explanation of the basis for its decision.  In addition, the law should provide a process for review of decisions in accordance with established criteria and procedure.

Statistical Results

The country-by-country analysis of the responses to Question 1 is given in Annex 2-1.

A summary analysis by region and development category is given below:

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 5=Y | 8=Y 1=N | 13=Y 1=N | 93%=Y 7%=N |
| Americas | 2=Y | 9=Y 1=N | - | 11=Y 1=N | 92%=Y 8%=N |
| Asia and Pacific | 1=Y | 8=Y 1=N | 2=Y 2=N | 11=Y 3=N | 79%=Y 21%=N |
| Europe and CIS | 13=Y | 13=Y | - | 26=Y | 100%=Y |
| Arab States | - | 3=Y 1=N | - | 3=Y 1=N | 75%=Y 25%=N |
| TOTAL | 16=Y | 38=Y 3=N | 10=Y 3=N | - | - |
| % of response | 100%=Y | 93%=Y 7%=N | 77%=Y 23%=N | - | - |

The country-by-country analysis of the responses to Question 2 is given in Annex 2-1.

A summary analysis by region and development category is given below:

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 4=Y 1=N | 4=Y 5=N | 8=Y 6=N | 57%=Y 43%=N |
| Americas | 2=Y | 8=Y 2=N | - | 10=Y 2=N | 83%=Y 17%=N |
| Asia and Pacific | 1=Y | 7=Y 2=N | 4=Y 0=N | 12=Y 2=N | 86%=Y 14%=N |
| Europe and CIS | 13=Y | 11=Y 2=N | - | 24=Y 2=N | 92%=Y 8%=N |
| Arab States | - | 3=Y 1=N | - | 3=Y 1=N | 75%=Y 25%=N |
| TOTAL | 16=Y | 33=Y 8=N | 8=Y 5=N | - | - |
| % of response | 100%=Y | 80%=Y 20%=N | 62%=Y 38%=N | - | - |

5.3	Financial

    - Questions grouped under this category

Q5. Do you have a need for any spectrum redeployment?

- If so, do you have a strategy for achieving this redeployment in respective frequency bands and for given radiocommunication services?

- Please define the established strategy and describe the nature of the consultation, if any, with users regarding the potential costs resulting from the planned redeployment.

Q6. What is the total cost of national spectrum management functions performed by your Government (expressed in Swiss francs)?

- What is the source of  the funding required to accomplish these spectrum management functions?

Q7. Do you have a method for establishing spectrum users' fees?

- If so, please give a brief description of the method used in establishing those fees.

71 countries responded to the questionnaire on WTDC-98 Resolution 9.

63 countries completed the second part of the questionnaire. The calculations which follow are based on that figure. A country-by-country listing is given in Annex 2-2.

1) Question 5: Do you have a need for any spectrum redeployment?

47 countries replied YES, representing 75% of the countries having replied to the second part of the questionnaire.

If so, do you have a strategy for achieving this redeployment in the respective frequency bands and for given radiocommunication services?

29 countries replied YES, representing 62% of countries with a need for spectrum redeployment and 46% of the countries having replied to the second part of the questionnaire.

Please define the established strategy used for achieving this redeployment in the respective frequency bands and for given radiocommunication services.

| Strategy | Number of replies | Usable replies | Percentage |
| --- | --- | --- | --- |
| Under study | 6 | 6 | 30% |
| Public consultation | 4 | 4 | 20% |
| Time-limited licences | 1 | 1 | 5% |
| Mandatory | 3 | 3 | 15% |
| Information | 5 | 5 | 25% |
| Other | 1 | 1 | 5% |
| Not specified | 9 | 0 | 0% |
| Total | 29 | 20 | 100% |

Describe the nature of the consultation, if any, with users regarding the potential costs resulting from the planned redeployment.

Only 9 countries replied to this question, representing 31% of the countries having a strategy for achieving this redeployment in the respective frequency bands and 14% of the countries having replied to the second part of the questionnaire.

| Costs borne by | Number of replies | Percentage |
| --- | --- | --- |
| State | 1 | 12% |
| New entrants | 2 | 22% |
| Old licensees | 2 | 22% |
| Funds | 2 | 22% |
| Negotiation | 2 | 22% |
| Total | 9 | 100% |

2) Question 6 : What is the total cost of national spectrum management functions?

Only 38 of the 63 countries, representing 60% of the countries having replied to the second part of the questionnaire, know the total cost of national spectrum management functions. The disparities between countries are such that there may be some doubt as to what costs are included (e.g. investment, operating costs, etc.). In the next questionnaire, the question should therefore be more clearly formulated by making a distinction between investments and operating, monitoring and management costs.

What is the source of the funding required to accomplish these spectrum management functions?

Even if countries do not know the total cost of the functions, only 14 countries, representing 22% of the countries having replied to the second part of the questionnaire do not know how spectrum management was funded.

| Financing | Number | Percentage |
| --- | --- | --- |
| State | 25 | 40% |
| Incumbent operator | 6 | 9% |
| Users' fees | 18 | 29% |
| Not specified | 14 | 22% |
| Total | 63 | 100% |

3) Question 7: Do you have a method for establishing spectrum users' fees?

50 of the 63 countries, representing 79% of the countries having replied to the second part of the questionnaire, replied that they had a method for establishing spectrum users' fees.

Description of the method

| Method | Number | Percentage |
| --- | --- | --- |
| Users' fees | 40 | 80% |
| Flat rate | 1 | 2% |
| Auction | 2 | 4% |
| Costs | 2 | 4% |
| Comparison | 2 | 4% |
| Telecommunication tariffs | 2 | 4% |
| Not specified | 1 | 2% |
| Total | 50 | 100% |

5.4	Spectrum planning

5.4.1	Questions grouped under this category:

Q3.	Do you have a national radio frequency spectrum allocation table?

Q4.	Do you have technical specifications for national spectrum use?

Q9.	Do you notify frequency assignments to the ITU?

Q10.	Do you have a policy and planning function for national spectrum management (i.e. a national strategy for future use of the spectrum)?

Q11.	Do you perform technical analyses of frequency assignment requests?

5.4.2	Q3.	Do you have a national radio frequency spectrum allocation table?

A national table of frequency allocations is a basic tool for an effective spectrum management process. It provides a general plan for spectrum use and the basic structure to ensure efficient use of the spectrum and the prevention of RF interference between services. Through use of the table, manufacturers will have a guide to where in the spectrum to design and build equipment, and users will know where to operate. As described in the National Spectrum Management Handbook, the International Table of Frequency Allocations, Article S5 of the Radio Regulations forms the basis for national tables and in some countries this may be used as the national table. However, other countries have included additional information on national use, varying in detail from showing which service operates when the Radio Regulations offer a choice, to spectrum available for government and non-government use, and, for specific sub-bands, channel arrangements and equipment specifications in use. Example extracts of national allocation tables and strategies are given in Annex 2.3.

Statistical Results

The country-by-country analysis of the responses to Question 3 is given in Annex 2-3.

A summary analysis by region and development category is given below:

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 4=Y 1=N | 4=Y 5=N | 8=Y 6=N | 57%=Y 43%=N |
| Americas | 2=Y | 8=Y 2=N | - | 9=Y 2=N | 83%=Y 17%=N |
| Asia and Pacific | 1=Y | 8=Y 1=N | 3=Y 1=N | 12=Y 2=N | 86%=Y 14%=N |
| Europe and CIS | 13=Y | 10=Y 3=N | - | 23=Y 3=N | 88%=Y 12%=N |
| Arab States |  | 2=Y 2=N |  | 2=Y 2=N | 50%=Y 50%=N |
| TOTAL | 16=Y | 32=Y 9=N | 7=Y 6=N | - | - |
| % of response | 100%=Y | 78%=Y 22%=N | 54%=Y 46%=N | - | - |

5.4.3 	Q4.	Do you have technical specifications for national spectrum use?

Technical specifications or standards are used to publish maximum or “limit” values for certain technical characteristics of equipment and the associated measurement methods. Article S3 of the Radio Regulations deals with the technical characteristics of stations and requires equipment to satisfy the provisions of the Regulations so that interference to other services is avoided. Transmitter frequency tolerance and spurious emissions are technical characteristics specifically mentioned in the Regulations and limit values are given for different services. Other technical characteristics are used for frequency planning purposes or sometimes for interoperability. The limit values may be agreed on a national, regional or an international basis, especially if there is a global circulation or safety-of-life requirement. Many countries prefer to adopt standards developed by global or regional standards bodies because a standardisation program is costly and the adoption of common standards removes barriers to trade.

Statistical Results

The country-by-country analysis of the responses to Question 4 is given in Annex 2-3.

A summary analysis by region and development category is given below:

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 4=Y 1=N | 7=Y 2=N | 11=Y 3=N | 79% 21% |
| Americas | 2=Y | 6=Y 4=N | - | 8=Y 4=N | 67% 33% |
| Asia and Pacific | 1=Y | 16=Y 3=N | 4=Y 0=N | 21=Y 3=N | 88% 12% |
| Europe and CIS | 12=Y 1=N | 12=Y 1=N | - | 24=Y 2=N | 92% 8% |
| Arab States | - | 2=Y 1=N | - | 2=Y 1=N | 67% 33% |
| TOTAL | 15=Y 1=N | 42=Y 10=N | 11=Y 2=N |  |  |
| % of response | 94%=Y 6%=N | 80%=Y 20%=N | 85%=Y 15%=N |  |  |

5.4.4	Q9.	Do you notify frequency assignments to the ITU?

The coordination, notification and registration of frequency assignments through the ITU-BR is a crucial activity to protect national radiocommunication systems from interference. For some services and frequency bands the procedures are specified and required as provisions of the Radio Regulations. For other systems which operate completely within national borders, administrations may decide that notification is unnecessary, or may establish simpler bilateral or multilateral arrangements with neighbouring countries.

Statistical Results

The country-by-country analysis of the responses to Question 9 is given in Annex 2-3.

A summary analysis by region and development category is given below:

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 3=Y 2=N | 5=Y 4=N | 8=N | 57% 42% |
| Americas | 2=Y | 4=Y 5=N | - | 6=Y 5=N | 58% 42% |
| Asia and Pacific | 1=Y | 7=Y 1=N | 4=Y 2=N | 12=Y 3=N. | 86% 14% |
| Europe and CIS | 13=Y | 13=Y | - | 26=Y | 100% |
| Arab States | - | 4=Y | - | 4=Y | 100% |
| TOTAL | 16=Y | 33=Y 7=N | 8=Y 5N |  |  |
| % of response | 100%=Y | 80%=Y 20%=N | 62%=Y 38%=N |  |  |

5.4.5	Q10.	Do you have a policy and planning function for national spectrum management (i.e. a national strategy for future use of the spectrum)?

The primary purpose of policy making and planning in national spectrum management is to ensure that the radio spectrum is allocated to meet, both in the short and longer term, the often competing requirements from different users and services. This must take account of international obligations and technical constraints as well as the national social, economic and political realities. Establishment of a national allocation table provides a basis for current and near-future needs, however, spectrum requirements and uses will change considerably over time, especially due to economic growth and advances in technology. A national strategy for future use of the spectrum is developed from the national table, incorporating timescales for foreseen changes, showing plans for phasing out old systems and introducing replacement technology. This enables these changes to be accommodated more easily, for different options to be considered and an opportunity for consultation with key players representing the radiocommunication industry, operators and users.. Example extracts of national spectrum strategies are given in Annex 2-3.

Statistical Results

The country-by-country analysis of the responses to Question 10 is given in Annex 2-3.

A summary analysis by region and development category is given below:

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 3=Y 2=N | 4=Y 4=N | 7=Y 6=N | 54%=Y 46%=N |
| Americas | 2=Y | 8=Y 2=N | - | 10=Y 2=N | 83%=Y 17%=N |
| Asia and Pacific | 1=Y | 8=Y 1=N | 3=Y 1=N | 12=Y 2=N | 86%=Y 14%=N |
| Europe and CIS | 10=Y 3=N | 13=Y 0=N | - | 23=Y 3=N | 88%=Y 12%=N |
| Arab States | - | 2=Y 2=N | - | 2=Y 2=Y | 50%=Y 50%=Y |
| TOTAL | 13=Y 3=N | 34=Y 7=N | 7=Y 5=N |  |  |
| % of response | 81%=Y 19%=N | 83%=Y 17%=N | 58%=Y 42%=N |  |  |

5.4.6	Q11.	Do you perform technical analyses of frequency assignment requests?

The purpose of technical analyses is to ensure that a frequency assignment to a new station will not cause unacceptable interference to existing services. This usually involves providing sufficient isolation, for example in frequency, distance or time occupancy, between the new and existing stations, taking into account the technical characteristics (for example radiated power, bandwidth and protection criteria) for the systems concerned. The analyses can range from a simple manual assessment to detailed computer modeling using terrain data for propagation calculations. Technical analysis requires an accurate knowledge (preferably a database) of existing users.

Statistical Results

The country-by-country analysis of the responses to Question 11 is given in Annex 2-3.

A summary analysis by region and development category is given below:

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 3=Y 2=N | 7=Y 2=N | 10=Y 4=N | 71%=Y 29%=N |
| Americas | 2=Y | 7=Y 2=N | - | 9=Y 2=N | 83%=Y 17%=N |
| Asia and Pacific | 1=Y | 7=Y 1=N | 2=Y 1=N | 10=Y 2=N | 86%=Y 14%=N |
| Europe and CIS | 11=Y 2=N | 13=Y | - | 24=Y 2=N | 88%=Y 12%=N |
| Arab States | - | 3=Y 1=N | - | 3=Y 1=N | 75%=Y 25%=N |
| TOTAL | 14=Y 2=N | 40=Y 6=N | 9=Y 3=N |  |  |
| % of response | 88%=Y 12%=N | 85%=Y 15%=N | 75%=Y 25%=N |  |  |

5.5	Spectrum control (including monitoring and technical analyses of interference)

5.5.1	Questions grouped under this category

Q12. Do you perform radio monitoring?

-number of fixed monitoring stations?

- facilities available at fixed monitoring stations:

-- monitoring up to ____ MHz

-- direction finding up to ____MHz

- number of mobile monitoring stations?

- facilities available at mobile monitoring stations:

-- monitoring up to ____ MHz

-- direction finding up to ____MHz

Q13. Do you perform technical analyses of radio frequency interference complaints?

Response to Question 12:

This question involves the available monitoring facilities both fixed and mobile and to which frequency range the stations are able to make observations.

It should be noted that the purpose of spectrum monitoring by means of monitoring stations is to assist in the overall process of spectrum management including the assignment and planning of frequencies. To that end, spectrum monitoring provides technical information from which conformity with the technical and operating standards established in the framework of spectrum management can be ascertained. For its part, the spectrum management programme supports the monitoring process by providing precise and comprehensive data on the assignments in force and by designating specific tasks to monitoring operations. Monitoring of emissions is an essential component in frequency spectrum management, and monitoring stations are an essential tool for effective spectrum management at the national level.

In the response to the questionnaire there is a very significant difference between the developed and non-developed countries.

The Developed Countries seem to have very well equipped monitoring facilities, both fixed and mobile and many  countries have remote-control facilities to provide country-wide coverage.

On the other hand the Developing Countries lack both fixed and mobile monitoring facilities and some countries, at present, do not have any facilities at all.

In particular, the Developing Countries are seeking the assistance of ITU to set up a plan to realise the necessary resources to build up their monitoring facilities and/or asking the ITU to help them with resources.

Regarding the frequency ranges of the fixed- and mobile monitoring equipment, it is indicated that most countries who do have facilities are able to monitor up to 1300 or 2700 MHz. Some of them are able to monitor up to 40 GHz and some indicated the availability of satellite monitoring facilities. Direction-finding equipment is also available mostly up to 1300 or 2700 MHz, although a number of countries have direction-finding equipment available only in the frequency range up to 30 MHz.

In this connection, it must be borne in mind that the usable spectrum has been extended to include increasingly higher frequency bands. This means that fixed monitoring stations must be distributed as widely as possible across the territory of the countries concerned and/or that a large number of mobile monitoring stations must be used. For various reasons, including economic, administrations may find it impossible to monitor all the bands and all the major parameters.  It is therefore recommended that they harmonize their plans and activities in order to meet the desired objectives.

5.5.2	Monitoring Stations

Seventy-five responses were received, including four from telecommunication operators (see Annex 2-4). The 71 responses from administrations can be categorised as follows:

| Region | Developed countries (D'd Cs) | Developing countries (D'ing Cs) | LDCs | Total | % of responses |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 5 | 9 | 14 | 20 |
| Americas | 2 | 10 | - | 12 | 17 |
| Asia-Pacific | 1 | 8 | 5 | 14 | 20 |
| Europe and CIS | 13 | 14 | - | 27 | 38 |
| Arab States | - | 4 | - | 4 | 5 |
| Totals | 16 | 41 | 14 | 71 | 100 |

On the basis of the 189 ITU Member States, the percentage of responses received by region has been calculated as follows:

Africa:			30%

Americas:		34%

Asia-Pacific:		37%

Europe and CIS:	49%

Arab States:		21%

It can be noted that, of the 71 responses from ITU Member States, certain Member States stated that they did not carry out monitoring of emissions or were in the process of acquiring monitoring stations:

| Region | No monitoring | Monitoring planned |
| --- | --- | --- |
| Africa | 6 (LDCs) | 1 |
| Asia-Pacific | 2 (LDCs) | 1 |
| Europe | 2 (D'd Cs) | - |
| Arab States | 1 (D'd Cs) | 1 |

As the three Member States in question submitted planning data, that data was taken into account. The following statistical analyses therefore concern 60 countries:

| Region | Developed countries (D'd Cs) | Developing countries (D'ing Cs) | LDCs | Total | % of responses |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 4 | 4 | 8 | 15 |
| Americas | 2 | 10 | - | 12 | 20 |
| Asia-Pacific | 1 | 8 | 3 | 12 | 20 |
| Europe and CIS | 12 | 13 | - | 25 | 41 |
| Arab States | - | 3 | - | 3 | 4 |
| Totals | 15 | 38 | 8 | 60 | 100 |

5.5.3	Fixed stations

A detailed analysis of the fixed stations declared by the 60 administrations in question is given in Annex 2-4:

- Chart 1 (fixed measurement stations)

- Chart 2 (fixed radio direction-finding stations)

- Chart 3 (distribution of fixed stations by frequency band)

The analysis produces the following table:

| Region | Measurement stations |  | Direction-finding and measurement stations (MS) |  |  | H/V/UHF stations |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty | % of total | Qty | % of total | % of MS | Meas. | DF | MS + DF |
| Africa | 7 | 1 | 6 | 2 | 86 | - | 1 | - |
| Americas | 195 | 34 | 147 | 41 | 75 | 75 | 33 | 33 |
| Asia-Pacific | 84 | 15 | 64 | 13 | 76 | - | 1 | - |
| Europe and CIS | 273 | 48 | 135 | 37 | 49 | 2 | 2 | - |
| Arab States | 9 | 2 | 7 | 2 | 77 | 7 | 3 | - |
| Total | 568 | 100 | 359 | 100 | 63 | 84 | 40 | 33 |

In total, 568 fixed measurement stations and 359 fixed radio direction-finding stations were declared.  Of the 568 fixed measurement stations, 359 stations also perform the radio direction-finding function, i.e. 63 per cent, in frequency bands limited to 3 GHz. This means that fixed monitoring (or by means of remote controlled stations) must be located in those areas where the higher frequencies are heavily used. For the other areas mobile monitoring stations can be used.

It should be noted that 15 per cent of the total number of fixed stations perform monitoring of both the HF and V/UHF bands.  The distinction between HF and V/UHF bands is altogether apparent and reflects the different types of operation in each of them.

Analysis of the data by national administration shows that eight administrations (12 per cent) have 370 fixed stations, i.e. 65 per cent of the overall total of fixed stations.

| Region | Stations | % of total | Administrations |
| --- | --- | --- | --- |
| Americas | 131 | 23 | 1 (D'd Cs) + 2 (D'ing Cs) |
| Asia-Pacific | 40 | 7 | 1 (D'd Cs) + 1 (D'ing Cs) |
| Europe and CIS | 199 | 35 | 4 (D'd Cs) |

The above table shows the considerable infrastructures established by the developing countries, which are a function both of the size of their national territories and of their economic development and therefore of the development of radiocommunication services.

5.5.3.1		HF stations

There are eighty-four measurement stations in the band 9 kHz – 20 MHz, of which 66 per cent fall between 9 kHz and 100 kHz. In respect of the radio direction-finding capacity, only 40 per cent of stations can offer this function between 300 kHz and 20 MHz and 12 per cent between 9 kHz and 100 kHz.

5.5.3.2		V/UHF stations

Based on Annex 2-4 Chart 3, these stations can be grouped into three significant categories according to frequency band.

| Frequency band | Fixed measurement stations/Regions |  |  |  |  | Total | Measurement stations with DF |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Africa | Americas | Asia- Pacific | Europe and CIS | Arab States |  |  |
| 20 MHz – 1 GHz | 7 (1%) | 195 (34%) | 24 (15%) | 273 (48%) | 9 (2%) | 568 (100%) | 80% |
| 1 GHz – 2 GHz | 3 (<1%) | 157 (35%) | 26 (6%) | 260 (58%) | 4 (<1%) | 450 (100%) | 50% |
| 2 GHz – 3 GHz | 1 (<1%) | 89 (28%) | 26 (8%) | 196 (62%) | 4 (1%) | 316 (100%) | 36% |

It can be observed that the number of stations offering the radio direction-finding function diminishes as one moves up in frequency.

5.5.3.3		SHF stations

Above 3 GHz, only five countries (8%) [1 (Europe), 3 (Americas) and 1 (Asia), including two developing countries], have installed 33 different types of station accounting for six per cent of all declared stations. These stations provide only the measurement function, without radio direction-finding.

5.5.4	Mobile stations

The main function of mobile monitoring stations is to carry out all monitoring of emission characteristics that cannot be undertaken (easily) by fixed centres, either on account of the magnitude of the measurement in question, or of difficulties resulting from spectrum congestion or to locate the exact position of a transmission. This is particularly true for all monitoring carried out on frequencies above 30 MHz, where the low transmitter power, higher antenna directivity and, in particular, propagation characteristics make effective measurements from fixed positions impossible.

A detailed analysis of the number of mobile stations declared by the 60 administrations is given in Annex 2-4:

- Chart 4 (mobile measurement stations)

- Chart 5 (mobile radio direction-finding stations) and

- Chart 6 (distribution of mobile stations by frequency band)

The breakdown of the data is as follows:

| Region | Measurement stations (MS) |  | Measurement + DF stations |  |  | H/V/UHF stations |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty | % | Qty | Total % | % MS | MS | DF | MS + DF |
| Africa | 4 | < 1 | 3 | 1 | 75 | - | - | - |
| Americas | 183 | 31 | 175 | 29 | 96 | 114 | 92 | 92 |
| Asia-Pacific | 151 | 25 | 151 | 26 | 100 | - | - | - |
| Europe and CIS | 257 | 43 | 253 | 43 | 98 | 3 | 3 | 3 |
| Arab States | 5 | < 1 | 4 | 1 | 80 | - | - | - |
| Total | 600 | 100 | 586 | 100 | 98 | 117 | 95 | 95 |

The above table demonstrates the importance of mobile stations in the management of frequency monitoring, with 95 per cent of mobile stations offering both measurement and radio direction-finding functions.

Analysis of the data shows that nine administrations (15 per cent) have 366 stations, accounting for 61 per cent of the overall total for mobile stations.

| Region | Stations | % of total | Administrations |
| --- | --- | --- | --- |
| Americas | 118 | 20 | 2 (D'd Cs) |
| Asia-Pacific | 80 | 13 | 1 (D'ing Cs) |
| Europe and CIS | 168 | 20 | 6 (D'd Cs) |

5.5.4.1			HF stations

These total 114 measurement stations, of which 95 (83%) with radio direction-finding.

5.5.4.2			V/UHF stations

As is the case for fixed stations (see § 5.1.2), we can distinguish three groups of stations according to frequency band.

| Frequency band | Mobile measurement stations / Regions |  |  |  |  | Total | MS with DF |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Africa | Americas | Asia- Pacific | Europe and CIS | Arab States |  |  |
| 20 MHz – 1 GHz | 4 (<1%) | 183 (31%) | 151 (25%) | 257 (43%) | 5 (<1%) | 600 (100%) | 98% |
| 1 GHz – 2 GHz | 1 (<1%) | 113 (25%) | 118 (26%) | 215 (48%) | 3 (<1%) | 450 (100%) | 97% |
| 2 GHz – 3 GHz | 1 (<1%) | 63 (19%) | 107 (32%) | 165 (48%) | 3 (<1%) | 339 (100%) | 95% |

5.5.4.3			SHF stations

Seventy-four measurement stations are operational up to 105 GHz, accounting for 12 per cent of all mobile stations. Radio direction-finding is available only in 14 stations (2 per cent of all mobile stations), this low percentage being explained by the high cost of equipment for satellite radio direction-finding.

5.5.5		General comment

The answers show:

a)	A greater use of mobile stations with regard to fixed stations: + 9.5 per cent

Monitoring stations have come to acquire considerable importance, particularly for measurements of coverage for broadcasting services and mobile radiocommunication networks, for example cellular networks. There are 600 mobile stations in these categories, 95 per cent of which offer both measurement and radio direction-finding functions, as against 568 fixed stations.

Furthermore, an analysis by frequency band gives the following results:

- HF:	MobileFixed= 1,36

- V/UHF:	MobileFixed= 1,09

- SHF:	MobileFixed= 2,24

b)	Frequency band: most of the monitoring relates to following frequency ranges

100 kHz – 20 MHz (MS + DF)

20 MHz – 1 GHz (Mobile)

20 MHz – 1.3/2 GHz (Fixed)

c) 	Relative availability of the radio direction-finding function in mobile and fixed stations according to frequency band:

- HF:	MobileFixed= 1,04

- V/UHF	MobileFixed= 1,32

- SHF:	Mobile = 14 ; Fixed = 0

### 5.6	Information technology

5.6.1	Questions grouped under this category

Q8. Do you maintain centralized databases for spectrum management?

- What is the approximate size of your database (expressed in number of records)?

- Do you have a computerized data base management system (DBMS)?

- What DBMS system do you use?

- Are these frequency assignment records available to the public?

Q14. What computers and operating systems are in use for national spectrum management?

5.6.2	Provisional analysis

A country-by-country analysis is given in Annex 2-5.

The summary of the responses to questions 8 and 14 and contained in Annex 2-5 is incomplete and the detailed responses need to be treated with some caution as they are based on a “snapshot” of administrations’ operations in 1999. In addition, the rapid pace of change in Information Technology (IT) makes it difficult to assess easily the responses, especially in relation to some requests for assistance and the size of the database. A particular problem is that information related to IT development has not always been recorded under questions 8 and 14 and therefore could not be included in the summary due to the guidelines used for developing the report. Hence, while the responses contain some valuable information, particularly for the development of handbooks, they require a much more detailed analysis in order to extract this information, to target specific forms of assistance to administrations and to identify limitations in administrations’ hardware and software capabilities.

While some administrations have advanced IT systems, other administrations, mainly the Developing Countries, appear to have no IT facilities and this indicates a large variation in capability. Considering that IT facilities provided by the ITU are continually being updated, this variation in capabilities has implications for the provision of assistance to administrations and setting the minimum IT standards needed to operate ITU software. IT systems should continue to be monitored (for example in further questionnaires) in order to identify relative changes in system capability and how to meet such changes.

  - Organisational

5.7.1	Questions grouped under this category

Q15: Number of technical/professional staff in national spectrum management?

Q16: Number of support staff in national spectrum management?

Q17: Describe your country's spectrum management structure

5.7.2 Introduction

The prime objective of national spectrum management is to enable a country to manage effectively its use of the finite resources of the radio-frequency spectrum and satellite orbits, within the framework of ITU treaty obligations. There is usually an identifiable authority in each country having the necessary legal powers and resources to carry out spectrum management functions. The organizational structures differ from country to country according to particular requirements and resources. The general model for spectrum management organizations is a central authority responsible for national coordination of all frequency use and international representation.

5.7.3 Questions 15 and 16 (Staff employed in spectrum management duties):

Of the administrations that replied to the questionnaire, 57 provided information about the numbers of technical/professional and support staff in their national spectrum management. The numbers provided are summarized in Annex 2.6 Table 1.

The average number of technical/professional staff is 99 (ranging from 1 to 1050) and for support staff 86 (ranging from 1 to 1181). For calculation purposes, when only one figure was provided for the sum of technical/professional and support staff, it was divided by 2. These were not considered in the range.

In the analysis of the replies, since the numbers alone may not be representative of the effort put into spectrum management by each administration, an attempt was made to establish relationships between the numbers informed and some other important factors that may influence the necessary staff to carry out spectrum management activities.

Some relationships that appear to be interesting for analysis are between the number of technical and support staff and: the population of the countries, the size of the spectrum management database and the stage of development of the countries. These relationships are shown in Annex 2.6 Table 2.

It should be noted that, in some of the replies, the number of staff involved in spectrum management was considered as the total number of employees of the institution in charge of spectrum management, which may be an independent agency, a branch of a larger Ministry or even a private company. In some other replies, only the personnel directly involved were reported. These different interpretations on the questions may account for part of the large range of numbers found in the replies.

Conclusions:

Even though the numbers of staff presented in the replies represent good examples of how different administrations are dealing with spectrum management, they alone should not be used as a basis for comparison, since the number of people involved in spectrum management duties is largely dependent not only on the population and stage of development of the countries but also on various other factors like the degree of automation used, the number of frequency assignments in each particular country and the type of institution in charge of  spectrum management.

Although there is a distinct relationship between the number of staff,  the population of the countries and the size of the spectrum management database, for the same reasons mentioned above, it is not recommendable that the decision on the number of staff in spectrum management be based on the on population of the country or size of the database alone.

5.7.4 Question 17 (structure of the national spectrum management organization)

Of the administrations that replied to the questionnaire, 33 supplied a copy of the organizational chart of their spectrum management structure. Others either provided descriptions of the structure or indicated a web site where the chart could be found. The administrations that supplied their organizational charts, descriptions and web sites are indicated in Annex 2-6 Table 1.

Some administrations supplied the organizational charts of the whole institution or regulatory body, while others focused only on the branch that deals specifically with spectrum management.

The charts are a good source of information on the approach each country adopts in dealing with spectrum management, specially on the degree of independence the branch or regulatory body has from the central government.

Even though there is a worldwide trend towards the establishment of independent regulatory agencies or commissions, in many countries the regulatory functions for spectrum management are still performed by an office or department within a larger executive entity or ministry.

Administrations wishing to establish new regulatory bodies or review existing ones may find very useful information in these organizational charts.

5.8	Use of ITU Handbooks

5.8.1	Questions in this category

Q18. Do you use the ITU-R Handbooks and Reports on:

- National Spectrum Management, version 1995?

- Spectrum Monitoring, version 1995?

- Computer-aided Techniques for Spectrum Management, version 1999?

- HF Broadcasting System Design, version 1999?

- Report SM.2012, Economic Aspects of Spectrum Management, version 1997?

- Windows Basic Automated Spectrum Management System (WinBASMS) Software Version 1997, Manual Version 1997

What additional information/handbooks do you need from the ITU?

A country-by-country analysis given in Annex 2-7.

To assist the administrations, responsible for the proper and efficient use of the spectrum, a number of Handbooks, Reports and other tools are developed within ITU-R. For this reason it is important to know whether these Handbooks etc. are used by and meet the needs of the administrations.

Also it is important to know what additional information or handbooks are missing, in order to give guidance to ITU-R in the development of additional Handbooks or Reports.

It should be noted that there are available Handbooks and Reports developed within the ITU-D but these were not dealt with in this Questionnaire.

5.8.2	Response to Question 18:

Analysis of the responses show that the Handbooks and Reports developed within ITU-R are well known, but not in use in all countries. The reason for not using them is not always given. It is also important to note that a number of responding countries do not have these publications and requested that copies be sent to them. Some administrations remarked that they lacked the  professionally qualified staff able to use and apply the information in the Handbooks, WinBASMS and Reports.

Of all the published ITU-R Handbooks, the Handbook on National Spectrum Management, version 1995 and the Spectrum Monitoring Handbook, version 1995 are used in most of the responding countries. The other Handbooks/Reports are significantly less used as the Handbooks indicated above.

Most of the responding Developed Countries indicate that they are using the Handbook on Computer aided Techniques for Spectrum Management, version 1999 and the Report SM.1012, Economic Aspects of Spectrum Management, version 1997.

A number of Developing Countries not yet using the WinBASMS software indicated that they need this software. Administrations who have used WinBASMS request a new version.

The responses to the question for additional information/Handbooks were various:

- There is a need for regularly updated versions of the National Spectrum Management Handbook and the Spectrum Monitoring Handbook.

- A number of administrations indicated that they would like to use the Handbooks etc. in combination with training for their personnel. In this respect, it was also suggested that there is a need for more ITU seminars on Spectrum Management.

- A significant number of Developing Countries indicated that, although they did not have the Handbooks and Reports, they knew the ITU had published them. The countries concerned requested that these publications be sent to them, together with all available ITU-R Recommendations on Spectrum Management, Radio Regulations on CD-ROM etc.

- Requests were received for BR and BDT to co-operate in the preparation of  handbooks or reports on satellite regulations and regulations and procedures for national spectrum management (e.g. radio services, license requirements etc. including spectrum fees). However it should be noted that Report SM.2012, Economic Aspects of Spectrum Management, version 1997 has a section dealing with fees.

6.	Requests for assistance in spectrum management (identified in the Questionnaire)

  - Introduction

Eleven of the questions in the Questionnaire invited administrations to identify problems and indicate if any assistance from the ITU was required in solving them.  It was envisaged that the responses to this invitation  would identify areas where ITU could initiate studies to produce recommendations, reports or handbooks targeted to meet specific needs, ideally with the developing countries participating in the work. However, it is clear from some of the responses that a more direct type of assistance is required in some cases, for example financial assistance and consultancy services.

Contributions were received from two members of the Joint Group, the representatives from Cameroon and Côte d’Ivoire, that identified a comprehensive list of items for ITU assistance (see Annex 3-1).

6.2	Results of the replies

The country-by-country analysis of the requests for assistance is given in Annex 3-2. A summary analysis is given below. None of the developed countries requested assistance, therefore Table 6-1 gives the number of developing and least developed countries requesting assistance expressed as a percentage of the total number of responses (55) received from these countries.

Table 6-1

| # Question | % of Developing and Least Developed countries requesting Assistance | # Example Comments |
| --- | --- | --- |
| Q1: Do you have a national law governing spectrum management? | 25% | “Current legislation is not sufficient to manage and protect the spectrum. Additional legislation is required. ITU assistance may be required on legal issues, regulation of spectrum”. |
| Q2: Have you published regulations and procedures for national spectrum management (e.g. radio services, license requirements etc.) | 25% | “ITU assistance is required for the development of procedures and regulations” |
| Q3: Do you have a national radio frequency spectrum allocation table? | 24% | “With the introduction of a new telecommunication authority in {country name supplied}, ITU help is needed to help frame our system.” |
| Q4: Do you have technical specifications for national spectrum use? | 25% | “We have developed several technical specifications only for a few services, but not in detail. We do not have the staff or the expertise to develop radio standards. We need assistance from the ITU to develop more technical specifications for national spectrum management use for various radiocommunication services.” |
| Q8: Do you maintain centralized databases for spectrum management? | 9% | “Require a more user-friendly database management program – e.g. updated version of WinBASMS” |
| Q9: Do you notify frequency assignments to the ITU? | 38% | “For the electronic notification, more assistance, example files and software support as well as a detailed description of the IFIC database would be required.” |
| Q10: Do you have a policy and planning function for national spectrum management (i.e. a national strategy for future use of the spectrum)? | 29% | “We have recently started this function but lack the expertise (inexperienced staff). We definitely need quite a lot of assistance for this important function.” |
| Q11: Do you perform technical analyses of frequency assignment requests? | 33% | “Inadequate analysis is performed, only harmonics and sub-harmonics are performed. We need software to perform proper interference analysis.” |
| Q12: Do you perform radio monitoring? | 24% | “Unable to carry out any monitoring. Equipment out of order. Automated spectrum management and monitoring system currently being developed. Assistance to be requested. ” |
| Q13: Do you perform technical analyses of radio frequency interference complaints? | 24% | “Regulations and guidelines required to set up the process, e.g. through a consultant.” |
| Q14: What computers and operating systems are in use for national spectrum management? | 20% | “For computerising the national spectrum management, we need some consultation and organisation of a meeting with ITU experts to get the necessary guidance, and visits to see computer systems already operational at ITU and in developed countries.” |

General study of the Resolution 9 questionnaire reveals that officials in charge of spectrum management in many developing countries report a lack of operational resources; in addition, it is noteworthy that a large number of these countries say that they do not have any technical facilities. A large majority of developing countries have expressed the need for assistance in establishing an operational modern spectrum management system capable of meeting user expectations and serving the public good.

After analysing all the replies to the questionnaire (Parts I and II), we can identify some guidelines for the different types of technical assistance which the developing countries expect from ITU:

1)	Methodologies to put in place a national Table of Frequency Allocations, supplementing the provisions laid down in the Radio Regulations and taking account of work already accomplished locally.

2)	Determination of operational structures for the smooth running of spectrum-related activities: administration, frequency allocation, analysis and monitoring. Furthermore, according to specific national factors, provision of expertise for identifying human resources, technical means and operational procedures for efficient management of the spectrum.

3)	Economic and financial aspects:

a)	identification of the financial resources required for the operational and investment budgets;

b)	according to specific national features, implementation of a charging policy for frequencies (fees) to support satisfactory national economic development.

4)	Dissemination of information

It has been observed, especially with regard to handbooks, that the developing countries do not possess or know about the different ITU publications. This could be attributable to a lack of awareness of the reference material available within ITU on the part of operational spectrum management staff. The financial aspect may also play a role in the acquisition of documentation in general. A mechanism could be put in place by ITU through the regional offices in order to inform developing countries in real time of the status of existing or scheduled publications emanating from the relevant ITU study groups, in order to avoid an “information gap”.

5)	Expertise

The developing countries call for:

- strengthening of ITU assistance to administrations through the training of national experts at headquarters and short missions by BR staff to national units that so request;

- more training in the form of very specific and targeted seminars.

. In addition, the regional Centres of Excellence being established might be an effective vehicle for training in spectrum management.

7.	Proposed further action

Proposals for further action fall into three main areas of responsibility:

- within the Joint Group for WTDC-98 Resolution 9

- within the Telecommunication Development Sector

- within the Radiocommunication Sector

7.1	Within the Joint Group for WTDC-98 Resolution 9

With the completion of this current stage of the report and the production of the Tri-lingual Frequency Allocation Table, the Joint Group will have finished Stage 1 of the requirements specified in WTDC-98 Resolution 9. The Joint Group has prepared a draft revision of Resolution 9 and an Annex describing the assistance required by African developing countries which will be considered by WTDC-02. If this adopted, then the Joint Group can continue with Stage 2 of the Report.

7.2	Within the Telecommunication Development Sector

For the Telecommunication Development Sector, proposals for further action ideally will have to fit into the functional framework of the Telecommunication Development Sector: Action within the study groups, the programmes – in particular Programmes 1 and 2 and the Special Programme for LDCs – and within the projects and direct assistance when financing is provided by UNDP, fund-in-trust or other sources. An overview of this framework is given in Annex 8.

7.3	Within the Radiocommunication Sector

Most of the actions to be taken in the Radiocommunication Sector will be the result of further analysis of the requests for assistance to identify how best to develop technical material that will help to solve at least some of the problem areas described in the responses. Some of this work will fall within the scope of ITU-R Study Group 1. Many other issues and actions might be identified by WTDC-02, depending on the contributions received.

Annex 1

An example of a trilingual version of the International Allocation Table presented in the Radio Regulations that is under development and intended to facilitate comparison between frequency use in the Regions
and between national allocation tables

TRILINGUAL INTERNATIONAL SPECTRUM ALLOCATION TABLE

For the frequency range from 27.5 to 960 MHz

as presented in the Radio Regulations Article S5 (1998 edition)

and as presented in Joint ITU-R/D Circular Letter CA/08-CA/71 dated November 1999

| Frequency Range in MHz | ITU Region 1 Europe and Africa | ITU Region 2 The Americas | ITU Region 3 Asia-Pacific |
| --- | --- | --- | --- |

| 27.5-28 | METEOROLOGICAL AIDS AUXILIAIRES DE LA MÉTÉOROLOGIE AYUDAS A LA METEOROLOGÍA FIXED FIXE FIJO MOBILE MOBILE MÓVIL | METEOROLOGICAL AIDS AUXILIAIRES DE LA MÉTÉOROLOGIE AYUDAS A LA METEOROLOGÍA FIXED FIXE FIJO MOBILE MOBILE MÓVIL | METEOROLOGICAL AIDS AUXILIAIRES DE LA MÉTÉOROLOGIE AYUDAS A LA METEOROLOGÍA FIXED FIXE FIJO MOBILE MOBILE MÓVIL |
| --- | --- | --- | --- |

| 41.015-44 | FIXED FIXE FIJO MOBILE MOBILE MÓVIL S5.160 S5.161 | FIXED FIXE FIJO MOBILE MOBILE MÓVIL S5.160 S5.161 | FIXED FIXE FIJO MOBILE MOBILE MÓVIL S5.160 S5.161 |
| --- | --- | --- | --- |

| ## Legal Aspects | Q1: Do you have a national law governing spectrum management? Q2: Have you published regulations and procedures for national spectrum management (e.g. radio services, license requirements etc.) |  |  |
| --- | --- | --- | --- |
| ## Country | Q1 | Q2 | # Comment |
| Algeria | Y | Y |  |
| Armenia | Y (1998) | Y | NP |
| Austria | Y (1997) | Y | NP |
| Bangladesh | Y (1933) | N | Project |
| Belarus | Y | Y |  |
| Belgium | Y | NA |  |
| Belize | Y (1987) | Y | Yes |
| Bolivia | N | Y |  |
| Brazil | Y (1997) | Y | NP |
| Bulgaria | Y (1998) | Y |  |
| Burkina Faso | Y (1998) | N | Y |
| Burundi | Y (1997) | Y | NP |
| Cameroon | Y (1998) | N |  |
| Canada | Y (1996, 1997) | Y | NP |
| Chad | Y | N | Y |
| China | Y (1997) | Y | Y |
| Colombia | Y (1990) | Y | NP |
| Côte d’Ivoire | Y (1995) | Y | N |
| Croatia | Y | N | NP |
| Cyprus | Y (1994) | Y | Y? |
| Czech Republic | Y (1992) | Y |  |
| Denmark | Y (1999) | Y | NP |
| Democratic People's Republic of Korea | Y (1998) | Y | NP |
| Egypt | N | N |  |
| Eritrea | Y (1998) | N | Y |
| Estonia | Y (1991) | Y | N |
| Ethiopia | N | N |  |
| Finland | Y (1995) | Y | N |
| France | Y (1996) | Y | N |
| Gabon | Y | Y | N |
| Georgia | Y (1999) | N | Y |
| Germany | Y (1995) | Y | N |
| Greece | Y (1994) | Y | N |
| Guyana | Y (1973) | N | Y |
| Hungary | Y (1999 | Y |  |
| Indonesia | Y (1999) | Y |  |
| Iran | Y (1984) | Y | Y |
| Japan | Y (1999) | Y | N |
| Republic of Korea | Y (2000) | Y | N |
| Lao | N | Y | N |
| Latvia | Y (1993) | Y | N |
| Lesotho | Y (1979) | Y |  |
| Liberia | Y (1978) | N |  |
| Libya | Y (1990) | Y | N |
| Lithuania | Y (1998) | Y | N |
| Madagasgar | Y (1999) | Y |  |
| Maldives | Y | Y |  |
| Mali | Y (1961) | Y | Y |
| Malta | Y (1998) | Y | N |
| Mexico | Y 1995 | Y | N |
| Myanmar | N | Y | Y |
| Namibia | Y (1990) | Y | Y |
| Nicaragua | Y | Y | Y |
| Norway | Y (1998) | Y | N |
| Pakistan | Y (1996) | N | Y |
| Peru | Y | Y | N |
| Portugal | Y (1997) | Y | N |
| Samoa | Y (1988) | Y | Y |
| Seychelles | Y (1998) | Y | N |
| Slovakia |  |  | No Part 2 |
| Spain | Y (1998) | Y | No |
| Suriname | Y (1994) | N | N |
| Sweden | Y (1999) | Y | N |
| Switzerland | Y (1998) | Y |  |
| Thailand | Y (1992) | Y | N |
| Turkey | Y (1993) | Y | N |
| United Arab Emirates | Y (1973) | Y | N |
| United Kingdom | Y (1998) | Y | N |
| United States | Y (1996) | Y | N |
| Venezuela | Y (1940) | Y |  |
| Vietnam | N | Y | Y |

| Country | Reply | Question 5 |  |  |  |  |  |  |  |  |  |  |  |  |  | Question 6 |  |  |  |  | Question 7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Redeployment | Redeployment strategy |  |  |  |  |  |  |  | Costs covered |  |  |  |  | Known cost | Financing |  |  |  | Method | Description of method |  |  |  |  |  |  |
|  |  |  | In existence | Under study | Consultation | Time | Mandatory | Information | Other | Not specified | State | New entrants | Former beneficiaries | Funds | Negotiation |  | State | Operator | Fees | Not specified |  | Bandwidth | Flat rate | Auction | Costs | Market | Operator | Information not supplied |
| Algeria | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |
| Armenia | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |
| Austria | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Bangladesh | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |
| Belarus | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Belgium | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Belize | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 |  |  |  |  |  | 1 |  |
| Bolivia | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Brazil | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  | 1 | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Bulgaria | 1 | 1 | 1 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Burkina Faso | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | 1 |  |  | 1 | 1 |  |  |  |  |  |  |
| Burundi | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Cameroon | 1 | 1 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |
| Canada | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 | 1 |  |  |  | 1 |  |  |  |  |  |  | 1 |
| Chad | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| China | 1 | 1 | 1 |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 | 1 |  |  |  | 1 |  |  |  |  | 1 |  |  |
| Colombia | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 | 1 |  |  |  |  |  |  |
| Côte d’Ivoire | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Croatia | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |
| Cyprus | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  | 1 |  | 1 |  |  |  |  |  |
| Czech Republic | 1 | 1 | 1 |  |  |  |  | 1 |  |  |  |  |  | 1 |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Dem. P. Rep. of Korea | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Denmark | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Egypt | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |
| Eritrea | 1 | 1 | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |
| Estonia | 1 | 1 | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Ethiopia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Finland | 1 | 1 | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| France | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  | 1 |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Gabon | 1 | 1 | 1 |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Georgia | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Germany | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 |  |  |  | 1 |  |  |  |
| Greece | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |
| Guyana | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Hungary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Indonesia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Iran | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Italy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Japan | 1 | 1 | 1 |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Rep of Korea | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Lao | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Latvia | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Lesotho | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | 1 |  |  | 1 |  |  |  |  |  | 1 |  |
| Liberia | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Libya |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lithuania | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |  |  |  |
| Madagascar | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Maldives | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Mali | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |
| Malta | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |
| Mexico | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Myanmar | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Namibia | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Nicaragua | 1 | 1 | 1 |  | 1 |  |  |  |  |  |  | 1 |  |  |  | 1 |  | 1 |  |  | 1 | 1 |  |  |  |  |  |  |
| Norway | 1 | 1 | 1 |  |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Pakistan | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 |  |  |  |  | 1 |  |  |
| Peru | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | 1 | 1 |  |  |  |  |  |  |
| Portugal | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 |  |  |  | 1 |  |  |  |
| Samoa | 1 | 1 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  | 1 |
| Seychelles | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Slovakia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spain | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Suriname | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |
| Sweden | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| Switzerland |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thailand | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Turkey | 1 | 1 | 1 |  |  |  | 1 |  |  |  |  | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |
| United Arab Emirates | 1 | 1 | 1 |  | 1 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| United Kingdom | 1 | 1 | 1 |  | 1 |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 |  |  | 1 |  |  |  |  |
| United States | 1 | 1 | 1 |  | 1 |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 1 |  |  | 1 |  |  |  |  |
| Venezuela | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |
| Vietnam | 1 | 1 | 1 |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 | 1 |  |  |  | 1 | 1 |  |  |  |  |  |  |
| Total | 65 | 49 | 29 | 6 | 4 | 1 | 3 | 5 | 1 | 11 | 1 | 2 | 2 | 2 | 3 | 38 | 25 | 6 | 19 | 14 | 51 | 41 | 1 | 2 | 2 | 2 | 2 | 2 |

| Spectrum Planning | Q3. Do you have a national radio frequency spectrum allocation table? Q4. Do you have technical specifications for national spectrum use? Q9. Do you notify frequency assignments to the ITU? Q10. Do you have a policy and planning function for spectrum management? ## Q11. Do you perform technical analyses of frequency assignment requests? |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # Country | Q3 | Q4 | Q9 | Q10 | Q11 |  |
| Algeria | Y | Y | Y | Y | Y |  |
| Armenia | Y | Y | Y | Y | Y |  |
| Austria | Y | Y | Y | Y | Y |  |
| Bangladesh | N | N | Y | N | N |  |
| Belarus | Y | Y | Y | Y | Y |  |
| Belgium | Y | Y | Y | Y | Y |  |
| Belize | Y | N | Y | N | Y |  |
| Bolivia | Y | N | N | Y | Y |  |
| Brazil | Y | Y | Y | Y | Y |  |
| Bulgaria | Y | Y | Y | Y | Y |  |
| Burkina Faso | N | N | Y | N | Y |  |
| Burundi | Y | Y | N | Y | Y |  |
| Cameroon | N | N | Y | Y | N |  |
| Canada | Y | Y | Y | Y | Y |  |
| Chad | N | N | N | N | N |  |
| China | Y | Y | Y | Y | Y |  |
| Colombia | Y | Y | N | Y | N |  |
| Côte d’Ivoire | Y (not public) | Y | Y | Y | Y |  |
| Croatia | Y | Y | Y | Y | Y |  |
| Cyprus | N | N | Y | Y | Y |  |
| Czech Republic | Y | Y | Y | Y | Y |  |
| Denmark | Y | Y | Y | Y | Y |  |
| Democratic Peoples Republic of Korea | Y | Y (some bands) | Y | Y | Y |  |
| Egypt | N | N | Y | N (planned) | N |  |
| Eritrea | Y | Y | Y | Y | Y |  |
| Estonia | Y | Y | Y | Y | Y |  |
| Ethiopia | N | Y | Y | N | Y |  |
| Finland | Y | Y | Y | Y | Y |  |
| France | Y | Y | Y | Y | Y |  |
| Gabon | Y | Y | N | Y | Y |  |
| Georgia | N | Y | Y | Y | Y |  |
| Germany | Y | Y | Y | Y | Y |  |
| Greece | Y | N | Y | Y | Y |  |
| Guyana | N | N | N | Y | N |  |
| Hungary | Y | Y | Y | Y | Y |  |
| Indonesia | Y | Y | Y | Y | Y |  |
| Iran | Y | Y | Y | Y | # |  |
| Japan | Y | Y | Y | Y | Y |  |
| Republic of Korea | Y | Y | Y | Y | Y |  |
| Lao | N | Y | Y | Y | N |  |
| Latvia | Y | Y | Y | Y | Y |  |
| Lesotho | Y | Y | Y | Y | Y |  |
| Liberia | Y | Y | N | # | # |  |
| Libya | Y | ! | Y | Y | Y |  |
| Lithuania | Y | Y | Y | Y | Y |  |
| Madagasgar | N | Y | Y | N | N |  |
| Maldives | Y | Y | N | N | Y |  |
| Mali | N | N | N | N | Y |  |
| Malta | N | Y | Y | N | Y |  |
| Mexico | Y | Y | N | N | Y |  |
| Myanmar | Y | Y | Y | Y | # |  |
| Namibia | Y | Y | N | N | N |  |
| Nicaragua | Y | Y | Y | Y | Y |  |
| Norway | Y | Y | Y | N | N |  |
| Pakistan | Y | N | Y | Y | Y |  |
| Peru | N | N | N | Y |  |  |
| Portugal | Y | Y | Y | Y | Y |  |
| Samoa | Y | Y | Y | N | Y |  |
| Seychelles | Y | Y | Y | N | Y |  |
| Slovakia |  |  |  |  |  | No Part II |
| Spain | Y | Y | Y | N | N |  |
| Suriname | Y | Y | Y | Y | Y |  |
| Sweden | Y | Y | Y | Y | Y |  |
| Switzerland | Y | Y | Y | N (planned) | Y |  |
| Thailand | Y | Y | Y | Y | Y |  |
| Turkey | Y | Y | Y | Y | Y |  |
| United Arab Emirates | N | Y | Y | N (planned) | Y |  |
| United Kingdom | Y | Y | Y | Y | Y |  |
| United States | Y | Y | Y | Y | Y |  |
| Venezuela | Y | Y | Y | Y | Y |  |
| Vietnam | Y | N | N | Y | Y |  |

Examples of national frequency allocation tables

The following three national frequency allocation tables are included as examples of the different formats and structures.

1. United States of America

2. Brazil

3 Japan

Example extract of a national table of frequency allocations (1): United States of America

| Extract from Radio Regulations, 1998 edition | United States Spectrum Uses |
| --- | --- |
| 37.5-38.25 FIXED MOBILE Radio astronomy S5.149 | 37.5– 38 Private Sector Usage: This band is used primarily by the Private Land Mobile Radio Services consisting of public safety and industrial/business licensees. Federal & Private Sector Usage. Radio Astronomy. Continuum observations are performed in this band that study electromagnetic radiation from the planet Jupiter and from the Sun. 38– 38.25 Federal Usage: Land Mobile Radio. This band is used primarily for tactical and training operations by some Federal agencies for combat net radio operations that provide command and control for combat, combat support, and combat service support units. Frequencies also used for air-to-ground communications for close air support requirements and experimental testing. Maritime Mobile. U.S. Coast Guard ship-to-ship and ship-to-shore communications. Federal & Private Sector Usage. Radio Astronomy. Continuum observations are performed in this band to study electromagnetic radiation from the Sun and the planet Jupiter. |
| 38.25-39.986 FIXED MOBILE | 38.25– 39 Federal Usage: Land Mobile Radio. This band is extensively used for land mobile radio communications in the operation, protection, and maintenance of national parks, forests, wildlife refuge areas, etc. Frequencies in this band are also used for reservation programs, law enforcement, public safety operations, control of power generation transmission and water facilities, environmental data collection, fish management, and wildlife telemetry programs. However, this band is used primarily for tactical and training operations by some Federal agencies for combat net radio operations that provide command and control for combat, combat support, and combat service support units. Frequencies also used for air-to-ground communications for close air support requirements. |

Example extract of a national table of frequency allocations (2): Brazil

MHz

| # REGION 2 | # BRAZIL |
| --- | --- |
| 29,7-30,005 FIXED MOBILE | 29,7-30,005 FIXED LAND MOBILE |
| 30,005-30,01 SPACE OPERATION (satellite identification) FIXED MOBILE SPACE RESEARCH | 30,005-30,01 SPACE OPERATION (satellite identification) FIXED LAND MOBILE SPACE RESEARCH |
| 30,01-37,5 FIXED MOBILE | 30,01-37,5 FIXED LAND MOBILE |
| 37,5-38,25 FIXED MOBILE Radio astronomy S5.149 | 37,5-38,25 FIXED MOBILE Radio astronomy S5.149 |
| 38,25-39,986 FIXED MOBILE | 38,25-39,986 FIXED LAND MOBILE |

#### MHz

| # USAGE | ## ALLOTMENT | ## REGULATION |
| --- | --- | --- |
| 29,7-30,005 Telephony in fixed and mobile services |  | MC Administrative Ruling n° 280/79 (Published 03.14. 79) DENTEL Instruction n° 11/81 (Published 08.18. 81) |
| 30,005-30,01 Official and private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 30,01-33,555 Official and private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 33,555-33,915 Radio taxi - cab systems |  | Anatel Resolution nº 239/2000 (Published 11.30. 2000) |
| Official, private and public correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 33,915-34,475 Official and private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 34,475-34,83 Radio taxi - cab systems |  | Anatel Resolution nº 239/2000 (Published 11.30. 2000) |
| Private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 34,83-35,525 Private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 35,525-36 Paging |  | MC Administrative Ruling n° 1306/96 (Published 10.30.96) |
| Private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 36-37,5 Official and private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 37,5-38,25 Official, private and public correspondence |  | DENTEL Instruction n° 4/89 (Published 0510..89) |
| 38,25-38,31 Private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 38,31-38,57 Radio taxi - cab systems |  | Anatel Resolution nº 239/2000 (Published 11.30. 2000) |
| Private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 38,57-38,73 Private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |
| 38,73-39,83 Radio taxi - cab systems |  | Anatel Resolution nº 239/2000 (Published 11.30. 2000) |
| Private correspondence |  | DENTEL Instruction n° 4/89 (Published 05.10. 89) |

Example extract of a national table of frequency allocations (3): Japan

| Read only |  |  | To be completed |  |
| --- | --- | --- | --- | --- |
| Allocation to services |  |  | Japanese Allocation to services |  |
| Region 1 | Region 2 | Region 3 | National Allocation | Remarks |
|  |  |  | 898-901 MOBILE | Cellular Telephone (FDMA System) (land Mobile Station) Cellular Telephone (TDMA System) (Land Mobile Station) Cellular Telephone (CDMA System) (Land Mobile Station) |
|  |  |  | 901-903 MOBILE | Local Area Disaster - Prevention Radio |
|  | 902-928 FIXED Amateur Mobile exept aeronauticaul mobile Radioallocation S5.150 S5.325 S5.126 |  | 903-905 MOBILE | Personal Radio |
|  |  |  | 905-915 MOBILE 174 | MCA (Multi-Channel Access) Land Mobile Communication (Land Mobile Station, Directive Station) Digital MCA (Multi-Channel Access ) Land Mobile Communication (Land Mobile Station, Directive Station) |
|  |  |  | 915-940 MOBILE 174 | Cellular Telephone (FDMA System) (Land Mobile Station) Cellular Telephone (TDMA System) (Land Mobile Station) Cellular Telephone (CDMA System) (Land Mobile Station) |

Common Table of Allocations prepared by a sub-region or country group

The following two tables contain extracts of frequency allocation tables prepared by regional telecommunication organisations or country groups. Such groups and organisations are formed by countries that decide amongst themselves that there are mutual benefits in co-operating in telecommunication matters including spectrum management. In some cases, especially where there is a close geographical proximity of several national borders, frequency co-ordination is simplified by harmonising frequency use and technical criteria for sharing frequencies. There may be other benefits in such co-operation, for example in having common equipment specifications and free circulation and use of equipment. One of the products of this co-operation may be a “Common Frequency Allocation Table”. This may vary in complexity, from a simple combination of the national tables to a longer term commitment to harmonise use so that, eventually, the national tables are largely the same as the common table.

It is emphasised that these tables are included only as examples of format and structure.

Inter-American Telecommunication Commission (CITEL)

On the following page is presented an extract from the CITEL on-line searchable database.  The CITEL database permits a comparison of national spectrum allocations between countries in the Americas region. The user can specify a frequency range and the countries of interest; based on the search criteria selected  a report is then generated in *.rtf format that can be either viewed on line or printed. The CITEL web page can be reached at --

http://oasforum.oas.org/citelituv2.nsf/searchform?openform

European Conference of Postal and Telecommunications Administrations (CEPT)

The CEPT European Common Allocation Table (ECA) has grown in complexity as the degree of harmonisation of spectrum use has increased between the 44 countries concerned. Member administrations of CEPT commit themselves to applying frequency harmonisation measures through “Decisions” developed by the European Radiocommunication Committee (ERC) (to be called the Electronic Communication Committee at the end of 2001). Frequency use is further harmonised by the common  application  of equipment specifications based on the standards produced by the European Telecommunications Standards Institute (ETSI). The applicability of ERC Decisions and ETSI standards to particular frequency sub-bands is shown in this extract from the ECA.

Extract of the on-line frequency allocation table produced by CITEL

| EU | 838 -862 | MHz | FIXED BROADCASTING S5.312 S5.314 S5.315 S5.316S5.319 S5.321 EU2 EU13 |
| --- | --- | --- | --- |
| USA-NG | 849 -851 | MHz | FIXED AERONAUTICAL MOBILE BROADCASTING S5.317 S5.318 NG63 |
| USA-NG | 851 -866 | MHz | FIXED LAND MOBILE BROADCASTING S5.317 S5.318 NG31 NG63 |
| EU | 862-890 870 | MHz | FIXED MOBILE except aeronautical mobile BROADCASTING S5.322 S5.319 S5.323 EU2 EU13 |
| USA-NG | 866 -869 | MHz | FIXED LAND MOBILE BROADCASTING S5.317 S5.318 NG63 |
| USA-NG | 869 -890 | MHz | FIXED LAND MOBILE BROADCASTING S5.317 S5.318 NG31 NG63 |
| EU | 870 -876 | MHz | FIXED MOBILE except aeronautical mobile BROADCASTING S5.322 S5.319 S5.323 EU2 EU13 |
| EU | 876 -880 | MHz | FIXED MOBILE except aeronautical mobile BROADCASTING S5.322 S5.319 S5.323 EU2 EU13 |
| EU | 880 -890 | MHz | FIXED MOBILE except aeronautical mobile BROADCASTING S5.322 S5.319 S5.323 EU2 EU13 |
| ITU | 890-902 | MHz | FIXED MOBILE except aeronautical mobile Radiolocation S5.318 S5.325 |
| ITU | 890-902 | MHz | FIJO MOVIL salvo móvil aeronáutico Radiolocalización S5.318 S5.325 |
| CAN | 890-902 | MHz | FIXED MOBILE except aeronautical mobile Radiolocation C5A S5.318 S5.325 |
| EU | 890-942 915 | MHz | FIXED MOBILE except aeronautical mobile BROADCASTING S5.322 Radiolocation S5.323 EU13 EU14 |
| USA-G | 890-902 | MHz | FIXED MOBILE except aeronautical mobile Radiolocation S5.318 S5.325 US116 US268 G2 |
| USA-NG | 890-902 894 | MHz | FIXED LAND MOBILE except aeronautical mobile Radiolocation S5.318 S5.325 US116 US268 NG63 NG151 |
| USA-NG | 890-902 | MHz | FIXED MOBILE except aeronautical mobile Radiolocation S5.318 S5.325 US116 US268 |
| USA-NG | 894 -896 | MHz | FIXED AERONAUTICAL MOBILE except aeronautical mobile Radiolocation S5.318 S5.325 US116 US268 |
| USA-NG | 896 -901 | MHz | FIXED LAND MOBILE except aeronautical mobile Radiolocation S5.318 S5.325 US116 US268 |
| ITU | 902-928 | MHz | FIXED Amateur Mobile except aeronautical mobile Radiolocation S5.150 S5.325 S5.326 |
| ITU | 902-928 | MHz | FIJO Aficionados Móvil salvo móvil aeronáutico Radiolocalización S5.150 S5.325 S5.326 |
| CAN | 902-928 | MHz | FIXED RADIOLOCATION C5A Amateur Mobile except aeronautical mobile Radiolocation S5.150 S5.325 S5.326 |
| USA-G | 902-928 | MHz | FIXED RADIOLOCATION G59 Amateur Mobile except aeronautical mobile Radiolocation S5.150 S5.325 S5.326 US215 US218 US267 US275 G11 |

| Frequency Band | RR Region 1 Allocation and relevant footnote | European Common Allocation | # Utilisation | Footnote on utilisation | ERC Document | Available Standard | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 862-870 MHz | FIXED MOBILE except Aeronautical Mobile BROADCASTING S5.322 S5.319 S5.323 | MOBILE S5.323 EU13 | Cordless Telephones | ###### EU2 |  | I-ETS 300 131 |  |
|  |  |  | Defence systems | # EU2 |  |  |  |
|  |  |  | SRD in 868-870 MHz | EU2 | ERC REC 70-03 | EN 300 220-1 |  |
|  |  |  | Social Alarms in 869.2-896.25 MHz | EU2 | ERC DEC (97)06 | EN 300 220-1 |  |
|  |  |  | Wireless Audio in 863-865 MHz | EU2 | ERC REC 70-03 | EN 301 357 |  |
|  |  |  | Radio microphones used on a tuning range basis in 863-865 MHz | EU2 | ERC REC 70-03 | I-ETS 300 422 |  |
| 870-876 MHz |  |  | Digital land mobile TETRA | EU2 | ERC DEC (96)04 ERC Rec T/R22-05 | ETS 300 394-1 | ML paired with 915-921 MHz |
|  |  |  | Defence systems |  |  |  |  |
| 876-880 MHz |  |  | Digital land mobile | EU2 |  |  | ML paired with 921-926 MHz |
|  |  |  | UIC railway systems |  | ERC REC T/R25-09 |  |  |
|  |  |  | Defence systems |  |  |  |  |
| 880-890 MHz |  |  | EGSM | EU2 | ERC DEC (97)06 | TBR 31 Ed 2 | ML paired with 925-935 MHz |
|  |  |  | Defence systems |  |  |  |  |

Example of national spectrum management strategy

The following table is an extract from the United Kingdom’s spectrum strategy. Many of the issues and associated strategies are specific to the UK, therefore it is included only as an example of one option for the format and structure of a national strategy.

The strategy shows:

- national requirements, obtained through consultation with users, manufacturers operators etc.;

- commitments to harmonise with neighbouring countries by implementation of CEPT ERC Decisions;

- requirements to implement the outcome of a previous WRC; and

- information on what changes might be expected from future WRCs.

As well as providing direction and co-ordination of objectives for the administration, spectrum users are provided with detailed information and gain confidence in the planning process to incorporate into their own business plans.

| ## Frequency Range |  |  |  | Allocation to United Kingdom Services | ##### ISSUES | ##### STRATEGY | ###### Time-scales |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 420 | MHz | 450 | MHz | FIXED MOBILE RADIOLOCATION LAND MOBILE Amateur-Satellite Amateur S5.269, S5.270, S5.271, S5.284, S5.285, S5.286 | TECHNOLOGY DEVELOPMENT Interference to vehicle radio keys in the band 433.72-434.12 MHz. 433.92 MHz band is recommended by CEPT/ERC as a harmonised band for SRDs. | The RA has developed a policy, in agreement with other CEPT administrations, that speech applications will not be allowed in the band 433.72-434.12 MHz. Review of the use of the 433.050 - 434.790 MHz frequency band, with a view to harmonise the band. The traffic information service will be moved to an alternative assignment at 449.10625 MHz. | Ongoing |
|  |  |  |  |  | PUBLIC NETWORKS Introduction of civil PAMR and PBR in a 410-430 MHz CEPT aligned block of spectrum. | PAMR licensed for two operators in low parts of duplex allocation. Sharing with government services under discussion. | Ongoing |
|  |  |  |  |  | PRIVATE BUSINESS RADIO Resolution of Continental Interference suffered by users in the band (UHF1). Increasing requirement to harmonise allocations in CEPT. Note: ERC Decision on TETRA (ERC/DEC(96)04) Migration of SRBR into European harmonised band 446.0 - 446.1 MHz. | Evaluate the possibilities and consequences of long term alignment of users in the entire band with CEPT recommendation T/R 25-08. Discuss prospects for re-alignment with users currently in the band as opportunities for individual re-alignment arise. Investigate possibilities for obtaining spectrum for private TETRA systems. Await production of relevant ERC Decision. | 2005 Ongoing |
| 420 | MHz | 450 | MHz | Continued from previous page | AMATEUR WRC-97 Res.COM 5.5 permits the use of wind profilers in the band 440-450 MHz. | Wind profiler radars to have due regard to minimizing interference to other services, dependent on the status of those services. | Ongoing |
|  |  |  |  |  | PROGRAMME MAKING AND SPECIAL EVENTS No current issues. | No changes planned in medium term. |  |
|  |  |  |  |  | SCIENCE SERVICES Proposal for an allocation to Earth Exploration Satellite Service (active) on agenda for WRC-99. | Protection of existing services should be ensured. Contribute to discussions within CEPT. | WRC-99 |
|  |  |  |  |  | MoD Radar/Land mobile communications. Sharing with amateurs and civil use in urban areas. | Replacement band for VHF. Give up channels for civil TETRA in 410 - 430 MHz. Plan military use of TETRA in 410 - 430 MHz. |  |

| Spectrum control (including monitoring and technical analyses of interference) | Q12. Do you perform radio monitoring? Q13. Do you perform technical analyses of radio frequency interference complaints? |  |  |
| --- | --- | --- | --- |
| ## Country | Q12 | Q13 | Comments |
| Algeria | Y | Y |  |
| Armenia | Y | Y |  |
| Austria | Y | Y |  |
| Bangladesh | N | N |  |
| Belarus | Y | Y |  |
| Belgium | Y | Y |  |
| Belize | Y | N |  |
| Bolivia | Y | Y |  |
| Brazil | Y | Y |  |
| Bulgaria | Y | Y |  |
| Burkina Faso | Y | Y |  |
| Burundi | N | Y |  |
| Cameroon |  |  |  |
| Canada | Y | Y |  |
| Chad | Y | N |  |
| China | Y | Y |  |
| Colombia | Y | Y |  |
| Côte d’Ivoire | Y (2F, 2M) | Y |  |
| Croatia | Y | Y |  |
| Cyprus | Y | Y |  |
| Czech Republic | Y | Y |  |
| Denmark | Y | Y |  |
| Democratic People's Republic of Korea | Y | Y |  |
| Egypt | N [planned] | N [planned} |  |
| Eritrea | Y | Y |  |
| Estonia | Y | Y |  |
| Ethiopia | N | N |  |
| Finland | Y | Y |  |
| France | Y | Y |  |
| Gabon | Y | Y |  |
| Georgia | Y | Y |  |
| Germany | Y | Y |  |
| Greece | Y | Y |  |
| Guyana | Y | Y |  |
| Hungary | Y | Y |  |
| Indonesia | Y (8F 80M) | Y |  |
| Iran | Y | Y |  |
| Japan | Y | Y |  |
| Republic of Korea | Y | Y |  |
| Lao | Y | N |  |
| Latvia | Y | Y |  |
| Lesotho | Y | Y |  |
| Liberia | Y | Y |  |
| Libya | N | ! |  |
| Lithuania | Y | Y |  |
| Madagasgar | N | Y |  |
| Maldives | Y | Y |  |
| Mali | N | N |  |
| Malta | Y | Y |  |
| Mexico | Y | Y |  |
| Myanmar | Y | ! |  |
| Namibia | N [planned] | N |  |
| Nicaragua | Y | Y |  |
| Norway | Y | Y |  |
| Pakistan | N [planned] | Y |  |
| Peru | Y | Y |  |
| Portugal | Y | Y |  |
| Samoa |  |  |  |
| Seychelles | Y | Y |  |
| Slovakia |  |  | No Part II |
| Spain | Y | Y |  |
| Suriname | Y | Y |  |
| Sweden | Y | Y |  |
| Switzerland | Y | Y |  |
| Thailand | Y | Y |  |
| Turkey | Y [planned] | Y |  |
| United Arab Emirates | Y | Y |  |
| United Kingdom | Y | Y |  |
| United States | Y | Y |  |
| Venezuela | Y | Y |  |
| Vietnam | Y | Y |  |

Annex 2-4: Chart 1

Fixed measurement stations

Annex 2-4: Chart 2

Fixed radio direction-finding stations

Annex 2-4: Chart 3

Distribution of fixed stations by frequency band

Annex 2-4: Chart 4

Mobile measurement stations

Annex 2-4: Chart 5

Mobile radio direction-finding stations

Annex 2-4: Chart 6

Distribution of mobile stations by frequency band

| Information Technology | Q8. Do you maintain centralized databases for spectrum management? Q14. What computers and operating systems are in use for national spectrum management? |  |  |
| --- | --- | --- | --- |
| Country | Q8 | Q14 | Comment |
| Algeria | Y 304 | UNIX/ORACLE/PC |  |
| Armenia | Y 2500 | MS/PC |  |
| Austria | Y 250000 | IBM/MS/M-F/PC |  |
| Bangladesh | Y 5000 | MS/PC |  |
| Belarus | Y 50000 | MS/PC |  |
| Belgium | Y 80000 | # |  |
| Belize | Y 4047 | MS/PC |  |
| Bolivia | Y 5750 | MS/PC |  |
| Brazil | Y 47 000 000 | MVS,MS/IBM,PC |  |
| Bulgaria | Y 12 500 | MS/PC |  |
| Burkina Faso | N | MS/PC |  |
| Burundi | Y 150 | # |  |
| Cameroon | # | PC/MS/DOS/WIN |  |
| Canada | Y 500 000 | UNIX,MS/PC |  |
| Chad | # | # |  |
| China | [N] 2 000 000 | MS/PC |  |
| Colombia | Y 10 000 | MS/PC |  |
| Côte d’Ivoire | N | 0 |  |
| Croatia | Y 100 000 | MS/PC |  |
| Cyprus | N | MS/PC |  |
| Czech Republic | Y 30 000 | MS/PC |  |
| Denmark | Y 46 000 | MS/PC |  |
| Democratic Peoples Republic of Korea | N | MS/PC |  |
| Egypt | Y 29 400 | MS/PC |  |
| Eritrea | Y 100/year | MS/PC |  |
| Estonia | N | MS/PC |  |
| Ethiopia | Y 20000 | DELL/NT |  |
| Finland | Y | HP,UNIX,MS/PC |  |
| France | Y 150 000 | UNIX,MS/Work Stns |  |
| Gabon | Y 300 | 4D/Mac |  |
| Georgia | N | MS/PC |  |
| Germany | N | MS/PC |  |
| Greece | Y 30 000 | MS/PC |  |
| Guyana | Y 16 000 | MS/PC |  |
| Hungary | Y 400 000 | MS,Solaris,UNIX/PC, |  |
| Indonesia | Y 250 000 | MS/PC |  |
| Iran | Y 240 000 | # |  |
| Japan | Y 2.2 GB | UNIX,MS/MF,WS |  |
| RoKorea | Y 53 292 | UNIX/WS |  |
| Lao | Y 4 000 | MS/DEC |  |
| Latvia | N | MS/PC |  |
| Lesotho | N 110 | MS/PC | Not Y2K |
| Liberia | Y 1 000 | 0 |  |
| Libya | # | # |  |
| Lithuania | Y 50 000 | MS/PC |  |
| Madagasgar | N | MS/PC |  |
| Maldives | N | MS/PC |  |
| Mali | N | MS/PC |  |
| Malta | Y 5 000 | MS/PC |  |
| Mexico | Y 230 000 | UNIX,ORACLE/SUN |  |
| Myanama | Y 4 500 | # |  |
| Namibia | Y 10 000 | # |  |
| Nicaragua | Y 5 000 | MS/PC |  |
| Norway | Y 25 000 | MS,Oracle/PC,SUN |  |
| Pakistan | Y 8 500 | MS/PC |  |
| Peru | N | MS,UNIX/PC |  |
| Portugal | Y 120 000 | MS/PC |  |
| Samoa |  |  |  |
| Seychelles | N | MS/PC |  |
| Slovakia |  |  | No Part II |
| Spain | Y | MS/PC |  |
| Suriname | Y 375 | MS/PC |  |
| Sweden | Y 100 000 | MS,Solaris,UNIX/ PC, SUN |  |
| Switzerland | Y 200 000 | MS,HP,Unix/PC,WS |  |
| Thailand | Y 100 000 | Unix,DOS/PC |  |
| Turkey | Y | Unix,MS/SUN,PC |  |
| United Arab Emirates | Y 23 000 | MS/PC |  |
| United Kingdom | Y 223 000 | Unix,MS/PC |  |
| United States | Y 1 000 000 | Unix,MS,SUN,Novell/ SUN,PC |  |
| Venezuela | Y | MS |  |
| Vietnam | Y 30 000 | MS,Solaris/SUN,HP |  |

Key:

MS	=	MicroSoft®

WIN	=	MicroSoft® Windows

NT	=	MicroSoft® NT

MF	=	MicroSoft® Fortran

DOS	=	Disk Operating System

PC	=	Personal Computer

Mac	=	Macintosh®

UNIX

SUN	=	Sun Microsystems Inc

Oracle	=	Oracle Corporation

Novell	=	Novell Networks

HP	=	Hewlett Packard

#	=	Not supplied

Analysis of Answers to Part II - Questions 15, 16 and 17

Table 1

| Administration | Question 15: Number of Technical/ Professional Staff (employed on spectrum management duties) | Question 16: Number of Support Staff (employed on spectrum management duties) | Question 17 : Structure of the Spectrum Management Organisation |
| --- | --- | --- | --- |
| Algeria | 83 | 7 | Diagram supplied |
| Argentina |  |  | www.cnc.gov.ar |
| Armenia | 40 | 40 | Diagram supplied |
| Austria | 15 | 75 | Diagram supplied |
| Bangladesh | 5 | 4 | Diagram supplied |
| Belarus |  |  | Description |
| Belgium | 51 | 12 | www.bipt.be |
| Belize | 3 | 2 | Diagram supplied |
| Bolivia | 10 | 10 | Diagram supplied |
| Brazil | 317 | 295 | Diagram supplied |
| Bulgaria | 17 | 7 | Diagram supplied |
| Burkina Faso | 4 | 1 | Diagram supplied |
| Burundi | 8 | 20 |  |
| Cameroon | 17 | 4 |  |
| Canada | 350 | 100 | Diagram supplied |
| CEPT |  |  |  |
| Chad |  |  |  |
| China | 75 | 25 |  |
| Colombia | 30 | 30 | www.mincomunicaciones.gov.co |
| Côte d’Ivoire | 19 | 6 |  |
| Croatia | 20 | 10 |  |
| Cyprus | 7 | 4 | www.ctu.cz |
| Czech Republic | 128 | 36 |  |
| Denmark | 38 total |  | Diagram supplied |
| Democratic People’s Republic of Korea |  |  |  |
| Egypt | 12 | 18 |  |
| Eritrea | 5 | 2 | Diagram supplied |
| Estonia | 35 | 10 |  |
| Ethiopia |  |  |  |
| Finland | 30 | 30 | Diagram supplied |
| France | 250 | 70 | Diagram supplied |
| Gabon | 20 | 4 | Diagram supplied |
| Georgia | 20 | 30 |  |
| Germany | 900 total |  | www.regtp.de |
| Greece | 5 | 10 | www.eett.gr |
| Guyana | 7 | 19 |  |
| Hungary |  |  | www.mav.hu |
| Indonesia |  |  |  |
| Iran | 120 | 50 | Diagram supplied |
| Italy |  |  | Www.comunicazioni.it |
| Japan | 2500 total |  | Diagram supplied |
| Korea | 55 | 1181 | Diagram supplied |
| Lao | 5 | 17 | Diagram supplied |
| Latvia | 50 | 20 | Diagram supplied |
| Lesotho | 2 | 0 | Diagram supplied |
| Liberia | 6 | 8 |  |
| Libya |  |  |  |
| Lithuania | 50 | 50 | Diagram supplied |
| Madagascar | 5 | 2 | Diagram supplied |
| Maldives | 7 | 4 | Description |
| Mali |  |  |  |
| Malta | 8 | 4 |  |
| Mexico | 15 | 15 | Description |
| Myanmar |  |  |  |
| Namibia | 1 | 0 | www.ncc.org.na |
| Nicaragua | 10 | 5 | Diagram supplied |
| Norway | 40 | 3 | www.npt.no |
| Pakistan | 55 | 64 | Www.pak.gov.pk/govt |
| Peru | 16 | 14 | Diagram supplied |
| Portugal | 79 | 8 | www.icp.pt |
| Republic of Korea |  |  |  |
| Seychelles | 3 | 2 |  |
| Slovakia |  |  | Www.telecom.gov.sk |
| Spain |  |  | Www.sgc.mfom.es |
| Suriname | 4 | 2 |  |
| Sweden | 80 total |  | Diagram supplied |
| Switzerland | 84 | 0 | Diagram supplied |
| Thailand | 190 | 170 | Diagram supplied |
| Turkey | 25 | 10 | Diagram supplied |
| United Arab Emirates |  |  |  |
| United Kingdom | 245 | 297 | Diagram supplied |
| United States | 1050 | 350 | Diagram supplied |
| Venezuela |  |  | Diagram supplied |
| Vietnam | 205 | 0 | Diagram supplied |
|  | 99 | 86 |  |

Analysis of Answers to Part II - Questions 15, 16 and 17

Table 2

| Administration | Population | Number of Technical/ Professional Staff (employed in spectrum management duties) | Number of Support Staff (employed in spectrum management duties) | Size of Spectrum Management Database (Given as number of records unless otherwise stated) |
| --- | --- | --- | --- | --- |
| Algeria | 31,471,000 | 83 | 7 | 304 |
| Argentina | 37,031,802 |  |  |  |
| Armenia | 3,809,000 | 40 | 40 | 2500 |
| Austria | 8,094,000 | 15 | 75 | 250000 |
| Bangladesh * | 128,133,000 | 5 | 4 | 5000 |
| Belarus | 10,004,000 |  |  | 50000 |
| Belgium | 10,246,000 | 51 | 12 | 80000 |
| Belize | 254,000 | 3 | 2 | 4047 |
| Bolivia | 8,281,000 | 10 | 10 | 5750 |
| Brazil | 158,232,252 | 317 | 295 | 47000000 |
| Bulgaria | 8,152,000 | 17 | 7 | 12500 |
| Burkina Faso * | 11,946,000 | 4 | 1 |  |
| Burundi * | 6,054,000 | 8 | 20 | 150 |
| Cameroun | 15,422,000 | 17 | 4 |  |
| Canada | 30,764,000 | 350 | 100 | 500000 |
| Chad * | 7,977,000 |  |  |  |
| China | 1,264,536,000 | 75 | 25 | 2000000 |
| Colombia | 40,037,000 |  |  | 10000 |
| Côte d’Ivoire |  |  |  |  |
| Croatia | 4,600,000 | 20 | 10 | 100000 |
| Cyprus | 882,000 | 7 | 4 |  |
| Czech Republic | 10,275,000 | 128 | 36 | 30000 |
| Denmark | 5,330,000 | 38 total |  | 46000 |
| Democratic People’s Republic of Korea | 21,688,000 |  |  |  |
| Egypt | 68,344,000 | 12 | 18 | 29400 |
| Eritrea * | 4,142,000 | 5 | 2 | 100 assignment/year |
| Estonia | 1,433,000 | 35 | 10 |  |
| Ethiopia | 64,117,000 | 3 | 1 | 20000 |
| Finland | 5,177,000 | 30 | 30 | 130000 frequencies |
| France | 59,353,000 | 250 | 70 | 150000 |
| Gabon | 1,226,000 | 20 | 4 | 300 |
| Georgia | 5,454,000 | 20 | 30 |  |
| Germany | 82,141,000 | 900 total |  |  |
| Greece | 10,596,000 | 5 | 10 | 30000 |
| Guyana | 698,000 | 7 | 19 | 1600 |
| Hungary | 10,020,000 |  |  | 400000 |
| Indonesia | 212,207,000 |  |  | 250000 |
| Iran | 67,411,000 | 120 | 50 | 240000 |
| Italy | 57,820,000 |  |  |  |
| Japan | 126,876,000 | 2500 |  | 2.2 GByte |
| Korea | 47,275,000 |  |  | 53292 |
| Lao * | 5,218,000 | 5 | 17 | 4000 |
| Latvia | 2,416,000 | 50 | 20 |  |
| Lesotho * | 2,143,000 | 2 | 0 | 110 |
| Liberia * | 3,164,000 | 6 | 8 | 1000 |
| Libya | 5,114,000 |  |  |  |
| Lithuania | 3,697,000 | 50 | 50 | 50000 |
| Madagascar * | 14,858,000 | 5 | 2 |  |
| Maldives * | 286,000 | 7 | 4 |  |
| Mali * | 11,234,000 |  |  |  |
| Malta | 390,000 | 8 | 4 | 5000 |
| Mexico | 99,639,000 | 15 | 15 | 230000 |
| Myanmar | 48,852,000 |  |  | 4500 |
| Namibia | 1,771,000 | 1 | 0 | 10000 |
| Nicaragua | 5,074,000 | 10 | 5 | 5000 |
| Norway | 4,487,000 | 40 | 3 | 25000 |
| Pakistan | 150,648,000 | 55 | 64 | 8500 |
| Peru | 27,136,000 | 16 | 14 |  |
| Portugal | 10,013,000 | 79 | 8 | 120000 |
| Republic of Korea | 47,275,000 |  |  |  |
| Seychelles | 82,000 | 3 | 2 |  |
| Slovakia | 5,401,000 |  |  |  |
| Spain | 39,466,000 |  |  |  |
| Suriname | 434,000 | 4 | 2 | 375 |
| Sweden | 8,866,000 | 80 total |  | 100,000 |
| Switzerland | 7,142,000 | 84 | 0 | 200000 |
| Thailand | 62,043,000 | 190 | 170 | 100000 |
| Turkey | 65,311,000 | 25 | 10 |  |
| United Arab Emirates | 2,835,000 |  |  | 23000 |
| United Kingdom | 57,750,000 | 245 | 297 | 223000 |
| United States | 275,600,000 | 1050 | 350 | 100000000 |
| Venezuela | 23,916,810 |  |  |  |
| Vietnam | 79,939,014 | 205 | 0 | 30000 |

* Least Developed Countries

Source of population data:	(1) www.worldpop.org/prbdata.htm

(2) www.cia.gov/cia/publications/factbook/index.html

| Use of ITU Handbooks | Q18. Do you use the ITU-R Handbooks and Reports |  |
| --- | --- | --- |
| Country | Handbook (for letter key see below) | # Comments, including additional information requirements |
| Algeria | a, e, | Manuals are not available to all staff |
| Armenia | none | Request for Russian language versions of these handbooks |
| Austria | a, b, c, | Request for more frequent updates of these handbooks |
| Bangladesh | f |  |
| Belarus | a,b, |  |
| Belgium | b |  |
| Belize | a, | Request for Windows s/w intermodulation and harmonic interference analysis, path profile analysis |
| Bolivia | a,b, |  |
| Brazil | a,b,c,e | Request for updates |
| Bulgaria | a,b,e | Request for c,d,f |
| Burkina Faso | a,b,e,f | Request for ITU-R recommendations on spectrum management |
| Burundi | a | Request for handbook on Telecommunications Regulations |
| Cameroon | a,b,e | Assistance requirements in respect of: strategy for frequency usage planning; use of MSPACE and WinBASMS planning softwares; administration of spectrum management projects. |
| Canada | a,b,c,e, |  |
| Chad |  |  |
| China | a,b,c,d,e | Request for all ITU-R handbooks |
| Colombia |  |  |
| Côte d’Ivoire | a,b,e, |  |
| Croatia |  | Requesting latest additions of handbooks |
| Cyprus | b | Request for all Windows based spectrum management software |
| Czech Republic | b |  |
| Denmark | b,c |  |
| Democratic People's Republic of Korea | a,b, | Lists of ITU-R and ITU-T recommendations published since 1995 |
| Egypt | a,b, |  |
| Eritrea | a,b,e, | Training required with some handbooks. Request BR/BDT cooperation to produce handbook on National Regulations and National Spectrum Fees |
| Estonia | a,b,f |  |
| Ethiopia | a,b,c,d,e,f |  |
| Finland | c |  |
| France | b,e |  |
| Gabon |  |  |
| Georgia | a,b,e | Request c,d,f |
| Germany | a,b,c,d,e |  |
| Greece | a,b, | Request development of regulatory framework for installing antennas |
| Guyana | a,b,e,f | Request c,d and updated versions of a,e |
| Hungary | (a,b,c,d,e |  |
| Indonesia | a,b,e, | Request ITU Recommendations |
| Iran | a,b,d | Request c |
| Japan | not used |  |
| Rep. of Korea |  |  |
| Lao |  | Request for all ITU-R hand books |
| Latvia | a,b,c,d,e |  |
| Lesotho | a,b, |  |
| Liberia | a,b, | Request latest versions of hand books a,b,c,d,e |
| Libya | a, |  |
| Lithuania | a,b,c, | Request for latest version of b |
| Madagasgar | not used |  |
| Maldives |  | Request latest versions of b,c |
| Mali |  |  |
| Malta | a |  |
| Mexico |  |  |
| Myanmar |  | Request a,b,c,f |
| Namibia |  |  |
| Nicaragua | a,b,e | Request for latest versions of all handbooks and WinBASMs |
| Norway | a,e | Request for handbook on satellite regulation |
| Pakistan | a,b, | Request for several ITU-R handbooks and the Radio Regulations |
| Peru | a,b, |  |
| Portugal | a,b,c,d,e,f |  |
| Samoa |  | Request for a,b,c,d,e f |
| Seychelles | a,b,c, | Request for f |
| Slovakia |  |  |
| Spain | b,c |  |
| Suriname | f |  |
| Sweden |  |  |
| Switzerland | a,b |  |
| Thailand | a,b,e |  |
| Turkey | a,b,c,d,e,f |  |
| United Arab Emirates |  |  |
| United Kingdom | a,b,e | Request an update on ITU-R SM.1413, the inclusion of the BR validation rules within SM.1413 and the issuing of this document in electronic format with validation software for space and terrestrial notification. |
| United States | a,b,c,d,e,f | Request update of b |
| Venezuela | a,b | Request latest versions |
| Vietnam | a,b, | Request latest versions of c,f |

Key:

a)	National Spectrum Management, version 1995

b) 	Spectrum Monitoring, version 1995

c)	Computer-aided Techniques for Spectrum Management, version 1999?

d)	HF Broadcasting System Design, version 1999?

e)	Report SM.2012, Economic Aspects of Spectrum Management, version 1997

f)	Windows Basic Automated Spectrum Management System (WinBASMS) Software Version 1997, Manual Version 1997

### The following list of requests for assistance in spectrum management is part of contributions to the Joint Group from two members representing Cameroon and Côte d’Ivoire.

# I	HISTORICAL OVERVIEW

In most developing countries, especially in sub-Saharan Africa, up until the nineties, user networks used twin copper wires or coaxial cables for large capacity or multimedia signals.

Telecommunications networks were operated by the State, which also regulated the sector, with the results we all know.

During the first decade of the nineties, some government bodies operated analogue or digital mobile telephone networks reserved for the rich and a select few. That was the period of shared networks and fledgling cellular networks (analogue and digital).

The State was more concerned with exploiting a moribund and increasingly over subsidised telephone network than managing frequencies. This spectrum management was a sideline in a monopoly situation wherein the State itself was the operator and could thus use its sovereign powers to impose a particular mode of conduct on all stakeholders.

For security reasons, or in order to “keep up with the Joneses”, light equipment had been installed to carry out surveillance on HF and occasionally on VHF.

# II	THE CURRENT SITUATION IN SPECTRUM MANAGEMENT

A)	Institutional and legal framework

The union between computers, broadcasting and telecommunications and the coming of global telecommunications infrastructure have fostered not only the emergence of new services which require more and more of the spectrum to function, but also a harmonisation of management methods to ensure network interconnection in the three regions of ITU-R.

Most of these countries, especially the developing countries, have undertaken broad based reforms of their respective telecommunications sectors. These reforms mainly consist of:

- enacting framework laws on telecommunications, which aim mostly at setting up new institutions characterised by the separation of the functions of regulation and telecommunications operation;

- the creation of statutory and regulatory bodies;

- opening the sector to private initiative and capital. The arrival of the private sector, an inevitable factor of fund mobilisation, has accelerated the introduction of new frequency-hungry management technologies;

- greater attention paid to spectrum management activities which remain within the purview of the State, but whose daily application is given over either to an autonomous body or to the structure in charge of legislation and/or regulation in the telecommunications sector.

With regard to spectrum in particular, the profound reforms undertaken mostly within the past five years aimed essentially at:

- improving the quality of service offered to the various clients in an environment characterised by the introduction of competition into certain areas of the telecommunications market, and especially in the mobile telephone market;

- ensuring the effective and efficient introduction of new services and a continuing search for fair treatment of the State, public operators, private users, etc…

Although the institutional framework has been set up, several obstacles, due mainly to the fledgling structures, still hamper the effective exploitation of the frequency spectrum.

# B	Major obstacles to spectrum management

  - Inexperience within the structures set up and the lack or inadequacy of well-trained and experienced human resources

Some government bodies or specific spectrum management entities set up following reforms in several areas quickly came up against new obstacles due to technological changes which engender new requests for bands in high demand.

The devolution of personnel following the setting up of the institutions arising from the various reforms has not yet had an impact on personnel with spectrum management skills.

This obvious lack of expertise in the resolution of technical, economic or legal problems associated with spectrum management has as its corollary the initiative by management of organising internal training and retraining programmes and seeking other avenues for co-operation with more experienced structures so as to bring the staff in charge of such matters quickly up to standard.

  - Absence of implementing instruments for laws

The recent nature of the reforms undertaken has not given most government bodies the time required to finalise the drafting of implementing instruments for the laws enacted or to complete the setting up of the institutions essential for the smooth functioning of spectrum management (for example, in Cameroon, the structure in charge of spectrum management is not functional).

  - Lack of an adequate regional information and coordination structure in Africa.

Spectrum management is not learned in conventional telecommunications training establishments. Radiocommunication knows no administrative borders. The absence of a structure to co-ordinate national spectrum management bodies and the absence of an official information transmission framework means that the specific problems of African countries are not often taken into account in the appropriate fora.

Most systems, products and applications used were developed for markets and environments not suited to the actual needs of developing countries.

  - Lack of financial resources at local level

The miniaturization of equipment and the complexity of signals transmitted mean that administrations must acquire interactive spectrum monitoring and management systems whose cost is not always within reach of the African bodies concerned. This lack of technical equipment is a major obstacle to the resolution of the rampant piracy observed in our respective administrations.

If it is easy to note the considerable influx of foreign capital in the setting up of infrastructure in the telecommunications sector within the framework of the liberalisation and privatisation already begun, it is no less obvious that the problem of financing for spectrum management entities still remains to be solved. Is this because of their status of government structures which are by definition subject to the requirements of public administrative establishments?

It is obvious that bilateral and multilateral aid should help solve this problem of financing projects for spectrum management. The ITU and Regional Organisations should, in this respect, seek out mechanisms that could aid the search for financing.

  - Level of training, information and education of spectrum users

Spectrum management requires a knowledge of the statutory and regulatory provisions which are not always known to all stakeholders (operators and licensees). The level of access to information and the lack of skill and interest on the part of the population to assimilate and comply with the body of legislation in force jeopardise the smooth development and the efficient use of the spectrum. Licensees, customs officers, operators and direct and indirect consumers of radio communication services should be conversant with the regulations so as to avoid interference of any kind. The regular education and information of frequency users must be a continuous effort on the part of frequency managers.

  - Absence or inadequacy of spectrum monitoring equipment

There is practically no automatic and interactive frequency monitoring system in our administrations. Equipment has been acquired by some countries to carry out, manually and sporadically, monitoring of HF and/or V-UHF bands. These systems are not flexible and have proven to be ineffectual in fighting piracy because of the slow pace of mission accomplishment due essentially to delays in the deployment and processing of information.

III)	ASSISTANCE REQUIRED

In order to avoid the major obstacles cited above, administrations must implement a certain number of internal and external provisions.

Although the internal measures, that is those which obviously depend on the willingness of government to attach particular importance to this activity, can be implemented as quickly as possible, the development of this activity in sub-Saharan Africa cannot be done without external technical and scientific assistance. This assistance can be provide in the following fields:

- Assistance to raise the awareness of policy-makers: Spectrum management has always been marginalized in our countries because it was not a priority for our administrations for obvious reasons. The State, as the main operator in a monopoly, made no financial gains from the use of its networks. With the restructuring of the sector, the coming of regulated competition and the high demand on the part of operators, this becomes a not insignificant source of revenue for States, if only they are aware of it and provide the financial means necessary for its development. The ITU and international donors (World Bank, International Monetary Fund) should play an essential role in this operation by organising seminars specifically for policy-makers.

- Training: National spectrum management is by definition secondary to the provisions of the Radio communications Regulations (RR), regional and sub-regional agreements of which our administrations are signatories, as well as national regulations which should be as binding as Radio communications Regulations (RR). Its implementation requires a profound knowledge of the said technical and statutory provisions, some of which are constantly changing, especially the ITU-R recommendations. Spectrum managers need to acquire the necessary skill in order to carry out these duties. The ITU is already organises seminars for government bodies on spectrum management, but these are not always adapted to the actual situation of the environment in which the government entities in charge of spectrum management have to operate. Such training should concern:

  - spectrum management in general;

  - concrete aspects concerning the setting up of a spectrum management structure in its real environment;

  - spectrum planning and strategies for introducing new services;

  - management of radio electric sites and rights of way.

3.	Assistance for the drawing up of national frequency allocation tables

These tables, which are the basis of spectrum management, must specify the services provided as well as their category of use. The provision to government administrations of information available in other countries is a fundamental element in the drawing up of theses tables.

4.	Assistance in setting up a spectrum monitoring system

It is universally acknowledged that automatic spectrum management and monitoring systems which are available on the market enable the simplification and easing of various spectrum management routines. These different systems, however, need to be adapted to the local context. To do so, some government administrations need assistance to identify their priorities as well as the programmes necessary for the acquisition of these systems.

The ITU should continue to develop projects such as WINBASMS which enable government administrations to familiarize themselves with the management of databases peculiar to the radio frequency spectrum. Special attention should be paid to regional projects such as the African network for broadcast monitoring.

5.	Assistance for the preparation of World Radio communication Conferences(WRCs)

During WRCs, which are held every 2½ years in ITU-R, government bodies come to a certain a number of decisions whose implementation requires the mobilisation of resources to a certain degree.

A mechanism to evaluate the implementation of these decisions should be designed and set up with the support of ITU and regional organisations so as to help government bodies to disseminate and implement all these decisions at national and sub-regional level.

The same strategy should also be employed when preparing for future conferences so that a consensus can be reached on urgent matters and adopt a common position for governments of the region.

It is therefore imperative that the ITU and the African Telecommunication Union (ATU) become more involved in the creation, promotion and the running of sub-regional and regional structures which prepare various WRCs so that the synergy indispensable for the presentation of joint proposals, thus guarantees that African peculiarities will be taken care of.

6.	Participation in study groups

Study groups are structures essential to the drafting of recommendations which commit the whole radio communications community. Participation in their deliberations is important, if not indispensable, to the taking into account of the concerns of developing countries. It is therefore essential to devise a strategy as quickly as possible so that SMEs and LDCs can contribute and participate effectively. This can be done with the support of the relevant ITU structures, by:

- Sub-regional co-ordination of government bodies which will take into account all opinions;

- The creation of a sub-regional network of spectrum managers which will be run by these structures, and the appointment of national co-ordinators for each of these questions.

7.	Disseminating certain ITU-R recommendations

Good spectrum management begins with a complete mastery of the instruments governing the sector and especially ITU norms and recommendations. A mechanism must be set up so that they are at the effective disposal of the main users. For this to occur, a certain number of centres using this information must be identified (appropriate ITU Bureaux, libraries of bodies in charge of spectrum management, etc.). The ITU Bureaux concerned have the duty to aid the creation of a reliable database of spectrum managers in their specialties. This can be done by a poll or a questionnaire and a mechanism to carry out daily updates of the information collected. The aim here is not necessarily to provide recommendations and related documents free of charge to these structures, but to provide them with more information on the publication of these tools which are indispensable to efficient spectrum management.

IV	EXAMPLE OF SPECIFIC NEEDS IN ONE COUNTRY: COTE D'IVOIRE

### Although intended to be specific to the needs of Cote d’Ivoire, it provides a representative collection of the requests made in the responses to the Questionnaire.

1.	Assistance in respect of computerization of spectrum management

- Establishment of a centralized database for administrative and technical spectrum management;

- Online notification of frequency assignments (installation of software and operating procedures);

- Technical analysis of frequency assignment requests using specialized software;

- Establishment of a database of radio sites (earth stations, radio-relay stations, broadcasting transmitters, etc.).

2.	Assistance in respect of management and technical monitoring of the spectrum

- Technical analyses of radio frequency interference complaints;

- Planning techniques for broadcasting networks and broadcasting modulation systems;

- Monitoring procedures for aeronautical stations and emissions;

- Radar monitoring techniques;

- Electromagnetic compatibility (EMC) analyses;

- Spectrum planning and engineering;

- Method for developing national technical standards;

- Practical methods for determining the coverage area of an earth station;

- Frequency and satellite orbital position management procedures.

The required assistance concerns both BR and BDT.

- For BR, it entails receiving engineers and technicians from Côte d’Ivoire in its services for a period to be determined by mutual agreement with the Côte d’Ivoire Administration;

- For BDT, it entails making an expert available to the country in the framework of technical cooperation.

In their current form, BR seminars do not meet this need for assistance.

V	CONCLUSIONS

Nowadays African countries are in dire need of the transfer of knowledge concerning spectrum management. Thus it is obviously an advantage to capitalize on experience in order to rework co-operation in this area.

In this context, support from the ITU and regional organisations is essential for:

- Creating an African network of frequency managers, if possible selected on the basis of their expertise and their actual place of work;

- Organising training seminars for frequency managers on matters peculiar to their respective sub-regions;

- Implementing WRC decisions and creating an African structure for the preparation of WRCs;

- Creating international synergy (government administrations, donors), for the financing of projects for the application of automatic spectrum management and monitoring systems;

- Disseminating recommendations and norms amongst the previously identified structures;

- Implementing spectrum planning strategies and introducing new services;

- The economic aspects of spectrum management and tarification;

- Managing radio electric sites and rights of way;

- Seeking out and facilitating national and regional expertise.

The implementation of these different programmes and projects can only be done through a proactive approach. In order to ensure the active participation of African countries in specific spectrum management activities, it is essential to encourage a strategy of permanent interface with a view to adopting common positions for all matters. Knowledge is acquired through basic and practical training. Spectrum management should be taught in schools as a basic subject in order to inculcate into students the fundamentals of this subject and derive the basic knowledge necessary for the future management of this limited natural resource. The ITU has an important role to play: it should send experts as teachers to training schools in Africa.

Lastly, it would not be out of place here to examine the possibility of creating the base of an African industry of radio communications products and/or services as well as an African standardisation structure. It is essential in this regard that the network of African Frequency managers be effective and that a prior harmonisation of national regulations be carried out.

| # Country | Q1: National Law | Q2: Nat Regulations | Q3: Allocation Table | Q4: Tech. Specifications | Q8: Central Database | Q9: ITU Notification | Q10: Spectrum Strategy | Q11: Analyses-Assignments | Q12: Monitoring | Q13: Analysis-Interference | Q14: IT systems |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Algeria | - |  |  |  |  |  |  |  |  |  |  |
| Armenia |  |  |  |  |  | Y |  |  | Y | Y |  |
| Austria | - |  |  |  |  |  |  |  |  |  |  |
| Bangladesh |  | Y | Y |  |  | Y | Y | Y |  | Y |  |
| Belarus | - |  |  |  |  |  |  |  |  |  |  |
| Belgium | - |  |  |  |  |  |  |  |  |  |  |
| Belize | Y | Y |  | Y | Y | Y |  | Y | Y | Y |  |
| Bolivia |  |  |  |  |  |  | Y | Y | Y |  |  |
| Brazil |  |  |  |  |  | Y |  |  |  |  |  |
| Bulgaria |  |  |  |  | Y | Y |  |  |  |  |  |
| Burkina Faso | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Burundi | Y |  |  |  | Y |  | Y |  |  |  |  |
| Cameroon |  |  |  |  | Y |  | Y |  | Y |  | Y |
| Canada | - |  |  |  |  |  |  |  |  |  |  |
| Chad | Y | Y | Y |  |  |  |  |  |  |  |  |
| China | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |  |
| Colombia |  |  |  |  |  | Y |  |  |  |  |  |
| Côte d’Ivoire |  |  |  |  |  |  |  |  |  |  |  |
| Croatia | - |  |  |  |  |  |  |  |  |  |  |
| Cyprus | Y | Y | Y | Y |  |  |  | Y |  |  |  |
| Czech Republic |  |  |  |  |  | Y |  |  |  |  |  |
| Denmark | - |  |  |  |  |  |  |  |  |  |  |
| Democratic People's Republic of Korea |  |  |  |  | Y | Y | Y | Y | Y | Y | Y |
| Egypt | - |  |  |  |  |  |  |  |  |  |  |
| Eritrea |  | Y | Y | Y | Y |  | Y | Y |  |  | Y |
| Estonia |  |  |  |  |  |  |  |  |  |  |  |
| Ethiopia | Y | Y | Y | Y | Y |  | Y | Y |  | Y |  |
| Finland | - |  |  |  |  |  |  |  |  |  |  |
| France | - |  |  |  |  |  |  |  |  |  |  |
| Gabon | - |  |  |  |  |  |  |  |  |  |  |
| Georgia |  | Y |  | Y | Y | Y | Y | Y |  |  |  |
| Germany | - |  |  |  |  |  |  |  |  |  |  |
| Greece | Y |  |  |  |  | Y |  |  |  |  |  |
| Guyana | Y | Y | Y | Y |  | Y | Y | Y | Y | Y | Y |
| Hungary | - |  |  |  |  |  |  |  |  |  |  |
| Indonesia | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Iran | Y | Y |  | Y | Y |  | Y | Y |  | Y | Y |
| Japan | - |  |  |  |  |  |  |  |  |  |  |
| Rep. of Korea | - |  |  |  |  |  |  |  |  |  |  |
| Lao |  |  | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Latvia | - |  |  |  |  |  |  |  |  |  |  |
| Lesotho |  |  |  |  | Y |  | Y |  | Y |  | Y |
| Liberia | Y |  | Y |  | Y | Y | Y | Y |  | Y |  |
| Libya | - |  |  |  |  |  |  |  |  |  |  |
| Lithuania |  |  |  |  |  | Y |  |  |  |  |  |
| Madagasgar | - |  |  |  |  |  |  |  |  |  |  |
| Maldives | Y |  |  | Y | Y | Y |  |  | Y |  | Y |
| Mali |  | Y | Y | Y |  | Y | Y | Y | Y | Y | Y |
| Malta |  |  | Y |  |  |  |  |  |  |  |  |
| Mexico | ? |  |  |  |  |  |  |  |  |  |  |
| Myanmar | Y | Y | Y | Y | Y |  |  |  |  |  |  |
| Namibia | Y |  | Y |  |  | Y |  |  |  |  |  |
| Nicaragua | ? |  |  |  |  |  |  |  |  |  |  |
| Norway | - |  |  |  |  |  |  |  |  |  |  |
| Pakistan |  | Y |  | Y |  |  |  | Y |  |  |  |
| Peru |  |  |  |  |  |  | Y |  |  |  | Y |
| Portugal | - |  |  |  |  |  |  |  |  |  |  |
| Samoa |  |  |  |  |  |  |  |  |  |  |  |
| Seychelles |  |  |  |  |  | Y |  | Y | Y | Y |  |
| Slovakia |  |  |  |  |  |  |  |  |  |  |  |
| Spain |  |  |  |  |  | Y |  |  |  |  |  |
| Suriname |  |  |  |  |  |  |  |  |  |  |  |
| Sweden | - |  |  |  |  |  |  |  |  |  |  |
| Switzerland | - |  |  |  |  |  |  |  |  |  |  |
| Thailand | - |  |  |  |  |  |  |  |  |  |  |
| Turkey | - |  |  |  |  |  |  |  |  |  |  |
| United Arab Emirates |  |  |  |  |  |  |  | Y | Y | Y | Y |
| United Kingdom | - |  |  |  |  |  |  |  |  |  |  |
| United States | - |  |  |  |  |  |  |  |  |  |  |
| Venezuela | ? |  |  |  |  |  |  |  |  |  |  |
| Vietnam | Y | Y |  | Y |  |  | Y | Y |  |  |  |

Table 4-1

Classification of ITU Member States

Listing Provided by the ITU-D Secretariat

| COUNTRY | DEVELOPED COUNTRY | DEVELOPING COUNTRY | LEAST DEVELOPED COUNTRY |
| --- | --- | --- | --- |
| Afghanistan |  |  | X |
| Albania |  | X |  |
| Algeria |  | X |  |
| Andorra | X |  |  |
| Angola |  |  | X |
| Antigua & Barbuda |  | X |  |
| Argentina |  | X |  |
| Armenia |  | X |  |
| Australia | X |  |  |
| Austria | X |  |  |
| Azerbaijan |  | X |  |
| Bahamas |  | X |  |
| Bahrain |  | X |  |
| Bangladesh |  |  | X |
| Barbados |  | X |  |
| Belarus | X |  |  |
| Belgium | X |  |  |
| Belize |  | X |  |
| Benin |  |  | X |
| Bhutan |  |  | X |
| Bolivia |  | X |  |
| Bosnia and Herzegovina |  | X |  |
| Botswana |  | X |  |
| Brazil |  | X |  |
| Brunei Darussalam |  | X |  |
| Bulgaria |  | X |  |
| Burkina Faso |  |  | X |
| Burundi |  |  | X |
| Cambodia |  |  | X |
| Cameroon |  | X |  |
| Canada | X |  |  |
| Cape Verde |  |  | X |
| Central African Rep. |  |  | X |
| Chad |  |  | X |
| Chile |  | X |  |
| China |  | X |  |
| Colombia |  | X |  |
| Comoros |  |  | X |
| Congo |  | X |  |
| Costa Rica |  | X |  |
| Côte d'Ivoire |  | X |  |
| Croatia |  | X |  |
| Cuba |  | X |  |
| Cyprus |  | X |  |
| Czech Rep. |  | X |  |
| Dem. Rep. of the Congo |  |  | X |
| Denmark | X |  |  |
| Djibouti |  |  | X |
| Dominica |  | X |  |

| COUNTRY | DEVELOPED COUNTRY | DEVELOPING COUNTRY | LEAST DEVELOPED COUNTRY |
| --- | --- | --- | --- |
| Dominican Rep. |  | X |  |
| Ecuador |  | X |  |
| Egypt |  | X |  |
| El Salvador |  | X |  |
| Equatorial Guinea |  |  | X |
| Eritrea |  |  | X |
| Estonia |  | X |  |
| Ethiopia |  |  | X |
| Fiji |  | X |  |
| Finland | X |  |  |
| France | X |  |  |
| Gabon |  | X |  |
| Gambia |  |  | X |
| Georgia |  | X |  |
| Germany | X |  |  |
| Ghana |  | X |  |
| Greece | X |  |  |
| Grenada |  | X |  |
| Guatemala |  | X |  |
| Guinea |  |  | X |
| Guinea-Bissau |  |  | X |
| Guyana |  | X |  |
| Haiti |  |  | X |
| Honduras |  | X |  |
| Hungary |  | X |  |
| Iceland | X |  |  |
| India |  | X |  |
| Indonesia |  | X |  |
| Iran (Islamic Republic of) |  | X |  |
| Iraq |  | X |  |
| Ireland | X |  |  |
| Israel | X |  |  |
| Italy | X |  |  |
| Jamaica |  | X |  |
| Japan | X |  |  |
| Jordan |  | X |  |
| Kazakhstan |  | X |  |
| Kenya |  | X |  |
| Kiribati |  |  | X |
| Korea (Dem. People R) |  | X |  |
| Korea (Rep. of) |  | X |  |
| Kuwait |  | X |  |
| Kyrgyzstan |  | X |  |
| Lao P.D.R. |  |  | X |
| Latvia |  | X |  |
| Lebanon |  | X |  |
| Lesotho |  |  | X |
| Liberia |  |  | X |
| Libya |  | X |  |
| Liechtenstein | X |  |  |
| Lithuania |  | X |  |
| Luxembourg | X |  |  |

| COUNTRY | DEVELOPED COUNTRY | DEVELOPING COUNTRY | LEAST DEVELOPED COUNTRY |
| --- | --- | --- | --- |
| Madagascar |  |  | X |
| Malawi |  |  | X |
| Malaysia |  | X |  |
| Maldives |  |  | X |
| Mali |  |  | X |
| Malta |  | X |  |
| Marshall Islands |  | X |  |
| Mauritania |  |  | X |
| Mauritius |  | X |  |
| Mexico |  | X |  |
| Micronesia |  | X |  |
| Moldova |  | X |  |
| Monaco | X |  |  |
| Mongolia |  | X |  |
| Morocco |  | X |  |
| Mozambique |  |  | X |
| Myanmar |  |  | X |
| Namibia |  | X |  |
| Nauru |  | X |  |
| Nepal |  |  | X |
| Netherlands | X |  |  |
| New Zealand | X |  |  |
| Nicaragua |  | X |  |
| Niger |  |  | X |
| Nigeria |  | X |  |
| Norway | X |  |  |
| Oman |  | X |  |
| Pakistan |  | X |  |
| Panama |  | X |  |
| Papua New Guinea |  | X |  |
| Paraguay |  | X |  |
| Peru |  | X |  |
| Philippines |  | X |  |
| Poland |  | X |  |
| Portugal |  | X |  |
| Qatar |  | X |  |
| Romania |  | X |  |
| Russia | X |  |  |
| Rwanda |  |  | X |
| Saint Lucia |  | X |  |
| Saint Vincent and the Grenadines |  | X |  |
| Samoa |  |  | X |
| San Marino |  | X |  |
| Sao Tome and Principe |  |  | X |
| Saudi Arabia |  | X |  |
| Senegal |  |  | X |
| Seychelles |  | X |  |
| Sierra Leone |  |  | X |
| Singapore | X |  |  |
| Slovakia |  | X |  |
| Slovenia |  | X |  |
| Solomon |  |  | X |
| Somalia |  |  | X |

| COUNTRY | DEVELOPED COUNTRY | DEVELOPING COUNTRY | LEAST DEVELOPED COUNTRY |
| --- | --- | --- | --- |
| South Africa |  | X |  |
| Spain | X |  |  |
| Sri Lanka |  | X |  |
| Sudan |  |  | X |
| Suriname |  | X |  |
| Swaziland |  | X |  |
| Sweden | X |  |  |
| Switzerland | X |  |  |
| Syria |  | X |  |
| Tajikistan |  | X |  |
| Tanzania |  |  | X |
| Thailand |  | X |  |
| The Former Yugoslav Republic of Macedonia |  | X |  |
| Togo |  |  | X |
| Tonga |  | X |  |
| Trinidad andTobago |  | X |  |
| Tunisia |  | X |  |
| Turkey |  | X |  |
| Turkmenistan |  | X |  |
| Tuvalu |  |  | X |
| Uganda |  |  | X |
| Ukraine | X |  |  |
| United Arab Emirates |  | X |  |
| United Kingdom | X |  |  |
| United States | X |  |  |
| Uruguay |  | X |  |
| Uzbekistan |  | X |  |
| Vanuatu |  |  | X |
| Vatican | X |  |  |
| Venezuela |  | X |  |
| Viet Nam |  | X |  |
| Yemen |  |  | X |
| Yugoslavia |  | X |  |
| Zambia |  |  | X |
| Zimbabwe |  | X |  |

Table 4-2: Statistical distribution of responses used in the analyses

| Region | Developed Countries | Developing Countries | Least Developed Countries | Total | % of response |
| --- | --- | --- | --- | --- | --- |
| Africa | - | 5 | 9 | 14 | 20 |
| Americas | 2 | 10 | - | 12 | 17 |
| Asia and Pacific | 1 | 8 | 5 | 14 | 20 |
| Europe and CIS | 13 | 14 | - | 27 | 38 |
| Arab States | - | 4 | - | 4 | 5 |
| TOTAL | 16 | 41 | 14 | 71 | 100 |

Taking into account that there are 189 Members States of the ITU, the ratio of responses received for each  region can be established :

Africa	:	30%

Americas	:	34%

Asia and Pacific	:	37%

Europe and CIS	:	49%

Arab States	:	21%

(Note: these are the focal points for information relevant only to the WTDC Resolution 9 questionnaire. The ITU Global Directory provides the official contact information for ITU Member States)

| # SUBMITTING ADMINISTRATION or ORGANIZATION ITU Code/Region/Source Doc. | RESPONSE INCLUDES Part I | RESPONSE INCLUDES Part II | FOCAL POINTS DESIGNATED BY THE SUBMITTERS | Formats used- Word-- *.doc Spreadsheet-- *.xls Scanned-- *.pdf Power Point-- *.ppt Image-- *.gif Publisher-- *.pub Compressed-- *.zip | Language of submission Phone # Fax # Email address |
| --- | --- | --- | --- | --- | --- |

| Algeria ALG/1/1 | yes | yes | Houria Khenchelaoui Sous-Directeur de la Reglementation des Services Radio Ministere des P et T L1, Bol KRIM BELKACEM, Alger, Algeria | *.pdf | Submission in French Ph: 271 1220 Fx: 272 4957 |
| --- | --- | --- | --- | --- | --- |
| Argentina ARG/2/2 | yes | no | Roberto Eduardo Perez Director Ejecutiveo de Relaciones Externas e Iniciativas Estrategicas Compania de Radiocomunicaciones Moviles S.A. Tucuman 744 Piso 2 – (1049) Buenos Aires, Argentina | *.pdf | Submission in Spanish rree@movi.com.ar Ph: 54 11 4321 5006 Fx: 54 11 4978 7373 rree@movi.com.ar |
| Armenia ARM/1/3 | yes | yes | Arthur Andreassyan Administrative Director Armenia/Republican Centre of Telecommunications SCJSC Ministry of Posts and Telecommunications 2, Mkhitar Heratsi Str., Yerevan, 375025 Republic of Armenia | *.doc | Submission in English Ph: 3742 52 79 22 Fx: 3742 52 60 32 |
| Austria AUS/3/4 | no | yes | Gerd Lettner Federal Ministry for Science and Transport P.O. Box 127, A-1103 Vienna, Austria | *.pdf | Submission in English Gerd.lettner@bmv.gv.ost Ph: 43 17 9731 4200 Fx: 43 17 9731 4209 |
| Bangladesh BGD/3/5 | yes | yes | S.M. Zabed Robbani Divisional Engineer (Frequency and Wireless) Ministry of Posts and Telecommunication Frequency and Wireless Board Telecommunication Training Center Tejgaon, Dhaka 1208 Bangladesh | *.doc | Submission in English zrobbani@hotmail.com Ph: 880 2 9883181 Fx: 880 2 988 4299 |
| Belarus BLR/1/6 | no | yes | Anatoly Budai First Deputy Minister Ministry of Posts and Telecommunications 10 Skoryna Ave. 220050 Minsk, Republic of Belarus | *.pdf | Submission in English SRFC@mpt.gov.by Ph: 375 172 272 526 Fx: 375 172 260 848 |

| Belgium BEL/1/7 | no | yes | Michael Vanroogenbroek Ingenieur – Conseiller IBPT Ave. de l’ Astrononmie 14 Bte 21 B-1210 Bruxelles, Belgique | *.pdf | Submission is in French michael.vandroogenbroek@ibpt.be Ph: +32 2 226 8878 Fx: +32 2 226 8882 |
| --- | --- | --- | --- | --- | --- |
| Belize BLZ/2/8 | yes | yes | Clifford M. Slusher Director Office of Telecommunication P.O.Box 310 Belize City, Belize | *.pdf | Submission in English Ph: 501 2 24938 Fx: 501 2 24939 |
| Bolivia BOL/2/9 | yes | yes | Jose Alfredo Arce Jofre Director General de Comunicaciones Direccion General de Comunicaciones Avenida Mariscal Santa Cruz, Esquina Oruro, Edificio Palacio de Comunicaciones, Piso 4 Bolivia | *.doc *.xls | Submission in Spanish mtccom@caoba.entelnet.bo Ph: 591 2 378129 Fx: 591 2 371360 |
| Brazil B/2/10 | yes | yes | Haroldo Motta Engineer Agencia Nacional de Telecommnicacoes – ANATEL SAS – Quadra 6 Bloco H, 40 Andar Brazilia – DF Brazil 70 313-900 | *.zip *.doc | Submission in Portuguese and in English haroldo@anatel.gov.br Ph: 55 61 312 2373 Fx: 55 61 312 2328 |

| Bulgaria BUL/1/11 | yes | yes | Nikola Mantchev Committee of Posts and Telecommunications 6 Gourko St. Sofia 1000, Bulgaria --------- Grigor Grigorov Director of Frequency Management Directorate State Telecommunications Commission 6 Gourko St. Sofia, 1000, Bulgaria | *.doc | Submission in English nmanchev@cpt.bg Ph: 359 2 949 2606 Fx: 359 2 987 9540 ---------- spectrum@acpt.bg Ph: 359 2 949 2327 Fx: 359 2 949 2198 |
| --- | --- | --- | --- | --- | --- |
| Burkino Faso BFA/1/12 | yes | yes | Pousbilo Ouedraogo Chef Service Gestion de Frequences Office National des Telecommunications ONATEL BP 10 000 Ouagadougou 01 Burkina Faso | *.pdf | Submission in French pousbilo.ouedraogo@ties.itu.int Ph: 226 318721 Fx: 226 310331 |
| Burundi BDI/1/13 | yes | yes | Constaque Hakizimana Chef de Service Technique A.R.C.T Bujumbino Avenue Bururi No. 5 BP 6702 Burundi | *.pdf | Submission in French and provides some specific assignments Ph: 257 210276 Fx: 257 210269 |

| Cameroon CME/1/71 | yes | yes | SIMPLICE ZANGA YENE Chef de la Cellule des Etudes et de la Planification du Spectre à l’Agence de Régulation des Télécommunication du Cameroun BP 6132 ART Yaoundé République du Cameroun | *.doc | Submission in French Ph: (237) 230 380 Fx : (237) 233 748 Simplice.zanga@Camnet.cm |
| --- | --- | --- | --- | --- | --- |
| Canada CAN/2/14 | yes | yes | Thomas Racine Director, Automated Spectrum Management Systems Industry Canada 300 Slater Street, Room B1240 Ottawa, ON K1A 0C8 Canada | *.doc | Submission in English Ph: 1 613 990 4767 Fx: 1 613 990 7287 racine.thomas@ic.gc.ca Also see: http://strategis.ic.gc.ca/ssg/sf01608e.html |
| CEPT/ERC __/1/15 | yes | no | Reiner Liebler Chairman ERC WG FM Regulatory Authority for Telecommunications and Posts P.O. Box 80 01 D – 55003 Mainz Germany | *.pdf *.doc | The submission is an extract of ERC Report 25 reiner.liebler@regtp.de Ph: +49 6131 18 3110 Fx: +49 6131 18 5604 |
| Chad TCD/1/16 | no | yes | Guirdonan Mogalbaye Le Coordonnateur Office National des Posts et Telecommunications | *.pdf | Submission in French Fx: 41 22 730 58 06 Fx: 41 22 730 54 84 |

| China CHN/3/17 | yes | yes | Yuansheng Xie Director Radio Regulatory Department 13 West Chang An Ave. Beijing 100804 Peoples Republic of China | *.pdf | Submission in English Ph: 86 10 6834 7623 Fx: 86-10-6836 6494 |
| --- | --- | --- | --- | --- | --- |
| Columbia CLM/2/18 | yes | yes | Néstor Alonso Jiménez Estrada Profesional Especializado Ministerio de Comunicaciones Carreras 7 y 8 Calles 12ª y 13, Edificio Murillo Toro, Piso 4 Columbia | *.doc | Submission in Spanish intermin@mincomunicaciones.gov.co Ph: 5713442357 Fx: 5713442206 |
| Cote D’Ivoire CTI/1/19 | yes | yes | Le Directeur General Agence des Telecommunications de Cote d’Ivoire Immeuble Postel 2001 Rue Lecoeur- Plateur 18 BP 2203 Abidjan 18 Cote d’ Ivoire | *.pdf | Submission in French Ph: 20 34 42 56 or 20 34 42 55 Fx: 20 34 42 58 |
| Croatia HRV/1/20 | yes | yes | DOMINIK FILIPOVIĆ Director Croatian Institute of Telecommunications Prisavlje 14, HR-10000 Zagreb Croatia | *.doc | Submission in English dominik.filipovic@telekom.hr Ph: + 385 1 616 91 10 Fx: + 385 1 619 66 62 |
| Cyprus CYP/1/21 | no | yes | Andronicos Kakkouras Telecommunications Officer Ministry of Communications and Works Directorate of Telecommunications CY-1424 Lefkosia (Nicosia), Cyprus | *.pdf | Submission in English kakkouras@mcw.gov.cy Ph: +357 2 800206 Fx: +357 2 776254 |
| Czech Republic CZE/1/22 | yes | yes | Ludmila HALOUSKOVÁ Department of Frequency Spectrum Management Ministry of Transport and Communications-Czech Telecommunication Office Klimentská 27, CZ-225 02 Praha 1, Czech Republic | .doc .xls | Submission in English halouskoval@ctu.cz Ph: +420 2 24006 666 Fx: +420 2 2491 1658 |
| Denmark DNK/1/23 | yes | yes | Claus B. Mortensen Master of Law/Economics Officer The National Telecom Agency Holsteinsgade 63, DK-2100 Copenhagen, DENMARK | *.doc | Submission in English cbm@tst.dk Ph: +45 35 45 02 31 Fx: +45 35 45 00 16 |
| Egypt EGY/1/24 | no | yes | No contact specified | *.pdf | Submission in English |
| Eritrea ERI/1/25 | yes | yes | Mogos Zerai Teklehaimanot Director, Standards and Regulatory Division Ministry of Transport and Communications Department of Communications Harnet Street P.O.Box 4918 Asmara, Eritrea | *.pdf | Submission in English Ph: 29 11 126 965 Fx: 29 11 126 966 |
| Estonia EST/1/26 | yes | no | Arold Julia Head of Radiocommunication Development Office Technical Planning Department Estonian National Communications Board Adala 4d 10614 Tallinn Estonia | *.pdf | Submission in English arold@sa.ee Ph: +372 693 1163 Fx: +372 693 1155 |
| Ethiopia ETH/1/72 | yes | yes | GETACHEW KEBEDE HEAD OF FREQUENCY MANAGEMENT DIVISION ETHIOPIAN TELECOMMUNICATION AGENCY ETHIOPIAN TELECOMMUNICATION AGENCY P.O. BOX 9991 ADDIS ABABA ETHIOPIAN (FEDERAL DEMOCRATIC REPUBLIC OF ETHIOPIA) | *.doc | Submission in English Ph: 251-1-51 5766 Fx: 251-1-551253 tele.agency@telecom.net.et |
| Finland FIN/1/27 | yes | yes | Margit Huhtala Head of Frequency Division Telecommunications Administration Centre P.O. Box 313 FIN-00181 Helsinki Finland | *.pdf | Submission in English Refers to CEPT/ERC/WGFM European Allocation Table (ERC Report 25) info@thk.fi margit.huhtala@thk.fi Ph: +358 9 6966 425 Fx: +358 9 6966 410 http://www.thk.fi |
| France F/1,2,3/28 | no | yes | Ms. Béatrice DURAND ITU Affairs Agence Nationale des Fréquences BP 400 F- 94704 MAISONS ALFORT CEDEX FRANCE | *.pdf | Submission in French durand@anfr.fr Ph: +33 1 45 18 72 15 Fx: +33 1 45 18 73 13 |

| Gabon GAB/1/29 | Yes | Yes | Louis Nkogiie Ndong Chef de Division des Relations Techniques Internationales Office des Postes et Telecommunications B.P. 20 000 Libreville Gabon | *.pdf | Submission in French 2nl@opt.ga Ph: 241 78 71 21 Fx: 241 78 71 17 |
| --- | --- | --- | --- | --- | --- |
| Georgia GEO/1/30 | yes | yes | Alexander Tsivtsivadze Director Ministry of Post and Communications, Post and Communication Supervision Service 46, Tevdore Mghvdeli St., Tbilisi, 380054, Georgia | *.doc | Submission in English alexander.tsivtsivadze@ties.itu.int Ph: +995 32 933126 Fx: +995 32 984071 |
| Germany D/1/31 | yes | yes | Bernd-Dieter Wolko Administrative Officer Regulatory Authority for Telecommunications and Posts (RegTP) Canisiusstrasse 21 D-55122 Mainz Germany | *.pdf *.doc | Submission in English Refers to CEPT/ERC/WGFM submission which includes European Allocation Table (ERC Report 25) Bernd-dieter.Wolko@regtp.de Ph: 00 49 6131 18 3111 Fx: 00 49 6131 18 5604 |
| Greece GRC/1/32 | yes | yes | Nissim Benmayior Head of Department Minstry of Transport and Communications 2, Anastaseos Str. GR-101 91 Athens Greece | *.pdf *.doc | Submission in English ymegde@hol.gr Ph: +301 650 8571 Fx: +301 650 8570 |

| Guyana GUY/2/33 | yes | yes | Valmikki Singh Engineer-Spectrum Planning /International Relations National Frequency Management Unit 68 Hadfield Street D’Urban Park Georgetown, Guyana | *.pdf *.doc | Submission in English nfmu@guyana.net.gy Ph: 592 2 63976 Fx: 592 2 67661 |
| --- | --- | --- | --- | --- | --- |
| Hungary HNG/1/34 | yes | no | Dr. Gyorgy Demendi Deputy Head of Frequency Management Office Ministry of Transport, Communications and Water Manaagement P.O. Box 87 Budapest H-1h00 Hungary | *.pdf *.Zip *.doc | Submission in English demendi@cms.khvm.hu Ph: +36 1 461 3334 Fx: +36 1 461 3392 |
| Indonesia INS/3/35 | yes | no | Denny Setiawan Director, Spectrum Policy and Planning Directorate General of Posts and Telecommunications Gedung Sapta Pesona 7th Floor JL. Medan Merdeka Barat 17 Jakarta 10110 Indonesia | *.doc and *.pdf | Submission in English denny.setiawan@ties.itu.int Ph: +62 21 383 8363 Fx: +62 21 386 7500 The file named “Freq_Table.pdf “ with passsword “Dennyz” is included in the submission. |
| Iran IRN/3/36 | no | yes | Ali Asghar Dolatabadi Directorate General of Telecommunications Islamic Republic of Iran | *.pdf | Submission in English hamed@radtel.org Fx: +98 21 867999 |

| Italy (Telespazio) __/1/38 | yes | no | Emanuele D’Andria Head of Space Segment and Regulatory Affairs Telespazio S.P.A. Via Tiburtina, 965 00156 Roma, Italy |  | Submission in English emanuele_dandria@telepzio.it Ph: +39 06 4079 3370 Fx: +39 06 4079 3722 |
| --- | --- | --- | --- | --- | --- |
| Japan J/3/39 | yes | yes | Masahiko Tominaga Director of International Frequency Policy Office Telecommunications Bureau Ministry of Posts and Telecommunications 1-3-2 Kasumigaseki, Chiyoda-Ku Tokyo 100-8798, Japan | *.pdf *.doc | Submission in English sat-fpd@mpt.go.jp Ph: +81 3 3504 4884 Fx: +81 3 5251 7650 |
| Korea, Democratic Peoples Republic KRE/3/40 | yes | yes | Pak Kwang Sik Manager, Department of Radio Regulation Ministry of Posts and Telecommunications Oesong-dong, Central District Democratic Peoples Republic of Korea | *.pdf | Submission in English Provides exceptions to international table Ph: +850 2 3813180 Fx: +850 2 3814418 |
| Korea, Republic of KOR/3/41 | yes | yes | Jeong Yeol Director, Frequency Division, Radio and Broadcasting Bureau Ministry of Information and Communication 100, Sejongno, Chongno-gu Seoul, Republic of Korea | *.pdf | Submission in English jypark@mic.go.kr Ph: +82 2 750 2440 Fx: +82 2 750 2449 |

| Latvia LVA/1/42 | no | yes | Karlis Bogens Director, Latvia Telecommunication State Inspection 41/43 Str. Elizabetes Riga LV-1010, Latvia | *.pdf | Submission in English Ph: +371 733 3034 Fx: +371 782 1275 |
| --- | --- | --- | --- | --- | --- |
| Laos LAO/3/43 | no | yes | Phanoulangsy Phimmachanh Head, Frequency Management Division Frequency Management Division, Department of Posts and Telecommunications, Ministry of Communications, Transports, Posts and Constructions. Jawaharlal Nehru Street, 0100 Vientiane Lao PDR. | *.doc | Submission in English Ph: (856-21) 412299 Fx.: (856-21) 412279 laofreqm@laotel.com depostel@laotel.com phanoulangsy@hotmail.com |
| Lesotho ( Lesotho Telecommunications Corporation) __/1/44 | yes | yes | Tennyson Saoana Engineer, Lesotho Telecommunications Corporation P.O. Box 1037 Maseru, Lesotho | *.pdf | Submission in English Ph: +266 31 2700 Fx: +266 31 0170 |
| Liberia LBR/1/45 | yes | yes | Gabriel W. Pannoh Director of Technical Services Ministry of Posts and Telecommunications Post Office Box GPO Monrovia, Liberia | *.pdf | Submission in English Ph: +231 22 6079 Fx: +231 22 3878 |

| Libya LBY/1/46 | yes | yes | Mohamed Saleh Alsabey Head, International Relations Department General Directorate of Posts and Telecommunications P.O.Box 81686 Tripoli, Libya | *.pdf | Submission in English Ph: + 21 821 605 65 Fx: + 21 821 360 4102 |
| --- | --- | --- | --- | --- | --- |
| Lithuania LTU/1/47 | yes | yes | Arturas Medeisis Head of Section, Strategic Planning Lithuanian State Radio Frequency Service SRFS, Algrdo Str. 27 LT-2006 Vilnius, Lithuania | *.pdf *.doc | Submission in English medeisis@radio.lt Ph: + 370 2 33 0825 Fx: + 370 2 26 1564 |
| Madagascar MDG/1/48 | yes | yes | Devarieux Aubertin Rasendramiadana Directeur De La Gestion Des Frequencies Office Malagasy D’Etudes et de Regulation des Telecommunications (OMERT) BP:99991-101 Antananarivo Madagascar | *.pdf | Submission in French omert@dts.mg Ph: +261 22 421 19 Fx: +261 22 422 71 |
| Maldives MLD/3/49 | yes | yes | Abdulla Rasheed Director of Engineering Post and Telecommunication Section Ministry of Communication, Science and Technology Telecom Building Husnuheena Maagu Male’ 20-04 Republic of Maldives | *.pdf | Submission in English telecom@dhivehinet.net.mv Ph: +960 32 33 44/323 444 Fx: +960 32 0000 |

| Mali MLI/1/50 | yes | yes | Cheick Oumar Traore Chef Section Radio Decametrique et Gestion des Frequences BP 740 Bamako Mali | *.pdf | Submission in French cotraore@sotelma.ml Ph: +215280 Fx: +215086 |
| --- | --- | --- | --- | --- | --- |
| Malta MLT/1/51 |  |  | Henry Mifsud CONSULTANT DEPARTMENT OF WIRELESS TELEGRAPHY EVANS BUILDING, MERCHANTS STREET, VALLETTA CMR02, MALTA | *.doc *.pub | Submission in English henry.mifsud@magnet.mt Ph: 00356 25993612 Fx: 00356 247229 |
| Mexico MEX/2/52 |  |  | Reynaldo González Bustamante Director General de Planeación y Administración del Espectro Comisión Federal de Telecomunicaciones Bosque de Radiatas No. 44, 1er. Piso, Col. Bosques de Las Lomas México | *.doc *.pdf | Submission in Spanish rglezbus@cft.gob.mx Ph: (52) 5 2614005 Fx: (52) 5 2614041 |
| Myanmar BRM/3/53 | yes | yes | Kyi Than Director General, Posts and Telecommunications Department Ministry of Communications, Posts, and Telegraphs 125 Ground Floor, Pansodan St. Yangon (RGN) Myanmar (BRN) | *.pdf | Submission in English DG.PTD@mtpt400.stems.com Ph: +95 1 280 024 Fx: +95 1 286 365 |

| Namibia NMB/1/54 | no | yes | Jan Kryger Deputy General Namibian Communications Commission Private Bag 13309 Winohoer Namibia | *.pdf | Submission in English janhk@ncc.org.na Ph: 222 666 + 264 61 Fx: 222 790 |
| --- | --- | --- | --- | --- | --- |
| Nicaragua NCG/2/55 | no | yes | Hjalmar Ruiz Tuckler Director DAER Administracion Del Espectro Radioelectrico DAER-TELCOR Edificio INSS V-VI-Piso Apartado 2664 Managua, Nicaragua | *.pdf | Submission in Spanish jvado@telcor.gob.ni Ph: 222 7348 Fx: 222 7644 |
| Norway NOR/1/56 | yes | yes | Stein Gudbjorgsrud Head of Frequency Planning Norwegian Post and Telecommunication Authority P.O. Box 447 Sentrum N-0104 OSLO Norway | *.pdf | Submission in English stein.gudbjorgsrud@npt.no Ph: +47 22 824 802 Fx: +47 22 824 790 |
| Pakistan PAK/3/57 | yes | yes | Mahboob Ali Vice Chairman, Frequency Allocation Board Ministry of Communications 19 Mauve Area, G-8/1 Islamabad, Pakistan | *.pdf *.doc | Submission in English vcfab@paknet2.ptc.pk Ph: 92 51 926 0183 Fx: 92 51 926 0185 |

| Peru PRU/2/58 | yes | yes | Edgard Alvarado Barreto Sub Director de Sistemas de Radiocomunicaciones Ministerio de Transportes, Comunicaciones Vivineda Y Construccion Av. 28 De Julio No 800 Lima, Peru | *.pdf | Submission in Spanish Ealvarado@mtc.gob.pe Ph: 511 423 0998 Fx: 511 433 1450 |
| --- | --- | --- | --- | --- | --- |
| Peru (Telefonica) __/2/59 | no | no | Patricio Muller Gerente Larga Distancia de Telefonica del Peru | *.pdf | Submission in Spanish Ph: 511 225 1248 Fx: 511 225 1202 |
| Portugal POR/1/60 | yes | yes | MARIA LUÍSA MENDES Head of Spectrum Management and Engineering Directorate ICP – Instituto das Comunicações de Portugal Avª José Malhoa , nº 12 1099-017 Lisboa Portugal | *.doc | Submission in English luisa.mendes@icp.pt Ph: +351 721 1000 Fx: +351 721 1006 |
| Samoa SMO/3/73 | yes | yes | Fong David Assistant Director Monitoring / Spectrum - Ministry of Posts and Telecommunications - Private Bag Chan Mow Plaza Apia, Samoa | *.doc | Submission in English Ph: 0685 26117 Fx: 0685 24671 Email: mpt@samoa.ws |

| Seychelles, Republic of SEY/1/61 | yes | yes | Dr. G. Ah-Thew Telecom Division Ministry of Information Technology and Communications P.O.Box 1389 Oceangate House Victoria, Republic of Seycelles | *.pdf | Submission in English telecoms@seychelles.net Ph: 248 382 039 Fx: 248 225 325 |
| --- | --- | --- | --- | --- | --- |
| Slovak Republic SVK/1/62 | yes | no | MÁRIA ALAKŠOVÁ Ing. MINISTRY OF TRANSPORT, POSTS AND TELECOMMUNICATIONS OF THE SLAVAK REPUBLIC Námestie slobody 6, 821 05 Bratislava, Slovak Republic | *.doc | Submission in English maria.alaksova@telecom.gov.sk Ph: +421 7 52731434 Fx: +421 7 52731437 |
| Spain E/1/63 | yes | yes | Juan Canas Santos Consejero Tecnico Secretaria General de Comunicaciones Palacio de Comunicaciones Pza. Cibeles s/n 28071 Madrid, Espana (Spain) | *.pdf | Submission in Spanish juan.canas@sgc.mjom.es Ph: 34 91 346 1500 Fx: 34 91 396 2229 . |
| Sweden S/1/64 | yes | yes | Hedlund Jan Spectrum Engineer National Post and Telecom Agency P.O.Box 5398 S-102 49 STOCKHOLM, Sweden | *.doc | Submission in English jan.hedlund@pts.se Ph: +46 8 678 5565 Fx: +46 8 678 5505 |

| Switzerland (OFCOM) SUI/1/65 | yes | yes | Ivan Franic El. Ing. HTL OFCOM Zukunftstrasse 44 P.O.Box CH-2501 Biel-Bienne Switzerland | *.pdf | Submission in English Ivan.franic@bakom.admin.ch Ph: +41 (0) 32 / 327 5703 Fx: +41 (0) 32 / 327 5666 |
| --- | --- | --- | --- | --- | --- |
| Thailand THA/3/66 | yes | yes | Auraphan Suwanrat Deputy Director General Post and Telegraph Department 87 Phaholyothin 8 Road Bangkok 10400 Thailand | *.pdf | Submission in English includes national allocation table from 9 kHz to 1350 MHz Ph: 662 27 10 15160 Fx: 662 27 13514 or 13512 http://www.ptd.go.th |
| Turkey TUR/1/67 | yes | yes | Abdullah Karakas Head of Frequency Management Department REPUBLIC OF TURKEY MINISTRY OF TRANSPORTATION GENERAL DIRECTORATE OF RADIOCOMMUNICATIONS Telsiz Genel Müdürlüğü-TGM Ulaştırma Bakanlığı Sitesi Emek 06510 Ankara-TÜRKİYE | *.doc | Submission in English akarakas@tgm.gov.tr Ph: 2126010/210 Fx: 2213226 |
| United Arab Emirates UAE/1/68 | yes | yes | Sultan Ali Hassan Al Marzouki A/ Director, Department of Telecommunication Ministry of Communication P.O.Box 900 Abu Dhabi, United Arab Emirates | *.pdf | Submission in English Ph: 00 97 12 662 909 Fx: 00 97 12 668 180 |

| United Kingdom G/1/69 | yes | yes | Terry Jeacock DTI Radiocommunications Agency 14R/3A Wyndham House 189 Marsh Wall, London, E14 9SX United Kingdom | *.zip *.doc *.gif | Submission in English terence.jeacock@ties.itu.int Ph: +44 (0)207 211 0004 Fx: +44 (0)207 211 0021 |
| --- | --- | --- | --- | --- | --- |
| United States USA/2/70 | yes | yes | William Luther (non Federal Government issues) Chief, Radiocommunication Policy Federal Communications Commission Washington, DC, 20554 Norbert Schroeder (Federal Government issues) Program Manager National Telecommunications and Information Administration Department of Commerce Washington, DC, 20230 | *.doc | Submission in English wluther@fcc.gov Ph: 202 418 0729 Fx: 202 418 7270 nschroeder@ntia.doc.gov Ph: +202 482 6207 Fx: +202 501 6198 |
| Vietnam VTN/3/71 | yes | yes | Luu Van Luong Director General Radio Frequency Department (DGPT) 18 Nguyen Du Street Hanoi, Vietnam | *.pdf | Submission in English Ph: 84 4 822 6732 Fx: 84 4 822 6930 |

### International Telecommunication Union -- Joint ITU-R ITU-D Sectors Working Group for WTDC-98 Resolution 9

This working group - established through the decisions of ITU-R Study Group 1 (in Assen, Netherlands, in August 1999) and ITU-D Study Groups 1 & 2 (in Geneva, Switzerland, September 1999) - is responsible for implementing Resolution 9, adopted at the 1998 ITU Telecommunication Development Conference in Valetta.  In this resolution, the ITU-D Sector requested assistance in radio frequency spectrum management from the ITU-R Sector.  To better focus the nature and scope of the assistance needed, the working group developed a two-part questionnaire (Part I addresses national radio frequency spectrum allocations policy; Part II addresses national spectrum management organizational practices) that was distributed in November 1999 to all ITU Member States and to all Sector members of the ITU R and D Sectors.  About seventy-five responses to this questionnaire have been received and  work is now underway to prepare a summary report of the information presented in these responses.

There is a wealth of spectrum management information contained in the responses to the questionnaire that clearly could prove immediately useful  to some spectrum managers in many countries.  And because of the highly detailed nature of  the spectrum allocation information contained in the responses to the questionnaire that could still be summarized in future draft work-products of the working group’s, there is a need for a medium that will facilitate very careful review of these draft work-products.  Consequently, this website has been established both for the purpose of making the submitted information immediately accessible to those spectrum managers seeking assistance, and for facilitating a comprehensive review of future draft work-products produced by this working group.

On this website you will find copies of the original questionnaire and all of the available responses.  All of the responses are presented here in their submitted form; no translations are provided.  About twenty of the submissions were made electronically and the remaining submissions were made on paper; those responses available only in paper form were scanned and are presented here in *.pdf file format.  Consequently, to use this website requires that both the Microsoft Word word-processor and the Adobe Acrobat reader software are available to the user. For further information regarding this Web Page, please contact Norbert Schroeder (nschroeder@ntia.doc.gov); Tel: +202 4826207; Fax: +202 5016198.

Further details regarding the activities of this working group --  Co-Chaired by Mr. Terry Jeacock (UK) and Mr Zanga-Yene (Cameroon) and previously Mr. P.K. Garg (India) -- are available from the ITU Website at  http://www.itu.int/ITU-D-StGrps/SGP_1998-2002/JGRES09/Res9_Index.htm.

1.	Contributions

2.	List of Submitters and Focal Points

3.	Draft Report

4.	Copies of Questionnaire Distributed by ITU-R and ITU-D in November 1999

1.	Introduction

The purpose of this Annex is to give a brief overview of the services operating in the frequency range and developments foreseen. It is emphasised that this information is not intended to endorse any particular technology or approach. The Radio Regulations allocate frequency bands to Services and these allocations differ between the ITU Regions. In many cases, the same frequency band is allocated on a primary basis to more than one service and national footnotes allow further variations. In addition, within the same frequency band and the same service, actual use can be different between countries depending on national requirements. For example, a band allocated to the mobile service may be used in one country for private mobile use and in another for government mobile use, or there may be differences in technical parameters (channel bandwidth, modulation type, etc.). Therefore any reference to applications in the information below, frequency sub-band use is given only in general terms.

2.	Services identified

Table 7-1 lists the Services shown in Article 5 of the Radio Regulations (1998) as having a primary allocation in the frequency range 29.7 MHz to 960 MHz, together with the total amount of spectrum allocated in each region. A Table such as this cannot represent the actual use of the spectrum because it does not take into account the effect of footnotes or that many services have co-primary allocations or that there are services operating satisfactorily on a secondary basis. In the case of co-primary allocations, administrations will generally have decided, on a national basis or in co-operation with neighbouring countries, which of the co-primary services may use a particular sub-band. For example, the responses to the Questionnaire indicated that many of the sub-bands allocated on a co-primary basis to Fixed and Mobile services are used predominantly by mobile systems.

However, the Table does show a surprising similarity between the ITU-R Regions and clearly shows the services that have been allocated the largest amount of spectrum in this frequency range.

Table 7-1

| Service | Amount of spectrum allocated on a primary basis (MHz) |  |  |
| --- | --- | --- | --- |
|  | ITU-R Region 1 | ## ITU-R Region 2 | ITU-R Region 3 |
| # Broadcasting | 587.5 | 506 | 576 |
| Fixed | 499.175 | 487.075 | 883.675 |
| Mobile | 427.175 | 458.675 | 883.675 |
| Aeronautical-Mobile (R) | 19.025 | 19.025 | 19.025 |
| Aeronautical Radionavigation | 17.325 | 17.325 | 29.325 |
| Radio Astronomy | 16.45 | 21.1 | 19.5 |
| Amateur | 12 | 13 | 8 |
| Space Research | 11.905 | 11.905 | 11.905 |
| Radiolocation | 10 | 16 | 10 |
| Aeronautical-Mobile (OR) | 6 | 0 | 0 |
| Meteorological Aids | 5.85 | 5.85 | 5.85 |
| Meteorological-Satellite | 3.85 | 3.85 | 3.85 |
| Mobile-Satellite | 3.825 | 5.825 | 3.825 |
| Space Operation | 3.005 | 3.005 | 3.005 |
| Amateur Satellite | 2 | 2 | 2 |
| Earth Exploration-Satellite | 2 | 2 | 2 |
| Radionavigation Satellite | 0.3 | 0.3 | 0.3 |
| Standard Frequency and Time Signal-Satellite | 0.1 | 0.1 | 0.1 |
| Maritime Mobile | 0.075 | 4.075 | 0.075 |
| Radionavigation | 0 | 0 | 25 |

3.	Band usage and typical applications within each service

For many years this frequency range has been used extensively for a wide variety of services. The propagation characteristics are ideal to provide reliable communications within easily predictable service areas over relatively short ranges (typically 30 – 80 km) and considerable frequency re-use can be obtained by careful planning. As can be seen from the Table, two of its principal uses are to provide national broadcasting (sound and television) and mobile applications.

Various services may require different types of coordination in the context of the Radio Regulations

### 3.1	Broadcasting applications

As one of the largest uses of the frequency range 29.7 MHz to 960 MHz,  broadcasting has bands allocated at (approximately) 47-68 MHz  (Band 1); 87.5 – 108 MHz (Band 2); 174 – 230 MHz (Band 3); 470 – 610 MHz (Band IV) and 610 – 862 MHz (Band V).  There are Regional differences, for example, in Region 2 and some countries in Region 1, Bands 68- 72 MHz and 76 – 87.5MHz  are also used.

Using the approximate band designations above, Band 2 is used for sound broadcasting and Bands 1, 3, 4 and 5 for television broadcasting. While the systems used for analogue sound broadcasting there are several systems in use for analogue television.

To ensure effective use and equitable distribution of the spectrum, especially in border areas, “Regional Agreements” exist that establish frequency sharing criteria and frequency use plans between the countries concerned, for example the "Regional Agreement for the European Broadcasting Area Concerning the Use of Frequencies by the Broadcasting Service in the VHF and UHF Bands" adopted by the European VHF/UHF Broadcasting Conference (Stockholm, 1961)” and The "Regional Agreement Relating to the Use of the Band 87.5 - 108 MHz for FM Sound Broadcasting (Region 1 and Part of Region 3)" adopted by the Regional Administrative Conference for the Planning of VHF Sound Broadcasting (Region 1 and Part of Region 3) (Geneva, 1984).

### 3.2	Mobile applications

Land mobile systems operate in sub-bands throughout the range typically around 30, 80, 160, 450 and 950 MHz although there are many secondary allocations for mobile. There are many government, publicly operated and private applications for Defence, safety, despatch systems, trunked radio systems, public mobile telephone  (note – cellular public mobile services operate at 160, 460, 960 MHz)

The Maritime Mobile service has the international distress, safety and calling frequency of 156.8 MHz for VHF radiotelephony and 156.525 for digital selective calling to provide safety of vessels mainly in coastal waters. Other frequencies in this range may be used for maritime operational purposes.

The Aeronautical Mobile (R) service has internationally planned frequencies in the 118 – 137 MHz band for flight safety and air traffic control purposes.

The Mobile-satellite service has a number of small allocations mainly for low-data rate LEO systems. Within the mobile-satellite service, emergency position-indicating radiobeacons (EPIRBs) operate at 121.5 MHz and 243 MHz (S5.199).

### 3.3	Space Operations, Space Research

In this frequency range, a total of 3.005 MHz is available on a primary basis in all ITU Regions.

### 3.4	Meteorological-Satellite, Meteorological Aids

In this frequency range, a total of 3.85 MHz is available on a primary basis in all ITU Regions.

3.5	Aeronautical Radionavigation, Radionavigation, Radionavigation-satellite, Radiolocation

The largest use in this category is Aeronautical Navigation: VOR systems, Instrument Landing Systems  (ILS) and Glide Path.  Most radiolocation  systems operate in frequency bands above 960 MHz but there are some specialised radiolocation applications that require longer wavelengths, mainly long range radar systems.

### 3.6	Amateur, Amateur-Satellite

Within this frequency range, allocations to the Amateur service may be found at 50 MHz, 144 MHz, 220 MHz and 432 MHz. However, only 144 – 146 MHz is available on a world-wide primary basis. In addition to the purpose of self training in wireless telegraphy (S1.56), radio amateurs often assist in communications following natural disasters. More details can be found in the recently produced ITU-D hand book on “Disaster Communications”.

4.	Technology trends

As described above, this frequency range is particularly suited to a wide range of applications and there is an increasing demand for frequencies to meet the expanding requirements of existing and new applications. In some countries there is congestion and consequently an incentive to develop technology with improved efficiency that will make more effective use of the spectrum.

Narrower bandwidths and trunking, for example, have been introduced over the years for analogue systems but digital technology offers the most promising opportunity for most radio services, especially the use of data compression techniques, packet switching and robust modulation schemes such as Coded Orthogonal Frequency Division Multiplexing (COFDM).

Administrations are planning the transition from analogue to digital, especially for the two largest uses, Broadcasting and Mobile, and are considering various means to speed up the process, bearing in mind the often large user investment in analogue technology.

The ITU-D Handbook on New Technologies and New Services contains more detailed information.

5.	Application trends

The providers of telecommunication services are creating an ever increasing market in information, images and other facilities for business and entertainment. In turn, users are expecting to access this electronic market place using multi-purpose fixed or mobile terminals. The introduction of digital technology and the Internet are forcing the convergence of applications and the methods in which they are transported from source to user. Many of the new digital radiocommunication systems operating in the review range will be used in this new multi-media environment and there may be some regulatory impact.

An agenda item for WRC03 will touch on the subject of convergence when it examines a progress report from ITU on ‘Terrestrial Wireless Interactive Systems’. Recognising the multiple interests involved, ITU-R established Joint Task Group 1, 6, 8, and 9 to undertake the study.

6. Summary of the roles of the ITU-R Study Groups

A short introduction to the ITU-R Study Groups and their scope

6.1	Study  Group 1:  Spectrum Management (Spectrum planning, utilization, engineering, sharing & monitoring)

1. Development of principles and techniques for effective spectrum management, sharing criteria and methods, techniques for spectrum monitoring and long-term strategies for spectrum utilization and economic approaches to national spectrum management as well as, in association with the appropriate bodies of the ITU, facilitation of the collection and dissemination of information concerning computer programs prepared for the implementation of relevant Recommendations.

2. To provide assistance in matters within its competence to developing countries in cooperation with the Telecommunication Development Sector.

3. Study a limited number of specific urgent Questions concerning inter-service sharing and compatibility referred to it by the Radiocommunication Assembly or, if the Question arises during the interval between the Assemblies, by the decision of a meeting of the Study Group Chairpersons and Vice-Chairpersons or by the Director after consultation with interested Study Group Chairpersons and Administrations. The Radiocommunication Assembly or the Director, as the case may be, shall establish a time schedule for the completion of this work.

4. Development of Recommendations or of a Report to the Conference Preparatory Meeting in answer to those urgent Questions concerning inter-service sharing and compatibility requiring special attention. This course of action shall be followed if the matter cannot be dealt with more expeditiously through the mechanism of joint working parties, joint task groups or ad hoc rapporteur groups, as assigned by the Radiocommunication Assembly, or if the Question arises during the interval between Radiocommunication Assemblies, by the Director after consultation with interested Study Group Chairpersons and Administrations.

Recommendations and Reports in the SM Series

Most of the Recommendations or Reports in the SM series are generally applicable and not specific to the frequency range considered in Stage 1 of this Report.  However, in view of the scope of Study Group 1, especially its responsibilities with respect to providing assistance to developing countries, these Recommendations and Reports will be relevant to developing countries. Some are mentioned elsewhere in this Report, for example: the Handbooks on National Spectrum Management, Spectrum Monitoring, Computer-Aided Techniques; Report. The following Reports and Recommendations will also be of particular interest to developing countries:

- SM.2012 (Economic aspects of spectrum management)

- SM.1048 Design guidelines for a basic automated spectrum management system (BASMS)

- SM.1370 Design guidelines for developing advanced automated spectrum management systems (ASMS)

- SM.1392-1 Essential requirements for a spectrum monitoring station for developing countries

- SM.1393 Common formats for the exchange of information between monitoring stations

- SM.1394 Common format for memorandum of understanding between the agreeing countries regarding cooperation in spectrum monitoring matters

- SM.1413 Radiocommunication data dictionary

6.2	Study Group 3: Radiowave Propagation

Propagation of radio waves in ionized and non-ionized media and the characteristics of radio noise, for the purpose of improving radiocommunication systems.

Recommendations in the P Series: Radiowave propagation

P.310-9 (11/93) Definitions of terms relating to propagation in non-ionized media

P.341-5 (10/99) The concept of transmission loss for radio links

P.370-7 (10/95) VHF and UHF propagation curves for the frequency range from 30 MHz to 1 000 MHz. Broadcasting services

P.525-2 (11/93) Calculation of free-space attenuation

P.529-3 (10/99) Prediction methods for the terrestrial land mobile service in the VHF and UHF bands

P.833-2 (10/99) Attenuation in vegetation

P.843-1 (08/97) Communication by meteor-burst propagation

P.1144-1 (10/99) Guide to the application of the propagation methods of Radiocommunication Study Group 3

P.1406 (07/99) Propagation effects relating to terrestrial land mobile service in the VHF and UHF bands

6.3	Study Group 6: Broadcasting Service - Terrestrial and satellite

Radiocommunication broadcasting (terrestrial and satellite), including vision, sound, multimedia and data services principally intended for delivery to the general public.

Broadcasting makes use of point-to-everywhere information delivery to widely available consumer receivers. When return channel capacity is required (e.g. for access control, interactivity, etc.), broadcasting typically uses an asymmetrical distribution infrastructure that allows high capacity information delivery to the public with lower capacity return link to the service provider. The production and distribution of programs (vision, sound, multimedia, data, etc.) may employ contribution circuits among studios, information gathering circuits (ENG, SNG, etc.), primary distribution to delivery nodes, and secondary distribution to consumers.

The Study Group, recognizing that radiocommunication broadcasting extends from the production of programmes to their delivery to the general public, as detailed above, studies those aspects related to production and radiocommunication, including the international exchange of programs as well as the overall quality of service.

Recommendations in the BS Series: Broadcasting service (sound)

BS.415-2 (07/86) Minimum performance specifications for low-cost sound-broadcasting receivers BS.412-9 (12/98) Planning standards for terrestrial FM sound broadcasting at VHF BS.450-2 (10/95) Transmission standards for FM sound broadcasting at VHF BS.467 (07/70) Technical characteristics to be checked for frequency-modulation stereophonic broadcasting. Pilot-tone system BS.599 (07/82) Directivity of antennas for the reception of sound broadcasting in band 8 (VHF) BS.638 (07/86) Terms and definitions used in frequency planning for sound broadcasting BS.641 (07/86) Determination of radio-frequency protection ratios for frequency-modulated sound broadcasting BS.643-2 (10/95) System for automatic tuning and other applications in FM radio receivers for use with the pilot-tone system BS.773 (03/92) Radio-frequency protection ratios required by FM sound broadcasting in the band between 87.5 MHz and 108 MHz against interference from D/SECAM television transmissions BS.1114-1 (10/95) Systems for terrestrial digital sound broadcasting to vehicular, portable and fixed receivers in the frequency range 30-3 000 MHz BS.1195 (10/95) Transmitting antenna characteristics at VHF and UHF Published separately, with Rec. ITU-R BS.705-1

Recommendations in the BT Series: Broadcasting service (television)

BT.417-4 (03/92) Minimum field strengths for which protection may be sought in planning a television service

BT.419-3 (03/92) Directivity and polarization discrimination of antennas in the reception of television broadcasting

BT.470-6 (11/98) Conventional television systems

BT.565 (07/78) Protection ratios for 625-line television against radionavigation transmitters operating in the shared bands between 582 and 606 MHz

BT.655-6 (03/00) Radio-frequency protection ratios for AM vestigial sideband terrestrial television systems interfered with by unwanted analogue vision signals and their associated sound signals

BT.798-1 (11/93) Digital terrestrial television broadcasting in the VHF/UHF bands

BT.804 (03/92) Characteristics of TV receivers essential for frequency planning with PAL/SECAM/NTSC television systems

BT.805 (03/92) Assessment of impairment caused to television reception by a wind turbine

BT.1123 (11/93) Planning methods for 625-line terrestrial television in VHF/UHF bands

BT.1125 (11/93) Basic objectives for the planning and implementation of digital terrestrial television broadcasting systems

BT.1206 (10/95) Spectrum shaping limits for digital terrestrial television broadcasting

BT.1299 (10/97) The basic elements of a worldwide common family of systems for digital terrestrial television broadcasting

BT.1368-2 (03/00) Planning criteria for digital terrestrial television services in the VHF/UHF bands

6.4	Study Group 7: Science Services

1 Systems for space operation, space research, earth exploration and meteorology, including the related use of links in the inter-satellite service.

2 Radio astronomy and radar astronomy.

3 Dissemination, reception and coordination of standard-frequency and time-signal services, including the application of satellite techniques, on a worldwide basis.

Recommendations in the RA Series: Radioastronomy

RA.314-8 (03/92) Preferred frequency bands for radioastronomical measurements

RA.517-2 (03/92) Protection of the radioastronomy service from transmitters in adjacent bands

RA.1031-1 (10/95) Protection of the radioastronomy service in frequency bands shared with other services

Recommendations in the SA Series: Space applications and meteorology

SA.363-5 (11/93) Space operation systems. Frequencies, bandwidths and protection criteria

SA.364-5 (03/92) Preferred frequencies and bandwidths for manned and unmanned near-Earth research satellites

SA.509-2 (02/98) Generalized space research Earth station and radio astronomy antenna radiation pattern for use in interference calculations, including coordination procedures

SA.515-3 (06/97) Frequency bands and bandwidths used for satellite passive sensing

SA.577-5 (06/97) Preferred frequencies and necessary bandwidths for spaceborne active remote sensors

SA.609-1 (03/92) Protection criteria for telecommunication links for manned and unmanned near-Earth research satellites

SA.1165-1 (06/97) Technical characteristics and performance criteria for radiosonde systems in the meteorological aids service

SA.1258-1 (10/99) Sharing of the frequency band 401-403 MHz between the meteorological-satellite service, Earth exploration-satellite service and meteorological Aids service

6.5	Study Group 8: Mobile, radiodetermination, amateur and related satellite services

Systems and networks for the mobile, radiodetermination and amateur services, including related satellite services.

Recommendations in the M Series: Mobile, radiodetermination, amateur and related satellite services

M.441-1 (07/82) Signal-to-interference ratios and minimum field strengths required in the aeronautical mobile (R) service above 30 MHz

M.478-5 (10/95) Technical characteristics of equipment and principles governing the allocation of frequency channels between 25 and 3 000 MHz for the FM land mobile service

M.489-2 (10/95) Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz

M.539-3 (11/93) Technical and operational characteristics of international radio-paging systems

M.584-2 (11/97) Codes and formats for radio paging

M.586-1 (07/86) Automated VHF/UHF maritime mobile telephone system

M.622 (07/86) Technical and operational characteristics of analogue cellular systems for public land mobile telephone use

M.623 (07/86) Data transmission bit rates and modulation techniques in the land mobile service

M.633-2 (05/00) Transmission characteristics of a satellite emergency position-indicating radio beacon (satellite EPIRB) system operating through a low polar-orbiting satellite system in the 406 MHz band

M.690-1 (10/95) Technical characteristics of emergency position-indicating radio beacons (EPIRBs) operating on the carrier frequencies of 121.5 MHz and 243 MHz

M.1033-1 (02/97) Technical and operational characteristics of cordless telephones and cordless telecommunication systems

M.1039-2 (05/00) Co-frequency sharing between stations in the mobile service below 1 GHz and mobile earth stations of non-geostationary mobile- satellite systems (Earth-space) using frequency division multiple access

M.1041-1 (10/98) Future amateur radio systems (FARS)

M.1042-1 (10/98) Disaster communications in the amateur and amateur-satellite services

M.1043-1 (10/98) Use of the amateur and amateur-satellite services in developing countries

M.1044-1 (10/98) Frequency sharing criteria in the amateur and amateur-satellite services

M.1072 (11/93) Interference due to intermodulation products in the land mobile service between 25 and 3 000 MHz

M.1073-1 (02/97) Digital cellular land mobile telecommunication systems

M.1074 (11/93) Integration of public mobile radiocommunication systems

M.1076 (11/93) Wireless communication systems for persons with impaired hearing

M.1077 (11/93) Multi-transmitter radio systems using quasi-synchronous (simulcast) transmission for analogue speech

M.1084-3 (10/98) Interim solutions for improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service

M.1085-1 (02/97) Technical and operational characteristics of wind profiler radars for bands in the vicinity of 400 MHz

M.1087 (11/93) Methods for evaluating sharing between systems in the land mobile service and spread-spectrum low-Earth orbit (LEO) systems in the mobile-satellite service (MSS) below 1 GHz

M.1185-1 (10/97) Method for determining coordination distance between ground based mobile earth stations and terrestrial stations operating in the 148.0-149.9 MHz band

M.1221 (02/97) Technical and operational requirements for cellular multimode mobile radio stations

M.1222 (02/97) Transmission of data messages on shared private land mobile radio channels

M.1226 (02/97) Technical and operational characteristics of wind profiler radars in bands in the vicinity of 50 MHz

M.1230 (02/97) Performance objectives for space-to-Earth links operating in the mobile-satellite service with non-geostationary satellites in the 137-138 MHz band

M.1231 (02/97) Interference criteria for space-to-Earth links operating in the mobile-satellite service with non-geostationary satellites in the 137-138 MHz band

M.1232 (02/97) Sharing criteria for space-to-Earth links operating in the mobile-satellite service with non-geostationary satellites in the 137-138 MHz band

M.1307 (10/97) Automatic determination of location and guidance in the land mobile services

M.1462 (05/00) Characteristics of and protection criteria for radars operating in the radiolocation service in the frequency range 420-450 MHz

M.1470 (05/00) Methodology of sharing between MSS systems (Earth-to-space) and existing RNSS systems (space-to-Earth) in frequency bands 149.9-150.05 MHz and 399.9-400.05 MHz

M.1478 (05/00) Protection criteria for COSPAS-SARSAT search and rescue processors in the band 406-406.1 MHz

6.6	Study Group 9: Fixed Service

Systems and networks of the fixed service operating via terrestrial stations.

### Recommendations in the F Series: Fixed service

F.757-2 (05/99) Basic system requirements and performance objectives for fixed wireless access using mobile-derived technologies offering basic telephony services

F.758-2 (05/00) Considerations in the development of criteria for sharing between the terrestrial fixed service and other services

F.759 (03/92) The use of frequencies in the band 500 to 3 000 MHz for radio-relay systems

F.1103 (11/93) Radio-relay systems operating in bands 8 and 9 for the provision of subscriber telephone connections in rural areas

F.1105 (11/93) Transportable fixed radiocommunication equipment for relief operations

F.1113 (11/93) Radio systems employing meteor-burst propagation

F.1399 (05/99) Vocabulary of terms for wireless access

F.1400 (05/99) Performance and availability requirements and objectives for fixed wireless access to public switched telephone network

F.1401 (05/99) Frequency bands for fixed wireless access systems and the identification methodology

F.1402 (05/99) Frequency sharing criteria between a land mobile wireless access system and a fixed wireless access system using the same equipment type as the mobile wireless access system

Possible material from relevant web pages of ITU

[(list of relevant ITU-R recommendations for various digital systems)

7.	Pending issues in the Study Groups

Interested in subjects WRC03 and beyond…

Identify some issues (from Questions) and send to the Study Group Chairman for further advice and comment.

Overview of the functional framework of the Telecommunication Development Sector

Article 21 of the ITU’s Constitution specifies that the functions of the Telecommunication Development Sector shall be "to fulfil the purposes of the Union as stated in Article 1 of the Constitution and to discharge, within its specific sphere of competence, the Union’s dual responsibility as a UN specialised agency and executing agency for implementing projects under the UN development system or other funding arrangements so as to facilitate and enhance telecommunications development by offering, organising and co-ordinating technical co-operation and assistance activities".

In carrying out its functions, the Telecommunication Development Sector works through Telecommunication Development Conferences, their associated Development Study Groups and the Telecommunication Development Bureau (BDT).

The BDT provides the infrastructural support for a comprehensive and collaborative development programme for the enhancement of telecommunication systems and services. Whether advising on appropriate institutional structures, networks and services for efficient telecommunications or assisting in mobilising the financial and human resources necessary to introduce technological innovations, the Bureau’s work extends the ITU’s commitment to encouraging the application of new technology.

The work program of the Telecommunication Development Sector is specified in an Action Plan. The current one is the Valletta Action Plan that was decided by the WTDC-98 after examining the various reports and proposals received, particularly the report on the Buenos Aires Action Plan and the Strategic Report "Partners for development: New opportunities for the telecommunication sector".

The Plan consists of four chapters:

# Cooperation among the Members

World and Regional Telecommunication Development Conferences where policy issues and strategic orientations are discussed, two study groups providing analytical support to telecommunications development, the Telecommunication Development Advisory Group to advise the Director, and information collection and sharing.

# Six Programmes

To provide a well-focused and coherent assistance to developing countries in the areas of sector reform, technologies, rural development, financing, partnerships and human resource development, areas which are critical to the successful development of telecommunications worldwide.

# A Special Programme for the Least Developed Countries

Aiming in particular at creating the conditions for sustainable development in these countries focusing on the areas mentioned above.

# Projects and Direct Assistance

Either financed by multilateral sources or through funds-in-trust.

The resources available for these actions are limited in view of the huge investments and operating costs required worldwide. In ITU’s model, however, the telecommunications sector is self-sustainable if the internal revenues and resources are used in a catalytic way.

The proposed further action ideally will have to fit into this framework: Action within the study groups, the programs – in particular program 2 and the special program for LDCs – and within the projects and direct assistance when financing is provided by UNDP, fund-in-trust or other sources.

_______________

The term "redeployment" is used here to refer to a process of national scope in which an assessment is conducted 1) to determine if portions of spectrum can be identified that are in limited use; and 2) to determine if  such spectrum segments can be reallocated for use in delivering radiocommunication services that have expanding spectrum requirements.

The National Spectrum Management Handbook will be updated, starting in 2001. For questions and comments relating to this update, you are invited to contact Mr Bob Mayher (USA), the designated Rapporteur from ITU-R Study Group . Responses to the Questionnaire related to this Handbook will be taken into account in the coming update.

The Spectrum Monitoring Handbook is currently being updated and is expected to be available in the three working languages before July 2002.  For questions related to this revision or if you have any comments that you wish included in a future version of this Handbook, you are invited to contact Mr Jan Verduijn (NL), the designated Rapporteur from ITU-R Study  Group 1, Working Party 1C.

This Report SM.2012 was updated during the ITU-R Study Group 1 meeting in August 1999. The English language version is available now.Versions in the other working languages are expected to be published soon.

NP: No plan for up-dating the national law

Microsoft

Personal Computer