| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION DEVELOPMENT BUREAU ITU-D STUDY GROUPS | Document 2/186(Rev.1)-E 18 August 2000 Original: English |
| THIRD MEETING OF STUDY GROUP 1: GENEVA, 11 - 15 SEPTEMBER 2000 THIRD MEETING OF STUDY GROUP 2: GENEVA, 18 - 22 SEPTEMBER 2000 |  |

FOR ACTION

Question 16/2: 	Preparation of Handbooks for developing countries

STUDY GROUP 2

SOURCE:	RAPPORTEUR FOR QUESTION 16/2

TITLE:	HANDBOOK ON NEW TECHNOLOGIES AND NEW SERVICES:
FASCICLE 4

________

Action required: Participants are invited to send their comments to BDT Secretariat by May 2001 at the latest.  After inclusion of the comments received, Fascicle 4 will be published in 2001.  All the chapters relating to abbreviations will be completed during September meeting.  This fascicle has not yet been edited.

Action demandée:  La version française sera transmise dès que disponible aux participants concernés (probablement en avril 2001).  Les participants sont invités à transmettre leurs commentaires au Secrétariat du BDT au plus tard un mois après réception.  Après inclusion des amendements reçus, le Fascicule 4 sera publié en 2001.  Tous les chapitres relatifs aux abréviations seront complétés durant la réunion de septembre 2000.  Ce fascicule n’a pas encore été revu dans sa forme.

Acción solicitada:  La versión en español será transmitida cuando sea disponible a los participantes interesados (probablemente en el mes de abril del año 2001).  Se invita a los participantes a transmitir los comentarios a la Secretaria de la BDT a más tardar un mes después de su recepción.  Después de la inclusión de las enmiendas recibidas, el fascículo 4 será publicado en 2001.  Todos los capítulos relativos  a las abreviaciones serán completados durante la reunión de septiembre 2000.  Este fascículo no ha sido todavía revisado en su forma.

# FASCICLE 4

DIGITAL RADIO AND TELEVISION NETWORKS AND SERVICES

The mention of specific companies or products does not imply any endorsement or recommendation by the ITU.

### Table of contents

Chapter

1.	Introduction		7

1.1	General		7

1.2 	Structure of the Fascicle 4		8

2.	Digital Audio broadcasting		9

2.1 	Introduction		9

2.2	T-DSB objectives		9

2.2.1	CD Quality Reception		10

2.2.1.1 	Frequency Selective Fading		10

2.2.1.2 	Digital transmission in multipath environment		11

2.2.1.2.1  Frequency Interleaving		13

2.2.1.2.2  Time Interleaving		13

2.2.1.2.3  Channel error correction		13

2.2.2 	Multiple choice of programs		14

2.2.3 	Multimedia feature		14

2.2.4	Data service		14

2.2.4.1	Programme Associated data (PAD)		14

2.2.4.2	Fast Information data Channel (FIDC)		15

2.2.4.3	Packet mode		15

2.2.4.4 	MOT Protocol		15

2.2.4.5	Broadcast Website		15

2.2.5	Adaptive audio-bandwidth		16

2.2.6 	Spectrum efficiency		16

2.2.7 	Some feature for planning		17

2.3	Different T-DSB system		19

2.3.1 	T-DSB above 30 MHz		19

2.3.1.1	IBOC & IBAC		20

2.3.1.2 	ISDB of Japan		20

2.3.1.3 	Satellite digital Sound broadcasting		21

2.3.1.3.1  Worldspace U.S.A.		21

2.3.1.4 	S-DARS at 2.3 GHz		23

2.3.1.5 	Mediastar system		23

2.3.2	DSB system below 30 MHz		23

2.3.2.1	Medium-wave Band		23

2.3.2.1.1  Digital MW by Deutsch Telekom / Telefunken/
							  Continental		24

2.3.2.2	Short-wave Band		25

2.3.2.2.1  ‘Skywave-2000’ of Thomcast		25

2.3.2.2.2  System by Voice of America & Jet propulsion Lab		26

2.3.2.2.3  ‘USADR’ in Short Wave		26

2.3.2.2.4  Digital H.F. by CCETT, France		26

2.3.3 	Towards a Common goal of World-wide standard for
AM Broadcasting		27

2.4 	List of Abbreviations		28

3.	Digital Television Broadcast Services		31

3.1 	Overview		31

3.1.1 	Introduction		31

3.1.2	Structure of a Digital Television System		32

3.1.3	Transmission media for Digital Television System		34

3.1.4	Standards for Digital Television Broadcasting		34

3.1.4.1	DVB standards		34

3.1.4.2	ATSC Digital television Standard		35

3.1.4.3	ISDB Standards		35

3.1.5	Service Information (SI) System		35

3.1.6	Conditional Access System		36

3.2	Digital Satellite Broadcasting		37

3.2.1	DVB-S		37

3.2.2	DIRECT TV		42

3.3	Digital Terrestrial Broadcasting		42

3.3.1	Introduction		42

3.3.2	The DVB-T system		43

3.3.3	ISDB-T System		45

3.3.4	ATSC system		47

3.3.5	DVB-T Distribution Networks		47

3.3.6	Transmission Networks		48

3.4	List of Abbreviations		49

3.5	References		51

4.	Strategies for Digital Television Broadcasting		53

4.1	Introduction			53

4.2	Planning for the transition period		53

4.3	Post transition period		55

4.4	Planning Criteria		55

4.4.1	Coverage definitions for fixed and portable reception		55

4.4.2	Coverage area		55

4.4.3	Wanted signal Level considerations		55

4.4.4	Minimum signal Level		56

4.4.5	Digital Cable Broadcasting		57

4.4.5.1	DVB-C System		57

4.4.5.2	Cable networks		58

4.4.5.3	SMATV system		58

4.4.5.3.1  System A		59

4.4.5.3.2  System B		59

4.4.6	Interactive Television Services		59

4.4.6.1	Introduction		59

4.4.6.2	DVB specification for broadcast		59

4.4.6.3	System model		60

4.5	List of Abbreviations		61

4.6	References		62

5.	Data Broadcasting		63

5.1	Introduction			63

5.1.1	Maket potential		63

5.1.2	Emerging opportunity for broadcasters		63

5.2	What is data broadcast?		64

5.2.1	Data broadcast on Radio services		64

5.2.2	Digital broadcasting		64

5.2.3	Advantages of digital broadcasting		64

5.2.4	Broadcaster’s widening roles 		65

5.2.5	Essential requirements for a data system (Radio Frequency environment)	65

5.2.6	Requirements of an information dissemination system		66

5.3	Delivery mechanisms for data broadcasting		67

5.3.1	Terrestrial broadcasting		67

5.3.2	Broadcast radio		68

5.3.2.1	HF Radio		70

5.3.2.2 	Skywave 2000		71

5.3.2.3	AMDS (AM Data System)		72

5.3.3	FM Data systems		74

5.3.3.1	High-speed data systems in FM		74

5.3.3.2	DAB		76

5.3.3.3	DAB standards and further developments		76

5.3.3.4	DAB – a multimedia carrier		76

5.3.4	Broadcast television		78

5.3.4.1	DVB		79

5.3.4.2	Video source coding		79

5.3.4.3	Audio source coding		79

5.3.4.4	Multiplexing		80

5.3.4.5	Modulation and channel coding		80

5.3.4.6	Carrier systems		80

5.3.5	Data broadcasting in DVB		81

5.3.6	Obstacles in the path of migration to digital domain		81

5.3.7	Satellite data broadcasting		82

5.3.7.1	World Space Satellite to radio, multimedia broadcast system		82

5.3.8	Broadband wireless		83

5.3.8.1	Multichannel Multipoint Distribution Service (MMDS)		84

5.3.8.2	ISDN		85

5.3.8.2.1  Applications of ISDN		85

5.3.8.2.2  ISDN developments		86

5.3.8.2.3  Benefits of ISDN		86

5.3.9 Cable TV		87

5.3.9.1	Essential components of cable TV system		88

5.3.9.2	HFC Cable systems		88

5.3.9.2.1  Channel organization and topology of HFC 
						                 cable systems		89

5.3.9.2.2  Two-way HFC system		89

5.3.9.3	Interactive two-way TV services		90

5.3.9.4	High speed data using cable system		90

5.3.9.5	Cable telephony		91

5.3.9.6	Signal scrambling and security		91

5.3.9.7	Digital compression in Cable media		92

5.3.10	  Multimedia broadcasting		92

5.3.10.1  Specification of a typical DVB data system		94

5.3.10.2  Information Infrastructure for Digital MM application		94

5.3.11	  Future developments		95

5.3.11.1  Terminal equipment		95

5.3.11.2	 ATM a platform for multimedia		95

5.3.11.3  Application of multimedia services		96

5.3.11.4  The Information super-highway and Information society		96

5.3.11.5  Digital Broadcasting		97

5.3.11.6  Full Service networks (FSN)		97

5.3.11.7  Internet		97

5.3.11.8  The Broadcast/Web combination / Hybrid delivery		98

5.3.11.9  Issue for Digital Television Broadcasting		98

5.4	List of abbreviations		98

5.5	References		101

6.	Related Worldwide Web sites		102

Annex A: Multimedia broadcasting – some examples		103

CHAPTER 1

1.	INTRODUCTION

1.1	General

Since the 1920’s radio technology has achieved tremendous growth, today there are over 2000 million radio receivers in use. The power of sound broadcasting to inform, educate and entertain at any place is unmatched. The main attraction of sound broadcasting is the very low cost of the receiver affordable to the large population. Digital technology is now capable of modernising this powerful medium bringing the benefits such as:

- Improved service reliability and coverage,

- Terrestrial and satellite reception with the same receiver,

- More programme choice,

- Possibility to have data services.

From the other side, delivery of audio signals through Internet is increasing and is becoming a strong competitor to traditional broadcasting services.

Digital coding and transmission of television picture and sound signals offer a new opportunity to future development in the field of television information, education and entertainment. Digital techniques might be realised in a number of ways offering:

- More programme transmissions per channel

- Higher quality pictures and sound

- Increased service area coverage or reduced transmission power

- Increased capability for data services

A wide range of digital broadcasting television systems and equipment suitable for the satellite, cable or terrestrial broadcasting domains have been developed and already implemented in many countries. However, a very large viewing public is still equipped with analogue receiving equipment and development of the appropriate strategy for transition from analogue to digital broadcasting is necessary. Another aspect is related to manufacturers. They might be reluctant to market only digital equipment while a consumer demand for cheaper analogue equipment continues to exist. In developing countries all of these aspects should be studied carefully, in order to make transition smooth and possible to identify the timeframe where it will be possible to switch off the existing analogue broadcast.

The aim of Fascicle 4 is to facilitate developing countries decisions on systems and strategies towards digital radio and television networks and services.

Further reading material can be found on these topics in the following references:

ITU-R Handbook on “Digital Sound Broadcasting to vehicular, portable and fixed receivers”, Geneva, 2002

ITU-R Handbook on “Digital Television Broadcasting”, Geneva, 2001

1.2. 	Structure of the Fascicle 4

# This Fascicle is divided in five chapters including the Introduction as Chapter 1. Each of the chapters contains abbreviations and references enabling the reader to obtain more information about each subject.

# Chapter 1, Introduction

# Chapter 2, Digital Audio Broadcasting, is composed of the introduction and two subchapters. The subchapter on Terrestrial Digital Sound Broadcasting (T-DSB) deals with quality reception (frequency selective fading, digital transmission in multipath environment, frequency and time interleaving), multiple choice of programme, multimedia features and data services.

# The subchapter on Different T-DSB systems discusses systems above and below 30 MHz.

Chapter 3 deals with the Digital Television Broadcasting Service. It is divided in three subchapters.

Subchapter Overview offers a detailed structure of digital television systems with three main subsystems (source coding and compression, service multiplex and transport and RF transmission), different standards and planning aspects.

Subchapter Digital Satellite Broadcasting discusses a digital video broadcasting satellite system (DVB-S).

Subchapter Digital Terrestrial Broadcasting explains the DVB-T, ISDB and ATSC system, and distribution networks.

Chapter 4 discusses Strategies for digital television broadcasting. The strategy is defined for the transition (Subchapter 4.1) and post-transition period (Subchapter 4.2). In subchapter Planning criteria, coverage area, wanted signal level and minimum signal level are discussed. Planning criteria are discussed separately for a Digital Cable Broadcasting System (DVB-C), Cable networks used for distribution and Interactive television services.

Chapter 5, Data Broadcasting, studies data broadcasting as the delivery of digital data, binary or text information from central source to a large, potentially unlimited number of receivers, or along with traditional broadcast programmes.

The first subchapter discusses market potential and opportunity for broadcasters while the second subchapter defines data broadcasting and essential requirements for a data system.

Subchapter Delivery mechanisms for data broadcasting deals in detail with terrestrial broadcasting, data systems for AM and FM sound broadcasting, television broadcasting, data broadcast in digital video broadcasting systems, satellite data broadcasting, broadband wireless systems, cable TV distribution, multimedia broadcasting with some examples. Future developments are discussed in a separate subchapter.

CHAPTER 2

# 2. 	Digital Audio Broadcasting

2.1	Introduction

Radio broadcasting is an attractive medium for disseminating information and entertainment oriented programmes from a central place to a very large population. The main attraction of radio broadcasting is that the cost of the receiver is very low and affordable to the public. But this technology has become very old and has not kept pace with the various developments in technology  that would make it a more effective tool.

Existing radio broadcasting in analogue mode in the medium wave, short-wave and FM bands has some drawbacks and needs improvement in quality to match with the quality of digital radio production techniques. Broadcasting in these bands has a restricted bandwidth of 9-10 kHz only with a dynamic range of only some 40 dB. With digital transmission technology, these figures can be improved with frequency response extending up to 20 kHz, similar to Compact Disk (CD) quality, with a dynamic range of about 90 dB. In addition, digital transmission provides flexibility for transmitting multi-channel programmes with added attractive value added services.

In summary some benefits of digital broadcasting are:

- More programme choice

- Near CD quality

- Dynamic programming re-allocation (balancing quality expectations versus quantity)

- Improved service reliability and coverage

- Disturbance free reception on mobile and portable receivers

- User-friendly receivers with programme selection by programme content or language and not by transmission frequency

- Terrestrial and satellite reception with the same receiver

- The opportunity to offer data services with the prospect of additional income

- The chance to become competitive with other digital and non-digital media

- Today there are several digital sound broadcasting systems, some in experimental stages, some in Pilot stages, some in implementation stages and some in operation. The operating frequency ranges from 550 kHz to 1.5 GHz.

2.2	T-DSB objectives

For Terrestrial Digital Sound Broadcasting (T-DSB), the Digital System A developed under the Eureka-147 project in Europe is the most mature and is already in use not only in Europe but in many countries over the world. It was completely defined under the ETSI standard ETS 300 401 and has been recommended by the ITU-R in Recommendation ITU-R BS.1114: "Systems for terrestrial digital sound broadcasting to vehicular, portable and fixed receivers in the frequency range 30-3 000 MHz".

2.2.1	CD Quality Reception

Analogue VHF signals suffer frequency selective fading when received by a simple vertical monopole (whip-antenna) in dense-urban area because of multipath reflections.

2.2.1.1	Frequency Selective Fading

Frequency selective fading affects the quality of the received audio signal in the form of impaired frequency response, non-linear distortion and poor signal-to-noise ratio. Such fading results from a combination of direct and delayed signals (reflected from terrestrials objects) at the receiving antenna.

Due to path difference of the direct and reflected signals, there is a phase difference between these signals. At some locations they are in phase (when signal strengthens) and in others, they are out-of-phase (creating nulls in the pass-band). Hence, some frequencies within the band are severely attenuated and some get enhanced in a dynamic way while the receiver in moving. Figure 2.1. gives an example of the spectrum of Eureka-147 system when it is affected by frequency selective fading .

Figure 2.1. Spectrum of the EUREKA-147 system affected by frequency selective fading

The phenomenon causes partial or total loss of information in FM and affects the received audio quality. As for mobile reception in a car, the situation is no different, and tends to be rather worse, due to continuous and rapid fluctuation of the signal.

In most difficult locations, where short multipath will exist due to the presence of large reflecting surfaces located rather close to the receiver (e.g., large buildings with flat sides), the width of a frequency selective fade can be as much as 300 kHz. This means that there could be complete loss of the FM signal when it is received with a small whip-antenna in a car. A directive, receive antenna can reduce greatly this type of fading in fixed location.  The use of antenna diversity (i.e., two antennas located at a distance of at least λ/2 with appropriate power combining) can also greatly reduce this type of fading in both fixed and mobile reception.

In the Eureka-147 system, the useful RF bandwidth is 1 500 kHz, thus ensuring that, even in such a difficult location, at least four-fifth of the bandwidth is received properly.

2.2.1.2 	Digital transmission in multipath environment

CD quality requires high-speed data-rate transmission. High data-rate not only occupies large bandwidth per channel, it creates additional problem for reception in multipath environment, other than the one mentioned before.

In a typical digital transmission, higher the data-rate is, smaller is the ‘symbol period’.  As such, if the data rate of the ‘information’ to be transmitted is high, the symbol period will need to be very small.

Small ‘symbol period’ easily triggers Inter Symbol Interference (ISI). The delayed symbols resulting from multipath present in almost every type of natural terrain will fall on to the following symbols in the stream and create severe interference and bit errors.

A technique, which uses several orthogonally spaced carriers for modulation, within the allocated bandwidth, has been found to be the best solution for digital transmission in a multipath situation. Here, all the carriers are simultaneously modulated with a small portion of the total data, thus resulting in a large symbol period. In short, this type of modulation is termed OFDM (Orthogonal Frequency Division Multiplexing). The carriers are orthogonal to each other so that there is nominally no mutual interference as shown in Figure 2.2 for 2 carriers. This unique technique was developed by the CCETT, France.

However, if the channel becomes time-variant as in the case of a receiver onboard a vehicle, there is a limit to stretching the symbol period because the decoding begins to be affected by the variation of the channel which can be seen as Doppler spread.  The right transmission mode has to be selected for the Eureka-147 system (transmission modes I,II,III and IV) depending on the carrier frequency and the maximum speed of the vehicle for which non-impaired reception has to be provided.

Fig-4.2.2.1.2.1

Figure 2.2  Orthogonal carriers

Figure 2.2. Orthogonal carriers

Further, it has been found that if a buffer, corresponding to 25% of the symbol period (guard interval), is inserted before each symbol to absorb the possible multipath interference (delayed symbols) caused by the natural terrain and man-made built up areas, then reception will not be affected by such delayed symbols as the receiver ignores them (the receiver is programmed to not decode the signal during this guard interval). The symbols delayed by more than this guard interval are expected to be attenuated enough to not cause inter-symbol interference.

The guard interval & symbol period of direct and delayed signals is shown in Figure 2.3.

Figure 2.3.  Guard interval absorbing the delayed symbols

To make transmission further rugged, the Eureka-147 system employs frequency and time interleaving for better reception. Frequency interleaving helps in fixed reception whereas time and frequency interleaving helps in mobile reception.

2.2.1.2.1		Frequency Interleaving

Loss of information due to frequency selective fading is further reduced in the Eureka-147 system by the application of ‘frequency interleaving’. In a fixed reception, attenuation takes place mostly on contiguous carriers of the OFDM as path length of the signal, delays and wavelength of the carriers remain more or less unaltered. For this reason, bit streams of every program is not loaded on to the same or adjacent carriers but spread over a large number of widely separated carriers within the bandwidth. Thus at no time are all the bits of the same program lost but rather some portions of every program is lost.  As will be seen in section 2.2.1.2.3, the channel error correction does the rest to recover the transmitted signal.

2.2.1.2.2	Time Interleaving

In a mobile reception environment, the randomisation of channel errors carried out by frequency interleaving can be improved greatly by time interleaving because of the fact that the channel impulse response (delay profile) changes continuously due to change in the surrounding. The time interleaving is performed by spreading successive bit streams several ‘Frames’ apart or delayed such that in the event of momentary (a few tens of ms) obstruction or interference in the signal, adjacent bits are not lost but they are re-constructed from different programs or data-streams that are received properly.  The higher is the speed, better is the performance of the time interleaving and better is the reception  (within the range permitted by the Doppler spread as seen in section 2.2.1.2). Since DVB-T does not have time-interleaving, it cannot perform as well in a mobile environment and can never replace T-DAB in a mobile multipath environment.

The interleaving and de-interleaving schemes in the transmitter and receiver respectively are arranged such that the total delay (delay in encoding and decoding) of each bit at the decoder output remains the same for every bit (about 400 ms in the Eureka-147 system).

2.2.1.2.3 Channel error correction

The time and frequency interleaving features mentioned above are used to randomise the channel errors so that the channel error correction scheme performs at its best.  In the Eureka-147 system, a convolutional coding scheme based on a simple mother code (1/2 rate, i.e., adding 1 bit of redundancy for each useful bit) is used with punctured coding to allow variation of the coding rate for different services in the transmission multiplex.  At the receiver, the demodulated bit stream containing the channel errors randomised by interleaving is presented to a Viterbi decoder that recovers most of these errors.  Convolutional coding is crucial to this modulation scheme and it is why the transmission is called Coded Orthogonal Frequency Division Multiplexing or COFDM.

All the above techniques are employed in the Eureka-147 system and thus ensure CD quality reception for fixed, portable and mobile receivers hitherto not possible with conventional transmission techniques.

2.2.2	Multiple choice of programs

One uncompressed CD quality stereo channel produces bit rate in the order of 1.5 Mbit/s. This with 1/2 rate convolution coding for error correction would result in about 3 Mbit/s.  This amount of data modulated by binary phase-shift keying (BPSK) would occupy a bandwidth of 3 MHz and for quaternary phase-shift keying (QPSK) 1.5 MHz. This is highly spectrum inefficient and cannot be allowed for broadcasting.

Thus, there is a serious need for bit rate reduction. Fortunately, the ISO Motion Picture Experts Group (MPEG) came out with perceptual compression algorithms based on human-psychophysical-limitations, that could reduce the data-rate to about 1/8th or in a range between 128 kbit/sand 256 kb/s without subjective impairment in sound quality.  This corresponds to a range between 192 kbit/s and 224 kbit/s for the MPEG layer 2 codec used in the Eureka-147 DAB system.  This was a milestone that made the dream of digital sound broadcasting a reality.

Such compression of the audio signal allows for a number of CD quality channels along with ancillary data being carried in a Eureka-147 multiplex in 1.5 MHz of RF spectrum.

For low quality programs like ‘News’, the same 1.5 MHz bandwidth can carry as many as 30 different channels. So, a digital transmission like that offered by the Eureka-147 system can offer a choice of several program channels to a listener.

2.2.3	Multimedia Feature

The large bandwidth of the Eureka-147 system allows for a large data pay-load. Such large pay-load not only allows for many channels of audio, but can also be used to transmit some video, text and other data services.   It could accommodate multicasting currently carried on the Internet and thus help relieve the Web from potential congestion when the number of users of the same program is large. It is interesting to note that ‘Deutsche Telekom’ has experienced with transmission of commercial video using the Eureka-147 system. Fraunhofer Institute in Erlangen, Germany, has developed software that encodes the video signal and transmits it as Motion Programme Associated Data. A new generation of data servers for DAB applications such as the  ‘Multimedia Data Servers’ will facilitate multimedia broadcasting through Digital Sound Broadcasting.

2.2.4	Data Service

The DAB multiplex can provide different modes of data services such as:

- Programme Associated Data (PAD)

- Fast Information Data Channel (FIDC)

- Packet Mode

2.2.4.1 	Programme Associated Data (PAD)

In each DAB-audio frame there is provision for 2 bytes of ancillary data called F-PAD (Fixed PAD) that can be used for data applications. The bit rate for F-PAD can be either 667 bit/s or 333bit/s depending on the sampling frequency of the audio signal (48 or 24 kHz). It can carry dynamic range control, music/speech indication, etc. Further, the Extended PAD (X-PAD) which can carry as much as 64 kbit/s (by removing it from the audio capacity) can be used for program related ancillary services such as dynamic label service, text and multimedia objects using the MOT protocol (see section 2.2.4.4),. One example can be for music information. When music is heard, one can access the data about the singer, composer or the current rating etc. Furthermore the X-PAD could carry a carrousel of pictures related to the program being heard.

2.2.4.2	Fast Information Data Channel (FIDC)

As part of the Fast-Information channel (FIC), some data can be transported on specific Fast-Information Groups (FIG’s). In this mode, data services with low data rates cumulating to up to 20 kbit/s can be carried.  Such information would be available to any listener tuned to the current multiplex whatever service he has selected on this multiplex. Some examples are Traffic Message channel (TMC) and Emergency Warning System (EWS). Traffic information is important for people in a moving vehicle. Since DAB is trying to attract this category of mobile audience, this service is very important. Information on traffic and road conditions can be overlaid on digital road maps if the receiver is so equipped and offers an overview of the road situation in an area and also helps to identify an alternate route. Due to its limited capacity, this FIDC is not used for multimedia applications.

Another useful service can be weather updates. The weather information service from the Meteorological department can be displayed as ‘easy to read’ graphics/charts if the receiver is so equipped. Stock market data (regional, national) can also be carried and represented in graphic form if the receiver is so equipped. Other data like flight schedules, railway reservation position, bullion rates etc. are other useful services that can be carried by FIDC.

2.2.4.3	Packet Mode

In this mode, data are split into two smaller entities on two hierarchy levels as Data Groups and packets.  Data Group header has information on repetition rate, multiplexing of different Data Groups like conditional access and MOT (see next section).  Each of the possible streams (up to 63) contained in the DAB multiplex can carry up to 1024 data packet streams.

2.2.4.4 	MOT Protocol

Multimedia Object Transport (MOT) protocol is used to carry multimedia over the DAB channel. Presently data types carried over the Internet such as JPEG, MPEG audio/video and HTML are supported.  A synchronous slideshow is possible with MOT. In this, a sequence of objects is presented automatically. A slide show consists of a sequence of JPEG images (carrousel). In timed slideshow, each slide is synchronised with the corresponding sound.

2.2.4.5 	Broadcast Website

This service provides Internet-like data service with or without local interactivity. Many of the services available on an Internet server can be broadcast in DAB mode without any need for interactivity. Weather Data, Traffic information, Tourist guide etc. are examples of such information. In some cases, limited interactivity is possible using local telephone lines for fixed receivers or mobile phones for car receivers.

Non-Multimedia Services

Global Positioning System (GPS) correction data can be transmitted through DAB and fed to the IEEE RS-232-C interface port of a GPS receiver to improve its resolution.

It is also possible to download updated software to the receivers to keep pace with the new standards, to test additional features and to incorporate necessary feedback. For this, the receiver has to wait for a few minutes until the downloading is done. Such updates help to reduce the efforts and cost compared to physically downloading the updates to individual receiver.

Other applications for closed user groups (like Internet) with conditional access are also possible.

2.2.5	Adaptive audio-bandwidth

As different types of information (audio, video & text) require data at different speed, the efficiency of the DAB transmission greatly depends on the content of the multiplex being carried.  This content will vary with time depending on the quality of the audio channels (high quality music, news, speech, etc.), their number and the capacity of the data services. Modern digital transmission systems such as the Eureka-147 system can allocate variable multiplex capacity per service.  Such flexible multiplex capability is possible through thetransmission of the  MCI (Multiplex Configuration Information) and transmitted through Fast Information Channel (FIC) of DAB-Frame. Due to this feature data-rate can be varied dynamically (every 24 msec) in accordance with the type of program being carried.  The change in data-rate is signalled via FIC. The net result is a fully utilized multiplex by adjusting the data services capacity (non-real time services) as a function of the need for the audio services (real time services) which is dependent on the quality of the program to be carried..

2.2.6	Spectrum Efficiency

To ensure high quality reception in high percentage of location (99% time, 99% locations), the RF bandwidth needed to be 1500 kHz even to carry less than the full multiplex such as one program to reduce the effect of the frequency selective fading as explained in section 2.2.1.1. The Eureka-147 system preserves spectrum efficiency by carrying several channels of audio, videotext and the data in the multiplex. Each type of data is protected separately (using various FEC code rates through punctured coding), depending on its importance, before multiplexing. For a nominal useful bandwidth of 1536 MHz where a QPSK modulation (2 bit/s/Hz) is used and removing the capacity taken by the guard interval which represents 20% of the total symbol period and the capacity of the FIC, the resulting raw bit rate that can be carried by the Eureka-147 system is about 2.3 Mbit/s.

In DAB, this multiplexed data is distributed over 1536 carriers (for VHF transmission in Mode-I). The spacing between the carriers is 1 kHz in Mode-I, 4 kHz in Mode II (for UHF) and 8 kHz in Mode III (for 1.5 GHz, 2.3GHz and 2.5 GHz bands). Later, another Mode IV was introduced with 2 kHz spacing for 1.5 GHz terrestrial transmission. There is a direct relation between the useful symbol period and the carrier spacing. For 1 kHz spacing, the useful symbol period is 1ms. For 2 kHz, 4 kHz and 8 kHz, the corresponding useful symbol periods are 0.5 ms, 0.25 ms and 0.125 ms respectively. The corresponding guard-intervals represent about 25% of the useful symbol period and the number of carriers in the channel bandwidth are also related to the useful symbol periods as indicated in Table 2.1,

| Parameter | Mode I | Mode II | Mode III | Mode IV |
| --- | --- | --- | --- | --- |
| Carrier spacing (kHz) | 1 | 4 | 8 | 2 |
| Useful symbol period (msec) | 1000 | 250 | 125 | 500 |
| Guard Interval (msec) | 246 | 62 | 31 | 123 |
| Total symbol period (msec) (D+ts) | 1246 | 312 | 156 | 623 |
| No. of carriers | 1536 | 384 | 192 | 768 |
| Total frame duration (ms) | 96 | 24 | 24 | 48 |

Table 2.1. Key parameters of the four transmission modes

2.2.7	Some features for planning

Modern low-power DSB transmitters seem to be more efficient and effective at providing smooth and uniform coverage in terms of power requirement, coverage area and interference. Here too Eureka-147 has an edge over the other terrestrial systems as it has built-in protection against multipath, thanks to the presence of the guard interval. A repeater transmitter operating on the same frequency can be used effectively for illumination of shadowed areas or high noise locations near traffic signals. A directional high-gain antenna is used for receiving the DAB signal from the main transmitter in a line-of-sight condition, amplified and re-transmitted at the same frequency with an omni-directional antenna close to the locations under discussion. Figure 2.4. shows the arrangement. Due to the intrinsic property of the Eureka-147 system, the receivers consider all signals present at their input as echoes matter whether they are active or passive and used their power sum as long as they fall within the Guard Interval to decode the signal. These ‘repeaters’ are often called ‘gap fillers’.

Figure. 2.4.  Gap Filler Transmitter

With the help of gap-fillers, small gaps such as those found in highly built urban cores, tunnel, bridges, etc.  Such gap-fillers can also be used to extend the coverage of a main transmitter by using higher power and sectoral antennas directed outward. However, because of the limitations resulting from the locations of the transmit sites, the related geometry and the width of the guard interval, the extension of the coverage is somewhat limited. For further coverage extension, Single Frequency Network is ideal. Here, the data multiplexed is distributed by other means to several transmitters, strategically located for uniform coverage of the target area, with power relative delay optimised for best coverage. In order to re-synchronize the data multiplex at each transmitter and adjust the relative delay, the multiplex needs to be time stamped at the origination point. For this reason, before distribution of ITU-T Rec. G-703 (Network Independent) data, frame structure is converted to ITU-T Rec. G-704 (Network Adapted) wherein Time Stamping is provided.

At each transmitter, the received multiplex is synchronised with time & frequency reference, received from a GPS receiver and both frequency and data of all the transmitters are thus locked and synchronised with this common reference. Studies indicate that the field-strength from two transmitters of equal power can be enhanced by as much as 7.5 dB for an availability of 99% when a field strength standard deviation of 5.5 dB is assumed. Figure 2.5. shows a graph that highlights the advantage of SFN planning.

Figure 2.5. Advantage of SFN planning

2.3	Different T-DSB Systems

2.3.1	T-DSB above 30 MHz

Beginning a new DSB service in VHF-TV and FM bands is difficult in most countries because of the existing heavy use of these bands for TV broadcasting. Fortunately, TV channel 12 has recently been freed in a number of countries in Europe and this has made some 7 to 8 MHz available around 235 MHz. Four blocks of 1.5 MHz were planned in 1995 in Wiesbaden to provide one service block per country in Europe for country-wide coverage either by stand-alone or SFN mode.

A frequency band from 1452 MHz-1492 MHz was allocated at the WARC-92 on a worldwide basis (except in the US) for satellite and terrestrial sound broadcasting.  Two other bands at 2.3 GHz and 2.5 GHz were also allocated for certain countries in the Americas and Asia for the same purpose.  Initially there was apprehension whether such a high frequency could work properly in terrestrial mode. Successful experimentation by the Communications Research Centre, Canada has put to rest all doubts. Broadcasting at 1.5 GHz proved to be equally good for small countries as well as for covering local areas such as cities where more radio programming is needed. At 1.5 GHz, the T-DSB transmitter density needs to be higher than at VHF for SFN and may not be economical for a large country. However, there is some benefit at 1.5 GHz that may be of special interest from the ‘noise’ point of view. In many developing countries, vehicular and man-made noise is very high even in the upper VHF-TV band. As a result, in spite of high sensitivity of the receiver, reception fails in many locations prematurely, due to poor C/N.  Vehicular and man-made noise of this type is not present at 1.5 GHz.

2.3.1.1	IBOC & IBAC

Because of the structure of the broadcast industry in the USA and the non-availability of spectrum for terrestrial DSB, the broadcast industry has proposed an In Band On Channel (IBOC) system that is aimed at progressively replacing the current FM broadcast by using the same channels in the FM band. The basic principle is to use, on a non-interfering basis the same FM channel which is used by the analogue host FM program for one stereo digital channel with some data. The digital channel is kept below the FCC mask by using the sidebands, such that there is nominally no interference to analogue reception.  The modulation used in these sidebands is very similar to the COFDM modulation of the Eureka-147 system.  However, since the operation is around 100 MHz, the symbol period, carrier spacing and the guard interval scaled accordingly.  The digital receiver uses a stronger error-correcting technique to be able to decode the necessary information from the multiple digital carriers in presence of the strong FM signal. Figure 2.6 shows the digital spectrum of the system along the analogue FM spectrum. It is understood that the system has gone through rigorous testing by the system proponent and that it has been recommended as a US broadcast standard to the FCC.

Figure 2.6. FM-IBOC spectrum

2.3.1.2 	ISDB of Japan

Japan has developed a family of digital broadcasting systems called Integrated Service Digital Broadcasting (ISDB) and as the name suggests, it integrates several services, for example terrestrial radio for reception in fixed, portable and cars with a small pocket receiver, Standard-Definition TV & High Definition TV in respective receivers and advanced data broadcasting services.  This family of systems is expected to operate in the TV VHF and UHF bands. It uses COFDM with some more flexibility than the Eureka-147 system for terrestrial transmissions with 13 contiguous blocks of 432 kHz each to fill a 6 MHz TV channel. Each of 432 kHz band contains several carriers. Figure 2.7 shows the spectrum of the system.

Figure 2.7. Spectrum for ISDB

2.3.1.3		Satellite - Digital Sound Broadcasting (S-DSB)

2.3.1.3.1	Worldspace U.S.A

The most notable system in this area is the system launched by WorldSpace, USA. Small hand held receivers are able to provide reception directly from a GSO Satellite. Uplinking can be done over a wide area covered by a 10o beamwidth receiving beam at the satellite and the receiver can tune to any two of the three 6.5o beams transmitted from the satellite as shown in the Figure 2.8. . The coverage shown is for ‘AsiaStar’ located at 105o longitude East. Each beam can offer up to 96 audio channels depending on the quality required. In open area, one can enjoy digital sound broadcasting along with the display of the station-name and other information. Figure 2.9. shows the picture of a WorldSpace prototype receiver.

Figure 2.8  AsiaStar coverage

Figure 2.9  World Space receiver

WorldSpace claims that limited amount of mobile reception is also possible, as long there is LOS between the satellite and the receive patch antenna.

The system already has 2 satellites over Asia (AsiaStar) and Africa (AfriStar at 21o longitude East), and a third one is planned to cover Central and South-America. Each satellite can provide 3 beams. The transmission from satellite is done at 1.5 GHz and uplinking can be done from various Feeder Link Stations simultaneously in the 7 GHz band.

It uses FDMA carriers for the uplink and TDM on a single carrier per beam for the downlink. Each carrier occupies a bandwidth of 2.5 MHz and uses a different frequency for each of the 3 beams. As a result, in comparison to the Eureka-147 system, it makes a better use of the non-linear satellite transponder due to its single carrier modulation it is less rugged but provides excellent quality as long LOS is present with US $150 hand held Receiver.

There are three carriers transmitted per satellite and each carrier is constituted of 96 primary channels of 16 kbit/s each.  The various services to be transmitted can be assigned a capacity varying from 8 kbit/s to 128 kbit/s in steps of 8 kbit/s.  Three versions of the MPEG Layer 3 audio coding scheme can be used: MPEG-1 Layer 3, MPEG-2 Layer 3 (half sampling rate) and the extension MPEG-2.5 (quarter sampling rate).

For each carrier, a separate TWT Amplifier is used. Since a single-carrier QPSK modulation is used, the transponder can be operated very close to saturation with minimal impact on the resulting bit-error ratio. Because reception is expected to take place in LOS and fixed conditions,  a coherent-QPSK de-modulation scheme can be used. Due to the near-optimum use of the satellite transponder, the absence of the guard interval and the use of coherent demodulation, there is an advantage of about 7 dB in satellite power over the use of the Eureka-147 system over satellite.

A detailed link budget analysis indicated that the WorldSpace system provides for a 9 dB margin at the beam centre can be provided.

Single-carrier modulation is preferable for satellite delivery because of the higher sensitivity of COFDM to inter-modulation distortion (IMD) products generated by the satellite transponder near saturation.  For consistent reception in non-line-of-sight conditions, very high link margins in the range of 20 to 30 dB would be required. As a result, neither World-Space nor Eureka-147 systems would be able to offer direct reception from satellite in Portables and Mobile receivers at the present level of satellite technology from GSO,.

For terrestrial DSB, the modulation used in the Eureka-147 system has proved to be the best approach.  As a result, WorldSpace, in trying to extend their service to non-LOS reception has developed the technology to carry their satellite multiplex terrestrially using a COFDM type modulation called Multicarrier Modulation (MCM).  This allows terrestrial repeaters to extend the coverage to built-up areas.

2.3.1.4 	S-DARS at 2.3 GHz

On January 12th, 1995 FCC allowed the 2310 to 2360 MHz frequency band exclusively for ‘Digital Audio Radio Services’ for satellite & complementary terrestrial services. Later, two systems (XM-Radio and Sirius) were allowed to operate in two slots of 12.5 MHz in the 2.3 GHz band.  These systems have started operation using single-carrier modulation on the satellite and multi-carrier modulation for the terrestrial re-transmitters.  XM-Radio uses two GSO satellites whereas Sirius uses three satellites on a highly-inclined elliptical orbit.

Besides the USA, only a few countries opted for 2.3 GHz for S-DSB transmission. India also was allocated the same 2.3 GHz and the Indian Space Research Organisation (ISRO) developed a satellite system for direct reception of digital audio in fixed conditions with a small patch-antenna.  Following the WARC-92, Mexico added a footnote to the Frequency Allocation Table for the use of the 2.3 GHz band.

2.3.1.5	Mediastar System

Daimler-Benz Aero-Space AG (DASA), Germany had planned to introduce a Commercial Direct Broadcast Satellite System for audio and Data Services to Mobile and Fixed Audiences based on Eureka 147 DAB standard in the late 80’s but decided in the mid 90’s to not pursue the project.

Lately, two new system proposals to provide a sound broadcasting service from satellite over Europe in the 1.5 GHz band were proposed.  One is known as Global Radio and the other is sponsored by Alcatel and WorldSpace.  These systems are expected to provide a service from satellite as well as a terrestrial extension as the WorldSpace system and the two US systems.  They are in an early phase of development.

2.3.2	DSB System below 30 MHz

2.3.2.1	Medium Wave Band

This band is heavily congested and there is no room for new services. Night time coverage area of analogue AM transmission gets severely reduced due to interference from powerful distant transmitters caused by skip mode propagation, and due to the reduction in transmit power to reduce the interference towards other areas. Age old MW transmission is a time-tested, consistent delivery method for daytime coverage over large areas even today. However, it is far from today’s expectation in terms of audio quality and service reliability especially in comparison to recorded audio material.

2.3.2.1.1	Digital MW by Deutsch Telekom / Telefunken/ Continental

In this context, Deutsch Telekom & Telefunken Sendertechnik together developed a Digital Sound Broadcasting (T2M) method by which not only nighttime coverage is kept almost unaffected, the transmission power requirement is also reduced. The system uses Amplitude phase shift keying or APSK - modulation. The experimental results, indicating daytime/night time coverage area and transmission Power requirement of analogue AM & Digital AM are given in Figure 2.13 and 2.14. Analogue & Digital transmission can co-exist as long interfering analogue carrier is not more than 30 dB above the digital carrier level.

Figure 2.13. Digital Audio Broadcasting in the Band below 30 MHz

Figure 2.14  Measured spectrum

2.3.2.2 		Short-wave Band

2.3.2.2.1	‘Skywave-2000' of Thomcast

This system will greatly reduce sporadicity, inconsistency and inferior quality reception caused by ionospheric propagation by the use of QAM and strong error correcting codes.  It uses COFDM modulated carriers occupying half of the nominal HF channel bandwidth, thus 4.5 kHz.  The remaining 4.5 kHz is kept for existing analogue transmission in SSB or VSB mode.  M/s Thomcast, accordingly has modified their High Power ShortWave Transmitter for such simulcast transmission.  The new receiver can work for both analogue and digital reception.   An integrated circuit has been developed which can process the digital part and deliver audio to the existing audio amplifier of the receiver.

The receivers will also have a small screen for displaying various related information about the transmission. In later stage the total bandwidth would be dedicated for digital transmission with higher data payload and with reduced transmission power.

2.3.2.2.2	System by Voice of America & Jet propulsion Lab

Successful experimentation indicates that HF transmission can be made better and continue for a long time by the use of data compression, error correction coding, digital phase modulation, channel equalisation and the use of lineariser in the HF transmitter. Experiments were also made to determine various parameters for optimum operation such as the number of transmission hops, operating frequency, data-rate and the most suitable order of modulation.

Unlike ‘SkyWave-2000’, the system uses the total HF channel bandwidth for digital transmission.

2.3.2.2.3	‘USADR’ in ShortWave

Unlike VOA/JPL system, USADR uses the same analogue channel as the analogue AM transmission for Digital Radio by reducing the level of the digital carriers low enough such that it meets the FCC RF mask for the AM Band. It uses OFDM carriers over a 40 kHz bandwidth and co-exists with the analogue carrier within the same channel. Close to the centre of the channel, over ± 10.2 kHz, the transmission speed in slower, resulting in larger ‘symbol period’ and beyond ±10.2 kHz, the symbol rate is faster. Perceptual Audio Coding (PAC) at data-rate of 48 kbit/s are planned to be used. The spectrum is shown in Figure 2.15.

Figure 2.15. In-Band On Channel

2.3.2.2.4	Digital H.F. by CCETT, France

The CCETT propose to use its famous Isotropic Orthogonal Transform Algorithm (IOTA) for HF circuit in addition to COFDM. It proved equally resistant to ‘multipath’ generated on the ionospheric paths. Instead of a convolutional coding, it uses ‘Turbo-Coding’, which provides an additional 2 dB coding gain. The use of IOTA provides another 2 dB gain i.e. a total of 4 dB advantage over the classical (COFDM + convolution) coding used in the Eureka 147 system.

2.3.3	Towards a Common goal for a World-wide standard for AM Broadcasting

As described above, several systems are emerging for distribution and delivery of digital sound broadcasting. All of them are divergent in nature. As a result, this is causing concerns and would eventually put financial pressure on the broadcaster & network operators. The problem is further compounded due to the maturity of the market in some parts of the world with varying degree of regulation risks making the market inaccessible.  In view of these factors, broadcasters, network operators and manufactures are finding investment in such a fragmented market very risky.

In this context, the consortium Digital Radio Mondiale (DRM) has been formed and is planning to introduce a single worldwide digital transmission system in the sound broadcasting bands below 30 MHz that would be a standard for both ITU-R and ISO. In early 2001, the DRM system became an ETSI Standard.  In April 2001, the DRM system became part of Recommendation ITU-R BS.1514 for digital sound broadcasting in bands below 30 MHz.  In March 2002, the DRM on-air system was endorsed by the International Electrotechnical Committee (IEC), which published its DRM Publicly Available Specification (PAS 62272-1). Extensive testing in shortwave bands is still being conducted in various part of the world.  The market launch of the DRM system is planned for 2003.

Some features of DRM

- Several ‘Modes’ to suit Broadcasters' needs e.g., more channels, less rugged, lower quality or highly rugged, lesser channel, higher quality or any combination thereof.

- Only one non-proprietary system to be used Worldwide below 30 MHz for digital broadcasting and information datacasting. The same DRM digital receiver will work in any part of the World, no matter where it has been bought.

- Initially, it will co-exist with analogue in simulcast mode, within a single channel. Later, the channel will be dedicated to digital transmission alone, which would be of better quality. However, analogue transmission will not stop completely but continue with some dedicated channels exclusively for the analogue receiver population. This phase has been termed ‘Multicast’.

- The system will be able to deliver audio and/or data (PAD, service information, multimedia and/or various data services). All these program components will be automatically and appropriately accommodated by the various receivers and classes of receivers.

- The quality of the audio program will be much better.  Improvement in compression will be at the transmitting end so that no change is required for the 1st generation of receivers.

- Data & information will be structured in such a way that transmission, reception & presentation will be possible. The data protocols will be fully technically specified.

- Spectral efficiency will be enhanced through the use of Single Frequency Networks. The system will be compatible with ITU channel spacing.

Keeping the above features in mind, DRM proposes to make use of both existing 9 kHz (2 x 4.5.kHz) and 10 kHz (2 x 5 kHz) analogue channels for DRM transmission in the early phase. In ‘Simulcast’ mode, the analogue channel will be placed at the centre between two 4.5 kHz or 5 kHz digital channels. The level of the analogue carrier will need to be 30 dB above the digital carrier as required in the T2M system. However, the total bandwidth for such simulcast transmission will be around 15 kHz approximately and will necessitate new HF channel re-distribution and co-ordination.

The DRM system combines some features of T2M system and some of the ‘SkyWave-2000’ system.  The DRM system overview is given in Figure 2.16.

Figure 2.16. DRM System

2.4. List of Abbreviations

AM	Amplitude modulation

APSK	Amplitude phase shift keying modulation

Block coding	Forward error correction coding based on combinational coding of a block of successive bits to add overhead bits to the transmission.  The combination is repeated on each successive block

BPSK	Binary phase-shift keying

C/N	Carrier to noise ratio (usually expressed in dB)

CCETT	Centre commun en télédiffusion et telecommunication, Rennes, France

CD quality	audio quality equivalent to that recorded on CD’s (16 bit per channel, right and left at 44.1 KHz sampling rate

CD	compact disk, digital audio recording on laser disk

COFDM	Coded Orthogonal Frequency Division Multiplexing

Convolutional	Forward error correction coding based on combinational coding of a number

coding	of successive bits to add overhead bits to the transmission.  The combination is repeated on each successive bit.

DB	decibel, relative logarithmic unit where 0 dB corresponds to a ratio of 1 and 10 dB to a ratio of 10

Digital System A	Eureka 147 DAB system, ETSI standard ETS 300 401, see ITU-R Handbook on “Digital Sound Broadcasting to vehicular, portable and fixed receivers”, Geneva, 2002

Doppler shift	Frequency shift of a signal caused by a mobile element in the transmission path (transmitter, receiver or reflective surface)

Doppler spread	Ensemble of different Doppler shifts on various multipaths

DRM	Digital Radio Mondiale

Dynamic range	Difference in dB between the maximum audio signal level and the level of noise in the audio channel.

DSB	Digital Sound Broadcasting

DVB-T	Terrestrial Digital Video Broadcasting

Dynamic range	Difference in dB between the maximum audio signal level and the level of noise in the audio channel

EWS	Emergency Warning System

FCC	Federal Communications Commission (US regulatory body for civilian communication systems)

FDMA	Frequency Division Multiple Access

FIC	Fast Information Channel

FIC	Fast-Information Channel

FIDC	Fast Information Data Channel

FIG	Fast-Information Groups

FM band	88-108 MHz, Band II, band where Frequency Modulated sound broadcasting takes place

F-PAD	Fixed PAD

GPS	Global Positioning System

GSO	Geostationary Orbit

HF	High Frequency band: 3 MHz - 30 MHz

HTML	HyperText Markup Language

IBOC	In Band On Channel: technique proposed in the USA to transmit DSB in the FM broadcasting band using the same spectrum as that used by the host FM channel

IEEE	Institute of Electrical and Electronics Engineers

IMD	Inter-modulation distortion

IOTA	Isotropic Orthogonal Transform Algorithm

ISDB	Integrated Service Digital Broadcasting

ISI	 Inter symbol interference

ISO	International Standards Organization

ITU-R	Radiocommunication Sector of the International Telecommunication Union

JPEG	ISO Joint Expert Group

LOS	Line of sight

MCI	Multiplex Configuration Information

MCM 	Multicarrier Modulation

Monopole	Simple antenna made of a ground plane and one side of a dipole

MOT	Multimedia Object Transport

MPEG	ISO Motion Picture Experts Group

Multipath	multiple signals carrying the same information seen at the receiver with various excess delays and attenuation caused by the various paths taken by the signals through reflections to reach the receiver

MW	Medium Wave band: 300 kHz - 3 MHz

OFDM	Orthogonal Frequency Division Multiplexing

PAD	Programme Associated Data

Punctured	based on a simple convolutional mother code (e.g., 1/2 rate, i.e., adding 1 bit

coding	of redundancy for each useful bit), some of the bits to be transmitted are removed for the transmission to reduce the overhead and reconstituted at the reception

QPSK 	quaternary phase-shift keying

S/N	Signal to noise ratio, usually expressed in dB

S-DARS	Satellite Digital Audio Radio Service (USA nomenclature)

S-DSB	Satellite Digital Sound Broadcasting

T-DAB	Terrestrial Digital Audio Broadcasting

TDM	Time Division Multiplex

T-DSB	Terrestrial Digital Sound Broadcasting

T-DSB	Terrestrial Digital Sound Broadcasting

TMC	Traffic Message Channel

TWTA	Travelling Wave Tube Amplifier

VHF	Very High Frrequency range: 30 MHz - 300 MHz

VOA/JPL	Voice of America & Jet propulsion Lab

X-PAD	Extended PAD

2.5 References

ITU-R Handbook on “Digital Sound Broadcasting to vehicular, portable and fixed receivers”, Geneva, 2002

CHAPTER 3

3.	DIGITAL TELEVISION BROADCAST SERVICES

3.1	Overview

3.1.1	Introduction

Digital coding and transmission of television pictures and sound signals offers the following advantages as compared to the existing analogue systems.

- more programme transmission per RF channel

- higher quality pictures and sound

- increased service area coverage

- reduced transmission powers

- some combination of the above

Both SDTV (Standard Definition Television) and HDTV (High Definition Television) signal formats are being considered for digital transmission.  The term SDTV is used to signify a digital television system that has the same resolution as the existing analogue systems such as PAL, NTSC etc. but which is expected to deliver a better quality picture on the advanced receivers because the artefacts of the current analogue systems would be avoided, unless of course the signal has to be converted to back to analogue for display on a conventional receiver.  The HDTV has a resolution of approximately twice that of conventional television in both horizontal and vertical dimensions and a picture aspect ratio of 16:9.

For SDTV, depending on the number of lines per frame and field rate, two systems are available viz the 625/50 Hz system and 525/60 Hz system.  ITU Recommendations BT 601 [2] and BT 656 [2] give all the details of these two systems.  Both systems have 720 active pixels per line.

For HDTV, a large number of presentation formats are under consideration.  In the USA, the formats used for digital terrestrial HDTV are shown in Table 3.1.

| Active Lines | Active Pixels per line | Aspect Ratio | Picture rate * |
| --- | --- | --- | --- |
| 1080 | 1920 | 16:9 | 60 I, 30 P, 24 P |
| 720 | 1280 | 16:9 | 60 P, 30 P, 24 P |

* Picture rate refers to number of fields or frames per second, Progressive (P) or Interlace (I) (see Rec. ITU-R 709-5).

Table  3.1. Formats used for digital terrestrial HDTV

The following rates are also available:

59.94 Hz, 29.97 Hz and 23.98 Hz.

Australia has adopted the 1080/1920/50 I format for digital terrestrial HDTV transmission, to be introduced in the near future.

3.1.2		Structure of a Digital Television System

The basic structure of a digital television system is shown in Figure 3.1.  It consists of three main subsystems.

1.	Source coding and compression

2.	Service multiplex and transport

3.	RF/transmission

Figure 3.1  Digital Television Broadcasting System

Source Coding and Compression

This refers to the bit rate reduction techniques, also known as data compression, appropriate for application to the video, and audio data streams.  The purpose of the coder is to minimise the number of bits needed to represent the audio and video information.

Digital television systems employ the MPEG-2 (Moving Picture Expert Group) standard for source coding of video/audio and service multiplexing including the ancillary data streams.  The term "ancillary data" includes control data, conditional access control data, and data associated with the program audio and video sources such as closed captioning.  "Ancillary data" can also refer to independent program services.  This standard was developed by the International Standards Organisation (ISO) and is comprised of three parts [3, 4, 5].  A second version of the standard was published in 1995 [6].

MPEG-2 is intended to be generic, supporting a diverse range of applications.  Different algorithmic elements or "tools", developed for many applications, have been integrated into a single bit stream syntax.

It is not necessary to implement the full syntax in all decoders hence a small number of subsets or "profiles" of the full syntax have been defined as shown in Table 3.2.  Also, within a given profile, a source format or "level" is defined which describes a set of constraints, such as maximum spatial/temporal sampling density, on parameters within the profile.  The profiles fit together such that a higher profile is a superset of a lower one.  A decoder based on a given profile and level must also be able to decode lower profiles and levels.

|  | Profile and maximum total bit-rate (Mbit/s) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Maximum sampling density (Hor/Vert/ Freq) | Simple profile (SP) | Main profile (MP) | SNR profile (scalable) | Special profile (scalable) | High profile (HP) | 4:2:2 |
| Level | High level (HL) (1920/1152/60) | - | MP@HL 80 Mbit/s | - | - | HP@HL 100Mbit/s +lower layers | - |
|  | High-1440 (1440/1152/60) | - | MP@H-14 60Mbit/s | - | Spt@H-14 60 Mbit/s +lower layers | HP@H-14 80 Mbit/s +lower layers | - |
|  | Main level (ML) (720/576/30) | SP@ML 15Mbit/s | MP@ML 15 Mbit/s | SNR@ML 15 Mbit/s +lower layers | - | HP@ML 20 Mbit/s +lower layers | 4:2:2@ML 720x68 50 Mb/s |
|  | Low level (LL) (352/280/30) | - | MP@LL 4 Mbit/s | SNR@LL 4 Mbit/s | - | - | - |
|  | ISO 11172 | - | - | - | - | - | - |
|  | (MPEG-1) 1.856 Mbit/s |  |  |  |  |  |  |

Table 3.2  MPEG profiles and levels

Note 1:  All decoders shall be able to decode ISO/IEC 11172 bit streams

Note 2:  SP@ML decoders are required to decode MP@LL bit streams

Note 3:	Currently MP@ML (Main profile at Main level) is used for SDTV broadcasting whereas for HDTV, MP@HL is used.

The MPEG-2 standard also provides tools for audio compression.  There is provision for five compressed audio channels (left, centre, right, right surround rear and left surround rear) as well as a special channel for low frequency effects.  There is also an option for up to seven channels of alternate language audio.

Service Multiplex and Transport

This refers to the means of dividing the digital data stream into "packets" of information, the means of uniquely identifying each packet or packet type and the appropriate methods of multiplexing video data stream packets, audio data stream packets and ancillary data stream packets into a single data stream.  Currently all available digital TV systems employ the MPEG-2 transport stream syntax [3] for the packetisation and multiplexing of video, audio and data.  The MPEG-2 transport stream syntax was developed for applications where channel bandwidth and recording media capacity is limited and the requirement for an efficient transport mechanism is paramount.  It was also designed to facilitate interoperability with the 
ATM transport mechanism.

RF/Transmission

This refers to channel coding and modulation.  The channel coder takes the data bit stream and adds additional information that can be used by the receiver to reconstruct the data from the received signal that, due to transmission impairments, may not accurately represent the transmitted signal.  This process is known as Forward Error Correction (FEC) coding.  This digital data stream information is used to modulate the transmitter carrier.

3.1.3	Transmission Media for Digital Television Broadcasting

Digital television broadcasting can be carried out via a number of transmission media such as satellite, terrestrial or cable.  Each of these media has some advantages as well as disadvantages.  A brief review of these is given below.

The main advantages of digital terrestrial transmission are as follows:  there are less shadow zones than with satellites and eventually universal reach is possible; portable or mobile reception is also more practical; Regional or local broadcasting can be achieved without duplicating the whole service; and one great advantage of the terrestrial media from the broadcast planner's point of view is the possibility of progressively increasing the coverage investment by add transmitters gradually over a period of time.

The benefits of digital satellite transmission are as follows: the capacity of the satellite channel is much greater than the terrestrial channel; it has about twice the data carrying capacity, and there are many more satellite channels available than terrestrial channels; and the coverage contour is large - one satellite beam can cover an entire country or part of a continental mass for viewers who have line of sight to the satellite.

Digital cable has its own benefits.  It has a larger capacity than terrestrial broadcasting, somewhat similar to satellite broadcasting.  It may be able to provide interactivity quite readily but this depends on the arrangements of the cable network.

3.1.4	Standards for Digital Television Broadcasting

3.1.4.1	DVB (Digital Video Broadcast) Standards

The DVB project was initiated in Europe.  It is a collaborative project involving a large number of organisations from Europe and around the world.  The DVB office, in cooperation with worldwide equipment manufacturers and research organisations has set down the framework for digital delivery system that can grow, mature and adapt over the years to come.  In order to take advantage of all special characteristics of every range of delivery media, including satellite, cable and terrestrial, including SMATV and MMDS, a set of standards optimised for the different delivery media have been developed.  The first DVB services are providing conventional TV quality, but the DVB systems can be used for transporting any data content or image and sound formats required by the service provider including HDTV.

A family of standards were developed keeping in view the following objectives [7].

- The source coding, framing and multiplexing are based on MPEG-2.

- Maximum commonality is preserved within each media and between media.

- Transmission systems should accommodate the 4:3, 16:9 and 2.21:1 picture

formats.

- The audio source coding should be either MPEG layer II (MUSICAM) or AC-3 (Dolby).

The three main standards of this family are the following [8,9,10].

1.  DVB-S:	The satellite delivery system for use in the 11/12 GHz band, configurable to meet a range of transponder bandwidths and power.

2.  DVB-T: 	The digital terrestrial TV system designed for use in terrestrial 7/8 MHz channels.

3.  DVB-C:	 Cable transmission system for use in 7/8 MHz CATV channels.

More details on these system will follow later.

3.1.4.2	The ATSC (Advanced Television Systems Committee) Digital Television Standard

This standard was developed in the USA by the Advanced Television Systems Committee for digital transmission of HDTV in the 6 MHz terrestrial channel.  It can also be used for multi-SDTV transmissions and further can be adapted for 7 or 8 MHz channels.  There are two transmission standards for satellite broadcasting of digital television in the North-America.  One proprietary system used by DirecTV and another one based on DVB-S used by Echo Star in the USA and ExpressView in Canada. [11]. A standard has been developed for digital cable transmission by CableLabs and has been documented by the ITU-T.

3.1.4.3 	ISDB (Integrated Services Digital Broadcasting) Standards [12]

This set of standards was developed in Japan by the DiBEG (Digital Broadcasting Expert Group) for digital transmission of video, audio and data.  Both HDTV and SDTV transmission is possible.  The relevant standards are :

ISDB-T	-	for terrestrial transmission

ISDB-S	-	for satellite transmission

ISDB-C	-	for cable transmission

3.1.5	Service Information (SI) System

The MPEG-2 systems standard [3] specifies Service Information (SI) data to enable configuration of the Integrated Receiver Decoder (IRD) to demultiplex and decode the various streams of programs within the multiplex.  This data is referred to by MPEG-2 as the Program Specific Information (PSI).  The SI data provides the user with information to assist in selection of services and/or events.  An event is a grouping of elementary broadcast data streams with a defined start and end time belonging to a common service, such as the first quarter of a basketball game or a commercial.  The specifications define that the IRD can automatically configure itself for the selected service.  Moreover SI can be used for VCR applications.

A unique feature of DTV is the capability to transmit an electronic program guide (EPG) database in the transport stream, which allows an IRD to build an on-screen grid of program information and contains control information to facilitate navigation.

The DVB standard [13] provides for three main tables (program association table, PAT, the program map table, PMT and the conditional access table, CAT) and eight more additional tables, which contain information on services and events carried by different multiplexer as well as on other networks.

The ATSC standard also has provisions for transmitting both EPG database as well as additional service information [14, 15, 16].

3.1.6	Conditional Access (CA) System

A conditional access system comprises a combination of scrambling and encryption to prevent unauthorised reception.  Scrambling is the process of rendering sound, pictures and data unintelligible.  Encryption is the process of protecting the secret keys that have to be transmitted with the scrambled signal in order for the descrambler in the receiver to reconstitute the signal.

The primary purpose of a CA system for broadcasting is to determine which individual receivers/set-top decoders shall be able to deliver particular program services, or individual programmes to the viewers.  The reasons why access may need to be restricted include:

- to enforce payment by viewers who want access to particular programmes or programme services

- to restrict access to a particular geographical area because of programme rights considerations

- to facilitate parental control (i.e. to restrict access to certain categories of programmes).

Two types of key signals are transmitted. These are, 1. Entitlement Control Messages (ECM) and 2. Entitlement Management Messages (EMM).  Both are transmitted in encrypted form.  ECMs are used to control the descrambler, whereas the EMMs are used to switch individual decoders, or groups of decoders, on or off.  The MPEG-2 standard does not require scrambling, nor does it specify which algorithms may be used.  The standard also does not specify the format of EMMs but it does provide a method for decoders to be able to identify and demultiplex streams that carry EMMs.  EMMs are sent using a predetermined packet ID that will be set at the multiplexer and is known to the decoder through the CA Table of PSI as mentioned earlier.  ECMs are sent in private data areas of the adaptation field or in transport packets marked as private.

DVB-CA System

The DVB project has designed a Common Interface [17] for use between the IRD and CA system.  The IRD contains only those elements that are needed to receive clear broadcasts.  The CA system is contained in a low priced proprietary module that communicates with IRD via the Common Interface.  This allows broadcasters to use CA modules that contain solutions from different suppliers.

The DVB project has given its backing to two CA approaches for the transmission of digital television via satellite namely Simulcrypt and Multicrypt [18].  These approaches are also relevant in cable and terrestrial transmission.

Simulcrypt :	The principle of the system is that the different ECMs and EMMs needed for the various access control systems are sent over the air together. Anyone decoder picks out the information it needs and ignores the other codes.

Multicrypt :	This uses common receiver/decoder elements which could be built into TV sets.  The Common Interface described above can be used to implement Multicrypt-Conditional Access modules from different system operators and can be plugged into different slots in the common receiver/decoder using Common Interface.

3.2	Digital Satellite Broadcasting

3.2.1	DVB-S

The DVB satellite system [8] has been defined for the broadcasting and distribution of digital multi-programme television and HDTV services in the 12 GHz Broadcast Satellite Service (BSS) and Fixed Satellite Service (FSS) bands.  The DVB-S system is also intended to provide Direct-to-Home (DTH) services for consumer Integrated Receiver Decoders (IRDs) as well as Satellite Master Antenna Television (SMATV) systems and cable systems.  In a such case, the  DVB signals need to be re-modulated for distribution purposes.

The DVB-S system is suitable for use on satellites whose transponder bandwidth lies within the range 24 MHz to 54 MHz.  By using a time-division-multiplex (TDM) a variety of digital vision, sound and data services can be superimposed on to a single RF carrier in the transponder.  The number of different services that can be multiplexed onto the carrier is a function of the television picture quality required and the marketing strategy of each broadcaster.

In the case of TDM, the satellite can be operated at saturation since there is only one carrier in the transponder.  However an alternative multicarrier system called frequency division multiple access (FDMA) can be used for satellite delivery.  In this case, it is necessary to operate the satellite transponder with an output back-off (OBO) in order to reduce the level of the intermodulation products.  The transponder operates at lower efficiency and the total power available to each carrier is lower than in the TDM case.  Furthermore taking into account the bandwidth resources in the transponder, the total capacity is lower in the case of FDMA because of the need to keep a guard band between the carriers.  However, FDMA technology has one advantage, it allows the possibility of uplinking multiple signals simultaneously to the same transponder from different sites, thus avoiding the need to transport all the TV programmes to one common site.

A very simplified block diagram of the DVB-S system is shown in Figure 3.2.  Source coding and multiplexing is based on the MPEG-2 standard.

Figure 3. 2.  DVB-S System Block Diagram

Channel Coding & Modulation

The MPEG transport stream packets generated by the video and audio coding and multiplexing processes are input to the channel coding process.  This process comprises four stages: Forward Error Correction (FEC) coding, outer coding, interleaving and inner coding followed by QPSK modulation.  The outer code used is Reed-Solomon (RS) wherein 16 check bytes are computed and appended to each 188 byte TS packets.  The RS code can detect and correct up to 8 byte errors in the total 204 bytes of the packet.  The interleaver placed between the outer and inner coders has the purpose of breaking down any lengthy bursts of errors reaching the outer decoder in the receiver.

The inner code used is known as convolutional code.  The code rates can be 1/2, 2/3, 3/4, 5/6 and 7/8.  This flexibility allows the user bit-rate to be optimised according to the available satellite effective isotropic radiated power (e.i.r.p.), receiving antenna size and service availability targets.

With the aim of allowing the receiver to detect the signal adequately at very low carrier to noise (C/N) ratios, a relatively simple channel modulation scheme has been adopted: Quadrature Phase Sight Keying (QPSK).  This scheme has a constant envelope amplitude and widely separated phase states which give a high degree of ruggedness against channel noise and interference.

Planning Aspects [7]

The most important feature of the DVB-S system is its flexibility to accommodate a wide range of possibilities, useful bit-rate, FEC, transponder BW, etc.  Table 3.3 gives examples of the useful bit-rate capacity achievable on a satellite transponder BW corresponding to a BW/RS = 1.28 where RS is the transmission symbol rate.  Other BW/RS values may be adopted for different service requirements, depending on the trade off implemented between the transmission capacity and the bit energy-to-noise ratio (Eb/No).  An important parameter is the antenna size required to receive the transmission in simple DTH applications.  To compute the required antenna size, it is first necessary to know the Eb/No required by the demodulator to ensure a BER (Bit Error Rate) of better than 2x10-4 after Viterbi Decoding.  The C/N can then be derived and finally the required antenna gain (and hence its size).  Figure 3.3. and Figure 3.4. illustrate some examples of the relationship between these parameters.

| Transponder Bandwidth (MHz) |  | Typical symbol rate (Mbaud) | Useful bit-rate capacity (Mbit/s) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -3 dB | -1 dB |  | Code rate 1/2 | Code rate 2/3 | Code rate 3/4 | Code rate 5/6 | Code rate 7/8 |
| 54 | 48.6 | 42.2 | 38.9 | 51.8 | 58.3 | 64.8 | 68.0 |
| 46 | 41.4 | 35.9 | 33.1 | 44.2 | 49.7 | 55.2 | 58.0 |
| 40 | 36.0 | 31.3 | 28.8 | 38.4 | 43.2 | 48.0 | 50.4 |
| 36 | 32.4 | 28.1 | 25.9 | 34.6 | 38.9 | 43.2 | 45.4 |
| 33 | 29.7 | 25.8 | 23.8 | 31.7 | 35.6 | 39.6 | 41.6 |
| 30 | 27.0 | 23.4 | 21.6 | 28.8 | 32.4 | 36.0 | 37.8 |
| 27 | 24.3 | 21.1 | 19.4 | 25.9 | 29.2 | 32.4 | 34.0 |
| 26 | 23.4 | 20.3 | 18.7 | 25.0 | 28.1 | 31.2 | 32.8 |

Table 3.3  -  Examples of useful bit-rate capacity

Figure 3.3  Examples of Antenna Diameter versus Symbol Rate

Figure 3.4  Examples of Availability versus Useful Bit-rate

Reference values to adopt when planning DVB-S services are detailed in Table 3.4.  They represent the most accurate assumptions for the performance of DVB-S equipment [19].

| Parameter | Euro Image reference values (dB) |
| --- | --- |
| Bandwidth limitation degradation | 0.1 - 0.4 |
| Non linearities (ISI) degradation | 0.0 - 0.7 |
| Noise bandwidth increase 10log(204/188) | 0.36 |
| Interference losses (intra-system) | 0.0 - 0.3 |
| Power reduction due to non-linearities | 0.9 |
| Modem implementation margin | 0.8 |
| Distortion due to SMATV distribution network SMATV-S SMATV-S equalized SMATV-IF | 1.00 - 1.85 0.25 - 0.50 3 |
| Required Eb/No AWGN (note 1) FEC 1/2 FEC 2/3 FEC 3/4 FEC 5/6 FEC 7/8 | 4.6 5.1 5.6 6.3 6.8 |
| Required Eb/No satellite (note 2) FEC 1/2 FEC 2/3 FEC 3/4 FEC 5/6 FEC 7/8 | 5.5 6.0 6.6 7.2 7.9 |
| Protection Ratios: Full Digital Scenario (BW/Rs = 1.28) CCI H-ACI (note 3) Hybrid Digital/Analogue Scenario CCI, D-to-A CCI, A-to-D H-ACI, D-to-A H-ACI, A-to-D | 16 12 23 17 15 13 |

Table 3.4  -  Reference values to adopt when planning DVB-S services

Note 1:			The values of Eb/No AWGN take into account the contribution from the modem implementation (margin= 0.8 dB) and the noise bandwidth increase (10 log (204/188)).

Note 2:			The required Eb/No satellite values add the following contributions to the required Eb/No AWGN: bandwidth limitation degradation and non-linearities (ISI).  Power reduction due to non-linearities and intra-system interference losses are not included in the required Eb/No satellite values.

Note 3:			H-ACI is adjacent channel interference from a signal that is half a channel width away (F-ACI is the case where the interfering signal is a full channel width away).

    - DIREC TV [20]

This is a proprietary system and one of the two United States' high power 12 GHz band DBS systems delivering digitally compressed TV to the home.  It transmits over 200 TV channels.  The user terminal consists of a small 18" satellite dish and a digital IRD that tunes each program and decompresses and translates the digital signal into analogue format.  DirecTV programming is distributed by three high power HS 601 satellites located at 101 degrees West (DBS-1, DBS-2, DBS-3) built by Hughes Electronics.  Each satellite features 16 120-watt 12 GHz band transponders with DBS-2 and DBS-3 each configured to provide 8 transponder at 240 watts.  This system also has capability to transmit HDTV signals as demonstrated at NAB 1999.

3.3	Digital Terrestrial Broadcasting

3.3.1	Introduction

VHF and UHF radio frequencies are used for terrestrial transmission of television pictures.  Like all RF channels, this frequency band suffers from the effects of noise, multipath and interference.  Multipath signal propagation results in the familiar picture ghosting of analogue transmissions, which is perhaps the most annoying visual artefact.  The effect is even worse when caused by a moving reflector such as an aircraft.  The worst-case environment for multipath is when the receiver is moving.  In this case the receiver will pick up signals via many dynamically varying paths.  This is made worse by the additional degradation caused by Doppler spread, whereby the various received signals suffer different Doppler frequency shifts depending on the geometry involved and the direction of the receiver displacement relative to the directions of the various local multipaths.  Another major problem is interference.  Interference in television networks is typically the result of mutual interference resulting from other transmitters in the network, either on the same or nearby frequencies, so called co-channel interference (CCI) and Adjacent Channel Interference (ACI) respectively.  In a mixed environment of analog and digital networks, there are therefore two types of CCI and ACI suffered by a digital transmission, depending on whether the interferer is analogue or digital.  Reception is also affected by Industrial Noise such as power switching in the vicinity of a receiver, e.g. ignition interference.

The various DTTB systems available today differ mainly in the channel coding and modulation techniques adopted to counter the problems mentioned above.  The modulation can be either Simple Carrier Modulation (SCM) or Multi Carrier Modulation (MCM).  In SCM, a single carrier is modulated at the full data rate required to carry the service information.  This arrangement provides for a relatively simple receiver/demodulator.  It is suitable for use in channels relatively free from multipath effects but less ideal for terrestrial channels where a highly sophisticated channel equaliser may be required in the receiver to deal with multipath effects.

In MCM, groups of sequential data packets are effectively stored for a period, known as the symbol period, during which a large number of carriers are generated and modulated in parallel.  Thus the data rate on each carrier is the total rate divided by the number of carriers.  While the demodulation algorithm (FFT) is inherently more complex than in the SCM case, the performance, in the presence of multipath effects and interference from analogue television transmissions, is inherently superior.

3.3.2	The DVB-T system

A simplified block diagram is shown in Figure 3.5.  Source coding as well as channel coding is identical to the one used in DVB-S except for a few differences explained below.  Unlike the satellite system, the terrestrial system allows for a two layer hierarchical system.  The transmitter in this case will emit two MPEG-2 transport streams that could contain the same programmes or entirely different programmes.  As an example, one transport stream could contain four programmes each encoded at 1.5 Mbit/s while the other transport stream carries the same four programmes each encoded at 4.5 Mbit/s.  By using a more rugged transmission scheme for the four 1.5 Mbit/s programmes, these could be received on portable receivers while the four 4.5 Mbit/s programmes can only be received on stationary receivers.  In the case of hierarchical transmission, the layer requiring to be more rugged will be carried by a QPSK signal while the layer requiring to be less rugged will be carried by 16 or 64 QAM.

Figure 3.5.  DVB Terrestrial Transmitting Chain

Another difference, in the case of the terrestrial system, is the presence of an inner interleaver.  This uses block based bit-wise interleaving combined with symbol-wise interleaving.  This interleaving combined with the OFDM modulation results in frequency interleaving.

DVB-T uses a multi-carrier modulation scheme called Coded Orthogonal Frequency Division Multiplex (COFDM) [21].  The major benefit of COFDM is the fact that the serial baseband bit stream which needs to be transmitted is distributed over many closely spaced individual carriers.  The individual set of such carriers which is processed at a given time is called a ‘COFDM Symbol'.  Due to the large number of carriers which handle part of the baseband bit stream in parallel, the duration of such a symbol (Ts) is considerably larger than the duration of one bit of the bit stream.  Thus before starting to evaluate each individual symbol, the receiver pauses during a short interval at the beginning of each symbol until channel conditions have become stable, which for instance means that all echoes of the previous symbol have vanished.  This is done by introducing a ‘guard interval’ of duration Tg at the beginning of each symbol (Figure 3.6.).  COFDM therefore turns echoes from destructive signals which cause inter symbol interference into constructive signals that add to the energy of the direct transmission path.  .

Figure 3. 6  Illustration of the Guard Interval

A special case of multipath exists when two or more transmitters are operated at exactly the same frequency and are transmitting the same signal at exactly the same time.  (This incidentally is known as Single Frequency Network (SFN)).  In this case, the receiver sees a multipath signal with an echo delay equal to the path difference between the transmitters and the receiver locations and this extra signal can be discarded like an echo if it arrives at the receiver during the guard interval.  The larger the Tg, the larger the maximum time differential between the signals from the transmitters can be.  On the other hand, the length of the guard interval should be kept as small as possible because it reduces the transmission channel efficiency and becomes more affected by time-variant channel effects such as Doppler spread.

DVB-T has two modes of operation.  Each symbol is constituted by a set of 1705 carriers in the ‘2K mode' and 6817 carriers in the ‘8K mode'.  Each symbol is transmitted with a duration Ts composed of two parts, a part with duration TU and a guard interval of duration Tg.  The possible values of Tg are 1/4 Tu, 1/8 Tu, 1/16 Tu and 1/32 Tu.  All data carriers in an OFDM symbol are modulated using one of five user-selectable modulation methods.  These are 1) QPSK, 2) 16-QAM, 3) 64-QAM, 4) non-uniform 16 QAM and 5) non-uniform 64 QAM.

The DVB-T COFDM signal also includes a number of pilot carriers for transmission of parameters and for signal recovery purposes (used at the receiver for synchronisation.

3.3.3	ISDB-T System

This standard was developed in Japan by the DiBEG (Digital Broadcasting Expert Group).  The source coding/multiplexing is based on the MPEG-2 standard.  As for channel coding and modulation, this standard uses more or less the same principles as the DVB-T specification with three major differences.

1.	Band-segmented transmission (BST)

2.	Hierarchical transmission methodology

3.	Differential modulation scheme

These extensions to basic COFDM specifications such as DVB-T are aimed at optimising the system for mobile multimedia transmission.

In BST the multiplex of carriers is split into 13 segments of carriers that can be independently modulated in the available modulation modes.  For example, one segment can be modulated with 64 QAM with a code rate of 2/3 and another with QPSK with a code rate of 1/2.  The segmented structure permits blocks of the spectrum to be used for different parts of the transmitted MPEG-2 transport stream (TS).  This implies a form of de-multiplexing and mapping process in the channel encoder/modulator that connects TS packets with COFDM segments.  In fact only four splits of the TS are permitted so that each part is usually carried by more than one segment.  Only the centre segment is used for single segment transmission.  This is shown graphically in Figure 3.7.

The advantage of this added complexity is that the centre segment can be received separately using a simplified receiver.  Obviously this does not allow the complete TS to be reconstructed at the receiver, but it does allow radio or data services to be delivered to a hand-held device.  Bit rates available are of the order 300 kbit/s - 1.5 Mbit/s.  The concept of partial reception may also be extended to partial transmission whereby the single segments alone might be transmitted in narrow channels.

Figure 3.7.  Segmentation and Hierarchy in ISDB-T

Another feature of the segmentation process is the ability to transmit hierarchically.  For example, a service for which universal coverage is required might use a 16 QAM rate 1/2 modulation scheme, whereas a Pay TV service that has no universal coverage mandate could be transmitted at higher bit rate using a 64 QAM, rate 2/3 modulation process.  In effect, the 16 QAM 1/2 segments would have greater coverage than the 64 QAM 2/3 segments.

Another example of the use of hierarchical transmission is in the transmission of hierarchically encoded HDTV.  If the TS contains hierarchically compressed video, the SD portion can be transmitted as above, with a highly robust modulation scheme e.g. for mobile reception.  The additional packets containing the extra information to extend the video to HD, could then be transmitted on a higher bit rate group of segments with corresponding reduced robustness but nevertheless receivable on fixed antenna terminals.

As an additional aid to mobile reception, the choice of modulation schemes has been extended from that in DVB-T, to include differential QPSK.  This allows a trade off of improved Doppler performance against performance in a gaussian channel.  Nevertheless DVB-T with coherent demodulation has been shown to work at over 150 km/h in low multipath conditions.

3.3.4	ATSC System

As already mentioned earlier this system was developed in the USA, for digital terrestrial HDTV transmission. Source coding and multiplexing is based on Main Profile/High level combination of MPEG-2.  However ATSC uses a single carrier modulation scheme called 8-VSB (Vestigial Side Band).  Figure 3.8 shows a block diagram of a VSB transmitter.  An RS (208, 188) outer code and a 2/3 rate trellis inner code are used.  The baseband data signal is filtered with a complex filter to produce quadrature and in-phase components for orthogonal modulation.  This actually corresponds to a phase and amplitude modulation scheme similar to the I/Q modulation scheme of QAM.  However as it is created from an amplitude only transmission scheme (with 8 levels).  Only a single side band is required of the otherwise full quadrature modulated signal.

Figure 3.8.  VSB Transmitter

Channel noise is dealt with in much the same way as for multicarrier systems, through the use of FEC schemes with soft decision decoding.  However multipath and interference need different strategies.

In the multipath environment, a high symbol rate carrier will be received with overlapping echoes of several tens of symbols.  The ISI (Inter Symbol Interference) can be removed by an equalisation process using a multi-tap digital filter. Implementations typically use two filler stages, one for pre-echoes of the order of 64 taps and one for post echoes typically of the order of 192 taps.  The weighting factor applied at each tap is calculated from the measured response of the channel to a known training sequence transmitted periodically with the data.  This is similar to the pilots used in the multicarrier systems.  Interference is removed at the receiver prior to demodulation by using a digital filtering process specially designed for the removal of the main tones that would be expected from a co-channel NTSC interferer.  The design of this comb filter effectively takes into account the most likely channel interferers.  For NTSC in 6 MHz a simple 1-tap filter can be used, but this needs to be programmed differently for operation in different channel bandwidths and when the CCI is from different television standards.  Spurious tones and other types of interferers are very difficult to remove in an 8VSB system and effectively look like noise after demodulation.

3.3.5		DVB-T Distribution Networks

The primary distribution network carries the digital TV signal from the TV production premises to the transmitter sites.  Two approaches are possible.  In the first approach, the COFDM modulator is situated at a central point and the modulated COFDM signal distributed to the transmitters using terrestrial analogue SHF links.  In the alternate approach, the MPEG-2 TS has to be distributed to all the COFDM modulators in the network.  The distribution network may use fixed terrestrial or satellite links and may include further levels of MPEG-2 multiplexing, for example to provide regional programme variations.

PDH Networks:  Plesiochronous Digital Hierarchy (PDH) was designed for digitised signals based on 64 kbit/s.  ITU Recommendations G 703 [22] specifies interfaces at various hierarchical levels, the interface at 34,368 Mbit/s is suitable for TS.  An interface between DVB TS and PDH networks has been specified [23].

SDH Networks:  Synchronous Digital Hierarchy (SDH) is an alternative to PDH using a simplified multiplexing and demultiplexing technique and offering improved network management capabilities.  An interface between DVB TS and SDH networks has been specified [24].

ATM Networks:  In the future, networks using the Asynchronous Transfer Mode (ATM) may be offered for primary distribution.  ATM uses a cell based multiplexing technique and may be carried over different kinds of transport networks including PDH & SDH.  ATM cells consist of a 5-Octet header followed by 48 payload octets.  Five different ATM Adaptation Layers (AALs) have been specified for adapting different types of signal to ATM networks.  AAL1 or AAL5 may be used for the transmission of an MPEG-2 TS; the main difference is that AAL1 specifies error defection and correction techniques whereas AAL5 does not.  The network adaptor specified by DVB for adaptation to PDH and SDH networks [23,24] is based on adaptation of the MPEG-2 TS into ATM cells using AAL1 and then adapts the ATM cells to PDH or SDH framing.

Satellite Distribution:  The TS can be distributed by satellite using the DVB-S specifications.  However, a demultiplexing operation will be required at each transmitter site to change the SI data to reflect the change of delivery medium.

3.3.6.	Transmission Networks

Two fundamental types of network architectures are possible: Multi Frequency Networks (MFN) and Single Frequency Networks (SFN).  MFN consists of transmitters with independent programme signals and with individual radio frequencies.  In SFN, all transmitters are synchronously modulated with the same signal and radiate on the same frequency.  As compared to MFN, SFN has many advantages, some of these are listed below.

- improved spectrum efficiency

- smaller frequency reuse distance

- improved power efficiency, better tailored to the desired coverage

- "gap fillers" without extra frequency resource

A single carrier system such as ATSC cannot be used in SFN.  The multi-carrier DVB-T system can be used for both MFN and SFN operation.  Gap fillers permit the elimination of residual shadowed areas by using passive reflectors or small active relays using the carrier frequency for the reflected or relayed signal. They create echoes voluntarily. The signal is picked up at a location where reception conditions are satisfactory and then re-amplified and rebroadcast on the same frequency.

Dense networks transmitting the same programme on the same frequency can be built.  The limitation to the use of the technique is the possibility that the signal received from distant transmitters are delayed more than allowed by the guard intervals and therefore behave as interfering signals.  As a rule of thumb, the guard interval should allow the signal to propagate over the distance between two transmitters in the network.  As this guard interval may vary from 224 µs (1/4 guard interval value, 8 K mode) to 7 µs (1/32 guard interval value, 2 K mode) the corresponding distances are about 65 km and 2 km respectively.  A variety of SFNs can therefore be implemented: large, (regional, possibly national depending on the programming variations required), local (e.g.. to serve an urban area).  The use of gap fillers to fill-in gaps in the coverage of the transmitters comes in addition to it.

SFN operation results in gain in frequency and power efficiency but it requires the synchronous operation of all the transmitters of the network.  Efficient methods are specified for this purpose [25].

3.4. List of Abbreviations

AALs	ATM Adaptation Layers

ACI	Adjacent channel interference (F-ACI: adjacent channel is a full channel away; H-ACI: adjacent channel is half a channel away)

ATM	Asynchronous Transfer Mode

ATSC	Advanced Television Systems Committee

ATSC	Advanced Television Systems Committee

AWGN	Average White Gaussian Noise

BER	Bit Error Ratio (commonly called bit error rate)

BSS	Broadcast Satellite Service

BST	Band-segmented transmission

BW	Bandwidth (channel)

C/N	Carrier to noise ratio (usually expressed in dB)

CA	Conditional Access

CCI	Co-channel interference

COFDM	Coded Orthogonal Frequency Division Multiplex

DiBEG	Digital Broadcasting Expert Group

DTH	Direct-to-Home

DTTB	Digital Terrestrial Television Broadcasting

DVB	Digital Video Broadcasting

DVB-C	Satellite Digital Video Broadcasting (EN 300 429)

DVB-S	Satellite Digital Video Broadcasting (EN 300 421)

DVB-T	Satellite Digital Video Broadcasting (EN 300 744)

e.i.r.p.	effective isotropic radiated power

Eb/No	Bit energy-to-noise density ratio

ECM	Entitlement Control Messages

EMM	Entitlement Management Messages

EPG	Electronic Program Guide

FDMA	Frequency Division Multiple Access

FEC	Forward Error Correction

HDTV	High Definition Television

IRD	Integrated Receiver Decoder

ISDB	Integrated Services Digital Broadcasting (Japan)

ISDB-C	Cable transmission standard for ISDB (ARIB standard)

ISDB-S	Satellite transmission standard for ISDB (ARIB standard)

ISDB-T	Terrestrial transmission standard for ISDB (ARIB standard)

ISI	Inter-symbol interference

ISO	International Standards Organization

MCM	Multi Carrier Modulation

MMDS	Multi-service Multi-channel Distribution  System

MPEG	Moving Picture Expert Group

NTSC	National Television System Committee TV standard

OBO	Output back-off (transponder)

OFDM	Orthogonal Frequency Division Multiplex

PAL	Phase-Alternate Line TV standard

PDH	Plesiochronous Digital Hierarchy

PSI	Program Specific Information

QAM	Quadrature Amplitude Modulation

QPSK 	Quadrature Phase Sight Keying

RS	Reed-Solomon (FEC block code)

SCM	Simple Carrier Modulation

SDH	Synchronous Digital Hierarchy

SDTV	Standard Definition Television

SFN	Single Frequency Network

SHF	Superhigh Frequency band

SI	Service Information

SMATV	Satellite Master Antenna TV distribution system

TDM	Time Division Multiplex

TDMA	Time Division Multiple Access

TS packets	Transport packet (ISO MPEG-2)

VCR	Video Cassette Recorder

VSB	Vestigial Side Band

3.5. References

- ITU-R Recommendation BT.601-3: Encoding parameters of digital television for studios.

- ITU-R Recommendation BT.656-1:  Interfaces for digital component video signals in 525-line and 625-line television systems operating at the 4:2:2 level of Recommendation 601.

- ISO/IEC IS 13818-1, International Standard (1994):  MPEG-2 Systems

- ISO/IEC IS 13818-2, International Standard (1994):  MPEG-2 Video

- ISO/IEC IS 13818-3, International Standard (1994):  MPEG-2 Audio

- ISO/IEC IS 13818-2 1995:  Information technology - Generic coding of moving pictures and associated audio information video.

- J. Sesena - "The DVB satellite, cable and SMATV systems why the technical choices were made" - EBU Technical Review Winter 95.

- ETS 300421 - "Digital broadcasting systems for television, sound and data services; Framing structure, channel coding and modulation for 11/12 GHz satellite services.

- ETS 300744 - "Digital broadcasting systems for television sound and data services; Framing structure, channel coding and modulation for digital terrestrial television".

- ETS 300429 -  "Digital broadcasting systems for television, sound and data services; Framing structure, channel coding and modulation for cable systems".

- Robert K Graves - "Digital TV - an ATSC Perspective" proceedings of ABU-AIBD seminar on Digital broadcasting: managing the transition 5-8 July'99, Kuala Lumpur, Malaysia.

- Hiroshi Miyazawa - "The Japanese approach to Digitisation" - Proc. ABU-AIBD Seminar July 99.

- EN 300468: "Digital Video Broadcasting (DVB); specifications for Service Information (SI) in DVB systems".

- Advanced Television Systems Committee - ATSC Digital Television Standard Document A/53 September 1995.

- ATSC Document T3/S8-050 - "Program guide for Digital Television".

- ATSC Document T3/S8-079 - "System Information for Digital Television".

- DVB Document A007 July 1995 - "Common interface specifications for Conditional Access and other Digital Video Broadcasting Decoder Applications".

- "Functional model of a Conditional Access system" - EBU project group B/CA .. EBU Technical Review Winter 1995.

- "Evaluation report on the performance of the DVB system on real satellites" - Euro Image project M1003/HIS/SAT/DS/R/341 HISPSAT, October 1995.

- Robert H Plummer - "HDTV via DBS": Proc. Montreux Television Symposium, June 1999.

- U Reimers - "DVB-T: the COFDM based system for terrestrial television" - IEEE Electronics and Communications Engineering Journal Feb. 1997.

- ITU-T Recommendation G703 - "Physical/electrical characteristics of hierarchical digital interfaces".

- ETS 300813 - "DVB; Interface to PDH networks".

- ETS 300814 - "Digital Video Broadcasting (DVB); Interfaces to Synchronous Digital Hierarchy (SDH) networks".

- TS 101191 - "Digital Video Broadcasting (DVB); specifications of a megaframe for SFN synchronisation".

## CHAPTER 4

4.	STRATEGIES FOR DIGITAL TELEVISION BROADCASTING

4.1	Introduction

Although digital transmission technology has developed apace in recent years, one can be certain that it will not replace the existing analogue system overnight, there will exist a period of transition during which the analogue and digital transmissions must coexist.  Hence planning will have to be done separately for the transition and post transition periods.

4.2	Planning for the Transition period

Presently terrestrial television bands (VHF & UHF) are heavily used by analogue television services and no new spectrum below 1000 MHz is likely to be allocated for television.  Hence any new digital services will have to share spectrum with analogue services and may need to co-exist for a period of 10 to 20 years during transition.  The growth of DTTB services will greatly depend upon the national perspective during the transition period.

It is desirable to reuse the same transmission sites as for existing analogue TV and to use a frequency planning approach similar to the one adopted for analogue services in the initial stage of implementation of DTTB.  This has a cost saving aspect for broadcasters and further it will not place unnecessary difficulties for the viewers.

Broadcasters may also wish to consider channels near the existing ones as this would permit them to reuse existing transmitting antennae and feeder cables.  However it should always be kept in mind that even when the transmitters are co-sited, there will be significant coverage differences between analogue and digital services.  Since in the transition period, analogue and digital services have to coexist, proper care should be taken to avoid mutual interference between them.

ITU-R Recommendation BT-1368 [26] gives details on the protection ratios (1) for wanted digital terrestrial TV systems interfered with by other digital systems; (2) for wanted digital terrestrial TV systems interfered with by analogue broadcast TV systems; and (3) for wanted analogue terrestrial TV systems interfered with by unwanted digital terrestrial TV systems.

The Recommendation tabulates protection ratios for co-channel, adjacent channels (both lower and upper), image channel and overlapping channel for the three cases mentioned above.terrestrial TV systems interfered with by analogue broadcast TV systems; and (3) for wanted analogue terrestrial TV systems interfered with by unwanted digital terrestrial TV systems.

The Recommendation tabulates protection ratios for co-channel, adjacent channels (both lower and upper), image channel and overlapping channel for the three cases mentioned above.

Use of Existing Infrastructure

As already mentioned, wherever possible, channels for digital broadcasting from a particular site are selected close to analogue channels.  In many cases this should allow viewers to reuse their existing receiving antenna.

As it is envisaged using the adjacent channels of analogue transmitters for digital TV broadcasting, the knowledge of the levels of the spurious emissions on these channels is of major interest.  In adjacent channels, the spurious emissions of analogue transmitters will be seen by the digital receivers as co-channel interference.  In order to minimise such spurious emissions, filters have to be used at the transmitter output.

It is also possible to use the existing transmit antenna for digital TV broadcasting.  In this case, channels chosen for digital terrestrial broadcasting have to be inside or close to the bandwidth for which the analogue antenna has been matched.  This can bring a similar coverage area for both the services and also most of the reception antennas should be suitable.  On the other hand possible ERP restrictions necessary to protect existing analogue channels broadcast from neighbouring sites cannot be satisfied.  It should also be kept in mind that existing antenna and feeders have to support the total multiplex power including all the peak power of digital channels.

The DVB-T recommends an RF combiner (see Figure 4.1.) that comprises two 3 dB couplers, two identical bandpass filters and a dummy land.  It has a selective input called "narrow band" and a wideband input: "broadband input".  The bandpass filters are tuned to the narrow band input channel.  In this case the digital signal is connected to the narrow band input and analogue channels are connected to the broadband input.  The filters are required to pass the digital signal yet block the channels of the broadband path.  Nevertheless the use of filters leads to a group delay variation that is the source of signal distortion.  In order to avoid such a problem a baseband pre-corrector has to be used.

Figure 4.1  RF DVB-T Combiner

An alternative approach is to use a new antenna dedicated to digital terrestrial TV, the main advantage of such an approach being the absence of high power RF combiner but the drawback is the spurious emissions which are not filtered.  Therefore it would be necessary to use specific filters at the transmitter output.  In this case, the new antenna diagram may lead to a different coverage area than the analogue one.  The bandwidth and matching of the antenna are specifically adapted to the broadcast of digital channels.  ERP restrictions may be required in order to protect the existing analogue TV servers.  Because of all these reasons, the cost of a new antenna can be very high.

4.3	Post Transition Period

During this period analogue services will no longer be there to pose a constraint to the size or number of digital services.  SFN with spectrum economy may be used for multiple national coverage.  Lower protection ratios of digital systems could mean a large increase in the number of regional or local television services.

4.4.	Planning Criteria

4.4.1	Coverage definitions for fixed and portable reception

Digital TV service coverage is characterised by a very rapid transition from near perfect reception to no reception at all and it thus becomes much more critical to be able to define which areas are going to be covered and which are not.  DVB-T has defined coverage for both fixed and portable reception.

Fixed antenna reception is defined as reception using a directional receiving antenna mounted at roof level.  Equivalent field strength required is calculated assuming an antenna height of 10 m above ground level (agl).

Portable antenna reception is defined as

Class A (outdoor):	Reception using a portable receiver with an attached antenna outdoors at no less than 1.5 m agl.

Class B (ground floor indoor):Reception using a portable receiver with an attached antenna

- indoors at no less than 1.5 m above floor level in rooms

- on the ground floor

- with a window in an external wall

4.4.2		Coverage area

In defining the coverage area for each reception condition, a three level approach is taken

1.	Location :  Smallest unit with dimensions of about 0.5 m x 0.5 m.  The location is regarded as covered if the requested carrier-to-noise and carrier-to-interference values are achieved for 99% of the time.

2.	Small area : Typically (100m x 100m)

The coverage of a small area is classified as

a) "Good" if at least 95% of receiving locations within it are covered.

b) "Acceptable" if at least 70% of locations within it are covered.

3.	Coverage area :  The coverage area of a transmitter or a group of transmitters is made up of the sum of the individual small areas in which a given class of coverage is achieved.

4.4.3	Wanted Signal Level considerations

Analogue systems have a fairly soft transition to failure characteristic.  As the input falls below the minimum value (specified for a particular picture quality) there will be a gradual reduction in picture quality.  Complete failures may not occur until the level is some 10 to 20 dB below the nominal value.

In contrast for digital services, rapid transitions between near perfection and failure may occur for a signal level reduction of less than 1 or 2 dB.  As such, location probability i.e. variation of signal level within a small area, is an important factor.  In digital service planning, aim is to provide at least the minimum signal level for 95% location probability for “good reception” and 70% location probability for “satisfactory reception”.  For indoor reception, the signal will be attenuated by building penetration loss.  This varies somewhat with the carrier frequency.  For portable reception, the antenna height of 10m above ground level is not realistic and a height of 1.5 m agl is assumed.  This will result in a signal loss termed "height loss".

4.4.4	Minimum Signal Level

The required minimum signal level or equivalent minimum field strength will depend upon a number of factors such as a carrier-to-noise ratio, receiver noise figures, feeder loss, antenna gain, etc.  For planning purposes, the field strength is calculated at 10m receiving antenna height.

In practise it is found that field strength values vary considerably from their median value within a given relatively small area around a nominal reference point.  It is expected that digital signals will also exhibit this type of variation, although it is also expected that the standard deviation will be less than that for relatively narrow band analogue signals.  It is thus possible to estimate the difference between the median signal level and a value appropriate for any given percentage of locations.

The DVB-T system has specified a set of minimum median power-flux densities/equivalent minimum median field strengths for both 75% and 95% location probability calculated for

a) Three different receiving conditions:

1)	Fixed antenna reception

2)	Portable outdoor reception

3)	Portable indoor reception at ground floor

b) Four frequencies representing Band 1, Band III, Band IV and Band V:

1)	65 MHz

2)	200 MHz

3)	500 MHz

4)	800 MHz

c) Five representative C/N ratios in the range of 2 dB to 26 dB in steps of 6 dB.

For each of the above configurations, the following parameters were determined:

a)	Minimum power-flux density at the receiving place

b)	Minimum equivalent field strength at the receiving place

c)	Minimum median power flux density at 10m, 50% of time and 50% locations

d)	Minimum median equivalent field strength at 10m, 50% of time and 50% of locations.

The parameters used for antenna gain were taken from Rec. ITU-R BT.419 [27] for fixed reception.  Building penetration loss figures were derived from Report ITU-R BS.1203 [28] for VHF and from measurements for UHF.  Portable antenna gain was also obtained by measurements.  These tables apply to conventional frequency planning with single transmitter coverage and not to SFN networks.  Also the signal level prediction method to be used will be based on Recommendation ITU-R P.1546 [29].

Annex 4 of ITU-R BT 1368 [26] presents the equivalent minimum field strength values for low VHF, high VHF and UHF bands.

4.4.5	Digital Cable Broadcasting

4.4.5.1	DVB-C System [10]

Many of the system attributes for satellite can also apply to cable where source coding, multiplexing, flexible capacity configuration, SI and CA etc. are concerned; there is therefore a great deal of commonality between the two systems.  Figure 4.2. illustrates the DVB-C system in the form of a block diagram.  However there are several significant differences between the satellite and cable media which make it necessary to have a different channel coding and modulation system.  Typically existing cable systems designed for analogue services have narrower channel bandwidth, better linearity and better channel dynamic range/signal-to-noise ratio than satellite channels.

The bandwidth available in the individual channels on existing cable systems is 7 or 8 MHz.  This contrasts poorly with the 27 to 54 MHz available per transponder on satellites and illustrates the bandwidth limited case of the conventional cable systems and means that satellite QPSK signal would provide insufficient capacity and would be sub-optimal for cable TV given the much better dynamic range of the channel.

Figure 4.2.  Cable System Block Diagram

Because the cable channels envisaged by the DVB project will also carry analogue services, there is a need to ensure compatibility.  Existing analogue cable systems have very good linearity and S/N ratio.  Performance is only limited by inter-modulation, return loss considerations and thermal noise.  The Shannon rule indicates that the higher S/N ratio of cable channels can to support higher order digital modulation schemes than QPSK.  The latter was chosen for the C/N ratio limited satellite channel.  The modulation system chosen for digital cable is higher order quadrature amplitude modulation (QAM) and the FEC comprises only an outer code using the identical 204:188 Reed-Solomon Code together with the same 12 byte interleave as used for satellite.  There is no inner code.

Currently DVB-C specifies 16, 32 or 64 QAM schemes.  This can be extended in future to include a greater number of constellations (e.g. 128 and 256 QAM).

Table 4.1 shows the region of possible transmission rates, the associated symbol rates and the occupied bandwidths for QAM with 16, 32 and 64 state constellations.

| Transmission rate (Mbit/s) | Symbol Rate ( Msymbol/s | Bandwidth ( MHz ) | Modulation |
| --- | --- | --- | --- |
| 18.9 - 25.2 | 5.13 - 6.84 | 5.90 - 7.86 | 16 - QAM |
| 26.1 - 31.9 | 5.66 - 6.92 | 6.51 - 7.96 | 32 - QAM |
| 32.6 - 32.81 | 5.89 - 6.89 | 6.78 - 7.92 | 64 - QAM |

Table 4.1  Relationship between transmission rate, symbol rate,

bandwidth and QAM constellations

Notes :  	Roll off factor = 15%

RS Code	= (204, 188, T=8)

No inner code

4.4.5.2	Cable Networks

Cable television networks are normally structured like trees with trunk routes, branches and sub-branches.  In order to achieve cost effective and reliable distribution of the television and sound programmes, coaxial cable and hybrid fibre/coaxial (HFC) technologies are used.

The DVB cable distribution system is designed to be fed by a combination of satellite signals local programme sources and signals incoming from contribution links; the appropriate programme content is assembled from those sources at the CATV head-end.  The signals arriving from a satellite will usually need to be de-multiplexed at the head-end in order to insert a number of them into the cable programme multiplex.

4.4.5.3	SMATV System

Any discussion of cable networks will not be complete without a reference to a SMATV (Satellite Master Antenna Television) system, which is meant for the distribution of television and sound signals to households in a building or a small number of closely spaced buildings.  The signals are received by a satellite dish antenna.  A cable network structure is used to distribute these signals transparently to a group of users.

The corresponding DVB standard is DVB-CS [30].  Two system configurations are defined.  They are called system A and system B.

4.4.5.3.1		System A:  The QPSK signal arriving from the satellite is demodulated, decoded and then re-modulated in a higher order QAM in order to be fit in a 8 MHz UHF channel.  This process is known as digital trans-modulation (SMATV-DTM).

4.4.5.3.2	System B:  Satellite QPSK signals are delivered directly to the users using one of the two options mentioned below.

- SMATV-IF:  The QPSK signals are distributed along the cable at an IF of between950 and 2050 MHz.

- SMATV-S:	Here, the QPSK signals are distributed in the extended cable S-band (230-470 MHz

4.4.6	Interactive Television Services

4.4.6.1	Introduction

From the point of view of the broadcasters, one of the great incentive for going digital is the possibility of introducing several new interactive services leading to new revenue streams.  The new interactive services currently under development in various parts of the world can be broadly classified into two main categories: those relating to fully interactive television services (Video on Demand (VOD) being the most widely known) and those relating to interactive data services (which may include programme related services such as voting or home shopping).  It is expected that the latter category, also known as narrow-band interactive services, will be introduced first.

4.4.6.2	DVB specifications for broadcast - related interactive TV services

These services [31] are proposed to be introduced using systems consisting of a high-bit rate down stream channel (up to the maximum bit rate of the broadcast channel) from service providers to consumers and low bit rate interaction channels (up to 150 kbit/s).

The following types of services could be supported by a set top box (STB) designed for digital video broadcasts enhanced with interactivity[32].  The required level of functionality increases in each case.

Local interaction:  In this case data or software is downloaded to the STB and consumers are able to decide what the box will select and process (no interaction path is required).  Also, software in the STB can be used to control other local devices (e.g. video cassette recorder, CD-ROM, CD-I).

Response to broadcast services:  In this case the user can respond to broadcast material in an anonymous manner, for instance by voting in a talent contest.  Another example is opinion polling.  These cases only require the number of votes to be counted.  The response can be immediate or by store and forward techniques.

Request for information:  In this case the user requests information from a remote database with, for instance, details of a sporting event or other ongoing broadcast services.  The data that is delivered can be conditional access controlled.  Once again this can be implemented using immediate delivery or store and forward techniques.

Purchase request under broadcast conditional access control:  This case covers the more general requirement for financial transactions associated with, for instance, shopping, banking etc.  It is necessary to prove the identity of the user rather than the STB, and to confirm financial status.  Delivery can be either over air or by mail.

Messaging and remote monitoring:  This case covers three types of services.  Firstly the user can initiate a message to the service or network provider.  Secondly the service provider can send messages for the users to the network provider.  Thirdly the network provider can send messages to the users STB or to the service provider.

4.4.6.3	System Model

Figure 4.2. shows the system model.  Two channels are provided between the user and the service provider.

Figure 4.2. DVB System Model for Interactive Broadcasting Services

The Broadcast Channel:  a unidirectional broadband broadcast channel including video, audio and data from the service provider to the users.  It may include the forward interaction path.

The Interaction Channel:  a bi-directional interaction channel between the user and service provider for interaction purposes.  It is formed by:

1.	The return interaction path from the user to the service provider.  This is used to make requests to the service provider or to answer questions.  It is a narrowband channel and it is also commonly known as the return channel.

2.	The forward interaction path from the service provider to the user.  It is used to provide information from the service provider to the user and any other required communication for the interactive service provision.

The broadcast channel can be transmitted via any of the channels specified by the DVB project.  These currently include terrestrial off-air, satellite, cable, SMATV and MMDS.

Currently specifications/standards for the following types of return channel are in an advanced stage of preparation.

1.	PSTN/ISDN return channel

2.	Cable television return channel

3.	Terrestrial off-air return channel

4.	LMDS return channel

The Systems for Interactive Services (SIS) group within the DVB project has also developed a set of network independent protocols for any broadcast related interaction network configuration [33].

4.5	List of Abbreviations

agl	above ground level

C/N	Carrier to noise ratio (usually expressed in dB)

CA	Conditional Access

CATV	Cable Television

CD-I	Compact Disk Interactive

CD-ROM	Compact Disk Read Only Memory

DTM	Digital trans-modulation

DTTB	Digital Terrestrial Television Broadcasting

DVB	Digital Video Broadcasting

DVB-C	Satellite Digital Video Broadcasting (EN 300 429)

DVB-T	Terrestrial Digital Video Broadcasting (EN 300 744)

ERP	Effective Radiated Power (relative to a dipole antenna, dB)

FEC	Forward Error Correction

HFC	Hybrid fibre/coaxial technologies

ISDN	Integrated Services Digital Network

LMDS	Local Multi-channel Distribution System

MMDS	Multi-service Multi-channel Distribution System

PSTN	Public Service Telephone Network

QAM	Quadrature Amplitude Modulation

QPSK	Quadrature Phase Sight Keying

RS	Reed-Solomon (FEC block code)

S/N	Signal to noise ratio (dB)

SFN	Single Frequency Network

SI	System Information

SIS	Systems for Interactive Services

SMATV	Satellite Master Antenna TV distribution system

STB	Set Top Box

UHF	Ultra High Frequency band (300 MHz - 3 GHz)

VHF	Very High Frequency band (30 MHz - 300 MHz)

VOD	Video on Demand

4.6	References :

- ITU-Recommendation BT.1368 - "Planning criteria for digital terrestrial television services in the VHF/UHF bands".

- ITU-R Recommendation BT.419 - "Directivity and polarisation discrimination of antennas in the reception of television broadcasting".

- ITU-R Report 1203 - "Digital sound broadcasting to vehicular, portable and fixed receivers using terrestrial transmitters in the UHF/VHF bands".

- ITU-R Recommendation P.1546 - "VHF and UHF propagation curves for the frequency range 30 MHz to 1000 MHz".

- ETS 300473 - "Digital broadcasting systems for television sound and data services; Satellite Master Antenna Television (SMATV) distribution systems" - ETSI May 1995.

- DVB Doc. No. A008 Oct.'95 - "European Digital Video Broadcasting project (DVB) : Commercial requirements for asymetric interactive services supporting broadcast to the home with narrow band return channels".

- G.S. Mills, W.H. Dobbie and G-Bensberg - "DVB specifications for broadcast related interactive TV services" - Electronics & Communications Engineering Journal February, 1997.

- ETS 300802 - "Digital Video Broadcasting (DVB); Network Independent protocols for DVB interactive services".

_____

CHAPTER 5

# 5.	DATA BROADCASTING

5.1	Introduction

The emergence of the information superhighway and hence an information society is a reality now. In the changing scenario, a major objective in front of the broadcasting community is to ensure that radio is not left behind in the coming era due to a failure to evolve. The broadcasters must now fight hard amidst fierce competition to ensure survival in the new era. The shift to the digital domain has erased the distinctions between TV, personal computer and radio, and the boundaries separating broadcasting, telecommunications and computer industry are no longer there. It's not difficult to predict that the media market of the future is going to be multi-channel, highly competitive, and have increasingly fragmented audiences. Retaining a competitive edge in the new market environment will mean examining all possible avenues of business growth, both traditional and non-traditional. Aggressive exploration leads rapidly to technologies that can offer business expansion opportunities in the emerging communications markets. A leading contender for business development is new wireless communications services that could be offered using existing broadcast spectrum resources.

5.1.1	Market Potential

Pursuit of new data broadcasting business opportunities could lead to a much-desired second revenue stream for broadcasters. It would be part of the rapidly expanding new wireless communications market. Wireless communications could become the biggest market for new communications services in the coming years and far beyond. An important reason to explore data broadcasting is because of its additional revenue stream potential. Equally important, however, making effective, maximum use of all broadcast spectrum through such pursuits as digital data broadcasting could help the industry protect its spectrum resources from new competitors anxious to gain access to these valued assets.

5.1.2	Emerging opportunity for broadcasters

Strategically, the significance of data broadcasting is that it points toward a greatly expanded and perhaps greatly profitable business environment for sound and television broadcasters. The adoption of a data broadcasting model is also a strategic -- perhaps even defensive -- move to position the industry against potential future spectrum demand by an expanding number of wireless industries and users. While the threat is not immediate, it is no less real. At some point broadcasters will need to strenuously protect their spectrum allocations from the demands of potential new spectrum users. The industry will be challenged to make the most effective and efficient use of all its spectrum resources or risk losing it. [Such aggressive and competitive language has no place in an ITU text.  If the public is better served by these new comers, then this provides for an overall better telecommunication environment which should be the overall purpose of the ITU.]

Today's developing data broadcasting market is a leading indicator of potential future digital market opportunities for the broadcasting industry. The conversion from analogue-based transmission to a digital communications platform could mean a lot more than just better sound and better pictures. The industry's leap into digital era really means that broadcasters can become major players in the digital wireless and data broadcasting businesses of the future. By leveraging the advantages of digital compression, there could be solid opportunities for radio and television broadcasters to develop a local multi-channel broadcasting platform from which new digital audio, video, text, or data services can be marketed to the emerging business and/or consumer markets of the future.

5.2	What is Data Broadcasting?

Data broadcasting is the delivery of digital data, binary or text information from a central source to a large, potentially unlimited number of receivers, independently or along with traditional broadcast programmes comprising of entertainment, information and education. Combining the traditional terrestrial broadcasting technology with emerging digital transmission and compression technologies, the resulting programme bouquet can be a multitude of new digital services provided by radio and television broadcasters to local audiences, consumers, and businesses. The broadcast channels could not only provide high quality sound and video signals to the users, but also provide valuable information to them accurately and fast making the broadcast service more productive, efficient and enjoyable.

5.2.1	Data Broadcasting on Radio services

Today there are over 2 Billion radio receivers in use on the planet. The power of sound radio to inform, educate and entertain at home and on the move is unmatched. Digital technology is now capable of modernising this grand old medium. Capability to receive data in the form of text, graphics sent through the medium either as independent entity or piggy riding along with sound signals, thanks to the digital advancements, has been a significant development.

5.2.2	Digital broadcasting

Digital broadcasting has been talked about for a long time but has only become a technical possibility in the past few years. In retrospect, it has become clear that the rush to create a standard for high definition television (HDTV), first in Japan and later in Europe and the United States, may have been something of a false dawn. What consumers seem to really want is multi-channel television rather than higher quality pictures. Fortunately, digital television can deliver both. The development of digital compression technologies, such as the MPEG-2 (Moving Picture Experts Group) standard for digital video and audio have drastically increased the capacity of the RF broadcast channels in terms of carrying large number of programmes. Progression and migration to digital techniques is likely to take the next ten to twenty years to complete, but it is already having an impact on the structure of broadcast networks.  Traditional one-way, broadcasting is being overtaken by narrowcasting, with some degree of two-way interactivity.  In the future, it may change again to become ‘bit-catching’; that is viewer-initiated retrieval of television programmes or any form of information from a digital video server.

5.2.3	Advantages of digital broadcasting

Digital technology offers many significant advantages. Greater spectrum efficiency will allow a much larger number of channels to be distributed over existing media. A satellite transponder, which currently carries one analogue TV channel and a handful of radio channels should be able to carry around eight compressed digital TV channels. Similar efficiency gains are likely on terrestrial television and on cable television. In the United States, it is planned to introduce digital high definition television (HDTV) broadcasts using over-the-air, while simulcasting with conventional television signals.

Digital methods offer a higher quality picture with better colour, higher definition, CD-quality sound and, if required, wide screen pictures. The multiple channel capacity could be used, for instance, for transmitting multiple language soundtracks with the same pictures.

The digital manipulation of images opens a whole range of new possibilities not available with current analogue techniques. Digital broadcasts can be recorded digitally if the receiver has appropriate equipment. Individual images could then be selected, downloaded to a computer, printed, resized, viewed as still images, inserted into other formats, etc.. On the production side, digital programming allows much more flexibility for subtitling, insertion of advertisements, display of multiple channels on one screen, etc.

Current technology uses VBI based teletext for transmission of data, including on-screen subtitles as well as separate pages of textual information. Digital data transmission will allow for a greater capacity and accuracy for the transmission of data, a much higher definition (permitting better use of still pictures and graphics) and still have space left over for non-entertainment applications.

The extra data capacity is potentially the most exciting new feature on offer and the most likely to lead to new applications further removed from traditional television entertainment. Some companies, such as Reuters or CNN, are already supplementing their news coverage with teletext feeds to deliver data direct to computers. This application is particularly appropriate for financial data on exchange rates, share prices and so forth. It could be tailored for delivery of electronic newspapers using information selected to be of interest to the reader. Closer to traditional television viewing, a viewer watching a sports programme could call up statistics or player profiles to display on screen during an event or for downloading or printing.

5.2.4	Broadcaster's widening roles

As digital signal transport providers, radio and television broadcasters can actively participate not only in the content segment of the marketplace but also in the transport and processing segments. "Broadcasting" as we commonly know it today will change. There is nothing inherent to the technology or the public policy that confine "radio" to be talk shows, news, sports, weather, and music. Nor are there similar limits constraining "television" to be syndicated programs, news, movies, or sports. "Broadcasting" will become a means of digitally transporting multimedia objects, some of which will be resolved by standard receivers as traditional broadcast fare. However, other bits will be intended for receipt by computers, personal digital assistants, and other smart devices. This could include HTML pages and other Internet resources. Policy will permit these "ancillary" private data uses.

5.2.5	Essential requirements for a data system (Radio Frequency environment)

A data broadcasting system, from the standpoint of the broadcasters, enables transmission of data in a one point to multi-point mode similar to audio/video programme broadcast. The same transmitter could carry both conventional broadcast and data signals with provisions to maintain the integrity of individual signals and avoid any interference between the two during transmission. A regular receiver having an add-on system to retrieve the data can deliver the data to the user. It is not essentially a two-way communication though it could be accomplished in a complex way. One of the main concerns of the broadcaster is that he is not assured of the correctness and completion of the information received at every receiver location. The receiver could be fixed or mobile. This necessitates the data transmission system to be robust enough to ensure that the data received is almost correct or correctable using advanced coding methods.  In general the factors that could be considered essential for most of the applications using the data broadcasting systems are:

- Reasonably high data rate that could be achieved in multiplexed transmission

- Immunity to local and external interference

- Satisfactory reception in the service area

- Adequate bit error performance

- Simpler low cost and small size Receiver/Decoder

5.2.6	Requirements of an information dissemination system

Data dissemination systems (Data Server) transmit the actual data along the wired or wireless media with limited or no interactivity between the user and the data provider. The target audience's needs, relevance of events happening and background are taken into account while scheduling beforehand the structure of content dissemination. This could be appropriate, in relation to broadcasting, as it is carried out today with mainly analogue/digital programmes providing video and audio content. As mentioned above, limited interactivity already exists, mostly at the user end, while he has the option to choose a certain bundle of programmes he desires. With data delivery, much more insight is needed in order to get the data system function efficiently, bring the benefits to all users and perform the tasks economically to make the service affordable to the users. The information distribution techniques already exist in the computer communication field, which the broadcasters can exploit.

For example, let us consider an application like the dissemination of an election results, the information server employed has the basic characteristics mentioned below:

1. There is a huge population of users (possibly millions) who want to access the data.

2. There is a tremendous degree of overlap among the interests of the population.

3. The users are following the event closely and are interested only in the new data and changes in the existing data.

4. The amount of data that must be sent to most users is fairly small.

When looking at these characteristics, it becomes clear that request/response method of data delivery as seen in normal computer communication is not necessary. With each request coming up and attended to by the server and with many users requiring the same data, soon there would be long delays in obtaining responses, system overload and the system could crash. To overcome the problem, a concept known as ‘push’ could be used and using data push, the transmission of data to users is initiated without requiring the users to explicitly request it. It is also more suited to a broadcaster's data delivery method. An interactive data service would invoke a ‘pull’ from the user to access needed information from the server and this requires a return channel. There may be need to have a trade off in the use of ‘push’ and ‘pull’ approach, as a system could be catering to different types of data needs unlike the election results dissemination mentioned above, such as News, entertainment, software distribution and so on. A flexible approach is needed that can support wide range of applications across varied environments like terrestrial, satellite and cable broadcasting in order to achieve data system efficiency and conserve bandwidth.

Useful solutions have emerged from recent studies subsequent to the impact of Internet. Concepts like brokered service (creation of intermediate centres in the transmission path for redistribution of information along with locally generated data), filtering of information at intermediate points or at the receiver and caching data at intermediate nodes are some options available to ensure timely and accurate information dissemination to users. Also, user profiles for a target service area could be used to optimize the information content flow to a specific area. The design issues one must consider while building a data broadcasting system are: available bandwidth, ‘push’ scheduling, client storage management, user profiles and feedback, data security and authenticity, data consistency, etc. Further the technical issues particular to the data broadcast system, such as the media, data rates and error performance must be taken into account.[8]

5.3	Delivery mechanisms for data broadcasting

Primary delivery mechanisms for current data broadcasting services include the following:

- Satellite transmission (4/6 GHz band VSAT and newer 12/14 GHz band services)

- Ancillary FM broadcast radio channels (FM subcarrier services)

- Ancillary Television broadcast channels - Vertical Blanking Interval (VBI)

- Cable Television (sideband, VBI or full channel)

- Other Dedicated Radio (RF) Communications Channels (paging networks)

- Telephone networks (land-line and cellular mobile)

In the following sections, different technologies are associated each delivery mechanism (not in the same order), the techniques involved in those and the further developments expected to happen will be examined.

5.3.1	Terrestrial Broadcasting

The terrestrial broadcast of radio and television programming has played a dominant role in information and entertainment delivery to urban and rural communities. These communities rely on broadcasters for public weather predictions and announcements, economic information as well as public safety and service announcements. These services have been provided using technology that was developed decades ago. With broadcast radio, the technology is almost a century old. The longevity of these technologies is due to the fact that they are based on standardized, reliable, and relatively inexpensive receivers. The transmission standard is an analogue one, in which the information (voice and video) is encoded as a continuously variable parameter. While this technology is proficient at distributing analogue signals such as voice and video, it does not allow digital data to be transmitted easily.

Several technologies are currently poised to deliver high quality voice and video as well as data services using conventional terrestrial broadcast frequency allocations. These include the development of high-definition television (HDTV), digital audio broadcast (DAB), and FM high-speed subcarrier (FM HSS) services. Each of these technologies approaches the problem of information delivery from a different perspective and offers various levels of enhanced service. They each have certain drawbacks to their use in exchange for their enhancements.

In urban or rural setting, there is no one dominant type of terrestrial service that is universally available. The accessibility of television and radio broadcasts can vary substantially in different parts of a country. Therefore, there is no one service that can accommodate the needs of every community.

5.3.2	Broadcast Radio

Two separate technologies are being tested to bring digital audio and data services to conventional radio broadcasts. The first, FM High Speed Subcarrier (HSS), incorporates digital data into the conventional FM broadcast by adding the digital data signal to the existing audio signal before FM modulation. Figure 5.1.shows this operation. The second, FM In-band On-channel (FM-IBOC), is a fully digital transmission that is transmitted in addition to the conventional FM. This separate signal is added to the conventional FM signal after the FM modulation, as shown in Figure 5.1. Unlike HDTV, these systems do not replace the analogue service, they provide additional services and are expected to be compatible with conventional AM or FM broadcasts. The additional services are available only to those users with a receiver capable of accessing the digital data.

Figure 5.1:   Insertion of FM HSS and FM-IBOC into a conventional FM broadcast

FM subcarriers have been used for many years in FM broadcast. In a conventional FM broadcast signal, the audio signal can be companded and limited and the carrier deviation can be adjusted such that the stereo sound does not require the full channel bandwidth that is allocated. The additional bandwidth available may be used to broadcast additional services by adding signals to the existing audio signal before FM modulation (as in FM HSS). These additional services currently include the Radio Broadcast Data System (RBDS), also known as RDS (Radio Data System) in Europe, a very low bit rate data stream used to identify stations by call letters and to provide information about program content, paging services; and low bandwidth audio broadcasts like SCA (Subsidiary Channel Authorisation using a subcarrier at 67 kHz) and other services. While some of these services do provide a very low bit rate digital service, prototypes for a new high-speed data service (FM HSS) have been developed by several proponents. This new high-speed data service is incorporated into the FM signal in the same way a conventional subcarrier is added. The differences between conventional subcarriers and the high-speed subcarrier systems are the data rate of the service and the bandwidth occupied by the subcarrier. Current conventional subcarrier systems provide, at most, a few hundred bits per second of data to a receiver. Current prototypes of FM HSS systems provide data rates up to 8 kbit/s; other systems are planned that will provide data rates up to 56 kbit/s.

The current FM HSS systems are designed to provide data services to mobile users. The primary interest is in providing mapping services and traffic information to automobiles as part of Intelligent Transportation Systems. The FM HSS systems are not designed to provide this type of service exclusively, however. The system architecture allows any type of data service to be provided. As in the HDTV system, the data can be addressed to a specific user or group of users with compatible receivers. FM HSS technology has advantages over HDTV technology; it is much more mature and is currently available for broadcast. Since FM subcarrier systems are typically installed and administered by a company that is separate from the broadcaster; there are no equipment costs required by the broadcast station to include this service. In fact, it generates revenues for the broadcaster in the form of fees paid by the subcarrier service provider.

A separate digital system, FM-IBOC, is being proposed for use in the United States as well as in Europe to broadcast digital near-CD quality audio to properly equipped receivers. FM-IBOC is broadcast on the same allocated frequencies as conventional FM. Unlike the Eureka-147 system, FM-IBOC can coexist with the conventional signal because the parameters of the conventional FM broadcast signal can be adjusted not to use its entire allocated channel bandwidth. Figure 5.2 shows how the FM-IBOC signal exists outside the conventional FM broadcast signal, but is still within the channel allocation of the conventional FM signal. As in the other digital broadcasting systems, compression is used to transmit the audio program, allowing for a dynamic data delivery capability.

Figure 5.2:  Frequency spectrum for conventional FM broadcast and DAB

FM-IBOC has a greater data rate available for audio services as compared to the FM-HSS systems, but the ancillary data rate available for non-audio services is comparable to that of the FM-HSS and is dynamically adjustable depending on the instantaneous capacity needed for digital audio. The ancillary data rate available is based on the current level of compression in the primary audio programming.  Both FM HSS and FM-IBOC systems provide one-way transmission only. A separate communication path from the user to the service provider is required for interactive services.

The entry of digital transmission into commercial broadcasting represents a revolution in the types of services that will be available to the public in the near future. HDTV, DAB, FM HSS and FM=IBOC all share a significant advantage for users in that they do not require the installation of new transmission media, such as cable or fibre. This advantage comes with a price in terms of high receiver costs, lower data rates, and the lack of interactive communication (communication from the user to the service provider requires a separate technology).

5.3.2.1	HF Radio

High frequency (HF) radio is one of the oldest forms of long-distance wireless communication, and is a rich and ongoing field of research.

Propagation of radio waves in the HF band of the spectrum, defined as 3-30 MHz, occurs in two primary modes: the ground wave, in which radio wave energy remains near the surface of the Earth, and the sky wave, in which radio waves are reflected from ionised layers in the Earth's upper atmosphere (ionosphere).

Ground wave propagation is relatively simple, in that the received signal can be viewed as a delayed and attenuated, but otherwise undistorted, replica of the transmitted signal.

At longer ranges (typically greater than 150 km), HF communication depends upon sky wave propagation. Unlike the ground wave, the sky wave returns are highly variable and dependent upon frequency, time of day, season, solar activity, and geographic location. In addition to this variability, sky wave signals also suffer a number of impairments, including the interruption of communications by ionospheric storms, a large number of possible propagation paths resulting in time dispersion of a signal, high levels of interference, large and rapid fades, and frequency dispersion of wide-band signals.

Despite these shortcomings, HF has remained the communications medium of choice for many applications, because a low transmit power can often provide extremely long range communications (without the need for repeaters or the use of satellites) when the proper transmission frequency is used.  Moreover, progress in HF technology since the 1970’s, has reduced the need for skilled operators.  Modern HF equipment incorporates a high degree of automation as a result of developments in automatic tuning and antenna matching systems, remotely controllable systems, and automatic link establishment systems.  In addition, hardware developments such as solid-state circuits, highly stable oscillators, frequency-agile synthesizers, fast tuning antenna couplers, and solid-state power amplifiers have improved HF communications considerably.  The advent of very large scale integration has led to smaller, lighter, more power efficient, and more reliable equipment.

HF equipment is low in cost and has a long operating life. HF transceivers cost on the order of $1,000, and HF modems typically cost between $200 and $1,000. Expected life cycles of the equipment may exceed 10 years. HF antennas are rapidly deployable and do not need the accurate siting required for line-of-sight transmission. Antennas can be either omnidirectional or directional, and can be wideband or narrowband, depending on the application.

The recent advances in HF technology, the low cost and durability of HF equipment, and the large area coverage of HF skywave propagation suggest that HF radio may be appropriate for some user services in typically rural areas. Typical bandwidths for civilian uses of HF radio are on the order of several kHz; data transmission rates are generally not more than 1200 bit/s. Thus, HF radio cannot be expected to support applications involving video, but can meet the bandwidth requirements for voice, data, and fax services including electronic commerce and computer networking applications.

Although not currently in widespread use, HF technology presently exists that would enable a remote rural user to access public information. An example architecture that could be used for this application is shown in Figure 5.3. A remote user equipped with a computer, a HF modem, and a HF transceiver can communicate via HF with a gateway that has Internet access or a public bulletin board system. The gateway consists of a HF transceiver, modem, computer, and appropriate interfaces.

Figure 5.3.  Remote user access to public information via HF radio

While HF radio can provide access to the Internet or public bulletin boards, it can only provide limited services due to the relatively narrow bandwidth that it can support.

5.3.2.2	Skywave 2000

Digital technology could be used in the HF broadcasting area also as proposed by Thomcast (BES, India Proceedings 1998). A project named “Skywave 2000” developed equipment for the short-wave band to transmit audio, data and multimedia in digital mode using the existing HF transmission infrastructure. HF transmissions have wide coverage area and traverse large distances enabling radio communication possible. They use the ionosphere in achieving this, however the signals are subjected to fading, absorption and other processes like Doppler effects, multimode phenomenon and multipath effects. Digital coding methods must match the channel through which signals pass. The digital transmission is compatible with the existing channel spacing specified by WARC recommendations. Present receivers could be used along with add-on modules to demodulate and decode the digital data. The digital modulation method suggested is COFDM (Coded Orthogonal Frequency Division Multiplexing) using 16 QAM or 64 QAM. The multicarrier based COFDM exhibits considerable robustness against multipath effects. Another major advantage is the possibility of using the concept of Single Frequency Network (SFN) with digital transmission to make better use of the congested HF bands. The system supports 16 kbit/s with existing bandwidth, and greater rates are possible in the future with wider bandwidths allocated. Some demonstrations of the system have been given in Europe and elsewhere and it promises a brighter future for HF broadcasting. Further work is in progress.

5.3.2.3	AMDS (AM Data System)

Supplementary data transmission on AM sound broadcast transmitters has always held great interest with the broadcasters. UK, France and Germany have actually developed systems for AM data transmission. Consequently Recommendations have been developed to this effect and they are awaiting approval by the ITU. High-power AM transmissions have large coverage areas and included ancillary data could carry useful services such as tuning information, time/date, Programme type, weather information, Radio Paging, telemetry and control, etc.. Due to the limited bandwidth, it can support a data rate of approximately 100 bit/s. The data is transmitted using phase modulation of the main carrier with a maximum deviation of 15 to 28.5 degrees. The British system offers a data rate of 25 bit/s while the German system can offer 200 bit/s. The draft Recommendation ITU-R BS 706-1 on "Data System in Monophonic AM sound broadcasting (AMDS)" describes the data coding and formatting methods including definitions of programme types, country identification codes, etc., similar to the FM data transmission standards. The system block diagram is given in Figure 5.4. The data system could be implemented in LF, MF and HF broadcast networks.

A: additional module (phase modulator)

B: additional module (output decoupling unit)

C: additional module (frequency demodulator and data signal regenerator)

Figure.5.4  Basic circuit diagram of a monophonic AM sound broadcasting system (AMDS)

for supplementary data transmission

5.3.3	FM Data systems

Radio data System (RDS) appeared in the 80’s, offering the capability to add data in analogue FM transmissions in the unused band above the stereo signal spectrum. EBU defined the specifications for the Radio Data System under the document EBU-3244. A subcarrier at 57 kHz is digitally modulated by the data signals after being coded and formatted into blocks and groups. The data information can be received by specially designed RDS receivers having the capability to demodulate and decode transmitted data and display textual information on a display. The data is transmitted at a rate of approximately 1200 bit/s (1187.5 bit/s to be exact). The service helps in the identification of stations, programmes and assist in automatic tuning to programmes. Other data services are also possible like paging, transparent data, in-house information, GPS correction data, etc. In the US also the same technique has been adopted and their version is called RBDS (Radio Broadcast Data Service). Both systems conform to standards and specification defined by ITU-R 643-1 and CENELEC EN 50067.

A project developed in the US employs RDS technology and HF communication in a two-way tracking and messaging network to track and communicate with trucks, trailers and rail-road cars. Vehicle communicates back to headquarters using HF transmitters to bounce a high frequency, low power signal off the atmosphere. The range of signal is more than 1600 km, so a few receive sites can be used to cover the entire US. RDS data meant for rolling stock are sent from central hub via satellite to the FM transmitter network in broadcast mode where the FM transmitters relay the data over the RDS channel. The RDS data is received by a smart-receiver on the vehicles as text display. It is possible to keep track of vehicles using the system and the transport fleet could be deployed more effectively and efficiently. The system could be implemented very fast and economically since the FM broadcast infrastructure is already in existence (Radio World Dec.1998). Similar projects and experimental trials have been reported by the RDS Forum, Radio Paging Association and other broadcasters in Europe.

5.3.3.1	High-Speed data systems in FM

High-speed data system capable of carrying data at higher speeds on FM analogue channels have emerged using subcarrier techniques and digital modulation methods. They are DARC (Data Radio Channel) developed and implemented by NHK in Japan, HSDS (High Speed Data Systems) developed by Seiko and used by commercial broadcasters in the US and STIC (Subcarrier Transmission Information Channel) developed in the USA. Of these, the DARC specifications are defined by ITU-R Recommendation BS.1194. Other systems are also in the process of being included in the Recommendation. The data rates supported are 16 kbit/s to 19 kbit/s. The subcarrier frequencies are in the range of 66.5 kHz to 76 kHz offering a bandwidth of +/- 15 kHz. The comparative summary and details of the systems are given in the Table 5.1. The high-speed data service is faster than RDS and more robust in terms of performance. In addition to text information, voice, picture and multimedia could also be transmitted over the high-speed data channel (Figure 5.5). The broadcaster can use his existing transmitter for these services without affecting the main sound programmes. He needs only to add the data encoder to transmit data. With a large coverage area available in commercial FM services, the radio users can benefit from these data services that should perform adequately even under mobile conditions. The user must have the data receiver, which is an enhanced version of the standard FM receiver with data demodulation, decoding facility and an LCD display or an outlet to a PC. Japanese companies such as Sanyo and Oki have developed the ASIC’s to decode DARC data signals from multiplexed FM transmissions. The technology could be very useful and profitable employing the existing FM services.

| Parameters | DARC | HSDS | STIC |
| --- | --- | --- | --- |
| Subcarrier Frequency | 76kHz | 66.5kHz | 72.2kHz |
| Bandwidth | 44kHz (-40dB) | 16kHz (60 to 76kHz at -60dB) | 16kHz (-50dB) |
| Channel data Bit rate | 16 kbit/s | 19 kbit/s | 18.05 kbit/s |
| Information data rate | Method A: 6.83 kbit/s Method B: 6.95 kbit/s Method C: 9.78 kbit/s | Packets- 10.51 kbit/s Small blocks-8.3 kbit/s | 7.6 kbit/s + short delay data |
| Modulation method | LMSK | Duobinary DSB suppressed carrier AM | /4 shifted DQPSK |
| Error Correction | (272,190) product code | Packets- Interleaved Hamming (12,8) Small blocks- Time spread packet with additional Hamming | Reed-Solomon (243,228) + 1/2 convolutional code |
| Error Detection | 14 bits CRC | 16 bit CRC | Accomplished by Reed-Solomon decoding |
| Injection Level | Varies from 3kHz to 7.5kHz | 3.75kHz to 7.5kHz nominally 7.5kHz | Nominally 7.5kHz |
| Additional Features |  |  |  |
| Power Saving | Yes | Yes | Yes |
| Receiver Addressing | Yes | Yes | Yes |
| Support of different data type | Yes | Yes | Yes |
| Compatibility with RDS | Yes | Yes | Yes |
| Multiple Station access | Yes | Yes, including time offset transmission for paging type data | Yes |
| Handle Re-broadcasting | Yes | Yes | Yes |
| Service Status | In operation in Japan Trials in Europe | In operation in USA | Trials in USA |

Table 5.1

Figure 5.5.  Services carried by FM HSS
(Courtesy : Sectra, Sweden)

5.3.3.2	DAB

The DAB technology (Eureka-147) was developed to provide very high quality audio reception under fixed and mobile conditions overcoming many of the deficiencies encountered in conventional analogue transmission techniques. The DAB system can also be used to carry a large variety of programme-associated and independent data services. The programme-associated category provides ancillary information on the audio programme received off-air. Independent data can range from real-time paging data to on-demand file downloads, games, finance, weather, news and many more directed to a dedicated receiver incorporated in a desktop, laptop or even a palmtop computer. DAB system's immunity to multipath and other reception impairments will guarantee error-free data reception under mobile environment. The receiver system is software intensive and support dynamic programming.  It will also support conditional access and any service that may be introduced in the future.

5.3.3.3	DAB Standards and further developments

Many standards have been defined to address the requirements of flexible audio coding, data transmission, change in transmission parameters and access methods. ETSI and ITU-R have defined the standards ETSI 300 401 and BS1114 respectively for DAB.  ETSI has also developed the Multimedia Object Transfer Protocol (MOT) for multimedia data. Standards for terrestrial and satellite mode delivery and reception have been finalised or are under process. In addition, a common interface at the receiver, named the Receiver Data Interface (RDI), has been developed to connect various dedicated application decoders to computers and printers. Decoders for data applications are also under development and efforts are being taken to implement a return channel for receiver control from an application terminal.

5.3.3.4	DAB - a multimedia carrier

Developments have taken place for using the Eureka-147 system as multimedia and data broadcasting system in addition to carrying conventional audio services. The system is opening many new opportunities to carry a number of non-audio services like text, still pictures, moving images, etc., Examples of DAB data services currently being implemented are given below. These services may be presented either in the form of text information (shown, for example, on a simple receiver display of 8 to 128 characters), still pictures or even video images on more sophisticated displays (Figure 5.6).

- Programme associated services such as current song title, name of interpreter and performer, lyrics, news headlines, CD covers, etc.

- News including events, traffic messages, weather, sport, stock market, travel and tourist information.

- Traffic navigation by means of transmitted digitised road maps, combined with position information provided via GPS (Global Positioning System).

- Advertisements and sales including sales catalogues, purchase offers, etc.

- Entertainment including games and non-commercial bulletin boards.

- Closed user group services such as banking information, electronic newspapers, fax printouts and remote teaching.

Figure 5.6 Network structure for carrying PAD information

Ideally multimedia should be interactive, in which case the consumer can communicate with the service provider's database. Since the broadcast services are one-way only, the return channel could be provided, for example, by a GSM telephone (in case of mobile DAB) or via PSTN (in case of a fixed receiver). A semi-interactive mode is also possible. In this instance, the information is downloaded by the service provider to the user's data terminal and stored as a database. All interactivity is then handled within the user's data terminal, but the database contents are to be updated regularly by the data service provider. The storage capacity of the user's terminal is part of a trade-off between the service transmission rate, the repetition rate and the cost of the memory.

A key to the success of DAB will be its ability to address each receiver individually. This will allow the service provider to customise the "bouquet" of services provided to each user, and to identify the user in an interactive transaction via the use of a smart card.

Developments are taking place on the suitable presentation of data. Currently, data services specified in the ETSI standard have text-based transmission and presentation. Transmission of data such as graphics and similar information has been addressed by ETSI with the definition of the MOT (Multimedia Object Transfer) protocol standard. In order to improve the man-machine interface, the DAB receiver should also support graphic user interface, such as, for example, Microsoft Windows. This will be of importance for screen-based services that seem to be relevant for stationary receivers and portable receivers. For mobile receivers, facility which would cause less distraction to drivers like synthesised speech could be useful.

For the user's data terminal, a unified transmission protocol will be very helpful, as no distinctions between different transport mechanisms would be necessary. A software-based language for object-oriented page description is emerging to define a communication and a presentation layer. Such a unified protocol could be used not only with DAB services, but also in other communication systems.

5.3.4	Broadcast Television

Analogue television has been in existence for many years. Several enhancements have been added to this basic system, including stereo audio encoding, additional audio programming channels, very low data rate digital transfer (closed captioning), Teletext and ghost cancelling. These are minor enhancements compared to the technological improvements represented by HDTV. In Europe the transmission standards are defined by the various versions of PAL and SECAM, with slightly differing parameters.

HDTV is the term used to describe a proposed video and audio delivery that departs from the NTSC/PAL/SECAM standards to provide significantly higher resolution video and multi-channel audio, as well as data services. The system currently deployed the United States offers several display resolutions (up to 1920 x 1080 pixels) and 5-channel compact disk (CD) quality surround sound.

After a period of competition between proponents of various systems, several organizations joined together and formed a consortium called the Grand Alliance. The Grand Alliance HDTV System is a transmission standard for digital television. It does not include any video production, display, or consumer equipment interface standards. The system does specify that the video and audio are to be digitised, compressed, and transmitted in a data stream according to the Motion Pictures Experts Group (MPEG-2) format. The data is to be broadcast using an 8-level vestigial sideband (8-VSB) modulation as per the ATSC proposal.

The specifications for the HDTV System call for a flexible data packet structure that can deliver many services to the receiver. Besides the video and audio packets, ancillary data packets are included in the specifications that allow the broadcast of any arbitrary data stream. The broadcast permits one-way data transmission only. Systems that require interactive communication need to provide an alternative means of communication back to the service provider. The packets can also be addressed to a specific receiver or group of receivers, allowing the data to be decoded only by those receivers that request it. Using the ancillary data service, some examples of services a broadcaster could provide include programming information, software delivery, video or audio delivery (like pay-per-view programming), and instructional materials.

The ability to send high-resolution video and audio in the same 6 MHz allocation used by conventional NTSC television relies on advanced digital compression technology. The compression algorithms produce a data flow that is dynamic in its data rate requirements. That is, when the program can be heavily compressed, such as a slowly changing scene or still, the data throughput required is low for the video. When the program contains rapidly changing scenes, the required data rate is higher. As a result, the entire 19 Mbit/s rate provided by the transmission system is not required for the video and audio at all times. There is room for other types of services to be broadcast, which may not be related to the primary video program.

The data rate available for ancillary services is dynamic and ranges from a few kbit/s to many Mbit/s, depending on the video and audio program content. Transmission of the ancillary data has a lower priority than transmission of the primary program, and must wait for an opening in the data stream for transmission. Such ancillary data cannot normally provide real-time services.

Data broadcast is particularly suited to rural areas. Covering large areas, as typically required in rural areas, can be achieved by using higher transmitter power, higher antenna height and/or higher antenna gain at either the transmitter or receiver rather than requiring additional wired transmission media

5.3.4.1	DVB

The digital coding and transmission of television picture and sound signals can offer considerably more than is possible with the older analogue coding techniques used in PAL, NTSC and SECAM television systems. It is recognised that the additional power and efficiency of digital signals might be useful in a number of ways, namely, more programme transmissions per channel, higher quality of pictures and sound, increased service area coverage, reduced transmission powers and some combinations of the above. Some tests and field trials in the UK and elsewhere have established the possibility of offering a larger number of channels of SDTV standard in a  regular 8MHz channel using digital methods. A similar increase in capacity in satellite mode delivery has also been demonstrated. A standardisation process has been completed for terrestrial, satellite and cable modes of operation. The enormous scope offered by the digital techniques have opened the possibility of using the medium for fast, high-capacity data applications also, offering a number of value added services.

5.3.4.2	Video Source coding

Systems for generic source coding and multiplexing have been developed by ISO-IEC MPEG. MPEG 2 standards have been included in DVB, ATSC and ISDB systems and ITU-R Recommendations have been developed accordingly.  These Recommendations are receiving increasing support from the broadcasters, service operators and industry. The key to the digital video broadcasting is the concept of compression based on the reduction of the capacity required to transmit the DCT coefficients within a frame and the process of removal of intra and inter-frame redundancy in the picture information prior to transmission. The technique involves transmission of differences only between sequential picture frames, following an alignment procedure, which takes account of picture element displacement due to motion (plus of-course the transmission of motion compensation information necessary for the re-alignment of picture elements in the receiver).

5.3.4.3	Audio source coding

The European DVB specification for audio is based upon MPEG-Layer 2 (MUSICAM and multi channel MUSICAM) as well as the Dolby AC-3 standards. The ATSC standard includes the AC-3 scheme whereas the ISDB standard uses the AAC coding scheme also developed by MPEG-2.  With MPEG 2 layer 2, compact disc stereo quality is maintained at a bit-rate of around 256 kbi/s. A hierarchy of bit-rates has been defined like-wise for FM stereo quality (128 kbit/s) and for many other quality levels (Mono, etc.,) with digital mode transmission of audio.  AC-3 provides some improvement over MPEG-Layer 2 for high quality audio signals and the more recent AAC provides even more quality improvement.  Work is still on-going in the field of audio compression scheme to provide a given audio quality at lower bit rates (e.g., CD quality at 128 kbit/s).

5.3.4.4	Multiplexing

The service multiplex methods developed and standardised by MPEG have received worldwide support from Administrations and broadcast community. The transport stream multiplexing methods specified in ISO-IEC 13818-1 form the basis of the DVB, ATSC and ISDB multiplexing specifications that are contained in ITU Recommendations. It is based on a fixed packet length transport stream (188 bytes). The overall system multiplexing approach can be thought of as a combination of multiplexing at two different layers. In the first layer, single programme transport streams are formed by multiplexing transport packets from one or more elementary bit streams, and in the second layer, many single programmes transport bit streams are combined to form a system of programmes.

5.3.4.5	Modulation and channel coding

The basic performance of a digital system depends on a theoretical error ratio requirement. Employing a convolutional error coding at the transmitter along with a soft decision Viterbi decoding scheme at the receiver, a bit error ratio of 2x10-4 is required for the transmission of almost perfect television picture, which in turn implies a requirement for a C/N ratio of about 10.4 dB for QPSK, 17.5 dB for 16QAM and around 24 dB for 64QAM. Even though margins of 1 to 2 dB need to be added to the C/N requirement to allow for the implementation in practical equipment, they compare favourably with those needed for analogue PAL where a C/N of 45 dB is require for picture grade of 4.5. Further improvements in digital system performance are gained from the use of sophisticated error coding / protection schemes in which some of the useful data carrying performance is efficiently traded for increased ruggedness. These schemes are also tailored to the characteristics of the channels for which they are implemented and therefore some differences exist for the satellite, cable and terrestrial media.

5.3.4.6	Carrier systems

In a single carrier system the useful data packets are multiplexed with the synchronising and framing information to form a continuous stream of input data to a conventional single carrier modulator. This scheme provides a relatively simple receiver/demodulator and is suitable for use in channels relatively free from multipath effects. It is useful for satellite broadcasting using QPSK and 8-PSK but less ideal for terrestrial channels. A sophisticated channel equaliser would be needed in terrestrial mode to deal with multipath effects. The system is also suitable in well-engineered (low reflection) cable networks. In a multicarrier system, like OFDM (Orthogonal Frequency Division Multiplexing), groups of sequential data packets are effectively stored for a period known as the symbol period during which a large number of carriers are generated and modulated in parallel with these packets. Thus the data rate of each carrier is divided by the number of carriers and there is some control of the positioning of the data elements in the transmitted spectrum for specific interference avoidance if needed. While the demodulation algorithm (FFT) is more complex, the performance in presence of multipath effects and interference is inherently superior. Time delayed echoes creating unwanted overlap of successive symbols resulting into Intersymbol Interference (ISI) can also be dealt with by the inclusion of a guard interval between symbols in order to absorb this symbol overlap. This allows the incoherent echoes from the previous symbol to die away before the receiver starts to integrate the current symbol. In so doing, the receiver can integrate all the power of the main signal as well as the delayed signal to create a S/N corresponding to the sum of all these signals, hence improving the reception performance by adding these phase-random signals on a power basis.  The multicarrier system is essentially for terrestrial delivery and reception.

5.3.5	Data Broadcasting in DVB

Though designed basically for providing superior quality television programmes delivery and distribution and optimal use of the spectrum, the enormous flexibility built into the system lends itself to provide a variety of services (HDTV, multi-programme bouquets, ancillary data services, possibly with interactive extension through return channel, more local services, etc.). It can include digital radio broadcasts, computer software, electronic magazines, real-time messaging, university lectures and business presentations with conditional access capability. This will create extra revenue earning possibilities for the broadcaster. Programme production industries could diversify into new potential areas of operation like multimedia information generation, delivery, presentation and so on. Consumers might have to spend some additional fee to have access to the system and services by way of acquiring new hardware and software. But they are assured of more reliable reception, better picture quality, more programme choice and new service innovations.

5.3.6 	Obstacles in the path of migration to digital domain

Though clearly advantageous in terms of reduced operational costs as well as capacity for greater programme content, flexibility and optimum spectrum utilisation compared to conventional analogue television, digital television does pose certain hard decisions to be taken at some time in the near future. Firstly, the timeframe to convert to all digital operation. Already, well-established analogue services are in existence in the satellite, terrestrial and cable domains. (Cable is in the process of migrating to Optical Fibre and digital operation). A simulcast solution has been suggested up to a certain period for the technologies to mature and settle. Apart from that, the introduction of new services, availability of new antennas and equipment (set-top boxes) are essential. The standardisation in the area of conditional access is also needed to rid consumers from system access hassles and provide transparent operation. There are actions to be taken in the area of spectrum management for different requirements and contingency measures for the transition period. The various factor to be considered are summarised below:

- The need for terrestrial and satellite receive antennae;

- The cost of receiving equipment;

- The number and quality of programmes to offer;

- The build-up of volume production of common critical components (e.g., MPEG chipsets);

- Public awareness and interest in new services and equipment;

- Technology advancements.

5.3.7	Satellite Data Broadcasting

Data services could be provided via geostationary satellites as they are better suited for transmission of large amounts of data. Data broadcasting from satellite direct-to-home and business users has a potential advantage, among others, of being lower cost. The existing infrastructure is used to distribute electronic content to various users' PCs. By delivering information more than 1000 times faster than the Internet to the user, data broadcasting is the quickest and cheapest way to download multimedia information with video, animation and CD quality sound. Information is packaged and comes from different service providers to the satellite uplink, where it is scheduled and transmitted to the consumer. The consumer looks at the transmitted version of the electronic programme guide (EPG) and selects to receive the desired information. Software packages and updates could be distributed with this method, allowing subscribers to request or purchase the package. Then at the time of transmission of the package, the consumer's computer would receive the package and store it on its hard disk. With many consumers wanting to download a new version of the package in the same timeframe, data broadcasting is a very economic way of distributing software. It need not be restricted to files. Many form of information could be sent overnight or during off-peak hours of transmission. The user terminal would consist in a satellite dish directed to the satellite, an in-door unit connected to the PC via a hard-wired co-axial cable interfaced with a MPEG-2 compliant Integrated Receiver Decoder (IRD). Once decoded, the data would be saved on the hard disk for later review by the user. The satellite covers wide geographical areas hence many subscribers could be served by the system using broadcast mode delivery. Since the satellite infrastructure, uplinks, etc., are already in place, the service could be implemented very quickly. During off-peak hours, a lot of transponders would be available for data broadcasting. For example, assuming 10 full transponders available, this would give possibly a data rate of more than 300 Mbit/s, allowing the system to transfer a gigabyte in 24 seconds. Although theoretically possible the consumer's system must be able to absorb all the data. Much of the data may not be required by the user, hence some filter mechanisms are needed in his system to select the desired information from the bulk of the data.

In summary satellite mode data broadcasting has the following advantages:

- Low-cost access to most of the services of the information superhighway;

- New revenue streams for satellite operators and DTH broadcasters;

- Electronic data delivered many times faster and cheaper than the Internet;

- Immediate operation of the service.

(Ref: TV Technology & Production, March 1999)

5.3.7.1	WorldSpace Satellite to Radio, Multimedia Broadcast System

A satellite system has been developed by WorldSpace Inc., that can deliver digital radio audio signals having qualities equivalent to AM mono, FM mono, FM stereo and near-CD stereo to small portable, low cost personal receiver. The system incorporates source coding using MPEG-2 Layer 3 which is claimed to achieve the above qualities at bit rates of 16, 32, 64, and 128 kbit/s respectively. Error rates over the system can be less than 10-10 and thus be also suitable for high quality digital image and data transmission for multimedia services. Uplinks are accessed using FDMA QPSK modulated carriers in the 7 GHz band, each supporting a prime rate of 16.056 kbit/s. These building blocks are combined to achieve the cited bit rates and corresponding qualities. Small low cost, VSAT sized broadcaster's earth terminals are used to access the uplinks. Downlinks use time division multiplexing carried on a single carrier per channel in the 1.5 GHz band (1467 to 1492 MHz). Each of the three beams from the satellite carries one RF channel (BW= 2.5 MHz). Conversion from the uplink FDMA carriers to the three single TDM carriers is done by an on-board baseband processor. The baseband processor also routes each uplink channel to any or all of the downlink beams. The system produces for high enough receive margins suitable for stationary and some LOS mobile outdoors reception.

The radio receiver is a key part of the system. It is designed for maximum convenience of use at a minimum cost. A low-end rudimentary receiver is expected to cost around US $200. Such a low cost radio receiver with a small, compact patch antenna having about 4 to 6 dBi gain will require virtually no pointing and will tune automatically to selected channels. More expensive receivers would have additional functionalities including a self-contained display and computer interfaces.

Broadcasters could franchise the satellite channels for providing service in a desired area. They would require small earth terminals equipped with 2.4 m diameter parabolic 10GHz dish antenna and 25 Watt power amplifiers to transmit a 128 kbit/s programme channel (comprising of 8 prime rate channels) to the satellite from a site in the country originating the programme. Alternately, programme channels can be connected to a shared uplink earth terminal via terrestrial links. The WorldSpace system will ultimately comprise three satellites to cover most of Asia, Africa and South America. (Ref: BES Review Vol. II, No. 3, July - Sept 96)  Two satellites are currently in operation over Africa and Asia.

5.3.8	Broadband Wireless

Broadband wireless communications could in principle use several physical media ranging from radio frequencies to light. The current focus is on the propagation of electromagnetic waves in frequency range form some hundreds of MHz to GHz. MMDS, MVDS and LMDS are among the newer systems that can be encompassed within the term 'Broadband Wireless'. MMDS (Multichannel Multi-point Distribution System) operate in the 2.5 - 3.6 GHz, while MVDS (Multi-point Video Distribution Service) and LMDS (Local Multi-point Distribution Service) operate at higher frequencies and within smaller cells, at 40 GHz and 28 GHz (upstream and downstream).

MVDS, MMDS and LMDS are identified as an alternative method for essentially local terrestrial television services. They can deliver many more channels than is possible with existing UHF terrestrial networks, with implicit requirement to use higher frequencies where spectrum is less congested. The rationale behind the developments was the desire to provide existing and future cable operators with a wireless radio based delivery mechanism. It was a means of augmenting normal cable coverage by dealing with pockets of franchised area usually considered not viable for cable deployment, an alternative to cable itself if economically viable and a means of providing a quick and efficient way to make available programming to customers.

MMDS is an adaptation of the system that has been in use in the US since the early 1960s to broadcast educational TV programmes to schools and colleges using frequencies around 2.5GHz. The system has evolved to become a well established, proven, mature TV distribution technology which has taken place alongside cable, broadcast and satellite as a medium for transporting TV programmes direct to home and to cable head-ends.

Though MMDS came too early, it is well placed to exploit the emerging technology in the digital domain to extend the range of TV channels available to the viewers. Digital techniques will enable four or more TV channels, to be compressed into the bandwidth of a single analogue 8 MHz PAL TV channel, without noticeable degradation of picture quality. During 1994, test transmissions of digitally compressed high-definition TV were carried out over an Irish MMDS system under the European EU RACE programme. The compressed 34 Mbit/s HDTV signal was successfully transmitted over an MMDS system using COFDM modulation. The experience has established the viability of 2.5 GHz MMDS technology for multichannel TV delivery direct to home particularly in rural areas. With digital capability MMDS is another potential candidate for multimedia and data transmission.

5.3.8.1	Multichannel Multipoint Distribution Service (MMDS)

MMDS is more commonly known by the oxymoron "wireless cable". MMDS is a radio alternative to cable TV distribution. It may be particularly important in rural areas because it can be used to distribute up to 33 TV channels in some situations where there is neither local broadcasting nor sufficient customer density to support a regular cable system. In this context, it is primarily an alternative to satellite TV, with the additional advantage that MMDS can supply local channels.

MMDS is allowed to radiate up to 100 Watts per channel using an omni-directional antenna. The receivers, usually at a home, use a high-gain directional dish antenna (about 30" in diameter) and a down-converter (that usually converts the selected channel to ordinary VHF TV channel 2 or 3). Although the range of the service is generally limited to line-of-sight operation , line-of-sight can be 16-32 km if the terrain is favourable. The receiver requires only a simple frequency conversion, since the TV signal is already broadcast in the proper analogue NTSC, PAL or SECAM format, suitable for viewing on an unmodified TV set. However, the down-converter must also provide for the necessary decoding and access to prevent pirate viewing.

Typical costs of MMDS for the rural homeowner include a typical $30 monthly fee for antenna and down-converter rental and program subscription costs. It is claimed that an MMDS system operating in a rural environment can earn a profit with only 800 subscribers (about 10% of the total number of subscribers required for profitable operation of a normal cable system).

Local multipoint distribution service (LMDS) may provide a similar service as MMDS at some future time. LMDS uses frequency bands near 30 or 40 GHz. LMDS might be considered a higher frequency version of MMDS. LMDS is expected to provide general two-way services, including voice, video, and data in a large city environment, in addition to TV. This suggests that the role of MMDS might also be extended at some future time to include general telecommunications.

As in the cable TV industry, the MMDS industry is experimenting with digital compression techniques. For example, one system being tested puts 10 TV channels on a single 6-MHz bandwidth. Other systems provide 4:1 compression. The prospect of MMDS with 200-300 video channels completely changes the economics of providing MMDS. In addition, return path channels are mostly unused. Some operators have proposed using the MMDS channels (2150-2160 MHz) for return path channels. The use of return path channels could allow interactive video services such as home shopping and video on demand as well as general two-way telecommunications.

5.3.8.2	ISDN

The integrated services digital network (ISDN) is a set of common standards for combining separate telephone and data networks. Voice and data services are integrated in an ISDN network because they are processed in the same way as digital bits. This simplifies management since one network can support both types of information. It also eliminates some of the limitations of using the telephone network for data communications. Users can simultaneously talk and transfer large amounts of data over a single line. For subscribers with an ISDN connection, there is no need for a modem since digital data no longer has to be converted to analogue for transmission over the telephone network. This produces faster and more reliable connections, as well as higher data transmission rates. ISDN is also more efficient than data transmission with a modem since control information is separated from the data being transferred, resulting in faster call set up and termination. The greater data transmission capability offered by ISDN should result in faster transfer and lower costs compared to the conventional telephone network.

ISDN is being standardized under the auspices of the ITU Telecommunication Standardization Sector (ITU-T), notably through the I-Series Recommendations. There are two types of ISDN implementations in use today that work over conventional copper wire telephone lines:

Basic rate access, which comprises two "B" or bearer channels of 64 kbit/s (56 kbit/s in North America and Japan) each allowing a total data transfer rate of 128 kbit/s or about 2.5 times faster than transfers using a telephone line and high speed modems. The B channel can also be used to carry a single voice channel or, with data compression, up to eight separate voice channels. There is also a "D" or data channel of 16 kbit/s, which carries control information or packet data.

The primary rate access consists of multiple B channels (30 in Europe, 23 in USA and Japan) providing service at 2.0 Mbit/s (1.544 Mbit/s).

There is no distinction between voice, text image or data on the B channel; they are all treated as strings of digital bits. When a call is made, the customer's equipment sends a signal on the D channel containing the information needed by the network to establish the call. Data or voice is then transmitted over the B channel. The customer's equipment sends a termination signal down the D channel to end the call.

5.3.8.2.1		Applications of ISDN

Extra telephone functionality: Information identifying the caller's telephone number (caller line identification, or CLI), for example, can be transmitted on the D channel. This information can be directed to an organization's computer database that can contain customer or supplier details, so that when the telephone is answered the respondent already has the details of the caller on-screen. This function has particular applications in telemarketing, emergency services, and customer service help lines.

Enhanced audio: ISDN's high quality audio capability can be used for sound broadcast applications such as FM-fidelity stereo. It should be possible to use ISDN lines for distributing music, for instance, to relay live concerts to remote areas.

Connection of computer systems: This includes linking local area networks (LANs) and, via ISDN adapters, providing personal computers with rapid data transfer capability. ISDN connections have become popular in Europe recently for residential users and small businesses with dial-up access to the Internet. The 64 kbit/s connection is more efficient than using modems that often operate at 28.8 kbit/s due to telephone line quality. (Faster rates, up to 56 kbit/s, are available with currentmodem if the quality of the telephone line allows it.) Indeed, one of the reasons why ISDN subscribers have grown more rapidly is because this has coincided with a rapid increase in usage of the Web browsing capabilities on the Internet which makes heavy use of graphic images.

High-level application support: Video codecs (coders/decoders) for desktop conferencing and videophones can work at ISDN speeds, either with still images at 56/64 kbit/s or full motion at 1.5/2.0 Mbit/s. The relevant ITU-T Recommendations for audio-visual services form part of the H Series.

ISDN is an alternative for leased lines. ISDN should be less expensive than fixed leased line charges, though this is not always the case; ISDN is often priced as a premium rate service even though it uses the same copper wires as normal voice telephony. ISDN may be favoured by those organizations whose requirements for transmission capacity vary considerably according to how much capacity they need and when they need it. With primary rate ISDN, it is possible for the Post and Telecommunications Operator to provide bandwidth on demand by offering fractional speeds in response to short term demand.

ISDN supports the digital fax transmission Group 4 facsimile which is up to eight times faster than a Group 3 fax and has much lower error rates. Group 4 can also be used for transmission of colour fax.

5.3.8.2.2 	ISDN developments

Existing narrowband ISDN technology has one big limitation. It assumes a basic channel of 64 kbit/s which cannot support services requiring large bandwidth such as full-motion video. Thus activity is underway to define a new set of standards for broadband-ISDN (B-ISDN) that would eliminate the fixed channel structure of narrowband ISDN. B-ISDN, for example, will provide bandwidths between 155 and 622 Mbit/s, 80-300 times greater capacity than the existing primary rate ISDN. This will support virtually any foreseeable commercial application. Customers could then be offered either a bandwidth-on-demand type service at fractional speeds, or a permanent bandwidth connection according to requirement and ability to pay.

The ITU-T work programme currently has more than 400 standards at different stages of drafting which relate in some way to ISDN. The goals of the standardization effort are to ensure global interconnectivity of national B-ISDN networks as well as standardized equipment specifications for all interfaces to the network, something, which has plagued narrowband ISDN. B-ISDN also aims is to achieve a tight integration with exchange and transport technologies to minimize limitations in the switching and transmission of broadband information. Asynchronous transfer mode (ATM) standards have been adopted as the preferred transmission mode for implementing B-ISDN. ATM is a cell-switching technique for the transfer of user information, where each cell has a fixed size of 53 bytes. This makes it particularly well-suited to transmission and switching of multimedia applications because of the ability to offer variable bit rates (bandwidth on demand) which is more difficult with packet-switching protocols, such as X.25 or frame relay.

5.3.8.2.3	Benefits of ISDN

There are a number of advantages for the users:

Higher quality of voice transmission is the most noticeable feature. Indeed, one US carrier uses the sound of a pin dropping in its advertisements to emphasize this feature. When voice, data or text is converted into a series of numerical values, it can be decoded by the called party's terminal with higher fidelity, a much lower error rate, and minimal interference.

Lower maintenance costs is a bonus mainly for the network operator but the subscriber also benefits since digitisation improves reliability. Progress towards digitisation in Morocco has been matched by a falling incidence of network faults.

Additional user functions become available to subscribers attached to digital networks. Many of these simply emulate functions available to users of business private branch exchanges, such as call forwarding or call waiting, but other functions actually utilize the data transmitted over the network. The most useful, and most controversial, is caller-ID, which allows the called party to see the number of the caller (unless the caller has specifically requested anonymity). Other functions include displaying cost of the call (for real-time billing and bill checking purposes), music on hold, teleconferencing, and allocating different dialing tones to different numbers addressable from the same telephone.

Use of data transmission facilities, both for receiving and making calls, is much easier on a digital network. Where ISDN service is available, data communications can be much cheaper because of the higher bandwidth, but even where ISDN is not used, a digital line will provide a much lower level of error transmission that an analogue line.

5.3.9	Cable TV distribution

Broadband cable television systems hold the key to large scale, cost-effective and efficient delivery of vast amount of entertainment, information and educational programming. Depending on any region's particular geography, regulations and economic situation, cable TV can serve an entire population with services from the most basic television programmes to state-of-the-art video, computer and telephone services. Satellite backed cable TV, an industry started in the US, provides one answer to the needs of the communication infrastructure. After initially focussing on entertainment and news TV, the worldwide cable industry is increasingly involved with TV programming that is informational and/or educational in nature. With today's evolving technology, a well-built cable system can also provide high quality wired or wireless telephony services and broadband links to home or office computers for high-speed data and multimedia communications. This section focuses on the technology, systems and services possible over the infrastructure- video, data and telephony. Some other points of interest would be signal scrambling, digital video compression and multimedia data.

A key factor of cable TV, versus other video communication technologies, is its self-contained spectrum. Signals moving over a well-built and well-maintained cable system are isolated within the cable, so they do not interfere with other signals using the same frequencies over the open, air-waves. Because of that, depending on the local government rules, cable can use the same frequencies assigned for other purposes in the over-the-air environment, and protective frequency spacing between signals is not necessary. (However in the US, FCC does mandate that cable systems not overlap with the over-the-air frequencies used by the Federal Aviation Administration for aeronautical voice communication and navigation information. This is to ensure the integrity of aeronautical communication against possible signal leakage from near-by cable systems).

A cable system has tremendous amount of capacity for the entire range of communication transmission. The first cable systems were used solely to retransmit the signals of off-air broadcast television stations to homes that otherwise could not see them due to far-off distance or blocked by congested urban buildings, mountains or geography. Use of satellite for TV opened possibilities of using cable for the distribution of new satellite services.

5.3.9.1	Essential components of cable TV system

The traditional cable system transmits signals one way, point to multipoint delivery to a city's entire population. The basic cable system is composed of co-axial cables (or fibre/coax). Essential parts of the system are head-end, trunk cable, distribution or feeder cable in the neighbourhood, drop cable to customer's home and consumer's terminal equipment (set-top box and TV receiver). The programme signals are gathered (satellite, terrestrial, local studio and character generator) at a central point, i.e., the head-end. The signals are modulated to desired cable channels, channel filtered, combined and amplified before being launched onto the cable. The head-end contains all other signal control equipment such as advertising insertion, scrambling and compression gear. At the input of the head end satellite dishes, microwave dishes, off-air antennas and videotape machines collect the appropriate programming. The head end also houses network management and quality control systems.

From the headend trunk lines of co-axial cable, 1 cm to 3 cm in diameter, reach out into the neighbourhoods or business areas to be served by the cable system. This is the tree trunk of the basic tree-and-branch architecture. Its purpose is to move the signal with minimum amount of degradation. The high loss due to coax cable is compensated by inserted line amplifiers.

The data system in the conventional analogue cable TV system is based on character generator equipment capable of adding alphanumeric or text material either in full-screen mode or as a scrolling line across the bottom of the screen. Such information might include programme promotions, stock market info, news alerts or emergency weather information. Simple low speed data services (such as text for closed captioning for hearing impaired viewers and sub-titles for language translation), one-way data services for personal computers and or cable-exclusive CD quality audio music services are possible. These signals require special equipment (like modem) at the receiving end for their reception. These signals are transmitted in the slots above or below the video channels and do not affect the system's capacity for video channels.

5.3.9.2	HFC Cable systems

Cable system has evolved further with the introduction of fibre and Hybrid Fibre Coaxial cable (HFC). Digital technology has made inroads into the cable domain. Digital video compression digitises and multiplexes 4 to 10 TV channels into the space currently used to transmit one analogue TV channel. Compression technology is being further developed to squeeze even more channels into 6 MHz spectrum space. Fibre and HFC have improved signal quality, increased channel capacity, decreased ongoing maintenance costs and created the infrastructure upon which future interactive or two-way voice, video and data services will be transmitted. Fibre, especially with its low losses, is important in large systems because it can span large distances without signal degradation and without requiring amplifiers. With fibre acting as trunk penetrating deep into the network, coaxial cables provided with optical/RF interfaces could serve the branches and coax cable could serve the subscriber drops.

The head end functions remain the same as in basic all-coax cable systems. Programming is gathered together from satellite, off-air, microwave, videotape and local sources. The analogue and digital signals are converted to optical signals that can be sent down on the fibre cables. Additional equipment are added at the head end to aid data services, for example, modems, file servers and other data links. Consumers also add broadband cable modem to their computers to enable data reception in addition to the TV set top box. A set top box with cable modem capabilities will likely be offered in the future, which would provide entertainment, information, education, neighbourhood specific programming and advertisements. Businesses, Government offices and educational institutions can be cabled with their own dedicated fibres using cable system to transmit internal data, voice and video. The system could also directly link the subscriber to a long distance telephone provider.

5.3.9.2.1 	Channel organisation and topology of HFC cable systems

- 30 channels at 6 MHz each analogue services

- 16 channels, 96 MHz, of open spectrum for non-video digital services

- 20 channels that use standard digital compression ranging from 4:1 to 10:1 to offer another 80-200 SDTV channels offering near video-on-demand service or pay channels.

- Unscrambled channels, especially analogue, not requiring a set-top box (basic service) while digital and data services need a set-top box and a cable modem.

Advanced topology supports linking a regional distribution system called hubs through a fibre backbone to other fibre/coax cable systems in a geographical area. Redundant paths such as ‘dark fibre’ could be established to ensure further reliability. The resources at these locations are file servers, head end modems, satellite downlinks, access point to telephone companies, PC switches, compression, advertising insertion and advanced video programme guides. Regional hubs also allow the transmission of education programming - including class sessions - or government/public affairs programming across city or cable franchise boundaries. Regional hubs are a logical gateway for province wide, state-wide or nationwide network interconnections.

5.3.9.2.2.	Two-way HFC system

Hybrid fibre and coax systems can transmit both analogue and digital signals at the same time over the same plant. Digital technology means that video, voice and data can be sent together, over the same cable, instead of having to be transmitted over independent technologies. This capability creates possibilities for hugely increased number and kinds of television channels as well as two-way communications of telephony and data/computer services. TV channel selection ca be done through local interactivity assisted by electronic programme guides, enabling the user to choose what he wants.

Depending on the services to be offered, file servers and other gear are to be added at the head end and possibly at the neighbourhood optical nodes. The cable system and associated other systems must follow the same protocols to permit easier connection. For example, when a telephone call leaves the cable plant and enters a large PSTN or long distance network, the protocols must match. The technologies existing in external systems such as SONET (Synchronous Optical Network), ATM (Asynchronous Transfer mode), FDDI (Fibre Distributed Data Interface) etc. necessitate installation and integration of hardware and software corresponding to those. Several players existing today offer these systems and services.

5.3.9.3	Interactive two-way TV services

Interactive TV services require digital transmission over a two-way infrastructure, such as a two-way HFC plant. The possibilities of interactivity could be as follows:

- The consumer uses a TV remote control through an Electronic Programme Guide channel, makes a selection and switch immediately to the desired programme. If the programme is a movie he could have full VCR like controls to pause, stop or rewind.

- Ability to view from a different position (related with a sports programme) could be possible using the TV remote by making a selection from several viewing options.

- The viewer uses the remote to order a PPV programme or purchase an item on a home-shopping channel.

- Several people, each at their TV set, could play the same video game competing with each other.

- Video Conferencing could be possible which could be used for business meeting, educational classes and residential picture phones.

An example application is tele-health in which voice and video conferencing facilities over fibre telecommunications infrastructure would provide doctor-to-doctor or doctor-to-patient consultations, transmission and review of medical images, etc. This capability extends the medical resources found in major urban centres to patients and doctors in distant rural areas.

5.3.9.4	High speed data using cable system

High-speed data to homes and businesses are available now on HFC systems. Modems, servers and other large data moving/switching equipment are added to the head end including reverse (upstream) lasers and amplifiers along the cable to provide two-way data communication. The frequency band at the lower end of the cable system pass-band, 5 to 40 MHz is reserved for the reverse channel. At the user end, the drop cable is split to link to a modem/computer as well as to the STB/TV. The residential telephone modems that are available today offer data speeds up to 56 kbit/s, while the cable modem could theoretically move data at speeds of 38 Mbit/s downstream and nearly 10Mbit/s upstream. This would provide capability to transfer complicated graphics and multimedia files in seconds, which would take hours with conventional telephone modem. One more advantage is that it is always available (“always-on”) without going through process of dialling, finding a free phone line and establishing a connection.

The cable data services are deployed as on a shared access network, meaning that multiple users share the spectrum space available on the cable. Downstream data transfer is greater than upstream activity. Significantly, vast bulk of communication in either direction occurs in short bursts of activity. One user takes up a lot of spectrum downloading a file, but when the file download is over the spectrum is open for other users. The packages offered by service providers are unlimited Internet access, electronic mail, local-focused content, exclusive multimedia content, national information services, computer games and distance education. File servers at the head end could give access to CD-ROMs, encyclopaedias and other reference material. Homes, business, schools and libraries are hooked up with broadband cable modems. Other services that are offered are picture phones, environmental telemetry and control, security telemetry and control, home shopping, home banking, interactive games etc. Business users are offered facility to work from home, video conferencing, high speed fax transmission and telephony.

5.3.9.5	Cable Telephony

The two-way HFC cable system, with regulatory permission, can support telephone calls. The cable system needs to add at the head end or on the regional hub telephone switches that connects the cable network to the outside public telephone network. The voice signals travel on dedicated spectrum on the same fibre and coax that carry TV channels and two-way computer data. At the nodes specifically located in apartment buildings or a neighbourhood central point, the distribution cable enters a terminator box that splits the voice signals off onto twisted copper wire, handles connections and switching. A special cable telephone appliance that includes RF modem could be used which can extract voice off the drop cable. A traditional telephone instrument on twisted copper wire could also be used with the facilities mentioned above to separate voice signals from other cable signals. Several US companies offer such services to cable consumers.

Similar facilities could be made available with PCS (Personal Communication Service) with close interaction and collaboration between cable operators and PCS operators. PCS is a form of wireless telephone communication using a small portable phone. It is similar to cellular phone, but has potential to offer lower cost and greater convenience. Adding microcells to transfer a user's call from one cell to another as the user travels, PCS can use cable infrastructure to interconnect cells. Signals from the calling PCS phone travel over the air to the nearest cable node. The signals then travel along the cable to the node closest to the called party and then through the air to his phone. Necessary interfaces must be integrated with the cable plant to enable such communication.

5.3.9.6	Signal scrambling and security

Scrambling and trapping systems are used to ensure that TV households receive only the services they have paid for. Trapping is a simple mechanism in cables delivering analogue TV signals. Traps are located where drop cable taps into the feeder cable. There are two trapping schemes: positive and negative. With positive trapping an interfering carrier is inserted into the selected video channel at the head-end. At the consumer's side a positive trap is installed to remove the inserted interfering signal. A negative trap removes the channel signals from the drop cable to the customer, which he has not opted for. Scrambling is more sophisticated and used in both analogue and digital transmissions. There are two classes of scrambling technologies: RF synchronisation suppression and baseband scrambling systems. With RF scrambling there is a phase-modulated, gated or pulsed synchronisation suppression format and a sine wave synchronisation format. Consumers in the latter two cases use non-addressable converters and descramblers to view their channels; the cable operator must physically change these boxes. Phase modulated RF scrambling is used within addressable cable plants. A computer-generated data signal created at the head end controls addressable converter/descrambler boxes at the consumer site. From the head end, the operator can tell any particular converter box to decode any particular channel. PPV services are supported by addressable technology. Baseband scrambling systems are more secure, and encrypt both audio and video portions of the programme signals. But end-user converter boxes are more complex and are also more expensive.

5.3.9.7	Digital compression in Cable Media

Using digital compression, four to 10 standard television channels can be transmitted in the 6 MHz space previously used by one analogue TV channel. In June 1997, TCI conducted experiments for incorporating technology that allows it to distribute movies at a 24:1 compression ratio using a technology called statistical multiplexing. When cable operators place multiple TV channels into one 6 MHz slot by using digital compression, the result is known as standard definition television (SDTV). This is in comparison to the better picture quality of High Definition Television (HDTV).

Current thinking on cable channel lineups is to allocate the lower end of the downstream cable spectrum to analogue signals, the 50 to 450 MHz (or 550 MHz) band. A basic unscrambled tier of broadcast signals would be at the low end of the spectrum. At higher channel numbers, cable operators would supply trapped pay services or analogue scrambled services. Above 450 or 550 MHz, digital signals would be provided up to the band edge located at 750 MHz or 1 GHz. Thus, from 200 MHz (33 6-MHz channels) to 550 MHz (91 6-MHz channels) will be available. At digital compression rates of 12 movies per 6 MHz, 396 to 1,092 movies may be offered simultaneously.

At the headend, the cable system receives the digital signal using an integrated receiver/ transcoder (IRT). The IRT demodulates the satellite signal and recovers the digital data stream, performs error correction, decodes, decompresses and converts the digital bitstream to the analogue signals that may be displayed on today's TV sets. (Until digital TV sets are the norm, this conversion to analogue is necessary).

5.3.10	Multimedia broadcasting

Multimedia broadcasting or datacasting refers to the use of the existing broadcast infrastructure to transport digital information to a variety of devices (not just PCs). While the existing infrastructure of broadcast radio and television uses analogue transmissions, digital signals can be transmitted on subcarriers or sub-channels. Also, both the radio and television industries have begun a transition to digital transmissions.

Multimedia broadcasting will be developed in three basic dimensions. First, datacasting supports the transport of multiple data types. This means that more than the traditional real-time, linear, and prescheduled forms of audio and video programming will be available. Broadcast programming will become richer and more involving by expanding its creative palette to encompass different data types and leveraging the processing power of intelligent receivers and PCs on the client side. Second, while some of this data will be related to the main channel programming (i.e., conventional radio and television programming), other data wholly unrelated to conventional programming will be transported. And third, broadcast applications will interoperate seamlessly with other non-broadcast client-server applications such as World Wide Web sessions.

The essential characteristics of multimedia broadcasting include:

- digital data stream;

- asynchronous;

- bandwidth asymmetry;

- downstream backbone;

- high speed downstream (up to 20 Mbit/s);

- universal access;

- low cost;

- layered architecture;

- wireless and wired;

- mobile and fixed service; and

- use of existing infrastructure.

As noted earlier, most client-server applications are asymmetric in bandwidth requirements. As such, the publicly switched telephone network, through which most Internet users get connected, is an inferior transport solution. State-of-the-art modems achieve a 56 kbit/s bidirectional performance. For multimedia content, including streaming audio and video, this is not very satisfactory.

Hoping to capitalize on the presumed demand for more bandwidth to the home, both cable and telephone companies actively plotted fibre optic build-outs in their distribution plant. This would have supported broadband capacity to and from the home. Given the market turn away from video-on-demand products (at least for the moment) and towards the Internet, a new economic picture emerges.

There is no particular reason to optimise a network for symmetric performance (as with a fibre optics build-out), when upstream and downstream bandwidth can be allocated more efficiently with existing infrastructures. Indeed, the telephone industry's promising answer to re-purposing much of their existing copper plant is a rejuvenating technology known as Asymmetric Digital Subscriber Line (ADSL) which will offer very high bandwidths at the customer premises. This technology supports a minimum configuration of a downstream data rate of at least 1.544 Mbit/s per second (e.g., equivalent to a T-1 rate), and an upstream data rate of 128 kbit/s. With fibre architecture without amplifiers going deep in the network and reaching Central Offices that are located closer and closer to the subscriber end and using smaller and improved DSLAM technologies, the network providers can offers greater bandwidth shared by less customers as well as more reliable transmission capability.

Television broadcasters could transmit data such as HTML pages, Java applets, and Virtual Reality Modelling Language (VRML), streaming audio and video or binary file transfers in their broadcast signals for capture and display by smart TV receivers, PCs, personal digital assistants, or network computers. Consumers could select from this data stream only that which is interesting to them. Further, they could activate agents or filters to grab and store particular resources of interest to them. This data could be accessed later, at their convenience, rather than having to grab it and display it in real time from the data stream being currently broadcast.

This model so far assumes a one-way data stream. One interesting thing about this scenario is that from a technical viewpoint it is one-way. However, given the choice and control, it appears to be interactive to the client. This adds value. Another interesting thing is that since the telecommunications technology supporting this service is one-way, it can be available to remote and mobile users. There is no need to be tethered to any kind of wire to enjoy the benefits of this ‘push’ implementation of Internet service.

Of course, consumers will want more data than the broadcast station can pre-select for transmission over their facilities. The service model can be extended in a couple of ways. First, using a return link like a telephone (wired or wireless), the consumer can request a specific set of resources to be transmitted. Depending on the file sizes, the client request could be served via phone link or the request could be forwarded to the broadcast server which then loads the appropriate resources in the data broadcast queue for reception by the client.

A second model is that the client could have his or her Web browser connected to an Internet Service Provider (ISP) with the appropriate enabling technology to permit a multimedia connection to the Internet. In this scenario, the user is sitting at home surfing the Web. The person goes to a Web site where there are lots video objects. Until now, the Internet connection has been supported by a dial-up connection. When the person clicks on a 5-minute video news clip, the session software notes that this is a 3-Mbyte file and instead of pushing it through the 56 kbit/s phone connection, shunts it to a local broadcast station which loads it into the data broadcast queue. The large video file arrives at the client workstation in seconds instead of minutes.

5.3.10.1		Specification of a typical DVB Data system

The DVB Data Broadcasting specification is designed to allow operators to download software over satellite, cable or terrestrial links, to deliver Internet services over broadcast channels (using IP tunnelling), to provide various services such as interactive TV. These specifications conform to the ETSI standards.

At an early stage in the proceedings, MPEG-2 DSM-CC (Digital Storage Media - Command and Control) was chosen as the core of the DVB's data broadcasting specification.

The result is a specification based on a series of four profiles. Each profile corresponds to an application area. The four application areas covered by the DVB's Data Broadcasting specification are as follows:

- Data Piping - This is the simple, asynchronous, end-to-end delivery of data through DVB compliant broadcasting networks.

- Data Streaming - This supports data broadcast services that require a streaming-oriented, end-to-end delivery of data in either an asynchronous, synchronous or synchronised way through DVB compliant broadcast networks.

- Multi-protocol Encapsulation - The data broadcast specification profile for multi-protocol encapsulation supports data broadcast services that require the transmission of datagrams of communication protocols via DVB compliant broadcast networks.

- Data Carousels - The data broadcast specification for data carousels supports data broadcast services that require the periodic transmission of data modules through DVB compliant broadcast networks.

5.3.10.2		Information Infrastructure for Digital MM applications

This section is written with the public telecommunication network in mind, although there are excursions along the way to look at other networks, such as the Internet and broadcasting networks. Digitisation is driving the convergence of telecommunication, computing and television. Ultimately, it may be possible to provide all three services over a single network, but there will inevitably be a lengthy intermediate period during which hybrid solutions are adopted to provide immediate service. In the longer term, the best candidate for the information superhighway is fibre optic cable. One of the determining factors will be how quickly terminal devices evolve to utilize the huge increase in information flowing into the home and the office. The pace of technological changes seems to get faster, and consequently it remains difficult to forecast technological winners and losers. Nevertheless, there are some trends that cross-over between the three sectors.

Communication networks have traditionally been designed to carry different information types separately. Telephone networks were designed for voice, telex networks for text, local area networks (LANs) for data, broadcast networks for video, and so on. But this picture is changing. Most types of information can now be digitised: that is coded in the binary language of ones and zeros. The essence of a multimedia network is that it is no longer possible, nor necessary, to disentangle the components of a digital bit stream until it reaches the device for which it is intended, be that a telephone, a computer or a television. The function of future networks will be defined less by the information that travels over them than by what kind of terminals receive that information.

The trend towards digitisation is affecting all three segments of the information industry - computing, telecommunications, and broadcasting.

5.3.11		Future developments

5.3.11.1		Terminal equipment

There are essentially three user devices for accessing telephone, data and television broadcast networks: the telephone, the personal computer and the television. Other devices can be used such as fax machines for the telephone network or videotex terminals for data over the broadcast network but they are not general-purpose machines. How will technological convergence affect the devices needed to access future networks? Will there be one all encompassing super telephone/PC/television or tele-computer, or will they remain separate with different functionalities?

While televisions, computers and telephones are likely to remain separate for many years, the chances are that they might be found next to, or underneath, black boxes. These boxes, which will fit between a network connection (e.g., a telephone line, a cable television connection or a satellite dish) and the user's access device, will contain sophisticated software and semiconductor chips. The purposes of the boxes will be manifold: providing conditional access to pay-TV channels, sending secure messages for financial transactions, decoding digital signals for analogue receivers, and much more. The trick will lie in developing a single box that can carry out as many of these functions as possible, while still remaining small and easy-to-use.

5.3.11.2		ATM, a (profitable) platform for multimedia?

It should be abundantly clear from this report that no one really knows which multimedia services will prove popular and which will fail. That causes a problem for the engineers who are trying to develop future generation networks. The key design criteria must be flexibility as well as performance.

While X.25 packet switching provides good data communications at speeds up to 56/64 kbit/s (basic rate ISDN), and frame relay takes over until 1.5/2.0 Mbit/s (primary rate ISDN), neither are adequate for speeds above primary rate. Fortunately a solution is at hand. Asynchronous transfer mode (ATM) is the standard adopted by the ITU- Telecommunication Standardization Sector (ITU-T, formerly the CCITT) for handling broadband services (B-ISDN). This standard has subsequently been taken on board by most of the telecommunication community. The technology employs cells of fixed length (48 bytes payload plus 5 bytes header), enabling operators to offer a bandwidth-on-demand service at speeds up to 622 Mbit/s (see for instance, ITU-T Recommendation I.361). The facility offered may be at a continuous bit-rate, comparable with a leased line, or variable bit-rate, which is useful for bursty type transmission, such as LAN interconnect. Thus, service providers can buy as much or as little bandwidth as they need.

For Post and Telecommunications Operators, ATM offers a powerful and flexible tool. In the words of one commentator, it offers operators the ability to fly higher, use less fuel, and land on a shorter runway. ATM is already being used internally within Post and Telecommunications Operator networks. The key problem will be pricing. But ATM will use the same technology, and probably the same network, for delivering a similar type of service. A Post and Telecommunications Operator could probably charge a business customer 100$US per hour for a videoconference, but certainly couldn't charge the same for a video-on-demand service, even though the bandwidth requirements might be identical.  The less stringent requirement for latency in the later case would help in rationalizing a lower cost per hour. Some pragmatic decision is the need of the hour to contribute in the area of growing multimedia market.

5.3.11.3		Applications of Multimedia services

In many developing countries a great majority of the population suffers from a lack of access to efficient health services, education, basic information and communications means, as well as TV programmes.

Most of these needs can be solved by basic (or better) telecommunications infrastructures, which enable rural or mountainous areas to communicate with the rest of the country, or with other countries.

A step further is the utilization of telecommunication means for tele-health or tele-learning (for primary schools, high schools and Universities), access to public utility information (e.g. weather forecast, risks of inundation or earth tremor, etc.).

Multimedia applications and access to Internet may also be considered as necessary in developing countries (e.g. access by a nurse or a doctor in an isolated village to a medical web site to get up-to-date information about a disease, new medicines or to help them in a diagnosis).

Access to the TV programmes by the whole population must also be considered as a key factor for the development of the country, and the possibility of receiving added value services at low cost via digital TV channels must also be offered to developing countries in the near future.

5.3.11.4		The Information super-highway and the Information Society

The media future is a world where everyone, everywhere, has easy and ready access to anything in entertainment, education and information. If we examine the potential lanes on the future Information Superhighway, we see three lanes. All of them can offer multimedia and interactivity - to varying degrees.

5.3.11.5		Digital Broadcasting

The first of the lane is digital broadcasting. Through all the turbulence ahead, major elements of our broadcast services today will not change because; there will always be a demand for them. Just because there is interactive multimedia, people will still want to watch sports or new movies, from a source that they know to be reliable and up-to-date. There will always be programming that is of interest to the many at the same time. There will always be need for 'broadcasting'. The branding that known and respected broadcasters provide will always be an asset.

Digital broadcasting will progressively engulf all broadcasting systems, with the application of DAB, DVB-S, DVB-C and DVB-T in Europe, the ATSC and ISDB systems developed in the US and Japan. There would be a need for a common worldwide standard for digital broadcasting throughout the world, though it would take considerable effort to realise it.

5.3.11.6		Full Service Networks (FSN)

The second lane on the Information Superhighway may be called ‘Full Service networks'. These are services supplied by a dedicated server, and often are locally provided. Into this comes the Video-on-Demand services that have been on trial in Europe and elsewhere. They need to deliver something approximating broadcast television picture quality, and thus about 1.5 Mbit/s or more, delivered on demand into the home.

There are different sets of hardware, and different specifications, being used for the field trials. A set of protocols for such systems has been drawn together in groups such as the DAVIC consortium. This concept could provide the backbone of the world's Information Superhighway. It could, but it probably won't, at least for some years.

5.3.11.7		Internet

There is a third lane on the Information Superhighway. This is the Internet and is its spectacularly successful child, the World Wide Web. By the year 2000, there was one billion web pages to access. Despite being arguably something of a technical ragbag, the web has won through because the infrastructure is already there, and it offers a vast choice of content. Any one anywhere can set himself or herself as either a content user or supplier. There are few, if any, barriers to entry, or use. It has won because of its democracy and its low cost.

Broadcasters must be a part of the action on the Web. They can play the same role that they play in broadcasting: the public guardians of news, culture, and national identity, and act as the stimulator of creativity. In a world of a million sources of information, branding and trust will be essential determinants of the services that people use. Broadcasters need to leverage their status in broadcast delivery into the world of the Web.

The range of Web browser plug-ins, and applications, continues to grow dramatically. It is as if a giant providential hand were driving the web forward. It appears to be moving in the direction of broadcasting. In the last decade, we saw the birth of ‘streaming' systems designed to ensure that audio or video arrives at the right pace. We have seen the rise of ‘Push Media' and ‘Pull Media'. These supply a diet of web pages and services to suit your taste. They can be there, always on, in the foreground or in the background. The first Push Media software has so far delivered over 1.7 million sets - this easily outshines the growth-rate of any broadcast system of recent years. There is no argument that Webcasting will be a major force in media delivery.

5.3.11.8	 	The Broadcast/Web Combination - Hybrid Delivery

The Web is likely to become a flexible mixture of ‘unicast' services for the individual, and ‘multicast' services, where a community of users wants the same information at the same time.

In parallel with the development in the Web of multicasting, there is almost a reverse move in the digital broadcasting environment. Several manufacturers believe that combining a Web channel with a broadcast channel will be an important way to speed up Web delivery. They see the combination of the Web and broadcasting, which might be called ‘hybrid delivery' to be the next giant leap forward in Web delivery. Both a telephone channel and a broadcast path together combine to give fast access to Web pages, particularly those that are commonly required.

If we follow the current trajectories of the Web and digital broadcasting we arrive at a world where all media can be used with flexibility for either multicast or unicasting. This seems to be as close as we can get today to a vision of the likely shape of the information superhighway.

5.3.11.9		Issues for Digital Television Broadcasting

In television broadcasting, we can look forward to greater computation capacity being available in STUs (Set Top Units) and receivers. All the features of a PC, such as downloadable or CD-loadable applications will become possible with the home receiver. Electronic Programme Guides will be the first ‘Application' for digital television receivers and they will be downloadable, or available on cheap local storage devices like CD-ROM or DVD.

A critical element to make all this possible is the operating system used in the television receiver, or the programming language that needs to be used by downloadable applications. It is clear that a common worldwide "Application Programming Interface (API)", which is itself capable of upgrading and evolution, would be a major asset for the broadcasting industry and the public. The search for this open API, or some means of allowing different APIs to be addressed by the same software, is a major technical challenge for the broadcasting community now.  Work is being done on such harmonization of the interpretive (XML) and executive (JAVA) aspects of the receiver terminal in Working Party 6M of the ITU-R.

There are several proprietary APIs already in use. DAVIC developed an API system that includes MHEG and Java elements. The harmonisation of the API situation, at least to an environment, in which service providers can live, is very important today.

The current debate about the form and nature of the Application Programming Interface for digital receivers is critically important, and is, in reality nothing more than a manifestation of a digital convergence, towards a time when Multimedia Home Terminal will receive and interpret all multimedia delivery means.

Engineers and ideas of vision will be needed to reach the goal.

5.4. 	List of Abbreviations

8-PSK	Eight-phase phase shift keying

AAC	Advanced Audio Coding, ISO Standard under MPEG-4 for non-backward multi-channel digital sound coding standard

AC-3	Multichannel digital audio coding developed by Dolby in the USA

ADSL	Asymmetric Digital Subscriber Line

AMDS	AM Data System (Recommendation ITU-R BS 706)

API	Application Programming Interface

ATM	Asynchronous transfer mode

ATSC	Advanced Television Systems Committee (USA)

B-ISDN	Broadband ISDN

C/N	Carrier to noise ratio (usually expressed in dB)

CD-quality	Audio quality equivalent to that recorded on CD’s (16 bit per channel, right and left at 44.1 KHz sampling rate

CLI	Caller Line Identification

CNN	Cable News Network based in Atlanta, Georgia, USA

COFDM	Coded Orthogonal Frequency Division Multiplexing

CRC	Cyclic Redundancy Check

DAB	Digital Audio Broadcasting

DARC	Data Radio Channel, High-speed data system developed by NHK in Japan

DAVIC	Digital Audio-Visual Council

DCT	Discreet Cosine Transform

DQPSK	Differential Quaternary Phase Shift Keying

DSB	Dual Sideband

DSM-CC	Digital Storage Media - Command and Control

DTH	Direct-to-home

DVB	Digital Vide Broadcasting

DVD	Digital Versatile Disk

ETSI	European Telecommunications Standards Institute

FDDI	Fibre Distributed Data Interface

FDMA	Frequency Division Multiple Access

FFT	Fast Fourier Transform

FM	Frequency Modulation

FM-HSS	FM High-speed subcarrier

FM-IBOC	FM In-band On-channel

FSN	Full Service Network

GPS	Global Positioning System

GSM	Global System for Mobile

HDTV	High-definition Television

HF	High Frequency band (3 MHz - 30 MHz)

HFC	Hybrid Fibre Coaxial cable

HSDS	High Speed Data Systems, High-speed data system developed by Seiko in Japan

HTML	Hypertext Markup Language

IEC	International Electrotechnical Commission

IRD	Integrated Receiver Decoder

IRT	Integrated receiver/ transcoder

ISDN	integrated services digital network

ISI	Inter-symbol interference

ISO	International Standards Organization

ISP	Internet Service Provider

ITU	International Telecommunication Union

ITU-R	Radiocommunication Sector of the ITU

ITU-T	Telecommunication Standardization Sector of the ITU

JAVA	Platform independent programming language developed by SUN Corporation in the USA

LAN	Local area network

LF	Low Frequency band (30 kHz - 300 kHz)

LMDS	Local Multi-point Distribution Service

LMSK	Level Controlled Minimum Shift Keying

LOS	Line of sight

MF	Medium Frequency band (300 kHz - 3 MHz)

MHEG	Multimedia and Hypermedia information coding Expert Group

MMDS	Multichannel Multi-point Distribution System

Mono	Monophonic

MOT	Multimedia Object Transfer Protocol

MPEG	Moving Picture Experts Group

MVDS	Multi-point Video Distribution Service

NTSC	National Television Systems Committee (USA) TV standard

PAL	Phase-alternate Line TV standard

PCS	Personal Communication Service

PPV	Pay-per-view

PTO	Post and Telecommunications Operator

QAM	Quadrature Amplitude Modulation

RBDS	Radio Broadcast Data Service (USA and Canada)

RDI	Receiver Data Interface

RDS	Radio data System (EBU-3244)

SCA	Subsidiary Channel Authorisation

SDTV	Standard Definition Television

SECAM	Sequentiel à mémoire TV standard

SFN	Single Frequency Network

SONET	Synchronous Optical Network

STB	Set-top box

STIC	Subcarrier Transmission Information Channel developed by MITRE Corporation in the USA

STU	Set Top Unit

TDM	Time Division Multiplex

VBI	Vertical Blanking Interval

VCR	Video Cassette Recorder

VRML	Virtual Reality Modelling Language

VSAT	Very Small Aperture Antenna

XML	Extensible Markup Language

5.5. 	References:

- “Data Broadcasting- Leveraging Spectrum Assets into New Business Expansion Opportunities” Marcia L. De Sonne, Director, NAB

- “More ways to Access the Web are coming to Europe’s Homes” – Valerie Thompson, BYTE, March 1998

- Multimedia Broadcasting and the Internet – Richard V. Ducey, Research and Information Group, NAB

- “A Framework Scalable Dissemination Systems” – Michael Franklin, Univ. of Maryland, Stanley Zdonik, Brown University

- Broadcast Engineering Society, India, Quarterly Journal

- Radio World Bi-montly Publication

- ETSI Documents on Digital Video Broadcasting, Digital Audio Broadcasting

- A Framework scalable dissemination based Systems - Micheal Franklin, Univ. of Maryland; Stanley Zdonik, Brown University

6.	Related Worldwide Web sites

www.nab.org

www.atsc.org

www.fcc.gov

www.rds.org.uk

www.intercast.org

www.wavephore.com

www.direcpc.com

www.databroadcast.com

www.sectra.se

www.itis.fr

www.bayern.de

ANNEX A:	  Multimedia broadcasting -- some examples

A.1.	Intercast Industry Group

The Intercast Industry Group is a not-for-profit alliance of computer, television, cable, and online companies creating a "new" medium, which combines television and the Internet on the PC. Their charter is to "develop technology, PC platforms, software applications, and content for broad market deployment." Their goal is to "create an industry accepted, open medium which will spawn industry-wide implementations." According to Intercast, their technology is a transport standard for digital data associated with TV signals.

Intercast technology (i.e., Internet + Broadcast = Intercast) is a means for transmitting HTML pages over broadcast television signals. The HTML pages can either be related to the traditional television programming or not. Links embedded in the HTML pages can be supported if the intercast-equipped PC also has a modem link for full interactivity.

For example, one implementation of this technology is that while a program or commercial is being broadcast, related HTML pages could be simultaneously broadcast to PCs. These pages could contain more details about the program, related information, scheduling information, plot summaries, or promotional material. For commercials, maps showing local dealers, full pricing information, current inventory stocks, or additional benefits can be presented in HTML pages.

A.2	WavePhore multimedia datacasting

WavePhore is a public company which uses different parts of the analogue television signal to net a 384 kbit/s throughput. The primary user platform is the PC. Its WavePhore Networks venture is part of the Intercast alliance. In addition to analogue TV, WavePhore can also use other transport infrastructures such as FM radio, satellite, microwave, or cable.

A.3	DirecPC

DirecPC, is a PC version of the direct broadcast satellite service from GM's Hughes Networks Systems division. It is a data service using digital satellite signals for transport from the server to the client at up to 400 kbit/s. The client must have a DBS antenna (50 cm) installed and the necessary reception electronics, subscription service, and PC. Back channel or upstream capacity can be enabled via a dial-up connection. The service is packaged and marketed as a cable version of the Internet with a basic subscription, which includes news, financial, sports, and other information in multimedia formats. DirecPC has its own implementation of a datacast technology.

A.4	Datacast partners

According to estimates by former NAB executive, John D. Abel, Datacast Partners, a television station can equip itself for $50,000 to upgrade its facilities for datacasting. Datacast is planning a system test using currently deployed (i.e., analogue) television technology to provide a 700 kbit/s service. Datacast will use Digideck's technology, which translates radio frequency (RF) signals to bitstream data for plug-in PC cards. Versions of the PC cards could also be incorporated into TV sets.

Abel's view of an appropriate match between broadcasting's transport strengths and content affinity is that a database that is relatively large and constantly changing is well suited to a datacast application. He provides the Yellow Pages as an example of a database that could be updated daily and rebroadcast for local storage. In addition to database services, news and entertainment information can be datacast using this technology. This service plans to derive revenues from advertising and data management fees.

________________

Dynamic range: Difference in dB between the maximum audio signal level and the level of noise in the audio channel.