| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION DEVELOPMENT BUREAU ITU-D STUDY GROUPS | Document 2/202(Rev.1) 3 April 2001 Original: English |
| FOURTH MEETING OF STUDY GROUP 1: CARACAS (VENEZUELA), 3 - 7 SEPTEMBER 2001 FOURTH MEETING OF STUDY GROUP 2: CARACAS (VENEZUELA), 10 - 14 SEPTEMBER 2001 |  |

FOR ACTION

Question 9/2: 	Identify study group Questions in the ITU-T and ITU-R Sectors which are of particular interest to developing countries and systematically, by way of annual progress reports, inform them of the progress of work on the Questions to facilitate their contributions to the work on those Questions as well as, ultimately, to benefit from their outputs in a timely manner

STUDY GROUP 2

SOURCE:	RAPPORTEUR’S GROUP ON QUESTION 9/2

TITLE:	HIGH ALTITUDE PLATFORM STATION: AN OPPORTUNITY TO CLOSE THE INFORMATION GAP

________

Action:

The meeting is requested to endorse this draft report for publication.

Abstract:

This report provides an update on technical and regulatory developments with respect to High Altitude Platform Stations (HAPS).

_____

1.	INTRODUCTION

High Altitude Platform Station, or HAPS, is a new technology that can revolutionize the wireless industry.  Thanks to the recent advances in power, material, propulsion, and telecommunications technologies, platforms can now be kept stationary in the upper atmosphere for a very long duration and serve as high-speed Internet gateways and relays for portable multimedia devices.  Such stratospheric platforms have tremendous economic and technology advantages as compared with either space or ground-based networks.

2.	STRATOSPHERIC INFRASTRUCTURE

Currently there are several major organizations that are devoted to the development and commercialization of high altitude platform based systems for telecommunications as well as for environmental monitoring and remote sensing. Chief among them are Sky Station International, based in Washington, DC, USA, the Japanese MPT/STA (Ministry of Posts and Telecommunications/Science and Technology Agency) R&D Project, and ESTEC (European Space Agency) HALE (High Altitude Long Endurance) Aerostatic Platform Project. Sky Station International will commence commercial deployment in the year 2002. The Sky Station platforms will consist of an extremely strong, lightweight, multi-layer skin containing buoyant helium, a station keeping system consisting of GPS and an advanced propulsion system, a telecommunications payload, thin film amorphous silicon solar panels for daytime power, and regenerative fuel cells for night-time power. Baseline platform characteristics of an example system are provided in Table 1.

Although stratospheric platforms are an old idea, they only recently have become practical through the development of several enabling technologies.  The enabling technologies are high efficiency solar cells and fuel cells that are both lightweight and durable, high strength ultrathin fiber and helium impermeable seal, thermal and pressure control/management techniques, as well as advanced phased antenna array and MMIC (microwave monolithic integrated circuit) technologies.

Table 1: Stratospheric platform parameters (Example system)

Operating Altitude		20-22 Kilometers

Lift at Altitude			0.062 kg/m3

Hull Volume			371,000 m3

Gross Lift at Altitude	23 metric tons

Hull Area				30,000 m2

Envelope Weight		7,500 kg

Payload					1,200 kg

Dimensions				220 m by 50 m

Speed					200 km/hour

Operating Frequencies	47.2-47.5 GHz /47.9-48.2 GHz

27.5-28.35 GHz/31.0-31.3 GHz

1885-1980 MHz/2110-2160 MHz

A global network of domestically operated stratospheric platforms is deployed in a population-based heterogeneous manner instead of the orbital dynamics-based homogeneous spacing of low-earth orbital satellites.  Gateway earth stations connect each stratospheric platform to the public switched telephone network, high-speed frame relay and ATM (asynchronous transfer mode) network, as well as the Internet.  Portable and fixed communication devices are used to send and receive digital information via the stratospheric platforms and gateway earth station.  Channel speeds range up to 2 Mbps for a  3rd generation portable user terminal, 45 Mbps for a transportable 23 dBi antenna, and up to OC3 (155 Mbps) for a fixed steerable high gain antenna.

From the stratosphere, communication links of high-angles can be established across large land areas.  For example, at 23 km altitude, links with 30° angles of elevation extend out to a radial distance of 40 km from the center of coverage.  Throughout any urban area centered on a stratospheric platform, the angles of elevation exceed 50°. The footprint of each stratospheric platform is approximately 1000 km in diameter to the coverage horizon.

Considerable effort has gone into ensuring the safety of stratospheric platforms.  They are deployed from cleared airspace like the hundreds of high altitude science research and weather balloons that are launched worldwide each day and they reach their stratospheric altitude within only a few hours.  They then move under their own power through the stratosphere to fixed locations above metropolitan areas at altitudes high above all commercial and most military aircraft.  Redundant safety systems prevent deflation and system failures and provide advance warning so that the platforms can be maneuvered to service centers or unpopulated areas for recovery and repair.  Each platform will be subject to inspection, approval and regulation by aviation authorities such as the Federal Aviation Administration, ICAO, and national regulators in each administration.

3.	STRATOSPHERIC TELECOMMUNICATIONS APPLICATIONS

Stratospheric platforms are designed with telecommunications technology capable of providing full duplex digital channels of from 14.4 Kbps to 155 Mbps.  At these speeds compressed voice and web TV applications can be supported at the low-end, and high-speed OC-3 LAN, MAN & WAN channels can be implemented at the high end.  Generally, stratospheric platforms will enable a complete blending of digital telephony, computer and video information to be delivered to hand-held multimedia terminals, wireless local loop terminals, and fixed wireless networks.  A list of stratospheric services is provided in Table 2.

Table 2: Stratospheric services

Digital Telephony, Fax and E-mail	14.4 Kbps

Full Motion Videophone Service	64 – 384 Kbps

High Speed Web Surfing, Web TV and File Transfers	128 Kbps - 45 Mbps

OC 3 LANs, MANs, & WANs	155 Mbps

Stratospheric transmissions can connect mobile or pocket telephones to desktop phones via millimeter wave, submillimeter wave transmission, or conventional microwave bands.  Similarly, laptop or notebook computers can transmit or receive information directly via a stratospheric platform (FIGURE 1), or indirectly through a Bluetooth transcoding relay station. It can also connect through to a gateway ground station and the public switched telephone network (PSTN) to either cellular or desktop phones or databases such as the World Wide Web anywhere in the world.

Figure 1. HAPS-based direct addressing high-speed wireless data and Internet Access.

The stratospheric platform system can be used to provide backhaul links to thousands of Bluetooth-based nanobase-stations, which serve as the hubs for inexpensive DECT (Digital Enhanced Cordless Terminal) phones over a short range of about 100 meters. The recurrence cost of such telephony service is around $30 per subscriber once it has been fully subscribed. In this way the Stratospheric Telecommunication System (STS) can become a developing nation’s lowest-cost high-speed digital backbone.

The figure below (FIGURE 2) illustrates the concept. Note that the stratospheric platform can provide more than 700 beams, each of which in turn, can support more than 100 Bluetooth base-stations. Since each base-station can support up to 200 DECT subscribers, potentially each platform can provide basic semi-mobile telephony service to 14 million subscribers. In addition, each stratospheric network can also provide high-speed Internet service either through the inherent packet-data capabilities of the Bluetooth protocol, or through a stratospheric access device directly. The phase 1 Bluetooth protocol can provide up to 1 Mbps packet data access. Later generation Bluetooth will achieve 11 Mbps packet data capabilities. Through a direct stratospheric access device, the maximum throughput can reach 45 Mbps, equivalent to that of a T-3.

Figure 2. HAPS-based Wireless Local Loop using Bluetooth nano-basestations to provide DECT telephony and high-speed Internet Access.

One of the most common multimedia applications of HAPS technology will be the provision of video telephony between two service subscribers in the same coverage areas.  This can be accomplished through either a stratospheric access device, a 3rd generation phone service such as UMTS or WCDMA, or a Bluetooth enabled PC or PDA (personal digital assistant). Here the HAPS overcomes the bandwidth limitations of traditional infrastructure that have impeded videophone to date.  Alternatively, two HAPS users can communicate with each other in different coverage areas via gateway groundstations and the PSTN or directly through a high-speed DWDM (dense wavelength division multiplexing) link between two stratospheric platforms.

It is also possible for communications to be maintained between a HAPS user and a HAPS non-user.  Here the multi-protocol gateway ground station and PSTN/Internet serve to provide protocol compatibility.  If the PSTN lacks adequate bandwidth or ATM switching capability, gateway groundstations or direct DWDM laser links can be used with different antennas inter-connecting multiple stratospheric platforms.  This will enable stratospheric communicators in one coverage area to enjoy direct communications with stratospheric communicators in adjacent coverage areas, although a “last-leg” of PSTN must be used to connect the non-HAPS subscriber.  For example, if agreed between neighboring Administrations, adjacent coverage areas in several countries can be interconnected by stratospheric platforms alone for long distance links, including transoceanic platforms interconnected by satellite or inter-platform laser transmission.

4.	STRATOSPHERIC REQUIREMENTS

Morgan Stanley Dean Witter Research has projected that by the end of year 2000, more than 155 million users will go online. By 2005 it is expected that there will be 300 million people using Internet worldwide as a ubiquitous secure multimedia communications medium connecting businesses and homes for business, entertainment and educational applications. A large majority of these users can be expected to prefer high-speed wireless connections over low speed wired connections.

It is widely recognized that more than half the people in the world have never placed a phone call.  This stunning demonstration of the information gap between developed and developing countries requires a cost-effective solution that brings the developing countries high-speed channels (to close the gap) at affordable prices.  Simply providing phone circuits to developing countries, while developed countries go broadband, will not close the gap.  And the information gap cannot be closed at dollar per minute tariffs or with thousand dollar terminals.  What is needed are flexible broadband channels, accessible via a low cost DECT-based telephone or a Bluetooth enabled intelligent device such as a low cost PC, and delivered to developing country markets for under $10 per month.  Note that it is projected by year 2002 the cost of a Bluetooth access device will drop below $10 on average. These needs can be fully addressed with the unique stratospheric architectural advantages of large coverage area, close proximity to earth and low-cost technology.

5.	THE STRATOSPHERE’S NATURAL ADVANTAGES

Stratospheric platforms have intrinsic advantages that enable HAPS to provide communications capacity to metropolitan areas at a low infrastructure cost per subscriber.  A single platform can provide a metropolitan area with full duplex broadband service to more than one million subscribers.  If the frequencies are divided into narrowband channels, a platform can provide basic telephone service to a substantially larger number of subscribers.

For example, a HAPS platform at 21 kilometers altitude using just 100 MHz of bandwidth in each direction in the 47.2-47.5 and 47.9-48.2 GHz, can generate 700 spot beams over an 80 kilometer diameter coverage area with a minimum elevation angle of 150.  Assuming a frequency reuse factor of 7, the total metropolitan capacity of the system is 7.68 Gbps which can be directed to provide high speed services to high density populations at a low cost, ideal for the developed and the developing world.

In regions that are susceptible to heavy rainfalls, the provisional Ka-band designation of 850 MHz for downlink and up to 300 MHz for uplink will provide much higher availability figure for critical telecommunications functions. Using a direct radiating large aperture phased array antenna onboard HAPS, up to 700 spot beams can be generated to cover a radius of 50 km and beyond, with a minimum elevation angle of approximately 230. Lower availability coverage is also possible beyond the 50-km radius, as long as there is line-of-sight, for up to 500 km.  The 850 MHz downlink bandwidth and 300 MHz uplink bandwidth can be partitioned into 7 bands each to produce a 7-cell frequency reuse pattern. This gives rise to a downlink band of 110 MHz per spot beam, with a capacity of 110 Mbps per beam. The uplink bands are further subdivided into 35 frequency subbands with 1 MHz bandwidth each. Using 16-QAM (quadrature amplitude modulation) for the uplink with 2.5 bits/s/Hz spectral efficiency (with FEC, or forward error correction), and QPSK with 1 bits/s/Hz spectral efficiency, each HAPS broadband access device can support a maximum downlink speed of 110 Mbps and a maximum uplink speed of 2.5 Mbps.  Higher uplink speed is possible with channel aggregation for up to 87.5 Mbps maximum uplink speed.

Since the Ka-band WAD (wireless access device) for HAPS initially will be expensive, the aforementioned Bluetooth nano-basestations can be integrated into HAPS WAD to provide ultra-low-cost broadband access. Each 100 mW Bluetooth basestation can support up to 200 DECT subscribers for circuit-switched voice, using inexpensive DECT phones. Each spot beam can link to about 100 such Bluetooth basestations. Hence for each HAPS, approximately 14 million subscribers can be supported. DECT phone provides limited mobility within its 100-meter radius (outdoors). The slow reconnection time when the mobile station (DECT phone) crosses from one Bluetooth coverage cell to another one essentially prevents its use within a fast moving vehicle. So the WAD-Bluetooth network does not challenge 2nd Generation (2G)  or 3rd Generation (3G) cellular systems.

The additional benefit of the WAD-Bluetooth based access network is that any Bluetooth-enabled device such as a PC, a cell phone, or a PDA device, can be linked to the Bluetooth basestation to provide high-speed packet data at a maximum speed of about 1Mbps.

The benefit of using HAPS to deploy 3G cellular network is also readily apparent. Due to its physical dimension, HAPS can house a 15 m by 15 m phased array antenna with tens of thousands of direct radiating elements to provide spot beams as small as 300 meters within a radius of 20 km from the center of coverage.

Instead of the conventional waveguide structure that is too bulky and heavy for such array application, a DM (directly modulated) RF/fiber feed structure is used to feed and steer multiple beams simultaneously. DM RF/fiber uses the RF signal to directly modulate the output of a laser diode to convert the RF signal into an (analogue) optical signal to be transmitted over a single-mode fiber. Furthermore, multiple RF signals can be transmitted over a single fiber using DWDM technique to further reduce the weight and cost of such feed structure. The extremely low loss characteristics of the optical fiber, as well as the ready availability of the low cost optical delay lines and MEMS (micro-electric-mechanical system)-based all optical switches makes it possible to provide coarse-steering of hundreds of spot beams simultaneously within a millisecond using the optical delay-line matrix switching technique.

Additional fine beam shaping and forming can be accomplished using digital beamformers that rely on fast DSP (digital signal processing) technique to sculpt the beams. The main benefit of such a multi-beam payload for the 3G system is that a single HAPS can provide the equivalent of from 700 – 1000 cellular. In addition, the steered beams can be used to reallocate radio resources dynamically to release radio traffic congestion and prevent traffic hotspots.

Since HAPS is like the tallest tower, it also has a much better look angle for most users, which translates into higher average link quality.

In a TDMA-based 3 G system such as UWC-136, such a strategy will produce better link quality but will not substantially increase the capacity of the existing network, unless the frequency plan is revised to increase the network capacity. This is possible since HAPS can help to reduce substantially the inter-cellular interference. It can also extend the coverage to regions previously not covered by the terrestrial network. With a CDMA-based 3G system, such as DS-CDMA or MC-CDMA, the link quality can be further improved through a soft-handover procedure between HAPS and one or more terrestrial basestation(s). The big reduction of the same-cell and adjacent cell interference afforded by HAPS also translates immediately into a direct increase in net capacity since the capacity of the CDMA network is largely a function of the interference level.

6.	STRATOSPHERIC IMPLEMENTATION SCHEDULE

The Sky Station International will begin deployment starting in 2002.  At a deployment rate of one platform per week, 90 percent of the world’s population could be covered by 2008.  Most major cities would be covered by 2004.  It is hard to imagine a more efficient means of achieving universal broadband service and thus closing the information gap.  Estimates of platform and payload costs from international manufacturers are less than half of satellite system costs, due to the avoidance of rocket launch costs, resulting in a capital cost per subscriber of not more than $20 for each 32 kb/s DECT channels.

Global rollout of HAPS multimedia services can proceed quickly by focusing on new urban markets.  Since the stratospheric platform footprint is as large as 1,000-km diameter, each urban market covered also brings multimedia service to a vast rural region as well.  For developing countries the STS at 47 GHz and at 27-31 GHz offers an unprecedented opportunity to leapfrog into broadband multimedia parity with the most economically developed states. For those developing countries wishing for an early deployment of the IMT-2000 (3rd generation mobile) service, a single HAPS platform can support the same number of users as 700 – 1,000 sectored radio towers. And since the HAPS system uses the same IMT-2000 handsets without modification, the economy of scale is at work here on the terminal side as well.

As early as 2004, every country in the world can expect to have at least partial coverage with stratospheric multimedia service.  Countries can then rollout stratospheric platforms to smaller markets within their borders, as they deem most advisable.  With the stratospheric telecommunications service, access to full multimedia information will be a birthright of peoples everywhere, from Africa to the Americas and from Asia to Europe.

7.	STRATOSPHERIC REGULATORY ISSUES

From a regulatory perspective, the ITU and the domestic telecommunications authorities that have considered the matter have concluded that stratospheric services are best defined as a high density fixed service using stations located in the stratosphere.  The service is high density because of the extraordinarily high number of communication circuits it can provide in a rather small, urban area.  The technology creates a fixed service because most user terminals will be accessed through a fixed antenna.  Another reason the HAPS has been considered a fixed service is because the platforms do not meet the definition of a space service and therefore must be a terrestrial service.

Each stratospheric system will operate within national airspace, not transnational outer space and, therefore, will be regulated as a domestic service by the country it serves.  This will enable domestic authorities to regulate the frequency use and the system technology.  However, in order for HAPS to be economically feasible in global application, stratospheric systems also require an international designation to enable worldwide compatibility of components and systems as well as investor confidence necessary to rapidly build-out the service.

On June 2, 2000, the World Radiocommunications Conference granted the extension of Resolution 122, as well as the approval of two new resolutions.  Resolution 122 was originally approved at the WRC-97 conference to designate the radio frequency bands 47.2-47.5 GHz and 47.9-48.2 GHz for use by high altitude platform stations (“HAPS”), thus establishing regulatory procedures to enable the filing by countries wishing to deploy HAPS-based systems, such as Sky Station’s.  The extension of Resolution 122 includes the provisional approval of the use of the two Ka bands 27.5-28.35 GHz and 31.0-31.3 GHz for countries in the Asian Pacific region where heavy rain can sometimes cause service outages for links using the original HAPS bands in the 47/48 GHz range.

The first new resolution adopted by WRC-2000 permits the use of HAPS as an optional platform for the deployment of terrestrial IMT-2000 (3rd generation cellular) services. The resolution allocated the IMT-2000 bands in the 1885-1980 MHz (in regions 1 and 3, 2010-2025 MHz is also included), and 2110 –2160 MHz (2110-2170 MHz in regions 1 and 3) ranges nonexclusively for use by HAPS.  The second new resolution calls for the study of the feasibility of use of HAPS in terrestrial frequency bands above 3 GHz. This opens up the way for the International Telecommunications Union (“ITU”) to study the issues of sharing and interference to existing services from HAPS as well as possible future allocations of other frequency bands for HAPS on a co-primary basis.

Finally, HAPS obtained several agenda items for the next World Radiocommunications Conference (“WRC–03”) to examine additional services and frequencies for use by HAPS.

Because the 47/48 GHz bands are strongly susceptible to heavy rain attenuation, they may not be suitable for regions or countries with heavy rainfalls. Thus it would be desirable to seek other bands in the Fixed Service that are less attenuated by rain. The provisional permission granted by WRC-2000 for HAPS to use the two Ka-bands 27.5-28.35 GHz and 31.0-31.3 GHz can potentially alleviate much of the rain attenuation problem in the wetter regions. However, such approval should be formalized at the next WRC meeting in order to ensure that these regions will enjoy the low cost, fast roll-out, broadband benefits of HAPS-based telecommunications infrastructure.

8.	CONCLUSION

A global stratospheric telecommunications system employing fixed platforms in the stratosphere could meet the world’s demand for affordable high-speed wireless communications.

__________

http://www.mpath.com/papers/greatbrands/sld007.html

“Trends and Technologies in Personal Wireless Local Loop Communications,” Montgomery Securities, Volume 27, September 23, 1996, Montgomery Securities