|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | TELECOMMUNICATION DEVELOPMENT BUREAU ITU-D STUDY GROUPS | Document 2/220-E 25 October 2001 Original: French |
| FOURTH MEETING OF STUDY GROUP 1: CARACAS (VENEZUELA), 3 - 7 SEPTEMBER 2001 FOURTH MEETING OF STUDY GROUP 2: CARACAS (VENEZUELA), 10 - 14 SEPTEMBER 2001 |  |  |

FOR INFORMATION

Question 12/2:	Examine broadband communications over traditional copper wires on aspects of technologies, systems and applications

STUDY GROUP 2

SOURCE:	MALI

TITLE:	MALI'S EXPERIENCE WITH BROADBAND COMMUNICATIONS

________

Abstract:

HDSL was introduced in Mali in 1997. The characteristics of the system and the difficulties encountered are described in this document.

# I	Country overview

a)	Introduction

The Republic of Mali is situated between latitudes 10° and 26º North and longitudes 4° and 12º West, and has a land area of 1 241 231 km2. It is a land-locked country in West Africa bordered to the north by Algeria, to the east by Niger, to the south by Burkina Faso, Côte d'Ivoire and Guinea and to the west by Senegal and Mauritania. In all, Mali has 7 200 km of borders.

At the end of 2000, Mali's population was estimated at 10 206 244, broken down as follows:

Rural population: 7 650 601

Urban population: 2 555 643

The country's population density is 8.22 inhabitants per square kilometre.

The capital city, Bamako, has a population of 1 059 318.

b)	Telecommunications

Telecommunications in Mali are administered by the Société des télécommunications du Mali (SOTELMA), which is a State corporation established on 9 October 1989. Until February 2001, SOTELMA held a monopoly on the provision of telecommunication services in the country.

SOTELMA has juridical personality and financial autonomy, and reports to the Minister of Communications. It has its own board of directors.

SOTELMA's objects are:

–	to operate the public telecommunication service;

–	to develop telecommunication services.

The business plan drawn up by SOTELMA expresses this mission in terms of offering telecommunication services to the largest possible number of users throughout the country's territory, whilst assuring good quality of service at affordable prices and guaranteeing the company's profitability.

As at 31 December 2001, the total number of subscribers is reckoned at 49 726, of whom 39 222 are fixed subscribers and 10 530 are mobile subscribers (6 003 on AMPS and 4 500 on GSM).

Of the 39 041 (non-cellular) main telephone lines, 70 per cent are concentrated in Bamako. In addition, the 10 530 mobile lines are all installed in Bamako. For fixed main telephone lines, the density in Bamako is 2.75 (versus 0.40 for the country as a whole).

Main telephone lines, productivity, density

| Year | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of staff | 1 465 | 1 421 | 1 402 | 1 383 | 1 338 | 1 357 | 1 360 | 1 338 | 1 353 | 1 357 |
| Population (thousands) | 8 289 | 8 464 | 8 646 | 8 832 | 9 013 | 9 198 | 9 299 | 9 790 | 10 006 | 10 206 |
| Main lines | 12 464 | 12 837 | 13 812 | 15 209 | 17 164 | 21 294 | 26 330 | 31 191 | 40 165 | 45 226 |
| Productivity (staff per thousand main lines) | 118 | 111 | 102 | 91 | 78 | 63 | 52 | 42 | 34 | 30 |
| Density | 0.15 | 0.15 | 0.16 | 0.17 | 0.19 | 0.23 | 0.28 | 0.32 | 0.40 | 0.44 |

Breakdown of telephone lines nationwide as of 31 December 2000

|  | Government | Private |  |  | SOTELMA |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Business | Household | Payphones | Lines | Payphones |  |
| Bamako (fixed) | 1 347 | 12 746 | 11 220 | 1 516 | 538 | 244 | 27 611 |
| Bamako (cellular) | 144 | 3 128 | 2 644 | 1 | 86 |  | 6 003 |
| Kayes | 166 | 1 224 | 980 | 68 | 54 | 24 | 2 516 |
| Koulikoro | 162 | 610 | 1 040 | 72 | 46 | 35 | 1 965 |
| Sikasso | 203 | 1 132 | 825 | 94 | 47 | 37 | 2 338 |
| Ségou | 198 | 992 | 895 | 115 | 40 | 23 | 2 263 |
| Mopti | 175 | 680 | 282 | 45 | 40 | 22 | 1 244 |
| Tombouctou | 105 | 222 | 254 | 8 | 24 | 20 | 633 |
| Gao | 132 | 273 | 186 | 31 | 22 | 9 | 653 |
|  | 2 632 | 21 007 | 18 326 | 1 950 | 897 | 414 | 45 226 |
| Totals |  | 41 283 |  |  | 1 311 |  |  |

# II	HDSL in Mali

a)	Introduction

Mali's telephone network is characterized by a very low level of development (0.45 lines per hundred inhabitants), which means that it ranks below other countries having a similar level of development.

The under-development of the national telecommunication network is evident in the lack of connection capacity in the local networks, which are either saturated or antiquated.

A great effort has been made during the past five years to modernize and extend local networks through three major projects carried out by Société DTA (two projects) and ALCATEL (one project).

Thanks to these projects, it has been possible to modernize some of the facilities and extend the network.

Nevertheless, despite the various activities undertaken with a view to developing the network, it remains under-equipped and can not keep pace with the continually growing demand.

The problem is particularly acute in certain areas of Bamako, where entire neighbourhoods still lack appropriate facilities. Moreover, nearly all cross-connect areas are reaching saturation point. This situation with the network is creating a large backlog of unmet demand. Even worse for the development of Bamako's urban network is the fact that more than half the demand is attributable to business needs, with nearly a quarter of this demand concentrated in and around the city's commercial centre.

In addition, the lack of network capacity in Bamako serves to stifle a significant volume of demand.

It was in this context that HDSL was introduced in Bamako's network in 1997. By the end of 2000, there were 1 004 subscribers, representing 3.6 per cent of Bamako's main lines. The HDSL systems that have been installed are operating satisfactorily. The problems identified by the maintenance service have been the result of power outages which have occurred mainly during the rainy season.

Another problem relates to the quality of the copper pairs used. With the network largely being in a state of disrepair in the centre of Bamako, some of the copper pairs in use are not in good condition.

It is because of the problems cited above, and particularly the last item relating to the quality of the copper wires, that it has not been possible to make greater use of HDSL systems in Mali.

For all these reasons, SOTELMA is considering a plan to modernize the system and extend Bamako's urban network.

This plan has three main components:

•	a project in the city centre, on the left bank of the River Niger;

•	a local network project on the right bank of the River Niger;

•	extension of the existing local loop.

Under the project, which is supported by various studies that have been carried out, more than 30 000 fixed lines are to be provided.

The equipment used is as follows:

•	PG-Flex (2/32) (320 subscribers; 10 units)

•	PG-Flex (2/24) (384 subscribers; 16 units)

•	PG-Plus (2/32) (300 subscribers; 50 units)

This equipment is used in the city centre, where the network has been operating to capacity for at least five years.

b)	Description PG-FLEX

PG-Flex is a small capacity universal subscriber carrier system supporting up to 32 subscriber channels, including POTS and ISDN services. The system is based on HDSL transmission technology and the remote power is powered from the Central Office. Using two 24 (0.5 mm) gauge unconditioned copper pairs, the remote terminal may be located up to 3.3 km from the Central Office terminal. Standard 19- or 23-inch shelves contain multiple systems; circuit cards may be hot-swapped without affecting other systems installed in the shelf. POTS channel units use 64-kbps A-Law PCM encoding to allow Group 3 facsimile or high-speed modem operation on all channels.

System Flex configuration. A PG-Flex system consists of:

•	Central Office Terminal (COT). Two complete systems may be installed in a 19-inch COT Shelf and four complete systems may be installed in a 23-inch COT Shelf. Each system requires one COT Line Unit and up to three (23-inch shelf) or six (19-inch shelf) Channel Units. A common Alarm Unit or Pair Gain Test Controller Interface Unit in each shelf provides an interface for maintenance alarm relays and metallic access to the remote subscriber lines. The PG-Flex FLL-716 Central Office Terminal (COT) Line Unit, is the Central Office end of a subscriber carrier system that uses two pairs of unconditioned copper wire as a carrier for up to 32 independent subscriber channels between a COT and a Remote Terminal (RT). The FLL-716 Line Unit uses Pair Gain's High-bit-rate Digital Subscriber Line (HDSL) technology to provide the equivalent of 2 048 Mbps digital transmission rates, plus signalling over two copper pairs without the need for repeaters, loop conditioning, or pair selection.

•	Remote Terminal (RT). The RT Enclosure provides a weatherproof housing for the PG-Flex remote electronics and subscriber terminations. The enclosure accepts one common RT Line Unit and up to four Channel Units. The Remote Terminal communicates with, and is line-powered from, the COT over two pairs of 19- to 26-gauge (0.9 mm to 0.4 mm) twisted-pair telephone wire. Each pair carries both power and HDSL signalling.

HDSL transmission. PG-Flex utilizes HDSL transmission technology between the COT and RT and provides up to 32 DSO, plus signalling, over two copper pairs without the need for repeaters, loop conditioning, or pair selection. Adaptive equalization, scrambling, and a four-level 2B1Q line coding scheme increase range and minimize crosstalk.

The line is a two-pair, 1 110-kbps full-duplex 2B1Q transmission format. The dual HDSL lines provide 32 64-kbps channels with signalling, and an embedded operations channel for management control. The signal characteristics on the carrier pairs comply with TR-NWT-001210, Generic Requirements for High-bit-rate Digital Subscriber Line (HDSL lines provide 32 64-kbps channels with signalling control. The signal characteristics on the carrier pairs comply with TR-NWT-001210, Generic Requirements for High-bit-rate Digital Subscriber Line (HDSL) Systems.

For the system configuration, the maximum distance between the COT and the RT is 3.3 km, assuming the HDSL lines are 24 gauge (0.5 mm).The table shows the maximum distance between the COT and RT for various gauge wire. Due to the nature of HDSL transmission technology, the HDSL line do not require any special conditioning and may include unterminated bridge taps, but cannot include load coils.

Subscriber drop testing. For subscriber drop testing from the Central Office, PG-Flex is able to select and connect any subscriber drop to metallic test pair at the RT. PG-Flex extends this

connection back to the COT where technicians can switch it onto a test access bus, or to the corresponding subscriber line on the COT channel card.

PG-FLEX distances

| Wire gauge | Loop length |  |
| --- | --- | --- |
|  | 16/32 Channel system | Resistance |
| 26 AWG (0.4 mm) | 2.5 km | 686  |
| 24 AWG (0.5 mm) | 3.3 km | 569  |
| 22 AWG (0.6 mm) | 4.2 km | 457  |
| 16 AWG (0.9 mm) | 5.9 km | 322  |

Test access is achieved by placing +116 V on the subscriber's COT tip lead for 1.5 seconds, then removing the = 116 V. Depending on the configuration of the PG-Flex system, the selected subscriber drop (assuming a working metallic bypass pair) will be connected to either the corresponding COT subscriber line or to the test access bus.

The test connection will be dropped when –116V is applied to the subscriber's COT tip for 1.5 seconds, then removing the –116 V.

Test access may also be activated through the RS-232 maintenance Port located on the front of the COT and RT Line Units, using an ASII terminal.

c)	Conclusion

The HDSL systems that have been installed in Mali have been operating satisfactorily. The problems identified by the maintenance service relate, by and large, to power outages occurring principally during the rainy season.

Another problem relates to the quality of the copper pairs used. With the network having deteriorated generally in Bamako's city centre, some copper pairs are not in good condition.

Typical PG-Flex configuration

32 subscriber lines

PG-Flex system central office terminal (COT)

HDSL, two copper pairs, carrying both power and signalling

To central office terminal

32 POTS circuits

Battery central office alarm

Test access

PG-Flex system remote terminal (RT)

____________