| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION DEVELOPMENT BUREAU ITU-D STUDY GROUPS | RGQ11/2/002-E 25 February 2000 Original: English only |
| Meeting of Rapporteur’s Group on Question 11/2, Geneva, 23-24 February 2000 |  |

Question 11/2: 	Examine digital broadcasting technologies and systems, including cost/benefit analyses, assessment of demands on human resources, interoperability of digital systems with existing analogue networks, and methods of migration from analogue to digital technique

STUDY GROUP 2

SOURCE:	ASIA-PACIFIC BROADCASTING UNION

TITLE:	ABU TASK GROUP (P2) ON DIGITAL SOUND BROADCASTING

_____

The Asia-Pacific Broadcasting Union decided to apply an integrated approach in addressing the DSB studies so that comprehensive guidance could be provided to the members.  Consequently, at the 1997 meeting the ABU Engineering Committee established a Task Group (P-2) on “Digital Sound Broadcasting studies within the ABU”. The Terms of Reference of the Task Group are:

To prepare a comprehensive report on DSB addressing the total system–source to reception — covering relevant aspects of the available system choices and implementation strategies, including:

- Summaries of system tests conducted by ABU members and others

- Characteristics of available systems and their implications for broadcasters.

The Group consists of :

Mr. K.M. Paul, AIR (Coordinator)

Mr. Peter Kepreotes, ABC

Mr. Xie Jinhui, RTPRC

Mr. Shinil Chung, KBS

Mr. Toru Kuroda, NHK

Mr. Yong Wui Pin, RCS

The Task Group provided an interim report to the 1998 meeting of the ABU Engineering Committee. A draft of the final report has now been completed and is being coordinated within the Group. It is expected that the report will be distributed to all ABU members by March 2000, possibly earlier.

The contents of the 87 page report are listed in the Annex.

Draft ABU Report on DIGITAL SOUND BROADCASTING (DSB)

CONTENTS

PAGE No.

- INTRODUCTION								1

- WHY DSB?										3

  - Importance of Digital Technology						3

  - Limitations in analogue broadcasting						3

  - Features of DSB								3

    - Delivers high quality audio							3

    - Overcomes the effect of multipath fading					4

    - Minimises the effect of frequency selective fading				4

    - DSB is multi efficient								4

    - Capable of providing Multimedia Services					5

  - Necessity to keep pace with technological changes				5

- DSB SYSTEM REQUIREMENTS AS ENVISAGED BY ITU-R		6

- TERRESTRIAL-DSB SYSTEMS						8

  - Eureka-147 System (System A of ITU-R)					8

  - BST-OFDM System of NHK, Japan						13

  - IBOC System of the USA							13

    - FM-IBOC (USA Digital Radio)						13

    - FM-IBOC (AT&T/Amati)							14

    - FM-IBAC/IBRC								15

4.3.4	IBOC-present status								16

- WORLDWIDE T-DSB PROPAGATION EXPERIMENTS			17

  - Introduction										17

  - Canada											17

5.2.1	Coverage constraints and solutions for hybrid system in L-Band			17

5.2.2	Indoor reception measurements in L-Band					18

5.2.3	Wideband channel characterisation measurements at 1.5 GHz			19

5.3	China												19

5.4	E.B.U. Countries								19

5.4.1	Man-made noise								19

5.5	Finland											21

5.6	Germany										21

5.6.1	Internal network gain in a single frequency network				21

5.6.2	Standard deviation of measurements						22

5.6.3	Coverage measurement along a long distance route				22

5.6.4	Building penetration loss							22

5.7	India												23

5.7.1	Coverage area of DAB transmitters						23

5.7.2	Ignition noise									23

5.7.3	H.T. power lines in proximity of receiver					23

5.7.4	Location probability of DSB signal						24

5.7.5	Polarisation discrimination							24

5.7.6	Antenna height gain								24

5.7.7	Gap fillers										25

5.8	Netherlands										25

5.8.1	Building penetration loss							25

5.9	UK measurements								25

5.9.1	Coverage of individual transmitters						26

5.9.2	Coverage of the full network							26

5.9.3	Building penetration loss in VHF/UHF bands					26

- CHOICE OF FREQUENCY BAND						28

  - VHF Band I										28

    - Man-made noise								28

    - Propagation factors								28

    - Hardware constraints								29

  - VHF Band II										29

  - VHF Band III									29

  - UHF Band IV/V								29

  - L-Band											30

  - Conclusion										30

- DSB SERVICE PLANNING							31

7.1	Basic Criteria for DSB Service Planning					31

  - Interference Considerations							32

  - Protection Ratios								32

  - Basic Planning Parameters							33

    - Minimum wanted field strength when using

the ITU-R Rec. PN-370 propagation prediction model				33

    - Location and time percentage requirements					33

    - Receiving antenna height gain correction					34

    - Minimum wanted field strengths to be used for DSB planning			34

  - Special planning features							35

    - Building penetration loss							35

    - Polarisation										35

    - Propagation factors influencing the choice of polarisation			35

    - Beam tilt										35

    - Beam shaping									35

  - Man-made noise								35

  - Planning of DSB Services							36

    - T-DSB transmitter network structures						36

    - Definition of distances								36

    - A single transmitter case							36

    - An Example on calculation of coverage radius and re-use

distance for DSB single transmitter case					37

    - Factors influencing the re-use distance						39

    - Re-use distance between two SFNs carrying different

Programmes										40

7.8	Computer planning methods							42

7.9	Gap fillers										42

- DSB BELOW 30 MHZ							46

  - Digital Modulation Systems							46

    - VOV/JPL System								47

    - T2M System										49

    - Thomcast’s Skywave-2000							51

    - IBOC of USA Digital Radio							52

    - CCETT (France) COFDM System						53

8.2		DRM for a common World Wide Standard					54

- SATELLITE-DSB SYSTEMS INCLUDING FIELD

EXPERIMENTS								56

  - Introduction										56

    - ITU-R System-A								56

    - ITU-R System-B								56

    - ITU-R System-D								56

    - Digital System-D with Terrestrial MCM Extension				57

  - BBC Experiment with System-A in L-Band					58

    - Transmission Parameters and Experiments					58

      - Modes												58

      - Transponder Input/Output Characteristics					58

      - Link Margin for Fixed Reception						59

      - Mobile Reception								60

      - Reception with Portable Receiver						60

  - Australian Experiment with System-A in L-Band				60

    - Transmission Parameters and Experiments					60

      - Satellite											60

      - The Transmission and Reception System					61

      - Experiments										61

    - Conclusion										62

  - Indian Experiment with ITU-R System-A in S-Band				62

9.4.1	Conclusion										63

  - WorldSpace Experiments in L-Band						64

  - Conclusion										65

10.	DSB RECEIVERS								66

  - Terrestrial Digital Radio Receivers							66

    - Eureka-147 System								66

    - AM and FM IBOC Radio Systems						68

  - Satellite Digital Radio Receivers							68

10.2.1	WorldSpace system								68

10.2.2	Eureka-147 S-DAB								70

10.3	Receivers by Philips Eindhoven						70

10.3.1	Philips DAB-452 test receiver							70

10.3.2	Philips DAB-752 professional receiver						70

10.4	New Era in the Availability of Consumer DSB Receiver			71

11.	COUNTRY-WISE GLOBAL STATUS OF DSB				72

12.	T-DSB versus DVB-T								83

13.	IMPLEMENTATION STRATEGIES					84

  - Introduction										84

  - Asia-Pacific Region – A wide ranging nations					84

  - Selection of Technology							85

  - Suitable Frequency band for DSB						85

  - National Spectrum Planning							85

  - Acquaintance with the technology						85

  - Pilot service										86

  - DSB – Awareness								86

  - Simulcasting										86

  - Production of New Programmes						86

  - Value added services								87

  - Listeners survey								87

  - International efforts								87

  - Conclusion										87

__________