Albania

FOCAL POINT REGARDING CORRESPONDENCE ON THIS QUESTIONNAIRE

(PARTS I, II AND III)

Please identify a focal point in your administration/organization who could provide a response to further correspondence regarding this questionnaire (see hereafter).

1.	Mr./Ms                         ANDONI		                       LORETA

Family Name	First Name

2.	Country   ALBANIA

3.	Name of the Administration/Organization       MINISTRY OF TRANSPORT AND TELECOMMUNICATION

4.	Title                  RADIOCOMMUNICATION SPECIALIST

5.	Address        SCANDERBEG SQUARE, TIRANA, ALBANIA

6.	Tel.:   +355682220903       Fax: +3554233772     E-Mail:   loandoni@hotmail.com

To be returned to:

ITU-D Study Groups Secretariat

Telecommunication Development Bureau

Fax: +41 22 730 54 84

E-Mail: devsg2@itu.int

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 942 – 960 MHz | FIXED MOBILE except aeronautical mobile BROADCASTING S5.322 S5.323 | MOBILE S5.323 | GSM Existing cellular networks | EU13 ERC Decision ERC/DEC/(94)01, FB paired with 897 - 915 MHz. | MOBILE S5.323 | GSM CIVIL / TRE |
| 960 – 1215 MHz | AERONAUTICAL RADIONAVIGATION S5.328 | RADIONAVIGACION AERONATIK S5.328 | Flight, Safety, Navigation and Information Distribution Systems (for example, DME, TACAN, SSR, MIDS) |  | RADIONAVIGACION AERONATIK S5.328 | Flight, Safety, Navigation and Information Distribution Systems (for example, DME, TACAN, SSR, MIDS) CIVIL / TRE A37: Fixed service should be terminated prior to 2003. |
| 1215 – 1240 MHz | EARTH EXPLORATION SATELLITE (active) RADIOLOCATION RADIONAVIGATION-SATELLITE (space to Earth) S5.329 SPACE RESEARCH (active) S5.330 S5.331 5.332 | RADIOLOCATION RADIONAVIGATION-SATELLITE (space to Earth) S5.329 EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION S5.331 S5.332 | Radar and Navigation Systems and Active Sensors; GNSS |  | RADIOLOCATION RADIONAVIGATION-SATELLITE (space to Earth) S5.329 EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION S5.331 S5.332 | Radar and Navigation Systems and Active Sensors; GNSS CIVIL / TRE A38: The operation of governmental users should be terminated prior to 2004. A39: Fixed services should be terminated prior to 2003. |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1240 – 1260 MHz | EARTH EXPLORATION SATELLITE (active) RADIOLOCATION RADIONAVIGACION –SATELITOR (space to Earth) S5.329 SPACE RESEARCH (active) Amateur S5.330 S5.331 S5.332 | RADIOLOCATION RADIONAVIGACION SATELITOR (space to Earth) S5.329 EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGACION Amateur S5.331 S5.332 | Radar and Navigation Systems and Active Sensors; GNSS |  | RADIOLOCATION RADIONAVIGACION SATELITOR (space to Earth) S5.329 EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGACION Amateur S5.331 S5.332 | Radar and Navigation Systems and Active Sensors; GNSS CIVIL / TRE |
| 1260 – 1270 MHz | EARTH EXPLORATION SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Amateur S5.282 S5.330 S5.331 S5.332 | RADIOLOCATION EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGACION Amateur Amator-Satelitor ? S5.282 S5.331 S5.332 | Radar and Navigation Systems and Active Sensors |  | RADIOLOCATION EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGACION Amateur Amator-Satelitor S5.282 S5.331 S5.332 | Radar and Navigation Systems and Active Sensors; CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1270 – 1300 MHz | EARTH EXPLORATION SATELLITE (active) RADIOLOCATION RADIONAVIGACION –SATELITOR (space to Earth) (space to space) S5.329 S5.329A SPACE RESEARCH (active) Amateur S5.282 S5.330 S5.331 S5.333 S5.334 S5.335 | RADIOLOCATION EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION Amateur S5.331 S5.332 | Radar and Navigation Systems and Active Sensors | Wind profiler radars between 1270 MHz and 1295 MHz | RADIOLOCATION EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION Amateur S5.331 S5.332 | Radar and Navigation Systems and Active Sensors CIVIL / TRE |
| 1300 – 1350 MHz | AERONAUTICAL RADIONAVIGATION S5.337 Radiolocation S5.149 | AERONAUTICAL RADIONAVIGATION S5.337 RADIOLOCATION S5.149 | Radar and Navigation Systems | RA spectral line observations 1330 - 1400 MHz. | AERONAUTICAL RADIONAVIGATION S5.337 RADIOLOCATION S5.149 | Radar and Navigation Systems; CIVIL / TRE A40: The operation of fixed link should be terminated prior to 2003. |
| 1350 – 1400 MHz | FIXED MOBILE RADIOLOCATION S5.149 S5.338 S5.339 | FIXED RADIOLOCATION MOBILE S5.149 S5.339 | Defence systems. Low capacity fixed links | EU2 EU15A EU15 Channel plan for the fixed service in ERC Recommendation T/R 13- 01, Annex A. RA spectral line observations 1330 - 1400 MHz. | FIXED RADIOLOCATION MOBILE S5.149 S5.339 | Low capacity fixed links. Channel plan for the fixed service in ERC Recommendation T/R 13- 01, Annex A. CIVILE GOVERNMENTAL USERS |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1400 – 1427 MHz | EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.341 | EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.341 | Passive applications |  | EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.341 | ### Silent band CIVIL / TRE A41: The operation of fixed link should be terminated prior to 2003. |
| 1427 – 1429 MHz | SPACE OPERATIONS (Earth to space ) FIXED MOBILE except aeronautical mobile S5.341 | SPACE OPERATIONS (Earth to space ) FIXED MOBILE except aeronautical mobile S5.341 | Defence systems Low capacity fixed links | EU2 EU15A EU15 Channel plan for the fixed service in ERC ERC Recommendation T/R 13 - 01, Annex B. | SPACE OPERATIONS (Earth to space ) FIXED MOBILE except aeronautical mobile S5.341 | Low capacity fixed links. Channel plan for the fixed service in ERC ERC Recommendation T/R 13 - 01, Annex B. CIVIL / TRE GOVERNMENTAL USERS |
| 1429 – 1452 MHz | FIXED MOBILE except aeronautical mobile S5.341 S5.342 | FIXED MOBILE except aeronautical mobile S5.341 |  |  | FIXED MOBILE except aeronautical mobile S5.341 | CIVIL / TRE GOVERNMENTAL USERS |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1452 – 1492 MHz | FIXED MOBILE except aeronautical mobile BROADCASTING S5.345 S5.347 RADIODIFUZION – SATELITOR S5.345 S5.347 S5.341 S5.342 | BROADCASTING S5.345 S5.347 RADIODIFUZION- SATELITOR S5.345 S5.347 Fixed Mobile except aeronautical mobile S5.341 S5.342 | Digital Audio Broadcasting T-DAB Wiesbaden Special Arrangement 1995 | EU15 ERC Recommendation T/R 52-02. | BROADCASTING S5.345 S5.347 RADIODIFUZION- SATELITOR S5.345 S5.347 Fixed Mobile except aeronautical mobile S5.341 S5.342 | T-DAB CIVIL / TRE, KKRT ##### GOVERNMENTAL USERS |
| 1492 – 1517 MHz | FIXED MOBILE except aeronautical mobile S5.341 S5.342 | FIXED MOBILE except aeronautical mobile S5.341 S5.342 | Defence systems Low capacity fixed links | EU2 EU X EU15 Channel plan for the fixed service in ERC Recommendation T/R 13 - 01, Annex A. | FIXED MOBILE except aeronautical mobile S5.341 S5.342 | Low capacity fixed links. Channel plan for the fixed service in ERC Recommendation T/R 13 - 01, Annex A. CIVIL / TRE GOVERNMENTAL USERS |
| 1517 – 1525 MHz |  | FIXED MOBILE except aeronautical mobile S5.341 S5.342 | Unidirectional fixed links Defence systems | EU2 EU X EU15 | FIXED MOBILE except aeronautical mobile S5.341 S5.342 | Unidirectional fixed links CIVILE /ERT GOVERNMENTAL USERS |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Perdorimet, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1525 – 1530 MHz | SPACE OPERATIONS (space to Earth) FIXED MOBILE SATELLITE (space to Earth) Earth Exploration Satellite Mobile except aeronautical mobile S5.349 S5.341 S5.342 S5.350 S5.351 S5.352A S5.354 | SPACE OPERATIONS (space to Earth) FIXED MOBILE SATELLITE (space to Earth) S5.341 S5.342 S5.351 S5.353A S5.354 | Unidirectional fixed links Mobile satellite systems | EU15 | SPACE OPERATIONS (space to Earth) FIXED MOBILE SATELLITE (space to Earth) S5.341 S5.342 S5.351 S5.353A S5.354 | Unidirectional fixed links Mobile satellite systems CIVIL / TRE GOVERNMENTAL USERS |
| 1530 – 1533 MHz | SPACE OPERATIONS (space to Earth) MOBILE SATELLITE (space to Earth) S5.353A, Earth Exploration Satellite Fixed Mobile except aeronautical mobile S5.341 S5.342 S5.351 S5.354 | SPACE OPERATIONS (space to Earth) MOBILE SATELLITE (space to Earth) Earth Exploration Satellite Fixed Mobile except aeronautical mobile S5.341 S5.342S5.351 S5.353A S5.354 | Mobile satellite systems | EU15 | SPACE OPERATIONS (space to Earth) MOBILE SATELLITE (space to Earth) Earth Exploration Satellite Fixed Mobile except aeronautical mobile S5.341 S5.342S5.351 S5.353A S5.354 | Mobile satellite systems CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1533 – 1535 MHz | SPACE OPERATIONS (space to Earth) MOBILE SATELLITE (space to Earth) S5.353A Earth Exploration Satellite Fixed Mobile except aeronautical mobile S5.341 S5.342 S5.351 S5.354 | SPACE OPERATIONS (space to Earth) MOBILE SATELLITE (space to Earth) Earth Exploration Satellite Mobile except aeronautical mobile S5.341 S5.342 S5.351 S5.353A S5.354 | Mobile satellite systems | EU15 | SPACE OPERATIONS (space to Earth) MOBILE SATELLITE (space to Earth) Earth Exploration Satellite Mobile except aeronautical mobile S5.341 S5.342 S5.351 S5.353A S5.354 | Mobile satellite systems. CIVIL / TRE |
| 1535 – 1544 MHz | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.353A S5.354 S5.355 | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.353A S5.354 | Mobile satellite systems | EU15 | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.353A S5.354 | Mobile satellite systems. CIVIL / TRE |
| 1544 – 1545 MHz |  | MOBILE SATELLITE (space to Earth) S5.354 S5.356 | Search and rescue satellite systems including GMDSS Mobile satellite systems | EU15 | MOBILE SATELLITE (space to Earth) S5.354 S5.356 | Search and rescue satellite systems including GMDSS Mobile satellite systems. CIVIL / TRE |



| Frequency Band | Percaktimet e Rajonit RR1 dhe verejtjet perkatese | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1545 – 1555 MHz | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.353A S5.354 S5.355 | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.354 S5.357 S5.359 S5.362A | Mobile satellite systems | EU15 + EU15A | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.354 S5.357 S5.359 S5.362A | Mobile satellite systems. CIVIL / TRE |
| 1555 – 1559 MHz |  | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.354 S5.359 | Mobile satellite systems | EU15 | MOBILE SATELLITE (space to Earth) S5.341 S5.351 S5.354 S5.359 | Mobile satellite systems. CIVIL / TRE |
| 1559 – 1610 MHz | AERONAUTICAL RADIONAVIGATION RADIONAVIGACION SATELITOR (space to Earth) S5.341 S5.355 S5.359 S5.363 | AERONAUTICAL RADIONAVIGATION RADIONAVIGACION SATELITOR (space to Earth) S5.341 S5.359 | GNSS | EU15 | AERONAUTICAL RADIONAVIGATION RADIONAVIGACION SATELITOR (space to Earth) S5.341 S5.359 | GNSS CIVIL / TRE |
| 1610 - 1610.6 MHz | MOBILE SATELLITE (Earth to space ) AERONAUTICAL RADIONAVIGATION S5.341 S5.355 S5.359 S5.363 S5.364 S5.366 S5.367 S5.368 S5.369 S5.371 S5.372 | AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Earth to space ) S5.341 S5.355 S5.359 S5.364 S5.367 S5.368 S5.372 | Satellite personal communications systems | EU15 ERC Decision ERC/DEC/(97)03 | AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Earth to space ) S5.341 S5.355 S5.359 S5.364 S5.367 S5.368 S5.372 | Satellite personal communications systems. CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1610.6 – 1613.8 MHz | MOBILE SATELLITE (Earth to space ) AERONAUTICAL RADIONAVIGATION RADIO ASTRONOMY S5.149 S5.341 S5.355 S5.359 S5.363 S5.364 S5.366 S5.367 S5.368 S5.369 S5.371 S5.372 | AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Earth to space ) RADIO ASTRONOMY S5.149 S5.341 S5.359 S5.364 S5.366 S5.367 S5.368 S5.372 | Satellite personal communications systems Radio astronomy applications | EU15 ERC Decision ERC/DEC/(97)03 Important band for radio astronomy spectral line observations. | AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Earth to space ) RADIO ASTRONOMY S5.149 S5.341 S5.359 S5.364 S5.366 S5.367 S5.368 S5.372 | Satellite personal communications systems Radio astronomy applications. CIVIL / TRE |
| 1613.8 – 1626.5 MHz | MOBILE SATELLITE (Earth to space ) S5.SSS AERONAUTICAL RADIONAVIGATION Mobile Satellite (space to Earth) S5.341 S5.355 S5.359 S5.363 S5.364 S5.365 S5.366 S5.367 S5.368 S5.369 S5.371 S5.372 | AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Earth to space ) Mobile Satellite (space to Earth) S5.341 S5.359 S5.364 S5.365 S5.367 S5.368 S5.372 S5.373 | Satellite personal communications systems | EU15 ERC Decision ERC/DEC/(97)03 | AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Earth to space ) Mobile Satellite (space to Earth) S5.341 S5.359 S5.364 S5.365 S5.367 S5.368 S5.372 S5.373 | Satellite personal communications systems CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1626.5 – 1631.5 MHz | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.355 S5.357A S5.359 S5.362A S5.374 S5.375 S5.376 | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.359 | Mobile satellite systems |  | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.359 | Mobile satellite systems. CIVIL / TRE A42: The operation of fixed link should be terminated prior to 2003. |
| 1631.5 – 1636.5 MHz |  | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.359 S5.374 | Mobile satellite systems | EU15 | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.359 S5.374 | Mobile satellite systems. CIVIL / TRE |
| 1636.5 – 1645.5 MHz |  | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.359 | Mobile satellite systems | EU15 | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.359 | Mobile satellite systems. CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1645.5 – 1646.5 MHz | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.353A S5.354 S5.355 S5.357A S5.359 S5.362A S5.374 S5.375 S5.376 | MOBILE SATELLITE (Earth to space ) S5.341 S5.354 S5.375 | Search and rescue satellite system including GMDSS | EU15 | MOBILE SATELLITE (Earth to space ) S5.341 S5.354 S5.375 | Search and rescue satellite system including GMDSS. CIVIL / TRE |
| 1646.5 – 1656.5 MHz |  | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.354 S5.359 S5.362B S5.376 | Mobile satellite systems | EU15 | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.354 S5.359 S5.362B S5.376 | Mobile satellite systems. CIVIL / TRE A43: The operation of fixed link should be terminated prior to 2003. |
| 1656.5 – 1660 MHz |  | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.354 S5.359 S5.360 S5.362B S5.374 | Mobile satellite systems | EU15 | MOBILE SATELLITE (Earth to space ) S5.341 S5.351 S5.354 S5.359 S5.360 S5.362B S5.374 | Mobile satellite systems. CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1660 - 1660.5 MHz | MOBILE SATELLITE (Earth to space ) RADIO ASTRONOMY S5.149 S5.341 S5.351 S5.354 S5.362A S5.376A | MOBILE SATELLITE (Earth to space ) RADIO ASTRONOMY S5.149 S5.341 S5.351 S5.354 S5.376A S5.362B | In accordance with ECARadio astronomy applications Mobile satellite systems | EU15 Important band for radio astronomy. VLBI | MOBILE SATELLITE (Earth to space ) RADIO ASTRONOMY S5.149 S5.341 S5.351 S5.354 S5.376A S5.362B | Radioastronomy applications Mobile systems. CIVIL / TRE |
| 1660.5 – 1668.4 MHz | RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile S5.149 S5.341 S5.379 S5.379A | RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile S5.149 S5.341 S5.379A | Radioastronomy applications Mobile systems | EU2 EU15A EU15 Important band for radio astronomy: Continuum line and VLBI measurements. | RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile S5.149 S5.341 S5.379A | Radioastronomy applications Mobile systems. CIVIL / TRE GOVERNMENTAL USERS |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1668.4 – 1670 MHz | METEOROLOGICAL AIDS FIXED Mobile except aeronautical mobile RADIO ASTRONOMY S5.149 S5.341 | METEOROLOGICAL AIDS FIXED RADIO ASTRONOMY Mobile except aeronautical mobile S5.149 S5.341 | Radio astronomy applications Mobile systems Meteorological applications | EU2 EU15A EU15 Important band for radio astronomy. | METEOROLOGICAL AIDS FIXED RADIO ASTRONOMY Mobile except aeronautical mobile S5.149 S5.341 | Radio astronomy applications Mobile systems Meteorological applications. CIVIL / TRE |
| 1670 – 1675 MHz | METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space to Earth) MOBILE S5.380 S5.341 | METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space to Earth) MOBILE S5.380 Fixed S5.341 | TFTS (ground to air) Meteorological applications | ERC Decision ERC/DEC/(92) 01. ERC Recommendation T/R 42-01. | METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space to Earth) MOBILE S5.380 Fixed S5.341 | TFTS (ground to air) in compliance with ERC Recommendation T/R 42-01. Meteorological applications. CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1675 – 1690 MHz | METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space to Earth) MOBILE except aeronautical mobile S5.341 | METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space to Earth) MOBILE except aeronautical mobile S5.341 | Mobile systems Meteorological applications | EU2 EU15A EU15 | METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space to Earth) MOBILE except aeronautical mobile S5.341 | Sisteme te levizshme Aplikime meteorologjike. CIVIL / TRE GOVERNMENTAL USERS |
| 1690 – 1700 MHz | METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space to Earth) Fixed Mobile except aeronautical mobile S5.289 S5.341 S5.382 | METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space to Earth) Fixed Mobile except aeronautical mobile S5.341 S5.382 | Meteorological applications | EU2 EU15A EU15 | METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space to Earth) Fixed Mobile except aeronautical mobile S5.341 S5.382 | Aplikime meteorologjike. CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1700 - 1710 MHz | FIXED METEOROLOGICAL-SATELLITE (space to Earth) MOBILE except aeronautical mobile S5.289 S5.341 | FIXED METEOROLOGICAL-SATELLITE (space to Earth) Mobile except aeronautical mobile S5.341 | Meteorological applications | EU2 EU15A EU15 | FIXED METEOROLOGICAL-SATELLITE (space to Earth) I Levizshem perveç ` Aeronautik S5.341 | Meteorological applications. CIVIL / TRE GOVERNMENTAL USERS |
| 1710 – 1785 MHz | FIXED I LEVIZSHEM S5.380 S5.149 S5.341 S5.385 S5.386 S5.387 S5.388 | FIXED I LEVIZSHEM S5.149 S5.341 S5.385 | DCS 1800 | EU15 ERC Decision ERC/DEC/(95)03 ERC Recommendation T/R 22- 07 | FIXED I LEVIZSHEM S5.149 S5.341 S5.385 | DCS 1800 CIVIL / TRE A44: The operation of fixed link should be terminated prior to 2003. |
| 1785 – 1800 MHz |  | FIXED I LEVIZSHEM | Mobile systems | EU2 EU15 Radio microphones (under study) | FIXED I LEVIZSHEM | Mobile systems. CIVIL / TRE GOVERNMENTAL USERS |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1800 – 1805 MHz | FIXED MOBILE S5.380 S5.149 S5.341 S5.385 S5.386 S5.387 S5.388 | MOBILE S5.380 Fixed | TFTS (air to ground) | EU15 ERC Decision ERC/DEC/(92) 01 ERC Recommendation T/R 42-01 | MOBILE S5.380 Fixed | TFTS (air to ground), in accordance with ERC Recommendation T/R 42-01. CIVIL / TRE |
| 1805 – 1880 MHz |  | FIXED MOBILE | DCS 1800 | EU15 ERC Decision ERC/DEC/(95)03 ERC Recommendation T/R 22 - 07 | MOBILE | DCS 1800, in accordance with ERC Recommendation T/R 22 – 07. CIVIL / TRE A45: The operation of fixed link should be terminated prior to 2003. |
| 1880 – 1885 MHz |  | MOBILE Fixed | DECT | EU15 ERC Decision ERC/DEC/(94) 03 | MOBILE | DECT CIVIL / TRE |
| 1885 – 1900 MHz |  | MOBILE Fixed S5.388 | DECT | EU15 ERC Decision ERC/DEC/(94) 03 | MOBILE S5.388 | DECT CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | ## Utilisation ## Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 1900 - 1930 MHz | FIXED MOBILE S5.380 S5.149 S5.341 S5.385 S5.386 S5.387 S5.388 | FIXED MOBILE S5.388 | IMT-2000 | EU16 EU15 ERC Decision ERC/DEC/(97)07 | MOBILE S5.388 | UMTS / IMT-2000. CIVIL / TRE A46: The operation of fixed link should be terminated prior to 2003. |
| 1930 - 1970 MHz | FIXED MOBILE S5.388 |  |  |  |  |  |
| 1970 - 1980 MHz | FIXED MOBILE S5.388 |  |  |  |  |  |
| 1980 – 2010 MHz | FIXED MOBILE MOBILE SATELLITE (Earth to space ) S5.388 S5.389A S5.389B S5.389F | FIXED MOBILE MOBILE SATELLITE (Toke - hapesire) S5.388 S5.389 | IMT-2000 Satellite personal communications systems. | EU16 EU15 ERC Decisions ERC/DEC/(97)03, ERC/DEC/(97)04 and ERC/DEC/(97)07 | MOBILE MOBILE SATELLITE (Toke - hapesire) S5.388 S5.389 | UMTS / IMT-2000 Satellite personal communications systems. CIVIL / TRE A47: The operation of fixed link should be terminated prior to 2003. |
| 2010 – 2025 MHz | FIXED MOBILE S5.388 | FIXED MOBILE S5.388 | IMT-2000 | EU16 EU15 ERC Decision ERC/DEC/(97)07 | MOBILE S5.388 | UMTS / IMT-2000 CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | Percaktim i Pebashket Europian | Major Utilisation | Notes | Albanian Allocation | ## Utilisation, Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 2025 – 2110 MHz | SPACE OPERATIONS (Earth to space ) (space to Earth) EARTH EXPLORATION SATELLITE (Earth to space ) (space to space) FIXED MOBILE S5.391 SPACE RESEARCH (Earth to space ) (space to space) S5.392 | FIXED MOBILE S5.391 SPACE RESEARCH (Earth to space ) (space to space) SPACE OPERATIONS (Earth to space ) (space to space) EARTH EXPLORATION SATELLITE (Earth to space ) (space to space) S5.392 | Fixed links Space science services Mobile systems | EU2 EU16A EU15 Channel plan for the fixed service in ERC Recommendation T/R 13-01, Annex C. | FIXED MOBILE S5.391 SPACE RESEARCH (Earth to space ) (space to space) SPACE OPERATIONS (Earth to space ) (space to space) EARTH EXPLORATION SATELLITE (Earth to space ) (space to space) S5.392 | Fixed links. Channel plan for the fixed service in ERC Recommendation T/R 13-01, Annex C. Space science services Mobile systems CIVIL / TRE GOVERNMENTAL USERS |
| 2110 – 2120 MHz | FIXED MOBILE SPACE RESEARCH (deep space) (Earth to space ) S5.388 | FIXED MOBILE SPACE RESEARCH (Earth to space , deep space) S5.388 | IMT-2000 | EU16 EU15 ERC Decision ERC/DEC/(97)07 | MOBILE S5.388 | UMTS / IMT-2000 CIVIL / TRE A48: The operation of fixed link should be terminated prior to 2003. |
| 2120 – 2160 MHz | FIXED MOBILE S5.392A S5.388 | FIXED MOBILE S5.388 | IMT-2000 Fixed links Space science services Mobile systems | EU16 EU15 ERC Decision ERC/DEC/(97)07 EU2 EU16A EU15 Channel plan for the fixed service in ERC Recommendation T/R 13-01, Annex C. | MOBILE S5.388 | UMTS / IMT-2000 CIVIL / TRE A49: The operation of fixed link should be terminated prior to 2003. |
| 2160 – 2170 MHz | FIXED MOBILE S5.BBB S5.388 S5.392A |  |  |  |  |  |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | Utilisation, Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 2170 – 2200 MHz | FIXED MOBILE MOBILE SATELLITE (space to Earth) S5.388 S5.389A S5.389F S5.392A | FIXED MOBILE MOBILE SATELLITE (space to Earth) S5.388 S5.389A S5.392A | IMT-2000 Satellite personal communications systems | EU16 ERC Decisions ERC/DEC/(97)03, ERC/DEC/(97)04 and ERC/DEC/(97)07 | MOBILE MOBILE SATELLITE (space to Earth) S5.388 S5.389A S5.392A | Komponente satelitore te UMTS / IMT-2000 CIVIL / TRE A50: The operation of fixed link should be terminated prior to 2003. |
| 2200 – 2290 MHz | SPACE OPERATIONS (space to Earth) (space to space) EARTH EXPLORATION SATELLITE (space to Earth) (space to space) FIXED MOBILE S5.391 SPACE RESEARCH (space to Earth) (space to space) S5.392 | SPACE OPERATIONS (space to Earth) (space to space) EARTH EXPLORATION SATELLITE (space to Earth) (space to space) FIXED MOBILE S5.391 SPACE RESEARCH (space to Earth) (space to space) S5.392 | Fixed links Space science services Mobile systems | EU16A EU15 Channel plan for the fixed service in ERC Recommendation T/R 13-01, Annex C. RA VLBI | SPACE OPERATIONS (space to Earth) (space to space) EARTH EXPLORATION SATELLITE (space to Earth) (space to space) FIXED MOBILE S5.391 SPACE RESEARCH (space to Earth) (space to space) S5.392 | Fixed links. Channel plan for the fixed service in ERC Recommendation T/R 13-01, Annex C. Space science services. Mobile systems CIVIL / TRE GOVERNMENTAL USERS |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | Utilisation, Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 2290 – 2300 MHz | FIXED MOBILE except aeronautical mobile SPACE RESEARCH (deep space)(space to Earth) | FIXED MOBILE except aeronautical mobile SPACE RESEARCH (deep space)(space to Earth) | Mobile systems | EU2 RA VLBI | FIXED MOBILE except aeronautical mobile SPACE RESEARCH (deep space)(space to Earth) | Mobile systems CIVIL / TRE |
| 2300 – 2400 MHz | FIXED MOBILE Amateur Radiolocation S5.150 S5.282 S5.395 | FIXED MOBILE Amateur Radiolocation S5.150 S5.395 | Mobile systems | EU2 ERC Recommendation CEPT/ERC/REC 62-02. Parts of the band are used for aeronautical telemetry on a national basis. | FIXED MOBILE Amateur Radiolocation S5.150 S5.395 | CIVIL / TRE GOVERNMENTAL USERS |
| 2400 – 2450 MHz |  | FIXED MOBILE Amateur Amateur Satellite S5.282 S5.150 | ISM, SRD, WBDTS, R-Tags | EU2 ERC Recommendation CEPT/ERC/REC 70-03 | FIXED MOBILE Amateur Amateur Satellite S5.282 S5.150 | ISM (2400 – 2500MHz), secondary service, in accordance with ERC Recommendation CEPT/ERC/REC 70-03. CIVIL / TRE GOVERNMENTAL USERS |
| 2450 - 2483.5 MHz | FIXED MOBILE Radiolocation S5.150 S5.397 | FIXED MOBILE S5.150 S5.397 | ISM, SRD, WBDTS, R-Tags | EU2 ERC Recommendation CEPT/ERC/REC 70-03 | FIXED MOBILE S5.150 S5.397 | ISM (2400 – 2500MHz), secondary service, in accordance with ERC Recommendation CEPT/ERC/REC 70-03 CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | Utilisation, Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 2483.5 - 2500 MHz | FIXED MOBILE MOBILE SATELLITE (space to Earth) Radiolocation S5.150 S5.371 S5.397 S5.398 S5.399 S5.400 S5.402 | FIXED MOBILE MOBILE SATELLITE (space to Earth) S5.150 S5.397 | In accordance with ECA Satellite personal communications systems ISM | EU15 ERC Decision ERC/DEC/(97)03) | FIXED MOBILE MOBILE SATELLITE (space to Earth) S5.150 S5.397 | In accordance with ECA Satellite personal communications systems ISM (2400 – 2500MHz) CIVIL / TRE |
| 2500 - 2520 MHz | FIXED S5.409 S5.410 S5.411 Mobile except aeronautical mobile MOBILE SATELLITE (space to Earth) S5.403 S5.405 S5.407 S5.408 S5.412 S5.414 | MOBILE SATELLITE (space to Earth) Fixed Mobile except aeronautical mobile S5.403 S5.414 | Mobile satellite systems | EU15 | MOBILE SATELLITE (space to Earth) Fixed Mobile except aeronautical mobile S5.403 S5.414 | Mobile satellite systems CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | Utilisation, Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 2520 – 2655 MHz | FIXED S5.409 S5.410 S5.411 Mobile except aeronautical mobile RADIODIFUZION-SATELITOR S5.413 S5.416 S5.339 S5.403 S5.405 S5.408 S5.412 S5.417 S5.418 | FIXED Mobile except aeronautical mobile S5.339 S5.403 | Fixed links Defence systems | EU2 EU15 Channel plan for the fixed service in ERC Recommendation T/R 13-01, Annex D. | FIXED Mobile except aeronautical mobile S5.339 S5.403 | Fixed links, in accordance with ERC Recommendation T/R 13-01, Annex D. CIVIL / TRE GOVERNMENTAL USERS |
| 2655 – 2670 MHz | FIXED S5.409 S5.410 S5.411 Mobile except aeronautical mobile RADIODIFUZION-SATELITOR S5.413 S5.416 Earth Exploration Satellite (passive) Radio Astronomy Space Research (passive) S5.149 S5.412 S5.417 S5.420 | FIXED Mobile except aeronautical mobile Earth Exploration Satellite (passive) Radio Astronomy Space Research (passive) S5.149 S5.420 | Fixed links Mobile systems | EU2 EU15 Channel plan for the fixed service in ERC Recommendation T/R 13-01, Annex D. RA continuum measurements. | FIXED Mobile except aeronautical mobile Earth Exploration Satellite (passive) Radio Astronomy Space Research (passive) S5.149 S5.420 | Fixed links, in accordance with ERC Recommendation T/R 13-01, Annex D. CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | Utilisation, Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 2670 - 2690 MHz | FIXED S5.409 S5.410 S5.411 MOBILE except aeronautical mobile MOBILE SATELLITE (Earth to space ) Satelitor per Eksplorimin eTokes (passive) Radio Astronomy Space Research (passive) S5.149 S5.412 S5.419 S5.420 | MOBILE SATELLITE (Earth to space ) Fixed Mobile except aeronautical mobile Radioastronomi S5.149 S5.419 S5.420 | Mobile satellite systems | EU15 RA continuum measurements. | MOBILE SATELLITE (Earth to space ) Fixed Mobile except aeronautical mobile Radioastronomi S5.149 S5.419 S5.420 | Mobile satellite systems. CIVIL / TRE |
| 2690 - 2700 MHz | EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.421 S5.422 | EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.421 S5.422 | Passive applications |  | EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 S5.421 S5.422 | Passive applications CIVIL / TRE |

| Frequency Band | RR Region 1 Allocation and Relevant Footnotes | European Common Allocation | Major Utilisation | Notes | Albanian Allocation | Utilisation, Users in Albania |
| --- | --- | --- | --- | --- | --- | --- |
| 2700 - 2900 MHz | AERONAUTICAL RADIONAVIGATION S5.337 Radiolocation S5.423 S5.424 | AERONAUTICAL RADIONAVIGATION S5.337 Radiolocation S5.423 | Radars and Navigation Systems Meteorological radars |  | AERONAUTICAL RADIONAVIGATION S5.337 Radiolocation S5.423 | Radars and Navigation Systems Meteorological radars CIVIL / TRE |
| 2900 - 3100 MHz | RADIONAVIGATION S5.426 Radiolocation S5.425 S5.427 | RADIONAVIGATION S5.426 Radiolocation S5.425 S5.427 | Radars |  | RADIONAVIGATION S5.426 Radiolocation S5.425 S5.427 | Radar CIVIL / TRE |
| 3100 - 3300 MHz | RADIOLOCATION Satelitor per Eksplorimin eTokes (active) Space Research (active) S5.149 S5.428 | RADIOLOCATION Satelitor per Eksplorimin eTokes (active) Space Research (active) S5.149 S5.428 | Radars and Active Sensors |  | RADIOLOCATION Satelitor per Eksplorimin eTokes (active) Space Research (active) S5.149 S5.428 | Radars and Active Sensors CIVIL / TRE |
| 3300 -3400 MHz | RADIOLOCATION S5.149 S5.429 S5.430 | RADIOLOCATION | Radars |  | RADIOLOCATION | Radar CIVIL / TRE |

QUESTIONNAIRE - PART II

(To be completed by Administrations only)

General Questions on National Spectrum Management

1.	What legal or regulatory texts govern your national spectrum management processes?

Law No 8618, dated 14.6.2000, “On Telecommunication in the Republic of Albania” and National Frequency Allocation Plan.

Are any actions planned to change these legal texts or regulations? 		NO

2.	Have you publicly available regulations and procedures for national spectrum management (e.g. radio services, license requirements etc.)		YES

4.	Regulations for the technical characteristics of radiocommunications equipment

Do you specify that the technical characteristics of radiocommunications equipment must comply with certain requirements (often referred to as "equipment standards"), for example to avoid interference to other services and users?                             		YES

- Do you develop these technical requirements or equipment standards on a national basis or use those developed by other administrations or international/regional standards organisations:			Other

- Do you have a procedure to ensure that radiocommunications equipment complies with the technical requirements, for example:

Manufacturers Declaration of Compliance

5.	Spectrum re-deployment*

(* The term "redeployment" is used here to refer to a process of national scope in which an assessment is conducted 1) to determine if portions of spectrum can be identified that are in limited use; and 2) to determine if  such spectrum segments can be reallocated for use in delivering radiocommunication services that have expanding spectrum requirements. Some countries co-operate on a regional basis to identify suitable spectrum segments that may be  re-deployed to facilitate the introduction of new applications on a harmonised basis.)

- Has there been any spectrum redeployment* in your country or has a need for spectrum redeployment been identified? 		NO

- If so, do you have a method for achieving this redeployment in respective frequency bands and for given radiocommunication services?		NO

- Please define the established method and describe the nature of the consultation, if any, with users regarding the potential costs resulting from the planned redeployment.

6.	Spectrum management costs

- What is the cost of providing national spectrum management functions in your country (if there is more than one organisation or agency responsible for spectrum management please give the total costs if this information is available)?

98 000(Swiss Francs)

- What is the source of the funding required to accomplish these spectrum management functions?

The fees of licenses granted by TRE and NRTC.

- Management of frequency assignment records.

- Does your administration have a system (manual or computerised) to keep and maintain records of national frequency assignments and spectrum use (usually known as a Data Base Management System (DBMS))?	Manual

- Is there a single national DBMS or separate DBMS(s) for different users (for example a DBMS for assignments to government users and separate DBMS for assignments to non-government users)?		Single

What is the approximate size (at 2002) of your DBMS:

- number of frequency assignments                            approximately  1500 frequencies

- number of licences	              331 licenses

- Are these frequency assignment records made available to public? 		NO

- Is the DBMS computerised? 		NO

- What computerised DBMS do you use?	___________

- Co-ordination of frequency assignments with other countries:

- do you co-ordinate assignments to terrestrial stations           		YES

- do you co-ordinate assignments to space stations                  		YES

- Notification of frequency assignments.

Do you notify to the ITU those frequency assignments that are required to be notified by the Radio Regulations?		YES

If not, please explain why and list any difficulties: _______________________________

10.	Do you have a policy and planning function for national spectrum management (i.e. a national strategy for future use of the spectrum)? 		YES

11.	Do you perform technical analyses of frequency assignment requests? 		NO

12	Do you perform radio monitoring of terrestrial radio services? 	Only in broadcasting services

The decision No. 379, dated 31.5.2001, “On approval of National Frequency Plan”, of Council of Ministers decided the establishment of National Monitoring Centre in TRE.

## Fixed monitoring stations

- How many fixed monitoring stations do you have?  __1(Broadcasting services)

- Please provide a brief list of the facilities available at your fixed monitoring stations (for example: receivers, spectrum analysers, direction finding equipment):

_______________________________________________________________________

- What is the upper frequency limit of your fixed monitoring stations  _3000GHz

- What is the upper frequency limit of your fixed direction finding stations __30 kHh MHz

## Mobile monitoring stations

- How many mobile monitoring stations do you have? !(broadcasting services)

- Please provide a brief list of the facilities available in your mobile monitoring stations (for example: receivers, spectrum analysers, direction finding equipment)

______ receivers, spectrum analysers, direction finding equipment

- What is the upper frequency limit of your mobile monitoring stations  ______ MHz

- What is the upper frequency limit of your mobile direction finding stations ______ MHz

## Transportable monitoring stations

- How many transportable monitoring stations do you have? 1(broadcasting services)

- Please provide a brief list of the facilities available in your transportable monitoring stations (for example: receivers, spectrum analysers, direction finding equipment):

receivers, spectrum analysers, direction finding equipment ___

- What is the upper frequency limit of your transportable monitoring stations  ____ MHz

- What is the upper frequency limit of your transportable direction finding stations ___ MHz

- Do you perform space monitoring						NO

- Please provide a brief list of the facilities available at your space monitoring stations

___________________________________________________________________

- What tasks does your space monitoring station perform for GSO satellite monitoring?

- What tasks does your space monitoring station perform for non-GSO satellite monitoring?

## Does your Administration participate in the International Monitoring Programme of ITU? 	YES ___   NO ___

## Co-operation between Spectrum Management and Monitoring

Please indicate the amount of work (in percentages) performed by the monitoring service for:

- Frequency Management Department ________%

- Enforcement Department ___________ %

- License Department ____________ %

- Do you perform Inspections on Radio Stations 		YES

- What inspection techniques are used by your administration to determine that users of the spectrum are complying with national or international requirements?

Inspection on site

- What are the administrative procedures that determine your inspection policy (for example the number of inspections, type of notification provided prior to inspection, rules and regulations)?

Based on the “On Telecommunication in the Republic of Albania” law, Inspectors of TRE must carry proof of identity. They may inspect the premises where a licensee conducts the activity even without prior notice, to verify whether services are offered in conformity with this law and the conditions of the license

A licensee may not prevent inspectors from doing their duty and is obliged to cooperate by giving information to inspectors for the nature and use of radiocommuncations and telecommunications equipment, if they require such information.

There are two specialists in ERT in charge of inspection.

Inspectors may ask for an expert, when inspection requires expertise.

- What measurement equipment does your administration use to perform technical measurements at an inspection?

NA

- What technical parameters does your administration measure when inspecting a radio system?

NA

- What station records does your administration review when inspecting a radio station?

License conditions

14.	Do you perform technical analyses of radio frequency interference complaints?			YES

Do you have an established consultation process, involving Government and non-government organisation, for resolving these complaints?

Partially, only with government organisation.

15.	Use of computers for national spectrum management

## General

- Do you use computers for national spectrum management?	Manual

- Type of computers                                                                                    Desktop PC-s

- How many workstations: ___ or personal computers (PCs):    		 3

- Operating system(s) 	Win 2000

- Does your spectrum management system operate within a Local Area Network (LAN)?	NO

- Do you have access to the internet?	YES

- Does your administration provide a web site on the internet to disseminate spectrum management information?	NO

If yes, please provide the address (URL) of the web site:

Windows Basic Spectrum Management System (WinBASMS)

- Are you aware that a Windows Basic Spectrum Management System is available from the ITU at no cost?	YES

- Has your administration used WinBASMS? 	NO

- Has your administration had problems using WinBASMS?	NA

- Please list all problems that were encountered using WinBASMS.

__________________________________________________________________

- Would you recommend using WinBASMS if the problems identified in (d) have been corrected?	NA

- Do you need an enhanced spectrum management system if you answered no in (e)?	NA

## Advanced Automated Spectrum Management Systems (AASMS)

- Does your administration use an Automated Spectrum Management Systems (AASMS)	NO

- Has your administration had problems using your AASMS	NA

- Please list all problems that were encountered using your AASMS

________________________________________________________________________

- How would you propose to change the AASMS to correct or overcome these problems (please describe)?

- Organisation of spectrum management

- Please describe your country's spectrum management structure and enclose a copy of the organisation chart. The following aspects are of particular interest:

- Is the spectrum management organisation a separate ministry, department or agency reporting directly to the government or is it part of a larger government department (for example, a department responsible for all telecommunications)?

- Is the responsibility for spectrum management contained within a single organisation or is it shared between separate organisations (for example, some administrations have separate organisations for regulatory matters and  policy matters, other administrations have separate organisations for government users and non-government users)?

The organisational structure in Albania involved in frequency management process consist on:

General Directorate of Posts and Telecommunications (GDPT) is a legal, public,buxhetory person, and works for fulfillment of the objectives of policy on telecommunications. GDPT depends on the Minister of Transport and Telecommunications.

GDPT coordinate work for drafting the National Frequencies Allocation Table, as well as for its harmonisation with international policies for radio frequencies spectrum development;

Telecommunications Regulatory Entity (TRE) is a public legal entity operating in conformity with its regulation approved by the Board of TRE and independent in making its decisions. TRE should supervise the all frequency spectrum, in order to ensure effective, interference-free use of frequencies. TRE manages radio frequency spectrum for civil purposes and makes regulations for civil spectrum activities.

Regulations and activities falling within radiocommunications must be in conformity with rules and standards of international organizations, conventions and treaties where Republic of Albania is a party.

## Organs of Radio Frequency Management

- TRE manage frequency bands designated for civil purposes and public or private use, except broadcasting bands.

- National of Radio Television Council manage broadcasting bands

- Ministry of Defense, Ministry of Public Order and National Information Service manage frequency bands aimed for national defense use.

- Have there been recent changes in this organisational structure or are changes planned (for example to take account of any changes in your government's policy for telecommunications)?	NO

- Number of specialist staff in national spectrum management? 	Approximately 14 specialists

- Number of support staff in national spectrum management?	Approximately 8 specialists

17.	Do you use the ITU-R Handbooks and Reports on:

a)	National Spectrum Management, version 1995?	NO

b)	Spectrum Monitoring, version 2002?	NO

c)	Computer-aided Techniques for Spectrum Management, version 1999?  	NO

d)	Report SM.2012-1, Economic Aspects of Spectrum Management, version 2000? NO

18.	Identification of problems experienced in national spectrum management.

Please use the following table to describe problems experienced by your administration in national spectrum management. This information will be used by the ITU, in particular ITU-R Study Group 1, to identify future areas of work, within the normal study programme, so that effort may be focused on the development of recommendations and reports for subjects where assistance is most needed.

| Question | Please describe the spectrum management problem associated with the Question and the type of assistance that could be provided by the ITU. |
| --- | --- |
| Q1 |  |
| Q2 |  |
| Q3 |  |
| Q4 |  |
| Q5 |  |
| Q6 |  |
| Q7 | Problems caused by lack of computerized system to keep and maintain records national frequency assignments. |
| Q8 |  |
| Q9 |  |
| Q10 |  |
| Q11 | Lack of skills in the technical analyses process of frequency assignments. |
| Q12 | Lack of equipment and skills on frequency monitoring process. |
| Q13 |  |
| Q14 |  |
| Q15 |  |
| Q16 |  |
| Q17 | Lack of ITU literatures on managing and monitoring of frequency specter |

## QUESTIONNAIRE - PART III
(To be completed by administrations)
Information on the calculation of fees for frequency use

## 3.1	General questions

## Q1

•	Are there any legal texts on the establishment of frequency fees?

Reply: 	Yes

•	If yes, please indicate their references and the date on which they were last updated.

Reply:	1. Law No. 8713, dated 15.12.2000,“On taxes system in the Republic of Albania”.

2. The Decision, No. 71, dated 15.2.2001, of the Council of Ministers “For approval of the fees made to TRE”

## Q2

•	What procedure (regulatory, legislative, etc.) is used to review and update your system for setting frequency fees?

Reply: Based on government decision, the approach is that TRE and NRTC reveneus from frequency fees should cover the expenses made for frequency managing and monitoring.

•	Are reviews conducted at pre-established regular intervals? If yes, please specify:

Reply: No

•	Does recourse to market mechanisms (auctions, calls for tenders) to screen applicants for spectrum access require that parliament enact legislation, that the government make a decision, or any other measure? Please specify.

Reply: Yes, by government decision.

## Q3

•	Are the same approaches and principles used to set frequency fees for all users?

Reply: Yes

•	If yes, please complete the charts in APPENDIX 2.

•	If no:

	please indicate the methods used to calculate fees or the scales applied to agencies that use frequencies for non-commercial activities;

	then, please complete the charts in APPENDIX 2 for the agencies that use frequencies for commercial activities.

Reply:

## Q4

•	In addition to direct frequency fees, certain administrations require the payment of additional spectrum-related charges (for example, for spectrum access, spectrum replanning, management of equipment using the frequencies).

Does your Administration require such payments?

Reply:No

•	If yes, please specify:

	the users concerned;

	the methods used to calculate the charges or the scales applied and the corresponding amounts.

Reply:

## Q5

•	To which institution(s) are the frequency fees and any additional charges collected paid?

Reply: For the state budget and Telecommunication Regulatory Entity budget and National Radio Television Council budget.

## 3.2	Exemption from payment of frequency fees

## Q6

•	Are any applications partially or completely exempted from the payment of frequency fees?

Reply: No

•	If yes, please specify:

	the applications concerned;

	their respective rate of exemption;

	the method used to calculate the fees or the scale applied, if they differ from those indicated in rows 20 and 21 of the charts in APPENDIX 2.

Reply:

## Q7

•	Are any users partially or wholly exempted from the payment of frequency fees?

Reply: Based on “On taxes system in the Republic of Albania” Law, the foreign embassies in Albania and humanitarian organisation are wholly exempted from the payment of frequency fees.

•	If yes, please specify:

	the users concerned;

	their respective rate of exemption;

	the method used to calculate the fees or the scale applied, if they differ from those indicated in rows 20 and 21 of the charts in APPENDIX 2.

Reply:

## 3.3 	The application of frequency fees

Administrations are invited to respond to the questions asked in charts A to E in APPENDIX 2, dealing respectively with the fixed, mobile, satellite and broadcasting services and other applications.

The charts comprise:

	horizontally, three sections corresponding respectively:

	[rows 1 to 21]: to the variables which may be used to set the fees and to the methods applied. This section contains shaded cells corresponding to non-relevant situations;

	[row 22]: to the explanations, grounds and objectives;

	[rows 23 to 25]: to recourse to market mechanisms, as the case may be;

	vertically, the various applications relating to the service considered.

## 3.3.1	Approaches and principles for setting frequency fees

To answer this part of ITU-D Question 21/2, please complete rows 1 to 21 of the five charts (A to E) in APPENDIX 2.

In each chart, for any given application:

•	for the variables, administrations should reply:

	yes (by crossing out or deleting the letter "n") in the cells relating to the variables they use to set fees;

	no (by crossing out or deleting the letter "y") in the cells relating to the variables they do not use;

•	under "methods used" (rows 20 and 21), administrations should indicate, separately and depending on the case, the formulas or scales used to calculate the amount of the fees, preceded by the references indicated in the corresponding cells. Administrations are invited to explain the formulas and scales they use and how they are implemented.

Note:	An administration concerned by a cell in row 20 in respect of one application will not be concerned by the corresponding cell in row 21 in respect of the same application, and vice versa.

Example 1	Take Chart A ("fixed service") and the application "Radio relays".

•	To establish the corresponding fees, if the administration uses the variables "bandwidth", "centre frequency", "number of transmitting stations" and "duration of authorization/licence", it should reply "yes" in the cells situated at the intersection of rows 1, 2, 10 and 13 with the column "Radio relay". In all other cells in that column, it should reply "no".

•	To determine the amount of the fees:

	if the administration uses the following formula:
"Annual charge for a link  =  100 x f/f ", 
where f = bandwidth and f = centre frequency,
it could reply as follows:
"A1:
Annual charge for a link  =  100 x f/f "

	if the administration uses no formula, it should append the corresponding scale under reference A7.

## 3.3.2 	Explanations, grounds and objectives (row 22 in the charts)

For each of the cells in row 22, administrations are invited to provide information on the grounds for their choice, for the variables used to set the fees and for the methods applied to determine the amount of those fees.

Example 2	Following on from example 1, the administration could reply as follows:

"A13:

	the variable "bandwidth" was chosen to encourage economical use of the spectrum;

	the variable "centre frequency" was chosen to encourage the use of high frequencies;

	the variable "number of transmitting stations" was chosen to take account of spectrum and geographic occupancy;

	the variable "duration of authorization" was chosen in order to enable collection of a global amount corresponding to the total length of time the spectrum is occupied. It also reduces the risk of frequency hoarding and non-use."

Appendix 2

Chart A: FIXED service

|  | APPLICATIONS VARIABLES | Row No. | Radio relay | Local radio loop (incl. LMDS, MMDS) | Links between fixed stations (incl. HF) | Local radio networks | Other application(s): please specify |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spectrum-related variables | bandwidth | ## 1 | ## n | y | ## y | ## y | ## n |
|  | number of channels | ## 1bis | ## y | n | ## n | ## n | ## n |
|  | centre frequency, or band position in the spectrum | ## 2 | ## Y | n | ## n | ## n | ## n |
|  | exclusive / shared use | ## 3 | ## Y | y | ## y | ## y | ## n |
| Variables relating to geographic coverage | surface area allocated | ## 4 | ## n | n | ## n |  | ## n |
|  | distance between transmitter and receiver | ## 5 | ## y |  | ## y |  | ## n |
| # Variables relating to equipment and infrastructure | transmitter power | ## 6 | ## n | n | ## n | ## n | ## n |
|  | antenna height | ## 7 | ## n | n | ## n |  | ## n |
|  | bit rate or capacity | ## 8 | ## y | y | ## y | ## y | ## n |
|  | transmitting beam angle | ## 9 | ## n |  |  |  | ## n |
|  | number of transmitting stations | ## 10 | ## y | y | ## y | ## y | ## n |
|  | number of receiving stations | ## 11 | ## n | n | ## n | ## n | ## n |
|  | Degressivity | ## 12 | ## n | n | ## n | ## n | ## n |
| ## Socio-economic variables | duration of the authorization / licence | ## 13 | ## n | n | ## n | ## n | ## n |
|  | population density | ## 14 |  | n | ## n | ## n | ## n |
|  | total population covered | ## 15 |  | n |  |  | ## n |
|  | geographic location | ## 16 | ## n | ## n | ## n | ## n | ## n |
|  | operator's turnover | ## 17 |  | n | ## n |  | ## n |
|  | Gross domestic product | ## 18 | ## n | n | ## n | ## n | ## n |
| ## Other variable(s): please specify |  | ## 19 | n | n | ## n | ## n | ## n |

Appendix 2

| ## Methods used | calculation formulas and corresponding amounts | ## 20 | ## A1 | ## A2 | ## A3 | ## A4 | ## A5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scales | 21 | A6 | A7 | A8 | A9 | A10 |
| ### Explanations and grounds, objectives |  | ## 22 | ## A11 | ## A12 | ## A13 | ## A14 | ## A15 |
| ## Recourse to market mechanisms | Auctions | ## 23 | ## A16 | ## A17 | ## A18 |  | ## A20 |
|  | call for tenders | 24 | A21 | A22 | A23 |  | A25 |
|  | comparative selection (beauty contests) | 25 | A26 | A27 | A28 |  | A30 |

A6	 340 CHF/year for frequencies up to 1 GHz, 560 CHF/year for frequencies from 1 GHz to 5 GHz, 890 CHF/year for frequencies from 5 GHz to 11 GHz; 1340 CHF/year for frequencies above 11 GHz.

A draft for the new regime of frequencies fees is sent to the Government for approval. In this draft are taken into considerations also WLL services, and the new approach is quite different as the one expressed in A6. This new regime of fees considers the following variables:

“bandwidth” to encourage economical use of the spectrum.

“centre frequency” to encourage the use of high frequency.

“exclusive / shared” to encourage the effective use of the spectrum.

“bit rate or capacity”(digital equipment) is chosen to encourage economical use of the spectrum.

Appendix 2

CHART B: MOBILE service

|  | APPLICATIONS VARIABLES | Row No. | 2G mobile systems | 3G mobile systems | Radio- messaging | Private independent networks | Operated independent networks | Citizen band (CB) | RRI 446 (or family radio) | Other application(s): please specify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spectrum-related variables | bandwidth | ## 1 | ## y | NA | ## y | ## y | ## y | ## n | ## y | ## n |
|  | centre frequency, or band position in the spectrum | ## 2 | ## n | NA | ## n | ## n | ## n | ## n | ## n | ## n |
|  | exclusive / shared use | ## 3 | ## y | NA | ## n | ## n | ## n |  |  | ## n |
| Variables relating to geographic coverage | surface area allocated | ## 4 | ## n | NA | ## n | ## n | ## n |  |  | ## n |
|  | distance between transmitter and receiver | ## 5 |  |  |  | ## y | ## y |  |  | ## n |
| Variables relating to equipment and infrastructure | transmitter power | ## 6 |  |  |  | ## n | ## n | ## n | ## n | ## n |
|  | antenna height | ## 7 |  |  |  | ## n | ## n | ## n |  | ## n |
|  | bit rate or capacity | ## 8 | ## n | ## NA |  | ## n | ## n |  |  | ## n |
|  | transmitting beam angle | ## 9 |  |  |  |  |  |  |  | ## n |
|  | number of transmitting stations | ## 10 | ## n | NA | ## y | ## y | ## y | ## n | ## y | ## n |
|  | number of receiving stations | ## 11 | ## n | NA | ## n | ## n | ## n |  |  | ## n |
|  | degressivity | ## 12 | ## n | NA | ## n | ## n | ## n | ## n | ## n | ## n |
| ## Socio-economic variables | duration of the authorization / licence | ## 13 | ## n | NA | ## n | ## n | ## n | ## n |  | ## n |
|  | population density | ## 14 | ## n | NA | ## n | ## n | ## n |  |  | ## n |
|  | total population covered | ## 15 | ## n | NA | ## n | ## n | ## n |  |  | ## n |
|  | geographic location | ## 16 | ## n | NA | ## n | ## n | ## n |  |  | ## n |
|  | operator's turnover | ## 17 | ## n | NA | ## n |  | ## n |  |  | ## n |
|  | Gross domestic product | ## 18 | ## n | NA | ## n | ## n | ## n |  |  | ## n |
| Other variable(s): please specify |  | 19 | ## n | NA | ## n | ## n | ## n | ## n | ## n | ## n |

Appendix 2

| ## Methods used | calculation formulas and corresponding amounts | ## 20 | ## B1 | ## B2 | ## B3 | ## B4 | ## B5 | ## B6 | ## B7 | ## B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | scales | 21 | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
| Explanations and grounds, objectives |  | ## 22 | ## B17 | ## B18 | ## B19 | ## B20 | ## B21 | ## B22 | ## B23 | ## B24 |
| ## Recourse to market mechanisms | auctions | ## 23 | ## B25 | ## B26 | ## B27 | ## B28 | ## B29 |  |  | ## B32 |
|  | call for tenders | 24 | B33 | B34 | B35 | B36 | B37 |  |  | B40 |
|  | comparative selection (beauty contests) | 25 | B41 | B42 | B43 | B44 | B45 |  |  | B48 |

B9   - 140 CHF/year per GSM 900 channel; 1110 CHF/year per GSM 1800 channel;

B10 -  not yet introduced

B11 – 560 CHF/year for a local system; 1110 CHF/year for a regional system, 220 CHF/year for a national system

B12, B13

PMR - 55 CHF/year for a simplex system  (unlimited in number of mobile and fixed stations)

PMR - 167 CHF/year for a half-duplex system (1 base station and unlimited mobile stations)

Trunk systems - 167 CHF/year for regional

Trunk systems - 270 CHF/year for national

#### Resources to market mechanisms

B33, B34

Appendix 2

Chart C: SATELLITE service

|  | ###### APPLICATIONS VARIABLES | ## Row No. | ## VSAT | Earth stations | ## Satellite video reporting | ## Mobile satellite service | ## Satellite radiolocation | Other application(s): please specify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spectrum-related variables | Bandwidth | ## 1 | ## n | y | ## y | ## y | ## y | ## n |
|  | Number of channels | ## 1bis | ## n | n | ## n | ## n | ## n | ## n |
|  | Centre frequency, or band position in the spectrum | ## 2 | ## n | n | ## n | ## n | ## n | ## n |
|  | Exclusive / shared use | ## 3 | ## n | n | ## n | ## n | ## n | ## n |
| ## Variables relating to geographic coverage | Surface area allocated | ## 4 |  |  | ## n | ## n | ## n | ## n |
|  | Distance between transmitter and receiver | ## 5 |  |  |  |  |  | ## n |
| Variables relating to equipment and infrastructure | Transmitter power | ## 6 | ## n | n | ## n |  |  | ## n |
|  | Antenna diameter | ## 7 | ## n | n | ## n |  |  | ## n |
|  | bit rate or capacity | ## 8 | ## y | y | ## y | ## y | ## y | ## n |
|  | Transmitting beam angle | ## 9 | ## n | n | ## n |  |  | ## n |
|  | Number of transmitting stations | ## 10 | ## y | y | ## y | ## y |  | ## n |
|  | Number of receiving stations | ## 11 | ## n | n |  | ## n | ## n | ## n |
|  | Degressivity | ## 12 | ## n | n | ## n | ## n | ## n | ## n |
| ## Socio-economic variables | Duration of authorization / licence | ## 13 | ## n | n | ## n | ## n | ## n | ## n |
|  | Population density | ## 14 | ## n | n | ## n | ## n | ## n | ## n |
|  | total population covered | ## 15 |  |  |  | ## n | ## n | ## n |
|  | Geographic location | ## 16 | ## n | n | ## n | ## n | ## n | ## n |
|  | Operator's turnover | ## 17 | ## n | n | ## n | ## n | ## n | ## n |
|  | Gross domestic product | ## 18 | ## n | n | ## n | ## n | ## n | ## n |
| ## Other variable(s): please specify |  | ## 19 | n | n | ## n | ## n | ## n | ## n |

Appendix 2

| ## Methods used | Calculation formulas and corresponding amounts | 20 | C1 |  | C2 |  | C3 |  | C4 |  | C5 |  | C6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scales | 21 | C7 |  | C8 |  | C9 |  | C10 |  | C11 |  | C12 |  |
| ## Explanations and grounds, objectives |  | ## 22 |  | ## C13 |  | ## C14 |  | ## C15 |  | ## C16 |  | ## C17 |  | ## C18 |
| ## Recourse to market mechanisms | Auctions | 23 |  | C19 |  | C20 |  | C21 |  | C22 |  | C23 |  | C24 |
|  | call for tenders | 24 |  | C25 |  | C26 |  | C27 |  | C28 |  | C29 |  | C30 |
|  | Comparative selection (beauty contests) | 25 |  | C31 |  | C32 |  | C33 |  | C34 |  | C35 |  | C36 |

C7 – 220 CHF/year per each terminal

C8 – 2450 CHF/year per each station

C10 – licence fee for offering GMPCS services 2000 USD/year

##### Others

Mobile earth station  and SNG– 1000 USD for a week

2000 USD for a month

5000 USD for 3 month

Appendix 2

Chart D: BROADCASTING service

|  | APPLICATIONS VARIABLES | Row No. | Sound broadcasting |  |  |  | Television broadcasting |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Earth |  | Satellite |  | Earth |  | Satellite |  |
|  |  |  | Analogue | Digital | Analogue | Digital | Analogue | Digital | Analogue | Digital |
| Spectrum-related variables | Bandwidth | ## 1 | ## n | NA | ## y | ## NA | ## y | ## NA | ## y | ## NA |
|  | Centre frequency, or band position in the spectrum | ## 2 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |
|  | Exclusive / shared use | ## 3 | ## y | NA | ## y | ## NA | ## y | ## NA | ## y | ## NA |
| ## Variables relating to geographic coverage | Surface area allocated | ## 4 | ## n | NA |  |  | ## n | ## NA |  |  |
|  | Distance between transmitter and receiver | ## 5 |  |  |  |  |  |  |  |  |
| # Variables relating to equipment and infrastructure | Transmitter power | ## 6 | ## y | NA |  |  | ## y | ## NA |  |  |
|  | Antenna height | ## 7 | ## n | NA |  |  | ## n | ## NA |  |  |
|  | Bit rate or capacity | ## 8 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |
|  | Transmitting beam angle | ## 9 |  |  |  |  |  |  |  |  |
|  | Number of transmitting stations | ## 10 | ## n | NA |  |  | ## n | ## NA |  |  |
|  | Number of receiving stations | ## 11 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |
|  | Degressivity | ## 12 | ## n | NA | ## y | ## NA | ## n | ## NA | ## n | ## NA |
| ## Socio-economic variables | Duration of authorization / licence | ## 13 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |
|  | Population density | ## 14 | ## y | NA | ## y | ## NA | ## y | ## NA | ## y | ## NA |
|  | Total population covered | ## 15 | ## y | NA | ## y | ## NA | ## y | ## NA | ## y | ## NA |
|  | Geographic location | ## 16 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |
|  | Operator's turnover | ## 17 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |
|  | Gross domestic product | ## 18 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |
| ## Other variable(s): please specify |  | ## 19 | ## n | NA | ## n | ## NA | ## n | ## NA | ## n | ## NA |

Appendix 2

| ## Methods used | Calculation formulas and corresponding amounts | ## 20 | ## D1 | ## D2 | ## D3 | ## D4 | ## D5 | ## D6 | ## D7 | ## D8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scales | 21 | D9 | D10 | D11 | D12 | D13 | D14 | D15 | D16 |
| Explanations and grounds, objectives |  | ## 22 | ## D17 | ## D18 | ## D19 | ## D20 | ## D21 | ## D22 | ## D23 | ## D24 |
| ## Recourse to market mechanisms | Auctions | ## 23 | ## D25 | ## D26 | ## D27 | ## D28 | ## D29 | ## D30 | ## D31 | ## D32 |
|  | Call for tenders | 24 | D33 | D34 | D35 | D36 | D37 | D38 | D39 | D40 |
|  | Comparative selection (beauty contests) | 25 | D41 | D42 | D43 | D44 | D45 | D46 | D47 | D48 |

D9

- For areas with population up to 300,000 inhabitants:

audio broadcasting  320 CHF/year

video broadcasting  800 CHF/year

- For areas with population from  300,000  to 400,000 inhabitants:

audio broadcasting  380 CHF/year

video broadcasting  950 CHF/year

- For areas with population above 400,000 inhabitants:

audio broadcasting  450 CHF/year

video broadcasting  1110 CHF/year

- For national:

audio broadcasting  16700 CHF/year

video broadcasting  55600 CHF/year

##### Resources to market mechanisms

D 41

The variables “population density” and “total population covered” are chosen to encourage effective use of frequency spectrum.

The variable “exclusive / shared” is chosen to encourage the effective use of the spectrum.

The variable “transmitter power” is chosen to take in consideration the geographic occupancy.

Appendix 2

Chart E: other applications

|  | APPLICATIONS VARIABLES | Row No. | Radio amateur | Experimental networks | Low-range, low-power devices | Radio- navigation | Radio- location | Weather service | Other application(s): please specify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spectrum-related variables | bandwidth | ## 1 | n | n | ## n | ## n | ## n | ## n | ## n |
|  | centre frequency, or band position in the spectrum | ## 2 | n | n | ## n | ## n | ## n | ## n | ## n |
|  | exclusive / shared use | ## 3 |  | n |  | ## n | ## n | ## n | ## n |
| ## Variables relating to geographic coverage | surface area allocated | ## 4 |  | n |  | ## n | ## n | ## n | ## n |
|  | distance between transmitter and receiver | ## 5 |  | n |  |  |  |  | ## n |
| # Variables relating to equipment and infrastructure | transmitter power | ## 6 | n | n | ## y | ## n |  |  | ## n |
|  | antenna height | ## 7 | n | n |  |  |  |  | ## n |
|  | bit rate or capacity | ## 8 |  | n | ## y | ## n | ## n | ## n | ## n |
|  | transmitting beam angle | ## 9 |  | n |  | ## n |  | ## n | ## n |
|  | number of transmitting stations | ## 10 | n | n | ## y | ## y | ## y | ## y | ## n |
|  | number of receiving stations | ## 11 | n | n | ## n |  | ## n |  | ## n |
|  | degressivity | ## 12 | n |  | ## n | ## n | ## n | ## n | ## n |
| ## Socio-economic variables | duration of authorization / licence | ## 13 | n | n | ## n | ## n | ## n | ## n | ## n |
|  | population density | ## 14 |  |  |  |  |  |  | ## n |
|  | total population covered | ## 15 |  |  |  |  |  |  | ## n |
|  | geographic location | ## 16 |  |  |  |  |  |  | ## n |
|  | operator's turnover | ## 17 |  |  | ## n | ## n | ## n | ## n | ## n |
|  | Gross domestic product | ## 18 |  |  |  |  |  |  | ## n |
| ## Other variable(s): please specify |  | ## 19 | ## n | n | ## n | ## n | ## n | ## n | ## n |

Appendix 2

| ## Methods used | calculation formulas and corresponding amounts | ## 20 | ## E1 | ## E2 | ## E3 | ## E4 | ## E5 | ## E6 | ## E7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scales | 21 | E8 | E9 | E10 | E11 | E12 | E13 | E14 |
| Explanations and grounds, objectives |  | ## 22 | ## E15 | ## E16 | ## E17 | ## E18 | ## E19 | ## E20 | ## E21 |
| ## Recourse to market mechanisms | auctions | ## 23 |  |  |  | ## E25 | ## E26 | ## E27 | ## E28 |
|  | call for tenders | ## 24 |  |  |  | E31 | E32 | E33 | E34 |
|  | comparative selection (beauty contests) | 25 |  |  |  | E38 | E39 | E40 | E41 |

E1

For Albanian radio amateurs  2.22 CHF for 5 years

For foreign radio amateurs 5.6 CHF for 5 years

E4

Aeronautic services

Terrestrial station: 278 CHF/year

Airplane station: 167 CHF/year

Maritime services

Coast station: 278 CHF/year

Ship station: 112 CHF/year

The National Spectrum Management Handbook is currently being updated. You are urged to contact Mr Robert Mayher, Chairman ITU-R Study  Group 1 and the designated Rapporteur for revision of this Handbook if you have any comments that you wish included in this revision.