FOCAL POINT REGARDING CORRESPONDENCE ON THIS QUESTIONNAIRE

(PARTS I, II AND III)

Please identify a focal point in your administration/organization who could provide a response to further correspondence regarding this questionnaire (see hereafter).

1.	Mr./Ms                        Del Monte                                                        Howard

Family Name	First Name

2.	Country                     United Kingdom of Great Britain & Northern Ireland

3.	Name of the Administration/Organization       Radiocommunications Agency

4.	Title

5.	Address	               14R-5B Wyndham House

189 Marsh Wall

London E14 9SX

United Kingdom

6.	Tel.: +44 20 7211 0090  Fax: +44 20 7211 0047  E-Mail: howard.delmonte@ra.gsi.gov.uk

To be returned to:

ITU-D Study Groups Secretariat

Telecommunication Development Bureau

Fax: +41 22 730 54 84

E-Mail: devsg2@itu.int

Attachment 1

QUESTIONNAIRE - PART I

### FREQUENCY ORDER TABLE

### 960 – 3000MHz

| International Region 1 Allocation | European Common Allocation Table Report 25 | UK Allocations | Current Use | Strategy |
| --- | --- | --- | --- | --- |
| 960-1215 MHz AERONAUTICAL RADIONAVIGATION S5.328, S5.328A | 960-1215 MHz AERONAUTICAL RADIONAVIGATION S5.328 | 960-1215 MHz AERONAUTICAL RADIONAVIGATION S5.328 | # AERONAUTICAL Band managed by the CAA. 1164-1300 MHz - RNSS allocation identified at WRC-2000. RADIO ASTRONOMY Radio Astronomy on 962-970 MHz - used for pulsars. MoD Military uses a tactical navigation system (TACAN), Interrogation Friend or Foe (IFF) system and Joint Tactical Information Distribution System (JTIDS). | Possible use by the European Galileo initiative. No changes planned. To continue to support spectrum requirements, within the agreed frequency constraints. |
| 1215-1240 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION RADIONAVIGATION- SATELLITE (sE) S5.329 SPACE RESEARCH (active) S5.329A S5.330, S5.331, S5.332 | 1215-1240 MHz RADIOLOCATION RADIONAVIGATION- SATELLITE (sE) EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION S5.329, S5.331, S5.332 | 1215-1240 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION RADIONAVIGATION- SATELLITE (sE) S5.329 SPACE RESEARCH S5.332 | # AERONAUTICAL/MoD 1215-1365 MHz – Naval and air defence radars. 1227.6 MHz – Registered with the ITU for use by the satellites of the Global Positioning System (GPS). 1164 – 1300 MHz - RNSS allocation identified at WRC-2000. Possible use by the European Galileo initiative. Spaceborne radiolocation for Earth exploration. | Ensure protection of existing services and aircraft systems from interference effects. |
| 1240-1260 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION RADIONAVIGATION- SATELLITE (sE) (ss) SPACE RESEARCH (active) RADIONAVIGATION- SATELLITE (sE) (ss) S5.329, S5.329A Amateur S5.330, S5.331, S5.332, S5.334, S5.335 | 1240-1260 MHz RADIOLOCATION RADIONAVIGATION SATELLITE(sE) EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION Amateur S5.329, S5.331, S5.332 | 1240-1260 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION RADIONAVIGATION- SATELLITE S5.329 SPACE RESEARCH (active) Amateur S5.332 | # AMATEUR Amateur secondary at powers of up to 26 dBW PEP (into the antenna) for many modes including packet, TV, Morse, etc. Amateur services are permitted on a non-interference basis. # AERONAUTICAL/MoD 1215-1365 MHz – Naval and air defence radars. 1164 – 1300 MHz - RNSS allocation identified at WRC-2000. Civil and military radar, video radar data and security devices. WRC-97 accepted wind profiler radars on site protection basis. Spaceborne radiolocation for Earth exploration. | The Agency is currently looking into the possibility of permitting the use of spread spectrum techniques in the amateur service. Protection of existing services and aircraft systems from interference effects. Possible use by the European Galileo initiative. Monitor ICAO and European developments. Introduction of wind profiler radars to be monitored. |
| 1260-1300 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) RADIONAVIGATION- SATELLITE (sE) (ss) S5.329, S5.329A Amateur | 1260-1270 MHz RADIOLOCATION EARTH EXPLORATION SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION Amateur Amateur-Satellite S5.282, S5.331, S5.332 | 1260-1350 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Amateur Amateur-satellite (Es) | AERONAUTICAL/MoD UK RADIOLOCATION primary on 1300-1350 MHz. 1215-1365 MHz – Naval and air defence radars. Military radar video radar data and security devices Wind profiler radars operate within the band 1270-1295 MHz on a case-by-case, NIB, basis. 1164-1300 MHz - RNSS allocation identified at WRC-2000. # AMATEUR Amateur secondary at powers of up to 26 dBW for many modes including packet, TV, Morse, etc. on 1260-1325 MHz. Amateur services are permitted on a non-interference basis # AMATEUR-SATELLITE Amateur-satellite on 1260-1270 MHz (Es) and 1300-1375 MHz | Protection of existing services and aircraft systems from interference effects. Possible use by the European Galileo initiative. Monitor ICAO and European developments. Introduction of wind profiler radars to be monitored. Possible use by the European Galileo initiative. The Agency is currently looking into the possibility of permitting the use of spread spectrum techniques in the amateur service. No changes planned. |
| S5.282, S5.330, S5.331, S5.332, S5.334, S5.335 | 1270-1300 MHz RADIOLOCATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH (active) RADIONAVIGATION Amateur S5.331, S5.332 | S5.149, S5.282, S5.332, S5.337 |  |  |
| 1300-1350 MHz AERONAUTICAL RADIONAVIGATION S5.337 RADIONAVIGATION- SATELLITE (Es) Radiolocation S5.149 | 1300-1350 MHz AERONAUTICAL RADIONAVIGATION RADIOLOCATION S5.149, S5.337 | 1260-1350 MHz (cont.) |  |  |
| 1350-1400 MHz FIXED MOBILE RADIOLOCATION S5.149, S5.338, S5.339 | 1350-1400 MHz FIXED RADIOLOCATION MOBILE S5.149, S5.339 | 1350-1375 MHz FIXED MOBILE (except aeronautical mobile) Radiolocation S5.149, S5.334, S5.339. | # FIXED SERVICE Civil Fixed terrestrial point to point links, 1350-1375/1492-1517 MHz, in accordance with CEPT Recommendation T/R 13-01. # AERONAUTICAL Airport surveillance radars on secondary basis. 1300-1375 MHz – for use by wind profilers on a case by case, NIB. Radio Astronomy on 1330-1400 MHz, used for study of red-shifted hydrogen lines. Maritime seismic streamer tailbuoys on 1362.5 MHz. | Band is available for low capacity digital fixed links assignments in accordance with CEPT Recommendation T/R 13-01. Protection of existing services and aircraft systems from interference effects. |
| 1350-1400 MHz (cont.) | 1350-1400 MHz (cont.) | 1375-1400 MHz FIXED MOBILE (except aeronautical mobile) S5.149, S5.339 | # MoD 1375-1400/1427-1452 MHz - Used for radio relay equipment, tactical training and security surveillance video and data links, and aeronautical telemetry. # RADIO ASTRONOMY Radio Astronomy on 1330-1400 MHz, used for study of red-shifted hydrogen lines. # SHORT RANGE DEVICES SRDs operate in this band, see Appendix I. Maritime seismic streamer tailbuoys on 1389.5 MHz. | Protect military access for tactical radio relay and fixed telemetry. Phase out use of the band for airborne telemetry. Protect radio astronomy in 1400-1427 MHz. No changes planned. |
| 1400-1427 MHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341 | 1400-1427 MHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341 | 1400-1427 MHz EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341 | # RADIO ASTRONOMY Passive band. Radio Astronomy - used for hydrogen line studies, MERLIN, pulsars and mapping radio sources. | # Support initiatives designed to provide protection to science services in this band. |
| 1427-1429 MHz SPACE OPERATION (Es) FIXED MOBILE except aeronautical mobile S5.341 | 1427-1429 MHz SPACE OPERATION (Es) FIXED MOBILE except aeronautical mobile S5.341 | 1427-1429 MHz SPACE OPERATION (Es) FIXED MOBILE S5.341 | # MoD 1427-1452/1375-1400 MHz - Used for radio relay equipment, tactical training and security surveillance video and data links, and aeronautical telemetry. | Protect military access for tactical radio relay and fixed telemetry. Phase out use of the band for airborne telemetry. Protect radio astronomy in 1400-1427 MHz. |
| 1429-1452 MHz FIXED MOBILE except aeronautical mobile S5.341, S5.342 | 1429-1452 MHz FIXED MOBILE except aeronautical mobile S5.341 | 1429-1452 MHz FIXED MOBILE S5.341 | # MoD 1427-1452/1375-1400 MHz - Used for radio relay equipment, tactical training and security surveillance video and data links, and aeronautical telemetry. # FIXED SERVICE Civil, low to medium capacity, analogue and digital fixed links currently operate within the band 1450-1467.5, paired with MHz 1512.5-1530 MHz. | Protect military access for tactical radio relay and fixed telemetry. Phase out use of the band for airborne telemetry. Protect radio astronomy in 1400-1427 MHz. No new fixed link assignments in this band. Existing fixed links to move by 2007 to accommodate DAB in 1452-1492 MHz. A small number of links may be required to move earlier than 2007. |
| 1452-1492 MHz FIXED MOBILE except aeronautical mobile BROADCASTING S5.345, S5.347 BROADCASTING- SATELLITE S5.345, S5.347 S5.341, S5.342 | 1452-1492 MHz BROADCASTING BROADCASTING- SATELLITE Fixed Mobile except Aeronautical Mobile S5.341, S5.345, S5.347 | 1452-1492 MHz FIXED MOBILE (except aeronautical mobile) BROADCASTING- SATELLITE S5.345 BROADCASTING S5.345 S5.341 | # FIXED SERVICE This band was allocated to BROADCASTING-SATELLITE at WARC-92 but in the UK, and in a number of other countries, this will remain a secondary service until 1 April 2007. Civil, low to medium capacity, analogue and digital fixed links currently operate within the band 1450-1467.5, paired with MHz 1512.5-1530 MHz. # BROADCASTING 9 blocks of 1.5 MHz have been made available for terrestrial using Eureka 147 technology. # PMSE 1488-1491 MHz used by Audio links. | No new fixed link assignments in this band. Existing fixed links to move by 2007 to accommodate DAB. A small number of links may be required to move earlier than 2007. Government will consider future use of an additional 14 blocks of spectrum in this band taking into account of future international discussions (see Chapter 2.3). No changes planned. |
| 1492-1525 MHz FIXED MOBILE except aeronautical mobile S5.341, S5.342 | 1492-1517 MHz FIXED MOBILE except aeronautical mobile S5.341 | 1492-1517 MHz FIXED MOBILE (except aeronautical mobile) S5.341, S5.348, S5.348A | # FIXED SERVICE Fixed terrestrial point to point links, 1492.5-1517 MHz/1350-1374.5 MHz, in accordance with CEPT Recommendation T/R 13-01. Civil, low to medium capacity, analogue and digital fixed links currently operate within the band 1512.5-1530 MHz, paired with 1450-1467.5MHz | Band is available for low capacity digital fixed links assignments in accordance with CEPT Recommendation T/R 13-01. No new fixed link assignments in this band on channel plan, 1512.5-1530 MHz/1450-1467.5 MHz. Existing fixed links to move by 2007 to accommodate DAB in 1452-1492 MHz. A small number of links may be required to move earlier than 2007. |
|  | 1517-1525 MHz FIXED MOBILE except aeronautical mobile S5.341 | 1517-1525 MHz FIXED MOBILE (except aeronautical mobile) S5.341, S5.348, S5.348A | # FIXED SERVICE Civil, low to medium capacity, analogue and digital fixed links currently operate within the band 1512·5-1530 MHz, paired with 1450-1467.5 MHz. # PMSE Audio links operate in this band. # MOBILE SATELLITE SERVICE | Consider possible future requirement for this band for temporary unidirectional links and possible permanent studio to transmitter stereo broadcast links. No new fixed link assignments in this band. Existing fixed links to move by 2007. A small number of links may be required to move earlier than 2007. No changes planned. The Agency is supporting additional MSS spectrum requirements and is identifying and pursuing, through CEPT, suitable allocations to satisfy the WRC-03 agenda item 1.31. The target bands at present are 1518-1525 MHz (space-to-Earth) and 1683-1690/1670-1675 MHz (Earth-to-space), which are particularly suitable as MSS expansion bands, being close to the existing L-band allocations. |
| 1525-1530 MHz SPACE OPERATION (sE) FIXED MOBILE-SATELLITE (sE) S5.351A Earth exploration-satellite Mobile except aeronautical mobile S5.349 S5.341, S5.342, S5.350, S5.351, S5.352A, S5.354 | 1525-1530 MHz SPACE OPERATION (sE) FIXED MOBILE-SATELLITE (sE) S5.341, S5.351, S5.354 | 1525-1530 MHz SPACE OPERATION (sE) FIXED MOBILE-SATELLITE (sE) Earth Exploration-Satellite Mobile (except aeronautical mobile) S5·341, S5·351, S5.352A, S5.354 | # FIXED SERVICE Civil, low to medium capacity, analogue and digital fixed links currently operate within the band 1512.5-1530 MHz, paired with 1450-1467.5 MHz. # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. | No new fixed link assignments in this band. Existing fixed links to move by 2007. A small number of links may be required to move earlier than 2007. Facilitate the introduction of new MSS systems through multilateral co-ordinating and discussions |
| 1530-1535 MHz SPACE OPERATION (sE) MOBILE-SATELLITE (sE) S5.351A, S5.353A Earth exploration-satellite Fixed Mobile except aeronautical mobile S5.341, S5.342, S5.351, S5.354 | 1530-1533 MHz SPACE OPERATION (sE) MOBILE-SATELLITE (sE) Earth Exploration-Satellite Fixed Mobile except aeronautical mobile S5.341, S5.351, S5.353A, S5.354 | 1530-1533 MHz SPACE OPERATION (sE) MOBILE –SATELLITE (sE) Earth Exploration Satellite Fixed Mobile (except aeronautical mobile) S5.341, S5.351, S5.353A, S5.354 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
| 1530-1535 MHz (cont.) | 1533-1535 MHz SPACE OPERATION (sE) MOBILE-SATELLITE (sE) Earth Exploration Satellite Mobile except aeronautical mobile S5.341, S5.351, S5.353A, S5.354 | 1533-1535 MHz SPACE OPERATION (sE) MOBILE-SATELLITE (sE) Earth Exploration-Satellite Fixed Mobile (except aeronautical mobile) S5.341, S5.351, S5.353A, S5.354 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
| 1535-1559 MHz MOBILE-SATELLITE (sE) S5.351A | 1535-1544 MHz MOBILE-SATELLITE (sE) S5.341, S5.351, S5.353A, S5.354 | 1535-1544 MHz MOBILE-SATELLITE (sE) Land mobile-satellite (sE) S5.341, S5.351, S5.353A, S5.354 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
| S5.341, S5.351, S5.353A, S5.354, S5.355, S5.356, S5.357, S5.357A, S5.359, S5.362A | 1544-1545 MHz MOBILE-SATELLITE (sE) S5.341, S5.354, S5.356 | 1544-1545 MHz MOBILE SATELLITE (sE) S5.341, S5.354, S5.356 | # MOBILE SATELLITE Mobile-Satellite distress and safety communications. | Ensure the use of the band 1544-1545 MHz by MMS (sE) is limited to distress and safety communications. |
| 1535-1559 MHz (cont.) | 1545-1555 MHz MOBILE-SATELLITE (sE) S5.341, S5.351, S5.354, S5.357, S5.357A, S5.359 | 1545-1555 MHz MOBILE SATELLITE (sE) S5.341,S5.351, S5.354, S5.357, S5.357A | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. Public correspondence with aircraft via satellite may use this band (paired with 1646.5-1656.5 MHz). | Ensure continued protection for distress and safety communications in AMS(R ) use required. Facilitate the introduction of new MSS systems through multilateral co-ordinating and discussions. |
|  | 1555-1559 MHz MOBILE-SATELLITE (sE) S5.341, S5.351, S5.354, S5.359 | 1555-1559MHz MOBILE SATELLITE (sE) S5.341, S5.351, S5.354 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. | Facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
| 1559-1610 MHz AERONAUTICAL RADIONAVIGATION RADIONAVIGATION- SATELLITE (sE) (ss) S5.329A S5.341, S5.355, S5.359, S5.363 | 1559-1610 MHz AERONAUTICAL RADIONAVIGATION RADIONAVIGATION SATELLITE (sE) S5.341, S5.359 | 1559-1610 MHz AERONAUTICAL RADIONAVIGATION RADIONAVIGATION- SATELLITE (sE) S5.341 | # AERONAUTICAL/MoD 1559-1610 MHz – Allocated to aeronautical radionavigation services and provides an assignment at 1575.42 MHz for the primary downlink of GPS. | Protect aeronautical radionavigation use. |
| 1610-1610.6 MHz MOBILE-SATELLITE (Es) S5.351A AERONAUTICAL RADIONAVIGATION S5.341, S5.355, S5.359, S5.363, S5.364, S5.366, S5.367, S5.368, S5.369, S5.371, S5.372 | 1610-1610.6 MHz AERONAUTICAL RADIONAVIGATION MOBILE-SATELLITE (Es) S5.341, S5.359, S5.364, S5.366, S5.367, S5.368, S5.371, S5.372 | 1610-1610.6 MHz AERONAUTICAL RADIONAVIGATION MOBILE-SATELLITE (Es) S5.149, S5.341, S5.364, S5.366, S5.367, S5.368, S5.371 |  | No changes planned. |
| 1610.6-1613.8 MHz MOBILE-SATELLITE (Es) S5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION S5.149, S5.341, S5.355, S5.359, S5.363, S5.364, S5.366, S5.367, S5.368, S5.369, S5.371, S5.372 | 1610.6-1613.8 MHz AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Es) RADIO ASTRONOMY S5.149, S5.341, S5.359, S5.364, S5.366, S5.367, S5.368, S5.371, S5.372 | 1610.6-1613.8 MHz AERONAUTICAL RADIONAVIGATION MOBILE-SATELLITE (Es) RADIO ASTRONOMY S5.149, S5.341, S5.364, S5.366, S5.367, S5.368, S5.371, S5.372 | Radio Astronomy on 1610.6-1613.8 MHz. Used for MERLIN and hydroxyl line studies. | # Protect Radio Astronomy |
| 1613.8-1626.5 MHz MOBILE-SATELLITE (Es) S5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (sE) S5.341, S5.355, S5.359, S5.363, S5.364, S5.365, S5.366, S5.367, S5.368, S5.369, S5.371, S5.372 | 1613.8-1626.5 MHz AERONAUTICAL RADIONAVIGATION MOBILE SATELLITE (Es) Mobile satellite (sE) S5.341, S5.359, S5.364, S5.365, S5.366, S5.367, S5.368, S5.371, S5.372, S5.373 | 1613.8-1626.5 MHz AERONAUTICAL RADIONAVIGATION MOBILE-SATELLITE (Es) Mobile-Satellite (sE) S5.341, S5.364, S5.365, S5.366, S5.367, S5.368, S5.371 | # MOBILE SATELLITE NGSO MSS (sE and Es) systems (Iridium and Globalstar). AERONAUTICAL Worldwide aeronautical radionavigation band including associated ground or satellite based facilities. | Protection of the Radio Astronomy service required. No changes planned in the medium term. |
| 1626.5-1660 MHz MOBILE-SATELLITE (Es) S5.351A | 1626.5-1631.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.353A, S5.354, S5.359 | 1626.5-1631.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.353A, S5.354 | # MOBILE SATELLITE Up link for Maritime Mobile Satellite terminals. INMARSAT is an International Mobile Satellite service provider in this band. | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
|  | 1631.5-1636.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.353A, S5.354, S5.359, S5.374 | 1631.5-1636.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.353A, S5.354, S5.374 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
|  | 1636.5-1645.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.353A, S5.354, S5.359 | 1636.5-1645.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.353A, S5.354 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. Public correspondence with aircraft via satellite may use this band (paired with 1545-1555 MHz). | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
| S5.341, S5.351, S5.353A, S5.354, S5.355, S5.357A, S5.359, S5.362A, S5.374, S5.375, S5.376 | 1645.5-1646.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.354, S5.375 | 1645.5-1646.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.354, S5.375 | Up link for Mobile Satellite terminals. Distress and Safety operations. Public correspondence with aircraft via satellite may use this band (paired with 1545-1555 MHz). | # No changes planned. |
| 1626.5-1660 MHz (cont.) | 1646.5-1656.5 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.354, S5.359, S5.357A, S5.376 | 1646.5-1656.5 MHz MOBILE-SATELLITE (ES) S5.341, S5.351, S5.354, S5.357A, S5.376 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. Public correspondence with aircraft via satellite may use this band (paired with 1545-1555 MHz). AMS(R)S operates in the band 1646.5-1656.5 MHz. | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
|  | 1656.5-1660 MHz MOBILE-SATELLITE (Es) S5.341 S5.351 S5.354 S5.355 S5.359 S5.374 | 1656.5-1660 MHz MOBILE-SATELLITE (Es) S5.341, S5.351, S5.354, S5.374 | # MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. Up link for mobile satellite terminals. | Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. |
| 1660-1660.5 MHz MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY S5.149, S5.341, S5.351, S5.354, S5.362A, S5.376A | 1660-1660.5 MHz MOBILE-SATELLITE (Es) RADIO ASTRONOMY S5.149, S5.341, S5.351, S5.354, S5.376A | 1660-1660.5 MHz MOBILE-SATELLITE (Es) RADIO ASTRONOMY S5.149, S5.341, S5.351, S5.354, S5.376A | MOBILE SATELLITE INMARSAT is an International Mobile Satellite service provider in this band. # RADIO ASTRONOMY Used for MERLIN and hydroxyl line studies. | # Ensure continued protection to GMDSS and facilitate the introduction of new MSS systems through multilateral co-ordination and discussions. Protect Radio Astronomy. |
| 1660.5-1668.4 MHz RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical Mobile | 1660.5-1668.4 MHz RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical Mobile | 1660.5-1668 MHz RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile S5.149, S5.341, S5.379A | # RADIO ASTRONOMY Used for MERLIN and hydroxyl line studies. | Protect Radio Astronomy. |
| S5.149, S5.341, S5.379, S5.379A | S5.149, S5.341, S5.379A | 1668-1670 MHz FIXED RADIO ASTRONOMY SPACE RESEARCH (passive) | # RADIO ASTRONOMY Used for MERLIN and hydroxyl line studies. Limited protection from co-primary service. # HO/SE HO/SE 1668-1670 MHz – Fixed links, paired with 1698-1700 MHz. | Protect Radio Astronomy No changes planned. |
| 1668.4-1670 MHz METEOROLOGICAL AIDS FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY S5.149, S5.341 | 1668.4-1670 MHz METEOROLOGICAL AIDS FIXED RADIO ASTRONOMY Mobile except aeronautical mobile S5.149, S5.341 | S5.149, S5.341, S5.379A |  |  |
| 1670-1675 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (sE) MOBILE S5.380 S5.341 | 1670-1675 MHz METEOROLOGICAL AIDS METEOROLOGICAL SATELLITE (sE) MOBILE Fixed S5.341, S5.380 | 1670-1675 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL SATELLITE (sE) MOBILE S5.341, S5.380 | # FIXED SERVICE Civil fixed links currently on old frequency plan 1671-1676 MHz and 1686-1690 MHz. # MOBILE TFTS (Ground Station) at 1670-1672 MHz and 1673-1675 MHz. MoD Meteorological sondes for the meteorological Office and Army artillery units operate in the band 1670-1690 MHz. # MOBILE SATELLITE SERVICE Meteorological satellites operate on 1670.5 and 1671.5 MHz. Meteorological Aids operate between 1670-1672 MHz. | No new fixed link assignments in this band. Clearance of fixed links by 2007. Future allocations of TFTS to be examined by CEPT by end of 2002 in the light of minimal commercial use of this service. No changes planned. The Agency is supporting additional MSS spectrum requirements and is identifying and pursuing, through CEPT, suitable allocations to satisfy the WRC-03 agenda item 1.31. The target bands at present are 1518-1525 MHz (space-to-Earth) and 1683-1690/1670-1675 MHz (Earth-to-space), which are particularly suitable as MSS expansion bands, being close to the existing L-band allocations. |
| 1675-1690 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (sE) MOBILE except aeronautical Mobile S5.341 | 1675-1690 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL SATELLITE (sE) MOBILE except Aeronautical Mobile S5.341 | 1675-1690 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL SATELLITE (sE) MOBILE except aeronautical Mobile S5.341, S5.377 | # FIXED SERVICE Civil fixed links currently on old frequency plan 1671-1676 MHz and 1686-1690 MHz. # MoD Meteorological sondes for the meteorological Office and Army artillery units operate in the band 1670-1690 MHz. # HO/SE 1677-1685 MHz – Used for mobile applications. # MOBILE SATELLITE SERVICE | No new fixed link assignments in this band. Clearance of fixed links by 2007. Ensure sufficient bandwidth for meteorological sondes. No changes planned. The Agency is supporting additional MSS spectrum requirements and is identifying and pursuing, through CEPT, suitable allocations to satisfy the WRC-03 agenda item 1.31. The target bands at present are 1518-1525 MHz (space-to-Earth) and 1683-1690/1670-1675 MHz (Earth-to-space), which are particularly suitable as MSS expansion bands, being close to the existing L-band allocations. |
| 1690-1700 MHz METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (sE) Fixed Mobile except aeronautical Mobile S5.289, S5.341, S5.382 | 1690-1700 MHz METEOROLOGICAL AIDS METEOROLOGICAL SATELLITE (sE) Fixed Mobile except Aeronautical Mobile S5.341 S5.382 | 1690-1698 MHz METEOROLOGICAL- SATELLITE (sE) Fixed Land Mobile S5.289, S5.341, S5.377 | METEOROLOGICAL- SATELLITE (sE) Used by the meteorological satellite service. # MoD 1690-1710 MHz – Meteorological satellite downlink. | No changes planned. |
|  |  | 1698-1700 MHz METEOROLOGICAL SATELLITE (sE) FIXED S5.289, S5.341, S5.377 | METEOROLOGICAL SATELLITE (sE) Used by the meteorological satellite service. # MoD 1690-1710 MHz – Meteorological satellite downlink. # HO/SE 1698-1700 MHz – Fixed links, paired with 1668-1670 MHz. | No changes planned. |
| 1700-1710 MHz FIXED METEOROLOGICAL- SATELLITE (sE) MOBILE except aeronautical mobile S5.289, S5.341 | 1700-1710 MHz FIXED METEOROLOGICAL SATELLITE (sE) Mobile except Aeronautical Mobile S5.341 | 1700-1710 MHz FIXED METEOROLOGICAL SATELLITE (sE) S5.289, S5.341, S5.377 | METEOROLOGICAL SATELLITE (sE) Used by the meteorological satellite service. # MoD 1690-1710 MHz – Meteorological satellite downlink. | No changes planned. |
| 1710-1930 MHz FIXED MOBILE S5.380 S5.149, S5.341, S5.385, S5.386, S5.387, S5.388 | 1710-1785 MHz FIXED MOBILE S5.149, S5.341, S5.385 | 1710-1785 MHz FIXED MOBILE Space Operations (Es) S5.149, S5.341, S5.385 | # PUBLIC NETWORKS GSM 1800 (base receive) band at 1710-1785 MHz, paired with 1805-1880 MHz # FIXED SERVICE Some residual FIXED services between 1710 and 1750 MHz. Offshore (terrestrial point to point and transhorizon) fixed links on old frequency plan 1764-1792 MHz and 1872-1900 MHz. # MoD Satellite Telecommand, Telemetry and Control (TT&C) are provided for the SKYNET satellites at 1760-1840 MHz for the command uplinks and 2200-2290 MHz for the telemetry downlink. # RADIO ASTRONOMY Radio Astronomy. Used for MERLIN and hydroxyl line studies. | # Potential re-farming of band for IMT-2000 in the longer term. 1st consultation on additional IMT-2000 spectrum and re-farming for 2002. No new fixed link assignments in this band (point to point or transhorizon). Point to point links will be relocated as necessary to the 1.4 GHz band or higher where appropriate. For transhorizon links see comments under Fixed Service 1900-1980 MHz. No changes planned. |
| 1710-1930 MHz (cont.) | 1785-1800 MHz FIXED MOBILE | 1785-1800 MHz FIXED MOBILE Space Operation (Es) | # FIXED SERVICE Offshore (terrestrial point to point and transhorizon) fixed links on old frequency plan, 1764-1792 MHz/1872-1900 MHz. # PMSE Radiomicrophones operate in this band | No new fixed link assignments in this band (point to point or transhorizon). Point to point links will be relocated as necessary to the 1.4 GHz band or higher where appropriate. For transhorizon links see comments under Fixed Service 1900-1980 MHz. No changes planned. |
|  | 1800-1805 MHz MOBILE Fixed S5.380 | 1800-1805 MHz MOBILE Fixed Space Operation (Es) S5.380 | # MOBILE TFTS (aircraft stations) at 1800-1805 MHz. | Future allocations of TFTS to be examined by CEPT by end of 2002 in the light of minimal commercial use of this service. |
|  | 1805-1880 MHz FIXED MOBILE | 1805-1880 MHz MOBILE Fixed Space Operation (Es) | PUBLIC NETWORKS GSM 1800 (base transmit) band at 1805-1880 MHz, paired with 1710-1785 MHz. # FIXED SERVICE Offshore (terrestrial point to point and transhorizon) fixed links on old frequency plan 1872-1900 MHz and 1764-1792 MHz. | # Potential re-farming of band for IMT-2000 in the longer term. 1st consultation on additional IMT-2000 spectrum and re-farming for 2002. No new fixed link assignments in this band (point to point or transhorizon). Point to point links will be relocated as necessary to the 1.4 GHz band or higher where appropriate. For transhorizon links see comments under Fixed Service 1900-1980 MHz. |
| 1710-1930 MHz (cont.) | 1880-1885 MHz MOBILE Fixed | 1880-1885MHz FIXED MOBILE | # FIXED SERVICE Offshore (terrestrial point to point and transhorizon) fixed links on old frequency plan 1872-1900 MHz and 1764-1792 MHz. # MOBILE DECT on 1880-1900 MHz. | No new fixed link assignments in this band (point to point or transhorizon). Point to point links will be relocated as necessary to the 1.4 GHz band or higher where appropriate. For transhorizon links see comments in FS 1900-1980 MHz. Band will remain exclusively for services based on the DECT standard. If band is opened up for public services following review of deregulated bands, management strategy will need to be reviewed. |
|  | 1885-1900 MHz MOBILE Fixed S5.388 | 1885-1900 MHz FIXED MOBILE S5.388 | # FIXED SERVICE Offshore (terrestrial point to point and transhorizon) fixed links on old frequency plan 1872-1900 MHz and 1764-1792 MHz. # MOBILE DECT on 1880-1900 MHz. | No new fixed link assignments in this band (point to point or transhorizon). Point to point links will be relocated as necessary to the 1.4 GHz band or higher where appropriate. For transhorizon links see comments under Fixed Service 1900-1980 MHz. Band will remain exclusively for services based on the DECT standard. If band is opened up for public services following review of deregulated bands, management strategy will need to be reviewed. |
| 1710-1930 MHz (cont.) | 1900-1980 MHz FIXED MOBILE S5.388 | 1900-1980 MHz FIXED MOBILE S5.388 | # PUBLIC NETWORKS IMT-2000 band 1900-1920 MHz TDD licensed. IMT-2000 band 1920-1980 MHz FDD mobile transmit, paired with 2110-2170 MHz. # FIXED SERVICE | # Five operators including 1 new entrant licensed for IMT-2000 services following spectrum auctions in early 2000. Commercial services expected to be launched late 2002 or early 2003. No new point to point or transhorizon fixed links. Any existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators to ensure that all transhorizon links are removed as soon as possible. |
| 1930-1970 MHz FIXED MOBILE S5.388 |  |  |  |  |
| 1970-1980 MHz FIXED MOBILE S5.388 |  |  |  |  |
| 1980-2010 MHz FIXED MOBILE MOBILE-SATELLITE (Es) S5.388, S5.389A, S5.389B, S5.389F | 1980-2010 MHz FIXED MOBILE MOBILE-SATELLITE (Es) S5.388, S5.389A | 1980-2010 MHz FIXED MOBILE MOBILE SATELLITE (Es) S5.388, 5.389A, S5.389B | # FIXED SERVICE Fixed terrestrial point to point and links. | No new point to point transhorizon fixed links. Existing point to point links to be removed. |
| 2010-2025 MHz FIXED MOBILE S5.388 | 2010-2025 MHz FIXED MOBILE S5.388 | 2010-2025 MHz FIXED MOBILE S5.388 | # PUBLIC NETWORKS 2010-2025 MHz identified for IMT-2000 usage for self provided applications. # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. | # The Agency proposes to make this band available for UMTS TDD self provided applications operating in a self co-ordinating mode on a mainly licence-exempt basis. The regulatory regime adopted will take account of the outcome of the October 2001 consultation on the use of licence exempt spectrum for the provision of public telecommunications services. The timescale for making this spectrum available will depend on progress on international standards for the TDD technology employed. No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. |
| 2025-2110 MHz SPACE OPERATION (Es) (ss) EARTH EXPLORATION- SATELLITE (Es) (ss) FIXED MOBILE S5.391 SPACE RESEARCH (Es) (ss) S5.392 | 2025-2110 MHz FIXED MOBILE SPACE RESEARCH (Es) (ss) SPACE OPERATION (Es) (ss) EARTH EXPLORATION SATELLITE (Es) (ss) S5.391, S5.392 | 2025-2110 MHz FIXED MOBILE SPACE RESEARCH (Es) (ss) SPACE OPERATION (Es) (ss) EARTH EXPLORATION SATELLITE (Es) (ss) S5.391, S5.392 | # FIXED WIRELESS ACCESS Fixed Wireless Access (FWA) systems operate, in rural areas, in the band 2025-2110 MHz, paired with 2200-2290 MHz. Two FWA licences, each 2x 38.5 MHz. MoD # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. # SCIENCE SERVICES Metosat Data Dissemination uplink band | One FWA licensee made the decision not to deploy services in this band and the RA expects to issue a consultation document shortly on possible uses of this spectrum. To consider the possible provision of spectrum for Tactical Radio Relay in order to harmonise with NATO use. No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. No changes planned. |
| 2110-2120 MHz FIXED MOBILE SPACE RESEARCH (deep space) (Es) S5.388 | 2110-2120 MHz FIXED MOBILE SPACE RESEARCH (Es) (deep space) S5.388 | 2110-2120 MHz FIXED MOBILE SPACE RESEARCH (Es) (deep space) S5.388 | # PUBLIC NETWORKS IMT-2000 band 2110-2170 MHz FDD base transmit, paired with 1920-1980 MHz mobile transmit. # 2110-2170 MHz licensed for UMTS (base transmit). # FIXED SERVICE | # Five operators including 1 new entrant licensed for IMT-2000 services following spectrum auctions in early 2000. Commercial services expected to be launched late 2002 or early 2003. No new point to point or transhorizon fixed links. Any existing point to point links removed or to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. |
| 2120-2160 MHz FIXED MOBILE S5.388 | 2120-2170 MHz FIXED MOBILE | 2120-2170 MHz FIXED MOBILE | # PUBLIC NETWORKS IMT-2000 band 2110-2170 MHz FDD base transmit, paired with 1920-1980 MHz mobile transmit. 2110-2170 MHz licensed for UMTS (base transmit). # FIXED SERVICE | # Five operators including 1 new entrant licensed for IMT-2000 services following spectrum auctions in early 2000. Commercial services expected to be launched late 2002 or early 2003. No new point to point or transhorizon fixed links. Any existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. |
| 2160-2170 MHz FIXED MOBILE S5.388, S5.392A | 2120-2170 MHz (Cont.) S5.388 | 2120-2170 MHz (Cont.) S5.388 |  |  |
| 2170-2200 MHz FIXED MOBILE MOBILE-SATELLITE (sE) S5.388, S5.389A, S5.389F, S5.392A | 2170-2200 MHz FIXED MOBILE MOBILE-SATELLITE (sE) S5.388, S5.389A | 2170-2200 MHz FIXED MOBILE MOBILE-SATELLITE (sE) S5.388, S5.389A | # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. | No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. |
| 2200-2290 MHz SPACE OPERATION (sE) (ss) EARTH EXPLORATION- SATELLITE (sE) (ss) FIXED MOBILE S5.391 SPACE RESEARCH (sE) (ss) S5.392 | 2200-2290 MHz SPACE OPERATION (sE) (ss) EARTH EXPLORATION- SATELLITE (sE) (ss) FIXED MOBILE SPACE RESEARCH (sE) (ss) S5.391, S5.392 | 2200-2290 MHz FIXED MOBILE SPACE RESEARCH (sE) (ss) SPACE OPERATION (sE) (ss) EARTH EXPLORATION- SATELLITE (sE) (ss) S5.391, S5.392 | # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. # FIXED WIRELESS ACCESS Fixed Wireless Access (FWA) systems operate, in rural areas, in the band, 2200-2290 MHz paired with 2025-2110 MHz. Two FWA licences, each 2x 38.5 MHz. MoD Satellite Telecommand, Telemetry and Control (TT&C) is provided for the SKYNET satellites at 2200-2290 MHz for the telemetry downlink and 1760-1840 MHz for the command uplinks. | No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until such further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. One FWA licensee made the decision not to deploy services in this band and the RA expects to issue a consultation document shortly on possible uses of this spectrum. To consider the possible provision of spectrum for Tactical Radio Relay in order to harmonise with NATO use. |
| 2290-2300 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (deep space) (sE) | 2290-2300 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (deep space ) (sE) | 2290-2300 MHz FIXED MOBILE Space Research (sE) (deep space) | # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. | No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible.. |
| 2300-2450 MHz FIXED MOBILE Amateur Radiolocation | 2300-2400 MHz FIXED MOBILE Amateur Radiolocation | 2300-2302 MHz FIXED MOBILE | # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. | No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until such further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. |
| S5.150, S5.282, S5.395 | S5.150 | 2302-2310 MHz FIXED | HO/SE Fixed use | # No changes planned |
| 2300-2450 MHz (cont.) | 2300-2400 MHz (cont.) | 2310-2450 MHz FIXED MOBILE Amateur Amateur-Satellite Radiolocation | # FIXED WIRELESS ACCESS Amateur secondary at 2310-2450 MHz using powers of up to 26 dBW for many modes including packet, TV, Morse, etc. # Amateur Amateur-Satellite on 2400-2450 MHz. PMSE 2390-2690 MHz used for Temporary point-to-point video links; Radcameras and Mobile and portable video links (20 MHz channels). # MoD Military use including fixed, telemetry and mobile data and video links. # HO/SE Used for Video Surveillance Links # SHORT RANGE DEVICES SRDs, including WBDTS, operate in this band, see Appendix I. ISM at 2400-2500 MHz. | The Agency has no plans for further FWA licences in this band at present. Licence exempt bands are under review and a consultation document has been issued. No changes planned. Future frequency allocations for all PMSE services currently operating between 2390-2690 will need to be reassessed in the light of the allocation of 2500-2690 to IMT-2000 at WRC-2000. No changes planned. No changes planned. No changes planned. |
|  | 2400-2450 MHz FIXED MOBILE Amateur Amateur satellite S5.150 S5.282 | S5.150, S5.282, S5.396 |  |  |
| 2450-2483.5 MHz FIXED MOBILE Radiolocation S5.150, S5.397 | 2450-2483.5 MHz FIXED MOBILE S5.150 | 2450-2483.5 MHz FIXED MOBILE Radiolocation S5.150 | # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. # FIXED WIRELESS ACCESS # PMSE 2390-2690 MHz used for Temporary point-to-point video links; Radcameras and Mobile and portable video links (20 MHz channels). # SHORT RANGE DEVICES SRDs, including WBDTS, operate in this band, see Appendix I. ISM at 2400-2500 MHz. | No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until such further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. # The Agency has no plans for further FWA licences in this band at present. Licence exempt bands are under review and a consultation document will be issued in due course. Future frequency allocations for all PMSE services currently operating between 2390-2690 will need to be reassessed in the light of the allocation of 2500-2690 to IMT-2000 at WRC-2000. No changes planned. |
| 2483.5-2500 MHz FIXED MOBILE MOBILE-SATELLITE (sE) Radiolocation S5.150, S5.371, S5.397, S5.398, S5.399, S5.400, S5.402 | 2483.5-2500 MHz FIXED MOBILE MOBILE-SATELLITE (sE) S5.150, S5.371, S5.397, S5.402 | 2483.5-2500 MHz FIXED MOBILE (except aeronautical mobile) MOBILE-SATELLITE (sE) Radiolocation S5.150, S5.371, S5.397, S5.398, S5.399, S5.402 | # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. # PMSE 2390-2690 MHz used for Temporary point-to-point video links; Radcameras and Mobile and portable video links (20 MHz channels). | No new point to point or transhorizon fixed links. Existing point to point links removed or to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band – see band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. Future frequency allocations for all PMSE services currently operating between 2390-2690 will need to be reassessed in the light of the allocation of 2500-2690 to IMT-2000 at WRC-2000. |
| 2500-2520 MHz FIXED S5.409, S5.410, S5.411 MOBILE except aeronautical mobile MOBILE-SATELLITE (sE) S5.403 S5.405, S5.407, S5.408, S5.412, S5.414 | 2500-2520 MHz MOBILE- SATELLITE (sE) Fixed Mobile except aeronautical mobile S5.403, S5.414 | 2500-2520 MHz FIXED S5.409, S5.410, S5.411 MOBILE (except aeronautical mobile) Radiolocation MOBILE SATELLITE (sE) S5.403, S5.407, S5.408, S5.414 | PUBLIC NETWORKS IMT-2000 band 2500-2690 MHz identified at WRC-2000. # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. # PMSE 2390-2690 MHz used for Temporary point-to-point video links; Radcameras and Mobile and portable video links (20 MHz channels). # RADIOLOCATION UK Radiolocation secondary on 2500-2600 MHz. | Primary band for additional IMT-2000 spectrum within Europe.1st consultation on additional IMT-2000 spectrum and re-farming planned for 2002. Work ongoing in CEPT in response to European Commission mandate to ensure band is made available in a harmonised manner. CEPT Decision required by March 2003. Current users will need to be migrated from the band. No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible.. Future frequency allocations for all PMSE services currently operating between 2390-2690 will need to be reassessed in the light of the allocation of 2500-2690 to IMT-2000 at WRC-2000. No changes planned. |
| 2520-2655 MHz FIXED S5.409, S5.410, S5.411 MOBILE except aeronautical mobile BROADCASTING- SATELLITE S5.413, S5.416 S5.339, S5.403, S5.405, S5.408, S5.412, S5.417, S5.418 | 2520-2655 MHz FIXED MOBILE except aeronautical mobile S5.339, S5.403 | 2520-2655 MHz FIXED S5.409, S5.410, S5.411 MOBILE (except aeronautical mobile) BROADCASTING- SATELLITE S5.413, S5.416 Radiolocation S5.339, S5.403, S5.408 | PUBLIC NETWORKS IMT-2000 band 2500-2690 MHz identified at WRC-2000. # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. # PMSE 2390-2690 MHz used for Temporary point-to-point video links; Radcameras and Mobile and portable video links (20 MHz channels). # RADIOLOCATION UK Radiolocation secondary on 2500-2600 MHz. | Primary band for additional IMT-2000 spectrum within Europe.1st consultation on additional IMT-2000 spectrum and re-farming planned for 2002. Work ongoing in CEPT in response to European Commission mandate to ensure band is made available in a harmonised manner. CEPT Decision required by March 2003.Current users would need to be migrated from the band. No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. Future frequency allocations for all PMSE services currently operating between 2390-2690 will need to be reassessed in the light of the allocation of 2500-2690 to IMT-2000 at WRC-2000. No changes planned. |
| 2655-2670MHz FIXED S5.409, S5.410, S5.411 MOBILE except aeronautical mobile BROADCASTING- SATELLITE S5.413, S5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) S5.149, S5.412, S5.417, S5.420 | 2655-2670 MHz FIXED MOBILE except aeronautical mobile Earth exploration satellite (passive) Radio astronomy Space research (passive) S5.149 S5.420 | 2655-2670 MHz FIXED S5.409, S5.410, S5.411 MOBILE BROADCASTING- SATELLITE S5.413, S5.416 Earth Exploration-Satellite (passive) Space Research (passive) Radio Astronomy S5.149, S5.420 | PUBLIC NETWORKS IMT-2000 band 2500-2690 MHz identified at WRC-2000. # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. PMSE 2390-2690 MHz used for Temporary point-to-point video links, Radcameras and Mobile and portable video links. RADIO ASTRONOMY Radio Astronomy. Used for mapping and pulsars. | Primary band for additional IMT-2000 spectrum within Europe. 1st consultation on additional IMT-2000 spectrum and re-farming planned for 2002. Work ongoing in CEPT in response to European Commission mandate to ensure band is made available in a harmonised manner. CEPT Decision required by March 2003. Current users will need to be migrated from the band. No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. Future frequency allocations for all PMSE services currently operating between 2390-2690 will need to be reassessed in the light of the allocation of 2500-2690 to IMT-2000 at WRC-2000. No changes planned. |
| 2670-2690 MHz FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (Es) Earth exploration-satellite (passive) Radio astronomy Space research (passive) S5.409, S5.410, S5.411, S5.149, S5.419, S5.420 | 2670-2690 MHz MOBILE-SATELLITE (Es) Fixed Mobile except aeronautical mobile Radio Astronomy S5.149 S5.419 S5.420 | 2670-2690 MHz FIXED MOBILE (except aeronautical mobile) MOBILE-SATELLITE (Es) Earth Exploration-Satellite (passive) Space Research (passive) Radio Astronomy S5.149, S5.409, S5.410, S5.411, S5.419, S5.420 | PUBLIC NETWORKS IMT-2000 band 2500-2690 MHz identified at WRC-2000. # FIXED SERVICE Fixed terrestrial point to point and transhorizon links. # PMSE 2390-2690 MHz used for Temporary point-to-point video links, Radcameras and Mobile and portable video links. RADIO ASTRONOMY Radio Astronomy. Used for mapping and pulsars. | Primary band for additional IMT-2000 spectrum within Europe. 1st consultation on additional IMT-2000 spectrum and re-farming planned for 2002. Work ongoing in CEPT in response to European Commission mandate to ensure band is made available in a harmonised manner. CEPT Decision required by March 2003. Current users will need to be migrated from the band. No new point to point or transhorizon fixed links. Existing point to point links to be removed. Existing transhorizon links in the 1 & 2 GHz bands (excluding the 1980-2010 MHz band) will be allowed to continue operations until further notice. Negotiations are currently underway with operators that all transhorizon links are removed as soon as possible. Future frequency allocations for all PMSE services currently operating between 2390-2690 will need to be reassessed in the light of the allocation of 2500-2690 to IMT-2000 at WRC-2000. No changes planned. |
| 2690-2700 MHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.421, S5.422 | 2690-2700 MHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 2690-2700 MHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | # RADIO ASTRONOMY Passive band– used for mapping and pulsars. | No changes planned. |
| 2700-2900 MHz AERONAUTICAL RADIONAVIGATION S5.337 Radiolocation S5.423, S5.424 | 2700-2900 MHz AERONAUTICAL RADIONAVIGATION Radiolocation S5.337, S5.423 | 2700-2900 MHz AERONAUTICAL RADIONAVIGATION S5.337 Radiolocation S5.423 | MoD/DAP Airfield, Surveillance and Traffic Control Radars and associated airborne transponders. | Consideration being given within CEPT to limited sharing with low power cameras. |
| 2900-3100 MHz RADIONAVIGATION S5.426 Radiolocation S5.425, S5.427 | 2900-3100 MHz RADIONAVIGATION Radiolocation S5.425, S5.426, S5.427 | 2900-3100 MHz AERONAUTICAL RADIONAVIGATION S5.426 MARITIME RADIONAVIGATION Radiolocation S5.425, S5.427 | # RADIOLOCATION Civil, Maritime, Air Traffic Control, Naval and Range safety Radars. # MARITIME Shipborne interrogator-transponder system limited to 2930-2950 MHz. | UK to support upgrade of the Radiolocation service to primary status at WRC-03. |

Attachment 2

QUESTIONNAIRE - PART II

(To be completed by Administrations only)

General Questions on National Spectrum Management

The following general questions on national spectrum management are based in part on the functional requirements of spectrum management described in the handbook on "National Spectrum Management". If you need additional space to answer the questions please continue on a separate sheet of paper.

1.	What legal or regulatory texts govern your national spectrum management processes?

Wireless Telegraphy Act, last modified 1998_______________________

Are any actions planned to change these legal texts or regulations?	YES   NO__	A new draft Communications Bill is currently being considered

2.	Have you publicly available regulations and procedures for national spectrum management (e.g. radio services, license requirements etc.)?	YES   NO__

3.	Do you have a national radio frequency spectrum allocation table?	YES   NO__

4.	Regulations for the technical characteristics of radiocommunications equipment

Do you specify that the technical characteristics of radiocommunications equipment must comply with certain requirements (often referred to as "equipment standards"), for example to avoid interference to other services and users?	 YES   NO__

- Do you develop these technical requirements or equipment standards on a national basis or use those developed by other administrations or international/regional standards organisations:	National __Other 

- Do you have a procedure to ensure that radiocommunications equipment complies with the technical requirements, for example:

Type Approval: ___; Manufacturers Declaration of Compliance:  (RTTE); Other ___

5.	Spectrum re-deployment*

(* The term "redeployment" is used here to refer to a process of national scope in which an assessment is conducted 1) to determine if portions of spectrum can be identified that are in limited use; and 2) to determine if such spectrum segments can be reallocated for use in delivering radiocommunication services that have expanding spectrum requirements. Some countries co-operate on a regional basis to identify suitable spectrum segments that may be  re-deployed to facilitate the introduction of new applications on a harmonised basis.)

- Has there been any spectrum redeployment* in your country or has a need for spectrum redeployment been identified?	YES   NO__

- If so, do you have a method for achieving this redeployment in respective frequency bands and for given radiocommunication services?	YES   NO__

- Please define the established method and describe the nature of the consultation, if any, with users regarding the potential costs resulting from the planned redeployment.

Spectrum pricing is one method that was introduced partly with the intention of simplifying the spectrum redeployment process.  Another method is licence revocation.  Consultation involves user groups and other government organisations depending on the spectrum and services concerned.

6.	Spectrum management costs

- What is the cost of providing national spectrum management functions in your country (if there is more than one organisation or agency responsible for spectrum management please give the total costs if this information is available)?                            116m (Swiss Francs)

- What is the source of the funding required to accomplish these spectrum management functions?

Spectrum usage fees

7.	Management of frequency assignment records.

- Does your administration have a system (manual or computerized) to keep and maintain records of national frequency assignments and spectrum use (usually known as a Data Base Management System (DBMS))?	YES  NO__

- Is there a single national DBMS or separate DBMS(s) for different users (for example a DBMS for assignments to government users and separate DBMS for assignments to non-government users)?	Single__ Separate 

What is the approximate size (at 2002) of your DBMS:

- number of frequency assignments   	                   approx. 200,00

- number of licences	                   > 212,000

- Are these frequency assignment records made available to public?	YES__ NO 

- Is the DBMS computerized?	YES  NO__

- What computerized DBMS do you use?                       Oracle, MS Access, Proprietary

8.	Co-ordination of frequency assignments with other countries:

- do you co-ordinate assignments to terrestrial stations 	YES  NO__

- do you co-ordinate assignments to space stations	YES  NO__

- Notification of frequency assignments.

Do you notify to the ITU those frequency assignments that are required to be notified by the Radio Regulations ?	YES  NO__

If not, please explain why and list any difficulties: _______________________________

10.	Do you have a policy and planning function for national spectrum management (i.e. a national strategy for future use of the spectrum)? 	YES  NO__

11.	Do you perform technical analyses of frequency assignment requests?	YES  NO__

12	Do you perform radio monitoring of terrestrial radio services?	YES  NO __

## Fixed monitoring stations

- How many fixed monitoring stations do you have?                            One plus 20 remote sites

- Please provide a brief list of the facilities available at your fixed monitoring stations (for example: receivers, spectrum analysers, direction finding equipment):

Receivers (9kHz – 3GHz), spectrum analysers, direction finding equipment (9kHz – 30MHz)

What is the upper frequency limit of your fixed monitoring stations                   3,000 MHz

- What is the upper frequency limit of your fixed direction finding stations         3 GHz

## Mobile monitoring stations

- How many mobile monitoring stations do you have?                                    Three

- Please provide a brief list of the facilities available in your mobile monitoring stations (for example: receivers, spectrum analysers, direction finding equipment)                        Receivers (9kHz – 50GHz), spectrum analysers, direction finding equipment

- What is the upper frequency limit of your mobile monitoring stations                   50,000 MHz

- What is the upper frequency limit of your mobile direction finding stations    50,000 MHz

## Transportable monitoring stations

- How many transportable monitoring stations do you have?                 Three

- Please provide a brief list of the facilities available in your transportable monitoring stations (for example: receivers, spectrum analysers, direction finding equipment):

Receivers, spectrum analysers, direction-finding equipment

- What is the upper frequency limit of your transportable monitoring stations  1,200 MHz

- What is the upper frequency limit of your transportable direction finding stations 1,200 MHz

- Do you perform space monitoring				YES ___   NO  

- Please provide a brief list of the facilities available at your space monitoring stations

___________________________________________________________________

- What tasks does your space monitoring station perform for GSO satellite monitoring?

- What tasks does your space monitoring station perform for non-GSO satellite monitoring?

## Does your Administration participate in the International Monitoring Programme of  ITU? 	YES    NO ___

## Co-operation between Spectrum Management and Monitoring

Please indicate the amount of work (in percentages) performed by the monitoring service for:

- Frequency Management Department 30%

- Enforcement Department 20%

- License Department 0%

- Do you perform Inspections on Radio Stations 	YES  NO__

- What inspection techniques are used by your administration to determine that users of the spectrum are complying with national or international requirements?

Measure transmission characteristics

- What are the administrative procedures that determine your inspection policy (for example the number of inspections, type of notification provided prior to inspection, rules and regulations)?

Based on work programme policies

- What measurement equipment does your administration use to perform technical measurements at an inspection?

General communication tests sets

- What technical parameters does your administration measure when inspecting a radio system?

Carrier frequency, radiated power, modulation, spurious emissions

- What station records does your administration review when inspecting a radio station?

Licence documents, certificates and logs.

14.	Do you perform technical analyses of radio frequency interference complaints?	YES NO__

Do you have an established consultation process, involving Government and non-government organization, for resolving these complaints?	YES  NO__

15.	Use of computers for national spectrum management

## General

- Do you use computers for national spectrum management?	YES  NO__

- Type of computers	__________

- How many workstations: ________ or personal computers (PCs): _________

- Operating system(s) _____________________________________________

- Does your spectrum management system operate within a Local Area Network (LAN)?	YES NO__

- Do you have access to the internet?	YES NO__

- Does your administration provide a web site on the internet to disseminate spectrum management information?	YES  NO__

If yes, please provide the address (URL) of the web site:                                  www.radio.gov.uk

Windows Basic Spectrum Management System (WinBASMS)

- Are you aware that a Windows Basic Spectrum Management System is available from the ITU at no cost?	YES  NO__

- Has your administration used WinBASMS? 	YES  NO__

- Has your administration had problems using WinBASMS?	YES  NO__

- Please list all problems that were encountered using WinBASMS.

__________Too basic________________________________________________________	Would you recommend using WinBASMS if the problems identified in (d) have been corrected?	YES__ NO__

- Do you need an enhanced spectrum management system if you answered no in (e)?	YES__ NO__

## Advanced Automated Spectrum Management Systems (AASMS)

- Does your administration use an Automated Spectrum Management Systems (AASMS)	YES  NO__

- Has your administration had problems using your AASMS	YES__ NO 

- Please list all problems that were encountered using your AASMS

________________________________________________________________________

- How would you propose to change the AASMS to correct or overcome these problems (please describe)?

________________________________________________________________________

16.	Organisation of spectrum management

- Please describe your country's spectrum management structure and enclose a copy of the organization chart. The following aspects are of particular interest:

- Is the spectrum management organisation a separate ministry, department or agency reporting directly to the government or is it part of a larger government department (for example, a department responsible for all telecommunications)? Currently an agency within a government department

- Is the responsibility for spectrum management contained within a single organisation or is it shared between separate organisations (for example, some administrations have separate organisations for regulatory matters and policy matters, other administrations have separate organisations for government users and non-government users)?  Single

- Have there been recent changes in this organisational structure or are changes planned (for example to take account of any changes in your government's policy for telecommunications)?

Next year the Agency will become part of a new national telecommunications regulator that amalgamates five existing telecommunication organisations

- Number of specialist staff in national spectrum management? 	              245

- Number of support staff in national spectrum management?	              297

Organisation Chart

PART III

Attachment 3

QUESTIONNAIRE - PART III
(To be completed by administrations)
Information on the calculation of fees for frequency use

Information on this part will follow later.