ATTACHMENTS OF SAILSPACE-1 CRC

ATTACHMENT 1   ATTACHMENT FOR PARAMETERS B4A3A C9C1 AND C9C2

- B4A3A   STATEMENT FOR ANGLE ALPHA NOT PROVIDED

The “Alpha” and “Beta” angles are not fixed for any of the steerable beams of the SAILSPACE-1 space station, and therefore these Alpha and Beta angles are not provided. The  “Alpha” and “Beta” angles are not applicable for the KU4R and KU4E omnidirectional beam.

- C9C1   TYPE OF MODULATION/MULTIPLE ACCESS DATA/SPECTRUM MASK

TYPE OF MODULATION: BPSK/QPSK/32PSK/16PSK/8PSK/OQPSK

MULTIPLE ACCESS DATA: TDMA / FDMA

- C9C2 SPECTRUM MASK DIAGRAM

The following e.i.r.p. mask is applied in accordance with Recommendation ITU-R SM.1541: EIRP-ES = 40 log [F/50+ 1] dBsd. Where F is the frequency offset from the edge of the total assigned band, expressed as a percentage of necessary bandwidth.

ATTACHMENT 2   E KU1E/KU2E/KU3E/QV1E/QV2E/QV3E/KU4E GAIN VS ELEVATION ANGLE DIAGRAM

The beams are steerable; thus, considering any fixed point in the service area of beams, given the nature of steering beams, the gain of the beams for all elevation angles can be as large as the peak gain, which is equal to the max isotropic gain in B.3.a.1.

The omnidirectional KU4E beam is meant for use in any satellite latitude. The gain for any elevation angle is as much as the maximum isotropic gain in B.3.a.1.

ATTACHMENT 3  SPREADING LOSS DATA FOR E KU1E/KU2E/KU3E/KU4E

ATTACHMENT 4  SPREADING LOSS DATA FOR E QV1E/QV2E/QV3E

ATTACHMENT 5  COMMITMENT UNDER RESOLVES 3 OF RES 770

SAILSPACE-1 will adopt the GSO arc interference avoidance Angle strategy to ensure that NGSO protects the GSO system, to ensure the protection of the geostationary fixed-satellite service and broadcasting-satellite service networks from non-GSO FSS service systems. The satellites will only transmit when the minimum separation angle requirement is satisfied, this will minimize any interference potential in the downlink/uplink direction.

By adopting the above strategies, we can ensure that SAILSPACE-1 complies with the limits in NO. 22.5L, under resolves 3 of Res 770.