Attachment1:

For the beam KA2AE, KA2BE, KA2ME, KA3AE, KA3BE, KA3ME, KA4AE, KA4BE, KA4ME, QV1AE, QV1BE, QV1ME (E) filed in the inter-satellite service, the power density will be reduced and the boresight of each of the beams will be pointed such that the relevant Article 21 PFD limit is met; The minimum angle between the KA2AE, KA2BE, KA2ME, KA3AE, KA3BE, KA3ME, KA4AE, KA4BE, KA4ME, QV1AE, QV1BE, QV1ME beam boresight and the Earth's horizon is at least 60 degrees. As a result, the main gain of KA2AE, KA2BE, KA2ME, KA3AE, KA3BE, KA3ME, KA4AE, KA4BE, KA4ME, QV1AE, QV1BE, QV1ME directed toward the Earth is attenuated by no less than 40 dBc., thus ensuring that the EIRP density toward any point on the Earth will not exceed the following values:

| Beam Name | Maximum EIRP density toward any point on Earth（dBW/Hz） |
| --- | --- |
| KA2AE | -81 |
| KA2BE | -73.6 |
| KA2ME | -69.6 |
| KA3AE | -75.8 |
| KA3BE | -68.4 |
| KA3ME | -64.4 |
| KA4AE | -78 |
| KA4BE | -70.6 |
| KA4ME | -66.6 |
| QV1AE | -79.4 |
| QV1BE | -72 |
| QV1ME | -68 |