| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On Next Generation Networks |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2001-2004 |  | FGNGN-OD-0004 |
|  |  | Original: English |
| Group(s): | WG 6 | Geneva, 19 - 23 July 2004 |
| OUTPUT DOCUMENT |  |  |
| Source: | Editors |  |
| Title: | PSTN evolution to NGN |  |

This serves as the initial draft of PSTN evolution to NGN.

Contents

Page

1	Scope	4

2	References	4

3	Definitions	4

4	Abbreviations	5

5	Conventions	6

6	Evolution of PSTN to NGN	6

7	Transport	6

7.1	Evolution scenarios	8

7.1.1	Scenario 1	8

7.1.2	Scenario 2	10

7.1.3	Scenario 3	11

7.1.4	Scenario 4	12

8	Managements aspects	12

9	Signalling and control aspects	12

10	Service aspects of evolution	13

11	Security aspects of evolution	13

12	Emergency services	13

Appendix I: Examples of PSTN Transport evolution scenarios	14

II.1	Use of ATM as the packet based network	14

II.2	Use of ATM as a access network to IP in the core	15

II.3	Use of both ATM and IP for overflow of PSTN traffic	16

II.4	Use of IP as an access network	17

II.5	Use of PSTN in the access	18

Appendix II: Examples of PSTN service evolution	19

Introduction

NGN (Next Generation Network) is believed to provide new opportunities for and capabilities to the network and service providers. Considering that existing networks have different life span and vast amount of capital has been spent on them, complete replacement of their components is not considered to be either advisable or possible. So, a phased approach should be considered for evolution of existing networks to NGN.

PSTN (public switched telecommunications network) being one of the first networks, is considered to be prime candidate for evolution. For PSTN evolution to NGN a phased approach is considered in this draft.

PSTN EVOLUTION TO NGN

# Scope

PSTN (public switched telecommunications network) being one of the first networks in telecommunication is considered to be a prime candidate for evolution to Next Generation Network (NGN). Since vast amount of capital has been invested in PSTN also in any other network, evolution to NGN should be considered as a step-wise approach.

This draft describes possible ways of evolving PSTN to NGN. It provides steps for evolution of transport, management, signalling and control parts of PSTN to NGN. Examples of possible implementation scenarios are provided in the appendices.

Note: PSTN encompasses ISDN for telephony services. The PSTN assumes the emulation of PSTN service (see definition section for definition of PSTN/ISDN emulation).

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this draft. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this draft are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this draft does not give it, as a stand-alone document, the status of a Recommendation.

[x]	ITU-T Recommendation G.100 (2001), Definitions used in Recommendations on general characteristics of international telephone connections and circuits

[x]	ITU-T Recommendation G.964 (2001), V-interfaces at the digital local exchange (LE) – V5.1 interface (based on 2048 kbit/s) for the support of access network (AN)

[x]	ITU-T Recommendation Y.overview (2004), NGN overview

[x]	ITU-T Recommendation Q.310-332 (1988), Specifications of Signalling System R1

[x]	ITU-T Recommendation Q.400-490 (1988), Specifications of Signalling System R2

[x]	ITU-T Recommendation G.965 (2001), V-interfaces at the digital local exchange (LE) - V5.2 interface (based on 2048 kbit/s) for the support of access network (AN)

[x]	ITU-T Recommendation M.3400 (2000), TMN Management Functions

[x]	ITU-T Recommendation G.3010 (2000), Principles for a Telecommunications management network

# Definitions

This draft uses or defines the following terms:

3.1	Access Network (AN): See Recommendation G.964[x].

3.2	Next Generation Network (NGN): See Recommendation Y.2001 [x].

3.3	Public Switched Network (PSTN): See Recommendation G.100 [x].

3.4	PSTN/ISDN Emulation: The provision of PSTN/ISDN service to PSTN/ISDN terminals by the NGN (e.g. via a gateway).

3.5	PSTN/ISDN Simulation: The provision of PSTN/ISDN-like services to non-PSTN/ISDN terminals. Such terminals may not be able to provide all functionalities that a PSTN/ISDN terminal provides.

3.6	User Access Module (UAM):  A unit that physically terminates subscriber lines and converts the analogue signals into a digital format. The UAM is collocated with a Local Exchange (LE), and is connected to the Local Exchange.

3.7	Remote User Access Module (RUAM):  A unit that physically terminates subscriber lines and converts the analogue signals into a digital format. The RUAM is physically remote from the Local Exchange.

# Abbreviations

This draft uses the following abbreviations.

AS	Application Server

ATM	Asynchronous Transfer Mode

CAS	Channel Associated Signalling

CBR	Constant Bit Rate

CCS	Common Channel Signalling

CS	Call Server

IN	Intelligent Network

INAP	Intelligent Network Application Part

IP	Internet Protocol

ISDN	Integrated Service Digital Network

LE	Local Exchange

LL	Leased Line

OSS	Operation Support System

PBX	Private Branch Exchange

PCM	Pulse Code Modulation

POTS	Plain Old Telephone Service

PRI	Primary Rate Interface

PSTN	Public Switching Telecommunications Network

QoS	Quality of Service

SCE	Service Creation Environment

SCP	Service Control Point

SG	Signalling Gateway

SSF	Service Switching Function

SSP	Service Switching Point

STP	Signalling Transfer Point

TDM	Time Division Multiplexing

TE	Transit Exchange

TG	Transit Gateway

TMN	Telecommunication Management Network

UAM	User Access Module

AG	Access Gateway

VoIP	Voice over IP

# Conventions

[TBD]

# Evolution of PSTN to NGN

[editor note: text needed to estate what existing PSTN is and what approach needs to be taken]

In general PSTN networks are comprised of the following entities each with one or multiple functionalities:

- Transport (access plus core): UAM, RUAM, AN (Access Network) via V.5 interface connected to the core switches and core switches themselves

- Control and signalling: exchange hosts

- Management: management of exchanges

- Service: exchange hosts and auxiliary network (e.g. IN)

In PSTN most of the functionalities are located in a single exchange and may use proprietary protocols. However, in NGN it is expected that functionalities will be distributed amongst several elements.

# Transport

Figure 7-1 provides a high level view of PSTN. The PSTN transport is comprised of two parts access and core. The access part is comprised of all transmission facilities, UAMs, ANs, and PBXs. The core of PSTN transport is comprised of local and transit exchanges. A brief description of each part is provided below. Possible evolution scenarios are also provided.

Figure 7-1: General view on PSTN transport

The PSTN access elements which may exist individually or in a combination are:

- UAM (User Access Module) which is connected to exchanges via proprietary interfaces (locally or remotely)

- PBX (Private Branch Exchange) which is connected to the local exchange directly or via remote access modules

- ANs (Access Networks) which is connected to the local exchange via V5.x (G.964/G.965 [x , y]) open interfaces.

## Evolution scenarios

Possible scenarios for evolution of PSTN access and core may be as follows:

### Scenario 1

This scenario consists of two steps, which are executed consecutively.

Step 1

The aim of step 1 is to prepare the PSTN for future NGN architecture. First all switch functionalities such as control, billing, etc. is removed from some of the local exchanges and access elements such as UAM, PBX, and AN are connected to the remaining exchanges as remote units e.g. UAM becomes RUAM.

Step 2

Step 2 follows step 1 to complete replacement of PSTN with NGN. Call control and billing functions of the LE are performed by the call server (CS) and all functions of UAM, RUAM are done by access gateways (AGs). The transit and signalling gateways are introduced for interconnection with other operators’ PSTN.

Figure 7-2: Realisation of scenario 1

### Scenario 2

The aim of this scenario is to prepare the PSTN for future NGN architecture.  Contrary to scenario 1, this scenario directly replaces PSTN with NGN in one step. All call control and billing functions of the LEs are performed by the call server (CS) and all functions of UAM, RUAM are done by access gateways (AGs). The transit and signalling gateways are introduced for interconnection with other operators’ PSTN.

Figure 7-3: Realisation of scenario 2

### Scenario 3

The aim of this scenario is to deploy an overlay NGN to PSTN. That is the co-existence of NGN with PSTN.  Some of the UAMs are upgraded to AGs to replace LEs and additional new AGs may also be implemented for new subscribers and all these elements are controlled by the call server (CS). The signalling and transit gateways are introduced for interconnection with existing PSTN.

Figure 7-4: Realisation of scenario 3

### Scenario 4

In this scenario Class 4 exchanges (also called as trunking exchange, TE) are converted to transit gateway (TG) and new call servers (CSs) are introduced. The TGs are connected either using existing PSTN facilities or of the Packed Based Network.

Figure 7-5: Replacement of TEs with TGs

Examples of such variants are shown in Appendix I.

# Managements aspects

The management system of PSTN includes exchange network management, access network management, intelligent network management and OSS (Operation Support System). Recommendations M.3400 [x] and M.3010[x] provide management principles for PSTN.

Referring to the management module defined in TMN, the NGN management system is comprised of three planes, namely the network element management plane, the network control plane and the service management plane. The three planes each implement corresponding management functions to each layer in the NGN layered model. Standard interfaces between these planes will be defined.

[Editor note: further study is needed in collaboration with other NGN groups including SG4. Until the NGN Management Framework has been defined it would be difficult to discuss strategy for evolution]

# Signalling and control aspects

In PSTN, TDM type signalling such as R1 [x], R2 [x], CCS [x], CAS [x] and SS7 are used. Migrating to NGN may require signalling in the packed network. Also signalling and control processing may reside in more than one element in NGN.

The AG needs to support not only POTS analogue subscribers and ISDN digital subscribers, also PBX, V5 subscribers and UAM to extend the coverage. However, different signalling is needed for different access.

The following figure illustrates an evolution scenario.

Figure 9-1: Decentralization of Signalling  and control functions

The signalling and control functionalities in PSTN may be distributed into different elements in NGN. Also, these elements may use either PSTN or packet based technology for interconnection.

# Service aspects of evolution

PSTN services which are traditionally provided by PSTN exchanges may be transferred to ASs (Application Servers) as applications in NGN.

An example of PSTN service evolution is shown in Appendix II.

# Security aspects of evolution

[TBD]

# Emergency services

[TBD]

# Appendix I: Examples of PSTN Transport evolution scenarios

(This appendix does not form an integral part of this draft)

## II.1	Use of ATM as the packet based network

In this example the ATM networks is used to ensure proper QoS for voice connections.  One of the ways of VoIP realization is to use ATM technology (VoIPoATM) as a transport layer having the qualities guaranteeing QoS for voice transmission (e.g. using CBR service).

Figure I1-1: Example of ATM as a packet based network

## II.2	Use of ATM as a access network to IP in the core

In this example the ATM networks is used as an access to IP in the core for signalling connections.  The PSTN is the transport layer for voice connection.

Figure I1-2: Example of ATM as an Access network

## II.3	Use of both ATM and IP for overflow of PSTN traffic

In this example both ATM and IP are used for overflow of PSTN traffic.

Figure I1-3: Example of ATM and IP being used for overflow of PSTN traffic

## II.4	Use of IP as an access network

In this example the IP is used in the access for signalling connection to the IP core.  The PSTN is the transport layer for voice connection.

Figure I1-4: Example of IP as an Access network

## II.5	Use of PSTN in the access

In this example the PSTN is used in the access for signalling connection to the IP core.  The PSTN is the transport layer for voice connection.

Figure I1-5: Example of PSTN as an Access network

# ppendix II: Examples of PSTN service evolution

(This appendix does not form an integral part of this draft)

- Implementation of SSF function of the IN network in the control layer (using open interface INAP allows for treating the IN network elements as the elements of NGN service layer).

- Duplication/implementation of the service logic from PSTN host in NGN service layer (application server - AS). Division of service logic from control.

- Inclusion of SCP of the IN network into NGN service layer - SSP-SCP communication through NGN IP packet network.

- Common SCE for all elements of NGN service layer - optional step.

- For the separation of the service function during evolution of PSTN, service process in local exchange can be transferred simply to a tandem exchange through data configuration. Only tandem exchanges are upgraded according to the above described steps. In this way, the collection of information at billing centre becomes simpler too, because all the services are converged at the tandem exchanges, only the information of tandem exchanges needs to be collected rather than that of all the local exchanges.

Figure X. Service evolution from PSTN to NGN

___________________________