| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On Next Generation Networks |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2001-2004 |  | FGNGN-OD-00022 |
|  |  | Original: English |
| Group(s) | WG3 | Geneva, 19-23 July 2004 |
| OUTPUT DOCUMENT |  |  |
| Source: | WG3 Editor |  |
| Title: | Revision 1 of draft TR-e2eqos.1 |  |

This document proposes revision 1 of TR-e2eqos.1 based on NGNFG-ID-48 according to the meeting discussion and ad hoc editing group. Besides the format, the main changes are summarized as follows:

1 Text has been added for Section 6.5 based on the contribution from ZTE (FGNGN-ID-77).

2 Text has been added for Section 6.6 based on the contribution from ZTE (FGNGN-ID-79), and the content of that contribution has been put into the appendix to be further considered.

3 Inputs from ad hoc drafting group on requirements for general QoS signaling part of  TRQ.IPQoS.SIG.CS1 and the result of this effort documented in Annex A. Annex A summarizes the generic requirements (some user and some network requirements) with the understanding that it will be further developed and inserted into an appropriate section of the main document body. In particular, the text was not fully agreed. Further work is needed to refine the definition of QoS requests, NNI, etc.

ITU-T draft TR-e2eqos.1

Requirements and framework for end to end QoS architecture in NGN

Summary

This draft provides a general end-to-end QoS architecture framework for NGN to facilitate new applications and services. The purpose is to introduce capabilities that would allow multiple architectural approaches and future innovation.

Introduction

The topic of QoS in IP networks has long been the subject of research, development, standardization, and network experience. All indications point to a continuing cycle of experience and innovation. Since each recognized operating agency has a different regulatory environment, service offerings, geographic span, and network infrastructure, there must be flexibility within any global end-to-end architecture that allows each operator to adopt new innovations or to revise existing capabilities on their own time scale.

Table of Contents

1	Scope	8

2	References	8

3	Definitions	9

4	Abbreviations	10

5	Conventions	11

6	Requirements	11

6.1	Design Goals	11

6.2	User requirements	12

6.3	Network requirements	12

6.3.1	data plane	13

6.3.2	control plane	13

6.3.3	management plane	13

6.4	Interworking and Interoperability requirements	13

6.5	Flexibility requirements	13

6.6	Scalability requirements	13

6.7	Security	14

6.8	Reliability and Fault Tolerance	14

6.9	Mobility support requirements	15

6.10	Other requirements	15

7	End to end QoS and network performance classes	15

8	Generic reference Model	17

9	Interfaces and Functional Information Model	18

9.1	UNI	18

9.2	NNI	18

10	QoS mechanisms	18

10.1	QoS mechanisms in CPN	18

10.2	QoS mechanisms in access network	19

10.3	QoS mechanisms in core network	19

10.4	Interdomain and Interworking QoS mechanisms	19

11	Interaction with AAA system	19

12	Interaction with network management system	19

13	Other considerations	19

13.1	Business Considerations	19

Appendix 1 –Scalability consideration	20

Annex A.  Supplementary material to be inserted into the main document	21

Requirements	21

User-Network Signalling	21

Omitting Attributes of a User QoS Request	22

Special Case of User QoS Request to support Voiceband Channels	23

Flow Control for User QoS Requests and Re-requests	23

Network Response to User QoS Requests	23

User Answer to Network QoS Response	24

QoS Signalling at the Network – Network Interface	24

Attributes of a Network QoS Request	24

Omitting Attributes of a Network QoS Request	24

Performance Requirements for QoS Requests and Re-requests	24

Response to a Network QoS Request	25

Accumulating Performance for Additional Requests	25

QoS Release	25

Performance	26

Symmetry of information transfer capability	26

Contention resolution	26

Error reporting	26

Unrecoverable failures	26

Forward and backward compatibility	26

Parameters and values for Transport connections	26

User-Initiated QoS Resource Modification	26

Emergency Service	27

Reliability/Priority Attributes	27

# Scope

This draft provides:

-Requirements for end-to-end QoS architectures

-A general architectural model, and

-A framework consisting of elements common to specific end-to-end QoS architectures in NGN

Within this framework it is envisioned that a series of new drafts would be created based on new contributions, to serve additional architectural needs like centralized, distributed and hybrid approaches.

[Ed. Note: Relationship with general access QoS architecture needs to be clarified. Signaling requirement work in SG11 will be also considered.]

# References

The following ITU-T Recommendations and other references specified explicitly contain provisions of this Draft. All Recommendations and other references are subject to revision; users of this document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this document does not give it, as a stand-alone document, the status of a Recommendation.

- ITU-T E.860 (2002), Framework of a Service Level Agreement

- ITU-T E.360.x (2002), QoS Routing and Related Traffic Engineering Methods    for IP-, ATM- and TDM-Based Multiservice Networks

- ITU-T E.361 (2003), Qos Routing Support for Interworking of QoS Service Classes Across Routing Technologies

- ITU-T Recommendation Y.1001 (2000), A Framework for Convergence of Telecommunications Network and IP Network technologies.

- ITU-T Recommendation Y.GRM-NGN, General Reference Model for Next Generation Networks.

- ITU-T Recommendation Y.NGN-FRA, Functional Requirements and Architecture of the NGN.

- ITU-T J.170 (2002), IPCablecom security specification

- ITU-T J.174 (2002), IPCablecom interdomain quality of service

- ITU-T M.1079 (2003), Performance and quality of service requirements for International Mobile Telecommunications-2000 (IMT-2000) access networks

- ITU-T Y.1221 (2002), IP Packet Transfer Performance Objectives

- ITU-T Y.1540 (1999), IP Packet Transfer and Availability Performance Parameters

- ITU-T Y.1541 (2002), IP Packet Transfer Performance Objectives

- IETF RFC2990 (2000), Next Steps for the IP QoS Architecture

- IETF RFC3031 (2001), Multiprotocol Label Switching Architecture

- IETF RFC2475 (1998), An Architecture for Differentiated Services

- IETF RFC2702 (1999), Requirements for Traffic Engineering Over MPLS

- IETF RFC3209 (2001), RSVP-TE: Extensions to RSVP for LSP Tunnels

- IETF RFC3564 (2003), Requirements for Support of DiffServ-aware MPLS Traffic Engineering

- IETF RFC3270 (2002), Multi-Protocol Label Switching (MPLS) Support of Differentiated Services

- IETF RFC3272 (2002), Overview and Principles of Internet Traffic Engineering

- ITU-T Y.1291, An Architectural Framework for Support of Quality of Service (QoS) in Packet Networks

- ITU-T Y.123.QoS (2003), A QoS architecture for Ethernet-based IP access network

- Rec. E.800	Terms and definitions related to quality of service and network performance including dependability

- Rec. I.350	General aspects of quality of service and network performance in digital networks, including ISDNs

- Rec. I.356	B-ISDN ATM layer cell transfer performance

- Rec.Y.1540	Internet protocol data communication service - IP packet transfer and availability performance parameters

- Rec.Y.1541	Network performance objectives for IP-based services

- Rec. Y.1560	Parameters for TCP connection performance in the presence of middleboxes

- Rec. Y.1561	Performance and Availability Parameters for MPLS Networks

- Draft Rec. Y.NGN-GRM	General Reference Model for NGN

- ITU-T Y.1251: General architectural model for interworking

- ITU-T Y.1412 ATM-MPLS network interworking - Frame mode user plane interworking

- ITU-T Y.1411 ATM-MPLS network interworking - Cell mode user plane interworking

- ITU-T G,1010, End-user multimedia QoS categories

- GRQ

[Ed. Note. This section will be updated]

# Definitions

This docunment defines the following terms:

End-to-End: Within the context of this draft end-to-end means UNI-to-UNI,  that is from the User Network Interface (UNI) at the source host side to the UNI at the destination host. Note that end-to-end means from mouth to ear in other Recommendations concerning user perceiving.

Customer Premises Equipment (CPE): End-user system including private network elements connecting the customer applications to the access line.

Customer Premises Network (CPN): A private network administrated by the user, that may be individual, home or enterprise.

Access network: Implementation comprising those entities (such as cable plant, transmission facilities, etc.) which provide the required transport bearer capabilities for the provision of telecommunications services between a Service Node Interface (SNI) and each of the associated User-Network Interfaces (UNIs).

Core network: A portion of the delivery system composed of networks, systems equipment and infrastructures, connecting the service providers to the access network.

Connection-oriented network service: A network service that establishes logical connections between end users before transferring information.

Connectionless service: A service, which allows the transfer of information among service users without the need for end-to-end logical connection establishment procedures.

Relative QoS:  This term refers to a traffic delivery service without absolute bounds on the achieved bandwidth, packet delay or packet loss rates. It describes the circumstances where certain classes of traffic are handled differently from other classes of traffic, and the classes achieve different levels of QoS.

Absolute QoS: This term refers to a traffic delivery service with numerical bounds on some or all of the QoS parameters.  These bounds may be physical limits, or enforced limits such as those encountered through mechanisms like rate policing. The bounds may result from designating a class of network performance objectives for packet transfer.

Flow [IP flow]: A sequence of packets sent from a particular source to a particular destination to which the common routing is applied. If using IPv4, a flow is identified by IPv4 5-tuple including source/destination IP addresses, protocol ID, source/destination port numbers. If using IPv6, a flow is identified by IPv6 3-tuple including source/destination IP addresses, flow label.

Session: A period of communication between two terminals which may be conversational or non-conversational (for example retrieval from a database).

Interworking: This term is used to express interactions between networks, between end systems, or between parts thereof, with the aim of providing a functional entity capable of supporting an end-to-end communication. The interactions required to provide a functional entity rely on functions and on the means to select these functions.

Interoperability: The ability of two or more systems or applications to exchange information

and to mutually use the information that has been exchanged.

[Ed. Note. This section will be updated based on the terminology used in this document]

# Abbreviations

This document uses the following abbreviations:

IETF	Internet Engineering Task Force

ITU-T	International Telecommunication Union – Telecommunication Standardization Sector

IP	Internet Protocol

AS	Autonomous System

RSCF         Resource and Service Control Function

LSP	Label Switched Path

MPLS	Multiple Protocol Label Switching

DiffServ	Differentiated Service

RSVP	Resource ReSerVation Protocol

QoS	Quality of Service

SLA	Service Level Agreement

NP             Network Performance

CPE	Customer Premises Equipment

AN	Access Node

ER	Edge Router

BR	Border Router

BAS	Broadband Access

CPN	Customer Premises Network

CPE	Customer Premises Equipment

NGN	Next Generation Network

SNMP	Simple Network Management Protocol

UNI	User-to-Network Interface

NNI	Network-to-Network Interface

API	Application Programming Interface

[Ed. Note. This section will be updated based on the terminology used in this draft]

# Conventions

In this document, "shall" refers to a mandatory requirement, while "should" refers to a suggested but optional feature or procedure. The term "may" refers to an optional course of action without expressing a preference.

[Ed. Note. This section will be updated to make it consistent with other ITU documents.]

# Requirements

## Design Goals

The end-to-end QoS architecture is designed to provide necessary quality for a variety of services and applications. This architecture ensures the interworking between different administrative domains with possibly varied architectures to achieve end-to-end QoS services.

QoS is defined in Recommendation E.800 as follows: “Collective effect of service performance which determine the degree of satisfaction of a user of the service”. Based on this definition which is a fairly wide one encompassing many areas including subjective user satisfaction, one should provide mechanisms that would guarantee the requested services by measurement and compliance methods. This process also involves identification of parameters that can be directly observed and measured at the point at which the service is accessed by the users and network providers.

Recommendation I.350 defines Network Performance (NP) as the “NP is measured in terms of parameters which are meaningful to the network provider and are used for the purpose of system design, configuration, operation and maintenance. NP is defined independently of terminal performance and user actions”.

Hence the set of requirements for providing end to end QoS in the broad sense as defined above would be defined with the objectives that are measurable and quantifiable in some sense which will be made precise in this document. In the absence of fixed allocations of NP and CPN and terminal performance values, this architecture enables evaluation of end-to-end paths for their compliance with the objectives.

[Ed. Note. The following text is from NGN-WD-87, other sources will be added]

1)	Encompass CPEs

2)	Be independent of access technology and provide mechanisms that would allow negotiation between access and core networks that belong to different network providers

3)	Accommodate multiple administrative domains

4)	Support various services, conventional as well as new applications; real-time multimedia communications, VPN and new advanced applications like telepresence and virtual reality etc.

5)	Support convergence of connectionless and connection-oriented networks and technology

6)	Support applications and systems that require call control signaling as well as others that do not

7)          Support admission control and congestion control mechanisms based on measurements

8) 	Support QoS extensions in middleware systems for innovative new services

……….

The high performance expected from the IP networks evolving to NGN has many requirements besides the QoS. The other considerations in this context that are closely related to QoS as follows:

- Security

- Reliability

- Availability

- Resilience (Fault Tolerance)

The design goal would be therefore is an architectural framework that provides the collective requirements for users, both end users and others such as application providers and network providers.

[Ed. Note. This section will be updated]

## User requirements

[Ed. Note. This section will be updated]

## Network requirements

TBD

### data plane

### control plane

### management plane

TBD

## Interworking and Interoperability requirements

[Ed. Note. Requirements of the QoS architecture should be given to ensure the interworking between different domains, different technologies and different authorities. The section title might be modified based on the alignment to other ITU documents. Contributions are invited]

## Flexibility requirements

The end-to-end QoS architecture should support different kind of applications and related business models.

There should not be any restrictions for QoS mechanisms inside a network, for example the network could support MPLS, or it could be the IP network without MPLS support. It is up to carriers to choose according to their own network situation.

As for QoS resource control, there could be a distributed or centralized approach or even a hybrid approach to fulfil the admission control and resource management. Resource control function can be implemented in a separated device, or it can reside in the edge routers or other existing physical element. It depends on the architecture inside the network.

Also, the flexibility lies in the following aspects:

-The flexibility of the resource request initiator, it could be the user, or the proxy (like the service controller)

-The QoS framework shall support dynamic QoS behaviour (i.e., it shall be possible to modify QoS attributes during an active session.)

The flexibility is based on the interworking and interoperability consideration and it should not go beyond them.

## Scalability requirements

[Ed. Note. This section will describe the requirements for scalability. This is an important consideration for reference model evaluation. Contributions are invited]

Scalability is a major concern in new architectures, services and applications of NGN. The fundamental premise of scalability is to provide a system that will not be impacted adversely by growth of hardware, software and number of applications and users. It provides essentially a mechanism to gracefully grow the network as necessitated by demand.

NGN must be flexible and have reusable components, be reliable and scalable, and be able to provide QoS support for different applications. In this context scalability refers to the ability to handle additional users, services and applications. This means that network performance and QoS parameters do not suffer degradation and user perception is not noticeably impacted as network grows. Scalability also affects reliability and system administration. Furthermore, a set of recommended measurements and tests should be defined related to NGN scalability.

## Security

The QoS architecture described in this draft enhances the security of IP networks. And it does not raise any new security issues to IP networks.

MPLS technology can be used to implement network resource isolation between the different service classes. It prevents from the vulnerable best-effort traffic intruding into the reserved resource of LBN.

Resource requests are initiated by service control function not by hosts, which prevents from the malicious resource requests and the resulting illegal excessive resource reservation, exhaustion and even DoS (denial of service). All of resource requests are triggered by the service requests that have passed the user authentication and authorization. Should also cover resource reservation by hosts.

Admission control is helpful against fabrication attacks, unauthorized traffic and the resulting congestion.

Traffic marking can be  done  and checked edge routers. And the mark is trusted and reused by core routers. Initial markings can be done by hosts…

QoS (resource control) signalling can be out-of-band and path-decoupled, which can be delivered on the dedicated link with security encryption. Access nodes, edge routers and service control function should protect themselves from DoS attacks.

Still, the conventional network security mechanisms such as firewalls, intrusion detection software (IDS) and proxies are used against network attacks. If needed, authentication and integrity mechanisms can be used to protect UNI and NNI from interception, modification and fabrication attacks.

[Ed. Note. This section will describe the requirements for security and is subject to change. Contributions are invited]

## Reliability and Fault Tolerance

This section is about reliable service protection and rapid service restoration from the failure of an end-to-end QoS architecture. In user/data plane, the reliability and OAM function of routers is of much concern. In control and management plane, the reliability and backup of centralized admission control and resource management entities is of much concern. The interaction between different sub-layers may also make effects on the systematic reliability.

In the end-to-end QoS architecture described in this draft, the key function physical entities could be multihomed for redundancy backup. That is, a AN could be connected to multiple ERs; an ER/AN could be connected to multiple RCFs in a domain; a SCF could be connected to multiple RCFs; a RCF could be connectted to multiple RCFs in other domains. The redundancy of the physical entities could be more or less according to the network requirements.

The data consistency check by background process is used for avoiding the resource deadlock.

Routers should support MPLS OAM mechanisms at least including MPLS LSP fast failure detection and protection switching in conformance to ITU-T Y.1711 and Y.1720.

If a LSP is detected in failure, all service flows borne on the LSP should be rerouted rapidly if not receiving the call release form SCF. If the failure LSP is configured with one or more backup LSP, these flows traffic should be switched into the backup LSP rapidly and reliably.

If the failure LSP is configured without any backup LSP, RCF should immediately select the new paths for these flows traffic and release the previously selected paths resource. It is desired to seek the equivalent path to the previous failure path in the same domain for a service flow as much as possible. For the fast path reselection, a routing matrix table may be used to calculate and store the equivalent paths for a service flow so that the path can be recalculated and switched partially instantly according to the service type, available resource, policy, specific QoS requirement and so on.

If needed, the key function physical entities could be installed with one or more backup entities working in the active-standby mode or in the load-balancing mode, such as RCF, Edge Router and Transit router.

The cooperation and interaction between multi-layer protection mechanisms is for further study.

[Ed. Note. This section will describe the requirements for reliability and is subject to change. Contributions are invited]

## Mobility support requirements

[Ed. Note. This section will describe the requirements for mobility and is subject to change. Contributions are invited]

## Other requirements

[Ed. Note. This section will describe other requirements such as deployment feasibility, complexity and efficiency, that are relevant to QoS architecture and are not discussed elsewhere within this document. Contributions are invited]

# End to end QoS and network performance classes

| Table 1/Y.1541 – Provisional IP network QoS class definitions and network performance objectives |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Network performance parameter | Nature of network performance objective | QoS Classes |  |  |  |  |  |
|  |  | Class 0 | Class 1 | Class 2 | Class 3 | Class 4 | Class 5 Unspecified |
| IPTD | Upper bound on the mean IPTD (Note 1) | 100 ms | 400 ms | 100 ms | 400 ms | 1 s | U |
| IPDV | Upper bound on the 1  10–3 quantile of IPTD minus the minimum IPTD (Note 2) | 50 ms (Note 3) | 50 ms (Note 3) | U | U | U | U |
| IPLR | Upper bound on the packet loss probability | 1 × 10–3 (Note 4) | 1 × 10–3 (Note 4) | 1 × 10–3 | 1 × 10–3 | 1 × 10–3 | U |
| IPER | Upper bound | 1 × 10–4 (Note 5) |  |  |  |  | U |

Note: "U" means "unspecified" or "unbounded".

The following table gives some guidance for the applicability and engineering of the network QoS Classes. For each class, network operators make capacity planning and traffic engineering.

Table 2/Y.1541 – Guidance for IP QoS classes

| QoS class | Applications (examples) |  | Node mechanisms |  | Network techniques |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  | Real-time, jitter sensitive, high interaction (VoIP, VTC) |  | Separate queue with preferential servicing, traffic grooming |  | Constrained routing and distance |
| 1 |  | Real-time, jitter sensitive, interactive (VoIP, VTC). |  |  |  | Less constrained routing and distances |
| 2 |  | Transaction data, highly interactive (Signalling) |  | Separate queue, drop priority |  | Constrained routing and distance |
| 3 |  | Transaction data, interactive |  |  |  | Less constrained routing and distances |
| 4 | Low loss only (short transactions, bulk data, video streaming) |  | Long queue, drop priority |  | Any route/path |  |
| 5 | Traditional applications of default IP networks |  | Separate queue (lowest priority) |  | Any route/path |  |

[Ed. Note. This section will describe end to end performance classes based on Y.1541 and harmonization with 3GPP definitions and service classes. Contributions are invited]

# Generic reference Model

This section provides a general framework architecture that will be used as a guide to specific architectural needs that will be addressed in subsequent drafts that will follow this.  To motivate innovation and creation of new applications and to address the needs of users and providers of application and networks several types of architectures are envisioned such as centralized, decentralized and possible combinations of these two.

Figure 1 Generic QoS architecture

In Fig.1, a generic end-to-end QoS architecture is introduced to provide service and resource control functions in an abstract setting. In this setting a user application creates a set of low level requirements related to QoS, Security etc. This process can be implemented using a user process and the network access can be achieved by a gateway which can be provided by various types of network providers, access network providers or by another application provider. These requirements are signaled to a service and resource control function by the User Gateway. A user gateway functions as an agent with generic functions and acts as a proxy functional entity that creates resource and service allocation requests and oversees the validation process. Generic QoS Control and Management Function in this abstract setting is responsible to provide admission control, service control functions as well as allocation of resources based on the general constraint based requirements. Path selection based on QoS parameters and network route optimization are functions provided among other functions, such as Monitoring, SLS, Policy management, security, authentication etc.. Since this is a generic architecture, implementation details are not provided here. Some of the possible implementations are distributed, centralized or hybrid resource and service control. After the resources are determined and requested service level requirements are satisfied and the necessary resources are allocated, the provided services and resources are guaranteed. This guarantee mechanism is provided by a framework that establishes measurement and validation mechanisms for each resource request across entities. In addition to these a synchronization mechanism is provided to have the network measurement, resource allocations and other necessary data to be synchronized across the network.

# Interfaces and Functional Information Model

[Ed. Note. This section will describe interfaces and relevant requirements as well as Functional Information Model based. Signaling documents in SG11 will be referred and considered.Contributions are invited]

## UNI

## NNI

# QoS mechanisms

Some of the proposed mechanisms are provided below:

Admission Control

Resource Control and Reservation

Parameter Tuning

TE

QoS Routing Constraint-based routing

Traffic Data Analysis

Anomaly Analysis

Traffic Conditioning

Marking

Shaping

Policing

Congestion Avoidance

SLS Management

Capacity Management

Monitoring Measurement

Reliability Management

Policy Management

[Ed. Note. This section will describe QoS mechanisms that are needed in the generic and specific architecture interfaces Contributions are invited]

## QoS mechanisms in CPN

[Ed. Note. This section will describe QoS mechanisms in CPN. Contributions are invited]

## QoS mechanisms in access network

[Ed. Note. This section will describe QoS mechanisms in access networks. It is related to the findings of general access network QoS mechanisms that is being developed under Y.ipaqos]

## QoS mechanisms in core network

[Ed. Note. This section will describe QoS mechanisms in core networks. It will contain some of the material listed in NGN-WD-87 listed under Y.e2eqos. Contributions are invited]

## Interdomain and Interworking QoS mechanisms

[Ed. Note. Me3chanisms of the QoS architecture relevant to be given to ensure the interworking between different domains, different technologies and different authorities. Contributions are invited]

# Interaction with AAA system

[Ed. Note. This section will describe interaction of the architecture with AAA systems. It will contain some of the material listed in NGN-WD-87 listed under Y.e2eqos. Contributions are invited]

# Interaction with network management system

[Ed. Note. This section will describe interaction of the architecture with NMS. It will contain some of the material listed in NGN-WD-87 listed under Y.e2eqos. Contributions are invited]

# Other considerations

## Business Considerations

This general model allows users to access the networks and other users through a User Gateway functionality to provide various services. This is a generic device that allows various access network realizations and transport networks to provide end-to-end services. These gateways can be provided by different types of network providers (mobile, fixed access, or other types of service providers). Depending on the provider supplying the User Gateways there will be different requirements and functionalities. Service control, admission control and resource and QoS support will be based on these. New business models are needed for each case since the access and transport collaboration is quite different in fixed and wireless providers. This will involve service agreements and requirements, security issues, and revenue sharing as well as service verification among network entities. The mechanisms can support the centralized versions as well as totally distributed and hybrid implementations. Enabling peer-to-peer applications are also important consideration in any end-to-end architecture scenarios. One important consideration of many of these applications is to have charging and revenue sharing mechanisms and the provided architecture have functional blocks that can address each of these issues.

[Ed. Note. This section will describe general business considerations relevant to this framework draft. Contributions are invited]

Appendix 1 –Scalability consideration

The following components and methods need to be considered for scalable systems and scalability requirements.

Number and size applications

Number of client systems supported

Number of concurrent users

Load growth and application load profiles

Geographical distribution of load

Load distribution by time (daily, time of day, seasonal etc.)

Load balancing strategies

Administration and maintenance considerations

Caching and replication methods

Multicast mechanisms used to increase scalability

Number of simultaneous connections,

Size of data and control messages exchanged,

Usage of multi-threading,

Distribution and managing objects

For the distributed resource layer: network capacity, hardware configuration, replication mechanisms, fault-tolerance, transport layer, and bridging technologies.

Middleware products and their performance

Number and arrangement  of servers

Authentication and user profile management

Hardware and software limits of each product (CPU, buffers. etc.)

Configuration and tuning used in basic hardware and software (routers, gateways, softswitches etc.)

A detailed example study providing guidance needs to be developed that shows the impact of these factors on scalability.

In addition several other important factors that help provide scalable systems with necessary QoS support need to be will  developed such as:

Scalability testing

Monitoring and Measurements

Performance : response time, throughput, etc

Device, OS, application server, DB, I/O, and middleware limits- configuration parameters

Load Balancing

Methods of load balancing-measurement based auction based

Replication and cashing

Distributed and centralized schemes

Multicasting schemes

Modelling, Analysis and Simulation

The usage of modelling, analysis and simulation should be investigated on the determination of scalability requirements of NGN applications and architectures.

Annex A.  Supplementary material to be inserted into the main document

[Ed. Note. This part copies some sections from TRQ document with minor modification, and needs editing and further development,user/TE needs to beclarified]

QoS signalling requirements are expressed in terms of attributes include the following:

- the network QoS Class (i.e., Y.1541 [7]/Table 1);

- the network capacity required, at both the application and network (i.e., Y.1221 [9]) levels;

- the reliability/priority with which the service is to be sustained; and

- other elements of QoS.

- Note that the complete set of classes for reliability/priority is yet to be defined.

This document recognises that an automated system for obtaining User-to-User QoS on IP Networks, and on combinations of various network technologies, will require standard signalling protocols for communicating the requirements among the major entities.  For the purposes of this document, these entities are defined as:

- Users and their end Terminal Equipment (TE); and

- Network Service Providers/Operators and their equipment, especially equipment implementing the inter-working and signalling function between networks, and between users and networks.

# Requirements

Authentication of User and Network Peers is a prerequisite for QoS signalling.  Authentication may be accomplished by static extension of the zone of trust, or through an Authentication Protocol, which is beyond the scope of these requirements.

# User-Network Signalling

The following requirements apply to QoS Signalling between Users (or their terminal equipment) and the responsible network entity. Attributes of a User QoS Request

It shall be possible to derive the following service level parameters as part of the process of requesting service: (Some of these  may not be explicitly communicated by the user.)

- QoS class from Y.1541 [7]

- peak rate (Rp)

- peak bucket size (Bp)

- sustainable rate (Rs)

- sustainable bucket size (Bs)

- maximum allowed packet size (M)

It should be possible to derive the following service level parameters as part of the process of requesting service:

- the Reliability/Priority with which the service is to be sustained, and

- other elements of QoS.

Note that the complete set of classes for Reliability/Priority is to be defined.

Users must be able to initiate requests for service quality. Such requests may have the following attributes if the TE has this ability:

- the network QoS Class (e.g., Y.1541 [7]/Table 1);

- the network Capacity required, at both the application and network (e.g., Y.1221 [9]) levels;

- the Reliability/Priority with which the service is to be sustained; and

- other elements of QoS.

Note that the complete set of classes for Reliability/Priority is to be defined.

Optional attributes include the user Application type and quality from among several quality categories, when such categories are available.  The type of application may be completely specified from the chosen quality category.

Terminal Equipment (TE) may compose the detailed request on the user's behalf, possibly based on configurations set by the user or equipment installer. Many TE may have the flexibility to match the user's request for application quality with network QoS classes by selecting parameters such as source coder type and packet size.

# Omitting Attributes of a User QoS Request

Network QoS Class, Capacity, and Reliability/Priority are required attributes; others are optional. The Network Provider may assign default values for omitted attributes.

For example, Speech quality categories have been defined in ITU-T Rec. G.109 [12], but there is no comparable standard range of quality categories for Web browsing, financial transactions, or many other applications of networks (each is associated with a limited quality range in new ITU-T Rec. G.1010 [13). ITU-T Rec. P.911 [14] tabulates quality categories for Multimedia Communication (also known as video/audio/data conferencing) and Television applications. Users may simply wish to make requests for capacity, network QoS class, and reliability.

A user/TE may make a service request without providing explicit QoS parameters.In case where explicit QoS request is initiated by TE the following requirements apply.

Form of a Verifiable User QoS Request

The QoS requests made by the user/TE must be in terms that the network understands, especially the parameters for Network QoS. The Network QoS Classes and Network Capacity specifications in the signalling protocol must contain values that are verifiable by users (the classes in Y.1541 [7] meet that requirement). TE may conduct measurements to ensure that the committed performance and capacity levels are achieved by the network(s).

### Special Case of User QoS Request to support Voiceband Channels

When the user/TE request is for a voiceband channel (to support speech or voice band modems), the QoS request (or other associated message) should contain the preferred voiceband codec and packet size. Other optional parameters may be included to indicate, for example, the use of silence suppression, the need for network echo cancellation, and alternate codecs/packet sizes.

Many of the capacity attributes will be determined by this codec choice. Also, the network operation benefits from knowledge of the codec when the need for voice transcoding can be identified (and possibly avoided). However, much of the negotiation of application parameters takes place beyond the network's purview.

### Flow Control for User QoS Requests and Re-requests

The TE must wait X seconds before re-submitting a request, and may have a maximum of Y simultaneous requests outstanding.  Time-outs for re-submission will increase exponentially. The protocol must be "congestion-aware," using failed requests as implicit indications of congestion or using explicit notification of congestion, if available.

### Network Response to User QoS Requests

Network Service providers should be able to communicate the following messages and attributes (in the case of user-network interaction):

- An Identification Code for the request exchange, to be used in this response and all messages that follow (such as User ACK, or Release, and also in Network-Network messages). When used together with other information, such as Src address, each request can be uniquely referenced.

- The simple acknowledgement and acceptance of user/TE requests.

- The performance level expected. The ability to achieve a performance level that is better than an aspect of the QoS Class response, if the network operator desires.  This indication may be made for a single performance parameter, or for a combination of parameters.

- The ability to reject a request and, at the same time, to offer a modified service level that can be met. The response may modify the request and may include commitments to an alternate QoS Class, a lower capacity, and other indications such as those in item 3.

The processing of each request and determination of acceptance require considerable work on behalf of the network provider/operator.  Networks may wish to indicate a maximum time interval for which the response is valid.

### User Answer to Network QoS Response

The final decision to accept or reject an offered service is left to the user/TE. This completes a Request-Offer-Answer exchange.

## QoS Signalling at the Network – Network Interface

Network-network interfaces include the QoS related interfaces between network providers and between service provider and network providers in case that they have different ownership.〔Ｅｄ.note. This part needs to be further clarified〕

This section treats the case where multiple networks co-operate to realise the end-to-end connectivity desired.  Beyond the applications considerations mentioned above, network providers/operators primarily deal with Network QoS Classes, Network Capacity, and Reliability. Network-network signalling is the principal way for networks to determine multi-network compliance with QoS classes, since fixed performance allocations are not currently possible on IP Networks.

Network - Network signalling shall support the determination of the QoS Class offered to the user/TE, by communicating both the Network QoS Class requested, and the extent to which each specified parameter is already consumed.  This implies that each network knows the performance from the entrance node to the (most likely) exit node(s) for the network that has the best opportunity to complete the end-end path. Policies may also determine the next network chosen. The best-next network receives the network-network signalling request.

Networks shall determine if the desired capacity and reliability are available to support the specified Network QoS Class from entrance to exit node(s).

### Attributes of a Network QoS Request

The attributes of the network's request are:

- the network QoS Class (e.g., Y.1541 [7]/Table 1), along with the consumption of individual objectives that are specified by the class;

- the network Capacity required, at both the application and network (e.g., Y.1221 [9]) levels;

- the interconnecting point(s), where user/TE traffic will leave the requesting network and enter the next network;

- the Reliability/Priority with which the service is to be sustained; and

- other elements of QoS.

- Note that the complete set of classes for Reliability/Priority is yet to be defined.

Optional attributes include the user Application type and the quality category, when such categories are available and meaningful.

### Omitting Attributes of a Network QoS Request

Network QoS Class, Capacity, and Reliability/Priority are required attributes; others are optional.

### Performance Requirements for QoS Requests and Re-requests

An important aspect of the requirements for a signalling protocol is the performance requirement associated with that protocol. The most important areas where signalling performance requirements need to be established is the average / maximum latency for the establishment of service and the average / maximum latency for the re-establishment of service in the event of a network failure. The latency requirements described above for the signalling protocol depend on the performance characteristics of the underlying transport network. Because of this, performance requirements for the transport network must be specified along with the latency requirements for the signalling protocol. The combination of these factors leads to the following formal performance requirements for the signalling protocol.

- Networks designed to meet the signalling protocol requirements specified in this section should be capable of supporting the network performance objectives of QoS class 2 in Y.1541 [7].

- The average delay from the time of a UNI or NNI request for service to the acceptance or rejection of this service request by the network should be <800 msec.

- The maximum delay from the time of a UNI or NNI request for service to the acceptance or rejection of this service request by the network should be <1500 msec.

- The average delay from the time of a network failure to the time of re-establishment of service at any UNI or NNI interface should be <800 msec.  (This does not address restoration of failed links.).

- The maximum delay from the time of a network failure to the time of re-establishment of service at any UNI or NNI interface should be <1500 msec.

### Response to a Network QoS Request

Network providers shall be able to respond with the following messages and attributes (in the case of network-network interaction):

- The ability to correlate all responses and subsequent requests to the original request is required. An Identification Code is one example.

- The simple acknowledgement and acceptance of requests.

- The ability to indicate a performance level that exceeds an aspect of the request/response is required, but the indication to other entities is a network option.

- The terminating network supporting the destination UNI shall offer a modified service level if the original service level cannot be met. The modified service may include commitment to an alternate QoS Class, a lower capacity, etc.

It is possible that a chain of network-network QoS requests will encounter a network that does not support the QoS signalling protocol or QoS Classes in general.  If this network is an essential section of the end-to-end path, then several results are possible. One is to reject the request, but at the same time offer an Unspecified Class (e.g., Class 5 of Y.1541 [7]), possibly with the indication of some additional parameter values.

When making entrance-to-exit performance commitments, only one of the interconnecting links will be included for all networks, except the first network which shall include both the link to the UNI and the link to the NNI (subsequent networks will include the exit link to the next interface, either NNI or UNI).

### Accumulating Performance for Additional Requests

Signalling must communicate the consumption of the network (source-UNI to destination-UNI) QoS objectives.

## QoS Release

Users and Networks shall be able to signal when a previously requested network resource is no longer needed.

## Performance

For reasons of signalling performance, the following areas should be addressed:

the number of messages required to establish, maintain and clear QoS requests should be kept to a minimum; and

the format of the IP Signalling Protocol information should be chosen to minimize message-processing delays at the endpoints.

## Symmetry of information transfer capability

The QoS Signalling protocol shall support symmetric QoS Requests.

Asymmetric QoS Requests are optional. That is, the end-to-end requests may be bi-directional where the information transfer capability in each direction might be different.

## Contention resolution

The QoS Signalling protocol shall be able to resolve all contentions with respect to resource allocation and collision.

## Error reporting

The QoS Signalling Protocol shall include mechanisms for detecting and reporting signalling procedural errors or other failures detected by the TE/Network. Service failures may also be reported to the User.

## Unrecoverable failures

The TE and Network Entities shall include mechanisms for returning the QoS protocol instance to a stable state after detection of unrecoverable failures.

## Forward and backward compatibility

The QoS Signalling Protocol shall include a forward compatibility mechanism and backward compatibility rules.

## Parameters and values for Transport connections

The signalling protocol(s) at UNI and NNI interfaces should be capable of specifying the following additional parameters as part of the process of requesting service:IP header fields: source + destination address (RFC 791 [1], RFC 2460 [2]);Source + destination port as specified in RFC 768 [4] and RFC 793 [5].

## User-Initiated QoS Resource Modification

If the QoS request is initiated by User/TE explicitly, the following requirements apply.

Either User may be able to modify the resources associated with an active Transport connection, represented by the information contained in the Transport Connection messages.

Collision of connection resource modification requests shall be avoided by the Served User.

Modification shall be performed with no loss of IP transport contents.

The use of the preferred Transport Connection messages is to avoid the need for subsequent modification of the connection resources immediately after the establishment.

User/TE (IP Endpoints) should determine, through the use of end-end application level capability signalling, the ability and support to use resources beyond those currently in use.  The support / lack of support of the capability to modify Transport Connection messages, for a Transport connection must be indicated by the originating IP Endpoint. The terminating IP Endpoint must indicate the support / lack of support of the modification capability of the Transport Connection messages. Only when both Endpoints indicate modification support can modification be attempted.

This capability uses the following objects:

- Transport Connection message Modification Support Request,

- Transport Connection message Modification Support Response.

## Emergency Service

Requirements for support of Emergency Services may be specified in a future version of this document.  The protocol will identify reserved objects, bits, etc. This topic is treated in general under reliability and priority attributes.

## Reliability/Priority Attributes

Reliability/Priority attributes are the same for User-Network and Network-Network signalling requirements. Reliability for a service can be expressed in the form of a priority level with which that service requires a particular type of network function (e.g., Connection Admission Control Priority). Hence, reliability can be requested in the form of a Priority Class for that specific network function. Two types of network functions apply for Reliability/Priority classes: Connection Admission Control and Network Restoration. As an example, emergency services can signal for the highest available connection admission control priority during emergency conditions.

No formal standards exist with respect to the qualitative (e.g., number of priority classes) or quantitative (e.g., time-to-restore) aspects of reliability. From the viewpoint of signalling, there should be a limited number of Priority Classes for all network functions in order to ensure scalability (e.g., 4 classes). The signalling protocol needs to be able to provide the capability to effectively convey these priority requests once priority level attributes are established in standards forums.

The values of IP Loss Ratio, IP Transfer Delay, and IP Delay Variation as specified in Y.1221 [9] may be derived by specifying the QoS class from Y.1541 [7] as a signalling parameter.