INTERNATIONAL TELECOMMUNICATION UNION

Radiocommunication Bureau (Direct Fax N°. +41 22 730 57 85)

| Circular Letter CCRR/10 | 5 March 2001 |
| --- | --- |

To Administrations of Member States of the ITU

Subject:	Draft Modifications to the Rules of Procedure

To the Director General

Dear Madam/Sir,

1.	Following the conclusions of WRC-2000 (Istanbul, 2000), the Radiocommunication Bureau has identified a number of cases, which require consideration of new Rules of Procedure or, modifications to, or suppression of, existing Rules of Procedure. At its 20th Meeting (11 - 15 September 2000), the Radio Regulations Board (RRB) agreed to deal with these cases in subsequent meetings taking into account the workload of the RRB and the Bureau. Sets of draft Rules of Procedure, which were intended for submission to the 21st Meeting (20 - 24 November 2000) and 22nd Meeting (5 - 9 February 2001) of the RRB, were distributed for comment in Circular Letters CCRR/7 of 22 September 2000, CCRR/8 of 30 November 2000 and CCRR/9 of 15 December 2000.

2.	In this Circular Letter (CCRR/10), please find enclosed the set of draft Rules of Procedure which are intended for consideration by the RRB at its 23rd Meeting (14 - 18 May 2001). This set does not contain draft Rules of Procedure concerning Appendices S30, S30A, and S30B which will be distributed in a separate Circular Letter. The proposals in this Circular Letter relate to the following provisions.

-	Articles Nos. S5.149 and S5.565 (Annex 1).

-	Article No. S9.52C (Annex 2).

-	Regional Agreement GE89 (Annex 3).

-	Internal working methods of the RRB (Annex 4).

-	Articles Nos. S9.15 to S9.19, S9.36, S11.17, S11.32 and Appendix S7 (Annex 5).

-	Articles Nos. S5.418 and S5.418C (Annex 6).

-	Article No. S11.31 (Annex 7).

-	Articles Nos. S11.44 and S11.44.1 (Annex 8).

-	Receivability of Space Notices (Annex 9).

-	Article S9.11A (Annex 10).

3.	It should be noted that draft Rules of Procedure concerning Regional Agreement GE89 (Annex 3) does not necessarily relate to the decisions of WRC-2000.

4.	Proposed modifications to Rules of Procedure on “Receivability” were included in Circular Letter CCRR/9 of 15 December 2000 (Annex 1 of that Circular Letter). A number of comments were received from Administrations in respect of the proposed modifications published in that Circular Letter. The Radio Regulations Board considered the proposals at its 22nd Meeting (5 - 9 February 2001) and some changes were proposed to the draft as a result. However, the RRB deferred further consideration of the draft until its next (23rd) meeting (14 - 18 May 2001). The proposed revised draft is included at Annex 9 for comment by Administrations. In considering the draft modifications to the Rules, the RRB will take account of comments already received in response to the previous Circular Letter (CCRR/9) together with any further comments made in response to this Circular Letter.

5.	The Radiocommunication Bureau intends to recommend to the RRB that the modified Rule of Procedure on Receivability should have a prospective date of effect.  In this event, the Bureau proposes to publish a Circular Letter prior to that date to provide detailed information on implementation. This information will relate, in particular, to use of the software tools for preparing and validating satellite notices together with guidance on the detailed practical application of the associated validation rules.

6.	In accordance with Article S13.17 of the Radio Regulations, these proposals included in ten annexes are made available for comment by Administrations before being submitted to the RRB pursuant to Article S13.14.

7.	To enable the Radiocommunication Bureau to prepare, translate and post on the ITU website the consolidated submission to the RRB in time for the 23rd meeting, which commences on 14 May 2001, any comments that you may wish to submit should reach the Bureau not later than 16 April 2001.

Yours faithfully,

Robert W. Jones
					Director, Radiocommunication Bureau

Annexes: 10

Distribution:

-	Administrations of Member States of the ITU
-	Members of the Radio Regulations Board
- 	Director and Heads of Department of the Radiocommunication Bureau

ANNEX 1

Rules concerning Nos. S5.149 and S5.565 of the RR

MOD S5.149

There is no allocation to radio astronomy in the bands 73-74.6 MHz (Regions 1 and 3), 1330-1400 MHz, 3260-3267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 6 650-6 675.2 MHz, 22.01-22.21 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 36.43-36.5 GHz, 168.59–168.93 GHz, 171.11-171.45 GHz (except for KOR), 172.31-172.65 GHz (except for KOR), 173.52-173.85 GHz (except for KOR) and  195.75-196.15 GHz. Notification of frequency assignments to radio astronomy stations in the bands 73-74.6 MHz (Regions 1 and 3), 1 330-1 400 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 6 650-6 675.2 MHz, 22.01-22.21 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 36.43-36.5 GHz 168.59–168.93 GHz, 171.11-171.45 GHz (except for KOR), 172.31-172.65 GHz (except for KOR), 173.52-173.85 GHz (except for KOR) and 195.75-196.15 GHz will be considered by the Bureau not to be in conformity with the Table of Frequency Allocations.

Reason: Consequential to new allocations to Radio Astronomy Service by WRC-2000.

MOD S5.565

The bands above 275 GHz are not allocated to any service. However, the administrations may use the frequency band 275-1000 GHz for experimentation with, and development of, various active and passive services. The frequency assignments notified in this frequency band will be recorded in the Master Register with favourable regulatory finding, without any examination, for information only, with reference to footnote S5.565.

Reason: Consequential to the modification to S5.565 by WRC-2000.

ANNEX 2

Rules concerning No. S9.52C of the RR

MOD 9.52C

# 1	Case of administrations not responding

With respect to administration not responding, an administration having applied the procedure shall be regarded as having successfully completed the procedure of this Article for assignments for which there was no response.

# 2	Publication of Special Sections containing the status of the coordination procedures under Nos. S9.11 to S9.14 and S9.21

2.1	Any comment which does not explicitly express objection to the request for coordination is not considered as a disagreement under No. S9.52. In case of doubt concerning the nature of comments, the administration concerned should be consulted.

2.2	The appropriate Special Section shall include the following information:

a)	the names of administrations whose disagreement to the request for coordination were received within the regulatory deadline;

b)	a Note, which reads:

“Pursuant to No. S9.52C, all administrations other than those listed above shall be regarded as unaffected, and in the case of Nos. S9.11 to S9.14 the provision of Nos. S9.48 and S9.49 shall apply.”

Reason: Consequential to addition of new provision S9.53A by WRC-2000, which covers points mentioned in paragraph 2.1 of the rule.

Annex 3

PART  A6

Rules concerning the Regional Agreement relating to the planning of
VHF/UHF television broadcasting in the African Broadcasting Area
and neighbouring countries
(Geneva, 1989) (GE89)

# Examination of notices related to the non-planned services in the bands governed by the Regional Agreement GE89

1	Sections 5.2 and 5.3 of Article 5 of the GE89 Agreement specify the procedure to be followed for the examination of the notices related to the non-planned primary services in the bands governed by the Agreement. The bands and the services concerned are summarized in the Table below.

TABLE

| Frequency band (MHz) | Services and countries within the planning area | Provisions | Notes |
| --- | --- | --- | --- |
| 47-68 | FX: AFS, AGL, BOT, BDI, CME, COD, COG, IRN, LSO, MDG, MLI, MOZ, MWI, NMB, RRW, SOM, SDN, SWZ, TCD, TZA, ZWE | S5.165 S5.167 S5.171 | 1 |
|  | MO(-AER): AFS, AGL, BOT, BDI, CME, COD, COG, LSO, MDG, MLI, MOZ, MWI, NMB, RRW, SOM, SDN, SWZ, TCD, TZA, ZWE | S5.165 S5.171 | 1 |
|  | MO: IRN | S5.167 |  |
| 174-223 | FX: IRN MO: IRN |  |  |
| 223-230 | FX: IRN MO: IRN AL: ARS, BHR, IRN, OMA, QAT, UAE | S5.247 |  |
| 230-238 | FX: from all parties to the Agreement (excepting those referred to in No. S5.252) MO: from all parties to the Agreement (excepting those referred to in No. S5.252) AL ARS, BHR, IRN, OMA, QAT, UAE | S5.247 | 2 3 |

Reason: To correct apparent errors and to reflect more accurately the allocation situation, as well as the situation of some countries in respect to GE89 Agreement.

TABLE (end)

| Frequency band (MHz) | Services and countries within the planning area | Provisions | Notes |
| --- | --- | --- | --- |
| 246-254 | FX: from all parties to the Agreement (excepting those referred to in No. S5.252) MO: from all parties to the Agreement (excepting those referred to in No. S5.252) |  | 2 |
| 470-790 | FX: IRN MO: IRN |  |  |
| 790-862 | FX: from all parties to the Agreement MO: IRN |  |  |
| NOTE 1 – The additional allocations to countries referred to in No. S5.171 are limited to the band 54 – 68 MHz. NOTE 2 – In the frequency bands 230-238 MHz and 246-254 MHz, in the examinations under § 5.2 of the Agreement, account is taken of only those frequency assignments in the broadcasting service which are entered into Plan following a successful application of the procedure referred to in No. S9.21, as required by Resolution 1 (GE89) and No. S5.252. NOTE 3 – The additional allocation to countries referred to in No. S5.247 is limited to the band 223 - 235 MHz. |  |  |  |

2	The frequency assignment notices related to the aeronautical radionavigation service of Nigeria, whose allocation is governed by No. S5.251, shall not be subject to the examinations referred to in § 5.2 of Article 5 of the Agreement, since these notices are subject to the application of the procedure of No. S9.21.

3	The frequency assignment notices related to services and countries referred to in Nos. S5.164, S5.235, S5.243 and S5.316 shall not be subject to the examinations requested by § 5.2 of Article 5 of the Agreement, since their allocation is subject to not causing harmful interference to, or claiming protection from, the broadcasting service. Consequently they will be recorded in the MIFR under the conditions of No. S5.43 vis-à-vis the broadcasting service (symbol R in column 13B2).

Annex 4

PART  C

MOD

Rules concerning internal working methods of 
the Radio Regulations Board

# Introduction

These working methods are intended to complement basic provisions which have been included in Article 14 of the Constitution and Article 10 of the Convention and in accordance with the provisions of No. 147 of the Convention (Geneva, 1992).

# 1	Board meetings

1.1	A meeting of the Board will be held approximately every three months and the specific date for the next meeting will be decided at the end of each Board meeting. Any subsequent change of date will only be made with the agreement of all the members.

1.2	The duration of the meeting will be decided by the Chairman and the Executive Secretary, taking into consideration the agenda of the meeting.

1.3	A convening notice which includes the specific agenda and the duration of the meeting should be prepared by the Executive Secretary of the Board after approval by the Chairman at least three weeks before the meeting and sent to members of the Board.

1.4	The agenda should include the following, as required:

a)	approval or confirmation of the minutes of the previous Board meeting;

b)	approval of the new or revised Rules of Procedure (CS95);

c)	consideration of Review of Findings which cannot be resolved by the use of the Rules of Procedure (CV171);

d)	consideration of Reports on Harmful Interference (CV140, CV173);

e)	consideration of any other matters which cannot be resolved by the Bureau (CS96);

f)	matters concerning simplification of the Radio Regulations (Report on VGE meeting and exchange of views) (transitional arrangement) (Resolution 1, APP, Geneva, 1992);

g)	matters which should be referred to the Radiocommunication Conference (CS95, Resolution 1, APP, Geneva, 1992);

h)	any item requested by any member of the Board;

i)	any item requested by the Director of the Radiocommunication Bureau;

j)	miscellaneous (CS97, etc.).

1.5	All documentation should be prepared by the Executive Secretary and distributed to the members normally two weeks before the meeting.

1.6	Meeting attendance will be as follows:

–	Members

–	Executive Secretary/Director of the Radiocommunication Bureau

–	Secretary

–	Minute writer(s)

The Director of the Radiocommunication Bureau may be accompanied by any necessary staff of the Bureau on a case-by-case basis.

1.7	The Board shall endeavour to reach its decisions unanimously. If it fails in that endeavour, a decision shall be valid only if at least two-thirds of the members of the Board vote in favour thereof. Each member of the Board shall have one vote; voting by proxy is not allowed (see CV146).

1.8	The minutes should clearly indicate whether the decision was unanimous or by majority. Minutes should be approved after consultation between members of the Board and shall normally be circulated at least one month before the start of the following meeting sby the Executive Secretary.

1.9	A summary of decisions should be prepared by the Executive Secretary in a tabular form (subject, decision, follow-up) and approved by the Chairman of the Board.

# 2	Rules of Procedure

## 2.1	Establishment or revision of Rules of Procedure

2.2	Draft new Rules or draft revision of the existing Rules should be prepared by the Director of the Radiocommunication Bureau and the draft should be submitted to the Board for approval.

2.3	In submitting the draft Rules, the Director should also submit relevant material which explains the practical necessity of the new or revised Rules, as well as its possible impact on administrations, and other background information.

## 2.2	Possible review of Rules after publication

2.2.1	The Rules are effective when approved by the Board. However, if comments are received from any administration after publication, the Board will review the Rules taking into account those comments collected by the Director from administrations and submitted to the Board and revise the Rules, if appropriate (CV169).

2.2.2	If there is continuing disagreement, the matter shall be submitted to a forthcoming World Radiocommunication Conference (CS95).

# 3	Review of Findings

3.1	Any review of a Finding which is requested by an administration and which cannot be resolved by the use of the Rules of Procedure shall be submitted to the Board with the information indicated below (CV171):

a)	Brief explanation on the case including the history of the case.

b)	All relevant documents which were received from the concerned administrations and those relevant documents which were sent by the Director of the Radiocommunication Bureau to that administration.

c)	Brief statement by the Director to clarify the view of the Radiocommunication Bureau.

3.2	The Board will decide on the appropriate action.

# 4	Recommendations on interference

4.1	When an administration has requested an investigation for the resolution of a case of harmful interference in accordance with CV173, and the case has not been resolved within three months after efforts by the Director following the established procedures in the Radiocommunication Bureau, a report shall be submitted to the Board, which includes the following:

a)	Brief explanation of the case which will include the degree of reported interference, history of the reported interference and the status of notification of the concerned assignments.

b)	The statements by the administrations concerned which clarify the views of those administrations.

c)	Draft recommendation to those administrations.

4.2	The Board will decide on the appropriate action.

# 5	Any other matters that cannot be resolved by the Bureau through the application of the Rules of Procedure

The Director may raise any such matter. Such cases will be handled by the Board on a case-by-case basis (CS96).

Reason: Consequential to the modification to S13.18 by WRC-2000.

# ANNEX 5

Rules concerning Nos. S9.15 to S9.19, S9.36, S11.17, S11.32 
and Appendix S7 of the RR

MOD S9.15 to S9.19

1	The expression in Nos. S9.15, S9.17 and S9.17A of “band allocated with equal rights” is understood to mean bands with the same category of allocation. According to footnote No. 1 to § 1 of Appendix S5 the “equality of right” condition is extended to all coordination forms under Nos. S9.15 to S9.19.

2	Taking into account Article S59, Resolution 59, Resolution 541 and the discussion in WRC-2000, the Board concluded that provisions S9.17 – S9.19 and Appendix S7 as modified by WRC-2000 shall enter into force as of 1 January 2002 except with respect to submissions under Appendices S30 and S30A to which S9.17A, S9.19 and Appendix S7 shall apply as of 3 June 2000. Table S5-1 of Appendix S5 as modified by WRC-2000 with respect to application of S9.15 to S9.19 shall enter into force as of 1 January 2002.

3	See also Rules of Procedure concerning Appendix S7.

Reason: S59.5 states that provisions revised by WRC-2000 shall enter into force on 1 January 2002 except the cases where Resolutions listed in S59.6 stipulate other effective dates of application.  There is no description of the effective dates of application of S9.17-S9.19 and Appendix S7 in those Resolutions.  When the revision of Appendix S7 was discussed during WRC-2000, advancement of effective date of application was not mentioned.  However, in Resolution 541 it is stipulated that the Regions 1 and 3 Plan, the list and their associated procedures, together with Annexes thereto, shall enter into force as of 3 June 2000.  It is clear that during WRC-2000 Article 6 of Appendix S30 and Article 7 and Annex 4 of S30A were revised based on S9.17A, S9.19 and Appendix S7 as revised by WRC-2000.  Consequently the above-mentioned Rule of Procedure to clarify the situation relating to effective dates of application of S9.17 – S9.19 and Appendix S7 is proposed.

Res. 59 stipulates that provisions S9.15, S9.16 and Table S5-1 of Appendix S5 shall enter into force as of 3 June 2000.  On the other hand, Appendix S7 as modified and paragraph 3 of Annex 1 of Appendix S5 as suppressed by WRC-2000 are not included in the list of provisions which shall enter into force as of 3 June 2000.

Moreover, coordination among administrations under S9.17 has been initiated based on the former Appendix S7.  It might not be appropriate to apply the new Appendix S7 as of 3 June 2000 due to the fact that it might result additional coordination requirements to notices notified after 3 June 2000 but before 1.1.2002.  It should also be noted that the definitive version of software to determine coordination area based on new Appendix S7(WRC-2000) will be available to administrations before 1.1.2002 (preliminary version is available on the ITU Web site for testing).

Paragraph 2 of the previous Rule is moved to Rules of Procedure relating to Appendix S7 which deals with methods to identify coordination requirements.

MOD S9.36

1	Under this provision, the Bureau “shall identify any administrations with which coordination may need to be effected”. In applying Appendix S5 with respect to No. S9.21, the Bureau uses the following calculation methods and criteria2:

–	space network vs. space network: Appendix S8;

–	earth station vs. terrestrial stations (and vice versa): Appendix S7;

–	transmitting terrestrial stations vs. receiving space stations: criteria of Article S21;

–	transmitting space stations vs. terrestrial services: pfd limits defined in Article S21;

–	between stations of terrestrial services in some specific frequency bands: Rules of Procedure B4, B5 and B6 as appropriate.

Reason:	Consequential to the adoption by WRC-2000 of the revised footnote S5.488 and proposed deletion of Rules of Procedure B1 and B2.  It becomes absolutely clear that S9.21 coordination is no longer applied to the band dealt by Appendix S30 because of modification of S5.488.  Therefore, references to methods and criteria used in Appendix S30 were deleted.

2	For coordination requests under Nos. S9.11 to S9.14 and S9.21, it is to be noted that irrespective of the identification by the Bureau under No. S9.36 (see footnote S9.36.1), any administration, even one which was not identified, may object to the published assignment under No. S9.52 and any administration, including one identified by the Bureau, that has not commented on the proposed use within the regulatory time limit is considered to have no objection to that use in accordance with No. S9.52C.

3.	See also Rule of procedure concerning Appendix S5.

Reason:	Consequential to the draft revision of the Rules of Procedure relating to No. S9.27.

MOD S11.17

This provision and provisions of Nos. S11.18 to S11.21A identify assignments to terrestrial stations to be notified individually. All other assignments1, 2, 3 can be notified either as a typical station or as individual stations, as the administration concerned considers appropriate. The frequency assignments which shall be notified individually, under the procedure of Article S11, are the following:

1	Assignments to stations covered by the Allotment Plans of Appendices S25, S26 and S27 (No. S11.18) and by any Frequency Assignment Plan.

2	Assignments to stations of the broadcasting service in any band (No. S11.19).

3	Assignments to stations of all terrestrial services which are within the coordination area of an earth station (No. S11.20) if the notified bandwidth of the terrestrial station is situated wholly or partially within a frequency band which is allocated with equal rights to terrestrial and space services where coordination is required under Appendix S5, Table S5-1.

According to No. S11.20, no notification of a typical terrestrial station is receivable if the terrestrial station is within the coordination area of an earth station. In view of the current difficulties of the Bureau to ascertain, at the time of the receipt of the notice, whether a terrestrial station is situated within the coordination area of an existing earth station or one for which the coordination has been effected or initiated, the Board instructed the Bureau to encourage administrations to submit individual notices to terrestrial stations in every case where the notified bandwidth of the terrestrial station is situated wholly or partially in any of the bands shared between terrestrial and space services with equal rights if the allocation to the space service comprises the space-to-Earth direction. The Bureau may also accept a notification to a typical station in these bands, if the notifying administration so wishes, under the understanding that the subject notice form may be returned to the notifying administration at a later stage, if the Bureau’s examinations confirm that the notified geographical area of operation of the typical terrestrial station overlaps the coordination area of an Earth station. Such a notice, when published in Part A of the BR Weekly Circular, shall bear a special symbol making reference to this Rule of Procedure.

4	Assignments to any terrestrial stations in bands shared with space services with equal rights which exceed the limits of the terrestrial station parameters specified in Tables 8a, 8b, 8c and 8d of Appendix S7 and in No. S21.3 (No. S11.21).

The Board concluded that the first part of this provision is intended to afford appropriate protection to receiving earth stations when the terrestrial stations are using a high e.i.r.p. Given the variety of conditions specified in the referred Tables of Appendix S7, the Board decided that administrations shall submit individual notice whenever the e.i.r.p. exceeds the following limits:

50 dBW (for analogue modulation) and 37 dBW (for digital modulation), in any of the frequency bands below 3 GHz that are mentioned in Tables 8a and 8b;

55 dBW (for analogue modulation) and 42 dBW (for digital modulation), in any of the frequency bands between 3 GHz and 15 GHz that are mentioned in Tables 8b and 8c;

55 dBW (for analogue modulation) and 40 dBW (for digital modulation), in any of the frequency bands above 15 GHz that are mentioned in Tables 8c and 8d.

.

5	Assignments to terrestrial stations in the frequency bands listed in Table S21.2 (No. S11.21A).

The Board concluded that this provision is intended to protect the geostationary-satellite orbit. It should be applied to all terrestrial services in the bands referred to above, irrespective of their category of allocation.

Reason: Consequential to the modification to Appendix S7.

MOD S11.32

Paragraph 1 to 3

NOC to the RoP adopted at 22nd meeting of RRB

Paragraph 4

# 4	Examination of frequency assignment notices to stations in terrestrial services in the bands shared with equal rights with space service

4.1	In its examinations of a frequency assignment notice for a station of a terrestrial service, in the bands shared with equal rights with space services, from the point of view of its conformity with the procedures relating to coordination with respect to earth stations of other administrations, the Bureau takes into account those earth stations which are recorded in the Master Register. To this effect, the Bureau uses the coordination contour associated to the respective earth station and calculated in accordance with the calculation method and parameters being in force at the time of notification of the earth station.

If, within a period of three years following the date of notification of the terrestrial station, the Bureau receives a comment from another administration, indicating that the concerned assignment was included in a coordination procedure initiated by this later administration pursuant to No. S9.29 in respect to its earth station(s) coordination under No. S9.15 or S9.17, and was not agreed to, or was agreed with different technical characteristics, the Bureau will review the situation in accordance with the relevant provisions of Article S14 and will proceed accordingly.

Reason: WRC-2000 modified the method to determine coordination area under S9.15, S9.17 and S9.17A by adopting a new Appendix S7.  The Board is of opinion that the new calculation method in Appendix S7 shall not be applied retroactively.  See also minutes of the fourth Plenary meeting (Doc. 533).

Paragraph 5 to 6

NOC to the RoP adopted at 22nd meeting of RRB

ADD

Rule concerning

APPENDIX S7 to the RR

1	Cases have arisen in practice where the coordination contour around an earth station exceeds several hundreds of kilometers and overlaps only a very small part of the territory of an administration (less than a few tens of kilometers). Considering that several conservative assumptions are used in calculating the coordination distance, the Board decided that when the overlapping is less than 5% of the coordination distance, no coordination is required.

2	The examination with respect to the application of Nos. 9.15, 9.17 and 9.17A involves the application of the calculation method of Appendix S7, using the system parameter values specified in its Tables 7 to 9. In view of the fact that these tables contain multiple sets of parameters in several places (e.g. for analogue and digital modulation), which result in different coordination contours, and in order to ensure completeness in the relevant checks of the conformity with the coordination requirements, the Board decided to instruct the Bureau to use the set of parameters which produces the largest coordination area in a given frequency band whenever multiple sets of parameters are indicated in these Tables. In addition, as the system parameter tables have incomplete information in some of the columns, the Board instructed the Bureau to apply the following approach in this regard:

- To use the parameters in Table 7 for determining the coordination area for a transmitting earth station in a service not mentioned in that table, but allocated with equal rights, based on the fact that all parameters related to the earth station needed for the calculation can be found in the notice form;

- To use the parameters in Table 8 for determining the coordination area for a receiving earth station in respect to a terrestrial service not mentioned in that table, but allocated with equal rights, under the assumption that the concerned terrestrial service has the same potential of interference as the other terrestrial services listed in the table (see also paragraph 4 of Rules of Procedure under S11.17).

Reason: Most sets of parameters covered by Rules of Procedure Section B1 and B2 which are proposed to be suppressed are now included in the tables of Appendix S7.  However, some allocations of services are not listed in the tables.  Parameters available for different services in the tables can be used for certain cases of those allocations.  In case of Table 7 a service of a transmitting earth station can be different from those in the table because all parameters relating to the transmitting earth station necessary for the calculation can be found in the notice form. In case of Table 8 a service of a transmitting terrestrial station can be different from those in the table because assignments to any terrestrial stations exceeding the parameters listed in the table which are considered as the worst case will be dealt under the paragraph 4 of Rules of Procedure under S11.17.  If parameters are not available in the Tables, the Bureau should draw the attention of ITU-R in view of accelerating its studies requested in Resolution 74.

Paragraph 1 of the proposed Rule is moved from Rules of Procedure relating to S915 to S9.19 because it is more appropriate to include it in the rule concerning methods to identify coordination requirements.

SUP

# PART  B

## SECTION  B1

# Rules concerning calculation methodology for the “agreement area”
related to a transmitting earth station in the frequency bands
below 1 GHz, in application of the procedure of No. S9.21

SUP

# PART  B

## SECTION  B2

# Rules concerning calculation methodology for coordination contours

Reason: Most of the cases covered by the former Rules of Procedure Section B1 and B2 are now included in Appendix S7.  See also paragraph 4 of Draft Rules of Procedure relating to S9.15 to S9.19.

ANNEX 6

Rules of Procedure concerning S5.418 and S5.418C

ADD S5.418

1.	In accordance with footnote S5.418, as modified by WRC-2000, the use of the band 2630 - 2655 MHz by non-geostationary satellite systems in the broadcasting-satellite service (sound) in certain Region 3 countries listed in this footnote is now subject to the application of Resolution 539 (WRC-2000), as of 3 June 2000. Resolution 539 (WRC-2000) resolves that, before an administration notifies to the Bureau or brings into use a frequency assignment for a broadcasting-satellite service (sound) system using non-GSO satellite in the band 2630 - 2655 MHz, for which complete Appendix S4 coordination information, or notification information, has been received after 2 June 2000, it shall seek agreement of any administration in Regions 1, 2 and 3 having a primary allocation to terrestrial services in the same frequency band if the power flux-density produced on its territory exceeds the threshold values contained in the Resolution.

2.	Based on the above, for the application of MOD S5.418, the Board instructs the Bureau to act as follows:

  - For coordination requests (Article S9) for the non-GSO BSS (sound) systems received as of 3 June 2000, to establish the list of administrations the agreement of which is required based on power flux-density thresholds included in resolves 2, Resolution 539 (WRC-2000) and to publish this list in the relevant Special Section of its IFIC.

  - For notifications (Article S11) of the above-mentioned non-GSO BSS (sound) systems received as of 3 June 2000 for which agreement requirements under Resolution 539 (WRC-2000) were established and published in accordance with paragraph 2.a. above, to examine whether the required agreements (explicit) are properly reflected in the Forms of Notice and to establish the finding under S11.32 accordingly.

  - For request of assistance received from administrations in case of difficulties to obtain the required agreement from other administrations (identified as affected) to apply the corresponding/analogous provisions of Article S9 or former Article 11.

3.	The Board studied the relationship between the application of Resolution 539 (WRC-2000) to coordination requests of non-GSO BSS (sound) systems received as of 3 June 2000 and the procedure under No. S9.11. The Board’s conclusions are as follows:

a.	Coordination requests of non-GSO BSS (sound) systems received as of 3 June 2000 will be subject to the application of Resolution 539 (WRC-2000) only and not to the application of No. S9.11.

b.	For notifications (Article S11) of non-GSO BSS (sound) systems received as of 3 June 2000, the Bureau shall examine and establish the finding under S11.32 vis-à-vis Resolution 539 (WRC-2000) requirements only and not vis-à-vis No. S9.11 requirement.

Reason:	The Board understands S5.418, as modified by WRC-2000, together with Resolution 539 [COM4/6] (WRC-2000) as a decision of WRC-2000 to subject the use of the band 2630 – 2655 MHz by non-GSO BSS (sound) systems in certain Region 3 countries to seeking agreement from any administration in Regions 1, 2 and 3 having a primary allocation to terrestrial services if the PFD trigger values on its territory are exceeded and, at notification stage, to verify the existence of agreements from all such administrations and to accordingly establish the findings under S11.32, and by so-doing to replace No. S9.11 procedure by Resolution 539 procedure for such use.

#### ADD S5.418C

1.	In accordance with footnote S5.418C, as added by WRC-2000, the use of the band 2630 - 2655 MHz by geostationary-satellite networks is now subject to the application of the provisions of No. S9.13 with respect to non-geostationary satellite systems in the broadcasting-satellite service (sound), as of 3 June 2000. Resolution 33 (WRC-97) resolves that for satellite networks for which the API or the request for coordination has been received by the Bureau prior to 1 January 1999, the procedure in Sections A to C in Resolution 33 shall be applied. The Board studied the applicability of No. S9.13 coordination in the above context and understands No. S9.13 coordination to apply as follows:

| GSO Satellite network | Date of Receipt of API | Date of receipt of Coordination information | No. S9.13 applicability | Comments |
| --- | --- | --- | --- | --- |
| FSS |  3.6.2000 |  3.6.2000 | NO | S5.418C |
| (Region 2) | - |  3.6.2000 | YES | S5.418C |
| BSS | - |  3.6.2000 | NO | S5.418C |
|  |  |  3.6.2000 | YES | S5.418C |

Reason:	For reciprocity reasons, the Board understands No. S9.13 coordination to apply also to satellite networks for which the API or the coordination request has been received by the Bureau prior to 1.1.1999.

ANNEX 7

Rules concerning No.S11.31 of the RR

MOD S11.31

1	Provision No. S11.31.2 requires that the “other provisions” mentioned in No. S11.31 should be identified and included in the Rules of Procedure. This chapter intends to answer the above problem.

The regulatory examination under No. S11.31 includes the following4:

–	conformity with the Table of Frequency Allocations, including its footnotes and any Resolution or Recommendation which is referred to in such a footnote;

–	the successful application of No. S9.21, when mention is made of that provision in a footnote (See also Rules of Procedure relating to Nos. S9.21, S11.31.1 and S11.37);

–	all “other” mandatory provisions that are contained in Articles S21 to S57, in Appendices to the Radio Regulations and/or in Resolutions (except Resolution 49 (WRC-97/WRC-2000, as applicable) for which there is a separate procedure as contained in the Rules of Procedure concerning No. S11.44) that are relevant to the service in the frequency band in which a station of that service operates.

2	The list of these “other provisions”, referred to in No. S11.31.2, with respect to which the notices to stations in terrestrial (§ 2.1 to 2.5.2) or space services (§ 2.6 to 2.6.6) are examined, is given below:

2.1	Broadcasting service: Those contained in No. S23.7 concerning the power limit (50 kW) of the broadcasting transmitters operating in the Tropical bands in the frequency bands listed in No. S23.6.

2.2	Fixed service: Those of No. S24.2 which stipulate that F3E and G3E emissions are prohibited in the fixed service below 30 MHz;

2.3	Aeronautical mobile service: There are mandatory provisions only for the frequency bands that are allocated exclusively to the aeronautical mobile service. These provisions (obligatory channeling arrangement, permitted classes of emission, power limits) are contained in Appendices S26 and S27. The provisions of No. S43.4 also fall into this category of mandatory regulatory provisions, i.e. the prohibition of using the exclusive frequency allocations to the aeronautical mobile service for any kind of public correspondence.

2.4	Maritime mobile service:  Most of them are related to the frequency bands that are allocated exclusively to the maritime mobile service (obligatory channeling arrangements, permitted classes of emission, power limits, etc); however many of them are also applicable to the non-exclusive allocations to the maritime mobile service. A summary of the provisions that are applicable to the frequency assignments subject to notification is given in the Table below:

|  | Provision No. |
| --- | --- |
| Power limits | S52.56, S52.104 S52.117, S52.127 (Region 1 only), S52.143, S52.144, S52.172 S52.184-S52.186, S52.188, S52.200 (Region 1 only) S52.219, S52.220, S52.227 |
| Class of emission | S52.2, S52.3, S52.17, S52.37 S52.55, S52.98, S52.101, S52.177, S52.183, S52.188, S52.198, S52.217 |
| Mandatory sub-division | S52.10 (Region 1 only), S52.13, S52.39 and S52.40 Appendix S17 |

2.5	The list of these “other provisions”, referred to in No. S11.31.2, with respect to which notices to stations in terrestrial services5 in the bands that are shared with equal rights with space services are examined, is given below:

2.5.1	conformity with the limits concerning the maximum equivalent isotropically radiated power, in the context of services and frequency bands indicated in Table S21-2 (Nos. S21.3, S21.4, S21.5A and S21.6);

2.5.2	conformity with the limits concerning the power delivered by a transmitter to the antenna of a station in the fixed or mobile services (13 dBW in frequency bands between 1 GHz and 10 GHz, 10 dBW in frequency bands above 10 GHz), in the context of services and frequency bands indicated in Table S21-2 (Nos. S21.5 and S21.6).

2.6	The list of these “other provisions”, referred to in No. S11.31.2, applicable to space services, is given below so far as Articles S21 and S22 are concerned:

2.6.1	conformity with the power limits for earth stations as stipulated in provisions Nos. S21.8, S21.10 and S21.12, S21.13 account being taken of Nos. S21.9 and S21.116, and in provisions S22.26 to S22.28 or S22.32 (as appropriate) under the conditions specified in S22.30, S22.31 and S22.34 to S22.39, where the earth stations are subject to those power limitations (see also A.16 of Appendix S4).

2.6.2	conformity with the minimum angle of elevation of earth stations as stipulated in provisions Nos. S21.147 and S21.15.

2.6.3	conformity with the limits of power flux-density from space stations produced at the Earth’s surface as indicated in the Table S21-4 (No. S21.16), as well as with the epfd limits in Tables S22-1A to S22-1D (No. S22.5C), taking into account, as appropriate, the provisions of Nos. S21.17 and S22.5CA.

2.6.4	conformity with the limits of power flux-density from space stations produced at the geostationary-satellite orbit as indicated in Nos. S22.5 and S22.5A as well as with epfdis limits in Table S22-3 (No. S22.5F).

2.6.5	conformity with limit of equivalent power flux density from earth stations produced at the GSO (epfd) as indicated in the Table S22-2 (No. S22.5D).

2.6.6	conformity with the limit specified in Nos. S22.8, S22.13, S22.17 and S22.19.

3	Other provisions of Articles S21 and S22 will not be taken into account in the Regulatory examination under No. S11.31 and the Board understands that these provisions are to be applied between administrations.

Reason: To adjust the Rule to the modifications of Articles S21 and S22 as decided by WRC-2000.

# 5	Conformity with the Table of frequency allocations

The examination of conformity with the Table of frequency allocations consists of determining whether the assigned frequency and/or the necessary bandwidth of the emission is within the frequency band allocated to the service in which the station in question operates. Another element is to identify the category of the service according to the Table of Frequency Allocations. The following rules are applied in this connection:

5.1	Out-of band emissions: In the case where the assigned frequency is in a band which is not allocated to the service in which the station operates the notice receives an unfavourable regulatory finding. If the assigned frequency is on the edge of a band which is not allocated to the service the finding is also unfavourable.

5.2	Overlapping emissions: In the case where the assigned frequency is in the band which is allocated to the service, but the necessary bandwidth overlaps the immediately adjoining band which is not allocated to the service, the notice receives an unfavourable regulatory finding.

5.3	Receiving point of a terrestrial service is in a region where the service is not allocated: In the case of circuit whose transmitting point is in a country, Sub-Region or Region where the frequency is allocated to the service, but whose receiving point is not, an unfavourable regulatory finding is issued.

5.4	The relationship between the notifying administration and the territory in which the station is located is covered by Resolution 1 (Rev.WRC-97) (see also comments under the Rules of Procedure concerning No. S9.3 and Resolution 1 (Rev.WRC-97). The notification of assignments to space stations raises the following questions:

–	Should there be any relation between the territory of the notifying administration and the orbital position of a space station in the case of worldwide allocations?

–	Should there be any such relation in the case of Regional allocations or allocations to a group of countries or to individual countries?

In reply to these questions the Board reached the following conclusions:

a)	In the case of worldwide allocations without a specific restriction in a footnote, any administration may notify any orbital position and any service area in any part of the Earth which is visible from the space station.

b)	When an allocation is made with territorial restrictions, such as for national use, the service area must then be limited to that territory.

c)	In the case of a Regional allocation, as the limits separating the three Regions do not refer to the geostationary-satellite orbit, the orbital position is not taken into consideration when determining if the Regional allocation is respected; only the service area is used to formulate a Finding; this Finding is favourable if the service area is entirely located in the Region to which the allocation is made and unfavourable otherwise. When there is no specific restriction in a footnote, any administration, pertaining or not to the Region to which the allocation is made, may notify any orbital position and any service area within the Region to which the allocation is made.

d)	The § c) above applies equally to an allocation to a sub-Region or to a country.

e)	As indicated in c) and d) above, the service area notified by an administration is not necessarily the territory of the notifying administration. When the service area notified covers totally or partly the territory of another administration, it is assumed (unless advised to the contrary by an administration not accepting such practice) that an agreement exists between the administrations concerned. If, following the publication of an assignment in the weekly Circular, an administration objects to the notified service area, the Bureau informs the notifying administration of the comments received and will modify the service area only if the notifying administration so requests.

f)	A space station has a “coverage area” which generally encompasses the “service area”. Article S1 of the Radio Regulations does not contain a definition for these terms; however, the definitions given in Annex 5 to Appendix S30 may be used. Generally the coverage area results in an unavoidable transmission over the territory of other countries and the comments made in § e) above do not apply to such unavoidable overlap.

5.5	Categories of allocation: In the case when the assigned bandwidth overlaps two frequency bands that are both allocated to the service in question, with different categories of allocation, the favourable regulatory finding is accompanied by the indication of the status derived from the lowest of the two categories of allocation.

5.6	Tropical broadcasting bands:  The frequency bands listed in No. S23.6 are allocated on a shared basis to the tropical broadcasting, to fixed and mobile services (see also footnote No. S5.113). In the Tropical Zone (No. S5.16-S5.21) the broadcasting service has priority over the other services in these frequency bands and the frequency assignments to services other than the broadcasting service are indicated so as to show their lower status with respect to the frequency assignments to stations in the broadcasting service, while retaining their status with respect to the non-broadcasting assignments from both inside and outside the Tropical Zone (symbol V in column 13B2).

6	The No. S11.31 examination shall be made on the basis of the information on the status of the coordination agreement available to the Bureau in the Form of Notice. (See Rules of Procedure relating to No. S11.31.1).

7	The examination under Articles S21 and S22 may result in cases where the limits stipulated in these Articles are exceeded. When the agreement of other administrations is foreseen, the Bureau will formulate a favourable Finding No. S11.31 only if it is informed that such agreement exists. This agreement is treated by the Bureau separately from the coordination agreement.

ANNEX 8

Rules concerning Nos. S11.44 and S11.44.1 of the RR

MOD S11.44

1	The information concerning the date of bringing into use is normally to be provided in the following occasions:

–	in APS4 form of notice when submitted under No. S11.15; and

–	in any subsequent communication to the Bureau under Nos. S11.44B to S11.44I; and

–	in the confirmation of the date of bringing into use under No. S11.47.

It should be noted that the information concerning the date of bringing into use shall be provided for each assignment or group of assignments.

2	Number S11.44 provides that administrations may request an extension of the date of bringing into use. This extension cannot be more than two years. In addition, the extension is given only under specific conditions as enumerated in Nos. S11.44C to S11.44I. As mentioned in No. S11.44B, the extension cannot be granted if the complete “due diligence” information required by Resolution 49 (WRC-97/WRC-2000, as applicable) is not provided.

3	Number S11.44 also states that the Bureau shall cancel those frequency assignments which are not brought into use within the required period (5 years plus the extension granted by the Bureau). Before the Bureau cancels any frequency assignment, it needs to inform the administration at least three months before the expiry of the above period.

4	The Board noted from Resolution 49 (WRC-97/WRC-2000, as applicable) that the submission of the complete “due diligence” information by administrations is closely associated with the expiry date (five years) of the regulatory period. In fact, § 10 of Annex 1 to the Resolution instructs the Bureau to inquire about the complete “due diligence” information if this was not received at least 6 months before the 5 year deadline.

5	The Board concluded from the above that the Bureau shall inquire on the date of bringing into use of the assignments and the complete “due diligence” information before the 5 year period expires, if the above information is not communicated by the administration. The Board noted that the two inquiries concern similar information and they are to be effected at similar points in time. Consequently the Board decided that one inquiry shall be effected by the Bureau for both purposes. Six months before the expiry of the 5 year period counted from the date of receipt of the advance information submitted under No. S9.1, if the administration has not confirmed the date of bringing into use of the assignments of a satellite network and/or has not provided the complete “due diligence” information pursuant to Resolution 49(WRC-97/WRC-2000, as applicable), the Bureau shall request the administration to fulfill its obligations.

6	If no answer is received, a reminder shall be sent 3 months before the expiry of the 5 year period.

7	At the end of the five year period the following situations may be envisaged:

7.1	If the administration confirms that the assignments of the space station have been brought into use and it provides the complete “due diligence” information in accordance with Resolution 49 (WRC-97/WRC-2000, as applicable), the Bureau maintains the MIFR recording of the subject assignments (provisional recording is changed to definitive one) or, if the assignments of the network have not been recorded in the MIFR, the Bureau continues to take into account the coordination and/or advance publication files of the

subject network in the applicable regulatory procedures.

7.2	If the complete “due diligence” information is provided in accordance with Resolution 49 (WRC-97/WRC-2000, as applicable) but the assignments have not been brought into use and the administration requests an extension not more than two years referring to one or more circumstances mentioned in Nos. S11.44C to S11.44I, the Bureau examines this request for extension and, according to itsconclusion, either grants an extension or, if it is not in a position to grant the extension, it explains the reasons thereof to the administration. If the extension is granted, then the Bureau maintains the MIFR recording of the subject assignments (provisional recording) or, if the assignments of the network have not been recorded in the MIFR, the Bureau continues to take into account the coordination and/or advance publication files of the subject network in the applicable regulatory procedures till the end of the extended period. If the extension is not granted, the Bureau cancels the MIFR recording of the assignments in question and  or, if the assignments of the network have not been recorded in the MIFR, it will no longer take into account the coordination and/or advance publication files of the network in the applicable regulatory procedures (files are cancelled).

7.3	In any other cases (i.e. no complete “due diligence” information provided or no extension requested or no answer from the administration received) the Bureau shall cancel the MIFR recording of the subject assignments or, if the assignments of the network have not been recorded in the MIFR, it will no longer take into account the coordination and/or advance publication files of the network in the applicable regulatory procedures (files are cancelled).

8	If the assignments are brought into use and due diligence information has been provided for them, the Bureau continues to take into account the coordination and/or advance publication files of the network in the applicable regulatory procedures till the end of the 7 year regulatory period with or without extension under S11.44B-S11.44I, even if these assignments are not notified.

9	Three months before the end of the 7-year regulatory period of assignments, if the assignments are not notified under No. S11.15 i.e. no first notification (see APS4 form) on these assignments is received by the Bureau, it shall inform the notifying administration that the assignment will no longer be taken into account by the Bureau and administrations unless they are notified within the 7-year regulatory deadline.

10	At the end of the 7-year regulatory period, the Bureau shall take into account in its examination, only the assignments brought into use, for which due diligence information is provided and for which first notification has been received.

11	See also the Rules of Procedures concerning No. S11.47.

ADD S11.41.1

See the Rules of Procedure concerning No. S11.44.

Reason : Consequential to addition of new provision S11.44.1 by WRC-2000.

Note:  It should be noted that after the end of the 7 years regulatory period, there may be assignments which are brought into use and for which due diligence information is provided, and their first notification has been received but they were returned to administration as results of unfavourable findings (e.g. unsuccessful or inconclusive coordination).  There is no clear indication in the Radio Regulation how long these assignments are to be taken into account.

annex 9

# Rules concerning the Receivability of APS4 Forms of Notice 
submitted to the Radiocommunication
Bureau in the application of the Radio Regulatory
Procedures relating to space services

# 1	Forms of notice

1.1	Circular-letters CR/65 (22 November 1996) and CR/86 (25 March 1998) of the Bureau contain the forms of notice to be used for communication of particulars of radiocommunication stations and satellite networks. The forms of notice are derived primarily from Appendix S4 of the Radio Regulations. The above circular-letters include detailed instructions for completing the notices. Circular letter CR/144 (18 August 2000) established revised arrangements for mandatory electronic filing of notices pursuant to Resolution 55 (WRC-2000).

1.2	Circular Letter CR/144 also notes that there are practical difficulties which are delaying the application of the provisions of resolves 5 and 6 of Resolution 55 (WRC-2000) to notices under Appendices S30, S30A and S30B. Some additional time is needed to develop new forms of notice and the necessary software to capture and validate such notices electronically. Administrations were requested, as from the receipt of that Circular Letter, to start using the Annexes 2A and 2B of Appendix S4 (WRC-2000) data elements in a paper form for submissions under Appendices S30, S30A and S30B, until the time when new notice forms are made available to administrations and the relevant Data Processing software (e.g. SpaceCap, etc ) are available.

# 1bis.	Receipt of notices1

Having regard to the various means available for transmission and delivery of notices and other related correspondence, the Board has decided that:

a)	Mail received through the postal service shall be recorded as received on the first working day on which it is delivered to the ITU/BR’s offices in Geneva.

b)	E-mail or telefax documents shall be recorded as received on the actual date of receipt, irrespective of whether or not that is a working day at the ITU/BR’s offices in Geneva.

c)	In the case of E-mails, an administration is required to send, within 7 days of the date of the E-mail, a confirmation by either telefax or mail which shall be regarded as being received on the same date as the original E-mail.

- All mail must be sent to the following address:

Radiocommunication Bureau

International Telecommunication Union

Place des Nations

CH-1211 Geneva 20

Switzerland

- All telefaxes must be sent to:

+41 22 730 57 85 (several lines)

- All E-mail must be sent to:

brmail@itu.int

g)	Information received in the ITU/BR by E-mail shall be acknowledged by E-mail

# 2	Treatment of forms of notice by date order

According to provisions S11.282 and S11.29, complete notices are examined by date order of their receipt and the Bureau cannot act upon a notice having a technical bearing on an earlier notice until the earlier notice has been dealt with. While similar provisions do not exist in all the regulatory procedures defined in the Radio Regulations, nevertheless, several other provisions tacitly require the same general concept. The Board decided that the principle of treatment by date order of receipt of any submission is to be applied in each of the procedures described in Articles S9, S11, Appendices S30, S30A, S30B and Resolutions containing specific procedures.

Reasons	The proposed new paragraph 1 bis and modified paragraph 2 are intended to codify and clarify arrangements for receipt of notices and the date order in which they are processed. In particular, the arrangements reflect practical requirements in which notices are received in the ITU-R in electronic format by either mail, telefax or E-mail. Whilst both time and date of receipt of telefax and E-mail can be accurately determined, the same is not possible for deliveries by mail or courier; nor can relative times of receipt between all forms of transmission be accurately and reliably established.

# 3	Establishment of a date of receipt

3.1	In order to establish a formal date of receipt for the purpose of treatment, in date order, of the submissions (notices for advance publication, request for coordination, modification  to the Region 2 Plan or proposed new or modified assignments in the Regions 1 and 3 Lists under Appendices S30 or S30A, or request for application of Articles 6 or 7 of Appendix S30B, and notifications for recording in the Master International Frequency Register except notification relating to frequency bands subject to Appendix S30B), the Bureau shall examine inter alia the completeness and correctness of the information submitted by administrations. It shall also take account of the requirements of No. S9.1 when establishing the date of receipt of coordination information and notification information with respect to the date of receipt (when the coordination procedure of Section II of Article S9 is applicable) and the date of publication (when the coordination procedure of Section II of Article S9 is not applicable) of advance information, respectively.

3.1bis	Resolves 5 of Resolution 55 (WRC-2000) prescribes that, as from 3 June 2000, all notice forms (APS4/II and III), radio astronomy notices (APS4/IV) and API (APS4/V and VI) and due diligence information (Resolution 49 (WRC-97/WRC-2000 as applicable)) for networks and earth stations submitted to the Radiocommunication Bureau pursuant to Articles S9 and S11 shall be submitted in electronic format which is compatible with the BR electronic notice form capture software (SpaceCap).

3.1ter	The RRB noted the above mentioned requirement in the context of the associated considerings and recognizings of Resolution 55. It noted also the availability to administrations of capture and validation software from the Radiocommunication Bureau. Accordingly, the Board agreed that, in view of the processing delays of the notice forms in the BR, efficient measures are needed to enable the Bureau to address the backlog in processing. Accordingly, where a notice received by the Bureau does not contain all of the mandatory information as defined in the Table (Annex 2B) of APS4 of the Radio Regulations and the published list of validation rules, the Bureau shall regard the notice as incomplete. The Bureau shall immediately inform the administration and seek the information not provided. Further processing of the notice by the Bureau will remain in abeyance and a date of receipt (see paragraph 3.1 above) will not be established until the missing information is received.

3.1quater	The current version (V 2.25 or above) of the validation software available to administrations (as advised by Circular Letter) shall be used by the Bureau when assessing the completeness of APS4 Forms of notice, coordination requests and notification for satellite networks or satellite systems, including earth stations, submitted under Articles S9 and S11. Administrations are encouraged to run the validation software themselves in order to overcome any difficulties in the notices before they are submitted to the Bureau.

3.1quinto	The requirement in 3.1ter does not apply to notices in respect of Advance Publication Information (S9.1), radio astronomy notices (APS4/IV), notices under Appendices S30, S30A and S30B and due diligence information (Resolution 49 (Rev. WRC-2000)) pending the introduction of electronic filing and/or the availability of validation software for these procedures. In the case of notices under APS30, S30A and S30B, the Bureau shall establish the date of receipt as that recorded in accordance with paragraph 1bis above. Should the Bureau find that the information received in these cases is incomplete or incorrect it shall apply the same procedures as noted in paragraphs 3.2 to 3.7 below.

Reason	consequential to the adoption by WRC-2000 of Resolution 55, with a view to improvement of satellite network coordination and notification procedures.

3.2	a)	In the case of notices pursuant to Appendices S30, S30A and S30B, should the Bureau find that the information is incomplete or incorrect, it shall request the administration responsible for the station or network to provide the missing information or clarification within 30 days.

b)	In the case of requests for co-ordination or notification if, following the steps in paragraph 3.1ter above, the Bureau finds that, whilst the notice includes all mandatory information defined in APS4, further information or clarification is required, it shall request the administration responsible for the station or network to provide the further information or clarification within 30 days.

3.3	If the information or clarification is provided within that period of 30 days (counted from the date of the dispatch of Bureau’s message), the initial date of receipt established by the Bureau will be considered as the formal date of receipt for the purpose of any subsequent processing of the notice. (See however, paragraph 3.4 below).

3.4	Nevertheless, for replies received within the above period of 30 days, a new date of receipt is established in those cases (or for the concerned part of the station or network) where the information submitted subsequently is outside the scope and beyond the objective of the Bureau’s enquiry pursuant to paragraph 3.2 and paragraph 3.3 above. unless the new or modified data has no impact on the regulatory and technical examination.  The new date of receipt will be the date of receipt of the new or modified information. See also Rules of Procedure relating to provisions No. S9.27.

3.5	If the information or clarification is not provided within the above period of 30 days, the submission shall be considered incomplete and no formal date of receipt will be established by the Bureau. A new date of receipt will be established when the complete information is received,  irrespective of whether the newly provided information adds new affected administrations or not.

3.6	In case of the request for deletion of an assignment, a group of assignments, an emission, beams or other characteristics of a satellite network or satellite system, two situations may arise:

3.6.1	The satellite network or satellite system in question has not yet been examined and published by the Bureau. In that case, the initial date of receipt will be maintained.

3.6.2	The satellite network or satellite system in question has already been examined and published by the Bureau. In that case, the request for deletion shall be published in a corrigendum to previously published relevant special section. However, the technical bearing of the deletion will be examined by the Bureau in the date order of receipt of the request.

3.7	After one year, unless otherwise specified in the relevant procedures, any pending submissions containing incomplete information shall be returned to the notifying administration.

# 4	Other non-receivable submissions

There are, in addition to the above case of incomplete notice, other circumstances when a notice is not receivable. These cases are described in the following non exhaustive paragraphs.

4.1	An advance publication notice sent to the Bureau earlier than five years before the planned date of bringing into use of the satellite network is not receivable and shall be returned to the administration responsible for the network. (No. S9.1 refers.)

4.2	A notification received by the Bureau earlier than the date limits prescribed in provisions S11.24 to S11.26 (date limits relate to the date of bringing into use of a station or satellite network) is not receivable and shall be returned to the administration responsible for the network.

4.2bis	NOC

4.3	The Radio Regulations prescribe, in some cases, the application of multiple procedures which have to be applied, for the same stations or satellite network, in a sequential order, one after another. A typical example of such a case of multiple procedures is a geostationary satellite network to which the application of the advance publication, the coordination (in some cases more than one coordination category) and the notification procedures, in this order, are mandatory. In such cases, a notice for a particular procedure is receivable only if the previously applicable procedures have been effected. A notice for a request for coordination is not receivable if the advance publication information was not submitted to the Bureau.  A notification under Article S11 is not receivable if the advance publication information and coordination request, where applicable, were not received for the satellite network, and shall be returned to the notifying administration. The same shall also apply for the notification of an earth station whose associated space station is not supported by an advance publication.

Reason:	This part of the rule is considered no longer appropriate because of the practical difficulties in examining the requirement of coordination at the stage of the completeness check. Paragraph b) of the Rules of Procedure (ROP) relating to S11.32, which has similar effect is being applied following the decision of the 19th meeting of the RRB (21 - 25 February 2000).

4.4	A notification received under No. S11.2 or S11.9 relating to a satellite network/system for which either the regulatory time limit (5  2 years, if extension is granted) has expired or the due diligence information as prescribed by Resolution 49 (WRC-97) has not been provided, are not receivable and shall be returned to the notifying administration.

5	In each case where the Bureau returns a form of notice according to the above paragraphs, the necessary justification for such an action shall be provided to the notifying administration.

ANNEX 10

Rules concerning No.S9.11A of the RR

MOD S9.11A

Paragraph 1

NOC

Paragraph 2

# 2	Application of No.S9.11A to different services/frequency bands

Paragraph 2.1 and 2.2

NOC

Paragraph 2.3

MOD

2.3	While recognising the difficulties of harmonising the text of the footnotes to Article S5 introduced by WARC-92, WRC-95 and WRC-97 on the one hand and the text of the provision of No. S9.11A (including Nos. S9.12 to S9.16) and S9.17A, as appropriate with respect to the services to which this provision is applicable, on the other hand, the Board concluded that the procedure is applicable to all other space and terrestrial services with respect to those satellite services having allocations with equal rights and mentioned in the specific footnotes to which this provision applies. The frequency bands are those to which, in a footnote, reference is made to this provision in the Table of Frequency Allocations- (See Tables S9.11A-1 and S9.11A-2 below). In this Table, there is an indication of those other space services (in addition to the mobile-satellite and radiodetermination-satellite services as well as non-GSO MSS feeder links and non-GSO FSS included in the footnotes) to which this coordination procedure shall also apply. This application is subject to the same condition as that of the space services specifically mentioned in the footnotes, e.g. the coordination of space stations of the other space services (space-to-Earth), with respect to terrestrial services, is required only if the threshold values indicated in Annex 1 to Appendix S5 are exceeded.

Paragraph 2.4

ADD

2.4	WRC-2000 decided to delete the Table S5-1A of Appendix S5 under the condition that it is included in a Rule of Procedure with appropriate modifications (e.g. inclusion of terrestrial services etc.) (minutes of the Plenary (B17) refers). The extended version of the above-mentioned Table is contained in Tables S9.11A-1 and S9.11A-2, based on the following considerations:

a)	No. S9.14 applies to space-to-earth frequency allocations only, i.e. coordination of a transmitting space station in respect of receiving terrestrial stations when the threshold value is exceeded. In the absence of threshold value, the provisions of No. S9.50.1 could apply (see also Appendix S5). For the earth-to-space frequency allocations, the Board is of the opinion that no coordination is required between the involved space and terrestrial services, which both are considered on an equal basis.

b)	No. S9.15 applies to earth-to-space and space-to earth frequency allocations of a 
non-geostationary satellite network to which No. S9.11A refers, i.e. coordination of a transmitting earth station in respect of receiving terrestrial stations and coordination of a receiving earth station in respect of transmitting terrestrial stations, if the coordination area of the earth station in a non-geostationary satellite network includes the territory of another country (see also Appendix S5).

c)	No. S9.16 applies to space-to-earth frequency allocations of a non-geostationary satellite network to which No. S9.11A refers only, i.e. coordination of a transmitting terrestrial station within the coordination area of a receiving earth station in a non-geostationary satellite network.

Paragraph 3

3	Frequency allocation matters

Paragraph 3.1

NOC

Paragraph 3.2

MOD

3.2	WRC-97, in its Resolution 54 (WRC-97) instructed the Bureau to apply the provisions of the Resolution 46 (Rev.WRC-97)/No. S9.11A as of 22 November 1997 to those bands in which the Resolution is mentioned even though the footnotes to the Table of Frequency Allocations are not in force until a later date. The Board understands that the earlier date of implementation of the procedure does not influence the date of entry into force of the related allocations. Tables S9.11A-1 and S9.11A-2 below contains an indication of the dates of entry into force of the allocations concerned with the application of No. S9.11A.

Paragraph 3.3

NOC

Paragraph 4

NOC

Reason: WRC-2000 agreed to delete the Table S5-1A of Appendix S5 under the condition that it should be included in a Rule of procedure with appropriate modifications (e.g. inclusion of terrestrial services etc.) (refer minutes of the Plenary (B17)). The Board carefully studied the existing Rule of procedure on No. S9.11A (version 1998), taking due account of its evolution since the former Rule on Res.46/S9.11A (version 1994) and applied the following considerations to the relationship between the different space and terrestrial services in developing the draft modifications to the S9.11A Rule of procedure:

- Appendix S5, paragraph 1 states that ”For the purpose of effecting coordination under Article S9…the frequency assignments to be taken into account are those in the same frequency band as the planned assignment, pertaining to the same service or to another service to which the band is allocated with equal rights or higher category of allocation…” Footnote 1 restricts this application to assignments in bands allocated with equal rights in case of coordination under Nos. S9.15-S9.19. The board considered the application of coordination under Nos. S9.11A-S9.14 between frequency assignments in bands allocated with different category of allocation and taking into account Nos. S5.28-S5.31, it confirmed the practice applied by the Bureau since 1992 to examine coordination under Nos. S9.11A-S9.14 between services with equal status only (see Table 1 to RS46 Rule of Procedure (Edition 1994)). Noting however the text in paragraph 1 to Appendix S5, the Board considers that the attention of a future conference should be drawn to this discrepancy.

- The terminology used in No. S9.11A footnotes to the Table of Allocation to specify the space/terrestrial (or other services) relationship is various and somehow ambiguous in some cases. Whilst the terminology used in e.g., Nos. S5.221, S5.286B, S5.352A, S5.374…(“stations in the [space service]…shall not cause harmful interference to…or claim protection from…stations in the [terrestrial service]” could be interpreted as a total protection of existing and planned use of the terrestrial stations, leading to the non-application of coordination under Nos. S9.14-S9.16, the type of protection afforded by such terminology as “stations in the [space service]…shall not constrain the development and use of … stations in the [terrestrial service]” , as in e.g. Nos. S5.219, S5.286C, might be questioned. Taking into account Nos. S5.43 and S5.43A, and the decisions by WRC-97 on this issue (refer Summary Record of the 12th Meeting of Committee 5), the Board is of the opinion that whenever such above references are indicated in a footnote, this would be interpreted as not requiring the application of the coordination procedure between the involved stations. The Board has carefully reviewed the wording of each No. S9.11A footnotes and has applied the above principle, as appropriate, to develop Tables S9.11A-1 and S9.11A-2.

When considering the FSS allocations to non-GSO systems, Earth-to-space, in the bands 12.5 - 12.75 GHz in Region 1 and 12.7 - 12.75 GHz in Region 2, the Board has noticed that there was no footnote in the Table of frequency allocations, nor reference in any resolutions to the application of No. S9.12 coordination for these FSS allocations. However, the non-GSO FSS systems (Earth-to-space) in these bands would have to comply with the EPFD  limits listed in Article S22, Table S22.2 and such non-GSO FSS (Earth-to-space) system may request to be included in the S9.12 coordination of a non-GSO system (space-to-Earth). The Board considered if such asymmetry should be corrected (i.e. application of No. S9.12 also to both uplink allocations) and decided that at this stage the above 2 bands should not be listed in the Table on S9.11A applicability and that this issue might be drawn to the attention of a future conference. The Board noted a similar situation for the uplink allocation to FSS in the band 10.7 - 11.7 GHz in Region 1, except that in this case no EPFD  limits exist.

In order to easily identify the requirements for coordination of space services on one hand and the terrestrial services on the other and bearing in mind the widespread practices in administrations where different entities deal with space and terrestrial services, two separate tables are proposed in the Rule of procedure.

ADD

TABLE S9.11A-1

Applicability of the provisions of Nos. S9.11A - S9.15 to stations of Space Services

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 137 - 137.025 137.175-137.825 | S5.208 | MOBILE-SATELLITE (non-GSO) |  | SPACE OPERATION METEOROLOGICAL-SATELLITE SPACE RESEARCH |  | FIXED (S5.204, S5.205) LAND MOBILE (S5.204, S5.205) MARITIME MOBILE (S5.204, S5.205) AERONAUTICAL MOBILE (OR) (S5.204, S5.206) BROADCASTING (S5.207) |  | 1 2 |
| 137.025-137.175 137.825-138 | S5.208 | MOBILE-SATELLITE (non-GSO) |  | --- |  | Fixed (in countries other than those listed in S5.204, S5.205) Land mobile (in countries other than those listed in S5.204, S5.205) Maritime mobile (in countries other than those listed in S5.204, S5.205) Aeronautical mobile (OR) (in countries other than those listed in S5.204, S5.206) |  |  |
| 148-149.9 | S5.219 | MOBILE-SATELLITE (non-GSO) |  | --- (See S5.219) |  | --- (See S5.219, S5.221) |  |  |
| 149.9-150.05 | S5.220 | MOBILE-SATELLITE (non-GSO) |  | --- (See S5.220) |  | --- | Limited to LMSS System until 1.1.2015 (S5.224A) |  |
| 312-315 | S5.255 | Mobile-satellite (non-GSO) |  | --- |  | --- |  |  |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 387-390 | S5.255 | Mobile-satellite (non-GSO) |  | --- |  | --- |  |  |
| 399.9-400.05 | S5.220 | MOBILE-SATELLITE (non-GSO) |  | --- (See S5.220) |  | --- | Limited to LMSS System until 1.1.2015 (S5.224A) |  |
| 400.15-401 | S5.264 | MOBILE-SATELLITE (non-GSO) |  | METEO-SATELLITE SPACE RESEARCH |  | FIXED (S5.262) MOBILE (S5.262) METEOROLOGICAL AIDS |  | 1 2 |
| 454-455 | S5.286A | MOBILE-SATELLITE (non-GSO) (S5.286D, S5.286E) |  | --- |  | --- (See S5.286B, S5.286C) |  |  |
| 455-456 | S5.286A | MOBILE-SATELLITE (non-GSO) (R2, S5.286E) |  | --- |  | --- (See S5.286B, S5.286C) |  |  |
| 459-460 | S5.286A | MOBILE-SATELLITE (R2, S5.286E) |  | --- |  | --- (See S5.286B, S5.286C) |  |  |
| 1492-1525 | S5.348 | MOBILE-SATELLITE (R2, except USA (S5.344)) |  | --- |  | FIXED MOBILE |  | 3 |
| 1525-1530 | S5.354 | MOBILE-SATELLITE |  | SPACE OPERATION |  | FIXED (R1, R3, see also S5.352A) LAND MOBILE (S5.349) MARITIME MOBILE (S5.349) AERONAUTICAL MOBILE (S5.342, S5.350) |  | 1 |
| 1530-1535 | S5.354 | MOBILE-SATELLITE |  | SPACE OPERATION |  | AERONAUTICAL MOBILE (S5.342) |  | 1 |
| 1535-1545 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- |  |  |
| 1545-1550 | S5.354 | MOBILE-SATELLITE |  | --- |  | AERONAUTICAL MOBILE (R) (S5.357) |  | 4 |
| 1550-1555 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) AERONAUTICAL MOBILE (R) (S5.357) |  | 4 |
| 1555-1559 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) |  |  |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 1610-1626.5 | S5.364 | MOBILE-SATELLITE (except S), RADIODETERMINATION-SATELLITE (R2+S5.369) |  | --- |  | --- (See S5.364) |  |  |
| 1610-1626.5 | S5.364 | Radiodetermination-satellite (R1 (S5.371), R3, VEN (S5.370)) |  | --- |  | Fixed (S5.355) |  |  |
| 1613.8-1626.5 | S5.365 | Mobile-satellite |  | --- |  | Fixed (S5.355) |  |  |
| 1626.5-1631.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) |  |  |
| 1631.5-1634.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- (See S5.374) |  |  |
| 1634.5-1645.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) |  |  |
| 1645.5-1646.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- |  |  |
| 1646.5-1656.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) AERONAUTICAL MOBILE (R) (S5.376) |  |  |
| 1656.5-1660.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- (See S5.374) |  |  |
| 1675-1690 | S5.377 | MOBILE-SATELLITE (R2) |  | --- (See S5.377) |  | FIXED LAND MOBILE MARITIME MOBILE |  |  |
| 1690-1700 | S5.377 | MOBILE-SATELLITE (R2) |  | --- (See S5.377) |  | FIXED (R2 and R3 countries listed in S5.381 and S5.382, R1 countries listed in S5.382) LAND MOBILE (R2 and R3 countries listed in S5.381, R1 countries listed in S5.382) MARITIME MOBILE (R2 and R3 countries listed in S5.381, R1 countries listed in S5.382) |  |  |
| 1700-1710 | S5.377 | MOBILE-SATELLITE (R2) |  | SPACE RESEARCH (S5.384) |  | FIXED LAND MOBILE MARITIME MOBILE |  | 1 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 1980-1990 | S5.389A | MOBILE-SATELLITE |  | --- |  | FIXED (except R2 countries in S5.389B) MOBILE (except R2 countries in S5.389B) (see also S5.389F) | 1.1.2005 in R2 |  |
| 1990-2010 | S5.389A | MOBILE-SATELLITE |  | --- |  | FIXED MOBILE (see also S5.389F) |  |  |
| 2010-2025 | S5.389C | MOBILE-SATELLITE (R2) |  | --- |  | FIXED (R2) MOBILE (R2) (see also S5.390 and S5.389E) | 1.1.2002 (1.1.2000 in CAN, USA) |  |
| 2160-2170 | S5.389C | MOBILE-SATELLITE (R2) |  | SPACE RESEARCH S5.392A (RUS) |  | FIXED (R2) MOBILE (R2) (see also S5.390, S5.392A and S5.389E) | 1.1.2002 (1.1.2000 in CAN, USA) | 1 5 |
| 2170-2200 | S5.389A | MOBILE-SATELLITE |  | SPACE RESEARCH S5.392A (RUS) |  | FIXED MOBILE (see also S5.389F andS5.392A) |  | 1 5 |
| 2483.5-2500 | S5.402 | MOBILE-SATELLITE RADIODETERMINATION-SATELLITE (R2 & R1/R3 countries in S5.400) |  | --- |  | RADIOLOCATION (R2, R3, F)(S5.397, S5.399) FIXED MOBILE |  | 7 |
| 2483.5-2500 | S5.402 | Radiodetermination-satellite (R1&R3) |  | --- |  | --- (See S5.399) |  |  |
| 2500 -2515 | S5.414 S5.403 | MOBILE-SATELLITE (except countries in S5.412) |  | FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) |  | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) | 1 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 2515 - 2520 | S5.414 S5.403 | MOBILE-SATELLITE (except countries in S5.412) |  | FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | FIXED, LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) 1.1.2002 (AMSS in IND) | 1 |
| 2520 -2535 | S5.403 | MOBILE-SATELLITE (except AERONAUTICAL MOBILE-SATELLITE) (except countries in S5.412 and S5.417) |  | BROADCASTING-SATELLITE, FIXED SATELLITE (R2&3) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | 1.1.2002 (AMSS in IND) | 1 6 |
| 2630-2655 | S5.418A S5.418B | BROADCASTING-SATELLITE (Non-GSO) (sound) FIXED-SATELLITE (R2) (Non-GSO) |  | BROADCASTING-SATELLITE (non-GSO) |  | --- (see Res.539 (WRC-2000)) |  | 8 9 |
|  | S5.418C | BROADCASTING-SATELLITE (GSO) FIXED-SATELLITE (R2) (GSO) |  |  |  |  |  |  |
| 2655-2670 | S5.420 | MOBILE-SATELLITE (except AERONAUTICAL MOBILE-SATELLITE) (except countries in S5.412 and S5.417) |  | BROADCASTING-SATELLITE, FIXED SATELLITE (R2 and R3) |   | FIXED LAND MOBILE MARITIME MOBILE |  | 1 |
| 2670-2690 | S5.419 S5.420 | MOBILE-SATELLITE (except countries in S5.412) |  | FIXED SATELLITE (R2 and R3), AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.420A) |  | FIXED, LAND MOBILE MARITIME MOBILE | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) 1.1.2002 (AMSS in IND) | 1 |
| 5091-5150 | S5.444A | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | AERONAUTICAL-MOBILE SATELLITE (R) (S5.367) |  | --- (See S5.444A) |  |  |
| 5150 - 5250 | S5.447A S5.447C | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | RADIODETERMINATION-SATELLITE (S5.446), with date of bringing into use prior to 17.11.1995 |  | AERONAUTICAL RADIONAVIGATION MOBILE (S5.447) |  | 1 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 5150 - 5216 | S5.447B | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | RADIODETERMINATION-SATELLITE  (S5.446), with date of bringing into use prior to 17.11.1995 |  | AERONAUTICAL RADIONAVIGATION, MOBILE (S5.447) |  | 1 13 |
| 6700 - 7075 | S5.458B | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | non-GSO FIXED-SATELLITE in bands 6700 – 6725 MHz and 7025 – 7075 MHz |  | FIXED MOBILE |  | 1 13 |
| 10.7 - 11. 7 | S5.441 S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 11.7-12.2 (R2) | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 12.2-12.75 (R3) |  |  |  |  |  |  |  |  |
| 12.5-12.75 (R1) |  |  |  |  |  |  |  |  |
| 11.7-12.5 (R1) 11.7-12.2 (R3) 12.2-12.7 (R2) | S5.487A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 12.75-13.25 | S5.441 | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 13.75-14.5 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 15.43-15.63 | S5.511A | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | --- |  | AERONAUTICAL RADIONAVIGATION |  | 12 |
| 15.43-15.63 | S5.511A | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | --- |  | AERONAUTICAL RADIONAVIGATION |  |  |
| 15.63-15.65 | S5.511D | Non-GSO FIXED-SATELLITE |  | FIXED-SATELLITE |  | AERONAUTICAL RADIONAVIGATION |  | 1 12 |
| 17.3-18.1 (R1, R3) | S5.516 | Non-GSO FIXED-SATELLITE |  | Non-GSO FIXED-SATELLITE (R1, R2, R3) in the band 17.7 – 17.8 GHz |  | --- |  | 10 11 |
| 17.8-18.1 (R2) | S5.516 | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 17.8-18.6 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 18.8 - 19.3 | S5.523A | FIXED-SATELLITE |  | --- |  | FIXED MOBILE |  | 13 |
| 19.3 - 19.6 | S5.523B | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | --- |  | FIXED MOBILE |  |  |
| 19.3 - 19. 7 | S5.523D | FIXED-SATELLITE (GSO and non-GSO MOBILE SATELLITE SERVICE feeder link) |   | --- |  | FIXED MOBILE |  | 13 |
| 19.7 - 20.2 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 27.5 - 28.6 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 28.6 - 29.1 | S5.523A | FIXED-SATELLITE |  | --- |  | FIXED MOBILE |  |  |
| 29.1 - 29.5 | S5.535A | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | --- |  | FIXED MOBILE |  |  |
| 29.5 - 30 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |

Notes:

- The coordination of non-GSO space services (Earth stations) in respect of terrestrial services is to be effected under the provisions of No. S9.15. For the coordination of GSO space services (Earth stations) in respect of terrestrial 	services columns 4 and 5 of Table S9.11A, the provisions of No. S9.17 apply.

- Coordination thresholds indicated in Annex 1 to Appendix S5 apply only to MOBILE-SATELLITE service.

- For AERONAUTICAL MOBILE service for telemetry, the requirement for coordination is determined by band overlap only (S5.348).

- See Rule of procedure on No. S5.357.

5.	SPACE RESEARCH service is not subject to the application of the provisions of Nos. S9.14 and S9.15.

- The coordination of BROADCASTING-SATELLITE service in respect of terrestrial services is to be effected under the provisions of No. S9.11.

- RADIOLOCATION service is subject to the application of the provisions of Nos. S9.14 and S9.15 vis-à-vis stations in the MOBILE-SATELLITE service only.

- The coordination of non-GSO BROADCASTING-SATELLITE service  (sound) in respect of terrestrial services is subject to the provisions of Resolution 539. (WRC-2000).

- The coordination of non-GSO BROADCASTING-SATELLITE service (column 4) is subject to the provisions of No. S9.12 only.

- The coordination of space services (Earth stations) listed in columns 3 in respect of the terrestrial services to which the same band is allocated with equal rights is to be effected under the provisions of S9.17.

- Subject to application of the provisions of No. S9.12 only.

- Subject to application of the provisions of No. S9.14 in the frequency band 15.45 – 15.65 GHz only

- Not subject to application of the provisions of No. S9.14.

ADD

TABLE S9.11A-2

Applicability of the provisions of No. S9.16 to stations of Terrestrial Services

| 1 | 2 | 3 | 4 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz/GHz) | Footnote in Art. S5 | Terrestrial services to which No. S9.16 applies | Space services mentioned in a footnote referring to No. S9.11A in respect of which No. S9.16 applies, and other non-GSO space services in respect of which No. S9.16 applies equally |  | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 137 - 137.025 137.175-137.825 | S5.208 | FIXED (S5.204, S5.205) LAND MOBILE (S5.204, S5.205) MARITIME MOBILE (S5.204, S5.205) AERONAUTICAL MOBILE (OR) (S5.204, S5.206) BROADCASTING (S5.207) | MOBILE-SATELLITE (non-GSO (S5.209)) SPACE OPERATION METEOROLOGICAL-SATELLITE SPACE RESEARCH |  |  | 1 |
| 137.025-137.175 137.825-138 | S5.208 | Fixed (in countries other than those listed in S5.204, S5.205) Land mobile (in countries other than those listed in S5.204, S5.205) Maritime mobile (in countries other than those listed in S5.204, S5.205) Aeronautical mobile (OR) (in countries other than those listed in S5.204, S5.206) | Mobile-satellite (non-GSO (S5.209)) |  |  | 1 |
| 400.15-401 | S5.264 | FIXED (S5.262) MOBILE (S5.262) METEOROLOGICAL AIDS | MOBILE-SATELLITE (non-GSO (S5.209)) METEOROLOGICAL-SATELLITE SPACE RESEARCH |  |  | 1 |
| 1492-1525 | S5.348 | FIXED (R2) MOBILE (R2) | MOBILE-SATELLITE (R2, except USA (S5.344)) |  |  | 1, 2 |
| 1525-1530 | S5.354 | FIXED (R1, R3, see also S5.352A) LAND MOBILE (S5.349) MARITIME MOBILE (S5.349) AERONAUTICAL MOBILE (S5.342, S5.350) | MOBILE-SATELLITE SPACE OPERATION |  |  | 1 |
| 1530-1535 | S5.354 | AERONAUTICAL MOBILE (S5.342) | MOBILE-SATELLITE SPACE OPERATION |  |  | 1 |
| 1545-1550 | S5.354 | AERONAUTICAL MOBILE (R) | MOBILE-SATELLITE |  |  | 1, 3 |
| 1550-1555 | S5.354 | FIXED (S5.359) AERONAUTICAL MOBILE (R) | MOBILE-SATELLITE |  |  | 1 |
| 1555-1559 | S5.354 | FIXED (S5.359) | MOBILE-SATELLITE |  |  | 1 |
| 1613.8-1626.5 | S5.365 | Fixed (S5.355) | Mobile-satellite |  |  | 1 |
| 2160-2170 | S5.389C | FIXED (R2) MOBILE (R2) (see also S5.390, S5.392A and S5.389E) | MOBILE-SATELLITE (R2) |  | 1.1.2002 (1.1.2000 in CAN, USA) | 1 |
| 2170-2200 | S5.389A | FIXED MOBILE (see also S5.392A and S5.389F) | MOBILE-SATELLITE |  |  | 1 |

| 1 | 2 | 3 | 4 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz/GHz) | Footnote in Art. S5 | Terrestrial services to which No. S9.16 applies | Space services mentioned in a footnote referring to No. S9.11A in respect of which No. S9.16 applies, and other non-GSO space services in respect of which No. S9.16 applies equally |  | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 2483.5-2500 | S5.402 | RADIOLOCATION (R2, R3, F) (S5.397, S5.399) FIXED MOBILE | MOBILE-SATELLITE RADIODETERMINATION-SATELLITE (R2+ R1/R3 countries in S5.400) |  |  | 1, 4 |
| 2500 -2515 | S5.414 S5.403 | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | MOBILE-SATELLITE (except countries in S5.412) FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) |  | 1.1.2005 (until 2005: S9.21: (MSS -AMSS)) 1.1.2002 (AMSS in IND) | 1 |
| 2515 - 2520 | S5.414 S5.403 | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | MOBILE-SATELLITE (except countries in S5.412) FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) 1.1.2002 (AMSS in IND) | 1 |
| 2520 -2535 | S5.403 | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | MOBILE-SATELLITE (except AERONAUTICAL MOBILE-SATELLITE) (except countries in S5.412) BROADCASTING-SATELLITE, FIXED SATELLITE (R2&3) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | 1.1.2002 (AMSS in IND) | 1, 5 |
| 5150 - 5216 | S5.447B | AERONAUTICAL RADIONAVIGATION MOBILE (S5.447) | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) RADIODETERMINATION-SATELLITE (S5.446), with date of bringing into use prior to 17.11.1995 |  |  | 1 |
| 6700 - 7075 | S5.458B | FIXED MOBILE | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link (S5.458B)) |  |  | 1 |
| 15.43-15.63 | S5.511A | AERONAUTICAL RADIONAVIGATION | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link (S5.511A)) |  |  | 1, 6 |
| 15.63-15.65 | S5.511D | AERONAUTICAL RADIONAVIGATION | FIXED-SATELLITE (limited to non-GSO (S5.511D)) |  |  | 1 |
| 18.8 - 19.3 | S5.523A | FIXED MOBILE | FIXED-SATELLITE |  |  | 1 |
| 19.3 - 19.7 | S5.523D | FIXED MOBILE | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  |  | 1 |

Notes:

- The coordination of the terrestrial services under the provisions of No. S9.16 is to be effected only in respect of earth stations of non-GSO satellite 	networks. For the coordination of the terrestrial services in respect of earth stations of GSO satellite networks, the provisions of No. S9.18 apply.

- For the aeronautical mobile service for telemetry, the requirement for coordination is determined by band overlap only (S5.348).

- See Rule of procedure on No. S5.357.

- The radiolocation service is subject to the application of the provisions of No. S9.16 vis-à-vis stations in the mobile-satellite service only.

- For the coordination of the terrestrial services in respect of earth stations of the broadcasting-satellite service the provisions of No. S9.19 apply.

- Stations in the aeronautical radionavigation service in this band are subject to power limits indicated in Recommendations ITU-R S.1340 (see 	No. S5.511C)

2	 	For cases not covered under this paragraph, the Bureau, in collaboration with the appropriate ITU-R Study Groups, continue to develop applicable calculation methods and criteria in the form of Rules of Procedure to be submitted to the RRB for approval.

1	 	Frequencies for common use listed in Section V of the Preface to the IFL shall not be notified.

2	 	Frequency assignments to stations in the amateur service shall not be notified (No. S11.14).

3	 	Frequency assignments to broadcasting stations in the high frequency bands allocated to broadcasting service between 5 900 kHz and 26 100 kHz which are subject to the procedure of Article S12 shall not be notified under Article S11 (see No. S11.14).

WRC-2000 suppressed the former Appendix S7 and replaced it by a new one. The new Appendix has a completely different structure from the former one. Amongst other things, the former Table II (Parameters required for the determination of coordination distances for a receiving earth station) has been replaced by a set of four new Tables (Tables 8a, 8b, 8c and 8d) which all specify the parameters of transmitting terrestrial stations for determining the coordination distances for a receiving earth station.  The Board considers that the continuing reference to “Table II of Appendix S7” in provision No. S11.21 is a simple omission from WRC-2000, and that the correct reference should be “Tables 8a, 8b, 8c and 8d of Appendix S7”.

The e.i.r.p. given in Tables 8c and 8d of Appendix S7 is derived from a total e.i.r.p. of 55 dBW.

In cases where the Bureau is not in a position to publish notification data under No. S11.28 within three years following the date of notification, a comment received three months after the date of publication shall be taken into account by the Bureau.

4	 	With respect to the application of this provision to assignments of the Broadcasting-satellite service submitted under Resolution 33 (Rev.WRC-97) see comments under Rules of Procedure concerning No. S23.13.

5	 	In bands shared by terrestrial and space radiocommunication services, the administration may use passive repeaters in the fixed service (radio-relay systems). While generally the passive repeater is situated close to the transmitting or receiving station, it usually involves a major change in the direction of the maximum radiation which may further affect the orbit; for this reason, the Board decided that administrations shall be requested to notify both parts of the link as separate stations, i.e., transmitting stations to passive repeater and passive repeater to receiving stations; and that each of the notices, containing information in accordance with Appendix S4, is treated as a separate assignment representing a separate station.

6	 	See Rule of Procedure relating to No. S21.11.

7	 	See Rule of Procedure relating to No. S21.14.

1	 	Whilst this Rule of Procedure applies to space services, the arrangements referred to in paragraph 1bis apply equally to submissions relating to terrestrial services.

2	 	The Board notes that there is an inconsistency between the English (and Spanish) and French texts of S11.28. While the English (and Spanish) texts stipulate that “it shall be examined in the date order of their receipt”, the French text stipulates that “…il les examinera dans l’ordre ou il les recoit”. There is no mention of “date” in the French text. The current practice of processing in the date order of their receipt will continue until the matter is considered by the next WRC.