ANNEX 1
Draft revised Rules concerning No. S9.11A of the RR

Remarks by the Radiocommunication Bureau:

- Proposed modifications to the Rules of Procedure on No. S9.11A were set out in Annex 10 to Circular Letter CCRR/10 of 5 March 2001. A number of comments were received from administrations in respect of those proposed modifications. The Radio Regulations Board gave initial consideration to the principles embodied in the draft Rules of Procedure, as noted in the “Reasons”, at its 23rd meeting (14 – 18 May 2001). However RRB deferred further consideration of the draft until its next (24th) meeting (10 – 18 September 2001).

- In the light of the comments from administrations relating to the readability/completeness of the tables attached to the proposed Rules on No. S9.11A, the Bureau continued an in-depth examination of the applicability of No. S9.11A in particular in the frequency ranges 10.7 – 11.7 GHz, 12.5 – 12.75 GHz, 17.3 – 18.6 GHz, 19.3 – 19.7 GHz, 19.7 – 20.2 GHz and 29.1 – 29.5GHz, which has resulted in further modifications that are proposed for your comments.

  - 10.7 – 11.7 GHz, 12.5 – 12.75 GHz
In considering the FSS allocations to non-GSO systems (Earth-to-space) in the bands 12.5 - 12.75 GHz in Region 1 and 12.7 - 12.75 GHz in Region 2, and BSS allocation in the band 12.5 – 12.75 GHz in Region 3, the Board has noticed that there was no footnote in the Table of Frequency Allocations, nor reference in any resolutions, to the application of No. S9.12 coordination for these FSS and BSS allocations. However, non-GSO FSS systems (Earth-to-space) would have to comply with the EPFD  limits listed in Article S22, Table S22.2 and such non-GSO FSS (Earth-to-space) systems may request to be included in the S9.12 coordination of a non-GSO system (space-to-Earth). The Board considered whether such asymmetry should be corrected (i.e. with application of No. S9.12 also to both uplink allocations) and decided that, at this stage, non-GSO FSS  (Earth-to-space) in the above two bands as well as non-GSO BSS (R3) should be subject to No. S9.12 and listed in column 4 of Table S9.11A-1 on an interim basis until WRC-03, and that this issue, including the relationship between non-GSO FSS or BSS systems and GSO FSS or BSS and terrestrial systems, should be drawn to the attention of WRC-03. The Board noted a similar situation for the uplink allocation to FSS in the band 10.7 – 11.7 GHz in Region 1, except that in this case no EPFD  limits exists. The Board decided to apply a similar approach as the one for the bands 12.5 – 12.75 GHz.

  - 17.3 – 18.6 GHz
In considering the BSS allocation in the band 17.3 – 17.8 GHz in Region 2 and the FSS allocation (Earth-to-space) in the band 18.1 – 18.4 GHz, the Board has noticed that there is no restriction to the use of non-GSO BSS systems or non-GSO FSS systems in these allocations. The Board considered whether the application of No. S9.12 should also apply to non-GSO BSS systems in the band 17.3 – 17.8 GHz and to non-GSO FSS (Earth-to-space) systems in the band 18.1 – 18.4 GHz and decided that, at this stage, non-GSO BSS and non GSO FSS (Earth-to-space) should be subject to No. S9.12 and listed in column 4 of Table S9.11A-1 on an interim basis until WRC-03, and that this issue, including the relationship between non-GSO FSS or BSS systems and GSO FSS or BSS and terrestrial systems, should be drawn to the attention of WRC-03.

  - 19.3 – 19.7 GHz
Further clarifications were added on the date of receipt of the coordination information of GSO FSS networks.

  - 19.7 – 20.2 GHz and 29.5 – 30 GHz
In considering the MSS allocations (space-to-Earth) in the bands 19.7 – 20.2 GHz and 29.5 - 30 GHz, the Board has noticed that the relationship between non-GSO MSS/non-GSO FSS and GSO MSS systems is not clearly defined (e.g. no reference to GSO MSS systems in No. S22.2). The Board considered whether such non-GSO MSS systems should be subject to the application of No. S9.12 and also to other limits, which apply to non-GSO FSS systems. The Board decided that, at this stage, non-GSO MSS should be subject to No. S9.12 and listed in column 4 of Table S9.11A-1 on an interim basis until WRC-03, and that this issue, including the relationship between non-GSO MSS systems and GSO FSS or MSS and terrestrial systems,  should be drawn to the attention of WRC-03.

- In considering the proposed draft modifications to the Rules, RRB will consider comments already received in response to Circular Letter CCRR/10, together with any further comments made in response to this circular letter.

MOD S9.11A

Paragraph 1

NOC

Paragraph 2

# 2	Application of No. S9.11A to different services/frequency bands

Paragraphs 2.1 and 2.2

NOC

Paragraph 2.3

MOD

2.3	While recognising the difficulties of harmonising the text of the footnotes to Article S5 introduced by WARC-92, WRC-95 and WRC-97 on the one hand and the text of the provision of No. S9.11A (including Nos. S9.12 to S9.16) and S9.17A, as appropriate with respect to the services to which this provision is applicable, on the other hand, the Board concluded that the procedure is applicable to all other space and terrestrial services with respect to those satellite services having allocations with equal rights and mentioned in the specific footnotes to which this provision applies. The frequency bands are those to which, in a footnote, reference is made to this provision in the Table of Frequency Allocations- (See Tables S9.11A-1 and S9.11A-2 below). In this these Tables, there is an indication of those other space services (in addition to the mobile-satellite and radiodetermination-satellite services as well as non-GSO MSS feeder links and non-GSO FSS included in the footnotes) to which this coordination procedure shall also apply. This application is subject to the same condition as that of the space services specifically mentioned in the footnotes, e.g. the coordination of space stations of the other space services (space-to-Earth), with respect to terrestrial services, is required only if the threshold values indicated in Annex 1 to Appendix S5 are exceeded.

Paragraph 2.4

ADD

2.4	WRC-2000 decided to delete Table S5-1A of Appendix S5 subject to the condition that it be included in a Rule of Procedure with appropriate modifications (e.g. inclusion of terrestrial services etc.) (minutes of the Plenary (B17) refer). The extended version of the above-mentioned table is contained in Tables S9.11A-1 and S9.11A-2, based on the following considerations:

a)	No. S9.14 applies to space-to-Earth frequency allocations only, i.e. coordination of a transmitting space station in respect of receiving terrestrial stations when the threshold value is exceeded. In the absence of threshold value, the provisions of No. S9.50.1 could apply (see also Appendix S5). For the Earth-to-space frequency allocations, the Board is of the opinion that no coordination is required between the involved space and terrestrial services, which both are considered on an equal basis.

b)	No. S9.15 applies to Earth-to-space and space-to-Earth frequency allocations of a 
non-geostationary satellite network to which No. S9.11A refers, i.e. coordination of a transmitting earth station in respect of receiving terrestrial stations and coordination of a receiving earth station in respect of transmitting terrestrial stations, if the coordination area of the earth station in a non-geostationary satellite network includes the territory of another country (see also Appendix S5).

c)	No. S9.16 applies to space-to-Earth frequency allocations of a non-geostationary satellite network to which No. S9.11A refers only, i.e. coordination of a transmitting terrestrial station within the coordination area of a receiving earth station in a non-geostationary satellite network.

Paragraph 3

3	Frequency allocation matters

Paragraph 3.1

NOC

Paragraph 3.2

MOD

3.2	WRC-97, in its Resolution 54 (WRC-97) instructed the Bureau to apply the provisions of the Resolution 46 (Rev.WRC-97)/No. S9.11A as of 22 November 1997 to those bands in which the Resolution is mentioned even though the footnotes to the Table of Frequency Allocations are not in force until a later date. The Board understands that the earlier date of implementation of the procedure does not influence the date of entry into force of the related allocations. Tables S9.11A-1 and S9.11A-2 below contains an indications on of the dates of entry into force of the allocations concerned with the application of No. S9.11A.

Paragraph 3.3

NOC

Paragraph 4

NOC

Reason: WRC-2000 agreed to delete the Table S5-1A of Appendix S5 under the condition that it should be included in a Rule of procedure with appropriate modifications (e.g. inclusion of terrestrial services etc.) (refer minutes of the Plenary (B17)). The Board carefully studied the existing Rule of procedure on No. S9.11A (version 1998), taking due account of its evolution since the former Rule on Res.46/S9.11A (version 1994) and applied the following considerations to the relationship between the different space and terrestrial services in developing the draft modifications to the S9.11A Rule of procedure:

- Appendix S5, paragraph 1 states that ”For the purpose of effecting coordination under Article S9…the frequency assignments to be taken into account are those in the same frequency band as the planned assignment, pertaining to the same service or to another service to which the band is allocated with equal rights or higher category of allocation…” Footnote 1 restricts this application to assignments in bands allocated with equal rights in case of coordination under Nos. S9.15-S9.19. The board considered the application of coordination under Nos. S9.11A-S9.14 between frequency assignments in bands allocated with different category of allocation and taking into account Nos. S5.28-S5.31, it confirmed the practice applied by the Bureau since 1992 to examine coordination under Nos. S9.11A-S9.14 between services with equal status only (see Table 1 to RS46 Rule of Procedure (Edition 1994)). Noting however the text in paragraph 1 to Appendix S5, the Board considers that the attention of a future conference should be drawn to this discrepancy.

- The terminology used in No. S9.11A footnotes to the Table of Allocation to specify the space/terrestrial (or other services) relationship is various and somehow ambiguous in some cases. Whilst the terminology used in e.g., Nos. S5.221, S5.286B, S5.352A, S5.374…(“stations in the [space service]…shall not cause harmful interference to…or claim protection from…stations in the [terrestrial service]” could be interpreted as a total protection of existing and planned use of the terrestrial stations, leading to the non-application of coordination under Nos. S9.14-S9.16, the type of protection afforded by such terminology as “stations in the [space service]…shall not constrain the development and use of … stations in the [terrestrial service]” , as in e.g. Nos. S5.219, S5.286C, might be questioned. Taking into account Nos. S5.43 and S5.43A, and the decisions by WRC-97 on this issue (refer Summary Record of the 12th Meeting of Committee 5), the Board is of the opinion that whenever such above references are indicated in a footnote, this would be interpreted as not requiring the application of the coordination procedure between the involved stations. The Board has carefully reviewed the wording of each No. S9.11A footnotes and has applied the above principle, as appropriate, to develop Tables S9.11A-1 and S9.11A-2.

In order to easily identify the requirements for coordination of space services on one hand and the terrestrial services on the other and bearing in mind the widespread practices in administrations where different entities deal with space and terrestrial services, two separate tables are proposed in the Rule of procedure.

ADD

TABLE S9.11A-1

Applicability of the provisions of Nos. S9.11A - S9.15 to stations of Space Services

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 137 - 137.025 137.175-137.825 | S5.208 | MOBILE-SATELLITE (non-GSO) |  | SPACE OPERATION METEOROLOGICAL-SATELLITE SPACE RESEARCH |  | FIXED (S5.204, S5.205) LAND MOBILE (S5.204, S5.205) MARITIME MOBILE (S5.204, S5.205) AERONAUTICAL MOBILE (OR) (S5.204, S5.206) BROADCASTING (S5.207) |  | 1 2 |
| 137.025-137.175 137.825-138 | S5.208 | Mobile-Satellite (non-GSO) |  | --- |  | Fixed (in countries other than those listed in S5.204, S5.205) Land mobile (in countries other than those listed in S5.204, S5.205) Maritime mobile (in countries other than those listed in S5.204, S5.205) Aeronautical mobile (OR) (in countries other than those listed in S5.204, S5.206) |  |  |
| 148-149.9 | S5.219 | MOBILE-SATELLITE (non-GSO) |  | --- (See S5.219) |  | --- (See S5.219, S5.221) |  |  |
| 149.9-150.05 | S5.220 | MOBILE-SATELLITE (non-GSO) |  | --- (See S5.220) |  | --- | Limited to LMSS System until 1.1.2015 (S5.224A) |  |
| 312-315 | S5.255 | Mobile-Satellite (non-GSO) |  | --- |  | --- |  |  |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 387-390 | S5.255 | Mobile-satellite (non-GSO) |  | --- |  | --- |  |  |
| 399.9-400.05 | S5.220 | MOBILE-SATELLITE (non-GSO) |  | --- (See S5.220) |  | --- | Limited to LMSS System until 1.1.2015 (S5.224A) |  |
| 400.15-401 | S5.264 | MOBILE-SATELLITE (non-GSO) |  | METEO-SATELLITE SPACE RESEARCH |  | FIXED (S5.262) MOBILE (S5.262) METEOROLOGICAL AIDS |  | 1 2 |
| 454-455 | S5.286A | MOBILE-SATELLITE (non-GSO) (S5.286D, S5.286E) |  | --- |  | --- (See S5.286B, S5.286C) |  |  |
| 455-456 | S5.286A | MOBILE-SATELLITE (non-GSO) (R2, S5.286E) |  | --- |  | --- (See S5.286B, S5.286C) |  |  |
| 459-460 | S5.286A | MOBILE-SATELLITE (non-GSO) (R2, S5.286E) |  | --- |  | --- (See S5.286B, S5.286C) |  |  |
| 1492-1525 | S5.348 | MOBILE-SATELLITE (R2, except USA (S5.344)) |  | --- |  | FIXED MOBILE |  | 3 |
| 1525-1530 | S5.354 | MOBILE-SATELLITE |  | SPACE OPERATION |  | FIXED (R1, R3, see also S5.352A) LAND MOBILE (S5.349) MARITIME MOBILE (S5.349) AERONAUTICAL MOBILE (S5.342, S5.350) |  | 1 |
| 1530-1535 | S5.354 | MOBILE-SATELLITE |  | SPACE OPERATION |  | AERONAUTICAL MOBILE (S5.342) |  | 1 |
| 1535-1545 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- |  |  |
| 1545-1550 | S5.354 | MOBILE-SATELLITE |  | --- |  | AERONAUTICAL MOBILE (R) (S5.357) |  | 4 |
| 1550-1555 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) AERONAUTICAL MOBILE (R) (S5.357) |  | 4 |
| 1555-1559 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) |  |  |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 1610-1626.5 | S5.364 | MOBILE-SATELLITE (except S), RADIODETERMINATION-SATELLITE (R2+S5.369) |  | --- |  | --- (See S5.364) |  |  |
| 1610-1613.8 | S5.364 | Radiodetermination-satellite (R1 (S5.371), R3, VEN (S5.370)) |  | --- |  | Fixed (S5.355) |  |  |
| 1613.8-1626.5 | S5.364 S5.365 | Radiodetermination-satellite (R1 (S5.371), R3, VEN (S5.370)) Mobile-satellite |   | --- |  | Fixed (S5.355) |  |  |
| 1613.8-1626.5 | S5.365 | Mobile-satellite |  | --- |  | Fixed (S5.355) |  |  |
| 1626.5-1631.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) |  |  |
| 1631.5-1634.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- (See S5.374) |  |  |
| 1634.5-1645.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) |  |  |
| 1645.5-1646.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- |  |  |
| 1646.5-1656.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | FIXED (S5.359) AERONAUTICAL MOBILE (R) (S5.376) |  |  |
| 1656.5-1660.5 | S5.354 | MOBILE-SATELLITE |  | --- |  | --- (See S5.374) |  |  |
| 1675-1690 | S5.377 | MOBILE-SATELLITE (R2) |  | --- (See S5.377) |  | FIXED LAND MOBILE MARITIME MOBILE |  |  |
| 1690-1700 | S5.377 | MOBILE-SATELLITE (R2) |  | --- (See S5.377) |  | FIXED (R2 and R3 countries listed in S5.381 and S5.382, R1 countries listed in S5.382) LAND MOBILE (R2 and R3 countries listed in S5.381, R1 countries listed in S5.382) MARITIME MOBILE (R2 and R3 countries listed in S5.381, R1 countries listed in S5.382) |  |  |
| 1700-1710 | S5.377 | MOBILE-SATELLITE (R2) |  | SPACE RESEARCH (S5.384) |  | FIXED LAND MOBILE MARITIME MOBILE |  | 1 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 1980-1990 | S5.389A | MOBILE-SATELLITE |  | --- |  | FIXED (except R2 countries in S5.389B) MOBILE (except R2 countries in S5.389B) (see also S5.389F) | 1.1.2005 in R2 |  |
| 1990-2010 | S5.389A | MOBILE-SATELLITE |  | --- |  | FIXED MOBILE (see also S5.389F) |  |  |
| 2010-2025 | S5.389C | MOBILE-SATELLITE (R2) |  | --- |  | FIXED (R2) MOBILE (R2) (see also S5.390 and S5.389E) | 1.1.2002 (1.1.2000 in CAN, USA) |  |
| 2160-2170 | S5.389C | MOBILE-SATELLITE (R2) |  | SPACE RESEARCH S5.392A (RUS) |  | FIXED (R2) MOBILE (R2) (see also S5.390, S5.392A and S5.389E) | 1.1.2002 (1.1.2000 in CAN, USA) | 1 5 |
| 2170-2200 | S5.389A | MOBILE-SATELLITE |  | SPACE RESEARCH S5.392A (RUS) |  | FIXED MOBILE (see also S5.389F andS5.392A) |  | 1 5 |
| 2483.5-2500 | S5.402 | MOBILE-SATELLITE RADIODETERMINATION-SATELLITE (R2 & R1/R3 countries in S5.400) |  | --- |  | RADIOLOCATION (R2, R3, F)(S5.397, S5.399) FIXED MOBILE |  | 7 |
| 2483.5-2500 | S5.402 | Radiodetermination-satellite (R1&R3) |  | --- |  | --- (See S5.399) |  |  |
| 2500 -2515 | S5.414 S5.403 | MOBILE-SATELLITE (except countries in S5.412) |  | FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) |  | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) | 1 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 2515 - 2520 | S5.414 S5.403 | MOBILE-SATELLITE (except countries in S5.412) |  | FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | FIXED, LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) 1.1.2002 (AMSS in IND) | 1 |
| 2520 -2535 | S5.403 | MOBILE-SATELLITE (except AERONAUTICAL MOBILE-SATELLITE) (except countries in S5.412 and S5.417) |  | BROADCASTING-SATELLITE, FIXED SATELLITE (R2&3) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | 1.1.2002 (AMSS in IND) | 1 6 |
| 2630-2655 | S5.418A S5.418B | BROADCASTING-SATELLITE (Non-GSO) (sound) FIXED-SATELLITE (R2) (Non-GSO) |  | BROADCASTING-SATELLITE (non-GSO) |  | --- (see Res.539 (WRC-2000)) |  | 8 9 |
|  | S5.418C | BROADCASTING-SATELLITE (GSO) FIXED-SATELLITE (R2) (GSO) |  |  |  |  |  |  |
| 2630-2655 | S5.418A S5.418B S5.418C | BROADCASTING-SATELLITE (Non-GSO) (sound) (S5.418) BROADCASTING-SATELLITE (GSO) FIXED-SATELLITE (R2) ( GSO and Non-GSO) |  | BROADCASTING-SATELLITE (non-GSO) |  | --- (see Res.539 (WRC-2000)) |  | 8 9 |
| 2655-2670 | S5.420 | MOBILE-SATELLITE (except AERONAUTICAL MOBILE-SATELLITE) (except countries in S5.412 and S5.417) |  | BROADCASTING-SATELLITE, FIXED SATELLITE (R2 and R3) |   | FIXED LAND MOBILE MARITIME MOBILE |  | 1 |
| 2670-2690 | S5.419 S5.420 | MOBILE-SATELLITE (except countries in S5.412) |  | FIXED SATELLITE (R2 and R3), AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.420A) |  | FIXED, LAND MOBILE MARITIME MOBILE | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) 1.1.2002 (AMSS in IND) | 1 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz/GHZ) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 5091-5150 | S5.444A | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | AERONAUTICAL-MOBILE SATELLITE (R) (S5.367) |  | --- (See S5.444A) |  |  |
| 5150 - 5216 | S5.447B | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | RADIODETERMINATION-SATELLITE  (S5.446), with date of bringing into use prior to 17.11.1995 |  | AERONAUTICAL RADIONAVIGATION, MOBILE (S5.447) |  | 1 13 14 |
|  | S5.447A S5.447C | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  |  |  |  |  |  |
| 5216 - 5250 | S5.447A S5.447C | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | RADIODETERMINATION-SATELLITE (S5.446), with date of bringing into use prior to 17.11.1995 |  | AERONAUTICAL RADIONAVIGATION, MOBILE (S5.447) |  | 1 13 |
| 6700 - 7075 | S5.458B | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | non-GSO FIXED-SATELLITE in bands 6700 – 6725 MHz and 7025 – 7075 MHz |  | FIXED MOBILE |  | 1 13 |
| 10.7 - 11. 7 | S5.441 S5.484A | Non-GSO FIXED-SATELLITE |  | Non-GSO FIXED-SATELLITE (R1) |  | --- |  | 10 11 |
| 11.7-12.5 | S5.484A S5.487A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 12.5-12.7 | S5.484A S5.487A | Non-GSO FIXED-SATELLITE |  | Non-GSO FIXED-SATELLITE (R1) Non-GSO BROADCASTING-SATELLITE (R3) |   | --- |  | 10 11 |
| 12.7-12.75 | S5.484A | Non-GSO FIXED-SATELLITE (R1, R3) |  | Non-GSO FIXED-SATELLITE (R1, R2) Non-GSO BROADCASTING-SATELLITE (R3) |   | --- |  | 10 11 |
| 12.75-13.25 | S5.441 | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 13.75-14.5 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (GHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 15.43-15.63 | S5.511A | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | --- |  | AERONAUTICAL RADIONAVIGATION |  | 12 14 |
|  |  | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  |  |  |  |  |  |
| 15.63-15.65 | S5.511D | Non-GSO FIXED-SATELLITE |  | FIXED-SATELLITE |  | AERONAUTICAL RADIONAVIGATION |  | 1 12 |
| 17.3-17.7 | S5.516 | Non-GSO FIXED-SATELLITE (R1, R3) |  | Non-GSO BROADCASTING-SATELLITE (R2) |  | --- |  | 10 11 |
| 17.7-17.8 | S5.516 | Non-GSO FIXED-SATELLITE (R1, R3) |  | Non-GSO FIXED-SATELLITE in the band 17.7 – 17.8 GHz (R1, R2, R3) Non-GSO BROADCASTING-SATELLITE (R2) |  | --- |  | 10 11 |
| 17.8-18.1 | S5.516 | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
|  | S5.484A |  |  |  |  |  |  |  |
| 18.1-18.4 | S5.484A | Non-GSO FIXED-SATELLITE |  | Non-GSO FIXED-SATELLITE |  | --- |  | 10 11 |
| 18.4-18.6 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 18.8 – 19.3 | S5.523A | FIXED-SATELLITE |  | --- |  | FIXED MOBILE |  | 13 |
| 19.3 – 19.6 | S5.523B | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  | --- |  | FIXED MOBILE |  |  |
|  | S5.523D | FIXED-SATELLITE (GSO with coordination information received as of 18.11.1995 and non-GSO MOBILE SATELLITE SERVICE feeder link) (see also S5.523C) |   |  |  |  |  | 13 14 |

| 1 | 2 | 3 |  | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (GHz) | Footnote in Art. S5 | Space services mentioned in a footnote referring to No. S9.11A to which Nos. S9.12 to S9.15 apply, or referring to Nos. S9.12 – S9.13, as appropriate |  | Other space services to which Nos. S9.12 to S9.14 apply equally |  | Terrestrial services in respect of which Nos. 9.14 and S9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 19.6 – 19. 7 | S5.523D | FIXED-SATELLITE (GSO with coordination information received as of 22.11.1997 and non-GSO MOBILE SATELLITE SERVICE feeder link) (see also S5.523E) |   | FIXED-SATELLITE (GSO with coordination information received as of 22.11.1997 and non-GSO) (see also S5.523E) |  | FIXED MOBILE |  | 13 14 |
| 19.7 – 20.1 | S5.484A | Non-GSO FIXED-SATELLITE |  | Non-GSO MOBILE-SATELLITE (R2) |  | --- |  | 10 11 |
| 20.1 – 20.2 | S5.484A | Non-GSO FIXED-SATELLITE |  | Non-GSO MOBILE-SATELLITE |  | --- |  | 10 11 |
| 27.5 - 28.6 | S5.484A | Non-GSO FIXED-SATELLITE |  | --- |  | --- |  | 10 11 |
| 28.6 - 29.1 | S5.523A | FIXED-SATELLITE |  | --- |  | FIXED MOBILE |  |  |
| 29.1 - 29.5 | S5.535A | FIXED-SATELLITE (GSO (see also S5.523C and S5.523E) and non-GSO MOBILE SATELLITE SERVICE feeder link) |  | --- |  | FIXED MOBILE |  |  |
| 29.5 – 29.9 | S5.484A | Non-GSO FIXED-SATELLITE |  | Non-GSO MOBILE-SATELLITE (R2) |  | --- |  | 10 11 |
| 29.9 - 30 | S5.484A | Non-GSO FIXED-SATELLITE |  | Non-GSO MOBILE-SATELLITE |  | --- |  | 10 11 |

Notes:

- The coordination of non-GSO space services (Earth stations) in respect of terrestrial services is to be effected under the provisions of No. S9.15. For the coordination of GSO space services (Earth stations) in respect of terrestrial 	services columns 4 and 5 of Table S9.11A, the provisions of No. S9.17 apply.

- Coordination thresholds indicated in Annex 1 to Appendix S5 apply only to MOBILE-SATELLITE service.

- For AERONAUTICAL MOBILE service for telemetry, the requirement for coordination is determined by band overlap only (S5.348).

- See Rule of procedure on No. S5.357.
5.	SPACE RESEARCH service is not subject to the application of the provisions of Nos. S9.14 and S9.15.

- The coordination of BROADCASTING-SATELLITE service in respect of terrestrial services is to be effected under the provisions of No. S9.11.

- RADIOLOCATION service is subject to the application of the provisions of Nos. S9.14 and S9.15 vis-à-vis stations in the MOBILE-SATELLITE service only.

- The coordination of non-GSO BROADCASTING-SATELLITE service  (sound) in respect of terrestrial services is subject to the provisions of Resolution 539. (WRC-2000).

- The coordination of non-GSO BROADCASTING-SATELLITE service (column 4) is subject to the provisions of No. S9.12 only.

- The coordination of space services (Earth stations) listed in columns 3 in respect of the terrestrial services to which the same band is allocated with equal rights is to be effected under the provisions of S9.17.

- Subject to application of the provisions of No. S9.12 only.

- Non-GSO FIXED-SATELLITE service (space-to-Earth) subject to application of the provisions of No. S9.14 in the frequency band 15.45 – 15.65 GHz only

- Not subject to application of the provisions of No. S9.14.

- For the coordination of GSO or non-GSO space services (specific earth station) in respect of other earth stations operating in the opposite direction of transmission, see also the provisions of No. S9.17A.

ADD

TABLE S9.11A-2

Applicability of the provisions of No. S9.16 to stations of Terrestrial Services

| 1 | 2 | 3 | 4 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz/GHz) | Footnote in Art. S5 | Terrestrial services to which No. S9.16 applies | Space services mentioned in a footnote referring to No. S9.11A in respect of which No. S9.16 applies, and other non-GSO space services in respect of which No. S9.16 applies equally |  | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 137 - 137.025 137.175-137.825 | S5.208 | FIXED (S5.204, S5.205) LAND MOBILE (S5.204, S5.205) MARITIME MOBILE (S5.204, S5.205) AERONAUTICAL MOBILE (OR) (S5.204, S5.206) BROADCASTING (S5.207) | MOBILE-SATELLITE (non-GSO (S5.209)) SPACE OPERATION METEOROLOGICAL-SATELLITE SPACE RESEARCH |  |  | 1 |
| 137.025-137.175 137.825-138 | S5.208 | Fixed (in countries other than those listed in S5.204, S5.205) Land mobile (in countries other than those listed in S5.204, S5.205) Maritime mobile (in countries other than those listed in S5.204, S5.205) Aeronautical mobile (OR) (in countries other than those listed in S5.204, S5.206) | Mobile-satellite (non-GSO (S5.209)) |  |  | 1 |
| 400.15-401 | S5.264 | FIXED (S5.262) MOBILE (S5.262) METEOROLOGICAL AIDS | MOBILE-SATELLITE (non-GSO (S5.209)) METEOROLOGICAL-SATELLITE SPACE RESEARCH |  |  | 1 |
| 1492-1525 | S5.348 | FIXED (R2) MOBILE (R2) | MOBILE-SATELLITE (R2, except USA (S5.344)) |  |  | 1, 2 |
| 1525-1530 | S5.354 | FIXED (R1, R3, see also S5.352A) LAND MOBILE (S5.349) MARITIME MOBILE (S5.349) AERONAUTICAL MOBILE (S5.342, S5.350) | MOBILE-SATELLITE SPACE OPERATION |  |  | 1 |
| 1530-1535 | S5.354 | AERONAUTICAL MOBILE (S5.342) | MOBILE-SATELLITE SPACE OPERATION |  |  | 1 |
| 1545-1550 | S5.354 | AERONAUTICAL MOBILE (R) | MOBILE-SATELLITE |  |  | 1, 3 |
| 1550-1555 | S5.354 | FIXED (S5.359) AERONAUTICAL MOBILE (R) | MOBILE-SATELLITE |  |  | 1 |
| 1555-1559 | S5.354 | FIXED (S5.359) | MOBILE-SATELLITE |  |  | 1 |
| 1613.8-1626.5 | S5.365 | Fixed (S5.355) | Mobile-satellite |  |  | 1 |
| 2160-2170 | S5.389C | FIXED (R2) MOBILE (R2) (see also S5.390, S5.392A and S5.389E) | MOBILE-SATELLITE (R2) |  | 1.1.2002 (1.1.2000 in CAN, USA) | 1 |
| 2170-2200 | S5.389A | FIXED MOBILE (see also S5.392A and S5.389F) | MOBILE-SATELLITE |  |  | 1 |

| 1 | 2 | 3 | 4 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz/GHz) | Footnote in Art. S5 | Terrestrial services to which No. S9.16 applies | Space services mentioned in a footnote referring to No. S9.11A in respect of which No. S9.16 applies, and other non-GSO space services in respect of which No. S9.16 applies equally |  | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |
| 2483.5-2500 | S5.402 | RADIOLOCATION (R2, R3, F) (S5.397, S5.399) FIXED MOBILE | MOBILE-SATELLITE RADIODETERMINATION-SATELLITE (R2+ R1/R3 countries in S5.400) |  |  | 1, 4 |
| 2500 -2515 | S5.414 S5.403 | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | MOBILE-SATELLITE (except countries in S5.412) FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) |  | 1.1.2005 (until 2005: S9.21: (MSS -AMSS)) 1.1.2002 (AMSS in IND) | 1 |
| 2515 - 2520 | S5.414 S5.403 | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | MOBILE-SATELLITE (except countries in S5.412) FIXED SATELLITE (R2&3), RADIODETERMINATION-SATELLITE (S5.404) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | 1.1.2005 (until 2005: S9.21: MSS (-AMSS)) 1.1.2002 (AMSS in IND) | 1 |
| 2520 -2535 | S5.403 | FIXED LAND MOBILE MARITIME MOBILE RADIOLOCATION (F) | MOBILE-SATELLITE (except AERONAUTICAL MOBILE-SATELLITE) (except countries in S5.412) BROADCASTING-SATELLITE, FIXED SATELLITE (R2&3) AERONAUTICAL MOBILE-SATELLITE (J, IND) (S5.415A) |  | 1.1.2002 (AMSS in IND) | 1, 5 |
| 5150 - 5216 | S5.447B | AERONAUTICAL RADIONAVIGATION MOBILE (S5.447) | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) RADIODETERMINATION-SATELLITE (S5.446), with date of bringing into use prior to 17.11.1995 |  |  | 1 |
| 6700 - 7075 | S5.458B | FIXED MOBILE | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link (S5.458B)) |  |  | 1 |
| 15.43-15.63 | S5.511A | AERONAUTICAL RADIONAVIGATION | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link (S5.511A)) |  |  | 1, 6 |
| 15.63-15.65 | S5.511D | AERONAUTICAL RADIONAVIGATION | FIXED-SATELLITE (limited to non-GSO (S5.511D)) |  |  | 1 |
| 18.8 - 19.3 | S5.523A | FIXED MOBILE | FIXED-SATELLITE |  |  | 1 |
| 19.3 - 19.7 | S5.523D | FIXED MOBILE | FIXED-SATELLITE (limited to non-GSO MOBILE SATELLITE SERVICE feeder link) |  |  | 1 |

Notes:

- The coordination of the terrestrial services under the provisions of No. S9.16 is to be effected only in respect of earth stations of non-GSO satellite 	networks. For the coordination of the terrestrial services in respect of earth stations of GSO satellite networks, the provisions of No. S9.18 apply.

- For the aeronautical mobile service for telemetry, the requirement for coordination is determined by band overlap only (S5.348).

- See Rule of procedure on No. S5.357.

- The radiolocation service is subject to the application of the provisions of No. S9.16 vis-à-vis stations in the mobile-satellite service only.

- For the coordination of the terrestrial services in respect of earth stations of the broadcasting-satellite service the provisions of No. S9.19 apply.

- Stations in the aeronautical radionavigation service in this band are subject to power limits indicated in Recommendations ITU-R S.1340 (see 	No. S5.511C).

# ANNEX 2
Draft new Rules concerning APPENDIX S4 to the RR

An. 2A

ADD

A.17 a)

The Board noted that Annex 2A of Appendix S4, item A.17 a) requires that “for non-geostationary-satellite systems operating in the radionavigation-satellite service in the band 5 010-5 030 MHz, the aggregate power flux-density produced at the Earth’s surface in the band 5 030-5 150 MHz in a 150 kHz bandwidth and in the band 4 990-5 000 MHz in a 10 MHz bandwidth, as defined in No. S5.443B” be provided by the notifying administration. According to the provisions of No. S5.443B and resolves 3), Resolution 604 (WRC-2000), this data element is, however, also required for submission of information on geostationary-satellite systems operating in the radionavigation-satellite service in the band 5 010-5 030 MHz. The reference to geostationary-satellite systems in item A.17 a) was probably omitted by inadvertence at WRC-2000. To correct the inconsistency, the Board decided that Administration will be requested to provide, in addition to the relevant characteristics listed in Appendix S4, for any geostationary-satellite system operating in the radionavigation-satellite service in the band 5 010-5 030 MHz, the aggregate power flux-density produced at the Earth’s surface in the band 5 030-5 150 MHz in a 150 kHz bandwidth and in the band 4 990-5 000 MHz in a 10 MHz bandwidth, as defined in No. S5.443B. The Bureau, in the completeness examination of the submitted data, will then also take account of the item A.17 a) requirement for geostationary-satellite systems.

ADD

A.17 b)

The Board noted that Annex 2A of Appendix S4, item A.17 b) correctly reflect the requirement as indicated in the first sentence of No. S5.551G. A similar requirement however also exists for geostationary-stations in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service (space-to-Earth) operating in the band 42 – 42.5 GHz (refer 2nd sentence of No. S5.551G and resolves of Resolution 128 (Rev. WRC-2000)). No. S5.551G requirement for geostationary-satellite stations under Annex 2A of Appendix S4, part A.17 was probably omitted by inadvertence at WRC-2000. To correct the inconsistency the Board decided that Administration will be requested to provide, in addition to the relevant characteristics listed in Appendix S4, the calculated value of the power flux-density in the band 42.5-43.5 GHz produced by any geostationary station in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service (space-to-Earth) operating in the band 42-42.5 GHz at the site of a radio astronomy station, as defined in No. S5.551G. The Bureau, in the completeness examination of the submitted data, will then also take account of No. S5.551G requirement for geostationary-satellite stations.

Reasons: To correct inconsistency between Annex 2A of Appendix S4, part A.17 and requirements provided in No. S5.443B/Resolution 604 (WRC-2000) and No. S5.551G/Resolution 128 (Rev. WRC-2000)

ANNEX 3
Draft new Rules concerning RESOLUTION 57 (WRC-2000)

ADD

Modification of bringing into use and administrative due diligence requirements

as a consequence of allocation changes

above 71 GHz

- Resolution 57 (WRC-2000) concerns the modification of the date of bringing into use and administrative due diligence requirements of satellite network assignments above 71 GHz as a consequence of allocation changes above 71 GHz agreed by WRC-2000. In its resolves 3 and 2, Resolution 57 indicates, respectively, that “…the notifying administration shall have until 31 December 2000 to resubmit to the Bureau the Appendix S4 advance publication information and coordination information…reflecting the proposed modification…above 71 GHz” and that “…there shall be no change in the date that the advance publication or coordination information is considered as having been received by the Bureau.” In its resolves 1 and 4, Resolution 57 also indicates, respectively, that “…Bureau will extend the notified date of bringing into use under No. S11.44 up to 3 June 2007 at the request of the notifying administration” and that “the provisions contained in Nos. S11.44B to S11.44I are applicable with respect to the date of bringing into use communicated to the Bureau under resolves 3. “

- In view of the above, the Board agreed that:

  - Taking account of resolves 1 and 4 of Resolution 57 (WRC-2000), the latest date for bringing into use frequency assignments resubmitted under resolves 3 of the Resolution shall be 3 June 2007, including any extension that may be granted according to the conditions set forth in Nos. S11.44B to S11.44I.

  - Taking account of resolves 2 and 3 of the Resolution, the latest planned date for bringing into use assignments in the additional band above 71 GHz, as notified in the advance publication and associated coordination information, shall be 5 years from the date of receipt of the resubmitted advance publication information. This applies to frequency bands above 71 GHz not already covered by an earlier API. Any postponement of the date of bringing into use (beyond the original 5 years) under Nos. S11.44B to S11.44I shall be receivable if the date of bringing into use is no later than 3 June 2007.

  - In accordance with resolves 3 of the Resolution, any resubmitted advance publication and coordination information relating to the frequency band above 71 GHz received by the Bureau prior to 31 December 2000 shall keep the date of receipt of the original advance publication and coordination information.

- The Board carefully studied the wording of resolves 6 of Resolution 57 (WRC-2000) and concluded that it should have been reviewed by WRC-2000 after the Conference decided to suppress the original text of the same section, as contained in resolves 6 of Resolution [COM5/8] (WRC-2000 Document 495). Consequently, and taking into account the minutes of the Tenth Plenary Meeting (WRC-2000 Document 539), the Board is of the opinion that resolves 6 of Resolution 57 (WRC-2000) is not applicable.

ANNEX 4

Draft new Rules of Procedure relating to
§§ 5.1.3, 5.1.4 and 5.1.5 of Article 5 of Appendix S30A (WRC-2000), and
Articles Nos. S9.17 and S9.17A of the Radio Regulations
(Notification of assignments in a Regions 1 and 3 feeder-link Plan or List)

1.	In revising the Appendix S30A feeder-link Plans for Regions 1 and 3, WRC-2000 also adopted revised versions of Article 4 for the coordination and of Article 5 for notification, examination and recording in the MIFR of frequency assignments submitted under Appendix S30A.

2.	In doing so, WRC-2000 has excluded from the procedure of Article 4 of Appendix S30A the coordination of the transmitting feeder-link earth stations with respect to receiving earth stations operating in the opposite direction of transmission, as well as with respect to terrestrial stations. The coordination of specific transmitting earth stations with associated receiving space station(s), which is still necessary before these specific earth stations are notified, should now be undertaken by the notifying administration directly with the other concerned administrations with respect to terrestrial stations and receiving earth stations operating in the opposite direction of transmission, in accordance with the relevant/corresponding provisions of Article S9 of the Radio Regulations (RR).

3.	§ 5.1.3 of Article 5 of Appendix S30A requires the coordination of a specific transmitting feeder-link earth station, which is notified under that Article with an e.i.r.p. greater than the sum of the values specified in the columns 11 and 12 (i.e. e.i.r.p. and power control) of the Regions 1 and 3 feeder-link Plan, with each administration whose territory lies wholly or partly within the coordination area established using the Appendix S7 methodology. The purpose of § 5.1.3 is therefore now covered by the relevant/corresponding provisions of Articles S9 and S11 of the RR.

4.	§ 5.1.4 of Article 5 of Appendix S30A requires the coordination of a specific transmitting feeder-link earth station, which is notified under that Article, with each administration whose territory lies wholly or partly within the coordination area established using the Appendix S7 methodology, in respect of notices concerning terrestrial stations and receiving earth stations operating in the opposite direction of transmission received by the Bureau prior to 3 June 2000 for recording in the International Master Frequency Register (MIFR). The purpose of § 5.1.4 is therefore covered by Notes 3 and 4 to the Remarks Column of the Regions 1 and 3 feeder-link Plans (see Section 9A.2 to Article 9A of Appendix S30A) and partially by the relevant/corresponding provisions of Article S9 and S11 of the RR.

5.	§ 5.1.5 of Article 5 of Appendix S30A describes the course of action in case an administration with which coordination is sought under § 5.1.4 does not respond within three months. Therefore, § 5.1.5 deals also with the coordination of specific transmitting feeder-link earth stations with respect to terrestrial stations and receiving earth stations operating in the opposite direction of transmission.

6.	In view of the above, and in accordance with the scope of application of Article 5 of Appendix S30A, as clarified in Annex 2 to Circular Letter CCRR/11 dated 2 March 2001, it is proposed that the Bureau no longer require the application of §§ 5.1.3, 5.1.4 and 5.1.5 of that Article in its examination of frequency assignments notified under that Article.

7.	Moreover, it is proposed to no longer take into account the cross-references to §§ 5.1.3, 5.1.4 and 5.1.5 contained in other provisions of Article 5 to Appendix S30A (WRC-2000) when examining the frequency assignments notified under that Article.

8.	Appendix 1 to this Annex contains a draft new Rules of Procedure on §§ 5.1.3, 5.1.4 and 5.1.5 of Article 5 to Appendix S30A (WRC-2000) which is proposed in order to inform administrations on this situation. This draft new Rules is submitted for consideration by the 24th meeting of the Radio Regulations Board.

9.	It should also be noted that, in addition to the draft new Rules of Procedure proposed in Appendix 1 to this Annex, as well as the draft new Rules of Procedure on the scope of application of Article 5 of Appendix S30A proposed in Annex 2 to CCRR/11 dated 2 March 2001, the Bureau is preparing draft new Rules of Procedure relating to relevant provisions of Article S9 and S11 of the RR, in order to clarify their application for the coordination and notification of specific transmitting feeder-link earth station with associated space station(s) in a Plan or a List of Appendix S30A. In doing so the Bureau is taking account of the purpose of both WRC-97 and WRC-2000 versions of Articles 4, 5 and 9A of Appendix S30A, as well as of Resolution 53 (Rev.WRC-2000) and Resolution 542 (WRC-2000). For completeness, a cross-reference to the appropriate new Rules of Procedure, once agreed, would be inserted in § 2 of the draft new rule on § 5.1.3 of Article 5 to Appendix S30A (WRC-2000).

10.	The draft new Rules proposed in Appendix 1 to this Annex should be brought to the attention of the Special Committee on Regulatory and Procedural Matters (SC), CPM-02 and WRC-03, for a possible revision at WRC-03 of Article 5 to Appendix S30A, as appropriate.

Appendix:	1

APPENDIX 1 TO ANNEX 4

Draft new Rules of Procedure relating to
§§ 5.1.3, 5.1.4 and 5.1.5 of Article 5 of Appendix S30A (WRC-2000)

Art. 5

## Notification, examination and recording

ADD

- 5.1.3

1.	In revising the Appendix S30A feeder-link Plans for Regions 1 and 3, WRC-2000 has excluded from the procedure of Article 4 of Appendix S30A the coordination of the transmitting feeder-link earth stations with respect to receiving earth stations operating in the opposite direction of transmission, as well as with respect to terrestrial stations. This coordination should now be undertaken by the notifying administration directly with the other concerned administrations with respect to terrestrial stations and receiving earth stations operating in the opposite direction of transmission, in accordance with the relevant/corresponding provisions of Article S9 of the Radio Regulations (RR).

2.	In view of the above, and considering the Rule of Procedure on the scope of application of Article 5 of Appendix S30A (WRC-2000), the Board concluded that §§ 5.1.3, 5.1.4 and 5.1.5 of that Article need no longer to be applied by the Bureau in its technical and regulatory examination of frequency assignments notified under that Article.

3.	Moreover, the Board concluded that the cross-references to §§ 5.1.3, 5.1.4 and 5.1.5 contained in other paragraphs/provisions of Article 5 of Appendix S30A (WRC-2000) shall also no longer be taken into account by the Bureau in its technical and regulatory examination of frequency assignments notified under that Article.

ADD

- 5.1.4

1.	See the Rule of procedure on § 5.1.3 of Article 5 of Appendix S30A (WRC-2000).

ADD

- 5.1.5

1.	See the Rule of procedure on § 5.1.3 of Article 5 of Appendix S30A (WRC-2000).

_______________

ANNEX 5

Draft revised Rules of Procedure relating to § 7.1 of Article 7 to Appendix S30B

1	Article 7 of Appendix S30B, which contains the procedure for the addition of a new allotment to the FSS Plan for a new Member State of the Union, stipulates that:

"7.1	The administration of a country which has joined the Union as a new Member State ‡ shall obtain a national allotment in Part A of the Plan by the following procedure.

7.2	The administration shall submit its request for an allotment to the Bureau, with the following information:

a)	the geographical coordinates of not more than 10 test points for determining the minimal ellipse to cover its national territory;

b)	the height above sea level of each of its test points and the rain zone or zones;

c)	any special requirement, other than a fixed orbital position, which is to be taken into account to the extent practicable.

7.3	Upon receipt of the complete information (mentioned in § 7.2 above), the Bureau shall find an appropriate orbital position, if necessary using the PDA concept, and shall enter the national allotment of the new Member State ‡ of the Union in Part A of the Plan.

7.4	For this purpose the Bureau shall consult, and if necessary seek the agreement of, any administrations that may be affected."

2	§ 3 of the current Rules of Procedure relating to § 7.1 of Article 7 to Appendix S30B stipulates that:

"3	In view of the difficulties of the Bureau to apply the PDA concept in its integrity, the Board decided that the Bureau shall provisionally apply the following procedures upon receipt of the request to find an appropriate orbital position for an allotment in Part A of the Plan for a new Member State of the Union under Article 7 of Appendix S30B. The Bureau shall:

3.1	study the orbit occupancy on a case-by-case basis, and select a few (not more than 3) likely suitable orbital positions;

3.2	using the criteria of Annex 4 of Appendix S30B to determine whether the new allotment at selected orbital position(s) are compatible with the allotments of the Part A, the existing networks contained in Part B of the Plan, the assignments which appear in the Appendix S30B List and the assignments with respect to which the Bureau previously received information in accordance with Article 6;

3.3	if the result of the exercise for all three selected positions is not satisfactory (affected administrations are identified) send the results to the requesting administration recommending that it may seek the agreement(s) of affected administration(s) and upon reaching the agreement may formally submit the request for the allotment on any of the proposed position to the Bureau;

3.4	enter the new allotment in Part A of the Plan and inform administrations in its circular telegram, indicating the characteristics of the allotment concerned if no administration was identified as affected in the above mentioned exercises or if agreement is reached and the request is resubmitted."

3	In reviewing the implementation of the rules quoted in § 2 above, the Bureau has identified that they could be revised in order to improve the accuracy of the technical examinations performed to process the requests submitted under Article 7 of Appendix S30B, as well as to automate to the extent practicable and thus accelerate those examinations. In particular, the Bureau's recent experiences on BSS replanning studies carried out before and at WRC-2000 could be applied in the processing of the above-mentioned requests. In addition, the rules quoted in § 2 above could also be revised in order to be aligned with those relating to § 6.12 of Article 6 of that Appendix.

4	In this regard, Appendix 1 to this Annex contains proposed draft revised Rules of Procedure relating to § 7.1 of Article 7 to Appendix S30B. It should be noted that a contribution on this issue was submitted to the Study Group 4 Chairman for consideration by ITU-R Working Party 4A. Working Party 4A considered the subject at its last meeting (Seattle, 23 April-02 May 2001) and approved the proposed course of action. In addition, an information document was submitted to the 23rd meeting of the Radio Regulations Board (Geneva, 14 – 18 May 2001) in order to provide preliminary information on this subject. The RRB noted the information provided by the Bureau and the intention to present a draft Rule of Procedure for consideration at the next (24th) meeting.

5	The proposals contained in Appendix 1 to this Annex are mainly based on the Bureau's recent experiences gained in the BSS replanning studies before and at WRC-2000. It is worth mentioning that the implementation of these proposals in the case of Appendix S30B would not require significant efforts by the Bureau, since most of them have already been implemented in various software utilities used during the BSS replanning activities.

6	In addition, the proposals contained in Appendix 1 to this Annex, once adopted, should also significantly improve the processing time of some requests submitted under Appendix S30B, and thus provide a positive answer to the concerns expressed by some administrations on this issue during the last meeting of the RAG (Geneva, 12-16 March 2001).

APPENDIX 1 TO Annex 5

Draft revised Rules of Procedure relating to § 7.1 of Article 7 to Appendix S30B

The Board, in reviewing the subject matter, decided to revise the current Rules of Procedure relating to § 7.1 of Article 7 to Appendix S30B, as follows:

Art. 7

## New allotments to new Member States of the Union

7.1

New allotment to the Plan for a new Member State of the Union

1	Appendix S30B contains provisions inviting the Bureau, when it is requested, to provide an allotment to a new Member State of the Union.

2	The Bureau, to the extent practicable5, should endeavour to find appropriate orbital positions compatible with the Plan using, if necessary, the PDA concept (defined in § 5.3 and 5.4 of Article 5 of Appendix S30B).

3	In view of the difficulties of the Bureau to apply the PDA concept in its integrity and until the time when a method to apply that PDA concept is available, the Board decided that the Bureau shall  apply the  procedures described below as soon as possible upon receipt of the request6 to find an appropriate orbital position for an allotment in Part A of the Plan for a new Member State of the Union under Article 7 of Appendix S30B.

3.1	The Bureau shall ensure that all submitted test-points are located within the national territory of the new Member State. Test-point locations shall be verified using the ITU Digitised World Map (IDWM). In addition, in the absence of a height above sea level, a value of zero metres shall be assumed by the Bureau. With respect to the rain climatic zone(s), this information shall be defined by the Bureau based on Recommendation ITU-R P.837-3.

3.2	In order to facilitate the implementation of the orbital position selection approach described in § 3.9 below, the new Member State may provide under § 7.2c) of Article of Appendix S30B its preferred orbital position(s) and/or its preferred orbital arc(s), bearing in mind that the implementation of these preferences might not be possible due to excesses of interference to or from other allotments, existing systems or assignments of Appendix S30B.

3.3	Using the rain climatic zones defined as described in § 3.1 above, the Bureau shall establish the required minimum elevation angles associated to each test-point in accordance with section 1.3 of Annex 1 to Appendix S30B. The service arc shall then be calculated in order to meet the required minimum elevation angles of all test-points. In cases where the required minimum elevation angles cannot be obtained with a non-zero service arc, a minimum service arc of at least 20 shall be defined, provided that all test-points remain visible from any orbital position within that service arc. This value of 20 is proposed to comply with the definition of the Predetermined Arc (PDA) (i.e. ideally 10 in the case of allotments (see Article 5 of Appendix S30B)).

3.4	With regard to the generation of the minimum ellipse to cover the national territory of the new Member State, the Bureau shall apply the same assumptions as those used during the BSS replanning studies, i.e. to use only a space station antenna beam pointing error of 0.1 for the generation of elliptical beams under Article 7 of Appendix S30B.

3.5	With regard to the transmitting and receiving space station antenna maximum gain values, as a function of the major and minor axes of the ellipse, instead of using the definition contained in § 1.7.2, Section A of Annex 1 to Appendix S30B the Bureau shall use the more precise formula defined in sections 3.13.1 of Annex 5 and 3.7.1 of Annex 3 of Appendices S30 and S30A, respectively.

3.6	With regard to the calculation of the maximum power density values, the Bureau shall assume the worst-case conditions in terms of space station antenna pointing error and rotational accuracy for the calculation of the antenna gain in the direction of each test-point, in order to ensure that the objective C/N ratios defined in section 1.2 of Annex 1 to Appendix S30B are met for all test points, i.e., assume the minimum gain value of the antenna, taking into account a pointing error of 0.1 and a rotational accuracy of 1.0.

3.7	With regard to the free-space loss, instead of using the definition contained in Annex II of Appendix S8, the Bureau shall use the formulae as described in the Bureau’s internal Rule relating to MSPACE manual.

3.8	With regard to the atmospheric losses, the Bureau shall use the latest relevant ITU-R Recommendations to calculate the atmospheric losses (i.e. rain attenuation and gaseous absorption) at a given test-point. All those relevant ITU-R Recommendations are referred to in one main ITU-R Recommendation: Recommendation ITU-R P.618-7.

3.9	With regard to the selection of orbital position, the Bureau shall  use an automated approach based on an iterative process similar to the one used during the BSS replanning studies performed at WRC-2000, as follows:

3.9.1	Once the service arc is calculated, as mentioned in § 3.3 above, an iterative process is implemented to identify suitable orbital position(s) within that arc for the allotment to the new Member State in question.

3.9.2	Taking into account the possible orbital position preferred by this new Member State (see § 3.2 above), the Bureau shall start the iterative process either from that preferred orbital position, or in the absence of such a preference, to start from the orbital position located in the middle of the orbital arc preferred by this new Member State (see § 3.2 above), or in case of no preferences, to start from the orbital position located in the middle of the service arc as defined in § 3.3 above.

3.9.3	The Bureau shall identify the nearest suitable orbital positions. Considering the non-regular orbital position spacing used in Appendix S30B on the one hand and in order to shorten the time required for the implementation of that iterative process on the other hand, the Bureau shall assume a minimum orbital position step of 0.1 in this process.

3.9.4	Each new possible orbital position shall be examined by the Bureau as follows:

	regenerate the elliptical beam parameters;

	recalculate the required power density values;

	using the criteria of Annex 4 of Appendix S30B,  determine whether the new allotment at  that orbital positionis compatible with the allotments of the Part A, the existing networks contained in Part B of the Plan, the assignments which appear in the Appendix S30B List and the assignments with respect to which the Bureau previously received information in accordance with Article 6 of that Appendix. This analysis has to be carried out in both the C-band and Ku-band.

3.10	In the case where none of the orbital positions assessed in § 3.9 above provides for the allotment in question, a solution in conformity with the Appendix S30B Annex 4 criteria, the Bureau shall repeat the orbital position selection process described in § 3.9 above with improved antenna patterns for this allotment. These improved antenna patterns are described for both earth and space station antennas in sections 1.6.5 and 1.7.2 of Annex 1 to Appendix S30B, respectively.

3.11	After this second study, if there is still no orbital position providing the allotment in question with a solution in conformity with the Appendix S30B Annex 4 criteria, the most appropriate orbital position(s) shall be identified with the aim to minimise the C/I excesses caused or received from other allotment(s), existing system(s) or assignment(s) of Appendix S30B, or by applying any other appropriate criteria agreed by the new Member State in question.

3.12	 The Bureau shall send these provisional results6 to the requesting administration of the new Member State recommending that it may seek the agreement(s) of affected administration(s). Upon reaching the required agreement(s) it may  submit  a request for  an allotment  at  one of the proposed position(s) to the Bureau.

3.13	Upon receipt of this submission, the Bureau shall examine again the compatibility of the proposed allotment with the allotments in Part A, the existing networks contained in Part B, the assignments which appear in the Appendix S30B List and the assignments with respect to which the Bureau has previously received information in accordance with Article 6 of Appendix 30B, using the latest updated Reference Situation files after the processing of all submissions received under Appendix S30B before the date of receipt of the subject request, § 3 above refers. If no information is received under § 3.12 above by the time of processing of the subject request, it shall be returned to that administration with an indication that subsequent resubmission will be considered in the order of date of receipt as appropriate.

3.14	The Bureau shall send the results of its calculations to the responsible administration of the new Member State. That administration may then, within a period of 30 days after having received these results, change or adjust characteristics previously submitted and send the changes to the Bureau within that 30 days' period together with a confirmation of any previous agreement and/or new required agreements as necessary.

3.15	Upon receipt of the information mentioned in §3.14 above, the Bureau shall re-examine the situation.

3.15.1	If the results of the re-examination do not show compatibility with the allotments in Part A, the existing networks contained in Part B, the assignments which appear in the Appendix S30B List and the assignments with respect to which the Bureau has previously received information in accordance with Article 6 of Appendix 30B, the submission shall be returned to that administration with an indication that subsequent resubmission will be considered in the order of date of receipt as appropriate.

3.15.2	Otherwise, the Bureau shall enter the new allotment in Part A of the Plan and inform administrations in its circular telegram, indicating the characteristics of  this new allotment  and any possible changes to other characteristics, if no administration was identified as affected in the above mentioned exercises or if the required agreements  are reached  and all other regulatory provisions are met.

3.16	If no information is received within the period referred to in §3.14 above, the submission shall be returned to that administration with an indication that subsequent resubmission will be considered in the order of date of receipt as appropriate.

_______________

ANNEX 6

Deletion of current Rules of Procedure relating to § 8.1 of Article 8 to Appendix S30B

It is proposed to delete the current Rules of Procedure relating to § 8.1 of Article 8 to Appendix S30B once the draft new Rules of Procedure contained in Annex 6 to Circular Letter CCRR/11, dated 2 March 2001, are approved.

Reasons:

1	Annex 6 to Circular Letter CCRR/11, dated 2 March 2001, contains draft new Rules of Procedure on Annex 2B of Appendix S4 (WRC-2000) to clarify its use in application of Appendix S30B of the Radio Regulations (RR). Appendix 1 of that Annex 6 contains a proposed clarification on the scope of application of Annex 2B to Appendix S4 (WRC-2000) with respect to the mandatory data for the submission of notices in application of Appendix S30B.

2	According to the draft new Rules of Procedure mentioned in § 1 above, the power density values mentioned in the current Rules of Procedure relating to § 8.1 of Article 8 to Appendix S30B have to be submitted by the administrations when notifying their frequency assignments under Article 8 of Appendix S30B in the same way as when applying the procedures under Article 6 of that Appendix. In addition, the procedures stipulated in §§ 2 and 3 of the current Rules have already been accomplished and the examinations/checks noted in §§ 1 and 4 of the current Rules are included in relevant provisions of Article S11 of the RR. This means that, if the draft new Rules of Procedure mentioned in § 1 above are approved, the current Rules of Procedure relating to § 8.1 of Article 8 to Appendix S30B are no longer required.

3	In light of the above and in order to avoid any future misunderstanding by the administrations or the Bureau, it is proposed to delete the current Rules of Procedure relating to § 8.1 of Article 8 to Appendix S30B provided that the draft new Rules of Procedure mentioned in § 1 above are approved.

ANNEX 7

Clarifications on the scope of application of Annex 2B to Appendix S4 (WRC-2000) for submission of notices related to the coordination and notification of specific earth stations with associated space stations subject to Appendix S30A,
in application of Article S9 and S11 of the Radio Regulations

1.	In adopting the revised version of Annex 2 of Appendices S30 and S30A (WRC-2000), WRC-2000 decided to definitively include in Appendix S4 (WRC-2000) the data elements which were previously contained in Annex 2. These data elements are related to the basic characteristics to be furnished in notices relating to space stations in the Broadcasting-Satellite Service and to the associated feeder-link stations in the Fixed-Satellite Service.

2.	In reviewing Appendix S4 in the context of its use as of 3 June 2000 for the submission of space notices in application of Appendices S30, S30A and S30B, the Bureau has identified some discrepancies between Annexes 2A and 2B of Appendix S4 (WRC-2000 Final Acts) on the one hand, and former Annex 2 to Appendix S30 and S30A and Annex 2 to Appendix S30B on the other hand. These discrepancies were not considered at WRC-2000 and thus do not result from explicit decisions taken at this Conference.

3.	In order to correct the above-mentioned discrepancies, proposed alignments to those data items in Annexes 2A and 2B of Appendix S4 (WRC-2000), which were not discussed at 
WRC-2000, with those of Annex 2 to Appendix S30B were submitted for consideration by the RRB at its 22nd Meeting, Geneva, 05 - 09 February 2001. The RRB considered the matter and instructed the Bureau to publish a Circular Letter on the required amendments and refinements to Annexes 2A and 2B of Appendix S4 (WRC-2000). In accordance with this RRB decision, Circular Letter CR/158 dated 4 May 2001 was published by the Bureau.

4.	In revising Appendix S30A, WRC-2000 also adopted revised versions of Article 4 for the coordination and of Article 5 for notification, examination and recording in the MIFR of frequency assignments submitted under Appendix S30A. In doing so, WRC-2000 has excluded from the procedure of Article 4 of Appendix S30A the coordination of the transmitting feeder-link earth stations with respect to receiving earth stations operating in the opposite direction of transmission, as well as with respect to terrestrial stations. The coordination of specific transmitting feeder-link earth stations with associated receiving space stations subject to Appendix S30A, which is still necessary before these specific earth stations are notified under Article S11 of the Radio Regulations (RR), should now be undertaken by the notifying administration directly with the other concerned administrations with respect to terrestrial stations and receiving earth stations operating in the opposite direction of transmission, in accordance with the relevant/corresponding provisions of Article S9 of RR.

5.	In view of the above, the Bureau reviewed further the data elements in Appendix S4 in the context of its use as of 3 June 2000 for the submission of notices related to the coordination and notification of specific feeder-link earth stations, with associated space stations in the Region 2 Plan or the Regions 1 and 3 Plans or Lists of Appendix S30A, in application of Article S9 and S11 of the RR. The Bureau has then noted that some further clarifications are required to precisely define the mandatory data elements of Appendix S4, which shall be submitted in the above-mentioned notices.

6.	It is worth mentioning that the clarifications, which are proposed in Attachment 1 of Appendix 1 to this Annex, have no impact on the use of Appendix S4 for submissions of notices in application of other parts of the Radio Regulations. In addition, the modifications of Annex 2B to Appendix S4, which were published in Circular Letter CR/158 dated 4 May 2001 according to the decision taken at the 22nd RRB meeting (Geneva, 5-9 February 2001), as well as those which are proposed in Annex 6 to CCRR/11 dated 2 March 2001, are not all reproduced in Attachment 1 hereafter. However, they still have to be taken into account unless otherwise proposed in Attachment 1.

7.	The above-mentioned changes should be brought to the attention of the Special Committee on Regulatory and Procedural Matter (SC), CPM-02 and WRC-03 for a possible revision of Appendix S4 at WRC-03, as appropriate.

Appendix:	1

APPENDIX 1 TO ANNEX 7

Clarifications on the scope of application of Annex 2B to Appendix S4 (WRC-2000) for submission of notices related to the coordination and notification of specific earth stations with associated space stations subject to Appendix S30A,
in application of Article S9 and S11 of the Radio Regulations

Attachment 1 to this Appendix contains an extract of Annex 2B to Appendix S4 (WRC-2000).

The clarifications on the scope of application of Annex 2B to Appendix S4 (WRC-2000) with respect to the mandatory data elements for the submission of notices related to the coordination and notification of specific feeder-link earth stations, with associated space stations in the Region 2 Plan or in a Regions 1 and 3 Plan or List of Appendix S30A, in application of Article S9 and S11 of the Radio Regulations, as contained in this Appendix, is proposed with revision marks (i.e.: new text underlined or deleted text strike through) to the version of Annex 2B to Appendix S4 which will appear in the Radio Regulations after inclusion of the amendments contained in the WRC-2000 Final Acts.

It is worth mentioning that the modifications of Annex 2B to Appendix S4, which were published in Circular Letter CR/158 dated 4 May 2001 according to the decision taken at the 22nd RRB meeting (Geneva, 5-9 February 2001), as well as those which are proposed in Annex 6 to CCRR/11 dated 2 March 2001, are not all reproduced in Attachment 1 hereafter. However, they still have to be taken into account unless otherwise proposed in Attachment 1.

ATTACHMENT 1 TO APPENDIX 1 OF ANNEX 7

Proposed clarification on the scope of application of Annex 2B to Appendix S4 (WRC-2000) with respect to the mandatory data elements for the submission of notices related to the coordination and notification of specific earth stations 
with associated space stations subject to Appendix S30A, in application of Article S9 and S11 of the Radio Regulations

(Extract of the Table of characteristics to be submitted for space and radio astronomy services)

A – General characteristics of the satellite network or the earth station

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for a satellite network space stations in the broadcasting-satellite service under Appendix S30 | Notice for a satellite network (feeder-link) stations under Appendix S30A | Notice for a satellite network stations in the fixed-satellite service under Appendix S30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.7.a.1 |  |  |  |  |  | X11 |  | X |  | A.7.a.1 |  |
| A.7.b |  |  |  |  |  | X11 |  | X |  | A.7.b |  |
| A.7.d |  |  |  |  |  | X11 |  | X |  | A.7.d |  |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

11	Not required for coordination under No. S9.7A or S9.7B.

B  –  Characteristics to be provided for each satellite antenna beam and for each earth station antenna

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for a satellite network space stations in the broadcasting-satellite service under Appendix S30 | Notice for a satellite network (feeder-link) stations under Appendix S30A | Notice for a satellite network stations in the fixed-satellite service under Appendix S30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

C – Characteristics to be provided for each group of frequency assignments for a satellite antenna beam or an earth station antenna

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for a satellite network space stations in the broadcasting-satellite service under Appendix S30 | Notice for a satellite network (feeder-link) stations under Appendix S30A | Notice for a satellite network stations in the fixed-satellite service under Appendix S30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.8.b |  |  | X1, 7 | X7 | X7 | X11, 15 |  |  |  | C.8.b |  |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

1	Only the value of maximum power density is mandatory.

7	One or the other of C.8.a or C.8.b is mandatory, but not both.

11	Not required for coordination under No. S9.7A or S9.7B.

15	In case of a specific earth station with an associated space station subject to an Appendix S30A Plan or List, the values submitted under this item shall include the maximum amount of power control which could be used.

D  –  Overall link characteristics

| Items in Appendix | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article S9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article S9 | Notification or coordination of a geostationary-satellite network (including Appendix S30B) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station | Notice for a satellite network space stations in the broadcasting-satellite service under Appendix S30 | Notice for a satellite network (feeder-link) stations under Appendix S30A | Notice for a satellite network stations in the fixed-satellite service under Appendix S30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

X  Mandatory information	O  Optional information	C  This information need only be furnished when it has been used as a basis to effect coordination with another administration

_______________

5	 	Note by the Radiocommunication Bureau:  Due to non-availability of a method to apply the PDA concept, the computer software currently available for the Appendix S30B applications (MSPACEG) is limited to the method of Annex 4 of Appendix S30B to carry out compatibility calculations between networks at fixed orbital positions. Consequently the Radiocommunication Bureau is not in a position to apply the PDA concept.

6	 	Note by the Radiocommunication Bureau:  This is to provide to that administration provisional results based on the current Reference Situation files at that time. It is understood that the proposed allotment is not entered in Part A of the Plan at that time.

6	 	Note by the Radiocommunication Bureau:  This is to provide to that administration provisional results based on the current Reference Situation files at that time. It is understood that the proposed allotment is not entered in Part A of the Plan at that time.