INTERNATIONAL TELECOMMUNICATION UNION

Radiocommunication Bureau (Direct Fax N°. +41 22 730 57 85)

| Circular Letter CCRR/17 | 15 December 2001 |
| --- | --- |

To Administrations of Member States of the ITU

Subject:	Draft modifications to the Rules of Procedure

To the Director General

Dear Madam/Sir,

Following the conclusions reached by WRC-2000 (Istanbul, 2000), the Radiocommunication Bureau identified a number of cases, which would appear to require new Rules of Procedure or the modification or suppression of existing Rules of Procedure.

In this Circular Letter, please find enclosed the set of draft Rules of Procedure which are intended for consideration by the RRB at its 26th Meeting (11 – 15 March 2002). The proposals in this Circular Letter relate to the following provisions:

-	Draft new Rule of Procedure concerning Nos. S5.523B, S5.523C, S5.523D and S5.523E of the RR (Annex 1).

-	Rules of Procedure concerning No. S5.351 of the RR (Annex 2).

-	Rule of Procedure on No. S5.554 of the RR (Annex 3).

- Rule of Procedure on the Box 40.5-42.5 GHz of the Frequency Allocation Table (Annex 4).

- Rules of Procedure concerning Resolution 84 (WRC-2000) (Annex 5).

- Draft new Rules of Procedure relating to Resolution 49 (Rev. WRC-2000) with respect to its application for satellite networks submitted under Appendices S30, S30A and S30B of the RR (Annex 6).

- Rules of Procedure concerning Resolution 122 (Rev. WRC-2000) (Annex 7).

- Draft modification to the current Rules of Procedure relating to Nos. S5.487 and S5.490 (Annex 8).

- Draft new Rules of Procedure relating to § 4.1.3 and § 4.2.6 of Article 4 of Appendices S30 and S30A (WRC-2000) of the RR (Annex 9).

- Draft new Rules of Procedure relating to §§ 5.1.3, 5.1.4 and 5.1.5 of Article 5 of Appendix S30A (WRC-2000), and Articles Nos. S9.17, S9.17A and S11.34 of the RR (Annex 10).

- Draft modification to the current Rules of Procedure relating to former §§ 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 of Appendix S30 (WRC-97) (Annex 11).

- Draft modification to the current Rules of Procedure relating to former §§ 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 to Appendix S30A (WRC-97) (Annex 12).

- Draft modification to the current Rules of Procedure relating to former § 6.1 of Article 6, and former § 7.6 of Article 7 to Appendix S30A (WRC-97) (Annex 13).

- Modification of the Rule of Procedure concerning No. S9.21 (Annex 14).

- Modification of the Rule of Procedure concerning No. S9.36 (Annex 15).

- Draft new Rule of Procedure relating to Section 3.4 of Annex 5 of Appendix S30 (WRC-2000) (Annex 16).

- Proposed modification to the Rule of Procedure in respect of No. S11.49 (Annex 17).

- Draft new Rule of Procedure relating to Sections 5 and 6 of Annex 1 to Appendix S30A (WRC-2000) (Annex 18).

In accordance with Article S13.17 of the Radio Regulations, these proposals are made available to administrations for comment before being submitted to the RRB pursuant to Article S13.14.

To enable the Radiocommunication Bureau to prepare, translate and post on the ITU website the consolidated submission to the RRB in time for the 26th Meeting, which commences on 11 March 2002, any comments that you may wish to submit should reach the Bureau not later than 11 February 2002. All e-mail comments should be sent to: mailto:brmail@itu.int.

Yours faithfully,

Robert W. Jones
					Director, Radiocommunication Bureau

Annexes: 18

Distribution:

-	Administrations of Member States of the ITU
-	Members of the Radio Regulations Board
- 	Director and Heads of Department of the Radiocommunication Bureau

RNS/YH/at

ANNEX 1

Draft new Rule concerning Nos. S5.523B, S5.523C, S5.523D and 
S5.523E of the RR

Remarks by the Radiocommunication Bureau:

1	The Radio Regulations Board at its 24th meeting (Geneva, 10-18 September 2001) approved proposed modifications to the Rule of Procedure concerning No. S9.11A. During the discussions, the Board recognized however that the application of this rule for the particular frequency range 19.3-19.7 GHz would be facilitated if a table reflecting the interrelationship between the different services allocated in this frequency range was prepared for consideration by the Board at a next meeting.

2	In the light of these comments, the Bureau undertook an in-depth examination of the applicability of the different coordination procedure (space network-to-space network (S9.7, S9.12, S9.12A, S9.13), space network-to-terrestrial stations, earth station-to-terrestrial station (S9.15, S9.17, S9.17A, S9.18)) in the frequency range 19.3-19.7 GHz which has resulted in a Rule of Procedure on Nos. S5.523B, S5.523C, S5.523D and S5.523E that is proposed for your comments.

ADD

S5.523B, S5.523C, S5.523D, S5.523E

Footnotes S5.523B, S5.523C, S5.523D and S5.523E provide information on the different constraints and procedures applying to the fixed-satellite service (FSS) in the frequency range 19.3-19.7 GHz. The Board studied the interrelationship between the different FSS usages and also vis-à-vis the terrestrial stations. Tables 19.3-19.6 GHz and 19.6-19.7 GHz below contain the Board’s conclusions on this matter.

Table 19.3-19.6 GHz

| CR request: Column vis-à-vis Row() | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (other) | GSO FSS  (CR 18.11.95) | GSO FSS  (18.11.95 CR) | Terrestrial |
| --- | --- | --- | --- | --- | --- | --- |
| Non-GSO FSS  (MSS feederlink) | S9.12 (S5.523B) | S9.12 (S5.523B/S5.523D) | NO CR (S5.523D) | S22.2 (S5.523C) | S9.12A (S5.523B) | --- (S5.523B) |
| Earth station | --- | S9.17A | S9.17A | S9.17A | S9.17A | S9.15 |
| Non-GSO FSS  (MSS feederlink) | S9.12 (S5.523B/ S5.523D) | S9.12 (S5.523D) | NO CR (S5.523D) | S22.2 (S5.523D) | S9.12A (S5.523D) | --- PFD limits (S5.523D) |
| Earth station | S9.17A | --- | --- | --- | --- | S9.15 |
| Non-GSO FSS  (other) | NO CR (S5.523D) | NO CR (S5.523D) | NO CR (S5.523D) | S22.2 (S5.523D) | S22.2 (S5.523D) | --- PFD limits (S5.523D) |
| Earth station | S9.17A | --- | --- | --- | --- | S9.17 |
| GSO FSS  (CR 18.11.95) | S22.2 (S5.523C) | S22.2 (S5.523D) | S22.2 (S5.523D) | S9.7 | --- | --- PFD limits |
| Earth station | S9.17A | --- | --- | --- | --- | S9.17 |
| GSO FSS  (18.11.95 CR) | S9.13 (S5.523B) | S9.13 (S5.523D) | S22.2 (S5.523D) | S9.7 | S9.7 | --- PFD limits |
| Earth station | S9.17A | --- | --- | --- | --- | S9.17 |
| Terrestrial | --- | S9.16 (S5.523D) | S9.18 (S5.523D) | S9.18 | S9.18 | --- |

Table 19.6-19.7 GHz

| CR request: Column vis-à-vis Row () | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (other) | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (other) | GSO FSS  (CR 21.11.97) | GSO FSS  (CR 21.11.97) | GSO FSS  (21.11.97 CR) | GSO FSS  (21.11.97 CR) | Terrestrial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-GSO FSS  (MSS feederlink) | S9.12 | S9.12 | S9.12 | NO CR | S22.2 (S5.523E) | S22.2 (S5.523E) | S9.12A (S5.523D) | S9.12A (S5.523D) | (S5.523D) |
| Earth station | --- | --- | S9.17A | S9.17A | --- | S9.17A | --- | S9.17A | S9.15 |
| Non-GSO FSS  (other) | S9.12 | S9.12 | S9.12 | NO CR | S22.2 (S5.523E) | S22.2 (S5.523E) | S9.12A (S5.523D) | S9.12A (S5.523D) | (S5.523D) |
| Earth station | --- | --- | S9.17A | S9.17A | --- | S9.17A | --- | S9.17A | S9.15 |
| Non-GSO FSS  (MSS feederlink) | S9.12 | S9.12 | S9.12 (S5.523D) | NO CR (S5.523D) | S22.2 (S5.523E) | S22.2 (S5.523E) | S9.12A (S5.523D) | S9.12A (S5.523D) | PFD limits (S21/section V) (S5.523D) |
| Earth station | S9.17A | S9.17A--- | --- | --- | S9.17A | --- | S9.17A | --- | S9.15 |
| Non-GSO FSS  (other) | NO CR | NO CR | NO CR (S5.523D) | NO CR (S5.523D) | S22.2 (S5.523D) | S22.2 (S5.523D) | S22.2 (S5.523D) | S22.2 (S5.523D) | PFD limits (S21/section V) (S5.523D) |
| Earth station | S9.17A | S9.17A--- | --- | --- | S9.17A | --- | S9.17A | --- | S9.17 |
| GSO FSS  (CR 21.11.97) | S22.2 (S5.523E) | S22.2 (S5.523D) | S22.2 (S5.523E) | S22.2 (S5.523D) | S9.7 | S9.7 | --- | --- | --- |
| Earth station | --- | --- | S9.17A | S9.17A | --- | S9.17A | --- | S9.17A | S9.17 |
| GSO FSS  (CR 21.11.97) | S22.2 (S5.523E) | S22.2 (S5.523D) | S22.2 (S5.523E) | S22.2 (S5.523D) | S9.7 | S9.7 | --- | --- | PFD limits (S21/section V) |
| Earth station | S9.17A | S9.17A | --- | --- | S9.17A | --- | --- | --- | S9.17 |
| GSO FSS  (21.11.97 CR) | S9.13 (S5.523D) | S9.13 (S5.523D) | S9.13 (S5.523D) | S22.2 (S5.523D) | S9.7 | S9.7 | S9.7 | S9.7 | --- |
| Earth station | --- | --- | S9.17A | S9.17A | --- | S9.17A | --- | S9.17A | S9.17 |
| GSO FSS  (21.11.97 CR) | S9.13 (S5.523D) | S9.13 (S5.523D) | S9.13 (S5.523D) | S22.2 (S5.523D) | S9.7 | S9.7 | S9.7 | S9.7 | PFD limits (S21/section V) |
| Earth station | S9.17A | S9.17A | --- | --- | S9.17A | --- | S9.17A | --- | S9.17 |
| Terrestrial | --- | --- | S9.16 (S5.523D) | S9.18 (S5.523D) | --- | S9.18 | --- | S9.18 | --- |

ANNEX 2

Rules concerning

ARTICLE S5 OF THE RR

MOD

S5.351

1	This provision permits, in derogation of the definitions contained in No. S1.76 and S1.82, the use of the bands allocated to a mobile-satellite service by a station at a specified fixed point (without being a coast earth station or an aeronautical earth station).

2	The exceptional circumstances referred to in this provision cannot be evaluated by the Bureau.

3	The Board therefore concluded that assignments notified under this provision shall receive a favourable regulatory finding.

Reason:	To correct the errors in the references which were inadvertently introduced in the editorial process, consequently to the changes in the numbering scheme. The former Rule of Procedure on RR 726A (which was renumbered to S5.351) was making references to RR 71 (definition of coast earth station) and to RR 77 (definition of aeronautical earth station). The provision No. RR 71 was renumbered to S1.76 in the 1998 edition of the Radio Regulations, and provision No. RR 77 was renumbered to S1.82.

ANNEX 3

Rule of Procedure on No. S5.554

MOD

S5.554

This provision does not provide additional allocation for the fixed-satellite service in the frequency bands specified therein. It (exceptionally) authorizes links between land stations at specified fixed points within the mobile-satellite or radionavigation-satellite service. Land station in the context of the latter two services means land earth station, which, in accordance with its definition, is a feeder link earth station. Therefore, a space or earth station in the fixed-satellite service (class of station EC or TC) is not authorized in the frequency bands listed in S5.554 (except in the band 123-130 GHz where there is FSS allocation) and links between specific (as opposed to typical) feeder-link earth stations (e.g. class of station VA, or TI, or the like) are authorized within the mobile-satellite or radionavigation-satellite service.

Reason:	Footnotes S5.351 and S5.554 are not similar. S5.351 prohibits the use of feeder links where such use is normally authorized by definition, while S5.554 does not. S5.351 permits the use of the bands allocated to a mobile-satellite service by an earth station at a specified fixed point (without it being a feeder-link earth station) in place of a mobile earth station, while S5.554 authorizes links between feeder-link earth stations within the mobile-satellite or radionavigation-satellite service. Therefore, reference in this Rule to the Rule concerning S5.351 is not appropriate. The Rule now clarifies the classes of stations that are authorized under S5.554 within the mobile-satellite or radionavigation-satellite service.

ANNEX 4

ADD

Rule of Procedure
Box 40.5-42.5 GHz of the Frequency Allocation Table

1	Before WRC-97 there was no FSS allocation in the band 40.5-42.5 GHz and there was secondary allocation to the Fixed Service (FS).

2	WRC-97 made FSS allocation in that band for Regions 2 and 3 and, under S5.551D, to certain countries in Region 1 and upgraded the FS allocation to primary. The FS allocation went into effect on 1.1.1999. For FSS it went into effect on 1.1.1999 for Region 2, while for Regions 1 and 3 it was postponed until 1.1.2001, subject to the review of this date by WRC-2000 (see S5.551D, S5.551E and Resolution 134 (WRC-97)). Also, to protect the radio astronomy (RA) in the neighbouring band (42.5-43.5 GHz), the implementation of FSS systems in the band 41.5-42.5 GHz was prohibited until technical and operational measures had been identified and agreed within ITU-R (see S5.551B and Resolution 128 (WRC-97)).

3	The complement of the above measures was intended to protect FS and RA until appropriate measures have been agreed.

4	WRC-2000 adopted hard PFD limits for FSS in the band 40.5-42.5 GHz and made them applicable from 3.6.2000 (see S59.6 (WRC-2000) and Resolution 59 (WRC-2000), resolves 6) to protect FS. To protect RA stations in the band 42.5-43.5 GHz, WRC-2000 adopted S5.551G, which contains hard PFD limits in that band for FSS networks operating in the band 41.5-42.5 GHz, and made these limits applicable from 3.6.2000 (see S59.6 (WRC-2000) and resolves and urges administrations 2 of Resolution 128 (WRC-2000)).

5	Having put in place measures for protection of FS and RA, WRC-2000 suppressed S5.551B, S5.551D, S5.551E and Resolution 134 (WRC-97) and modified Resolution 128. Suppression of footnotes took effect on 1.1.2002, while the suppression of Resolution 134 and modification of Resolution 128 took effect immediately after WRC-2000, i.e. on 3.6.2000.

6	In view of the above, the Board considered the effective date of FSS allocation in the band 40.5-42.5 GHz and the applicability of hard PFD limits to FSS and concluded as follows:

| Region | Allocation effective | Application of hard PFD limits of S5.551G and Table S21-4 |
| --- | --- | --- |
| Region 2 | From 1.1.1999 | To FSS networks for which coordination or notification request is received as from 3.6.2000 |
| Region 3 | From 3.6.2000 |  |
| Countries of Region 1 listed in S5.551D (WRC-97) | From 3.6.2000 to 31.12.2001 |  |
| Region 1 | From 1.1.2002 |  |

Reason:	To clarify the use of the band 40.5-42.5 GHz by the FSS. The background is contained in the Rule itself.

SUP S5.551B, SUP S5.551E

Reason:	WRC-2000 suppressed Nos. S5.551B and S5.551E.

ANNEX 5

ADD

Rules concerning

RESOLUTION 84 (WRC-2000)

Power flux-density limits in the band 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service

resolves 2

The Board considered resolves 2 of Resolution 84 (WRC-2000) and, to assist administrations and the Bureau in applying it, decided to instruct the Bureau to act as follows with respect to GSO FSS networks using the band 37.5-40 GHz in any Region:

1)	At coordination stage (Article S9), for coordination requests submitted as from 3.6.2000, and for coordination requests submitted before 3.6.2000 but not followed by request for notification before that date, to establish and publish in a coordination special section the list of administrations in Region 2 on whose territory the power flux-density produced exceeds the values in Table S21-4 minus 12 dB;

2)	At notification stage (Article S11), for notification requests received as from 3.6.2000, to examine whether the required agreements (explicit), as established in 1) above, are properly reflected in the forms of notice and to establish the finding under S11.32 accordingly;

3)	For requests for assistance received from administrations in case of difficulties to obtain the required agreement from other administrations (identified as affected) to apply the corresponding/analogous provisions of Article S9.

Reason: 	The Board noted that, in the frequency band 37.5-40 GHz, resolves 2 of Resolution 84 (WRC-2000) provides, in the interim period before WRC-03, a mechanism for protection of terrestrial services in Region 2 from GSO FSS networks in Region 2. In view of the worldwide allocation of the band 37.5-40 GHz to GSO FSS, the Board considers that the intention of resolves 2 is to protect terrestrial services in Region 2 from GSO FSS use in any Region.

Also, the Board noted that there is no procedure associated with seeking, obtaining and verifying that the required agreements have indeed been obtained. To assist the administrations and the Bureau in applying resolves 2 of Resolution 84, the Board considers that it is appropriate to take an approach similar to the application of Resolution 77 as contained in the Rule of Procedure on S5.488.

ANNEX 6

Draft new Rules of Procedure relating to Resolution 49 (Rev.WRC-2000) 
with respect to its application for satellite networks submitted under 
Appendices S30, S30A and S30B of the RR

The following draft new Rules of Procedure relating to Resolution 49 (Rev.WRC-2000) are proposed in order to clarify the implementation of that Resolution for notices submitted under Appendices S30, S30A and S30B, taking into account the new provisions adopted by WRC-2000 with respect to Appendices S30 and S30A (including Resolution 533 (Rev.WRC-2000)), and the Rule of Procedure adopted at the 20th meeting of the Radio Regulations Board (RRB) (Geneva, 11-15 September 2000) relating to the use of Appendix S4 (WRC-2000) for submissions under Appendix S30B (see Section 6.3 of the Summary of Decisions, Document RRB2000/223).

RESOLUTION 49 (Rev.WRC-2000)

Administrative due diligence applicable to some satellite 
radiocommunication services

1	The Board considered the references to “Annex 2 of Appendix S30B/30B” in resolves 1 and in § 3 of Annex 1 of that Resolution. In accordance with the Rules of Procedure adopted on this issue at its 20th meeting (Geneva, 11-15 September 2000), the Board concluded that these references shall be understood as references to Appendix S4 of the Radio Regulations.

2	The Board considered the references to “relevant provisions of Article 4 of Appendix 30, Article 4 of Appendix 30A” in resolves 2 and to “relevant provisions of Article 4 of Appendix S30/30 and the relevant provisions of Article 4 of Appendix S30A/ 30A” in Section 5 of Annex 1 of that Resolution. Considering the adoption at WRC-2000 of new Article 4 of Appendices S30 and S30A (WRC-2000) and of Resolution 533 (Rev.WRC-2000), the Board concluded that these references shall be understood as references to § 4.1.3 (Regions 1 and 3) or § 4.2.6 (Region 2) of Article 4 of Appendices S30 and S30A (WRC-2000), and that the extension period referred to in resolves 3.2 or 4.2 of Resolution 533 (Rev.WRC-2000) shall be taken into account in the implementation of Resolution 49 (Rev.WRC-2000). See also the Rules of Procedure relating to § 4.1.3 and § 4.2.6 of Article 4 of Appendices S30 and S30A (WRC-2000).

ANNEX 7

Rules concerning

RESOLUTION 122 (Rev.WRC-2000)

# Use of the bands 47.2-47.5 GHz and 47.9-48.2 GHz by high altitude platform stations (HAPS) in the fixed service and by other services and the potential use of bands in the range 18-32 GHz by HAPS in the fixed service

Resolves and instructs

1	Resolves 2 of Resolution 122 (Rev.WRC-2000) stipulates that, on a provisional basis, the procedures of Article S9 shall be used for coordination between satellite systems and systems using HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz.

2	Having regard to the ongoing studies undertaken by the ITU-R study groups on appropriate criteria and calculation methods as well as the applicable Article S9 provisions involving satellite systems and HAPS (refer Resolution 122 (Rev.WRC-2000)), the Board concluded that, the Bureau shall proceed as follows:

2.1	To examine those assignments to satellite systems and HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz only with respect to No. S11.31 and to defer the examination of such systems under No. S11.32 as well as the application of any Article S9 coordination procedure between the satellite systems and HAPS until further decision by WRC-03. When the examination with respect to No. S11.31 leads to a favourable finding, the assignments shall be entered provisionally in the Master Register with the appropriate symbol in Column 13B3 (Finding Action);

2.2	To report to WRC-03 the list of involved assignments either at the coordination stage or recorded in the MIFR, as appropriate, for further consideration.

Reason:	In order to treat the cases in instructs 2 of Resolution 122, the requirement of coordination procedures of S9 has to be cleared for the coordination between satellite systems and systems using HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz. Recognizing that the study of ITU-R is in progress and WRC-03 in any case will review the notification process, provisional favourable finding without application of coordination procedures of S9 is proposed.

DRAFT

ANNEX 8

Draft modification to the current Rules of Procedure relating to 
Nos. S5.487 and S5.490

The following modifications to the current Rules of Procedure relating to Nos. S5.487 and S5.490 are proposed in order to align the corresponding texts with the modifications to No. S5.487 adopted at WRC-2000, thus avoiding any future misunderstanding in its application by the administrations or the Bureau.

MOD

S5.487

This provision stipulates that “services ... shall not cause harmful interference to, or claim protection from, ...”.  This would lead to a contradiction with Articles 4, 6 and 7 of Appendix S30, containing procedures which lead one to consider that the fixed-satellite, the fixed, the broadcasting and the mobile, except aeronautical mobile, services have equality of rights with the broadcasting-satellite service. The Board considers that in this case it should be deemed when applying Appendix S30 that the service concerned has equality of rights; however, if, despite the application of the procedures of Appendix S30, harmful interference is actually caused to a broadcasting-satellite station, the fixed, fixed-satellite, broadcasting or mobile, except aeronautical mobile, station shall cease this interference.

MOD

S5.490

Number S5.43 states that “… a service or stations in a service may operate ... subject to not causing harmful interference…”. This provision stipulates that “… services shall not cause harmful interference to ...”. Despite this difference in wording, the Board is of the view that No. S5.43 would apply in this case. This would lead to a contradiction with Articles 4 and 6 of Appendix S30, containing procedures which lead one to consider that the fixed, the broadcasting and the mobile, except aeronautical mobile, services have equality of rights with the broadcasting-satellite service. The Board considers that in this case it should be deemed when applying Appendix S30 that the service concerned has equality of rights; however, if, despite the application of the procedures of Appendix S30, harmful interference is actually caused to a broadcasting-satellite station, the fixed, broadcasting or mobile, except aeronautical mobile, station shall cease this interference.

ANNEX 9

Draft new Rules of Procedure relating to § 4.1.3 and § 4.2.6 of Article 4 of Appendices S30 and S30A (WRC-2000) of the RR

The following draft new Rules of Procedure relating to § 4.1.3 and § 4.2.6 of Article 4 of Appendices S30 and S30A (WRC-2000) of the RR are proposed in order to link the coordination time period defined in these paragraphs with the extension period referred to in resolves 3.2 or 4.2 of Resolution 533 (Rev.WRC-2000).

4.1.3 and 4.2.6

ADD

5	WRC-2000 included in § 4.1.3 of Article 4 of Appendices S30 and S30A a footnote on the fact that “the provisions of Resolution 533 (Rev.WRC-2000) apply”. Considering that resolves 3 or 4 of that Resolution apply to all Regions, the Board concluded that the extension periods referred to in resolves 3.2 or 4.2 of that Resolution are applicable under § 4.1.3 and § 4.2.6 of Article 4 of Appendices S30 and S30A, although the footnote mentioned above is not referred to in § 4.2.6 of Article 4 of Appendices S30 and S30A.

ANNEX 10

Draft new Rules of Procedure relating to §§ 5.1.3, 5.1.4 and 5.1.5 of 
Article 5 of Appendix S30A (WRC-2000), and Articles Nos. S9.17, 
S9.17A and S11.34 of the Radio Regulations (Notification of 
assignments in a Regions 1 and 3 feeder-link Plan or List)

1	At its 24th meeting (Geneva, 10-18 September 2001), the Radio Regulations Board (RRB) considered draft new Rules of Procedure relating to §§ 5.1.3, 5.1.4 and 5.1.5 of Article 5 of Appendix S30A (WRC-2000). The RRB agreed to postpone consideration of the draft new Rules of Procedure pending the consideration of draft Rules in respect of related provisions of Articles S9 and S11 (see item 6.18 of the Summary of Decisions of the 24th RRB meeting in Document RRB2001/280).

2	The Bureau reviewed the situation and, based on the discussions on this subject at the 24th RRB meeting, the Bureau developed draft new Rules of Procedure relating to Articles Nos. S9.17, S9.17A and S11.34 of the Radio Regulations (RR) which are provided below.

3	The Board is invited to consider these draft new Rules at its 26th meeting (Geneva, 11-15 March 2002) together with the draft new Rules of Procedure relating to §§ 5.1.3, 5.1.4 and 5.1.5 of Article 5 of Appendix S30A (WRC-2000), which were made available to administrations for comment in Annex 4 to CCRR/12 dated 3 July 2001 and which are not reproduced in the present Annex.

S9.17 and S9.17A

1	In revising the Appendix S30A feeder-link Plans for Regions 1 and 3, WRC-2000 has excluded from the procedure of Article 4 of that Appendix the coordination of the transmitting feeder-link earth stations with respect to receiving earth stations operating in the opposite direction of transmission, as well as with respect to terrestrial stations. This coordination should now be undertaken by the notifying administration directly with the other concerned administrations with respect to terrestrial stations and receiving earth stations operating in the opposite direction of transmission, in accordance with the relevant/corresponding provisions of Article S9 of the Radio Regulations (RR).

2	Taking into account the situation described in paragraph 1 above, the Board considered the following situations:

2.1	Notification of a specific feeder-link earth station under Article S11 with an associated space station/network in a WRC-2000 Regions 1 and 3 feeder-link Plan at 14 or 17 GHz, once this space station/network has been notified under Article 5 of Appendix S30A. In this case, the Board concluded that Notes 3 and 4 of Section 9A.2 to Article 9A of Appendix S30A (WRC-2000) shall be taken into account, if applicable pursuant to the implementation of Resolution 53 (Rev.WRC-2000), in the coordination of such a specific feeder-link earth station under Articles Nos. S9.17 and S9.17A of the RR. Otherwise, this specific feeder-link earth station shall not be coordinated under Articles Nos. S9.17 and S9.17A of the RR, provided that its characteristics are identical to those of the specific feeder-link earth station associated to the corresponding WRC-2000 Regions 1 and 3 feeder-link Plan assignment(s).

2.2	Notification of a specific feeder-link earth station under Article S11 with an associated space station/network in the RARC SAT-R2 Region 2 Plan (Geneva, 1983), once this space station/network has been notified under Article 5 of Appendix S30A. In this case, the Board concluded that such a specific feeder-link earth station shall not be coordinated under Articles Nos. S9.17 and S9.17A of the RR, provided that its characteristics are identical to those of the specific feeder-link earth station associated to the corresponding RARC SAT-R2 Region 2 Plan assignment(s).

2.3	Notification of a specific feeder-link earth station under Article S11 with an associated space station/network in the appropriate feeder-link Plan or List, which has being successfully coordinated under former Article 4 of Appendix S30A (WRC-97) with no restriction indicated for that space station/network in the relevant Part B Special Section on pending coordination(s) with respect to terrestrial station and/or receiving earth station operating in the opposite direction of transmission (i.e. former §§ 4.2.1.2, 4.2.1.3, 4.2.3.2 and 4.2.3.3 of Appendix S30A (WRC-97)). In this case, the Board concluded that such a specific feeder-link earth station shall not be coordinated under Articles Nos. S9.17 and S9.17A of the RR, provided that the coordination area of this specific feeder-link earth station is within the coordination areas successfully coordinated for the space station/network in question.

2.4	Notification of a specific feeder-link earth station under Article S11 with an associated space station/network in the appropriate feeder-link Plan or List, which has being successfully coordinated under former Article 4 of Appendix S30A (WRC-97) but with restrictions indicated for that space station/network in the relevant Part B Special Section on pending coordination(s) with respect to terrestrial station and/or receiving earth station operating in the opposite direction of transmission (i.e. former §§ 4.2.1.2, 4.2.1.3, 4.2.3.2 and 4.2.3.3 of Appendix S30A (WRC-97)). In this case, the Board concluded that the restrictions referred to above shall be taken into account in the coordination of such a specific feeder-link earth station under Articles Nos. S9.17 and S9.17A of the RR. Nevertheless, should the coordination area of this specific feeder-link earth station be beyond the coordination areas considered for the coordination under former Article 4 of Appendix S30A (WRC-97) of the associated space station/network in question, that specific feeder-link earth station would have to be coordinated under Articles Nos. S9.17 and S9.17A of the RR as in the other cases referred to in paragraph 2.5 below.

2.5	In other cases of notification of a specific feeder-link earth station under Article S11 with an associated space station/network in the appropriate feeder-link Plan or List, the Board concluded that Articles Nos. S9.17 and S9.17A of the RR apply with no restriction and no condition.

3	See also the Rules of Procedure relating to Article No. S11.34.

S11.34

# 3	Examination of the notification of a specific feeder-link earth station with an associated space station/network in the appropriate feeder-link Plan or List of Appendix S30A

3.1	In revising the Appendix S30A feeder-link Plans for Regions 1 and 3, WRC-2000 has included a footnote to the title of Article 5 of that Appendix in order to exclude from the scope of application of that Article the notification of specific feeder-link earth stations with associated space station/network in the appropriate feeder-link Plan or List of Appendix S30A (see the Rules of Procedure relating to the scope of application of that Article). Accordingly, the notification of these specific feeder-link earth stations shall be performed under Article S11 of the Radio Regulations (RR).

3.2	In view of the above, the Board concluded that a specific feeder-link earth station with an associated space station/network in the appropriate feeder-link Plan or List of Appendix S30A is considered to be in conformity with Article No. S11.34 of the RR when its location is within the service area of the associated space station/network and when this associated space station/network was notified with favourable findings under Article 5 of Appendix S30A.

3.3	In addition, for such a specific feeder-link earth station for which Articles Nos. S9.17 and/or S9.17A of the RR apply, the Board concluded that the conformity with the procedures relating to coordination with other administrations shall be checked in accordance with Article No. S11.32 of the RR (see the Rules of Procedure relating to Articles Nos. S9.17 and S9.17A of the RR). Otherwise, if Articles Nos. S9.17 and/or S9.17A of the RR do not apply, the Board concluded that Article No. S11.32 of the RR does not apply neither, and that in this case the specific feeder-link earth station in question is considered to be in conformity with Article No. S11.34 of the RR.

ANNEX 11

Draft modification to the current Rules of Procedure relating to former 
§§ 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 of Appendix S30 (WRC-97) to align its text with the decisions adopted at WRC-2000

The following modifications are proposed in order to align the text with the provisions contained in the new Article 5 of Appendix S30 (WRC-2000) taking into account the WRC-2000 decision to adopt the revised Regions 1 and 3 downlink Plan and the new Regions 1 and 3 downlink List of additional uses.

Art. 5

Notification, examination and recording

MOD

5.2.1 b)

1	The Board has considered the question whether the examination with respect to conformity with the Plan3 means only the columns of Articles 10 and 11 of Appendix S30, as updated or whether it also includes an examination with respect to the technical criteria given in Annex 5 to Appendix S30 which were used for the establishment of the Plans. The Board concluded that some of the technical criteria contained in Annex 5 need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the Plan is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the Plan concerned as updated (see § 3.1 of Article 3). If the characteristics are different then the examination under § 5.2.1 d) is carried out. For the items below, any characteristics for which the procedure of Article 4 has been successfully applied could be notified.

b)	to ensure that the protection criteria resulting from the Region 2 Plan, or from the Regions 1 and 3 Plan and List, as appropriate, are not exceeded. To this effect, the following characteristics are examined:

–	beam identification (as indicated in columns 1 and 2 of Articles 10 and 11 respectively of Appendix S30);

–	nominal orbital position (as indicated in columns 2 and 3 of Articles 10 and 11 respectively of Appendix S30);

–	channel number/frequency (as indicated in column 3 of Article 10 and in column 5 of the Table titled “Minimum equivalent protection margin of assignments in the Regions 1 and 3 Plan (sorted by orbital position)” of Article 11 of Appendix S30);

–	boresight coordinates (as indicated in column 4 of Articles 10 and 11 of Appendix S30);

–	in the case of an elliptical beam:

–	antenna beamwidth (as indicated in column 5 of Articles 10 and 11 of Appendix S30),

–	ellipse orientation (as indicated in columns 6 and 5 of Articles 10 and 11 respectively of Appendix S30),

–	antenna rotational accuracy (same as or better than that of § 3.14 of Annex 5 to Appendix S30);

–	polarization (as indicated in columns 7 and 10 of Articles 10 and 11 respectively of Appendix S30);

–	power plus co-polar antenna gain (as indicated in columns 8 and 11 of Articles 10 and 11 respectively of Appendix S30), and in the case of shaped beam the cross-polar antenna gain (as indicated in column 8 of Article 11 of Appendix S30);

–	service area (test points shall be located within the service area);

–	class of emission and bandwidth (as indicated in column 12 of Article 11 of Appendix S30 in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 and 3.8 of Annex 5 to Appendix S30);

–	antenna characteristics (same as or better than those indicated in columns 6 or 7 as appropriate of Article 11 of Appendix S30 in the case of the Regions 1 and 3 Plan, or otherwise same as or better than Fig. 9 or 10 as appropriate of Annex 5 to Appendix S30);

–	antenna pointing accuracy (same as or better than that referred to in § 3.14 of Annex 5 to Appendix S30);

–	station keeping tolerance (same as or better than that mentioned in § 3.11 of Annex 5 to Appendix S30);

–	modulation characteristics (same as in column 12 of Article 11 of Appendix S30 in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 of Annex 5 to Appendix S30);

–	energy dispersal (same as § 3.18 of Annex 5 to Appendix S30);

–	the power flux-density identified in Note 10 of the Region 2 Plan or in Note 5 of the Regions 1 and 3 Plan, to determine whether the limits are met or whether there is an agreement with the affected administrations.

2	The Board has considered the question whether the examination with respect to conformity with the Regions 1 and 3 List means only the columns of the Tables in Part I of Annex 2 to Resolution 542 (WRC-2000), as updated, or whether it also includes an examination with respect to the technical characteristics published by the Bureau for each network of the List in the corresponding Part B Special Section of the Weekly Circular or the IFIC. The Board concluded that all technical characteristics published in the Part B Special Section for a given network need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the List is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the List, as updated, and those specified in the Part B Special Section of a given network. If the characteristics are different then the examination under § 5.2.1 d) is carried out.

b)	to ensure that the protection criteria resulting from the Regions 1 and 3 Plan and List are not exceeded. To this effect, the characteristics specified in the columns of the List, as updated, and those specified in the Part B Special Section of a given network are examined.

MOD

5.2.2.1

This paragraph implicitly relates to the cases where the Bureau reaches a favourable Finding with respect to § 5.2.1 a) and § 5.2.1 c) and an unfavourable Finding with respect to § 5.2.1 b) but a favourable Finding with respect to § 5.2.1 d). In this event, the frequency assignment shall be recorded in the Master Register.

MOD

5.2.2.2

Part of this paragraph deals with interim systems which are submitted in application of Resolution 42 (Rev.Orb-88) for Region 2.

In case of Regions 1 and 3, should the Bureau reach a favourable Finding with respect to §§ 5.2.1 a) and 5.2.1 c) but an unfavourable Finding with respect to §§ 5.2.1 b) and 5.2.1 d), the assignments in question shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this Finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

MOD

5.3.1

1	See item 2 of the Rules of Procedure relating to § 4.2.6 of Article 4 of Appendix S30.

2	For any notification other than that relating to the modification to the Region 2 Plan, the date of bringing into use of assignments can be extended at the request of the notifying administration.

Nevertheless, the total period for the implementation (bringing into use) of the assignments shall be limited to 8 years, plus any applicable extension in accordance with resolves 3.2 or 4.2 of Resolution 533 (Rev.WRC-2000), counted from the date of receipt of the complete information of Appendix S4 or former Annex 2 to Appendix S30.

See also the Rules of Procedure relating to §§ 4.1.3 and 4.2.6 of Article 4 of Appendix S30.

ANNEX 12

Draft modification to the current Rules of Procedure relating to former §§ 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 to Appendix S30A (WRC-97) to align its text with the decisions adopted at WRC-2000

The following modifications are proposed in order to align the text with the provisions contained in the new Article 5 of Appendix S30A (WRC-2000) taking into account the WRC-2000 decision to adopt the revised Regions 1 and 3 feeder-link Plans and the new Regions 1 and 3 feeder-link Lists of additional uses.

Art. 5

Notification, examination and recording

MOD

5.2.1 b)

1	The Board has considered the question whether the examination with respect to conformity with the Plan2 means only the columns of Articles 9 and 9A of Appendix S30A, as updated or whether it also includes an examination with respect to the technical criteria given in Annex 3 to Appendix S30A which were used for the establishment of the Plans. The Board concluded that some of the technical criteria contained in Annex 3 need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the Plan is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the Plan concerned as updated (see § 3.1 of Article 3). If the characteristics are different then the examination under § 5.2.1 d) is carried out. For the items below, any characteristics for which the procedure of Article 4 has been successfully applied could be notified.

b)	to ensure that the protection criteria resulting from the Region 2 Plan, or from the Regions 1 and 3 Plan(s) and List(s), as appropriate, are not exceeded. To this effect, the following characteristics of the receiving space station are examined:

–	space station beam identification (as indicated in columns 1 and 2 of Articles 9 and 9A respectively of Appendix S30A);

–	nominal orbital position (as indicated in columns 2 and 3 of Articles 9 and 9A respectively of Appendix S30A);

–	channel number/frequency (as indicated in column 3 of Article 9 and in column 5 of the Tables titled “Minimum equivalent protection margin in the Regions 1 and 3 feeder-link Plan in the frequency band 14.5-14.8 GHz (sorted by orbital position)” and “Minimum equivalent protection margin in the Regions 1 and 3 feeder-link Plan in the frequency band 17.3-18.1 GHz (sorted by orbital position)” of Article 9A of Appendix S30A);

–	boresight coordinates (as indicated in column 4 Articles 9 and 9A of Appendix S30A);

–	in the case of elliptical beam:

–	antenna beamwidth (as indicated in column 5 of Articles 9 and 9A of Appendix S30A);

–	ellipse orientation (as indicated in columns 6 and 5 of Articles 9 and 9A respectively of Appendix S30A);

–	antenna rotational accuracy (same as or better than § 3.7.4 (Regions 1 and 3) or 4.6.4 (Region 2) of Annex 3 to Appendix S30A);

–	polarization (as indicated in columns 7 and 10 of Articles 9 and 9A respectively of Appendix S30A);

–	service area (test points shall be located within the service area);

–	class of emission and bandwidth (as indicated in column 13 of Article 9A in the case of Regions 1 and 3 Plan of Appendix S30A, or otherwise as indicated in § 3.1 and 3.8 of Annex 5 to Appendix S30);

–	antenna characteristics (same as or better than those indicated in columns 6 or 7 as appropriate of Article 9A of Appendix S30A in the case of Regions 1 and 3 Plan, or otherwise same as or better than § 4.6 of Annex 3 to Appendix S30A);

–	antenna pointing accuracy (same as or better than § 3.7.4 (Regions 1 and 3) or § 4.6.4 (Region 2) of Annex 3 to Appendix S30A);

–	system noise temperature (see Note 7 of Article 9A and § 3.8 of Annex 3 to Appendix S30A as appropriate for Regions 1 and 3, and § 4.7 of the same Annex for Region 2);

–	station keeping tolerance (same as or better than that of § 3.16 of Annex 3 to Appendix S30A);

–	modulation characteristics (same as in column 13 of Article 9A of Appendix S30A in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 of Annex 5 to Appendix S30);

–	range of automatic gain control (same as § 3.10 of Annex 3 to Appendix S30A for Regions 1 and 3, and 4.9 of the same Annex for Region 2).

–	e.i.r.p.: Columns 8 and 11 of Articles 9 and 9A respectively of Appendix S30A;

–	antenna diameter: § 3.5.1 or 4.4.1 of Annex 3 to Appendix S30A;

–	reference patterns: Fig. 6 or Fig. A of Annex 3 to Appendix S30A (as indicated in Column 9 of Article 9A of Appendix S30A for the Regions 1 and 3 Plan);

–	transmit power: § 3.6 or 4.5 of Annex 3 to Appendix S30A;

–	the location of the associated earth station to be associated with test points within the service area,

–	energy dispersal (same as § 3.18 of Annex 5 to Appendix S30).

In relation to the transmitting power, the Board noted that according to §§ 3.11 and 4.10 of Annex 3 to Appendix S30A, the use of power control shall remain within the limits indicated in those paragraphs.

2	The Board has considered the question whether the examination with respect to conformity with the Regions 1 and 3 Lists means only the columns of the Tables in Part II of Annex 2 to Resolution 542 (WRC-2000), as updated, or whether it also includes an examination with respect to the technical characteristics published by the Bureau for each network of the Lists in the corresponding Part B Special Section of the Weekly Circular or the IFIC. The Board concluded that all technical characteristics published in the Part B Special Section for a given network need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the Lists is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the List concerned, as updated, and those specified in the Part B Special Section of a given network. If the characteristics are different then the examination under § 5.2.1 d) is carried out.

b)	to ensure that the protection criteria resulting from the Regions 1 and 3 Plan and List concerned are not exceeded. To this effect, the characteristics specified in the columns of the List concerned, as updated, and those specified in the Part B Special Section of a given network are examined.

3	See also the Rules of Procedure relating to the scope of application of Article 5 of Appendix S30A [and to § 5.1.3 of that Article].

MOD

5.2.2.1

This paragraph implicitly relates to the cases where the Bureau reaches a favourable finding with respect to § 5.2.1 a), § 5.2.1 c) and § 5.2.1 f) and an unfavourable finding with respect to § 5.2.1 b) but a favourable finding with respect to § 5.2.1 d).

However, considering the Rules of Procedure relating to the scope of application of Article 5 of Appendix S30A [and to § 5.1.3 of that Article], the Board concluded that § 5.2.2.1 relates to the cases where the Bureau reaches a favourable finding with respect to § 5.2.1 a) and § 5.2.1 c) and an unfavourable finding with respect to § 5.2.1 b) but a favourable finding with respect to § 5.2.1 d).

In this event the frequency assignment shall be recorded in the Master Register.

MOD

5.2.2.2

Part of this paragraph deals with interim systems which are submitted in application of Resolution 42 (Rev.Orb-88) for Region 2.

In case of Regions 1 and 3, should the Bureau reach a favourable finding with respect to § 5.2.1 a) and § 5.2.1 c) but an unfavourable finding with respect to § 5.2.1 b) and § 5.2.1 d), the assignments in question shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

MOD

5.3.1

1	See § 2 of the Rules of Procedure relating to § 4.2.6 of Article 4 of Appendix S30A.

2	For any notification other than that relating to the modification to the Region 2 Plan, the date of bringing into use of assignments can be extended at the request of the notifying administration.

Nevertheless, the total period for the implementation (bringing into use) of the assignments shall be limited to 8 years, plus any applicable extension in accordance with resolves 3.2 or 4.2 of Resolution 533 (Rev.WRC-2000), counted from the date of receipt of the complete information of Appendix S4 or former Annex 2 to Appendix S30A.

See also the Rules of Procedure relating to §§ 4.1.3 and 4.2.6 of Article 4 of Appendix S30A.

ANNEX 13

Draft modification to the current Rules of Procedure relating to
former § 6.1 of Article 6, and former § 7.6 of Article 7 to 
Appendix S30A (WRC-97)

The following modifications are proposed in order to align the text with the provisions contained in the new Articles 6 and 7 of Appendix S30A (WRC-2000) taking into account of the WRC-2000 decisions to revise Appendix S7 and Article 4 of Appendix S30A of the Radio Regulations and to delete former Resolution 519 (Orb-88).

Art. 6

Coordination, notification and recording of receiving terrestrial
assignments when FSS feeder-links are involved

MOD

6.1

1	The paragraphs of Article 6 do not mention interim systems implemented in accordance with Resolution 42 (Rev.Orb-88). Such systems may be implemented in the frequency band 17.7-17.8 GHz for Region 2 shared with equal rights with terrestrial services.

Such usage may affect terrestrial stations.

2	This paragraph refers to “a feeder-link earth station located on the territory of another administration and included in the service area of an assignment to a broadcasting-satellite service feeder-link space station which is in conformity with the appropriate regional feeder-link Plan”. This earth station is to be considered a typical earth station located at the worst location.

3	In order to evaluate the interference, an Administration A, intending to use terrestrial stations, needs to know the fixed-earth station existing or planned. In order to take them into account administrations may calculate the coordination area as indicated in § 1.4.6 of Appendix S7 around the service area mentioned in § 6.1.

Art. 7

Coordination, notification and recording of FSS assignments
when feeder-links to BSS assignments are involved

MOD

7.7

The comments under § 6.5 apply.

ANNEX 14

Modification of the Rule concerning No. S9.21

MOD

S9.21

# 1	(NOC)

# 2	(Suppress paragraph 2)

Reason:	WRC-97 amended § 2 of Appendix S5 with respect to its version of WRC-95 so that it now reads: “For application of No. S9.21, the agreement of an administration may be required with respect to the frequency assignments … which may affect or be affected, as appropriate …” (underlining added). For the establishment of potentially affected administrations the Bureau will thus, like in other forms of coordination, take into account both directions of potential interference: from the assignment under No. S9.21 to other assignments and vice versa.

# 3	(NOC)

# 4	(NOC)

(Renumber existing paragraphs 3 and 4 to paragraphs 2 and 3)

ANNEX 15

Modification of the Rule concerning No. S9.36

MOD

S9.36

1	Under this provision, the Bureau “shall identify any administrations with which coordination may need to be effected”. In applying Appendix S5 with respect to No. S9.21, the Bureau uses the following calculation methods and criteria3:

–	space network vs. space network: Appendix S8;

–	earth station vs. terrestrial stations and vice versa: Appendix S7;

–	transmitting terrestrial stations vs. receiving space stations: criteria of Article S21;

–	transmitting space stations vs. terrestrial services: pfd limits defined in Article S21 (where such limits are not applicable as hard limits to the service which is subject to No. S9.21), or coordination threshold pfd values applicable to other services in the same frequency band (e.g. pfd values of Annex 1 to Appendix S5);

–	receiving space stations vs. transmitting terrestrial stations: frequency overlap;

Reason:	To use, for the purpose of identifying administrations with which coordination may need to be effected, technical criteria available in the frequency band in which No. S9.21 applies to a service indicated in a footnote referring to No. S9.21.

–	between stations of terrestrial services in some specific frequency bands: Rules of Procedure B4, B5 and B6 as appropriate.

2	(NOC)

annex 16

Draft new Rule of Procedure relating to Section 3.4 of Annex 5 
of Appendix S30 (WRC-2000)

1	The following new Rule of Procedure relating to Section 3.4 of Annex 5 to Appendix 30 (WRC-2000) is proposed in order to include protection ratios for Region 2, which were inadvertently omitted when WRC-2000 reviewed this Section.

2	The Board is invited to consider this draft new Rule at its 26th meeting (Geneva, 11-15 March 2002). This omission should be brought to the attention of the Special Committee on Regulatory and Procedural Matters (SCRPM), CPM-02 and WRC-03 for a possible revision of Section 3.4 of Annex 5 to Appendix 30 at WRC-03.

Annex 5

Technical data used in establishing the provisions and associated Plans and the Regions 1 and 3 List, which should be used for their application

3.4

1	The Board notes that the WRC-2000 Final Acts include a modification to Section 3.4 of Annex 5 to Appendix 30 (WRC-2000). However, that modification omits the last three paragraphs from the previous Section 3.4, apparently in error. These three paragraphs specify the protection ratios for Region 2, which are not mentioned elsewhere in that Appendix.

2	Accordingly, until the matter can be addressed by a future WRC, the Board agrees that the following protection ratios, previously prescribed, should continue to apply as follows:

“In Region 2, the following protection ratios have been adopted for the purpose of calculating the overall equivalent protection margin32bis:

28 dB for co-channel signals;

13.6 dB for adjacent-channel signals;

–9.9 dB for second adjacent-channel signals.

In Region 2, as a guide for planning, the reduction in the overall C/I ratio due to co-channel interference in the feeder link is taken as equivalent to a degradation in the downlink co-channel C/I ratio of approximately 0.5 dB not exceeded for 99% of the worst month, but the feeder-link and downlink Plans are evaluated on the basis of the overall equivalent protection margin, which includes the combined downlink and feeder-link contributions.

In Region 2, an overall equivalent protection margin of 0 dB, or greater, indicates that the individual protection ratios have been met for the co-channel, the adjacent channels and the second adjacent channels.”

annex 17

Proposed modification to the Rule of Procedure in respect of No. S11.49

MOD

S11.49

# 1	Suspended assignments

1.1	Under the provisions of No. S11.49 the Board understands that an administration may suspend the use of a frequency assignment to a space station for a period not exceeding two years and still continue to enjoy the protection acquired by virtue of the coordination agreements already obtained. The Bureau may be informed of such suspensions either by the administration at its own initiative (No. S11.49) or in response to an inquiry made under No. S13.6*.

1.2	Although No. S11.49 only mentions the use of recorded assignments to a space station the Board considers in a similar manner that the suspension of a recorded frequency assignment to a space station implies that the operation of recorded assignments to associated earth station(s) will also be suspended.

1.3	The Board noted also that the suspension of a recorded assignment under No. S11.49 may apply more than once during the lifetime of this assignment, provided that the assignment is brought back into regular use prior to the expiry of the period of two years on each occasion.

1.4	The Board decided that the procedure described below shall apply. The procedure will only be valid for suspended assignments which are not modified before being brought back into use.

# 2	Recording of a suspension of use

2.1	When the Bureau is informed, either under No. S11.49 or in response to an inquiry under No. S13.6, that the use of a frequency assignment to a space station or earth station recorded in the Master Register is suspended, this information is published in Part II of the BR IFIC (in order to inform all administrations) and the entry in the MIFR will be amended to include the date D of resumption of use indicated by the notifying administration.

2.2	Frequency assignments to space stations or earth stations whose suspension is notified for a period of not more than 2 years will continue to be taken into account for the purposes of the examination of other assignments in accordance with Nos. S9.36, S11.31.1, S11.32, S11.32A and S11.33 until the time that the consultation concerning their resumption of use is completed (see 2.4 below).

2.3	Frequency assignments to space and earth stations whose suspension is notified for a period of more than 2 years will not be taken into account for the purpose of examinations of other assignments under Nos. S9.36, S11.31.1, S11.32-S11.32A and S11.33 as of the date of such notification or after confirmation from the administration of the period of suspension exceeding 2 years and shall be cancelled according to Nos. S11.49 or S13.6.

2.4	At the expiry of the period of suspension of the use of a frequency, the notifying administration is consulted as to the date of resumption of use. According to the results of the consultation, the Bureau will apply the following procedures:

2.4.1	When the administration confirms that the use has been resumed at the originally indicated date (not later than two years after the date of suspension) or before, this information is published in Part II of the BR IFIC and the MIFR is modified to indicate this situation.

2.4.2	When the administration notifies that the use will be resumed at a date later than two years after the date of suspension, these assignment shall be cancelled according to the provisions Nos. S11.49 and S13.6. For those stations for which the use may be resumed later than the 2-year period, the administration responsible for the assignment shall apply again the relevant coordination procedure of Article S9.

annex 18

Draft new Rules of Procedure relating to Sections 5 and 6 of Annex 1 to Appendix S30A (WRC-2000)

1	The following draft new Rules of Procedure relating to Sections 5 and 6 of Annex 1 to Appendix 30A (WRC-2000) are proposed in order to clarify the implementation of these Sections thus avoiding any future misunderstanding by the administrations or the Bureau. In addition, it is proposed to align the title of the current Rules of Procedure relating to Annex 1 of Appendix 30A with the provisions contained in the new Articles and Annexes of Appendix 30A (WRC-2000).

2	The Board is invited to consider these draft new Rules at its 26th meeting (Geneva, 11-15 March 2002). These changes should be brought to the attention of Special Committee on Regulatory and Procedural Matter (SC), CPM-02 and WRC-03 for a possible revision of Annex 1 to Appendix 30A at WRC-03, as appropriate.

An. 1

Limits for determining whether a service of an administration is affected by proposed modifications to the Region 2 Plan or by proposed new or modified assignments to the Regions 1 and 3 feeder-link Lists

5

The Board considered the reference to “total RF bandwidth of the feeder-link carriers (24 MHz for Region 2 and 27 MHz for Regions 1 and 3)” contained in the last two lines of the first paragraph of Section 5 of Annex 1 to Appendix 30A (WRC-2000). Considering that many proposed modifications to the Region 2 Plan or proposed new or modified assignments to the Regions 1 and 3 feeder-link Lists include feeder-link carriers with necessary bandwidth different from 24 MHz for Region 2 or 27 MHz for Regions 1 and 3, the Board concluded that the reference to “(24 MHz for Region 2 and 27 MHz for Regions 1 and 3)” shall be disregarded by the Bureau in the implementation of Section 5 of Annex 1 to Appendix 30A (WRC-2000). In addition, the Board concluded that the same reference contained in data item No. C.8.i of Annex 2A to Appendix S4 shall also be disregarded. The Board further noted that the maximum power density per hertz averaged over the total RF bandwidth corresponds generally to the maximum power density per hertz averaged over the necessary bandwidth of the proposed feeder-link carrier.

6

1	The Board, in reviewing the implementation of the protection criteria and methods of Appendix 30A (WRC-2000), noted that the text of Section 6 of Annex 1 to Appendix 30A (WRC-2000) requires further clarification or complementary information with regard to the implementation of the corresponding protection criteria and method for sharing between the services involved.

2	Section 6 of Annex 1 to Appendix 30A (WRC-2000) contains the criteria applicable to protect Region 2 receiving feeder-link space stations in the band 17.8-18.1 GHz not subject to a Plan from proposed new or modified assignments in the 17 GHz Regions 1 and 3 feeder-link List.

3	In order to clarify the implementation of this Section thus avoiding any future misunderstanding by the administrations or the Bureau, the Board, in reviewing the subject matter in conjunction with the practices currently used by the Bureau, decided to instruct the Bureau to apply the following course of action:

3.1	When the service area of the proposed new or modified assignment in the 17 GHz Regions 1 and 3 feeder-link List is defined by a contour, the Bureau will apply the same methodology as the one used in the compatibility analysis between FSS systems, i.e.:

An administration in Region 2 is identified among those whose agreement is required when, under assumed free-space propagation conditions, the power flux-density at the Region 2 receiving feeder-link space station arriving from any portion of the service area, including the geographical location(s) of the associated specific feeder-link earth station(s) if any, of the proposed new or modified assignment in the Regions 1 and 3 feeder-link List exceeds the T/T limits prescribed in Section 6 of Annex 1 to Appendix 30A (WRC-2000).

3.2	In the absence of available service area contour of the proposed new or modified assignment in the 17 GHz Regions 1 and 3 feeder-link List, the methodology described in 3.1 above will be applied but instead of verifying the T/T compliance of the power flux-density arriving from any portion of the service area, the examination will consider each of the feeder-link test-points associated with the service area, including the geographical location(s) of the associated specific feeder-link earth station(s) if any, of the corresponding assignment.

______________

3	 	Any time the “Plan” is referred to, this means the current version of the Plan as updated on the date of Bureau’s examination in the case of the Region 2 Plan, and, in the case of the Regions 1 and 3 Plan, the current version of the Plan as may be updated pursuant to the possible application of § 4.1.26 or § 4.1.27 of Article 4 of Appendix S30.

2	 	Any time the “Plan” is referred to, this means the current version of the Plan as updated on the date of Bureau’s examination in the case of the Region 2 Plan, and, in the case of the Regions 1 and 3 Plan(s), the current version of the Plan(s) as may be updated pursuant to the possible application of § 4.1.26 or § 4.1.27 of Article 4 of Appendix S30A.

3	 	For cases not covered under this paragraph, the Bureau, in collaboration with the appropriate Radiocommunication Study Groups, continue to develop applicable calculation methods and criteria in the form of Rules of Procedure to be submitted to the RRB for approval.

3	2bis 	The definitions in §§ 1.7, 1.8, 1.9, 1.10 and 1.11 of this Annex apply to these calculations.

*	In lieu of an incorrect reference in S11.49 to No. S13.13.