INTERNATIONAL TELECOMMUNICATION UNION

Radiocommunication Bureau (Direct Fax N°. +41 22 730 57 85)

| Circular Letter CCRR/23 | 19 December 2003 |
| --- | --- |

To Administrations of Member States of ITU

Subject:	Draft Rules of Procedure

To the Director General

Dear Madam/Sir,

Please find enclosed proposals for suppression or modification of some current Rules of Procedure as well as for the approval of new Rules of Procedure. The majority of these proposals are related to decisions of WRC-03. These proposals are presented in 3 Annexes as follows:

Annex 1:	a)	New Rule of Procedure relating to No. 5.446A of Article 5 of the Radio Regulations;

Annex 2:	a)	Modification of the Rules of Procedure relating to Nos. 5.416 and 5.418C and suppression of the Rule of Procedure relating to No. 5.418 of Article 5 of the Radio Regulations;

b)	Modification of the Rule of Procedure relating to No. 9.11A of Article 9 of the Radio Regulations;

c)	Modification of the Rule of Procedure relating to No. 5.329 of Article 5 of the Radio Regulations;

d)	Modification of the Rule of Procedure relating to No. 9.36 and suppression of the Rule of Procedure relating to No. 9.42 of Article 9 of the Radio Regulations;

Annex 3:	a)	Modification of the Rule of Procedure relating to No. 5.492 of the Radio Regulations;

b)	Suppression of the Rules of Procedure relating to § 5.3.1 of Appendices 30 and 30A and the scope of application of Article 5 of Appendix 30A of the Radio Regulations;

c)	Modification of the Rules of Procedure relating to § 4.1.1 e), 4.1.3, 4.2.3 c), 4.2.3 e), 4.2.3 f), 4.2.11, 5.2.1 d), 5.2.2.2 and Annex 4 of Appendix 30 of the Radio Regulations;

d)	Modification of the Rules of Procedure relating to § 4.1.3, 4.2.2 c), 4.2.11, 5.2.1 d), 5.2.2.2 and 6.1 of Appendix 30A of the Radio Regulations;

e)	Modification of the Rules of Procedure relating to § 6.24, 6.43, 6.56 and Annex 2 of Appendix 30B of the Radio Regulations, and

f)	New Rule of Procedure relating to § 4.1.7bis of Appendices 30 and 30A of the Radio Regulations.

In accordance with No. 13.17 of the Radio Regulations, these proposals are made available to administrations for comment before being submitted to the RRB pursuant to No. 13.14.

To enable the Radiocommunication Bureau to prepare, translate and post on the ITU website the consolidated submission to the RRB in time for the 33rd Meeting, scheduled for 15-19 March 2004, any comments that you may wish to submit should reach the Bureau no later than 13 February 2004. All e-mail comments should be sent to: brmail@itu.int.

In this connection, the Bureau wishes to inform you that the RRB, at its 32nd meeting (1-5 December 2003) approved modified Rules concerning its working methods (Part C of the Rules of Procedure), which are now aligned to the amendments to the Constitution and Convention as adopted by the Plenipotentiary Conference (Marrakesh, 2002). These Rules will enter into force on 1 January 2004. Pending the standard distribution of the modified Rules of procedure that were approved by the RRB at its 32nd meeting, the Bureau considers that it would be beneficial for all administrations if they are already informed on those modifications to Part C of the Rules of procedure which deal with establishment or revision of Rules of Procedure, given the fact that the relevant approval mechanism will apply to the draft Rules of Procedure that are included in this Circular letter. Your attention is therefore drawn to paragraph 2.1.1 of the new Rules of Procedure in Part C, which stipulate the following:

“2.1.1	In the preparation and development of Rules of Procedure the following steps shall be applied:

a)	any practice used by the Bureau in the application of the provisions of the Radio Regulations shall be identified and proposed for inclusion in the Rules of Procedure;

b)	preparation, by BR, of a draft Rule of Procedure (RoP);

c)	the draft RoP prepared by BR shall be made available to the administrations for comments in a circular letter and on the RRB home page of the ITU website at least ten weeks before the meeting;

d)	any comments on these draft Rules of Procedure from administrations shall be submitted to the Bureau at least four weeks before the meeting;

e)	posting on the RRB home page of the ITU website and collection of the comments of administrations;

f)	consideration of the draft RoP and related comments, received within the time-limit, by the Board for decision. Comments submitted by administrations to draft Rules of Procedures that have not been received within the time limit shall not be examined by the Board (cf. 13.12bis(f)). However, until the 1st of January 2005, any such late submissions will be put on the agenda of the next Board meeting;

g)	publication of the approved RoP in a circular letter and in electronic form.”

Yours faithfully,

V. Timofeev
	Director, Radiocommunication Bureau

Annexes: 3

Distribution:

-	Administrations of Member States of the ITU

-	Members of the Radio Regulations Board

- 	Director and Heads of Department of the Radiocommunication Bureau

ANNEX 1

# Rules concerning

## ARTICLE 5 of the RR

ADD

5.446A

1)	This provision stipulates that the use of the bands 5 150-5 350 MHz and 5 470-5 725 MHz by the stations in the mobile service shall be in accordance with Resolution 229 (WRC-03). Accordingly, Resolution 229 (WRC-03) specifies that the use of these bands, by the mobile service, will be for the implementation of wireless access systems (WAS) including radio local area networks (RLAN) (see resolves 1) and, in addition to this, it specifies the maximum e.i.r.p. levels for stations in the mobile service (see resolves 2, 4 and 6).

As far as the band 5 150-5 350 MHz is concerned, the situation is rather simple, given the fact that the provisions of Resolution 229 (WRC-03) are applicable to all stations in the mobile service, except the cases referred to in No. 5.447, which apply to the band 5 150-5 250 MHz and where other (e.g. less stringent) conditions may be established in the context of the application of the procedure of RR 9.21.

On the other hand, the situation in the band 5 470-5 725 MHz is more complex, bearing in mind that other provisions are applicable to stations in the mobile service (e.g. those indicated in Nos. 5.451, 5.453 and in Table 21-2 of Article 21), which are stipulating different conditions (e.g. power limits) than the ones indicated in Resolution 229 (WRC-03). Consequently, administrations referred to in No. 5.453 (for the band 5 650-5 725 MHz) and in No. 5.451 (for the band 5 470-5 725 MHz) may implement other applications in the mobile service, which are not necessarily WAS, subject to compliance with the power limits set forth in No. 5.451 and in Table 21-2 of Article 21.

2)	Given the fact that, for the implementation of WAS, high deployment densities are expected, such implementation options could be adequately covered through notifications in the form of typical stations. The notification of terrestrial stations in the form of typical stations is normally possible with no restrictions in the bands 5 150-5 250 MHz and 5 470-5 670 MHz. However, provision No. 11.21A, in conjunction with Table 21-2, does not provide for the possibility of notifying terrestrial stations in the mobile service, in the form of typical stations, for the band 5 670-5 725 MHz, for the countries listed in No. 5.453. The strict application of these provisions would mean that the countries listed in No. 5.453 cannot notify their WAS applications in the form of typical stations, even though they conform with the limits of Resolution 229 (WRC-03). The Board concluded that such a restricted interpretation of all the relevant provisions for the band 5 670-5 725 MHz, for the countries listed in No. 5.453, would result in unnecessary burden for both the administrations listed in No. 5.453 and the Bureau. Consequently, the Board instructed the Bureau to accept notifications for mobile stations, in the form of typical stations, from the administrations listed in No. 5.453, provided that the maximum e.i.r.p. does not exceed 1 W, which implies that each typical station notice receivable in the band 5 670-5 725 MHz (with an e.i.r.p. of less than or equal to 1 W) is deemed to be part of a WAS.

Reason:	The Table of Frequency Allocation contains allocations to the mobile service for different applications: a) for wireless access systems (including RLAN), which are subject to certain conditions as specified in Resolution 229 (WRC-03), and b) for other systems/applications in the mobile service, which are subject to different conditions as specified in other provisions. As the Bureau has no means to differentiate, from the notified data, as to whether the frequency notice concerned relates to a WAS or to another system or application in the mobile service, this Rule is intended to provide guidance to the Bureau as to how the Bureau should treat notices to the mobile service in the concerned bands, without imposing too much burden on the administrations.

ANNEX 2

# Rules concerning

## ARTICLES 5 and 9 of the RR

MOD

5.416

See comments under the Rule of procedure concerning No. 5.415.

Reason: WRC-03 added provision No. 9.6.3 and also modified Appendix 5, Table 5-1, and therefore clarified the regulatory arrangement under No. 9.11 to be applied to frequency assignments for BSS systems under No. 5.416. Consequently, this Rule is updated.

Effective date of application of this modification to the Rule: upon approval (applies to the submissions received after 4 July 2003).

SUP

5.418

Reason: WRC-03 modified Resolution 539 and clarified the regulatory arrangement to be applied to frequency assignments for BSS (sound) systems using non-geostationary orbit in the band 2 630-2 655 MHz. Consequently, this Rule can be suppressed.

Effective date of suppression of the Rule: upon approval (applies to the submissions received after 4 July 2003).

MOD

5.418C

1	In accordance with provision No. 5.418C, as modified by WRC-03, the use of the band 2 630-2 655 MHz by geostationary-satellite networks is subject to the application of the provisions of No. 9.13 with respect to non-GSO satellite systems in the BSS (sound) pursuant to No. 5.418, as of 3 June 2000. Resolution 33 (Rev.WRC-03) resolves that for satellite networks for which the API has been received by the Bureau prior to 1 January 1999, only the procedure in Sections A to C in Resolution 33 (Rev.WRC-03) shall be applied.

2	The Board undertook an in-depth examination of the different procedures and provisions that apply to satellite systems in the band 2 630-2 655 MHz and noted

the difficulty in linking No. 5.418C reference to “notification information” of GSO BSS systems to the No. 22.2 application referred to in No. 5.418A.

3	In the above context, and taking into account of WRC-03 discussion and decision, the Board understands No. 9.13 coordination to apply as described in the Table below. -

| Reason: WRC-03 modified Resolution 33 and clarified the procedure to apply to GSO satellite networks for which the API has been received by the Bureau prior to 1 January 1999 and the request for coordination following that date. In addition, WRC-03 also clarified cases of coordination involving GSO and non-GSO BSS (sound) systems operated under No. 5.418 (see also Rule on Band 2 605-2 655 MHz). The Rule has been updated accordingly. GSO satellite network |  | Date of receipt of coordination information (No. 9.6) | Date of receipt of notification information (No. 11.2) | No. 9.13 applicability |  |
| --- | --- | --- | --- | --- | --- |
| BSS (No. 5.418) |  | < 3.6.2000 | < 3.6.2000 | NO |  |
|  |  | < 3.6.2000 |  3.6.2000 | NO |  |
|  |  |  3.6.2000 |  3.6.2000 | YES |  |

Band 2 605-2 655 MHz

1	Provisions Nos. 5.416, 5.417A, 5.417B, 5.417C, 5.417D, 5.418, 5.418A, 5.418B and 5.418C provide information on the different constraints and procedures applying to the broadcasting-satellite service (BSS) and fixed-satellite service (FSS) in the frequency range 2 605-2 655 MHz.

2	The Board undertook an in-depth examination of the different provisions and the applicability of the different coordination procedures (space network-to-space network (Nos. 9.7, 9.12, 9.12A and 9.13)) that apply to satellite systems in the band 2 605-2 655 MHz and noted the possible difficulty in assessing the service (BSS (sound), BSS (television), FSS) and the nature of the satellite network (GSO or non-GSO) to which Nos. 5.418A, 5.418B, 5.418C, and 5.417B, 5.417C, 5.417D should apply, taking due account of the dates of reception of the complete Appendix 4 coordination or notification information, as appropriate. Indeed, in the band 2 630-2 655 MHz, No. 5.418A refers to the application of the provisions of No. 9.12A for non-GSO systems in the BSS (sound) in certain countries listed in No. 5.418, in respect of GSO systems, without further details on the involved services; No. 5.418B refers to the application of the provisions of No. 9.12 for non-GSO systems in the BSS (sound) under No. 5.418, in respect of other non-GSO systems; and No. 5.418C refers to the application of No. 9.13 by GSO networks in respect of non-GSO systems in the BSS (sound), allocated under No. 5.418. The same wording is also used in Nos. 5.417B, 5.417C and 5.417D relating to BSS systems in the band 2 605-2 630 MHz.

3	Taking the above into account and in the light of WRC-03 discussions and decisions, in particular the addition of an explicit reference to No. 5.418 in Nos. 5.418B, 5.418C, and the explicit reference to No. 5.417A in Nos. 5.417B, 5.417C and 5.417D, the Board understands Nos. 5.418A, 5.418B and 5.418C, and Nos. 5.417B, 5.417C, 5.417D to only address cases of coordination as follows: non-GSO BSS (sound) (5.418, 5.417A) systems vis-à-vis any GSO systems under No. 9.12A, and vis-à-vis any non-GSO systems under No. 9.12, and vice versa, i.e. any GSO systems vis-à-vis non-GSO BSS (sound) (5.418, 5.417A) systems under No. 9.13, and any non-GSO systems vis-à-vis non-GSO BSS (sound) (5.418, 5.417A) systems under No. 9.12, as described in the Table below. This Table applies to coordination requirements between GSO and non-GSO satellite systems for which the API has been received following 1 January 1999 and complete coordination/notification information was received after 2 June 2000 in the band 2 630-2 655 MHz and after 4 July 2003 in the band 2 605-2 630 MHz .

| Coordination request (CR): Column vis-à-vis Row () (2 605-2 655 MHz) | Non-GSO BSS (sound)  (5.418, 5.417A) | GSO BSS  (5.416, 5.418, 5.417A) or FSS  (Region 2) | Non-GSO BSS  (5.416) or FSS  (Region 2) |
| --- | --- | --- | --- |
| Non-GSO BSS (sound)  (5.418, 5.417A) | 9.12 (5.418B, 5.417C) | 9.13 (5.418C, 5.417D) | 9.12 (5.418B, 5.417C) |
| GSO BSS  (5.416, 5.418, 5.417A) or FSS  (Region 2) | 9.12A (5.418A, 5.417B) | 9.7 | No CR (22.2) |
| Non-GSO BSS  (5.416) or FSS  (Region 2) | 9.12 (5.418B, 5.417C) | No CR 22.2 | No CR |

Reason:	Consequences of WRC-03 modifications to Nos. 5.416, 5.418, 5.418A, 5.418B, 5.418C and to Resolutions 539 and 33, and additions of Nos. 5.417A, 5.417B, 5.417C and 5.417D.

There is also a companion proposal to accordingly update Table 9.11A-1 of the Rule of Procedure on No. 9.11A.

Effective date of modification of this Rule: 5 July 2003.

MOD

9.11A

TABLE 9.11A-1
Applicability of the provisions of Nos. 9.11A-9.15 to stations of space services

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency band (MHz) | Footnote No. in Article 5 | Space services mentioned in a footnote referring to No. 9.11A to which Nos. 9.12 to 9.15 apply, or referring to Nos. 9.12-9.14, as appropriate | Other space services to which Nos. 9.12 to 9.14 apply equally | Terrestrial services in respect of which Nos. 9.14 and 9.15 apply equally, as appropriate | Date of provisional application of the allocation if later than 3.06.2000, or if different from the application date of the RR | Notes |

| 2 605-2 630 MHz | 5.417B 5.417C 5.417D | BROADCASTING-SATELLITE (sound) (5.417A) |  | BROADCASTING-SATELLITE (5.416) FIXED-SATELLITE (Region 2) |  | --- (see Resolution 539 (WRC-03)) |  | 8, 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 630-2 655 | 5.418A 5.418B 5.418C | BROADCASTING-SATELLITE (sound) (5.418) ) |  | BROADCASTING-SATELLITE (5.416) FIXED-SATELLITE (Region 2) |  | --- (see Resolution 539 (WRC-03)) |  | 8, 9 |

8	The coordination of the non-GSO BROADCASTING-SATELLITE service (sound) in respect of terrestrial services is subject to the provisions of Resolution 539 (WRC-03).

9	For the applicability of the forms of coordination (Nos. 9.12, 9.12A or 9.13) to be applied between services mentioned in columns 3 and 4, please refer to the Rule of Procedure on 2 605-2 655 MHz and the Rules of Procedure on No. 5.418C, as appropriate.

Reason:	Consequences of WRC-03 modifications to Nos. 5.418, 5.418A, 5.418B, 5.418C and additions of Nos. 5.417B, 5.417C and 5.417D. See also proposal to modify the Rule of Procedure on the band 2 609-2 655 MHz.

Effective date of modification: upon approval (applies to the submissions received after 4 July 2003).

# MOD

5.329

Assignments to stations of the radionavigation-satellite service if recorded need to indicate that they shall not cause harmful interference to assignments to stations of the radionavigation service of the countries listed in No. 5.331 and to stations of the radiolocation service (Symbol R in Column 13B2 and reference to No. 5.329 in Column 13B1).

Reason:	WRC-03 modified No. 5.329, made it applicable as of 5 July 2003, and the Rule is adjusted accordingly.

Effective date of modification of this Rule: 05.07.2003

MOD

9.36

NOC	1

NOC	2

-ted provisions to FSS, BSS and associated space operations in the frequency bands above 3 GHz, with an orbital arc threshold defined as follows:

| 3 |  |
| --- | --- |

-

SUP

9.42

Reason:	WRC-03 included in the Radio Regulations the substance of paragraph 3 (i.e. the extended application of coordination arc) of the Rule of Procedure on No. 9.36 by modifying Table 5-1 of Appendix 5 and made it provisionally applicable as of 5 July 2003. Paragraphs 3, 4 and 5 of this Rule, as well as the related Rule of Procedure on No. 9.42, are no more required and are proposed for suppression. This is also in line with Resolution 901 [COM4/19](WRC-03).

Effective date of modification and suppression of these Rules: After the Bureau completes the treatment of requests for coordination received before 5 July 2003.

SUP

# Rules concerning

# RESOLUTION  84 (WRC-2000)

Power flux-density limits in the bands 37.5-42.5 GHz for
the fixed-satellite service, broadcasting-satellite service
and mobile-satellite service

resolves 2

Reason:	WRC-03 abrogated Resolution 84 (WRC-2000) and the Rule is no more necessary. This abrogation removed the requirement, applicable to GSO FSS assignments in the band 37.5-40 GHz, to seek agreement from administrations in Region 2 in respect of their fixed services (resolves 2 of Resolution 84). This requirement was adopted by WRC-2000 as a precautionary measure until studies were completed as to whether the power flux-density limits of Article 21, Table 21-4, were sufficient to protect more sensitive fixed systems in Region 2. Studies, completed before WRC-03, confirmed that such additional measures were not necessary and WRC-03 abrogated Resolution 84. In view of this, the Bureau will, without re-publishing coordination special sections, remove from its database any previously established agreement requirements under Resolution 84 for frequency assignments at coordination stage and will not examine notifications with respect to compliance with such requirements. Preliminary indications are that no notification of relevant assignments was received during the existence of Resolution 84.

Effective date of suppression of this Rule: Upon approval.

ANNEX 3

# Rules concerning

## ARTICLE 5 of the RR

MOD

5.492

1	The Board concluded that the frequency bands covered by Appendix 30 are not allocated to the FSS in the Regions where the BSS is subject to the Plan of Appendix 30. Those transponders of the BSS which are also used for FSS purposes will be treated in accordance with Article 5 of Appendix 30.

2	Earth stations receiving FSS transmissions from the BSS transponders will be treated as earth stations of the BSS and are not to be notified as individual earth stations.

Reason:	WRC-03 agreed on the methodology to carry out the related compatibility analysis (see § 6 of AP30/30A related statements in Document 370 and § 28 of Document 410). In addition, WRC-03 decided not to include any specific item in Appendix 4 to indicate such a use.

Effective date of modification of this Rule: Upon approval.

# Rules concerning

## APPENDIX 30 to the RR

(Rules are arranged by paragraph numbers of Appendix 30)

Art. 4

MOD

4.1.1 e)

1	This examination shall consider only those administrations having assignments to FSS space stations whose necessary3 bandwidth overlaps the necessary3 bandwidth of the proposed new or modified assignment to the Regions 1 and 3 List.

2	Under this paragraph an assignment recorded in the Master Register shall be understood as defined in § 1a), 1b), 1c) and 1cbis) of Appendix 5.

3	In the case of inclusion of a new assignment in the Regions 1 and 3 List, different from the frequency assignments in the Plan or List for Regions 1 and 3 as established by WRC-2000, the limit prescribed in § 3 or Note 1 of Section 6 of Annex 1, as appropriate, shall be applied.

4	The Bureau, in applying Section 6 of Annex 1 shall, where applicable, compare the power flux-density values resulting from the proposed new or modified assignments to the Regions 1 and 3 List with those values in the Regions 1 and 3 Plan or List as appropriate. If it is not possible to do so, the Bureau should use the absolute limit expressed in Section 6 of Annex 1 to that Appendix.

Reason:	WRC-03 modified Section 6 of Annex 1 to Appendix 30. Consequently, this modification is proposed in order to avoid repetition in this Rule of some regulatory text contained in that Section or in § 4.1.1 e) of Appendix 30.

Effective date of modification of this Rule: Upon approval.

MOD

4.1.3

1	In the event that the Bureau cancels a frequency assignment in application of § 5.3.2 of Article 5 of this Appendix, the corresponding assignment, which has been submitted either under § 4.2.6 (except in the case of a request for replacement of an assignment in the Region 2 Plan) and entered in the Region 2 Plan, or under § 4.1.3 and entered in the Regions 1 and 3 List, shall also be removed from the Plan or the List according to the case. The Bureau does not need to recalculate the affected administration(s) as a result of the above-mentioned cancellation.

2	See also Rules of Procedure concerning Receivability of the Forms of Notice.

-

Reason:	WRC-03 modified § 4.1.3 and 4.2.6 of Appendices 30 and 30A. The substance of the proposed suppressed paragraphs was transferred to those revised regulatory provisions.

Effective date of modification of this Rule: Upon approval.

ADD

4.1.7bis

The agreement referred to in § 4.1.7bis/4.2.11 is the agreement of the administrations identified under § 4.1.1/4.2.3 and of those under § 4.1.7/4.2.10, respectively, which have been confirmed by the Bureau using the appropriate criteria.

Reason:	WRC-03 added new § 4.1.7bis to Article 4 of Appendices 30 and 30A similar to the existing § 4.2.11. Consequently, this new Rule is proposed in order to align the regulatory procedures.

Effective date of this new Rule: Upon approval.

MOD

4.2.3 c)

1	NOC

2	According to Resolution 42 (Rev.WRC-03), the Board decided that, when applying this paragraph, the Bureau shall not take account of the interim systems.

3	NOC

Reason:	Resolution 42 (Rev.Orb-88) was modified by WRC-03. This editorial modification is proposed in order to correct the reference to the Resolution.

Effective date of modification of this Rule: Upon approval.

MOD

4.2.3 e)

1	This examination shall consider only those administrations having assignments to FSS space stations whose necessary6 bandwidth overlaps the necessary6 bandwidth of the proposed modification to the Region 2 Plan.

2	See § 2 in the Rules of Procedure relating to § 4.1.1 e).

3	In the case of inclusion of a new assignment to the Region 2 Plan, different from the frequency assignments in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference, the limits prescribed in § 3 of Section 6 and in the second indent of Section 7 of Annex 1 shall be applied.

.

4	The Bureau, in applying Sections 6 and 7 of Annex 1 shall, where applicable, compare the power flux-density and DT/T values, respectively, resulting from the proposed modification to the Region 2 Plan with those values in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference. If it is not possible to do so, the Bureau should use the absolute limit expressed in § 6 and 7 of Annex 1 to that Appendix.

Reason:	WRC-03 modified Section 6 of Annex 1 to Appendix 30. Consequently, this modification is proposed in order to avoid repetition in this Rule of some regulatory text contained in that Section or in § 4.2.3 e) of Appendix 30.

Effective date of modification of this Rule: Upon approval.

MOD

4.2.3 f)

1	Until there is a Plan for Region 3 for the band 12.5-12.7 GHz, this examination shall consider only those administrations of Region 3 having broadcasting-satellite service assignments whose necessary9 bandwidth overlaps the necessary9 bandwidth of the proposed modification to the Region 2 Plan.

2	See § 2 in the Rules of Procedure relating to § 4.1.1 e).

Reason:	WRC-03 modified Section 3 of Annex 1 to Appendix 30. Consequently, this modification is proposed in order to align the text and avoid repetition in this Rule of some regulatory text contained in that Section or in § 4.2.3 f) of Appendix 30.

Effective date of modification of this Rule: Upon approval.

MOD

4.2.11

See Rules of Procedure relating to § 4.1.7bis.

Reason:	WRC-03 added new § 4.1.7bis to Article 4 of Appendices 30 and 30A similar to the existing § 4.2.11. Consequently, this modification is proposed in order to align the regulatory procedures.

Effective date of modification of this Rule: Upon approval.

Art. 5

MOD

5.2.1 d)

1	NOC

1.1	NOC

1.2	With respect to the compatibility with other inter-regional assignments in the same service or assignments in another service sharing the same frequency bands, as appropriate, the increase of the interference will be checked by calculating the power flux-density value, produced by the proposed new characteristics at any test-point or within the service area of the other assignments, or by calculating the T/T value in accordance with the method given in Case II of Appendix 8, and by comparing the resulting power flux-density or T/T values, according to the case, with those obtained with the previous11 characteristics of the subject assignment.

1.3	NOC

2	NOC

3	NOC

Reason:	This modification is proposed in order to clarify the application of the Rule.

Effective date of modification of this Rule: Upon approval.

MOD

5.2.2.2

Part of this paragraph deals with interim systems which are submitted in application of Resolution 42 (Rev.WRC-03) for Region 2.

In case of Regions 1 and 3, should the Bureau reach a favourable Finding with respect to § 5.2.1 a) and 5.2.1 c) but an unfavourable Finding with respect to § 5.2.1 b) and 5.2.1 d), the assignments in question shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this Finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

Reason:	Resolution 42 (Rev.Orb-88) was modified by WRC-03. This editorial modification is proposed in order to correct the reference to the Resolution.

Effective date of modification of this Rule: Upon approval.

SUP

5.3.1

Reason:	WRC-03 modified § 4.1.3, 4.2.6 and 5.3.1 of Appendices 30 and 30A. The substance of this Rule was transferred to those revised regulatory provisions.

Effective date of suppression of this Rule: Upon approval.

MOD

An. 4

## Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not subject to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the Plan, the List or proposed new or modified assignments in the List for Regions 1 and 3; in Region 1 (12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with respect to the Plan or proposed modifications to the Plan in Region 2; in Region 3 (12.2-12.5 GHz) with respect to the Plan, List or proposed new or modified assignments in the List for Region 1

(See Article 7)

Clarification on the implementation of Annex 4 to Appendix 30

1	The examination will consider only those administrations having assignments to BSS space stations subject to a Plan whose necessary15 bandwidth overlaps the necessary15 bandwidth of the proposed FSS (or BSS not subject to a Plan) assignment.

2	In the absence of available service area contour of the BSS assignment, the methodology described in Annex 4 to Appendix 30 will be applied but instead of verifying the power flux-density compliance over any portion of the service area, it will be verified at each of the BSS test-points associated with the service area of the corresponding BSS assignment.

Reason:	WRC-03 modified Annex 4 to Appendix 30. The substance of the proposed suppressed paragraphs was transferred to that revised regulatory provision.

Effective date of modification of this Rule: Upon approval.

# Rules concerning

## APPENDIX 30A to the RR

(Rules are arranged by paragraph numbers of Appendix 30A)

Art. 4

MOD

4.1.3

1	In the event that the Bureau cancels a frequency assignment in application of § 5.3.2 of Article 5 of this Appendix, the corresponding assignment, which has been submitted either under § 4.2.6 and entered in the Region 2 Plan, or under § 4.1.3 and entered in the Regions 1 and 3 List, shall also be removed from the Plan or the List according to the case. The Bureau does not need to recalculate the affected administration(s) as a result of the above-mentioned cancellation.

2	See also Rules of Procedure concerning Receivability of the Forms of Notice.

-

Reason:	WRC-03 modified § 4.1.3 and 4.2.6 of Appendices 30 and 30A. The substance of the proposed suppressed paragraphs was transferred to those revised regulatory provisions.

Effective date of modification of this Rule: Upon approval.

ADD

4.1.7bis

The agreement referred to in § 4.1.7bis/4.2.11 is the agreement of the administrations identified under § 4.1.1/4.2.3 and of those under § 4.1.7/4.2.10, respectively, which have been confirmed by the Bureau using the appropriate criteria.

Reason:	WRC-03 added new § 4.1.7bis to Article 4 of Appendices 30 and 30A similar to the existing § 4.2.11. Consequently, this new Rule is proposed in order to align the regulatory procedures.

Effective date of this new Rule: Upon approval.

MOD

4.2.2 c)

1	NOC

2	According to Resolution 42 (Rev.WRC-03), the Board decided that, when applying this paragraph, the Bureau shall not take account of the interim systems.

3	NOC

Reason:	Resolution 42 (Rev.Orb-88) was modified by WRC-03. This editorial modification is proposed in order to correct the reference to the Resolution.

Effective date of modification of this Rule: Upon approval.

MOD

4.2.11

See Rules of Procedure relating to § 4.1.7bis.

Reason:	WRC-03 added new § 4.1.7bis to Article 4 of Appendices 30 and 30A similar to the existing § 4.2.11. Consequently, this modification is proposed in order to align the regulatory procedures.

Effective date of modification of this Rule: Upon approval.

Art. 5

SUP

Scope of application

Reason:	WRC-03 modified the footnote to the title, § 5.1.2, 5.1.3, 5.1.4 and added § 5.1.2bis to Article 5 Appendix 30A. Consequently, this Rule can be suppressed.

Effective date of suppression of this Rule: Upon approval.

MOD

5.2.1 d)

1	NOC

1.1	NOC

1.2	With respect to the compatibility with other inter-regional assignments in the same service or assignments in another service sharing the same frequency bands, as appropriate, the increase of the interference will be checked by calculating the T/T valuesin accordance with the method given in Appendix 8, produced by the proposed new characteristics, and by comparing the resulting T/T values with those obtained with the previous5 characteristics of the subject assignment.

1.3	NOC

2	NOC

Reason:	WRC-03 replaced the former text in the fifth indent of § 5.2.1 d) of Appendix 30A with a new one and modified Section 3.15 of Annex 3 to Appendix 30A, thus removing the application of the cluster concept from the Region 1 and 3 Plans and Lists and consequently the Rules of Procedure relating to § 3.15 of Annex 3 of Appendix 30A were deleted. Therefore, paragraphs 3 and 4 of this Rule can be suppressed. In addition, modification in paragraph 1.2 is proposed in order to clarify the application of the Rule.

Effective date of modification of this Rule: Upon approval.

MOD

5.2.2.2

Part of this paragraph deals with interim systems which are submitted in application of Resolution 42 (Rev.WRC-03) for Region 2.

In the case of Regions 1 and 3, should the Bureau reach a favourable finding with respect to § 5.2.1 a) and 5.2.1 c) but an unfavourable finding with respect to § 5.2.1 b) and 5.2.1 d), the assignments in question shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

Reason:	Resolution 42 (Rev.Orb-88) was modified by WRC-03. This editorial modification is proposed in order to correct the reference to the Resolution.

Effective date of modification of this Rule: Upon approval.

SUP

5.3.1

Reason:	WRC-03 modified § 4.1.3, 4.2.6 and 5.3.1 of Appendices 30 and 30A. The substance of this Rule was transferred to those revised regulatory provisions.

Effective date of suppression of this Rule: Upon approval.

Art. 6

MOD

6.1

1	The paragraphs of Article 6 do not mention interim systems implemented in accordance with Resolution 42 (Rev.WRC-03). Such systems may be implemented in the frequency band 17.7-17.8 GHz for Region 2 shared with equal rights with terrestrial services:

Such usage may affect terrestrial stations.

2	NOC

3	NOC

Reason:	Resolution 42 (Rev.Orb-88) was modified by WRC-03. This editorial modification is proposed in order to correct the reference to the Resolution.

Effective date of modification of this Rule: Upon approval.

# Rules concerning

## APPENDIX 30B to the RR

MOD

6.24

1	NOC

2	NOC

3	NOC

4	NOC

5	NOC

6	NOC

7	NOC

Reason:	WRC-03 considered the treatment of multiple consecutive submissions from the same administration and decided to add new § 6.43bis and 6.56bis to Article 6 of Appendix 30B. The substance of § 8 and 9 of this Rule was transferred to those new regulatory provisions.

Effective date of modification of this Rule: Upon approval.

MOD

6.43

See also § 5, 6 and 7 in the comments made under the Rules of Procedure relating to § 6.12.

Reason:	WRC-03 added new § 6.43bis and 6.56bis to Article 6 of Appendix 30B. The substance of § 8 and 9 in the comments made under the Rules of Procedure relating to § 6.24 was transferred to those new regulatory provisions.

Effective date of modification of this Rule: Upon approval.

MOD

6.56

See § 5, 6 and 7 in the comments made under the Rules of Procedure relating to § 6.12.

Reason:	WRC-03 added new § 6.43bis and 6.56bis to Article 6 of Appendix 30B. The substance of § 8 and 9 in the comments made under the Rules of Procedure relating to § 6.24 was transferred to those new regulatory provisions.

Effective date of modification of this Rule: Upon approval.

MOD

An. 2

## Basic data to be furnished in notices relating to stations in the fixed-satellite service entering the design stage using frequency bands of the Plan

In order to establish a formal date of receipt for submission received by the Bureau, information relating to the modification of orbital position(s), in application of the “PDA concept” should be sent together with Appendix 4 data. See also Rules of Procedure relating to § 6.16.

Reason:	WRC-03 replaced the former Annex 2 to Appendix 30B with a new one that refers to data listed in Appendix 4. Consequently, the second part of this Rule can be suppressed.

Effective date of suppression of this Rule: Upon approval.

______________

3	 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

8	6 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

9	 	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

1	1 	As appearing in the appropriate Plan or List, according to the case.

1	5	In the absence of a clear indication of the precise frequency of each carrier within the assigned frequency band, the Bureau uses in its analysis the assigned frequency band (i.e. data item C.3 a) of Annex 2A of Appendix 4) instead of the necessary bandwidth (i.e. data item C.7 a) of Annex 2A of Appendix 4).

5	 	As appearing in the appropriate Plan or List, according to the case.