To Administrations of Member States of the ITU

Subject:	Processing of  the pending modifications/additions submitted under Article 4 of      Appendices  30/S30 and 30A/S30A, 
Implementation of Resolution 533/PLEN-4(WRC-97)

To the Director General

Dear Sir,

1.	Pursuant to the adoption of the revised Plan for the Broadcasting Satellite Service and the associated feeder-link for Regions 1 and 3 by WRC-97, as contained in Appendices 30/S30 (WRC-97) and 30A/S30A (WRC-97) and in accordance with Resolution 533/PLEN-4 (WRC-97), the Radiocommunication Bureau is instructed to review the requirement for coordination of the satellite networks submitted under Article 4 of Appendices 30/S30 (WRC-97) and 30A/S30A (WRC-97) including those which were already published in the Special Sections annexed to the Bureau Weekly Circulars.

2.	According to this Resolution, the Bureau shall apply in its examinations of Article 4 submissions of Regions 1 and 3 the new technical planning criteria and reference situation of the revised Plan for Regions 1 and 3, derived, in particular, from the application of the protection ratios used for EPM analyses in WRC-97 as defined in Annexes 5 and 3 to Appendices 30/S30 (WRC-97) and 30A/S30A (WRC-97) respectively (Rec. ITU-R BO.1297) instead of the protection ratios applied at WARC-77 and WARC-ORB-88.

3.	It is worth mentioning that for assignments which are in conformity with                              Appendices 30/S30 (WRC-97) and 30A/S30A (WRC-97) notified to the Bureau under Article 5 of these Appendices, brought into use before 27 October 1997 and for which the date of bringing into use has been confirmed to the Bureau in accordance with paragraph 5.2.8 of these Appendices, parameters used for the establishment of the initial Plans at WARC-77 and WARC-ORB-88, or those published in the corresponding Special Sections, as the case may be, will continue to be utilized.

4.	WRC-97 has also adopted the following new technical parameters:

4.1	reference receiving earth station antenna (Rec. ITU-R BO.1213) as defined in Figure 7bis, paragraph 3.7.2 of Annex 5 to Appendix 30/S30 (WRC-97),

- reference transmitting earth station antenna (Rec. ITU-R BO.1295) as defined in curves A' and B' of Figure A, paragraph  3.5.3 of Annex 3 to Appendix 30A/S30A (WRC-97),

./.

- reference receiving space station antenna (Rec. ITU-R BO.1296) as defined in curves A' and B' of  Figure B, paragraph  3.7.3 of Annex 3 to Appendix 30A/S30A (WRC-97),

- down-link transmitting space station fast roll-off antenna patterns as defined in Figure 11b, paragraph  3.13.3 of Annex 5 to Appendix 30/S30 (WRC-97),

4.5	receiving space station pointing accuracy: 0.1° instead of 0.2° (as defined in paragraph  3.7.4 of Annex 3 to Appendix 30A/S30A (WRC-97)),

4.6	angular rotation of the transmitting beam about its axis not exceeding ±1° instead of ±2° (as defined in Section  3.14 of Annex 5 to Appendix 30/S30 (WRC-97)),

4.7	reduced feeder-link receiver system noise temperature (as defined in Section 3.8 of Annex 3 to Appendix 30A/S30A (WRC-97)).

5.	For the revised Regions 1 and 3 Plan, WRC-97 has also applied the transmitting and receiving space station East-West station keeping error of ±0.1° (as defined in Section 3.11 of Annex 5 to                  Appendix 30/S30 (WRC-97) and Section 3.15 of Annex 3 to Appendix 30A/S30A (WRC-97)), as well as the new rain climatic zones (Rec. ITU-R P837-1) as defined in Figures 1 and 2, Section 2.1 of Annex 5 to Appendix 30/S30 (WRC-97) and Figures 1 and 3, Section 2.2 of Annex 3 to Appendix 30A/S30A (WRC-97).

6.	In view of the above, the Radiocommunication Bureau in its technical examination of pending Article 4 modifications/additions, as well as for the review of all Special Sections previously published, will take the following actions:

6.1	Application of the new technical parameters adopted by WRC-97:

- For transmitting and receiving space station East-West station keeping error, the value of ±0.1° or better if submitted by administration, will be used in the examination.

- For receiving space station pointing accuracy the value of 0.1° and for the angular rotation of the transmitting elliptical beam about its axis the value of ±1° or better if submitted by an administration, will be used in the examination. Administrations which have submitted values worse than these will be informed to take the required actions, as appropriate.

- Reference antenna patterns adopted by WRC-97 will be applied for the examination of the pending Article 4 networks as well as for the review of all Special Sections previously published instead of those referred to in Figure 7, paragraph 3.7.2 of Annex 5 to Appendix 30/S30 (WRC-97), and in curves A and B of Figures A and B, paragraphs 3.5.3 and 3.7.3 respectively of Annex 3 to Appendix 30A/S30A (WRC-97).

- In case of submissions using different reference antenna patterns than those referred to in Figure 7, paragraph 3.7.2 of Annex 5 to Appendix 30/S30 (WRC-97) and in curves A and B of Figures A and B, paragraphs 3.5.3 and 3.7.3 respectively of Annex 3 to Appendix 30A/S30A (WRC-97), their respective administrations will be consulted to determine whether they agree to the use of the new reference antenna pattern adopted by WRC-97. Based on the reply received, the Bureau will act accordingly.

- For all submitted test points, the rain climatic zones as defined in Figures 1 and 2, Section 2.1 of Annex 5 to Appendix 30/S30 (WRC-97) and Figures 1 and 3, Section 2.2 of Annex 3 to                      Appendix 30A/S30A (WRC-97), will be used.

- Those administrations which have submitted a higher value of feeder-link receiving system noise temperature compared with that adopted by WRC-97 will be consulted to determine whether they agree to the use of the new value.

6.1.7	A separate message will be sent to each individual administration which has submitted modification(s)/addition(s) under Article 4 of Appendices 30/S30 (WRC-97) and/or 30A/S30A (WRC-97) informing them of the situation of their respective submitted network in order to take appropriate action where required.

6.2	For the identification of affected administrations under provisions 4.3.1.1 and 4.2.1.1 of Article 4 of Appendices 30/S30 (WRC-97) and 30A/S30A (WRC-97) respectively, the new protection ratios will be used for EPM analyses as defined in Annexes 5 and 3 to Appendices 30/S30 (WRC-97) and 30A/S30A (WRC-97) respectively, instead of those applied for the establishment of the initial Plans at WARC-77 and            WARC-ORB-88, except for assignments referred to in paragraph 3 above.

In this examination, the Bureau, in accordance with Resolves 3 of Resolution 533/PLEN-4        (WRC- 97), will use the feeder-link and down-link files which have been derived from the merged Plan file adopted by WRC-97 and completed by including the feeder-links of Radiosat-6 and Radiosat-7 networks. Reference situations calculated from these feeder-link and down-link files will be published in a separate Circular Letter.

6.3	As a result of the individual consultation referred to in point 6.1.7 above, the Bureau will determine the revised list of administrations likely to be affected by the subject network, should the modified parameters of a network have an impact on:

6.3.1	the PFD levels already calculated by the Bureau in application of provisions 4.3.1.2 to 4.3.1.5 of Article 4 of Appendix 30/S30 (WRC-97),

6.3.2	the coordination areas already calculated by the Bureau in application of provisions 4.2.1.2 and 4.2.1.3 of Article 4 of Appendix 30A/S30A (WRC-97),

6.3.3	the DT/T level already calculated by the Bureau in application of provision 4.2.1.4 of Article 4 of Appendix 30A/S30A (WRC-97),

6.3.4	the DT/T level already calculated by the Bureau in application of provision 4.2.3.4 of Article 4 of Appendix 30A/S30A (WRC-97).

7.	It is worth mentioning that in order to avoid any further delay in the examination of all pending Article 4 networks, including those submitted by Administrations of Region 2 as well as for the Special Sections previously published, based on the new technical parameters and criteria adopted by WRC-97, only those parameters mentioned in paragraph 4 above will be subjected to a possible modification. Any other cases of modification that are requested by an Administration in reply to the Bureau's Telefax referred to in point 6.1.7 above, will be treated in accordance with the related Rules of Procedure.

Yours faithfully,

Robert W. Jones

Director,

Radiocommunication Bureau

Distribution:

-	Administrations of Member States of the ITU

-	Members of the Radio Regulations Board