| INTERNATIONAL TELECOMMUNICATION UNION Radiocommunication Bureau (Direct Fax No. +41 22 730 57 85) |  |
| --- | --- |

Circular Letter	4 June 1999

CR/119

To Administrations of Member States of the ITU

Subject:	Closure date for submission by administrations of the seasonal High Frequency Broadcasting Schedule for the period November 1999 - March 2000 (B-1999)

To the Director-General

Dear Sir or Madam,

1.	I wish to inform you that, in accordance with the provisions of No. S12.31 of the Radio Regulations, the Radiocommunication Bureau has set  11 August 1999 as the closure date for the receipt of the above-projected seasonal Broadcasting Schedules.

2.	In view of the need to issue the Tentative Schedule to administrations two months ahead of the implementation date (No. S12.34 of the Radio Regulations) and considering the time required for calculations, reproduction and dispatch, administrations are urged to send in their Schedules

before the closure date and, so far as possible, before 30 July 1999.

3.	Late submissions not only may fail to be incorporated in the Tentative Schedule but could seriously affect its value to administrations and that of any recommendation made by the Bureau. Utilisation of an incomplete Tentative Schedule, moreover, would lead to confusion and difficulty as soon as it is published.

4.	Requirements are to be submitted by administrations or organisations who are authorised by the administrations concerned to do so, such as broadcasters and frequency managers.  In the latter case, administrations are expected to advise the Bureau in writing, in advance, stating the names of the authorized organizations, their three-letter code for ease of identification and the scope of the authorizations (see S12.1); otherwise the requirements cannot be accepted by the Bureau.

5.	Submission of requirements will be in electronic format only.  Broadcasting requirements can be submitted to the Bureau using a 3 ½” computer diskette or can also be sent electronically to brmail@itu.int  (Resolution 535 of WRC-97 refers).

6.	A common electronic format will be used. A list of the data fields to be submitted for a requirement and their specifications is given in Annex 1.  The format of the electronic file of requirements is described in Annex 2.

7. 	The foreseen dates of dispatch to registered users of the CD-ROMs containing the updated schedule is indicated in Annex 3 together with the dates by which updated schedules shall be received by the Bureau in order to be incorporated.

Yours faithfully,

Robert W. Jones
	Director
	Radiocommunication Bureau

Annex: 3

Distribution:

-	Administrations of Member States of the ITU

-	Members of the Radio Regulations Board

ANNEX 1

INPUT DATA TO BE SUBMITTED FOR A REQUIREMENT

Administration Code (3-character string)

Mandatory, a three-letter administration code in accordance with the ITU’s designation.  An up-to-date reference list is included with the HFBC software package.

Alternate Frequencies/Bands (5-digit integer)

Optional.  Up to three alternate frequencies/Bands can be notified.  If notified, the Bureau will carry out the necessary analysis to select the most suitable frequency among the operating frequency and any alternate frequencies notified. For SSB usage, the nominal carrier frequency is to be notified.

Antenna Code (up to 3-digit integer)

Mandatory, a unique code representing transmitting antenna of specific technical parameters.

A list including antenna code and antenna definition, based on ITU-R BS 705, is to be maintained by the Bureau.  A new antenna code can be added upon request of administrations.   This is to avoid having the same antenna code for different transmitting antennas which may cause errors in determination of compatibility between requirements. An up-to-date reference list is included with the HFBC software package.

As the antenna codes between 1 and 76 are to be phased out, it is recommended that corresponding antenna codes in the range 100 to 950 should be used.

Antenna Design Frequency (up to 5-digit integer)

Mandatory, design frequency will be in kHz, within the range between 2 000 kHz and 30 000 kHz. 
0 or blank signifies that antenna is designed for the operating frequency.

Antenna slew angle (up to 2-digit integer)

Antenna slew angle is the difference between the azimuth of maximum radiation and the physical orientation of the antenna.  If a slewed antenna is in use, the slew angle must be notified.  The value notified must be in the range -30 to 30.  Default value is 0.

Azimuth of maximum radiation (up to 3-digit integer)

Mandatory.  If the transmitting antenna is directional, the value for the azimuth of maximum radiation must be notified.  This must be in the range 0 to 360 degrees (from True North).  If the antenna is non-directional,  0  shall be notified.

Broadcaster Code (3-character string)

Recommended. An up-to-date reference list containing codes, names and contact information of broadcasting organisations is included with the HFBC software package.

Site Code (3-character string)

Mandatory.  Unique code representing transmitting site.

A list including site code, site name, its geographical co-ordinates is to be maintained by the Bureau.  A new site can be added upon request of administrations.   This is to avoid having the same site code for different transmitter sites which may cause confusion in coordination of requirements. An up-to-date reference list is included with the HFBC software package.

Start Date (6-character string)

Mandatory.  The start date may not be earlier than the start of the schedule period.  The start date may not be the same as the stop date for a requirement.

Start Time (4-digit integer)

Mandatory.  A valid start time for this requirement must be notified using the 24 hour UTC system.

The value must be an integral multiple of 1 minute and may not be the same as the Stop Time.

Stop Date (6-character string)

Mandatory.  The stop date may not be later than the end of the schedule.  The stop date may not be the same as the start date for the same requirement.

Stop Time (4-digit integer)

Mandatory.  A valid stop time for this requirement must be notified using the 24 hour UTC system.

The value notified must be an integral multiple of 1 minute and may not be the same as the Start Time.

Target Service Area (30-character string)

Mandatory.  A set of CIRAF Zones/Quadrants must be notified representing the target area to be served.

A Zone number on its own may be used or it may be followed by S, SW, etc. to indicate a Quadrant. More than one Zone or Zone/Quadrant may be notified, provided that they are separated by a comma.

Transmitter power in kW (up to 4-digit integer)

Mandatory.  The power of the transmitter in kW must be notified.  The value notified must be an integer in the range 1 to 5000 (kW).

For DSB transmitters the carrier power is to be given; for SSB transmitters the peak envelope power is to be used.

ANNEX 2

ELECTRONIC FORMAT FOR INPUT/OUTPUT TEXT FILE

Line 1

| Item | Format | Start col. | Stop col. | Range | Examples | Note |
| --- | --- | --- | --- | --- | --- | --- |
| ; | A1 | 1 | 1 |  | ; |  |
| Season | A3 | 3 | 5 | Ref. table season.txt | A99 |  |
| Notifying organization | A3 | 7 | 9 |  | RNW | Administration or Freq. Management org. |
| Date sent | A11 | 11 | 21 | dd-mmm-yyyy format | 10-oct-1998 |  |

then, one line for each requirement:

| Item | Format | Start col. | Stop col. | Range | Examples | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency/Band (kHz) | I5 | 1 | 5 | Ref. table Rngfreq.txt | 9895, 6 |  |
| Start time (UTC) | I4 | 7 | 10 | 0000-2359 | 0125, 2300 |  |
| Stop time (UTC) | I4 | 12 | 15 | 0001-2400 | 0027 |  |
| Target Service Area | A30 | 17 | 46 | 1-85 | 27, 28sw, 18-20 |  |
| Station code | A3 | 48 | 50 | Reference table | SMG, FLE |  |
| Power (kW) | I4 | 52 | 55 | 1-5000 kW | 250 |  |
| Azimuth of Maxim. Radiation | I3 | 57 | 63 | 0 - 359 | 87 |  |
| Antenna Slew Angle | I3 | 65 | 67 | >= -30, =< +30 | -15 |  |
| Antenna Code | I3 | 69 | 71 | Ref. table Antenna.txt |  |  |
| Days of operation | A7 | 73 | 79 | 1-7 (Sunday=1) | 56 or 1234567 |  |
| Start date | A6 | 81 | 86 | >= Start date of season | 270399 |  |
| Stop date | A6 | 88 | 93 | <= Stop date of season | 250900 = (25 Sep. 2000) |  |
| Modulation | A1 | 95 | 95 | D=DSB, S=SSB -12, T=SSB -6 dB N=Digital. | D |  |
| Antenna design frequency (kHz) | I5 | 97 | 101 | 2 000 - 30 000 kHz | 7200 | If blank or zero operating freq. is assumed |
| Language (o) | A10 | 103 | 112 | Free format | English |  |
| Administration Code | A3 | 114 | 116 | Ref. table Admin.txt | USA |  |
| Broadcaster Code(r) | A3 | 118 | 120 | Ref. table Broadcas.txt | TWR |  |
| Frequency Manager Organ. Code (r) | A3 | 122 | 124 | Ref. table FMOrg.txt | FCC | If blank, identical to Administration code. |
| Identification (br) | I5 | 126 | 130 |  |  | BR or coordination group generated. |
| Old data (br) | I1 | 132 | 132 | 1 if no info is received | 1 | BR generated, output file only. |
| Alternate. Frequency 1/ Alternate. Band 1 (o) | I5 | 134 | 138 | band in MHz (6,7 etc) or freq. in kHz | 7150 |  |
| Alternate. Frequency 2/ Alternate. Band 2 (o) | I5 | 140 | 144 | band in MHz (6,7 etc) or freq. in kHz | 9 |  |
| Alternate. Frequency 3/ Alternate. Band 3 (o) | I5 | 146 | 150 | band in MHz (6,7 etc) or freq. in kHz | 11 |  |
| Notes (o) | A7 | 152 | 158 |  |  |  |

- Recommended

(o)   Optional

(br)  BR generated

ANNEX 3

HFBC on CD-ROM - Season B99

List of editions and closing dates for submissions

| Schedule Title | Date of edition | Date limit for submissions |
| --- | --- | --- |
| B99 Tentative | 30 August 1999 | 11.08.1999 |
| B99 Schedule 1 | 29 October 1999 | 13.10.1999 |
| B99 Schedule 2 | 23 December 1999 | 08.12.1999 |
| B99 Schedule 3 | 28 February 2000 | 09.02.2000 |
| B99 Final | 28 April 2000 | 12.04.2000 |