| INTERNATIONAL TELECOMMUNICATION UNION Radiocommunication Bureau (Direct Fax No. +41 22 730 57 85) |  |
| --- | --- |

Corrigendum 1 to
     Circular Letter	24 November 2000

CR/152

To Administrations of Member States of the ITU

Subject:	Application of Article S12 of the Radio Regulations: Closing date for receipt of High Frequency Broadcasting schedules for the season A2001 (March 2001 – October 2001)

To the Director General

Please replace page 3 (Annex 1) of the Circular Letter with the attached.

Yours faithfully,

Robert W. Jones
	Director
	Radiocommunication Bureau

Distribution:

-	Administrations of Member States of the ITU

-	Members of the Radio Regulations Board

ANNEX 1

ELECTRONIC FORMAT OF THE TEXT FILE TO BE USED FOR NOTIFICATION OF HF BROADCASTING

Line 1

| Item | Format | Start col. | Stop col. |  | Range | Examples | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ; | A1 | 1 |  | 1 |  | ; |  |
| Season | A3 | 3 |  | 5 | Ref. table season.txt | A01 |  |
| Notifying organization | A3 | 7 |  | 9 | Ref. table admin.txt or Ref. table authoris.txt | AFS or SNT | Administration or Authorized organization |
| Date sent | A11 | 11 |  | 21 | dd-MMM-yyyy format | 10-DEC-2000 | (in English) |

then, one line for each requirement:

| Item | Format | Start col. | Stop col. | Range | Examples | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency/Band (kHz) | I5 | 1 | 5 | Ref. table Rngfreq.txt | 9895 or 6 | Frequency in kHz or Band in MHz (6,7, etc.) |
| Start time (UTC) | I4 | 7 | 10 | 0000-2359 | 0125 |  |
| Stop time (UTC) | I4 | 12 | 15 | 0001-2400 | 0027 |  |
| Target Service Area | A30 | 17 | 46 | 1-85 | 27, 28SW, 18-20 | Caution: Some CIRAF zones are not divided into quadrants: 1-5,17,19-26,67,69-75 |
| Station code | A3 | 48 | 50 | Ref. table Site.txt | SMG |  |
| Power (kW) | I4 | 52 | 55 | 1-5000 | 250 | Caution: for less than 1 kW use 1 |
| Azimuth of Maxim. Radiation | I3 | 57 | 63 | 0 - 359 | 87 |  |
| Antenna Slew Angle | I3 | 65 | 67 | >=-30, =< +30 | -15 |  |
| Antenna Code | I3 | 69 | 71 | Ref. table Antenna.txt |  |  |
| Days of operation | A7 | 73 | 79 | 1-7 | 56 or 1234567 | Sunday=1 |
| Start date | A6 | 81 | 86 | >= Start date of season | 250301 | (25 March 2001) |
| Stop date | A6 | 88 | 93 | <= Stop date of season | 281001 | (28 October 2001) |
| Modulation | A1 | 95 | 95 | D=DSB, S=SSB -12, T=SSB -6 dB N=Digital. | D |  |
| Antenna design frequency (kHz) | I5 | 97 | 101 | 2000-30000 | 7200 | If blank or zero operating freq. is assumed |
| Language (o) | A10 | 103 | 112 | Free format | English |  |
| Administration Code | A3 | 114 | 116 | Ref. table Admin.txt | USA |  |
| Broadcaster Code(r) | A3 | 118 | 120 | Ref. table Broadcas.txt | TWR |  |
| Frequency Manager Organ. Code (r) | A3 | 122 | 124 | Ref. table FMOrg.txt | FCC | If blank, identical to Administration code. |
| Identification (br) | I5 | 126 | 130 |  |  | BR or coordination group generated. |
| Old data (br) | I1 | 132 | 132 | 1 if no info is received | 1 | BR generated, output file only. |
| Alternate. Frequency 1/ Alternate. Band 1 (o) | I5 | 134 | 138 | Ref. table Rngfreq.txt | 7150 | Frequency in kHz or band in MHz (6,7, etc.) |
| Alternate. Frequency 2/ Alternate. Band 2 (o) | I5 | 140 | 144 | Ref. table Rngfreq.txt | 9 | Frequency in kHz or band in MHz (6,7, etc.) |
| Alternate. Frequency 3/ Alternate. Band 3 (o) | I5 | 146 | 150 | Ref. table Rngfreq.txt | 11 | Frequency in kHz or band in MHz (6,7, etc.) |
| Notes (o) | A7 | 152 | 158 |  |  |  |

- Recommended

(o)   Optional

(br)  BR generated