INTERNATIONAL TELECOMMUNICATION UNION

Radiocommunication Bureau (Direct Fax N°. +41 22 730 57 85)

23 April 2002

| Circular Letter CR/177 |  |
| --- | --- |

To Administrations of Member States of ITU

Subject:	Rules of Procedure

To the Director-General

Dear Madam/Sir,

1	Pursuant to the provisions of Nos. 13.12 and 13.14 of the Radio Regulations, the Radio Regulations Board (RRB) at its 26th meeting held in Geneva (11-15 March 2002) approved new or modified Rules of Procedure and the suppression of some Rules.

2	Following the provisional entry into force of the 2001 edition of the Radio Regulations from 1 January 2002, the 1998 edition of the Rules of Procedure will be revised and reissued and will be published in both paper and electronic formats. The new edition will incorporate all revisions up to and including Revision No. 9. It will also incorporate the Rules of Procedure just approved at the 26th meeting of the RRB. Pending the availability of the new edition of the Rules of Procedure, the Rules noted below are included in the Annexes to this Circular Letter as indicated. All of the additions, modifications and suppressions of the Rules listed below take effect immediately.

2.1	A new Rule of Procedure relating to Nos. 5.523B, 5.523C, 5.523D and 5.523E (Annex 1).

2.2	Modifications to the Rules of Procedure relating to No. 5.351 (Annex 2).

2.3	Suppression of the Rules of Procedure relating to Nos. 5.487 and 5.490.

2.4	Modifications to the Rule of Procedure relating to No. 5.554 (Annex 3).

2.5	A new Rule of Procedure relating to the band 40.5-42.5 GHz in the Table of Frequency Allocations (Annex 4).

2.6	Suppression of the Rules of Procedure relating to Nos. 5.551B and 5.551E.

2.7	Modifications to the Rule of Procedure relating to No. 9.21 (Annex 5).

2.8	Modifications to the Rule of Procedure relating to No. 9.36 (Annex 6).

2.9	Modifications to the Rule of Procedure relating to No. 11.49 (Annex 7).

2.10	Additions to the Rules of Procedure relating to paragraphs 4.1.1 a) and 4.1.1 b) of Article 4 of Appendices 30 and 30A (Annex 8).

2.11	A new Rule of Procedure relating to paragraphs 4.1.3 and 4.2.6 of Article 4 of Appendices 30 and 30A (Annex 9).

2.12	Modifications to the Rules of Procedure relating to former paragraphs 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 of Appendix 30 (Annex 10).

2.13	Modifications to the Rules of Procedure relating to former paragraphs 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 of Appendix 30A (Annex 11).

2.14	Modifications to the Rules of Procedure relating to former paragraph 6.1 of Article 6 and former paragraph 7.6 of Article 7 of Appendix 30A (Annex 12).

2.15	A new Rule of Procedure relating to Sections 5 and 6 of Annex 1 to Appendix 30A (Annex 13).

2.16	A new temporary Rule of Procedure relating to Section 3.4 of Annex 5 of Appendix 30 (Annex 14).

2.17	A new Rule of Procedure relating to Resolution 49 (Rev.WRC-2000) in respect to its application to networks submitted under Appendices 30, 30A and 30B (Annex 15).

2.18	A new Rule of Procedure relating to Resolution 84 (WRC-2000) (Annex 16).

2.19	A new Rule of Procedure relating to Resolution 122 (Rev.WRC-2000) (Annex 17).

Yours faithfully,

Robert W. Jones
	Director, Radiocommunication Bureau

Annexes: 17

Distribution:

–	Administrations of Member States of ITU

–	Members of the Radio Regulations Board

RNS/mpc

annex 1

Rules of Procedure

ADD

5.523B, 5.523C, 5.523D, 5.523E

Footnotes 5.523B, 5.523C, 5.523D and 5.523E provide information on the different constraints and procedures applying to the fixed-satellite service (FSS) in the frequency range 19.3-19.7 GHz. The Board studied the interrelationship between the different FSS usages and also vis-à-vis the terrestrial stations. Tables 19.3-19.6 GHz and 19.6-19.7 GHz below contain the Board's conclusions on this matter.

Table 19.3-19.6 GHz

| CR request: Column vis-à-vis Row() | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (other) | GSO FSS  (CR 18.11.95) | GSO FSS  (18.11.95 CR) | Terrestrial |
| --- | --- | --- | --- | --- | --- | --- |
| Non-GSO FSS  (MSS feederlink) | 9.12 (5.523B) | 9.12 (5.523B/5.523D) | NO CR (5.523D) | 22.2 (5.523C) | 9.12A (5.523B) | --- (5.523B) |
| Earth station | --- | 9.17A | 9.17A | 9.17A | 9.17A | 9.15 |
| Non-GSO FSS  (MSS feederlink) | 9.12 (5.523B/ 5.523D) | 9.12 (5.523D) | NO CR (5.523D) | 22.2 (5.523D) | 9.12A (5.523D) | --- PFD limits (5.523D) |
| Earth station | 9.17A | --- | --- | --- | --- | 9.15 |
| Non-GSO FSS  (other) | NO CR (5.523D) | NO CR (5.523D) | NO CR (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | --- PFD limits (5.523D) |
| Earth station | 9.17A | --- | --- | --- | --- | 9.17 |
| GSO FSS  (CR 18.11.95) | 22.2 (5.523C) | 22.2 (5.523D) | 22.2 (5.523D) | 9.7 | --- | --- PFD limits |
| Earth station | 9.17A | --- | --- | --- | --- | 9.17 |
| GSO FSS  (18.11.95 CR) | 9.13 (5.523B) | 9.13 (5.523D) | 22.2 (5.523D) | 9.7 | 9.7 | --- PFD limits |
| Earth station | 9.17A | --- | --- | --- | --- | 9.17 |
| Terrestrial | --- | 9.16 (5.523D) | 9.18 (5.523D) | 9.18 | 9.18 | --- |

Table 19.6-19.7 GHz

| CR request: Column vis-à-vis Row () | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (other) | Non-GSO FSS  (MSS feederlink) | Non-GSO FSS  (other) | GSO FSS  (CR 21.11.97) | GSO FSS  (CR 21.11.97) | GSO FSS  (21.11.97 CR) | GSO FSS  (21.11.97 CR) | Terrestrial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-GSO FSS  (MSS feederlink) | 9.12 | 9.12 | 9.12 | NO CR | 22.2 (5.523E) | 22.2 (5.523E) | 9.12A (5.523D) | 9.12A (5.523D) | (5.523D) |
| Earth station | --- | --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.15 |
| Non-GSO FSS  (other) | 9.12 | 9.12 | 9.12 | NO CR | 22.2 (5.523D) | 22.2 (5.523D) | 9.12A (5.523D) | 9.12A (5.523D) | (5.523D) |
| Earth station | --- | --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.15 |
| Non-GSO FSS  (MSS feederlink) | 9.12 | 9.12 | 9.12 (5.523D) | NO CR (5.523D) | 22.2 (5.523E) | 22.2 (5.523E) | 9.12A (5.523D) | 9.12A (5.523D) | PFD limits (21/section V) (5.523D) |
| Earth station | 9.17A | 9.17A | --- | --- | 9.17A | --- | 9.17A | --- | 9.15 |
| Non-GSO FSS  (other) | NO CR | NO CR | NO CR (5.523D) | NO CR (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | 22.2 (5.523D) | PFD limits (21/section V) (5.523D) |
| Earth station | 9.17A | 9.17A | --- | --- | 9.17A | --- | 9.17A | --- | 9.17 |
| GSO FSS  (CR 21.11.97) | 22.2 (5.523E) | 22.2 (5.523D) | 22.2 (5.523E) | 22.2 (5.523D) | 9.7 | 9.7 | --- | --- | --- |
| Earth station | --- | --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.17 |
| GSO FSS  (CR 21.11.97) | 22.2 (5.523E) | 22.2 (5.523D) | 22.2 (5.523E) | 22.2 (5.523D) | 9.7 | 9.7 | --- | --- | PFD limits (21/section V) |
| Earth station | 9.17A | 9.17A | --- | --- | 9.17A | --- | --- | --- | 9.17 |
| GSO FSS  (21.11.97 CR) | 9.13 (5.523D) | 9.13 (5.523D) | 9.13 (5.523D) | 22.2 (5.523D) | 9.7 | 9.7 | 9.7 | 9.7 | --- |
| Earth station | --- | --- | 9.17A | 9.17A | --- | 9.17A | --- | 9.17A | 9.17 |
| GSO FSS  (21.11.97 CR) | 9.13 (5.523D) | 9.13 (5.523D) | 9.13 (5.523D) | 22.2 (5.523D) | 9.7 | 9.7 | 9.7 | 9.7 | PFD limits (21/section V) |
| Earth station | 9.17A | 9.17A | --- | --- | 9.17A | --- | 9.17A | --- | 9.17 |
| Terrestrial | --- | --- | 9.16 (5.523D) | 9.18 (5.523D) | --- | 9.18 | --- | 9.18 | --- |

ANNEX 2

Rules concerning

ARTICLE 5 OF THE RR

MOD

5.351

1	This provision permits, in derogation of the definitions contained in No. 1.76 and 1.82, the use of the bands allocated to a mobile-satellite service by a station at a specified fixed point (without being a coast earth station or an aeronautical earth station).

2	The exceptional circumstances referred to in this provision cannot be evaluated by the Bureau.

3	The Board therefore concluded that assignments notified under this provision shall receive a favourable regulatory finding.

ANNEX 3

Rule of Procedure on No. 5.554

MOD

5.554

This provision does not provide additional allocation for the fixed-satellite service in the frequency bands specified therein. It authorizes links between land stations at specified fixed points within the mobile-satellite or radionavigation-satellite service. Land station in the context of the latter two services means land earth station, which, in accordance with its definition, is a feeder link earth station. Therefore, a space or earth station in the fixed-satellite service (class of station EC or TC) is not authorized in the frequency bands listed in 5.554 (except in the band 123-130 GHz where there is FSS allocation) and links between specific (as opposed to typical) feeder-link earth stations (e.g. class of station VA, or TI, or the like) are authorized within the mobile-satellite or radionavigation-satellite service.

ANNEX 4

ADD

Rule of Procedure
Box 40.5-42.5 GHz of the Frequency Allocation Table

1	Before WRC-97 there was no FSS allocation in the band 40.5-42.5 GHz and there was secondary allocation to the Fixed Service (FS).

2	WRC-97 made FSS allocation in that band for Regions 2 and 3 and, under 5.551D, to certain countries in Region 1 and upgraded the FS allocation to primary. The FS allocation went into effect on 1.1.1999. For FSS it went into effect on 1.1.1999 for Region 2, while for Regions 1 and 3 it was postponed until 1.1.2001, subject to the review of this date by WRC-2000 (see 5.551D, 5.551E and Resolution 134 (WRC-97)). Also, to protect the radio astronomy (RA) in the neighbouring band (42.5-43.5 GHz), the implementation of FSS systems in the band 41.5-42.5 GHz was prohibited until technical and operational measures had been identified and agreed within ITU-R (see 5.551B and Resolution 128 (WRC-97)).

3	The complement of the above measures was intended to protect FS and RA until appropriate measures have been agreed.

4	WRC-2000 adopted hard pfd limits for FSS in the band 40.5-42.5 GHz and made them applicable from 3.6.2000 (see 59.6 (WRC-2000) and Resolution 59 (WRC-2000), resolves 6) to protect FS. To protect RA stations in the band 42.5-43.5 GHz, WRC-2000 adopted 5.551G, which contains hard pfd limits in that band for FSS networks operating in the band 41.5-42.5 GHz, and made these limits applicable from 3.6.2000 (see 59.6 (WRC-2000) and resolves and urges administrations 2 of Resolution 128 (WRC-2000)).

5	Having put in place measures for protection of FS and RA, WRC-2000 suppressed 5.551B, 5.551D, 5.551E and Resolution 134 (WRC-97) and modified Resolution 128. Suppression of footnotes took effect on 1.1.2002, while the suppression of Resolution 134 and modification of Resolution 128 took effect immediately after WRC-2000, i.e. on 3.6.2000.

6	In view of the above, the Board considered the effective date of FSS allocation in the band 40.5-42.5 GHz and the applicability of hard pfd limits to FSS and concluded as follows:

| Region | Allocation effective | Application of hard pfd limits of 5.551G and Table 21-4 |
| --- | --- | --- |
| Region 2 | From 1.1.1999 | To FSS networks for which coordination or notification request is received as from 3.6.2000 |
| Region 3 | From 3.6.2000 |  |
| Countries of Region 1 listed in 5.551D (WRC-97) | From 3.6.2000 to 31.12.2001 |  |
| Region 1 | From 1.1.2002 |  |

ANNEX 5

Modification of the Rule concerning No. 9.21

MOD

9.21

# 1	(NOC)

# 2	(Suppress paragraph 2)

# 3	(NOC)

# 4	(NOC)

(Renumber existing paragraphs 3 and 4 to paragraphs 2 and 3.)

ANNEX 6

Modification of the Rule concerning No.  9.36

MOD

9.36

1	Under this provision, the Bureau "shall identify any administrations with which coordination may need to be effected". In applying Appendix 5 with respect to No. 9.21, the Bureau uses the following calculation methods and criteria3:

–	space network vs. space network: Appendix 8;

–	earth station vs. terrestrial stations and vice versa, and earth station vs. other earth stations operating in the opposite direction of transmission: Appendix 7;

–	transmitting terrestrial stations vs. receiving space stations: criteria of Article 21;

- transmitting space stations vs. terrestrial services4:

•	pfd limits defined in Article 21 (where such limits are not applicable as hard limits to the service which is subject to No. 9.21); or

•	coordination threshold pfd values applicable to other services in the same frequency band (e.g. pfd values in Table 5-2 of Annex 1 to Appendix 5);

–	receiving space stations vs. transmitting terrestrial stations: frequency overlap within the visibility area of the satellite network;

–	between stations of terrestrial services in some specific frequency bands: Rules of Procedure B4, B5 and B6 as appropriate.

2, 3, 4 and 5 	(NOC)

Annex to the Rule of Procedure concerning No. 9.36

|  | Case 1 | Case 2 | # Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| In frequency band | F1 – F2 | F1 – F2 | F1 – F2 | F1 – F2 |
| Space service (A) under No. 9.21 (footnote No. 5.xxx refers) is | A | A | A | A |
| Another space service (B), not under No. 9.21, with which the same frequency band is shared is | - | B | B | B |
| Hard pfd limit (Article 21, a footnote or a Resolution refers) is applicable to space service | A | B | - | - |
| Coordination threshold pfd value is applicable (under e.g. No. 9.14) to space service | - | - | B | - |
| Agreement threshold pfd value used to identify, under No. 9.21, potentially affected administrations in respect of their terrestrial stations/services (NB: In Special Section CR/C and in the BR's database, this relation is indicated by the provision symbol 9.21/C, see Preface to the BR IFIC (space services), Table 11A.1, and also Attachment 1 to CR/172) | None No need for agreement in respect of terrestrial services is indicated in either the Special Section CR/C or BR's database. The hard pfd limit applicable to service A is considered to be there to protect terrestrial services from space service A. If that limit is observed (as checked under No. 9.35), the finding for the space service assignment is favourable, terrestrial services are protected and there is no need for an agreement under No. 9.21 in respect of terrestrial services. If the hard pfd limit is exceeded, the finding for the assignment is unfavourable and the agreement procedure is not applicable. | Hard pfd limit applicable to service B (4th indent of the Rule) If such pfd limit is good enough to protect terrestrial services from service B, it is good enough to protect them from service A, too. If such limit is not exceeded, an administration is not potentially affected with respect to 9.21/C. If that limit is exceeded, the finding for service A is still favourable (the limit is not a hard limit applicable to service A) and an administration on whose territory the limit is exceeded is considered as potentially affected with respect to 9.21/C. | Coordination threshold pfd value applicable to service B (4th indent of the Rule) If such pfd limit is good enough to indicate if terrestrial services are potentially affected or not from service B, it is good enough to indicate the same with respect to service A, too. | None (none exists) Frequency overlap with recorded terrestrial stations is used to indicate potentially affected administrations with respect to 9.21/C. Any administration, visible from the satellite, may disagree under No. 9.52 with respect to its terrestrial services. |

annex 7

Modification to the Rule of Procedure in respect of No. 11.49

MOD

11.49

# 1	Suspended assignments

1.1	Under the provisions of No. 11.49 the Board understands that an administration may suspend the use of a frequency assignment to a space station for a period not exceeding two years and still continue to enjoy the protection acquired by virtue of the coordination agreements already obtained. The Bureau may be informed of such suspensions either by the administration at its own initiative (No. 11.49) or in response to an inquiry made under No. 13.6.

1.2	The Board decided that the procedure described below shall apply. The procedure will only be valid for suspended assignments which are not modified before being brought back into use.

# 2	Recording of a suspension of use

2.1	When the Bureau is informed, either under No. 11.49 or in response to an inquiry under No. 13.6, that the use of a frequency assignment to a space station recorded in the Master Register is suspended, this information is published in Part II of the BR IFIC (in order to inform all administrations) and the entry in the MIFR will be amended to include the date of resumption of use indicated by the notifying administration.

2.2	Frequency assignments to space stations whose suspension is notified for a period of not more than 2 years will continue to be taken into account for the purposes of the examination of other assignments in accordance with Nos. 9.36, 11.31.1, 11.32, 11.32A and 11.33 until the time that the consultation concerning their resumption of use is completed (see 2.4 below).

2.3	Frequency assignments to space stations whose suspension is notified for a period of more than 2 years will not be taken into account for the purpose of examinations of other assignments under Nos. 9.36, 11.31.1, 11.32-11.32A and 11.33 as of the date of such notification or after confirmation from the administration of the period of suspension exceeding 2 years and shall be cancelled.

2.4	At the expiry of the period of suspension of the use of a frequency, the notifying administration is consulted as to the date of resumption of use. According to the results of the consultation, the Bureau will apply the following procedures:

2.4.1	When the administration confirms that the use has been resumed at the originally indicated date (not later than two years after the date of suspension) or before, this information is published in Part II of the BR IFIC and the MIFR is modified to indicate this situation.

2.4.2	When the administration notifies that the use will be resumed at a date later than two years after the date of suspension, the assignment shall be cancelled according to the provisions Nos. 11.49 and 13.6. For those stations for which the use may be resumed later than the 2-year period, the administration responsible for the assignment shall apply again the relevant coordination procedure of Article 9.

Annex 8

Additional sub-paragraphs to the Rules of Procedure 
relating to § 4.1.1 a) and § 4.1.1 b) of Article 4 of 
Appendices 30 and 30A (WRC-2000)

## Additional sub-paragraphs to the Rules of Procedure relating to § 4.1.1 a) and § 4.1.1 b) of Article 4 of Appendix 30A (WRC-2000)

4.1.1 a) and 4.1.1 b)

ADD (at the end)

In case of a request for replacement of an assignment/entry of an administration in the Region 2 Plan, the implementation of § 2.2 of the Rules of Procedure relating to § 4.2.6 of Appendix 30A requires the processing of the modified assignment requested by that administration under Article 4 of that Appendix based on the following conditions:

–	no interference effect from the initial assignment of the requesting administration is considered in the OEPM calculations of the modified assignment requested by that administration, and vice-versa; and

–	no aggregate interference effect from the subject initial assignment and the modified assignment of the requesting administration is considered in the OEPM calculations of other assignments, but only the worst interference effect from the two is to be considered.

The above-mentioned conditions apply only during the time-period afforded for the processing of the modified assignments under Article 4 of that Appendix. After that time-period, either the subject initial assignment or the modified assignment of the requesting administration will remain in the Plan, depending on the successful application or otherwise of the Article 4 procedure for this modified assignment.

A similar situation would occur in the case of the Regions 1 and 3 Plans and Lists for the implementation of § 4.1.27 of Appendix 30A.

## Additional sub-paragraphs to the Rules of Procedure relating to § 4.1.1 a) and § 4.1.1 b) of Article 4 of Appendix 30 (WRC-2000)

4.1.1 a) and 4.1.1 b)

ADD (at the end)

In the case of a request for replacement of an assignment/entry in the Region 2 Plan of an administration, the implementation of § 2.2 of the Rules of Procedure relating to § 4.2.6 of Appendix 30 requires the processing of the modified assignment requested by that administration under Article 4 of that Appendix based on the following conditions:

–	no interference effect from the initial assignment of the requesting administration is considered in the OEPM calculations of the modified assignment requested by that administration, and vice-versa; and

–	no aggregate interference effect from the subject initial assignment and the modified assignment of the requesting administration is considered in the OEPM calculations of other assignments, but only the worst interference effect from the two is to be considered.

The above-mentioned conditions apply only during the time-period afforded for the processing of the modified assignment under Article 4 of that Appendix. After that time-period, either the subject initial assignment or the modified assignment of the requesting administration will remain in the Plan, depending on the successful application or otherwise of the Article 4 procedure for this modified assignment.

A similar situation would occur in case of the Regions 1 and 3 Plan and List for the implementation of § 4.1.27 of Appendix 30.

ANNEX 9

Rules of Procedure relating to § 4.1.3 and § 4.2.6 of Article 4 of 
Appendices 30 and 30A (WRC-2000) of the RR

4.1.3 and 4.2.6

ADD

5	WRC-2000 included in § 4.1.3 of Article 4 of Appendices 30 and 30A a footnote on the fact that "the provisions of Resolution 533 (Rev.WRC-2000) apply". Considering that resolves 3 or 4 of that Resolution apply to all Regions, the Board concluded that the extension periods referred to in resolves 3.2 or 4.2 of that Resolution are applicable under § 4.1.3 and § 4.2.6 of Article 4 of Appendices 30 and 30A, although the footnote mentioned above is not referred to in § 4.2.6 of Article 4 of Appendices 30 and 30A.

ANNEX 10

Modification to the current Rules of Procedure relating to former 
§§ 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 of 
Appendix S30 (WRC-97) to align its text with 
the decisions adopted at WRC-2000

Art. 5

Notification, examination and recording

MOD

5.2.1 b)

1	The Board has considered the question whether the examination with respect to conformity with the Plan3 means only the columns of Articles 10 and 11 of Appendix 30, as updated or whether it also includes an examination with respect to the technical criteria given in Annex 5 to Appendix 30 which were used for the establishment of the Plans. The Board concluded that some of the technical criteria contained in Annex 5 need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the Plan is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the Plan concerned as updated (see § 3.1 of Article 3). If the characteristics are different then the examination under § 5.2.1 d) is carried out. For the items below, any characteristics for which the procedure of Article 4 has been successfully applied could be notified.

b)	to ensure that the protection criteria resulting from the Region 2 Plan3, or from the Regions 1 and 3 Plan and List, as appropriate, are not exceeded. To this effect, the following characteristics are examined:

–	beam identification (as indicated in columns 1 and 2 of Articles 10 and 11, respectively of Appendix 30);

–	nominal orbital position (as indicated in columns 2 and 3 of Articles 10 and 11, respectively of Appendix 30);

–	channel number/frequency (as indicated in column 3 of Article 10 and in column 5 of the table entitled "Minimum equivalent protection margin of assignments in the Regions 1 and 3 Plan (sorted by orbital position)" of Article 11 of Appendix 30);

–	boresight coordinates (as indicated in column 4 of Articles 10 and 11 of Appendix 30);

–	in the case of an elliptical beam:

–	antenna beamwidth (as indicated in column 5 of Articles 10 and 11 of Appendix 30),

–	ellipse orientation (as indicated in columns 6 and 5 of Articles 10 and 11, respectively of Appendix 30),

–	antenna rotational accuracy (same as or better than that of § 3.14 of Annex 5 to Appendix 30);

–	polarization (as indicated in columns 7 and 10 of Articles 10 and 11, respectively of Appendix 30);

–	power plus co-polar antenna gain (as indicated in columns 8 and 11 of Articles 10 and 11, respectively of Appendix 30), and in the case of shaped beam the cross-polar antenna gain (as indicated in column 8 of Article 11 of Appendix 30);

–	service area (test points shall be located within the service area);

–	class of emission and bandwidth (as indicated in column 12 of Article 11 of Appendix 30 in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 and 3.8 of Annex 5 to Appendix 30);

–	antenna characteristics (same as or better than those indicated in columns 6 or 7 as appropriate of Article 11 of Appendix 30 in the case of the Regions 1 and 3 Plan, or otherwise same as or better than Fig. 9 or 10 as appropriate of Annex 5 to Appendix 30);

–	antenna pointing accuracy (same as or better than that referred to in § 3.14 of Annex 5 to Appendix 30);

–	station keeping tolerance (same as or better than that mentioned in § 3.11 of Annex 5 to Appendix 30);

–	modulation characteristics (same as in column 12 of Article 11 of Appendix 30 in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 of Annex 5 to Appendix 30);

–	energy dispersal (same as § 3.18 of Annex 5 to Appendix 30);

–	the power flux-density identified in Note 10 of the Region 2 Plan or in Note 5 of the Regions 1 and 3 Plan, to determine whether the limits are met or whether there is an agreement with the affected administrations.

2	The Board has considered the question whether the examination with respect to conformity with the Regions 1 and 3 List means only the columns of the Tables in Part I of Annex 2 to Resolution 542 (WRC-2000), as updated, or whether it also includes an examination with respect to the technical characteristics published by the Bureau for each network of the List in the corresponding Part B Special Section of the Weekly Circular or the IFIC. The Board concluded that all technical characteristics published in the Part B Special Section for a given network need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the List is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the List, as updated, and those specified in the Part B Special Section of a given network. If the characteristics are different then the examination under § 5.2.1 d) is carried out.

b)	to ensure that the protection criteria resulting from the Regions 1 and 3 Plan and List are not exceeded. To this effect, the characteristics specified in the columns of the List, as updated, and those specified in the Part B Special Section of a given network are examined.

MOD

5.2.2.1

This paragraph implicitly relates to the cases where the Bureau reaches a favourable Finding with respect to § 5.2.1 a) and § 5.2.1 c) and an unfavourable Finding with respect to § 5.2.1 b) but a favourable Finding with respect to § 5.2.1 d). In this event, the frequency assignment shall be recorded in the Master Register.

MOD

5.2.2.2

Part of this paragraph deals with interim systems which are submitted in application of Resolution 42 (Rev.Orb-88) for Region 2.

In case of Regions 1 and 3, should the Bureau reach a favourable Finding with respect to §§ 5.2.1 a) and 5.2.1 c) but an unfavourable Finding with respect to §§ 5.2.1 b) and 5.2.1 d), the assignments in question shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this Finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

MOD

5.3.1

1	See item 2 of the Rules of Procedure relating to § 4.2.6 of Article 4 of Appendix 30.

2	For any notification other than that relating to the modification to the Region 2 Plan, the date of bringing into use of assignments can be extended at the request of the notifying administration.

Nevertheless, the total period for the implementation (bringing into use) of the assignments shall be limited to 8 years, counted from the date of receipt of the complete information of Appendix 4 or former Annex 2 to Appendix 30, plus any applicable extension in accordance with resolves 3.2 or 4.2 of Resolution 533 (Rev.WRC-2000).

See also the Rules of Procedure relating to §§ 4.1.3 and 4.2.6 of Article 4 of Appendix 30.

ANNEX 11

Modification to the current Rules of Procedure relating to
former §§ 5.2.1 b), 5.2.2.1, 5.2.2.2 and 5.3.1 of Article 5 to 
Appendix S30A (WRC-97) to align its text with 
the decisions adopted at WRC-2000

Art. 5

Notification, examination and recording

MOD

5.2.1 b)

1	The Board has considered the question whether the examination with respect to conformity with the Plan2 means only the columns of Articles 9 and 9A of Appendix 30A, as updated or whether it also includes an examination with respect to the technical criteria given in Annex 3 to Appendix 30A which were used for the establishment of the Plans. The Board concluded that some of the technical criteria contained in Annex 3 need to be taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the Plan is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the Plan concerned as updated (see § 3.1 of Article 3). If the characteristics are different then the examination under § 5.2.1 d) is carried out. For the items below, any characteristics for which the procedure of Article 4 has been successfully applied could be notified.

b)	to ensure that the protection criteria resulting from the Region 2 Plan2, or from the Regions 1 and 3 Plan(s) and List(s), as appropriate, are not exceeded. To this effect, the following characteristics of the satellite network are examined:

i)	For a receiving space station:

–	space station beam identification (as indicated in columns 1 and 2 of Articles 9 and 9A, respectively of Appendix 30A);

–	nominal orbital position (as indicated in columns 2 and 3 of Articles 9 and 9A, respectively of Appendix 30A);

–	channel number/frequency (as indicated in column 3 of Article 9 and in column 5 of the tables entitled "Minimum equivalent protection margin in the Regions 1 and 3 feeder-link Plan in the frequency band 14.5-14.8 GHz (sorted by orbital position)" and "Minimum equivalent protection margin in the Regions 1 and 3 feeder-link Plan in the frequency band 17.3-18.1 GHz (sorted by orbital position)" of Article 9A of Appendix 30A);

–	boresight coordinates (as indicated in column 4 Articles 9 and 9A of Appendix 30A);

–	in the case of elliptical beam:

–	antenna beamwidth (as indicated in column 5 of Articles 9 and 9A of Appendix 30A);

–	ellipse orientation (as indicated in columns 6 and 5 of Articles 9 and 9A, respectively of Appendix 30A);

–	antenna rotational accuracy (same as or better than § 3.7.4 (Regions 1 and 3) or 4.6.4 (Region 2) of Annex 3 to Appendix 30A);

–	polarization (as indicated in columns 7 and 10 of Articles 9 and 9A, respectively of Appendix 30A);

–	service area (test points shall be located within the service area);

–	class of emission and bandwidth (as indicated in column 13 of Article 9A in the case of Regions 1 and 3 Plan of Appendix 30A, or otherwise as indicated in § 3.1 and 3.8 of Annex 5 to Appendix 30);

–	antenna characteristics (same as or better than those indicated in columns 6 or 7 as appropriate of Article 9A of Appendix 30A in the case of the Regions 1 and 3 Plan, or otherwise same as or better than § 4.6 of Annex 3 to Appendix 30A);

–	antenna pointing accuracy (same as or better than § 3.7.4 (Regions 1 and 3) or § 4.6.4 (Region 2) of Annex 3 to Appendix 30A);

–	system noise temperature (see Note 7 of Article 9A and § 3.8 of Annex 3 to Appendix 30A as appropriate for Regions 1 and 3, and § 4.7 of the same Annex for Region 2);

–	station keeping tolerance (same as or better than that of § 3.16 of Annex 3 to Appendix 30A);

–	modulation characteristics (same as in column 13 of Article 9A of Appendix 30A in the case of the Regions 1 and 3 Plan, or otherwise as indicated in § 3.1 of Annex 5 to Appendix 30);

–	range of automatic gain control (same as § 3.10 of Annex 3 to Appendix 30A for Regions 1 and 3, and 4.9 of the same Annex for Region 2).

ii)	For an associated transmitting earth station:

–	e.i.r.p.: Columns 8 and 11 of Articles 9 and 9A, respectively of Appendix 30A;

–	antenna diameter: § 3.5.1 or 4.4.1 of Annex 3 to Appendix 30A;

–	reference patterns: Fig. 6 or Fig. A of Annex 3 to Appendix 30A (as indicated in Column 9 of Article 9A of Appendix 30A for the Regions 1 and 3 Plan);

–	transmit power: § 3.6 or 4.5 of Annex 3 to Appendix 30A;

–	the location of the associated earth station to be associated with test points within the service area;

–	energy dispersal (same as § 3.18 of Annex 5 to Appendix 30).

In relation to the transmitting power, the Board noted that according to §§ 3.11 and 4.10 of Annex 3 to Appendix 30A, the use of power control shall remain within the limits indicated in those paragraphs.

2	The Board has considered the question whether the examination with respect to conformity with the Regions 1 and 3 Lists means only the columns of the Tables in Part II of Annex 2 to Resolution 542 (WRC-2000), as updated, or whether it also includes an examination with respect to the technical characteristics published by the Bureau for each network of the Lists in the corresponding Part B Special Section of the Weekly Circular or the IFIC. The Board concluded that all technical characteristics published in the Part B Special Section for a given network need to be

taken into account in this examination. Therefore, the examination from the viewpoint of conformity with the Lists is carried out in two steps:

a)	to ensure that the characteristics notified are those specified in the columns of the List concerned, as updated, and those specified in the Part B Special Section of a given network. If the characteristics are different then the examination under § 5.2.1 d) is carried out.

b)	to ensure that the protection criteria resulting from the Regions 1 and 3 Plan and List concerned are not exceeded. To this effect, the characteristics specified in the columns of the List concerned, as updated, and those specified in the Part B Special Section of a given network are examined.

3	See also the Rules of Procedure relating to the scope of application of Article 5 of Appendix 30A.

MOD

5.2.2.1

This paragraph implicitly relates to the cases where the Bureau reaches a favourable finding with respect to § 5.2.1 a), § 5.2.1 c) and § 5.2.1 f) and an unfavourable finding with respect to § 5.2.1 b) but a favourable finding with respect to § 5.2.1 d).

However, considering the Rules of Procedure relating to the scope of application of Article 5 of Appendix 30A [and to § 5.1.3 of that Article], the Board concluded that § 5.2.2.1 relates to the cases where the Bureau reaches a favourable finding with respect to § 5.2.1 a) and § 5.2.1 c) and an unfavourable finding with respect to § 5.2.1 b) but a favourable finding with respect to § 5.2.1 d).

In this event the frequency assignment shall be recorded in the Master Register.

MOD

5.2.2.2

Part of this paragraph deals with interim systems which are submitted in application of Resolution 42 (Rev.Orb-88) for Region 2.

In case of Regions 1 and 3, should the Bureau reach a favourable finding with respect to § 5.2.1 a) and § 5.2.1 c) but an unfavourable finding with respect to § 5.2.1 b) and § 5.2.1 d), the assignments in question shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this finding and with such suggestions as the Bureau may be able to offer with a view to a satisfactory solution of the problem.

MOD

5.3.1

1	See § 2 of the Rules of Procedure relating to § 4.2.6 of Article 4 of Appendix 30A.

2	For any notification other than that relating to the modification to the Region 2 Plan, the date of bringing into use of assignments can be extended at the request of the notifying administration.

Nevertheless, the total period for the implementation (bringing into use) of the assignments shall be limited to 8 years, counted from the date of receipt of the complete information of Appendix 4 or former Annex 2 to Appendix 30A, plus any applicable extension in accordance with resolves 3.2 or 4.2 of Resolution 533 (Rev.WRC-2000).

See also the Rules of Procedure relating to §§ 4.1.3 and 4.2.6 of Article 4 of Appendix 30A.

ANNEX 12

Modification to the current Rules of Procedure relating to
former § 6.1 of Article 6, and former § 7.6 of Article 7 to 
Appendix S30A (WRC-97)

Art. 6

Coordination, notification and recording of receiving terrestrial
assignments when FSS feeder-links are involved

MOD

6.1

1	The paragraphs of Article 6 do not mention interim systems implemented in accordance with Resolution 42 (Rev.Orb-88). Such systems may be implemented in the frequency band 17.7-17.8 GHz for Region 2 shared with equal rights with terrestrial services.

Such usage may affect terrestrial stations.

2	This paragraph refers to "a feeder-link earth station located on the territory of another administration and included in the service area of an assignment to a broadcasting-satellite service feeder-link space station which is in conformity with the appropriate regional feeder-link Plan". This earth station is to be considered a typical earth station located at the worst location.

3	In order to evaluate the interference, an Administration A, intending to use terrestrial stations, needs to know the fixed-earth station existing or planned. In order to take them into account administrations may calculate the coordination area as indicated in § 1.4.6 of Appendix 7 around the service area mentioned in § 6.1.

Art. 7

Coordination, notification and recording of FSS assignments
when feeder-links to BSS assignments are involved

MOD

7.7

The comments under § 6.5 apply.

annex 13

Rules of Procedure relating to Sections 5 and 6 of Annex 1 
to Appendix 30A (WRC-2000)

An. 1

Limits for determining whether a service of an administration is affected by 
proposed modifications to the Region 2 Plan or by proposed new or 
modified assignments to the Regions 1 and 3 feeder-link Lists

5

The Board considered the reference to "total RF bandwidth of the feeder-link carriers (24 MHz for Region 2 and 27 MHz for Regions 1 and 3)" contained in the last two lines of the first paragraph of Section 5 of Annex 1 to Appendix 30A (WRC-2000). Considering that many proposed modifications to the Region 2 Plan or proposed new or modified assignments to the Regions 1 and 3 feeder-link Lists include feeder-link carriers with necessary bandwidth different from 24 MHz for Region 2 or 27 MHz for Regions 1 and 3, the Board concluded that the Bureau shall use those specified bandwidths in the implementation of Section 5 of Annex 1 to Appendix 30A (WRC-2000). In addition, the Board concluded that the same specified bandwidths shall be used by administrations when submitting information under data item No. C.8.i of Annex 2A to Appendix 4. The Board further noted that the maximum power density per hertz averaged over the total RF bandwidth corresponds generally to the maximum power density per hertz averaged over the necessary bandwidth of the proposed feeder-link carrier.

annex 14

Temporary Rule of Procedure relating to Section 3.4 of Annex 5 
of Appendix 30 (WRC-2000)

Annex 5

Technical data used in establishing the provisions and associated Plans and the Regions 1 and 3 List, which should be used for their application

3.4

1	The Board notes that the WRC-2000 Final Acts include a modification to Section 3.4 of Annex 5 to Appendix 30 (WRC-2000). However, that modification omits the last three paragraphs from the previous Section 3.4. These three paragraphs specify the protection ratios for Region 2, which are not mentioned elsewhere in that Appendix.

2	Accordingly, until the matter can be addressed by a future WRC, the Board agrees that the following protection ratios, previously prescribed, should continue to apply as follows:

"In Region 2, the following protection ratios have been adopted for the purpose of calculating the overall equivalent protection margin32bis:

28 dB for co-channel signals;

13.6 dB for adjacent-channel signals;

–9.9 dB for second adjacent-channel signals.

In Region 2, as a guide for planning, the reduction in the overall C/I ratio due to co-channel interference in the feeder link is taken as equivalent to a degradation in the downlink co-channel C/I ratio of approximately 0.5 dB not exceeded for 99% of the worst month, but the feeder-link and downlink Plans are evaluated on the basis of the overall equivalent protection margin, which includes the combined downlink and feeder-link contributions.

In Region 2, an overall equivalent protection margin of 0 dB, or greater, indicates that the individual protection ratios have been met for the co-channel, the adjacent channels and the second adjacent channels."

ANNEX 15

Rules of Procedure relating to Resolution 49 (Rev.WRC-2000) 
with respect to its application for satellite networks submitted 
under Appendices 30, 30A and 30B of the RR

RESOLUTION 49 (Rev.WRC-2000)

Administrative due diligence applicable to some satellite 
radiocommunication services

1	The Board considered the references to "Annex 2 of Appendix S30B/30B" in resolves 1 and in § 3 of Annex 1 of that Resolution. In accordance with the Rules of Procedure adopted on this issue at its 20th meeting (Geneva, 11-15 September 2000), the Board concluded that these references shall be understood as references to Appendix S4 of the Radio Regulations.

2	The Board considered the references to "relevant provisions of Article 4 of Appendix 30, Article 4 of Appendix 30A" in resolves 2 and to "relevant provisions of Article 4 of Appendix S30/30 and the relevant provisions of Article 4 of Appendix S30A/ 30A" in Section 5 of Annex 1 of that Resolution. Considering the adoption at WRC-2000 of new Article 4 of Appendices 30 and 30A (WRC-2000) and of Resolution 533 (Rev.WRC-2000), the Board concluded that these references shall be understood as references to § 4.1.3 (Regions 1 and 3) or § 4.2.6 (Region 2) of Article 4 of Appendices S30 and S30A (WRC-2000), and that the extension period referred to in resolves 3.2 or 4.2 of Resolution 533 (Rev.WRC-2000) shall be taken into account in the implementation of Resolution 49 (Rev.WRC-2000). See also the Rules of Procedure relating to § 4.1.3 and § 4.2.6 of Article 4 of Appendices 30 and 30A (WRC-2000).

ANNEX 16

ADD

Rules concerning

RESOLUTION 84 (WRC-2000)

Power flux-density limits in the band 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service

resolves 2

The Board considered resolves 2 of Resolution 84 (WRC-2000) and, to assist administrations and the Bureau in applying it, decided to instruct the Bureau to act as follows with respect to GSO FSS networks using the band 37.5-40 GHz in any Region:

1)	At coordination stage (Article 9), for coordination requests submitted as from 3.6.2000, and for coordination requests submitted before 3.6.2000 but not followed by request for notification before that date, to establish and publish in a coordination special section the list of administrations in Region 2 on whose territory the power flux-density produced exceeds the values in Table 21-4 minus 12 dB;

2)	At notification stage (Article 11), for notification requests received as from 3.6.2000, to examine whether the required agreements (explicit), as established in 1) above, are properly reflected in the forms of notice and to establish the finding under 11.32 accordingly;

3)	For requests for assistance received from administrations in case of difficulties to obtain the required agreement from other administrations (identified as affected) to apply the corresponding/analogous provisions of Article 9 (Nos. 9.60 to 9.65).

Reason: 	The Board noted that, in the frequency band 37.5-40 GHz, resolves 2 of Resolution 84 (WRC-2000) provides, in the interim period before WRC-03, a mechanism for protection of terrestrial services in Region 2 from GSO FSS networks in Region 2. In view of the worldwide allocation of the band 37.5-40 GHz to GSO FSS, the Board considers that the intention of resolves 2 is to protect terrestrial services in Region 2 from GSO FSS use in any Region.

Also, the Board noted that there is no procedure associated with seeking, obtaining and verifying that the required agreements have indeed been obtained. To assist the administrations and the Bureau in applying resolves 2 of Resolution 84, the Board considers that it is appropriate to take an approach similar to the application of Resolution 77 as contained in the Rule of Procedure on 5.488.

ANNEX 17

Rules concerning

RESOLUTION 122 (Rev.WRC-2000)

# Use of the bands 47.2-47.5 GHz and 47.9-48.2 GHz by high altitude platform stations (HAPS) in the fixed service and by other services and the potential use of bands 
in the range 18-32 GHz by HAPS in the fixed service

Resolves and instructs

1	Resolves 2 of Resolution 122 (Rev.WRC-2000) stipulates that, on a provisional basis, the procedures of Article 9 shall be used for coordination between satellite systems and systems using HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz.

2	Having regard to the ongoing studies undertaken by the ITU-R study groups on appropriate criteria and calculation methods as well as the applicable Article 9 provisions involving satellite systems and HAPS (refer Resolution 122 (Rev.WRC-2000)), the Board concluded that, the Bureau shall proceed as follows:

2.1	To examine those assignments to satellite systems and HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz only with respect to No. 9.35/11.31 and to defer the examination of such systems under No. 9.36/11.32 as well as the application of any Article 9 coordination procedure between the satellite systems and HAPS until further decision by WRC-03.

2.2	For notification when the examination with respect to No. 11.31 leads to a favourable finding, the assignments shall be entered provisionally in the Master Register with the appropriate symbol in Column 13B3 (Finding Action);

2.3	To report to WRC-03 the list of involved assignments either at the coordination stage or recorded in the MIFR, as appropriate, for further consideration.

__________

3	 	For cases not covered under this paragraph, the Bureau, in collaboration with the appropriate Radiocommunication study groups, continues to develop applicable calculation methods and criteria in the form of Rules of Procedure to be submitted to the RRB for approval.

4	 	Cases relevant to this indent are shown in the Annex to this Rule.

3	 	Anytime the "Plan" is referred to, this means the current version of the Plan as updated on the date of the Bureau's examination in the case of the Region 2 Plan, and, in the case of the Regions 1 and 3 Plan, the current version of the Plan as may be updated pursuant to the possible application of § 4.1.26 or § 4.1.27 of Appendix 30.

2	 	Anytime the "Plan" is referred to, this means the current version of the Plan as updated on the date of the Bureau's examination in the case of the Region 2 Plan, and, in the case of the Regions 1 and 3 Plan(s), the current version of the Plan(s) as may be updated pursuant to the possible application of § 4.1.26 or § 4.1.27 of Article 4 of Appendix 30A.

3	2bis 	The definitions in §§ 1.7, 1.8, 1.9, 1.10 and 1.11 of this Annex apply to these calculations.