| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 38 to Document 13-E |
|  |  |  | 13 March 2003 |
|  |  |  | Original: English/ French/ Spanish |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 38 |  |

Agenda item 1.38 – EESS (active) at 420-470 MHz

Introduction

According to Resolution 727, the United Nations Conference on Environment and Development (UNCED) (Rio de Janeiro, 1992) identified an urgent need for assessment and systematic observations of forest cover and rate of forest degradation in tropical and temperate regions and during WRC-97, many countries agreed to the principle that ITU should take action in response to the need identified by UNCED.

ITU-R studies have confirmed that the preferable frequency band within the range 420 to 470 MHz is the band 432-438 MHz that offers the most favourable sharing scenario. Studies leading to WRC-03 have shown that sharing between EESS (active) and existing services in the band 432-438 MHz is possible given certain characteristics and operational limitations on the EESS (active) instruments. These characteristics and operational limitations have been introduced in DRR ITU-R SA.1260 adopted by ITU-R Study Group 7 in February 2003.

Therefore, the following changes are proposed to the Radio Regulations in order to accommodate a new secondary allocation to the EESS (active) in the band 432-438 MHz, which will satisfy the agenda item.

Proposals

ARTICLE  5

Frequency allocations

MOD	EUR/13A38/1

410-470 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 430-432 AMATEUR RADIOLOCATION | 430-432 RADIOLOCATION Amateur |  |
| 5.271 5.272 5.273 5.274 5.275 5.276 5.277 | 5.271 5.276 5.277 5.278 5.279 |  |

MOD	EUR/13A38/2

| 432-438 AMATEUR RADIOLOCATION Earth exploration-satellite (active) ADD 5.XXX | 432-438 RADIOLOCATION Amateur Earth exploration-satellite (active) ADD 5.XXX |
| --- | --- |
| 5.138 5.271 5.272 5.276 5.277 5.280 5.281 5.282 | 5.271 5.276 5.277 5.278 5.279 5.281 5.282 |

MOD	EUR/13A38/3

| 438-440 AMATEUR RADIOLOCATION | 438-440 RADIOLOCATION Amateur |
| --- | --- |
| 5.271 5.273 5.274 5.275 5.276 5.277 5.283 | 5.271 5.276 5.277 5.278 5.279 |

ADD	EUR/13A38/4

5.XXX	The use of this band by sensors in the Earth exploration-satellite service (active) shall be in accordance with [the draft revision of] Recommendation ITU-R SA.1260.

____________