| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 167(Rev.1)-E |
|  |  |  | 19 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 5 |
| --- | --- |
| Working Group 5C |  |
| Agenda item 1.32 |  |

Working Group 5C has reached the following agreement on Resolution 84 issues of agenda item 1.32 “to consider technical and regulatory provisions concerning the band 37.5-43.5 GHz, in accordance with Resolutions 128 (Rev.WRC-2000) and 84 (WRC-2000)”.

ARTICLE  5

SUP	WG5C/167/1

5.551AA

MOD	WG5C/167/2

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |

MOD	WG5C/167/3

40.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551G |  |  |

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

MOD	WG5C/167/4

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| ... |  |  |  |  |  |  |
| 37.5-40 GHz | Fixed-satellite (non-geostationary-satellite orbit) Mobile-satellite (non-geostationary-satellite orbit) | –120 10, 19 | –120 + 0.75( – 5) 10, 19 |  | –105 10, 19 | 1 MHz |
| 37.5-40 GHz | Fixed-satellite (geostationary-satellite orbit) Mobile-satellite (geostationary-satellite orbit) | –127 19 | 5°-20° | 20°-25° | –105 19 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) 19 | –107 + 0.4 ( – 20) 19 |  |  |
| 40-40.5 GHz | Fixed-satellite | –115 | –115 + 0.5( – 5) |  | –105 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –115 10, 19 | –115 + 0.5( – 5) 10, 19 |  | –105 10, 19 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –120 19 | 5°-15° | 15°-25° | –105 19 | 1 MHz |
|  |  |  | –120 + ( – 5) 19 | –110 + 0.5 ( – 15) 19 |  |  |

MOD

TABLE  21-4 (end)

| SUP WG5C/167/5 Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –120 10, 19 | –120 + 0.75( – 5) 10, 19 |  | –105 10, 19 | 1 MHz |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –127 19 | 5°-20° | 20°-25° | –105 19 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) 19 | –107 + 0.4 ( – 20) 19 |  |  |

_______________

16	21.16.11

SUP	WG5C/167/6

_______________

17	21.16.12

SUP	WG5C/167/7

_______________

18	21.16.13

ADD	WG5C/167/8

_______________

19	21.16.14	When addressing the sharing conditions between the fixed service and the fixed-satellite service in the bands 37.5-40 GHz and 40.5-42.5 GHz, the power flux-density at the Earth’s surface from any FSS satellite should be no greater than the level(s) required to meet the FSS link availability and performance objectives of the subject applications, taking into account the technical and operational requirements of the overall satellite network design. In any case, the levels shall not exceed the applicable power flux-density limits in Table 21-4.

SUP	WG5C/167/9

RESOLUTION  84  (WRC-2000)

Power flux-density limits in the bands 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service

_____________