| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 194-E |
|  |  |  | 17 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 5 |
| --- | --- |
| second report from working group 5e to committee 5 |  |
| on agenda item 1.12 |  |
| Resolution 723, resolves 4 for 25.5-27 GHz |  |

The following report on agenda item 1.12, Resolution 723 (Rev.WRC-2000), resolves 4 for 25.5-27 GHz is presented to Committee 5 for consideration.

J. ZUZEK

Chairman, Working Group 5E, Box 507

ARTICLE  5

MOD	COM5/194/1

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 25.5-27 EARTH EXPLORATION-SATELLITE (space-to Earth) MOD 5.536A 5.536B FIXED INTER-SATELLITE 5.536 MOBILE SPACE RESEARCH (space-to-Earth) MOD 5.536A ADD 5.BE01 Standard frequency and time signal-satellite (Earth-to-space) |  |  |

MOD	COM5/194/2

5.536A	Administrations operating earth stations in the Earth exploration-satellite or space research services shall not claim protection from stations in the fixed and mobile services operated by other administrations. In addition, earth stations operating in the Earth exploration-satellite service or in the space research service should take into account Recommendations ITU-R SA.1278 and ITU-R SA.1625.     (WRC-03)

ADD	COM5/194/3

5.BE01	In Algeria, Saudi Arabia, Bahrain, Botswana, Brazil, Comoros, Cuba, United Arab Emirates, Estonia, Finland, Iran (Islamic Rep. of), Italy, Jordan, Kenya, Morocco, Nigeria, Qatar, Syrian Arab Republic, Sudan, Tanzania, Tunisia and Zambia, earth stations operating in the space research service in the band 25.5-27 GHz shall not claim protection from, or constrain the use and deployment of, stations of the fixed and mobile services.     (WRC-03)

ARTICLE 21

MOD	COM5/194/4

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 19.3-19.7 GHz 22.55-23.55 GHz 24.45-24.75 GHz 25.25-27.5 GHz | Fixed-satellite (space-to-Earth) Earth exploration-satellite (space-to-Earth) Inter-satellite Space research (space-to-Earth) | –115 | –115 + 0.5( – 5) | –105 | 1 MHz |

____________