| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 207-E |
|  |  |  | 18 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| Third series of texts submitted by Committee 5 to the Editorial Committee |  |

The following text on agenda item 1.15 (Resolution 604 (WRC-2000)) is proposed for consideration by Committee 8.

A. JAMIESON
	Chairman, Committee 5, Box 233

ARTICLE  5

MOD	COM5/207/1

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 000-5 010 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (Earth-to-space) 5.367 |  |  |

MOD	COM5/207/2

5 010-5 030 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) MOD 5.443B 5.367

MOD	COM5/207/3

5 030-5 150 AERONAUTICAL RADIONAVIGATION 5.367 5.444 5.444A

SUP	5.443A		COM5/207/4

5.443A

MOD	5.443B		COM5/207/5

5.443B	-In order not to cause harmful interference to the microwave landing system operating above 5 030 MHz, the aggregate power flux-density produced at the Earth’s surface in the band 5 030-5 150 MHz by all the space stations within any radionavigation-satellite service system (space-to-Earth) operating in the band 5 010-5 030 MHz shall not exceed –124.5 dB(W/m2) in a 150 kHz band. In order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, --radionavigation-satellite service systems operating in the band 5 010-5 030 MHz shall comply with the limits in the band 4 990-5 000 MHz defined in Resolution [COM5/1] (WRC-03).     (WRC-03)

ADD	RES. [COM5/1]	COM5/207/6

RESOLUTION  [COM5/1]  (WRC-03)

Protection of the radio astronomy service in the band 4 990-5 000 MHz from 
unwanted emissions of the radionavigation-satellite service (space-to-Earth) 
operating in the frequency band 5 010-5 030 MHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that unwanted emissions from space stations of the radionavigation-satellite service (RNSS) operating in the frequency band 5 010-5 030 MHz may cause interference to the radio astronomy service (RAS) in the band 4 990-5 000 MHz;

b)	that WRC-2000 decided to introduce a provisional power flux-density (pfd) limit in the band 4 990-5 000 MHz to protect the RAS, and invited ITU-R to conduct studies to review this limit;

c)	that protection requirements for the RAS are given in Recommendations ITU-R RA.769 and ITU-R RA.1513, and are different for GSO and non-GSO satellite systems,

noting

a)	that Recommendation ITU-R M.1583 provides a methodology based on the equivalent pfd (epfd) concept for calculation of interference resulting from unwanted emissions from non-GSO systems of the mobile-satellite or radionavigation-satellite services into radio astronomy stations;

b)	that Recommendation ITU-R RA.1631 provides antenna patterns and maximum antenna gain to be used for compatibility analyses between non-GSO systems and RAS stations based on the epfd concept;

c)	that Recommendation ITU-R RA.1513 recommends acceptable levels of data loss to radio astronomy observations, stating in particular that the percentage of data loss caused by any system should be lower than 2%,

resolves

1	that in order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, the power flux-density produced in this band by any GSO RNSS network operating in the 5 010-5 030 MHz band shall not exceed –171 dB(W/m2) in a 10 MHz band at any radio astronomy station;

2	that in order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, over the whole sky, for elevations higher than the operating elevation angle min1 specified for the radio telescope, the equivalent power flux-density (epfd) produced in this band by all space stations within any non-GSO RNSS system operating in the 5 010-5 030 MHz band shall not exceed –245 dB(W/m2) in a 10 MHz band at any radio astronomy station for more than 2% of the time, using the methodology in Recommendation ITU-R M.1583 and a reference antenna with the antenna patterns and the maximum antenna gain given in Recommendation ITU-R RA.1631;

3	that the limits referred to in resolves 1 and 2 shall apply to RNSS systems as of 3 June 2000;

4	that administrations planning to operate a GSO or a non-GSO RNSS system in the band 5 010-5 030 MHz, for which complete coordination or notification information, as appropriate, has been received by the Bureau after 2 June 2000, shall send to the Radiocommunication Bureau the value of the maximum level of pfd as referred to in resolves 1 or the value of the maximum level of epfd as referred to in resolves 2, as appropriate,

instructs the Radiocommunication Bureau

as of the end of WRC-03, to review all RNSS systems for which complete coordination or notification information, as appropriate, has been received by the Bureau before the end of WRC-03 for the band 5 010-5 030 MHz, and, if appropriate, to revise its findings regarding compliance with No. 5.443B, taking into account additional information received under resolves 4.

APPENDIX  4  (WRC-03)

ANNEX  2A

## A.17	Compliance with aggregate power flux-density limits

MOD	A.17 a)		COM5/207/7

a)	For systems operating in the radionavigation-satellite service in the band 5 010-5 030 MHz, the calculated aggregate power flux-density produced at the Earth’s surface by all space stations within any radionavigation-satellite service system in the band 5 030-5 150 MHz in a 150 kHz bandwidth, as defined in No. 5.443B.

ADD	A.17 aa)	COM5/207/8

aa)	For geostationary-satellite systems operating in the radionavigation-satellite service in the band 5 010-5 030 MHz, the calculated aggregate power flux-density produced at the Earth’s surface by any radionavigation-satellite service network in the band 4 990-5 000 MHz in a 10 MHz bandwidth, as defined in resolves 1 of Resolution [COM5/1] (WRC-03).

ADD	A.17bis 	COM5/207/9

## A.17bis	Compliance with equivalent power flux-density (epfd) limits

For non-geostationary-satellite systems operating in the radionavigation-satellite service in the band 5 010-5 030 MHz, the equivalent power flux-density produced by all space stations within any radionavigation-satellite service system in the band 4 990-5 000 MHz in a 10 MHz bandwidth as defined in resolves 2 of Resolution [COM5/1] (WRC-03).

SUP	RES. 604	COM5/207/10

RESOLUTION  604  (WRC-2000)

Studies on compatibility between the radionavigation-satellite service 
(space-to-Earth) operating in the frequency band 5 010-5 030 MHz
and the radio astronomy service operating
in the band 4 990-5 000 MHz

____________

1	 	Until adoption of a definition of min by ITU-R, and publication of notified radio astronomy observatory data, a value of 5° should be assumed in appropriate calculations.