| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 209-E |
|  |  |  | 18 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| fourth series of texts submitted by committee 5 to the editorial committee |  |

The attached text contains the decisions relating to Documents 194, 195 and 196 on agenda item 1.12.

ARTICLE  5

MOD	COM5/209/1

5 830-7 550 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 7 075-7 145 FIXED MOBILE 5.458 5.459 |  |  |
| 7 145-7 235 FIXED MOBILE SPACE RESEARCH (Earth-to-space) MOD 5.460 5.458 5.459 |  |  |
| 7 235-7 250 FIXED MOBILE 5.458 |  |  |

MOD	COM5/209/2

5.460	The use of the band 7 145-7 190 MHz by the space research service (Earth-to-space) is restricted to deep space; no emissions to deep space shall be effected in the band 7 190-7 235 MHz. Geostationary satellites in the space research service operating in the band 7 190-7 235 MHz shall not claim protection from existing and future stations of the fixed and mobile services and No. 5.43A does not apply.

ARTICLE  21

MOD	COM5/209/3

TABLE  21-2     (WRC-03)

| Frequency band | Service | Limit as specified in Nos. |
| --- | --- | --- |
| ... 7 145-7 235 MHz ... | Space research | 21.3 and 21.5 |

ARTICLE  21

MOD	COM5/209/4

TABLE  21-3

| ARTICLE 5 Frequency band |  | Services |
| --- | --- | --- |
| ... 5 850-7 075 MHz 7 190-7 235 MHz 7 900-8 400 MHz ... |  | Fixed-satellite Earth-exploration-satellite Meteorological-satellite Mobile-satellite Space operation |
|  |  | Space research |

MOD	COM5/209/5

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 25.5-27 EARTH EXPLORATION-SATELLITE (space-to Earth) MOD 5.536A 5.536B FIXED INTER-SATELLITE 5.536 MOBILE SPACE RESEARCH (space-to-Earth) MOD 5.536A ADD 5.BE01 Standard frequency and time signal-satellite (Earth-to-space) |  |  |

MOD	COM5/209/6

5.536A	Administrations operating earth stations in the Earth exploration-satellite or space research services shall not claim protection from stations in the fixed and mobile services operated by other administrations. In addition, earth stations operating in the Earth exploration-satellite service or in the space research service should take into account Recommendations ITU-R SA.1278 and ITU-R SA.1625, respectively.     (WRC-03)

ADD	COM5/209/7

5.BE01	In Algeria, Saudi Arabia, Bahrain, Botswana, Brazil, Comoros, Cuba, Egypt, United Arab Emirates, Estonia, Finland, Iran (Islamic Rep. of), Israel, Italy, Jordan, Kenya, Kuwait, Lithuania, Morocco, Nigeria, Oman, Qatar, Syrian Arab Republic, Somalia, Sudan, Tanzania, Tunisia, Uruguay, Zambia and Zimbabwe, earth stations operating in the space research service in the band 25.5-27 GHz shall not claim protection from, or constrain the use and deployment of, stations of the fixed and mobile services.     (WRC-03)

ARTICLE 21

MOD	COM5/209/8

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 19.3-19.7 GHz 22.55-23.55 GHz 24.45-24.75 GHz 25.25-27.5 GHz | Fixed-satellite (space-to-Earth) Earth exploration-satellite (space-to-Earth) Inter-satellite Space research (space-to-Earth) | –115 | –115 + 0.5( – 5) | –105 | 1 MHz |

ARTICLE  5

MOD	COM5/209/9

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 35.5-36 METEOROLOGICAL AIDS EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) 5.549 ADD 5.BE02 |  |  |

SUP	COM5/209/10

5.551A

ADD	COM5/209/11

5.BE02	In the band 35.5-36.0 GHz, the mean power flux-density generated by any Earth exploration-satellite service (active)/space research service (active) spaceborne sensor at the Earth’s surface for any angle greater than 0.8º from the beam centre shall not exceed –73.3 dB(W/m2) across this band.

NOC	COM5/209/12

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 36-37 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) 5.149 |  |  |

NOC	COM5/209/13

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 37-37.5 FIXED MOBILE SPACE RESEARCH (space-to-Earth) 5.547 |  |  |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) 5.551AA MOBILE SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |

ADD	COM5/209/14

Resolution  [COM 5/3]  (WRC-03)

Use of the frequency band 36-37 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the frequency band 36-37 GHz is allocated to the Earth exploration-satellite service (EESS) (passive) and to the space research service (passive) on a primary basis;

b)	that the frequency band 36-37 GHz is allocated to the fixed service and to the mobile service on a primary basis;

c)	that the EESS (passive) protection criteria are contained in Recommendation ITU-R SA.1029;

d)	that Recommendation ITU-R F.758 provides characteristics of fixed service point-to-multipoint systems operating in the band 36-37 GHz, but does not provide information on characteristics of fixed service point-to-point systems operating in this band;

e)	that the band 36-37 GHz is not available for high-density applications in the fixed service (see No. 5.547);

f)	that the EESS (passive) operating in the band 36-37 GHz may be interfered by the emissions of systems of active services,

recognizing

a)	that EESS (passive) systems may experience harmful interference if a high density of fixed-service stations is deployed in the band 36-37 GHz;

b)	that sharing criteria between EESS (passive) and fixed-service systems need to be defined in the band 36-37 GHz,

resolves

1	to invite ITU-R to conduct sharing studies between passive services and the fixed and mobile services in the band 36-37 GHz in order to define appropriate sharing criteria;

2	to recommend that a future competent conference review the results of the studies and consider the possible inclusion of the sharing criteria within the Radio Regulations,

invites administrations

1	to provide characteristics of active systems (fixed and mobile services) operating in the band 36-37 GHz;

2	to take into account that EESS (passive) systems may experience harmful interference if the band 36-37 GHz is heavily used by the fixed or mobile-service stations prior to the establishment of the sharing criteria.

SUP	COM5/209/15

RESOLUTION  730  (WRC-2000)

Use of the frequency band 35.5-35.6 GHz by 
spaceborne precipitation radars

____________