| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 214-E |
|  |  |  | 18 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 5 |
| --- | --- |
| FIRST REPORT FROM WORKING GROUP 5D TO COMMITTEE 5 |  |
| ON AGENDA ITEM 1.5 |  |

RESOLUTION 736, RESOLVES 2

The following report on agenda item 1.5, Resolution 736, resolves 2, is presented to Committee 5 for consideration.

A. HASHIMOTO
				Chairman, Working Group 5D, Box 5

ARTICLE  5

MOD

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D 5.448 5.448A ADD 5.BD01 |  |  |
| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) 5.448 5.448A ADD 5.BD01 |  |  |

ADD

5.BD01	Additional allocation: The band 5 250-5 350 MHz is also allocated to the fixed service on a primary basis in the following countries in Region 3: Australia, India, Japan, Papua New Guinea, Philippines and Viet Nam. The use of this band by the fixed service is included for the implementation of fixed wireless access (FWA) systems and shall comply with Recommendation ITU-R F.1613. In addition, the fixed service shall not claim protection from the radiodetermination, Earth exploration-satellite (active) and space research (active) services, although the provisions of No. 5.43A do not apply to the fixed service with respect to the Earth exploration-satellite (active) and space research (active) services. After implementation of FWA systems in the fixed service with protection for the existing radiodetermination systems, no more stringent constraints should be imposed on the FWA systems by future radiodetermination implementations.

____________