| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 219-E |
|  |  |  | 19 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 4 |
| --- | --- |
| Chairman, Working Group 4C |  |
| Sixth report of WG 4C to committee 4 on agenda item 1.14 |  |

The following texts are submitted to Committee 4:

–	Number 52.221A

–	Draft Resolution [COM4/3] (WRC-03)

–	Appendix 17

–	Draft Resolution [COM4/2] (WRC-03)

–	Resolution 207 (Rev.WRC-03)

A. ASHMAN
	Chairman, Working Group 4C, Box 139

# 1	Modification to Article 52 of the Radio Regulations

MOD

52.221A		2)	Calling on the carrier frequencies 12 290 kHz and 16 420 kHz shall be permitted only to and from rescue coordination centres (see No. 30.6.1), subject to the safeguards of Resolution [COM4/3] (WRC-03). The alternative carrier frequencies 12 359 kHz and 16 537 kHz may be used by ship stations and coast stations for calling on a simplex basis, provided that the peak envelope power does not exceed 1 kW.     (WRC-03)

# 2	Draft new Resolution on the use of the frequencies 12 290 kHz and 16 420 kHz

DRAFT  Resolution  [COM4/3]  (WRC-03)

Use of the carrier frequencies 12 290 kHz and 16 420 kHz for limited 
safety-related calling to and from rescue coordination centres

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that this Conference modified No. 52.221A to allow limited safety-related calling on the carrier frequencies 12 290 kHz and 16 420 kHz;

b)	that the limited safety-related calling function on these carrier frequencies will enhance the capability of those search and rescue organizations which maintain watch on these distress and safety frequencies to call ships not fitted for the GMDSS,

noting

a)	that regulation IV/4.8 of the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended, requires that SOLAS ships, while at sea, be capable of transmitting and receiving general radiocommunications to and from shore-based radio systems or networks;

b)	that general communications may consist of safety-related communications necessary for the safe operation of shipping,

further noting

that safety-related communications require adequate, effective and immediate access and protection,

recognizing

a)	that the International Maritime Organization notes that distress, urgency and safety radiocommunications include, but are not limited to:

•	transmissions of maritime safety information;

•	distress calls and traffic;

•	acknowledgment and relaying of distress calls;

•	search and rescue coordination communications;

•	ship movement service communications;

•	communications related to the safe operation of ships;

•	communications related to navigation;

•	meteorological warnings;

•	meteorological observations;

•	ship position reports; and

•	medical emergencies (e.g. MEDICO/MEDIVAC);

b)	that distress, urgency and safety communications are defined in Articles 32 and 33 of the Radio Regulations,

resolves

1	that the carrier frequencies 12 290 kHz and 16 420 kHz be used only for distress, urgency and safety communications, and limited safety-related calling;

2	that safety-related calling be initiated only after determination that other communications are not present on these frequencies;

3	that safety-related calling be minimized and not cause interference to distress, urgency and safety communications,

urges administrations

to encourage the coast and ship stations under their jurisdiction to use digital selective calling techniques,

instructs the Secretary-General

to bring this Resolution to the attention of the International Maritime Organization.

MOD

APPENDIX  17  (WRC-03)

Frequencies and channelling arrangements in the
high-frequency bands for the maritime mobile service

(See Article 52)

PART  A  –  Table of subdivided bands

In the Table, where appropriate1, the assignable frequencies in a given band for each usage are:

–	indicated by the lowest and highest frequency, in heavy type, assigned in that band;

–	regularly spaced, the number of assignable frequencies (f.) and the spacing in kHz being indicated in italics.

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 063 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for oceanographic data transmission c) | 4 063.3 to 4 064.8 6 f. 0.3 kHz |  |  |  |  |  |  |  |
| Limits (kHz) | 4 065 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for telephony, duplex operation a) i) | 4 066.4 to 4 144.4 27 f. 3 kHz | 6 201.4 to 6 222.4 8 f. 3 kHz | 8 196.4 to 8 292.4 33 f. 3 kHz | 12 231.4 to 12 351.4 41 f. 3 kHz | 16 361.4 to 16 526.4 56 f. 3 kHz | 18 781.4 to 18 823.4 15 f. 3 kHz | 22 001.4 to 22 157.4 53 f. 3 kHz | 25 071.4 to 25 098.4 10 f. 3 kHz |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |
| Frequencies assignable to ship stations and coast stations for telephony, simplex operation a) | 4 147.4 to 4 150.4 2 f. 3 kHz | 6 225.4 to 6 231.4 3 f. 3 kHz | 8 295.4 to 8 298.4 2 f. 3 kHz | 12 354.4 to 12 366.4 5 f. 3 kHz | 16 529.4 to 16 547.4 7 f. 3 kHz | 18 826.4 to 18 844.4 7 f. 3 kHz | 22 160.4 to 22 178.4 7 f. 3 kHz | 25 101.4 to 25 119.4 7 f. 3 kHz |
| Limits (kHz) | 4 152 | 6 233 | 8 300 | 12 368 | 16 549 | 18 846 | 22 180 | 25 121 |
| Frequencies assignable to ship stations for wide-band telegraphy, facsimile and special transmission systems | 4 154 to 4 170 5 f. 4 kHz | 6 235 to 6 259 7 f. 4 kHz | 8 302 to 8 338 10 f. 4 kHz | 12 370 to 12 418 13 f. 4 kHz | 16 551 to 16 615 17 f. 4 kHz | 18 848 to 18 868 6 f. 4 kHz | 22 182 to 22 238 15 f. 4 kHz | 25 123 to 25 159 10 f. 4 kHz |
| Limits (kHz) | 4 172 | 6 261 | 8 340 | 12 420 | 16 617 | 18 870 | 22 240 | 25 161.25 |
| Frequencies assignable to ship stations for oceanographic data transmission c) |  | 6 261.3 to 6 262.5 5 f. 0.3 kHz | 8 340.3 to 8 341.5 5 f. 0.3 kHz | 12 420.3 to 12 421.5 5 f. 0.3 kHz | 16 617.3 to 16 618.5 5 f. 0.3 kHz |  | 22 240.3 to 22 241.5 5 f. 0.3 kHz |  |
| Limits (kHz) | 4 172 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) j) m) p) | 4 172.5 to 4 181.5 18 f. 0.5 kHz | 6 263 to 6 275.5 25 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 181.75 | 6 275.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 280.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) m) p) |  | 6 281 to 6 284.5 8 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 284.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 186.75 | 6 284.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) h) p) | 4 187 to 4 202 31 f. 0.5 kHz | 6 285 to 6 300 31 f. 0.5 kHz | 8 342 to 8 365.5 48 f. 0.5 kHz | 12 422 to 12 476.5 110 f. 0.5 kHz | 16 619 to 16 683 129 f. 0.5 kHz |  | 22 242 to 22 279 75 f. 0.5 kHz | 25 161.5 to 25 171 20 f. 0.5 kHz |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 365.75 | 12 476.75 | 16 683.25 | 18 870 | 22 279.25 | 25 171.25 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 370.75 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Working frequencies assignable to ship stations for A1A or A1B Morse telegraphy e) f) p) |  |  | 8 371 to 8 376 11 f. 0.5 kHz |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 376.25 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) m) p) |  |  | 8 376.5 to 8 396 40 f. 0.5 kHz | 12 477 to 12 549.5 146 f. 0.5 kHz | 16 683.5 to 16 733.5 101 f. 0.5 kHz | 18 870.5 to 18 892.5 45 f. 0.5 kHz | 22 284.5 to 22 351.5 135 f. 0.5 kHz | 25 173 to 25 192.5 40 f. 0.5 kHz |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 549.75 | 16 733.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Calling frequencies assignable to ship stations for A1A or A1B Morse telegraphy g) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 554.75 | 16 738.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) m) p) |  |  |  | 12 555 to 12 559.5 10 f. 0.5 kHz | 16 739 to 16 784.5 92 f. 0.5 kHz |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (non paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK and for A1A or A1B Morse telegraphy (working) b) p) | 4 202.5 to 4 207 10 f. 0.5 kHz | 6 300.5 to 6 311.5 23 f. 0.5 kHz | 8 396.5 to 8 414 36 f. 0.5 kHz | 12 560 to 12 576.5 34 f. 0.5 kHz | 16 785 to 16 804 39 f. 0.5 kHz | 18 893 to 18 898 11 f. 0.5 kHz | 22 352 to 22 374 45 f. 0.5 kHz | 25 193 to 25 208 31 f. 0.5 kHz |
| Limits (kHz) | 4 207.25 | 6 311.75 | 8 414.25 | 12 576.75 | 16 804.25 | 18 898.25 | 22 374.25 | 25 208.25 |
| Frequencies assignable to ship stations for digital selective calling k) l) | 4 207.5 to 4 209 4 f. 0.5 kHz | 6 312 to 6 313.5 4 f. 0.5 kHz | 8 414.5 to 8 416 4 f. 0.5 kHz | 12 577 to 12 578.5 4 f. 0.5 kHz | 16 804.5 to 16 806 4 f. 0.5 kHz | 18 898.5 to 18 899.5 3 f. 0.5 kHz | 22 374.5 to 22 375.5 3 f. 0.5 kHz | 25 208.5 to 25 209.5 3 f. 0.5 kHz |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 18 899.75 | 22 375.75 | 25 210 |
| Limits (kHz) | 4 209.25 | 6 313.75 | 8 416.25 | 12 578.75 | 16 806.25 | 19 680.25 | 22 375.75 | 26 100.25 |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) n) o) p) | 4 209.5 to 4 219 20 f. 0.5 kHz | 6 314 to 6 330.5 34 f. 0.5 kHz | 8 416.5 to 8 436 40 f. 0.5 kHz | 12 579 to 12 656.5 156 f. 0.5 kHz | 16 806.5 to 16 902.5 193 f. 0.5 kHz | 19 680.5 to 19 703 46 f. 0.5 kHz | 22 376 to 22 443.5 136 f. 0.5 kHz | 26 100.5 to 26 120.5 41 f. 0.5 kHz |
| Limits (kHz) | 4 219.25 | 6 330.75 | 8 436.25 | 12 656.75 | 16 902.75 | 19 703.25 | 22 443.75 | 26 120.75 |
| Frequencies assignable to coast stations for digital selective calling l) | 4 219.5 to 4 220.5 3 f. 0.5 kHz | 6 331 to 6 332 3 f. 0.5 kHz | 8 436.5 to 8 437.5 3 f. 0.5 kHz | 12 657 to 12 658 3 f. 0.5 kHz | 16 903 to 16 904 3 f. 0.5 kHz | 19 703.5 to 19 704.5 3 f. 0.5 kHz | 22 444 to 22 445 3 f. 0.5 kHz | 26 121 to 26 122 3 f. 0.5 kHz |
| Limits (kHz) | 4 221 | 6 332.5 | 8 438 | 12 658.5 | 16 904.5 | 19 705 | 22 445.5 | 26 122.5 |
| Frequencies assignable to coast stations for wide-band and A1A or A1B Morse telegraphy, facsimile, special and data transmission systems and direct-printing telegraphy systems |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (end)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |
| Frequencies assignable to coast stations for telephony, duplex operation a) | 4 352.4 to 4 436.4 29 f. 3 kHz | 6 502.4 to 6 523.4 8 f. 3 kHz | 8 708.4 to 8 813.4 36 f. 3 kHz | 13 078.4 to 13 198.4 41 f. 3 kHz | 17 243.4 to 17 408.4 56 f. 3 kHz | 19 756.4 to 19 798.4 15 f. 3 kHz | 22 697.4 to 22 853.4 53 f. 3 kHz | 26 146.4 to 26 173.4 10 f. 3 kHz |
| Limits (kHz) | 4 438 | 6 525 | 8 815 | 13 200 | 17 410 | 19 800 | 22 855 | 26 175 |

a)	See Part B, Section I.

b)	See Part B, Section III.

c)	The frequency bands may also be used by buoy stations for oceanographic data transmission and by stations interrogating these buoys.

d)	See Part B, Section II.

e)	In the frequency bands to be used by ship stations for A1A Morse telegraphy working at speeds not exceeding 40 Bd, administrations may assign additional frequencies interleaved between the assignable frequencies. Any frequencies so assigned shall be multiples of 100 Hz. Administrations shall ensure a uniform distribution of such assignments within the bands.

f)	See Part B, Section V.

g)	See Part B, Section IV.

h)	For the conditions of use of the frequency 8 364 kHz, see Appendix 13.

i)	For the use of the carrier frequencies 4 125 kHz, 6 215 kHz, 8 291 kHz, 12 290 kHz and 16 420 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by single-sideband radiotelephony, see Article 31 and Appendix 13.

j)	For the use of the frequencies 4 177.5 kHz, 6 268 kHz, 8 376.5 kHz, 12 520 kHz and 16 695 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by NBDP telegraphy, see Article 31.

k)	For the use of the frequencies 4 207.5 kHz, 6 312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by digital selective calling, see Article 31.

l)	The following paired frequencies (for ship/coast stations) 4 208/4 219.5 kHz, 6 312.5/6 331 kHz, 8 415/ 8 436.5 kHz, 12 577.5/12 657 kHz, 16 805/16 903 kHz, 18 898.5/19 703.5 kHz, 22 374.5/22 444 kHz and 25 208.5/26 121 kHz are the first choice international frequencies for digital selective calling (see Article 54).

m)	Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy (working) (see Part B, Section II).

n)	The frequencies 4 210 kHz, 6 314 kHz, 8 416.5 kHz, 12 579 kHz, 16 806.5 kHz, 19 680.5 kHz, 22 376 kHz and 26 100.5 kHz are the exclusive international frequencies for the transmission of Maritime Safety Information (MSI) (see Articles 31 and 33).

o)	The frequency 4 209.5 kHz is an exclusive international frequency for the transmission of NAVTEX type information (see Articles 31 and 33).

p)	These sub-bands, except the frequency bands referred to in Notes j), n) and o), may be used for the initial testing and the possible future introduction within the maritime mobile service of new digital technologies. Stations using these sub-bands for this purpose shall not cause harmful interference to, and shall not claim protection from, other stations operating in accordance with Article 5.

PART  B  –  Channelling arrangements

Section I  –  Radiotelephony

Paragraphs 1 to 7 are dealt with in separate documents.

SUP	Paragraph 8

ADD

DRAFT  RESOLUTION  [COM4/2]  (WRC-03)

Review of the frequency and channel arrangements in the maritime MF and 
HF bands with a view to improving efficiency by considering the use 
of new digital technology by the maritime mobile service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the agenda of this Conference included consideration of the use of new digital technology in the MF and HF bands in the maritime mobile service;

b)	that the introduction of new digital technology in the maritime mobile service shall not disrupt the distress and safety communications in the MF and HF bands including those established by the International Convention for the Safety of Life at Sea, 1974, as amended;

c)	that changes made in Appendix 17 should not prejudice the future use of these frequencies or the capabilities of systems or new applications required for use by the maritime mobile service;

d)	that the requirement to use new digital technologies in the maritime mobile service is growing rapidly;

e)	that the use of new digital technology on maritime HF and MF frequencies will make it possible to better respond to the emerging demand for new services;

f)	that the maritime HF A1A Morse telegraphy and narrow-band direct-printing frequency bands are significantly under-utilized at present;

g)	that the ITU Radiocommunication Sector is conducting ongoing studies to improve the efficiency in these bands,

noting

a)	Resolution 347 (WRC-97);

b)	that different digital technologies have already been developed and used in the MF and HF bands in several radio services,

noting also

that this conference has modified Appendix 17 to permit, on a voluntary basis, the use of various channels or bands identified in the MF and HF bands for initial testing and future introduction of new digital technology,

resolves

1	that, in order to provide full worldwide interoperability of equipment on ships, there should be one technology, or more than one interoperable worldwide technology, implemented under Appendix 17;

2	that, as soon as the ITU-R studies are completed, a future competent conference should consider necessary changes to Appendix 17 to enable the use of new technology by the maritime mobile service,

invites ITU-R

to finalize studies currently ongoing:

–	to identify future requirements of the maritime mobile service;

–	to identify the technical characteristics necessary to facilitate use of digital systems in the MF and HF maritime mobile bands taking into account any relevant ITU-R Recommendations;

–	to identify the digital system(s) to be used by the MF/HF maritime service;

–	to identify any necessary modifications to the frequency plan contained within Appendix 17;

–	to propose a timetable for the introduction of new digital technology and any consequential changes to Appendix 17;

–	to recommend how digital technology can be introduced while ensuring compliance with the distress and safety requirements,

instructs the Secretary-General

to bring this Resolution to the attention of the International Maritime Organization, the International Civil Aviation Organization, the International Association of Marine Aids to Navigation and Lighthouse Authorities and the International Maritime Radio Association.

MOD

RESOLUTION  207 (Rev.WRC-03)

Measures to address unauthorized use of and interference to frequencies in the bands allocated to the maritime mobile service and 
to the aeronautical mobile (R) service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the HF frequencies currently used by the aeronautical and maritime mobile services for distress, safety and other communications, including allotted operational frequencies, suffer from harmful interference and are often subject to difficult propagation conditions;

b)	that WRC-97 considered some aspects of the use of the HF bands for distress and safety communications in the context of the Global Maritime Distress and Safety System (GMDSS), especially with regard to regulatory measures;

c)	that unauthorized operations using maritime and aeronautical frequencies in the HF bands are continuing to increase and are already a serious risk to HF distress, safety and other communications;

d)	that some administrations have resorted to, for example, transmitting warning messages on operational HF channels as a means of deterring unauthorized users;

e)	that provisions of the Radio Regulations prohibit the unauthorized use of certain safety frequencies for communications other than those related to safety;

f)	that enforcing compliance with these regulatory provisions is becoming increasingly difficult with the availability of low-cost HF single side-band (SSB) transceivers;

g)	that monitoring observations of the use of frequencies in the band 2 170-2 194 kHz and in the bands allocated exclusively to the maritime mobile service between 4 063 kHz and 27 500 kHz and to the aeronautical mobile (R) service between 2 850 kHz and 22 000 kHz show that a number of frequencies in these bands are still being used by stations of other services, many of which are operating in contravention of No. 23.2;

h)	that, in certain situations, HF radio is the sole means of communication for the maritime mobile service and that certain frequencies in the bands mentioned in considering g) are reserved for distress and safety purposes;

i)	that, in certain situations, HF radio is the sole means of communication for the aeronautical mobile (R) service and that this is a safety service;

j)	that WRC-2000 and this Conference have reviewed the use of the HF bands by the aeronautical mobile (R) and maritime mobile services with a view to protecting operational, distress and safety communications;

k)	that this Resolution identifies several interference mitigation techniques that can be employed by administrations on a non-mandatory basis,

considering in particular

a)	that it is of paramount importance that the distress and safety channels of the maritime mobile service be kept free from harmful interference, since they are essential for the protection of the safety of life and property;

b)	that it is also of paramount importance that channels directly concerned with the safe and regular conduct of aircraft operations be kept free from harmful interference, since they are essential for the safety of life and property,

resolves to invite ITU-R and ITU-D, as appropriate

to increase regional awareness of appropriate practices in order to help mitigate interference in the HF bands, especially on distress and safety channels,

urges administrations

1	to ensure that stations of services other than the maritime mobile service abstain from using frequencies in distress and safety channels and their guardbands and in the bands allocated exclusively to that service, except under the conditions expressly specified in Nos. 4.4, 5.128, 5.129, 5.137 and 4.13 to 4.15; and to ensure that stations of services other than the aeronautical mobile (R) service abstain from using frequencies allocated to that service except under the conditions expressly specified in Nos. 4.4 and 4.13;

2	to make every effort to identify and locate the source of any unauthorized emission capable of endangering human life or property and the safe and regular conduct of aircraft operations, and to communicate their findings to the Radiocommunication Bureau;

3	to participate in any monitoring programmes organized by the Radiocommunication Bureau or administrations, if so agreed among those administrations, without adversely affecting the rights of other administrations or conflicting with any provisions of the Radio Regulations, in accordance with item 4 in the Annex;

4	to make every effort to prevent unauthorized transmissions in bands allocated to the maritime mobile service and the aeronautical mobile (R) service;

5	to request their competent authorities to take, within their respective jurisdiction, such legislative or regulatory measures which they consider necessary or appropriate in order to prevent stations from unauthorized use of distress and safety channels or from operating in contravention of No. 23.2;

6	to take all necessary steps in such cases of contravention of No. 23.2 to ensure the cessation of any transmissions contravening the provisions of the Radio Regulations on the frequencies or in the bands referred to in this Resolution;

7	to employ as many of the interference mitigation techniques in the Annex as are appropriate for the maritime mobile and aeronautical mobile (R) services,

instructs the Radiocommunication Bureau

1	to seek the cooperation of administrations in identifying the sources of those emissions by all available means and in securing the cessation of those emissions;

2	when the station of another service transmitting in a band allocated to the maritime mobile service or to the aeronautical mobile (R) service has been identified, to inform the administration concerned;

3	to include the problem of interference to maritime and aeronautical distress and safety channels on the agenda of relevant regional radiocommunication seminars,

instructs the Secretary-General

to bring this Resolution to the attention of the International Maritime Organization and the International Civil Aviation Organization for such actions as they may consider appropriate.

Annex  to  Resolution  207  (Rev.WRC-03)

Interference mitigation techniques

This Annex lists several possible HF interference mitigation techniques that may be used to protect receivers either in combination or singly, depending on the resources of administrations. Use of any or all of these techniques is not mandatory.

# 1	Alternative modulation methods

The use of digitally modulated emissions, such as QPSK, to replace or supplement analogue SSB voice (J3E) and data (J2B) emissions. This initiative would need to be adopted internationally to allow the interoperability of equipment. For example, ICAO has adopted the HF data-link standard to provide packet data communications using automated link establishment and adaptive frequency control techniques as a supplement to analogue SSB voice communications (see ICAO Convention Annex 10).

# 2	Passive and dynamic/adaptive antenna systems

Use of passive and active/adaptive antenna systems to reject unwanted signals.

# 3	Channel barring

Administrations should ensure through their licensing, equipment standardization and inspection arrangements that, in compliance with No. 43.1, HF radio equipment cannot transmit on frequencies exclusively assigned to the aeronautical mobile (R) service, as detailed in Appendix 27, except for frequencies allocated for worldwide use and shared with the aeronautical mobile (OR) service (see Appendix 26/3.4).

# 4	Regional HF monitoring and direction-finding facilities

Collaboration and cooperation between regional administrations to coordinate the use of monitoring and direction-finding facilities.

# 5	Transmission of warning messages

Transmission of multilanguage warning messages on specific channels affected by strong or persistent interference. Such transmissions should be conducted after coordination with the users of the affected services and the administration(s) or competent authorities concerned.

# 6	Education and publicity initiatives

Administrations provide education and publicity initiatives on the proper use of the radio-frequency spectrum in these bands.

____________

1 	Within the non-shaded boxes.