| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 236-E |
|  |  |  | 23 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 5 |
| --- | --- |
| Report from ad hoc group 5 (1.28) to Committee 5 on agenda item 1.28 |  |

The following text on agenda item 1.28 is proposed for consideration by Committee 5.

ARTICLE  5

Frequency allocations

MOD	COM5/236/1

75.2-137.175 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 108-117.975 AERONAUTICAL RADIONAVIGATION 5.197 ADD 5.BA03 |  |  |

ADD	COM5/236/2

5.BA03	The band 108-117.975 MHz may also be used by the aeronautical mobile (R) service on a primary basis, limited to systems that transmit navigational information in support of air navigation and surveillance functions in accordance with recognized international aviation standards. Such use shall be in accordance with Resolution [COM5/2] (WRC-03) and shall not cause harmful interference to nor claim protection from stations operating in the aeronautical radionavigation service which operate in accordance with international aeronautical standards.

ADD	RES. [COM5/2]	COM5/236/3

draft  RESOLUTION  [COM5/2]  (WRC-03)

Use of the band 108-117.975 MHz by aeronautical services

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	the current allocation of the frequency band 108-117.975 MHz to the aeronautical radionavigation service;

b)	the current requirements of FM broadcasting systems operating in the frequency band 87-108 MHz;

c)	that digital sound broadcasting systems are capable of operating in the frequency band at about 87-108 MHz as described in Recommendation ITU-R BS.1114;

d)	the need for the aeronautical community to provide additional services in order to enhance navigation and surveillance systems through a communication data link;

e)	the need for the broadcasting community to provide digital terrestrial sound broadcasting services,

recognizing

a)	that precedence must be given to the aeronautical radionavigation service operating in the frequency band 108-117.975 MHz;

b)	that, in accordance with Annex 10 of the Convention of the International Civil Aviation Organization (ICAO), all aeronautical systems must meet Standards and Recommended Practices (SARPs) requirements;

c)	that within ITU-R, compatibility criteria between FM broadcasting systems operating in the frequency band 87-108 MHz and the aeronautical radionavigation service operating in the frequency band 108-117.975 MHz already exist as indicated in the most recent version of Recommendation ITU-R SM.1009;

d)	that all compatibility issues between FM broadcasting systems and ICAO standard ground-based systems for the transmission of radionavigation-satellite differential correction signals have been addressed,

noting

a)	that aeronautical systems are converging towards a communication data link environment to support aeronautical navigation and surveillance functions, which need to be accommodated in existing radio spectrum;

b)	that some administrations are planning to introduce digital sound broadcasting systems in the frequency band at about 87-108 MHz;

c)	that no compatibility criteria currently exist between FM broadcasting systems operating in the frequency band 87-108 MHz and the planned additional aeronautical systems in the adjacent band 108-117.975 MHz using aircraft transmission;

d)	that no compatibility criteria currently exist between digital sound broadcasting systems capable of operating in the frequency band at about 87-108 MHz and aeronautical services in the band 108-117.975 MHz;

e)	that surveillance functions include the observation of aircraft location, velocity and weather conditions for the purpose of air traffic control and situational awareness/collision avoidance between aircraft,

resolves

1	that the provisions of this Resolution and of No. 5.BA03 shall enter into force on [4 July 2003];

2	that any additional aeronautical systems planned to operate in the frequency band 108-117.975 MHz shall, as a minimum, meet the FM broadcasting immunity requirements contained in Annex 10 of the Convention of the International Civil Aviation Organization (ICAO) for existing aeronautical radionavigation systems operating in this frequency band;

3	that additional aeronautical systems operating in the band 108-117.975 MHz shall place no additional constraints on the broadcasting service or cause harmful interference to stations operating in the bands allocated to the broadcasting service in the frequency range 87-108 MHz;

4	that frequencies below 112 MHz shall not be used for these additional aeronautical systems excluding the ICAO systems identified in recognizing d) until all potential compatibility issues with the lower adjacent frequency band 87-108 MHz have been resolved,

invites ITU-R

1	to study any compatibility issues between the broadcasting and aeronautical services that may arise from the introduction of these additional aeronautical systems as referenced in noting b), or appropriate digital sound broadcasting systems, as described in Recommendation ITU-R BS.1114 and to develop new or revised ITU-R Recommendations as appropriate,

requests the Secretary-General

to bring this Resolution to the attention of ICAO.

_________________

The term “additional aeronautical systems” refers to systems that transmit navigational information in support of air navigation and surveillance functions in accordance with recognized international aviation standards.