| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 271-E |
|  |  |  | 24 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

| Source: Documents DT/54, DT/83 | COMMITTEE 4 |
| --- | --- |
| Working Group 4A |  |
| SIXTH REPORT FROM WORKING GROUP 4A TO COMMITTEE 4 |  |
| AGENDA ITEM 1.8.1 |  |

The following text, prepared by Working Group 4A, on agenda item 1.8.1 (Appendix 3) is presented for consideration by Committee 4.

ADD

1.146bis		out-of-band domain (of an emission):  The frequency range, immediately outside the necessary bandwidth but excluding the spurious domain, in which out-of-band emissions generally predominate. Out-of-band emissions, defined based on their source, occur in the out-of-band domain and, to a lesser extent, in the spurious domain. Spurious emissions likewise may occur in the out-of-band domain as well as in the spurious domain.

ADD

1.146ter		spurious domain (of an emission):  The frequency range beyond the out-of-band domain in which spurious emissions generally predominate.

MOD

3.6		Transmitting stations shall conform to the maximum permitted power levels for spurious emissions or for unwanted emissions in the spurious domain specified in Appendix 3.

MOD

3.7		Transmitting stations shall conform to the maximum permitted power levels for out-of-band emissions, or unwanted emissions in the out-of-band domain, specified for certain services and classes of emission in the present Regulations. In the absence of such specified maximum permitted power levels transmitting stations should, to the maximum extent possible, satisfy the requirements relating to the limitation of the out-of-band emissions, or unwanted emissions in the out-of-band domain, specified in the most recent ITU-R Recommendations (see Resolution 27 (Rev.WRC-2000)*).

_______________

* 	Note by the Secretariat:  Under consideration by this WRC.