| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 275-E |
|  |  |  | 24 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 5 |
| --- | --- |
| fourth report from working group 5d to committee 5 |  |
| agenda item 1.5 |  |

The following report on agenda item 1.5 is presented to Committee 5 for consideration.

A. HASHIMOTO
	Chairman, Working Group 5D, Box 5

ARTICLE 5

MOD

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD03 5.446 MOD 5.447 5.447B 5.447C |  |  |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD04 5.448 MOD 5.448A |  |  |
| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD04 5.448 MOD 5.448A |  |  |
| 5 350-5 460 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B SPACE RESEARCH (active) AERONAUTICAL RADIONAVIGATION 5.449 RADIOLOCATION ADD 5.BD05 |  |  |
| 5 460-5 470 RADIONAVIGATION 5.449 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B SPACE RESEARCH (active) RADIOLOCATION ADD 5.BD05 |  |  |
| 5 470-5 570 MARITIME RADIONAVIGATION MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD07 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B SPACE RESEARCH (active) RADIOLOCATION ADD 5.BD06 5.450 5.451 5.452 |  |  |
| 5 570-5 650 MARITIME RADIONAVIGATION MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD07 RADIOLOCATION ADD 5.BD06 5.450 5.451 5.452 |  |  |
| 5 650-5 725 RADIOLOCATION MOBILE except aeronautical mobile ADD 5.BD02 ADD 5.BD07 Amateur Space research (deep space) 5.282 5.451 5.453 5.454 5.455 ADD 5.BD08 ADD 5.BD10 |  |  |

| 5 725-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur | 5 725-5 830 RADIOLOCATION Amateur |
| --- | --- |
| 5.150 5.451 5.453 5.455 5.456 ADD 5.BD08 ADD 5.BD09 | 5.150 5.453 5.455 ADD 5.BD08 ADD 5.BD10 |

5 830-7 550 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) | 5 830-5 850 RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  |
| 5.150 5.451 5.453 5.455 5.456 ADD 5.BD08 ADD 5.BD09 | 5.150 5.453 5.455 ADD 5.BD08 ADD 5.BD09 |  |

ADD

5.BD02	The use of the bands 5 150-5 350 MHz and 5 470-5 725 MHz by the stations in the mobile service shall be in accordance with Resolution [COM5/16].     (WRC-03)

ADD

5.BD03	In the band 5 150-5 250 MHz, stations in the mobile service shall not claim protection from earth stations in the fixed-satellite service. The provisions of No. 5.43A do not apply to the mobile service with respect to FSS earth stations.     (WRC-03)

ADD

5.BD04	In the band 5 250-5 350 MHz, stations in the mobile service shall not claim protection from radiolocation, Earth exploration-satellite (active) and space research (active) services. Radiolocation and Earth exploration-satellite and space research (active) services shall not impose on the mobile service more stringent protection criteria, based on system characteristics and interference criteria, than those stated in Recommendations ITU-R M.1638 and ITU-R SA.1632, respectively.     (WRC-03)

MOD

5.448A	The Earth exploration-satellite (active) and space research (active) services in the frequency band 5 250-5 350 MHz shall not claim protection from the radiolocation service. The provisions of No. 5.43A do not apply.     (WRC-03)

MOD

5.448B	The earth exploration-satellite (active) service and space research (active) service operating in the band 5 350-5 570 MHz shall not cause harmful interference to the aeronautical radionavigation service in the band 5 350-5 460 MHz, the radionavigation service in the band 5 460-5 470 MHz and the maritime radionavigation service in the band 5 470-5 570 MHz.   (WRC-03)

ADD

5.BD05	In the frequency band 5 350-5 470 MHz, stations in the radiolocation service shall not cause harmful interference to, nor claim protection from, radar systems in the aeronautical radionavigation service operating in accordance with No. 5.449.     (WRC-03)

ADD

5.BD06	In the frequency band 5 470-5 650 MHz, stations in the radiolocation service, except ground-based radars used for meteorological purposes in the band 5 600-5 650 MHz, shall not cause harmful interference to, nor claim protection from, radar systems in the maritime radionavigation service.     (WRC-03)

ADD

5.BD07	In the band 5 470-5 725 MHz, stations in the mobile service shall not claim protection from radiodetermination services. Radiodetermination services shall not impose on the mobile service more stringent protection criteria, based on system characteristics and interference criteria, than those stated in Recommendation ITU-R M.1638.     (WRC-03)

MOD

5.447	Additional allocation:  in Israel, Jordan, Lebanon, Syria and Tunisia, the band 5 150-5 250 MHz is also allocated to the -aeronautical mobile service, on a primary basis, subject to agreement obtained under No. 9.21.     (WRC-03)

SUP

5.453	Additional allocation:  in Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, Cameroon, China, Congo, Korea (Rep. of), Egypt, the United Arab Emirates, Gabon, Guinea, India, Indonesia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kuwait, Lebanon, Libya, Madagascar, Malaysia, Nigeria, Oman, Pakistan, the Philippines, Qatar, Syria, the Dem. People’s Rep. of Korea, Singapore, Swaziland, Tanzania, Chad and Yemen, the band 5 650-5 850 MHz is also allocated to the fixed and mobile services on a primary basis.     (WRC-03)

ADD

5.BD08	Additional allocation:  In  [………..] the band 5 650-5 850 MHz is also allocated to the fixed service on a primary basis.     (WRC-03)

ADD

5.BD09	Additional allocation:  In [……..] the band 5 725-5 850 MHz is also allocated to the mobile service on a primary basis.     (WRC-03)

ADD

5.BD10	Additional allocation:  In […….] the band 5 650-5 725 MHz is also allocated to the aeronautical mobile service on a primary basis.     (WRC-03)

______________