| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 280-E |
|  |  |  | 25 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 4 |
| --- | --- |
| Chairman, Working Group 4B |  |
| SixTH REPORT FROM THE CHAIRMAN OF WORKING GROUP 4B TO COMMITTEE 4 ON Agenda Item 1.37 (EPFD Issues at 6/4 GHz Band) |  |

Working Group 4B considered at its eighth and ninth meetings the conclusions from Sub-Working Group 4B5 related to EPFD issues at 6/4 GHz band under agenda item 1.37. Working Group 4B adopted the modifications proposed to Article 22 as indicated in Annex 1 to this Document.

In addition, Working Group 4B decided that, with the adoption of the new Table 22-1E, the text “[22-1E]*” everywhere it appears in Resolution [COM4/6] (WRC-03) (see Annex 1 of Document 258) should now read “22-1E”, and the associated Editorial Note 1 should be deleted.

D. JANSKY
	Chairman, Working Group 4B, Box 46

Annex: 1

ANNEX 1

ARTICLE  22

Space services1

Section II  –  Control of interference to geostationary-satellite systems

MOD

22.5C	§ 6	1)	The equivalent power flux-density2, epfd, at any point on the Earth’s surface visible from the geostationary-satellite orbit, produced by emissions from all the space stations of a non-geostationary-satellite system in the fixed-satellite service in the frequency bands listed in Tables 22-1A to 22-1E, including emissions from a reflecting satellite, for all conditions and for all methods of modulation, shall not exceed the limits given in Tables 22-1A to 22-1E for the given percentages of time. These limits relate to the equivalent power flux-density which would be obtained under free-space propagation conditions, into a reference antenna and in the reference bandwidth specified in Tables 22-1A to 22-1E, for all pointing directions towards the geostationary-satellite orbit.     (WRC-03)

MOD

22.5CA		2)	The limits given in Tables 22-1A to 22-1E may be exceeded on the territory of any country whose administration has so agreed.     (WRC-03)

ADD

TABLE  22-1E

Limits to the epfd radiated by non-geostationary-satellite systems in
the fixed-satellite service in certain frequency bands12bis

| Frequency band (MHz) | epfd (dB(W/m2)) | Percentage of time during which epfd may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter and reference radiation pattern |
| --- | --- | --- | --- | --- |
| 3 700-4 200 |  | 100 | 4 | 1.8 m13ter |
|  |  | 100 | 4 | 2.4 m13ter |
|  |  | 100 | 4 | 3.7 m13ter |
|  |  | 100 | 4 | 4.5 m13ter |
|  |  | 100 | 4 | 5.5 m13ter |
|  |  | 100 | 4 | 8 m13ter |
|  |  | 100 | 4 | 10 m13ter |
|  |  | 100 | 4 | 15 m13ter |

_______________

13ter	The associated reference radiation pattern is defined as follows:

a)	for values of Dλ   100:

G(φ)  =  Gmax  –  2.5 × 10–3  Dλϕ2	for	0	   φ   <   φm

G(φ)  =  G1	for	φm	   φ   <   φr

G(φ)  =  29  –  25 log φ	for	φr	   φ   <   20°

G(φ)  =  –3.5	for	20°	   φ   <   26.3°

G(φ)  =  32  –  25 log φ	for	26.3°	   φ   <   48°

G(φ)  =  –10	for	48°	   φ      180°

where:

D:	antenna diameter; : wavelength, expressed in the same unit

φ:	off-axis angle of the antenna (degrees)

G1:	gain of the first side lobe  2  15 log  Dλ (dBi)

φm  20 λDGmax−G1 degrees

φr    15.85 Dλ−0.6degrees

Gmax  =  7.7 + 20 log Dλ (dBi)

b)	for values of 42  Dλ < 100:

G(φ)  =  Gmax  –  2.5 × 10–3  Dλϕ2	for	0	   φ   <   φm

G(φ)  =  G1	for	φm	   φ   <   100 λD

G(φ)  =  29  –  25 log φ	for  100 λD	   φ   <   20°

G(φ)  =  –3.5	for	20°	   φ   <   26.3°

G(φ)  =  32  –  25 log φ	for	26.3°	   φ   <   48°

G(φ)  =  –10	for	48°	   φ      180°

c)	for values of  Dλ    42:

G(φ)  =  Gmax  –  2.5 × 10–3  Dλϕ2	for	0	   φ   <   φm

G(φ)  =  G1	for	φm	   φ   <   100λD

G(φ)  =  32  –  25 log φ	for  100 λD	   φ   <   48°

G(φ)  =  –10	for	48°	   φ      180°

MOD

TABLE  22-2     (WRC-03)

Limits to the epfd radiated by non-geostationary-satellite systems in the
fixed-satellite service in certain frequency bands14, 16bis

| MOD Frequency band | epfd (dB(W/m2)) | Percentage of time epfd level may not be exceeded | Reference bandwidth (kHz) | Reference antenna beamwidth and reference radiation pattern15 |
| --- | --- | --- | --- | --- |
| 5 925-6 725 MHz | –183.0 | 100 | 4 | 1.5° Recommendation ITU-R S.672-4, Ls = –20 |
| 12.50-12.75 GHz 12.75-13.25 GHz 13.75-14.5 GHz | –160 | 100 | 40 | 4° Recommendation ITU-R S.672-4, Ls  –20 |
| 17.3-18.1 GHz (Regions 1 and 3) 17.8-18.1 GHz (Region 2) 16 | –160 | 100 | 40 | 4° Recommendation ITU-R S.672-4, Ls  –20 |
| 27.5-28.6 GHz | –162 | 100 | 40 | 1.55° Recommendation ITU-R S.672-4, Ls  –10 |
| 29.5-30 GHz | –162 | 100 | 40 | 1.55° Recommendation ITU-R S.672-4, Ls  –10 |

22.5H		5)	The limits specified in Nos. 22.5C to 22.5D and 22.5F apply to non-geostationary-satellite systems in the fixed-satellite service for which complete coordination or notification information, as appropriate, has been received by the Bureau after 22 November 1997. The limits specified in Table 22-1E and those specified in Table 22-2 for the frequency band 5 925-6 725 MHz apply to non-geostationary-satellite systems in the fixed-satellite service for which complete coordination or notification information, as appropriate, has been received by the Bureau after 5 July 2003. The limits in Tables 22-4A, 22-4A1, 22-4B and 22-4C do not apply to non-geostationary-satellite systems in the fixed-satellite service for which complete coordination or notification information, as appropriate, has been received by the Bureau before 22 November 1997.     (WRC-03)

____________