| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 333-E |
|  |  |  | 30 June 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | COMMITTEE 8 |
| --- | --- |
| TENth series of texts submitted by committee 6 to the editorial committee |  |

APPENDIX  30*  (WRC-03)

Provisions for all services and associated Plans and List** for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
             and 12.2-12.7 GHz (in Region 2)     (WRC-03)

(See Articles 9 and 11 of the Radio Regulations)

ARTICLE  1     (WRC-03)

General definitions

ADD	COM6/333/1

1.10	The broadcasting-satellite service subject to one of the Plans: The broadcasting-satellite service subject to one of the Plans referred to in this Appendix is the broadcasting-satellite service in the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3.

ARTICLE  2

Frequency bands

SUP	COM6/333/2

2.2

ADD	COM6/333/3

ARTICLE  2A   (WRC-03)

Use of the guardbands

2A.1	The use of the guardbands of this Appendix, as defined in § 3.9 of Annex 5, to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks in the broadcasting-satellite service (BSS) shall be coordinated with assignments of the BSS subject to a Plan using the provisions of Article 7 of the same Appendix.

2A.2	Coordination among assignments intended to provide the space operation functions and services not subject to a Plan shall be effected using the provisions of Nos. 9.7, 9.17, 9.18 and the associated provisions of Section II of Article 9, or § 4.1.1 d) or 4.2.3 d) of Article 4 of this Appendix, as appropriate. Advance publication information is not required. Coordination of modifications to the Region 2 Plan or assignments to be included in the Regions 1 and 3 List with assignments intended to provide these functions shall be effected using § 4.1.1 e) or 4.2.3 e) as appropriate, of Article 4 of this Appendix.

2A.3	Any assignment intended to provide these functions in support of a geostationary-satellite network in the BSS whose assignments are submitted under Article 4 of this Appendix shall be brought into use within the regulatory time-limit applicable to the corresponding BSS assignments submitted under Article 4 of this Appendix.

2A.4	Any assignment intended to provide these functions for the initial Plans (Region 2 Plans incorporated in the RR at WARC ORB-85 and the Regions 1 and 3 Plan adopted at WRC-2000), shall be brought into use within the regulatory time-limit referred to in § 4.1.3 or § 4.2.6 of this Appendix from the date of receipt by the Bureau of the complete Appendix 4 data.

2A.5	Assignments intended to provide these functions shall be notified under Article 11 of the Radio Regulations.

2A.6	Section II of Article 23 of the Radio Regulations does not apply to assignments in the guardbands intended to provide the above-mentioned functions.

ARTICLE  4     (WRC-03)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 32

MOD	COM6/333/4

_______________

2	The provisions of Resolution 49 (Rev.WRC-03) apply.

## 4.1	Provisions applicable to Regions 1 and 3

MOD	COM6/333/5

4.1.3	An administration, or one2bis acting on behalf of a group of named administrations, intending to include a new or modified assignment in the List shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. An assignment in the List shall lapse if it is not brought into use by that date.3 A proposed new or modified assignment not included in the List by that date shall also lapse3.

ADD	COM6/333/6

_______________

2bis  Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.

MOD	COM6/333/7

_______________

3	The provisions of Resolution 533 (Rev.WRC-2000) apply.

ADD	COM6/333/8

4.1.3bis	The regulatory time-limit for an assignment in the List may be extended once by no more than three years due to launch failure if the launch failure causes:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace the already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the regulatory time-limit3bis. In order to grant the administration this extension, the administration shall have, within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following to the Bureau before the regulatory time-limit of § 4.1.3:

–	date of launch failure;

–	due diligence information as required in Resolution 49 for the assignment with respect to the satellite that suffered the launch failure, if it has not been provided already.

Within one year of the request for extension, the administration shall provide to the Bureau updated Resolution 49 information for the new satellite under construction. Otherwise, the related frequency assignments shall lapse.

ADD	COM6/333/9

_______________

3bis  For launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as of 5 July 2003.

NOC	COM6/333/10

4.1.5	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected. The Bureau shall publish4, in a Special Section of its International Frequency Information Circular (BR IFIC), the complete information received under § 4.1.3, together with the names of the affected administrations, the corresponding fixed-satellite service networks, the corresponding broadcasting-satellite service assignments and terrestrial stations, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the assignment.

MOD	COM6/333/11

_______________

4	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than two months prior to the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

ADD	COM6/333/12

4.1.7bis	Except as provided under § 4.1.18 to 4.1.20, any inclusion of a new or modified frequency assignment in the Regions 1 and 3 List which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected.

NOC	COM6/333/13

4.1.10	An administration that has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of its BR IFIC referred to in § 4.1.5 shall be deemed to have agreed to the proposed assignment. This time-limit may be extended:

–	for an administration that has requested additional information under § 4.1.8, by up to three months; or

–	for an administration that has requested the assistance of the Bureau under § 4.1.21, by up to three months following the date at which the Bureau communicated the result of its action.

ADD	COM6/333/14

4.1.10bis	Thirty days prior to the expiry of the same four-month period, the Bureau shall dispatch a reminder telegram/fax to an administration which has not made its comments under § 4.1.10, bringing the matter to their attention.

ADD	COM6/333/15

4.1.10ter	After expiry of the deadline for comments in respect of the proposed assignment, the Bureau shall, according to its records, publish a Special Section indicating the list of administrations whose agreements are required for completion of the Article 4 procedure.

ADD	COM6/333/16

4.1.12bis	In application of § 4.1.12, an administration may indicate the changes to the information communicated to the Bureau under § 4.1.3 and published under § 4.1.5.

MOD	COM6/333/17

4.1.13	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the List, the assignment in question shall be maintained in the List until the end of the period referred to in § 4.1.3 above. After that date this assignment shall lapse unless the agreement of the administrations affected is renewed.

MOD	COM6/333/18

4.1.15	The Bureau shall publish4bis in a Special Section of its BR IFIC the information received under § 4.1.12, together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall be included in the List.

ADD	COM6/333/19

_______________

4bis	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than two months prior to the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

MOD	COM6/333/20

4.1.18	If, in spite of the application of § 4.1.16 and 4.1.17, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Regions 1 and 3 Plan, or in the Region 2 Plan or for which the procedure of § 4.2 of this Appendix has been initiated, and if the notifying administration insists that the proposed assignment be included in the Regions 1 and 3 List, the Bureau shall provisionally enter the assignment in the Regions 1 and 3 List with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the List only if the Bureau is informed that the new assignment in the Regions 1 and 3 List has been in use, together with the assignment which was the

basis for the disagreement, for at least four months without any complaint of harmful interference being made.

MOD	COM6/333/21

4.1.18bis	When requesting the application of § 4.1.18, the notifying administration shall undertake to meet the requirements of § 4.1.20 and provide to the administration in respect of which § 4.1.18 is applied, with a copy to the Bureau, a description of the steps by which it undertakes to meet these requirements. Once an assignment is entered in the List provisionally under the provisions of § 4.1.18, the calculation of the EPM4ter of an assignment in the Regions 1 and 3 List or for which the procedure of Article 4 of this Appendix has been initiated and which was the basis for the disagreement shall not take into account the interference produced by the assignment for which the provisions of § 4.1.18 have been applied.

ADD	COM6/333/22

_______________

4ter  For the definition of the equivalent protection margin, see § 3.4 of Annex 5.

NOC	COM6/333/23

4.1.19	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), or in § 4.1 (for assignments in the List or having initiated the procedure under § 4.1), as appropriate, then the status of the assignment in the List shall be reviewed accordingly.

NOC	COM6/333/24

4.1.20	Should harmful interference be caused by an assignment included in the List under § 4.1.18 to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

ADD	COM6/333/25

4.1.27bis	Should the assignments mentioned in § 4.1.26 and 4.1.27 over the national territory of the administration, not be brought into use within the regulatory time-limit mentioned in § 4.1.3, they would be retained in the List until the end of the World Radiocommunication Conference following immediately after the successful completion of procedure referred to in § 4.1.26 and 4.1.27 respectively, and thereafter they shall be removed from the List.

MOD	COM6/333/26

4.2.6	An administration, or one7bis acting on behalf of a group of named administrations, intending to make a modification to the Region 2 Plan shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. Modifications to that Plan shall lapse if the assignment is not brought into use by that date7ter. A request for modification not included in that Plan by that date shall also lapse7ter.

ADD	COM6/333/27

_______________

7bis  Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.

ADD	COM6/333/28

_______________

7ter  The provisions of Resolution 533 (Rev.WRC-2000) apply.

ADD	COM6/333/29

4.2.6bis	The regulatory time-limit for an assignment in the Region 2 Plan obtained through application of § 4.2 may be extended once by no more than three years due to launch failure if the launch failure causes:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace the already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the regulatory time-limit7quater. In order to grant the administration this extension, the administration shall have, within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following to the Bureau before the regulatory time-limit of § 4.2.6:

–	date of launch failure;

–	due diligence information as required in Resolution 49 for the assignment with respect to the satellite that suffered the launch failure, if it has not been provided already.

Within one year of the request for extension, the administration shall provide to the Bureau updated Resolution 49 information for the new satellite under construction. Otherwise, the related frequency assignments shall lapse.

ADD	COM6/333/30

_______________

7quater  For launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as of 5 July 2003.

NOC	COM6/333/31

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.3. The Bureau shall publish8, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks, the corresponding broadcasting-satellite service assignments and terrestrial stations, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the modification to the Region 2 Plan.

MOD	COM6/333/32

_______________

8	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than two months prior to the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

MOD	COM6/333/33

4.2.11	Except as provided under § 4.2.21.A to 4.2.21.D, any modification to a frequency assignment which is in conformity with the Region 2 Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected.

ADD	COM6/333/34

4.2.14bis	Thirty days prior to the expiry of the same four-month period the Bureau shall dispatch a reminder telegram/fax to an administration which has not made its comments under § 4.2.14, bringing the matter to their attention.

ADD	COM6/333/35

4.2.14ter	After expiry of the deadline for comments in respect of the proposed assignment, the Bureau shall, according to its records, publish a Special Section, indicating the list of administrations whose agreements are required for completion of the Article 4 procedure.

ADD	COM6/333/36

4.2.16bis	In application of § 4.2.16, an administration may indicate the changes to the information communicated to the Bureau under § 4.2.6 and published under § 4.2.8.

MOD	COM6/333/37

4.2.17	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the Plan, the assignment in question shall be maintained in the Plan until the end of the period referred to in § 4.2.6 above. After that date this assignment in the Plan shall lapse unless the agreement of the administrations affected is renewed.

MOD	COM6/333/38

4.2.19	The Bureau shall publish8bis in a Special Section of its BR IFIC the information received under § 4.2.16 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Region 2 Plan and will be considered as a frequency assignment in conformity with the Plan.

ADD	COM6/333/39

_______________

8bis  If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than two months prior to the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

ADD	COM6/333/40

4.2.21.A	If, in spite of the application of § 4.2.20 and 4.2.21, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Region 2 Plan, or in the Regions 1 and 3 Plan or List, or for which the procedure of § 4.1 or 4.2 of this Appendix has been initiated, and if the notifying administration insists that the proposed assignment be included in the Region 2 Plan, the Bureau shall provisionally enter the assignment in the Region 2 Plan with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the Region 2 Plan only if the Bureau is informed that the new assignment in the Region 2 Plan has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.

ADD	COM6/333/41

4.2.21.B	When requesting the application of § 4.2.21.A, the notifying administration shall undertake to meet the requirements of § 4.2.21.D and provide to the administration in respect of which § 4.2.21.A has been applied, with a copy to the Bureau, a description of the steps by which it undertakes to meet these requirements.

ADD	COM6/333/42

4.2.21.C	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44, the status of the assignment in the Region 2 Plan shall be reviewed accordingly.

ADD	COM6/333/43

4.2.21.D	Should harmful interference be caused by an assignment included in the Region 2 Plan under § 4.2.21.A to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the Region 2 Plan under § 4.2.21.A shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

ARTICLE  5

Notification, examination and recording in the Master International Frequency Register of frequency assignments to space stations 
in the broadcasting-satellite service

# 5.1	Notification

MOD 	COM6/333/44

5.1.1	Whenever an administration8ter intends to bring into use a frequency assignment to a space station in the broadcasting-satellite service, it shall notify this frequency assignment to the Bureau. For this purpose, the notifying administration shall apply the following provisions.

ADD	COM6/333/45

_______________

8ter  A frequency assignment may be notified by one administration acting on behalf of a group of named administrations. Any further notice (modification or deletion) relating to such an assignment shall, in the absence of information to the contrary, be regarded as having been submitted on behalf of the entire group.

ADD	COM6/333/46

5.1.2bis	In application of § 5.1.2 an administration may identify the characteristics of assignments in the Plans or the List as notification and send to the Bureau the changes thereto.

NOC	COM6/333/47

5.1.3	Each notice must reach the Bureau not earlier than three years before the date on which the frequency assignment is to be brought into use. In any case, the notice must reach the Bureau not later than three months before that date9.     (WRC-2000)

MOD	COM6/333/48

_______________

9  Where appropriate, the notifying administration shall initiate the procedure for modifying the Plan concerned or for including assignments in the Regions 1 and 3 List in sufficient time to ensure that this limit is observed. For Region 2, see also Resolution 42 (Rev.WRC-03) and § B of Annex 7.

MOD	COM6/333/49

### 5.2.1	The Bureau shall examine each notice:

a)	with respect to its conformity with the Constitution, the Convention and the relevant provisions of the Radio Regulations (with the exception of those relating to § b), c), d) and e) below);

b)	with respect to its conformity with the appropriate Regional Plan or the Regions 1 and 3 List, as appropriate; or

c)	with respect to the coordination requirements specified in the Remarks column of Article 10 or Article 11; or

d)	with respect to its conformity with the appropriate Regional Plan or the Regions 1 and 3 List, however, having characteristics differing from those in the appropriate Regional Plan or in the Regions 1 and 3 List, in one or more of the following aspects:

–	use of a reduced e.i.r.p.,

–	use of a reduced coverage area entirely situated within the coverage area appearing in the appropriate Regional Plan or in the Regions 1 and 3 List,

–	use of other modulating signals in accordance with the provisions of § 3.1.3 of Annex 5,

–	use of the assignment for transmission in the fixed-satellite service in accordance with No. 5.492,

–	in the case of Region 2, use of an orbital position under the conditions specified in § B of Annex 7;

–	in the case of the notification of Plan assignments, use of an e.i.r.p. which produces a pfd that exceeds the pfd limit of –103.6 dB(W/(m2 · 27 MHz)) given in Section 1 of Annex 1 to Appendix 30 on the territory of the notifying administration under the condition that the calculated pfd at test points of any Plan assignments, List assignments or proposed assignments submitted under Article 4 are equal to or below that of the original Plan assignments in the same channel of the administration applying this Section; or

e)	with respect to its conformity with the provisions of Resolution 42 (Rev.-WRC-03).

MOD	COM6/333/50

5.2.2.2	In the case of Region 2, where the Bureau reaches a favourable finding with respect to § 5.2.1 a) and 5.2.1 c), but an unfavourable finding with respect to § 5.2.1 b) and 5.2.1 d), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Rev.WRC-03). A frequency assignment for which the provisions of Resolution 42 (Rev.WRC-03) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Bureau shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Rev.WRC-03) and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments.

MOD	COM6/333/51

5.3.1	If an administration has not confirmed the bringing into use of a frequency assignment under § 5.2.8, the Bureau will make inquiries of the administration not earlier than six months after the expiry of the period specified in § 5.1.3. On receipt of the relevant information, the Bureau will either modify9bis the date of coming into use or cancel the entry.

ADD 	COM6/333/52

_______________

9bis  See also § 4.1.3 or 4.2.6 of Article 4.

ARTICLE  10

The Plan for the broadcasting-satellite service in
the frequency band 12.2-12.7 GHz in Region 2

10.2	TEXT  FOR  NOTES  IN  REMARKS COLUMN  OF  THE  PLAN

MOD	COM6/333/53

a)	The overall equivalent protection margin to be used for the application of Article 4 and Resolution 42 (Rev.WRC-03) shall be calculated on the following basis:

–	for the calculation of interference to assignments that are part of a group, only the interference contributions from assignments that are not part of the same group are to be included; and

–	for the calculation of interference from assignments belonging to a group to assignments that are not part of that same group, only the worst interference contribution from that group shall be used on a test point to test point basis.

APPENDIX  30A*  (WRC-03)

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 
and 11.7-12.2 GHz in Region 3) in the frequency bands 14.5-14.8 GHz2 
and 17.3-18.1 GHz in Regions 1 and 3,
           and 17.3-17.8 GHz in Region 2     (WRC-03)

(See Articles 9 and 11 of the Radio Regulations)

ARTICLE  1     (WRC-03)

General definitions

ADD	COM6/333/54

1.12	The BSS feeder-link subject to one of the Plans: The BSS feeder-link subject to one of the Plans referred to in this Appendix is the BSS feeder link in the frequency bands 14.5-14.8 GHz and 17.3-18.1 GHz in Regions 1 and 3 and 17.3-17.8 GHz in Region 2.

ARTICLE  2

Frequency bands

SUP	COM6/333/55

2.2

ADD	COM6/333/56

ARTICLE  2A   (WRC-03)

Use of the guardbands

2A.1	The use of the guardbands of this Appendix, as defined in § 3.1 and 4.1 of Annex 3, to provide space operation functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks for the broadcasting-satellite service (BSS) feeder link, shall be coordinated with assignments of the BSS feeder-link subject to a Plan using the provisions of Article 7 of the same Appendix.

2A.2	Coordination among assignments intended to provide the space operation functions and services not subject to a Plan shall be effected using the provisions of Nos. 9.7, 9.17, 9.17A, 9.18, and the associated provisions of Section II of Article 9, as appropriate. Advance publication information is not required. Coordination of modifications to the Region 2 feeder-link Plan or assignments to be included in the Regions 1 and 3 feeder-link List, with assignments intended to provide these functions shall be effected using § 4.1.1 d) of Article 4 of this Appendix.

2A.3	Any assignments intended to provide these functions in support of a geostationary-satellite network for the BSS feeder link whose assignments are submitted under Article 4 of this Appendix shall be brought into use within the regulatory time-limit of the corresponding BSS feeder-link assignments submitted under Article 4 of this Appendix.

2A.4	Any assignments intended to provide these functions for the initial Plans (Region 2 Plans incorporated in the RR at WARC ORB-85 and Regions 1 and 3 Plan adopted at WRC-2000), shall be brought into use within the regulatory time-limit referred to in § 4.1.3 or 4.2.6 of this Appendix from the date of receipt by the Bureau of the complete Appendix 4 data.

2A.5	Assignments intended to provide the above-mentioned functions shall be notified under Article 11 of the Radio Regulations.

ARTICLE  3     (WRC-03)

Execution of the provisions and associated Plans

MOD	COM6/333/57

3.3	The procedures for the use of interim systems in Region 2 for feeder-links in the fixed-satellite service for the bands covered by this Appendix are given in Resolution 42 (Rev.-WRC-03).

ARTICLE  4     (WRC-2000)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

## 4.1	Provisions applicable to Regions 1 and 3

MOD	COM6/333/58

4.1.1	An administration proposing to include a new or modified assignment in the feeder-link List shall seek the agreement of those administrations whose services are considered to be affected, i.e. administrations4, 4bis:

a)	of Regions 1 and 3 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is included in the Regions 1 and 3 feeder-link Plan with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a feeder-link frequency assignment included in the feeder-link List or for which complete Appendix 4 information has been received by the Radiocommunication Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is in conformity with the Region 2 feeder-link Plan, or in respect of which proposed modifications to that Plan have already been received by the Bureau in accordance with the provisions of § 4.2.6 with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

d)	having a feeder-link frequency assignment in the band 17.8-18.1 GHz in Region 2 in the fixed-satellite service (Earth-to-space) to a space station in the broadcasting-satellite service which is recorded in the Master Register or which has been coordinated or is being coordinated under the provisions of No. 9.7, or under § 7.1 of Article 7, with a necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment.

ADD	COM6/333/59

_______________

4bis  Coordination under 9.17 or 9.17A is not required for an earth station of an administration on the territory of which this earth station is located and for which the procedures of former § 4.2.1.2, and 4.2.1.3 of Appendix 30A (WRC-97) have been successfully applied by that administration before 3 June 2000 in respect of terrestrial stations or earth stations operating in the opposite direction of transmission.

MOD	COM6/333/60

4.1.3	An administration, or one4ter acting on behalf of a group of named administrations, intending to include a new or modified assignment in the feeder-link List shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. An assignment in the feeder-link List shall lapse if it is not brought into use by that date.5 A proposed new or modified assignment shall also lapse if it is not included in the List by that date5.

ADD	COM6/333/61

_______________

4ter  Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.

MOD	COM6/333/62

_______________

5 	The provisions of Resolution 533 (Rev.WRC-2000) apply.

ADD	COM6/333/63

4.1.3bis  the regulatory time-limit for an assignment in the List may be extended once by no more than three years due to launch failure if the launch failure causes:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace the already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the regulatory time-limit5bis. In order to grant the administration this extension, the administration shall have within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following to the Bureau before the regulatory time-limit of § 4.1.3:

–	date of launch failure;

–	due diligence information as required in Resolution 49 for the assignment with respect to the satellite that suffered the launch failure, if it has not been provided already.

Within one year of the request for extension, the administration shall provide to the Bureau updated Resolution 49 information for the new satellite under construction. Otherwise, the related frequency assignments shall lapse.

_______________

5bis  For launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as of 5 July 2003.

NOC	COM6/333/64

4.1.5	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected. The Bureau shall publish6, in a Special Section of its International Frequency Information Circular (BR IFIC), the complete information received under § 4.1.3, together with the names of the affected administrations, the corresponding fixed-satellite service networks and the corresponding feeder-links to broadcasting-satellite service assignments, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the assignment.

MOD	COM6/333/65

_______________

6 	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than two months prior to the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

ADD	COM6/333/66

4.1.7bis	Except as provided under § 4.1.18 to 4.1.20, any inclusion of a new or modified frequency assignment in the Regions 1 and 3 List which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all administrations whose services are considered to be affected.

NOC	COM6/333/67

4.1.10	An administration that has not notified its comments either to the administration seeking agreement or to the Bureau within a period of four months following the date of its BR IFIC referred to in § 4.1.5 shall be deemed to have agreed to the proposed assignment. This time-limit may be extended:

–	for an administration that has requested additional information under § 4.1.8, by up to three months, or

–	for an administration that has requested the assistance of the Bureau under § 4.1.21, by up to three months following the date at which the Bureau communicated the result of its action.

ADD	COM6/333/68

4.1.10bis	Thirty days prior to the expiry of the same four-month period, the Bureau shall dispatch a reminder telegram/fax to an administration which has not made its comments under § 4.1.10, bringing the matter to their attention.

ADD	COM6/333/69

4.1.10ter	After expiry of the deadline for comments in respect of the proposed assignment, the Bureau shall, according to its records, publish a Special Section, indicating the list of administrations whose agreements are required for completion of the Article 4 procedure.

ADD	COM6/333/70

4.1.12bis	In application of § 4.1.12, an administration may indicate the changes to the information communicated to the Bureau under § 4.1.3 and published under § 4.1.5.

MOD	COM6/333/71

4.1.13	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the List, the assignment in question shall be maintained in the List until the end of the period referred to in § 4.1.3 above. After that date this assignment shall lapse unless the agreement of the administrations affected is renewed.

MOD	COM6/333/72

4.1.15	The Bureau shall publish6bis in a Special Section of its BR IFIC the information received under § 4.1.12, together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall be included in the feeder-link List.

ADD	COM6/333/73

_______________

6bis  If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than two months prior to the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

MOD	COM6/333/74

4.1.18	If, in spite of the application of § 4.1.16 and 4.1.17, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Regions 1 and 3 Plan, or in the Region 2 Plan or for which the procedure of § 4.2 of this Appendix has been initiated, and if the notifying administration insists that the proposed assignment be included in the Regions 1 and 3 feeder-link List, the Bureau shall provisionally enter the assignment in the Regions 1 and 3 feeder-link List with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the feeder-link List only if the Bureau is informed that the new assignment in the Regions 1 and 3 feeder-link List has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.--

MOD	COM6/333/75

4.1.18bis	When requesting the application of § 4.1.18, the notifying administration shall undertake to meet the requirements of § 4.1.20 and provide to the administration in respect of which § 4.1.18 is applied, with a copy to the Bureau, a description of the steps by which it undertakes to

meet these requirements. Once an assignment is entered in the feeder-link List provisionally under the provisions of § 4.1.18, the calculation of the EPM6ter of an assignment in the Regions 1 and 3 feeder-link List or for which the procedure of Article 4 of this Appendix has been initiated and which was the basis for the disagreement shall not take into account the amount of interference produced by the assignment for which the provisions of § 4.1.18 have been applied.

ADD	COM6/333/76

_______________

6ter  For the definition of the equivalent protection margin, see § 1.7 of Annex 3.

MOD	COM6/333/77

4.1.19	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), or in § 4.1 (for assignments in the feeder-link List or having initiated the procedure under § 4.1), as appropriate, then the status of the assignment in the feeder-link List shall be reviewed accordingly.

MOD	COM6/333/78

4.1.20	Should harmful interference be caused by an assignment included in the feeder-link List under § 4.1.18 to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the feeder-link List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

ADD	COM6/333/79

4.1.27bis	Should the assignments mentioned in § 4.1.26 and 4.1.27 over the national territory of the administration not be brought into use within the regulatory time-limit mentioned in § 4.1.3, they would be retained in the List until the end of the World Radiocommunication Conference following immediately after the successful completion of procedure referred to in § 4.1.26 and 4.1.27, respectively and thereafter they shall be removed from the List.

MOD	COM6/333/80

4.2.1	When an administration intends to make a modification to the Region 2 feeder-link Plan, i.e.:

a)	to modify the characteristics of any of its frequency assignments in the fixed-satellite service which are shown in the Region 2 feeder-link Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

b)	to include in the Region 2 feeder-link Plan a new frequency assignment in the fixed-satellite service; or

c)	to cancel a frequency assignment in the fixed-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the Bureau (see Article 5 and Resolution 42 (Rev.WRC-03)).

MOD	COM6/333/81

4.2.2	An administration proposing a modification to the characteristics of a frequency assignment in conformity with the Region 2 feeder-link Plan, or the inclusion of a new frequency assignment in that Plan, shall seek the agreement of those administrations8, 8bis, 9:

a)	having an assignment for feeder-links in the fixed-satellite service (Earth-to-space) which is in conformity with the Regions 1 and 3 feeder-link Plan with the necessary bandwidth, any portion of which falls within the necessary bandwidth of the proposed assignment; or

b)	of Regions 1 and 3 having a feeder-link frequency assignment included in the feeder-link List or for which complete Appendix 4 information has been received by the Bureau in accordance with the provisions of § 4.1.3, and any portion of which falls within the necessary bandwidth of the proposed assignment; or

c)	of Region 2 having a feeder-link frequency assignment in the fixed-satellite service (Earth-to-space) in the same channel or an adjacent channel, which appears in the Region 2 feeder-link Plan or in respect of which proposed modifications to this Plan have been received by the Bureau in accordance with the provisions of § 4.2.6;

d)	which are considered affected.

ADD	COM6/333/82

_______________

8bis  Coordination under 9.17 or 9.17A is not required for an earth station of an administration on the territory of which this earth station is located and for which the procedures of former § 4.2.3.2 and 4.2.3.3 of Appendix 30A (WRC-97) have been successfully applied by that administration before 3 June 2000 in respect of terrestrial stations or earth stations operating in the opposite direction of transmission.

MOD	COM6/333/83

4.2.6	An administration, or one9bis acting on behalf of a group of named administrations, intending to make a modification to the Region 2 feeder-link Plan shall send to the Bureau, not earlier than eight years but preferably not later than two years before the date on which the assignment is to be brought into use, the relevant information listed in Appendix 4. Modifications to that Plan shall lapse if the assignment is not brought into use by that date9ter. A request for modification to that Plan shall also lapse if the assignment is not included in that Plan by that date9ter.

ADD	COM6/333/84

_______________

9bis  Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.

ADD	COM6/333/85

_______________

9ter  The provisions of Resolution 533 (Rev.WRC-2000) apply.

ADD	COM6/333/86

4.2.6bis	the regulatory time-limit for an assignment in the Region 2 Plan obtained through application of § 4.2 may be extended once by no more than three years due to launch failure if the launch failure causes:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace the already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the regulatory time-limit9quater. In order to grant the administration this extension, the administration shall have within one month of the launch failure or one month after 5 July 2003, whichever comes later, notified to the Bureau in writing of such failure, and shall also provide the following to the Bureau before the regulatory time-limit of § 4.2.6:

–	date of launch failure;

–	due diligence information as required in Resolution 49 for the assignment with respect to the satellite that suffered the launch failure, if it has not been provided already.

Within one year of the request for extension, the administration shall provide to BR updated Resolution 49 information for the new satellite under construction. Otherwise, the related frequency assignments shall lapse.

ADD	COM6/333/87

_______________

9quater  For launch failure which occurred before 5 July 2003, the maximum extension of three years shall apply as of 5 July 2003.

NOC	COM6/333/88

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.2. The Bureau shall publish10, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks and the corresponding feeder links to broadcasting-satellite service assignments, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the modification to the Region 2 feeder-link Plan.

MOD	COM6/333/89

_______________

10 	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than two months prior to

the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

MOD	COM6/333/90

4.2.11	Except as provided under § 4.2.21.A to 4.2.21.D, any modification to a frequency assignment which is in conformity with the Region 2 feeder-link Plan or any inclusion in that Plan of a new frequency assignment which would have the effect of exceeding the limits specified in Annex 1 shall be subject to the agreement of all affected administrations.

ADD	COM6/333/91

4.2.14bis	Thirty days prior to the expiry of the same four-month period the Bureau shall dispatch a reminder telegram/fax to an administration which has not made its comments under § 4.2.14, bringing the matter to their attention.

ADD	COM6/333/92

4.2.14ter	After expiry of the deadline for comments in respect of the proposed assignment, the Bureau shall, according to its records, publish a Special Section, indicating the list of administrations whose agreements are required for completion of the Article 4 procedure.

ADD	COM6/333/93

4.2.16bis	In application of § 4.1.16, an administration may indicate the changes to the information communicated to the Bureau under § 4.2.6 and published under § 4.2.8.

MOD	COM6/333/94

4.2.17	The agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires for an assignment in the Plan, the assignment in question shall be maintained in the Plan until the end of the period referred to in § 4.2.6 above. After that date this assignment in the Plan shall lapse unless the agreement of the administrations affected is renewed.

MOD	COM6/333/95

4.2.19	The Bureau shall publish10bis in a Special Section of its BR IFIC the information received under § 4.2.16 together with the names of any administrations with which the provisions of this Article have been successfully applied. The frequency assignment concerned shall enjoy the same status as those appearing in the Region 2 feeder-link Plan and will be considered as a frequency assignment in conformity with that Plan.

ADD	COM6/333/96

_______________

10bis	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The

Bureau shall send a reminder to the notifying administration, not later than two months prior to the deadline for the payment in accordance with Decision 482 unless the payment has already been received (see also Resolution [COM4/7] (WRC-03)).

ADD	COM6/333/97

4.2.21.A	If, in spite of the application of § 4.2.20 and 4.2.21, there is still continuing disagreement and the assignment which was the basis of the disagreement is not an assignment in the Region 2 feeder-link Plan, or in the Regions 1 and 3 feeder-link Plan or List, or for which the procedure of § 4.1 or 4.2 of this Appendix has been initiated, and if the notifying administration insists that the proposed assignment be included in the Region 2 feeder-link Plan, the Bureau shall provisionally enter the assignment in the Region 2 feeder-link Plan with an indication of those administrations whose assignments were the basis of the disagreement; however, the entry shall be changed from provisional to definitive recording in the feeder-link Plan only if the Bureau is informed that the new or modified assignment in the Region 2 feeder-link Plan has been in use, together with the assignment which was the basis for the disagreement, for at least four months without any complaint of harmful interference being made.

ADD	COM6/333/98

4.2.21.B	When requesting the application of § 4.2.21.A, the notifying administration shall undertake to meet the requirements of § 4.2.21.D and provide to the administration in respect of which § 4.2.21.A has been applied, with a copy to the Bureau, a description of the steps by which it undertakes to meet these requirements.

ADD	COM6/333/99

4.2.21.C	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44, the status of the assignment in the Region 2 feeder-link Plan shall be reviewed accordingly.

ADD	COM6/333/100

4.2.21.D	Should harmful interference be caused by an assignment included in the Region 2 feeder-link Plan under § 4.2.21.A to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the Region 2 feeder-link Plan under § 4.2.21.A shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

ARTICLE  5     (WRC-03)

Coordination, notification, examination and recording in the Master 
International Frequency Register of frequency assignments to
feeder-link transmitting earth stations and receiving
space stations in the fixed-satellite service11

MOD 	COM6/333/101

_______________

11 	Notification of assignments to transmitting feeder-link earth stations included in the Region 2 feeder-link Plan after 2 June 2000, or included in the feeder-link List, following successful application of Article 4, shall be effected applying the provisions of Article 11 of the Radio Regulations following completion of the Article 9 procedure of the Radio Regulations.

## 5.1	Coordination and notification

MOD	COM6/333/102

5.1.2	Whenever an administration11bis intends to bring into use a frequency assignment to a transmitting earth station or receiving space station in the fixed-satellite service in the bands between 14.5 GHz and 14.8 GHz and between 17.3 GHz and 18.1 GHz in Regions 1 and 3, and between 17.3 GHz and 17.8 GHz in Region 2, it shall notify this frequency assignment to the Bureau. For this purpose, the notifying administration shall apply the following provisions.

ADD	COM6/333/103

_______________

11bis  A frequency assignment to a space station or typical earth station as part of the satellite network may be notified by one administration acting on behalf of a group of named administrations. Any further notice (modification or deletion) relating to such an assignment shall, in the absence of information to the contrary, be regarded as having been submitted on behalf of the entire group.

ADD	COM6/333/104

5.1.2bis	Frequency assignments relating to a number of earth stations may be notified in the form of the characteristics of a typical earth station and the intended geographical area of operation. Individual notices of frequency assignments are however necessary in the case of earth stations whose coordination area includes the territory of another administration.

MOD	COM6/333/105

5.1.3	Before an administration in Region 1 or 3 notifies to the Bureau or brings into use any frequency assignment to a specific transmitting feeder-link earth station in the bands 14.5-14.8 GHz and 17.7-18.1 GHz with an e.i.r.p. greater than the sum of the values specified in columns 11 and 12 of the Regions 1 and 3 feeder-link Plan, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station using the method detailed in Appendix 7.

MOD	COM6/333/106

5.1.4	Before an administration in Region 1 or 3 notifies to the Bureau or brings into use any frequency assignment to a specific transmitting feeder-link earth station in the bands 14.5-14.8 GHz and 17.7-18.1 GHz, it shall effect coordination of this assignment with each administration whose territory lies wholly or partly within the coordination area of the planned earth station, using the method detailed in Appendix 7, in respect of notices concerning stations of the mobile and fixed services in the bands 14.5-14.8 GHz and 17.7-18.1 GHz and of the fixed-satellite service (space-to-Earth) in the band 17.7-18.1 GHz received by the Bureau prior to 3 June 2000 for recording in the International Master Frequency Register (Master Register) and subsequently recorded with a favourable finding11ter.

ADD	COM6/333/107

_______________

11ter  In cases where assignments from the WRC-97 Plans without Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna characteristics, the coordination status afforded by the WRC-97 Plans shall be preserved.

In cases where assignments from the WRC-97 Plans with Remarks were included in the WRC-2000 Regions 1 and 3 feeder-link Plan without change, or with conversion of modulation from analogue to digital, or a change from normal roll-off to fast roll-off antenna pattern, the compatibility shall be reassessed using the revised criteria and methodology in force and the Remarks of the WRC-97 Plans assignment shall either be maintained or reduced on the basis of the results of this analysis.

ADD	COM6/333/108

5.1.6bis	In application of § 5.1.2 an administration may identify the characteristics of assignments in the Plans or the List as notification and send to the Bureau the changes thereto.

## 5.2	Examination and recording

5.2.1	The Bureau shall examine each notice:

MOD	COM6/333/109

b)	with respect to its conformity with the appropriate Regional feeder-link Plan or the Regions 1 and 3 feeder-link List, as appropriate; or

MOD	COM6/333/110

d)	with respect to its conformity with the appropriate Regional feeder-link Plan or the Regions 1 and 3 feeder-link List, however, having characteristics differing from those in this Plan or in the Regions 1 and 3 feeder-link List in one or more of the following aspects:

–	use of a reduced e.i.r.p.,

–	use of a reduced coverage area entirely situated within the coverage area appearing in the Plan or in the Regions 1 and 3 feeder-link List,

–	use of other modulating signals in accordance with the provisions of § 3.1.3 to Annex 5 of Appendix 30,

–	in the case of Region 2, use of an orbital position under the conditions specified in § B of Annex 7 to Appendix 30,

–	in the case of Regions 1 and 3, use of the assignment for transmissions in the fixed-satellite service (Earth-to-space) other than for feeder links to the broadcasting-satellite service provided that such transmissions do not cause more interference, or require more protection from interference, than the feeder-link transmissions operating in conformity with the Plan or the List, as appropriate.

e)	for Region 2, with respect to its conformity with the provisions of Resolution 42 (Rev.-WRC-03);

SUP	COM6/333/111

_______________

MOD	COM6/333/112

5.2.2.2	In the case of Region 2, when the Bureau reaches a favourable finding with respect to § 5.2.1 a) and 5.2.1 c) but an unfavourable finding with respect to § 5.2.1 b) and 5.2.1 d), it shall examine the notice with respect to the successful application of the provisions of Resolution 42 (Rev.WRC-03). A frequency assignment for which the provisions of Resolution 42 (Rev.WRC-03) have been successfully applied shall be recorded in the Master Register with an appropriate symbol to indicate its interim status. The date of receipt of the notice by the Bureau shall be entered in Column 2d. In relations between administrations all frequency assignments brought into use following the successful application of the provisions of Resolution 42 (Rev.WRC-03) and recorded in the Master Register shall be considered to have the same status irrespective of the dates entered in Column 2d for such frequency assignments. If the finding with respect to § 5.2.1 e), where applicable, is unfavourable, the notice shall be returned immediately by airmail to the notifying administration.

MOD	COM6/333/113

5.3.1	If an administration has not confirmed the bringing into use of a frequency assignment under § 5.2.8, the Bureau will make inquiries of the administration not earlier than six months after the expiry of the period specified in § 5.1.3. On receipt of the relevant information, the Bureau will either modify12bis the date of coming into use or cancel the entry.

ADD	COM6/333/114

_______________

12bis  See also § 4.1.3 or § 4.2.6 of Article 4.

MOD	COM6/333/115

ARTICLE  6     (WRC-03)

Coordination, notification and recording in the Master International Frequency Register of frequency assignments to receiving terrestrial stations in Regions 1 and 3 in the bands 14.5-14.8 GHz and 17.7-18.1 GHz, and in Region 2 in the band 17.7-17.8 GHz, when frequency assignments to feeder-link transmitting earth stations for the broadcasting-satellite
service in conformity with the Regions 1 and 3 feeder-link Plan
or the Region 2 feeder-link Plan12ter are involved12quater

ADD	COM6/333/116

_______________

12ter  Only assignments included in the Region 2 feeder-link Plan before 3 June 2000 shall be taken into account.

12quater  These procedures do not replace the procedures prescribed for terrestrial stations in Articles 9 and 11 of the Radio Regulations.

MOD	COM6/333/117

ARTICLE  7     (WRC-03)

Coordination, notification and recording in the Master International Frequency Register of frequency assignments to stations in the fixed-satellite service (space-to-Earth) in Region 1, in the band 17.3-18.1 GHz,
 in Regions 2 and 3 in the band 17.7-18.1 GHz, 
to stations in the fixed-satellite service (Earth-to-space) in Region 2 in 
the band 17.8-18.1 GHz and to stations in the broadcasting-satellite 
service in Region 2 in the band 17.3-17.8 GHz when frequency 
assignments to feeder links for broadcasting-satellite stations 
in the 17.3-18.1 GHz band in Regions 1 and 3 or in the 
band 17.3-17.8 GHz in Region 2 are involved13bis

ADD	COM6/333/118

_______________

13bis  These provisions do not replace the procedures prescribed in Articles 9 and 11 of the Radio Regulations when stations other than those in the broadcasting-satellite service feeder-link subject to a Plan are involved.

Section I  –  Coordination of transmitting space or earth stations in the fixed-satellite 
service or transmitting space stations in the broadcasting-satellite service
with assignments to broadcasting-satellite service feeder links

MOD	COM6/333/119

7.1	The provisions of No. 9.714 and the associated provisions under Articles 9 and 11 are applicable to transmitting space stations in the fixed-satellite service in Region 1 in the band 17.3-18.1 GHz, to transmitting space stations in the fixed-satellite service in Regions 2 and 3 in the band 17.7-18.1 GHz, to transmitting earth stations in the fixed-satellite service in Region 2 in the band 17.8-18.1 GHz and to transmitting space stations in the broadcasting-satellite service in Region 2 in the band 17.3-17.8 GHz.

ARTICLE  9

Plan for feeder links for the broadcasting-satellite service in
the fixed-satellite service in the frequency band
17.3-17.8 GHz in Region 2

9.2	TEXT  FOR  NOTES  IN  REMARKS  COLUMN  OF  THE  PLAN

MOD	COM6/333/120

a)	The overall equivalent protection margin to be used for the application of Article 4 and Resolution 42 (Rev.WRC-03) shall be calculated on the following basis:

–	for the calculation of interference to assignments that are part of a group, only the interference contributions from assignments that are not part of the same group are to be included; and

–	for the calculation of interference from assignments belonging to a group of assignments that are not part of that same group, only the worst interference contribution from that group shall be used on a test point to test point basis.

__________